CONTENTS

EXECUTIVE SUMMARY . . . . . . . . . . . .   1

INTRODUCTION  . . . . . . . . . . . . . .   2

FACTORS AFFECTING SOFTWARE MUTABILITY . .   3

PROGRAM SIZE . . . . . . . . . . . .   3

THE PROCEDURAL PARADIGM AND THE

TIGHTLY COUPLED ARCHITECTURE  .   4

THE OBJECT ORIENTED PARADIGM AND

THE APPLICATION FRAMEWORK . . .   5

A BRIDGE FROM THE PROCEDURAL

TO THE OBJECT-ORIENTED PARADIGM  .   6

Data Encapsulation  . . . . . .   6

Inheritance . . . . . . . . . .   9

Polymorphism  . . . . . . . . .  10

POLYMORPHISM: ADDING AN ELEMENT TO

A FE PROGRAM  . . . . . . . . .  11

Inserting NewElement Into A PP

Program  . . . . . . . . .  12

Inserting NewElement Into An

OOP FE Program . . . . . .  12

UNBREAKABLE SOFTWARE . . . . . . . .  15

POLYMORPHISM: REPLACING THE SOLVER .  16

CONCLUDING REMARKS  . . . . . . . . . . .  16

REFERENCES  . . . . . . . . . . . . . . .  17

TABLE 1. A PARTIAL LIST OF THE TPROTEUS

OBJECT LIBRARY . . . . . . . . . . .  18

i

UNBREAKABLE FINITE ELEMENT PROGRAMS                         UNBREAKABLE FINITE ELEMENT PROGRAMS

by Peter N Roth, President

TCastle Software

EXECUTIVE SUMMARY                                  EXECUTIVE SUMMARY

Contemporary finite element programs are constructed using the

procedural paradigm and the tightly coupled architecture, and can

be very large. Mutation of this software is difficult, expensive,

and tedious. Smaller programs with equivalent capability can be

constructed with the Object-Oriented Paradigm (OOP) and the

application framework architecture. These programs are easier and

cheaper to modify and maintain.

The report opens with a discussion of the characteristics of

finite element programs. Then, using the example of "error

processing," the OOP concepts of data encapsulation and                                           __________________

inheritance are approximated in Fortran. This provides a bridge          ___________

from the procedural to the object-oriented paradigm.

The OOP concept of polymorphism is illustrated with a four-step                             ____________

approach:

- TProteus, the PC-based object-oriented Application                 TProteus,

Framework for Finite Elements is presented. This software,

and its comprehensive engineering library, was developed to

explore OOP.

- the efforts necessary to add an element to a procedural

program, and to TProteus, are compared.

- the inner workings of TProteus are revealed to show that

polymorphism produces "unbreakable software."

- the openness and security of the TProteus system is

demonstrated by changing the equation solver without

accessing the source code.

Because OOP programs are mutable and unbreakable, we conclude by

recommending the paradigm as the new standard for finite element

software.

1

INTRODUCTION                                     INTRODUCTION

Modern finite element texts (e.g., [1],[2],[3] ) often

present working programs in an appendix. The programs bear many

similarities: memory is "managed" by allocating storage from a

COMMON pool, and input data are delimited by keywords. The

programs are well-commented, are built using the procedural

paradigm (PP),  and are at most 6,000 lines of Fortran (the

dominant PP engineering language). The student is invited to

modify the program to suit his needs, and to carry the program

with him into his FE career. The commercial finite element

programs of the author's acquaintance are built along similar

lines.

The book-programs are not without caveats. In remarks on the

presentation of their book program pcFEAP [1], the authors write:                                             pcFEAP

"In present day practice there are many complex and

efficient system programs for finite element analysis

capable of dealing with very large numbers of variables and

formulations. The very complexity of such systems means that

it is difficult to update them in order to introduce new

developments of technology."

and later on,

"In practical engineering problems fixed algorithm programs

are often too restrictive and the user must continually

modify the program to solve a given problem - often at the

expense of another user! For this reason it is desirable to

have a program that has modules for variable algorithm

capabilities and, if necessary, can be modified without

interrupting other users' capabilities."

The commercial version of this difficulty is exemplified by the

engineer who enhances a program, and then learns that the next

"official" version of the program is incompatible with his

enhancement.

So part of the finite element lore is: software modification is

difficult. One may spend more time "fixing the program" than

analyzing the phenomenon of interest, which can lead to

expressions of frustration such as found in [2]:

____________________

Bracketed numbers indicate references listed at the end of

the paper.

In the Procedural Paradigm, data are passive entities that

are "processed" according to a fixed algorithm.

2

"Debugging is boring and frustrating, though the use of

interactive debug utilities can reduce the tedium. When you

are feeling particularly depressed by this experience,

remember everyone who has ever programmed has experienced                        ________

what you are feeling now. Misery loves company."

To bring more joy into the finite element arena, our programs

must be engineered  to meet our stringent numerical requirements

and provide the capability for large and complex analyses. But:

they must also be pleasantly mutable, whilst remaining

unbreakable. We address these additional two goals in this

report.

FACTORS AFFECTING SOFTWARE MUTABILITY                        FACTORS AFFECTING SOFTWARE MUTABILITY

The three major factors that affect the mutability of programs

are: the program size, the programming paradigm used to construct

the program, and, the program architecture. Because they are

inextricable, the programming paradigm and the architecture are

discussed together.

PROGRAM SIZE          PROGRAM SIZE

Size determines in large part the mental effort required to grasp

a program.

There are several ways to gauge program size; PP programs are

often measured by lines of code, a controversial criterion                            _____________

because it is rarely defined accurately. For example: does one

count comments? erroneous lines that have been removed?

scaffolding code that has been stripped away? debugging code? all

the lines in a COMMON statement every time it is included, or

only once? etc. Nonetheless, because it is familiar, we use the

lines-of-code measure below, if only in a qualitative sense.

The threshold of incomprehension appears to be somewhere around

10,000 lines for PP programs . When programs exceed this

threshold, the possibility of understanding the complete program

diminishes substantially. It is no accident that the

book-programs are all less than 6,000 lines of code!

Although discussion of complexity measures is beyond the scope of

this report, one program size measure derived from the study of

software complexity is worth noting in passing: the name-space

size. The name-space is a measure of the uniqueness and frequency

____________________

Programs containing fewer than 1,000 lines are of no

interest because they can be forced to work while every rule of

programming style is violated.

3

of use of program vocabulary. According to the hypothesis, the

larger the name-space, the more difficult the program is to

understand. Whereas PP programs require a different name for each

procedure and variable, OOP programs re-use procedure names and

hide many of the variables. By this measure, OOP programs are

"smaller" and easier to understand than the equivalent PP

programs.

THE PROCEDURAL PARADIGM AND THE TIGHTLY COUPLED ARCHITECTURE          THE PROCEDURAL PARADIGM AND THE TIGHTLY COUPLED ARCHITECTURE

Fortran  program architecture is ostensibly a top-down

procedural hierarchy in which driver procedures invoke servant

routines to perform a computation. The programs are tightly

coupled in three ways: through CALLs, COMMONs, and FILEs. By

tight coupling is meant the amount of programming effort required

to establish or break a connection between parts of a program.

CALL coupling is the loosest, and the easiest link to make or

break. That is, a subroutine is either called or not, and the

coupling is determined by a single program statement. Operating

system provided linker programs produce cross-reference maps that

show whence each subroutine is called, so the couplings are easy

to find, if not obvious. This is the "procedural" part of the PP,

and is the part that is relatively easy to change. The difficulty

arises with the data.

COMMON coupling is tighter than CALL coupling because the data

are passive and the software acreage to be observed is great. A

single statement may determine the presence of a COMMON in a

subroutine, but the access to the variables in the COMMON may

extend over the entire routine. Moreover, the presence of a

COMMON in a subroutine may be unnecessary, perhaps left over from

a previous modification; a programmer must spend time and effort

to make this determination. Changes to COMMON storage must be

made consistently at all accesses to the block. Coupling is

looser when COMMON is the same everywhere it occurs in a program

(made so by the use of the "include" directive). Instances of

COMMON in which the variable definitions differ force a

programmer to examine every instance.

FILE coupling is the tightest coupling in Fortran programs

because files are accessible from any subroutine without explicit

notice. File use can seem invisible; wily programmers will at

least flag file access in a subroutine with reference to a COMMON

carrying the unit number. Files are often accessed as serial

storage devices. Any change in the sequence of data written to a

____________________

Since Fortran is the dominant PP language, we will confine

our PP linguistic remarks to Fortran for the remainder of the

paper.

4

file must be compensated by matching changes to code that reads

the file. A single change ripples through not only the program,

but also through pre- and post-processor software.

These characteristics make it extremely difficult to flawlessly

mutate large Fortran programs. Hence, the reluctance on the part

of many engineers to get involved in the first place, and the

reluctance on the part of the developer to let anyone else have

access to the code.

THE OBJECT ORIENTED PARADIGM AND THE APPLICATION FRAMEWORK          THE OBJECT ORIENTED PARADIGM AND THE APPLICATION FRAMEWORK

For the same computational capabilities, OOP programs are smaller

and more understandable than their procedural counterparts. This

allows one to enhance large OO programs easily because each part

can be studied on its own, as an independent unit (which is what

an object is). It is no longer unreasonable for a solo

investigator to cope with more than 25,000 lines of code.[4]

Although the object-oriented paradigm extends the concept of

structured programming, the programs are themselves thought of as

a very loosely coupled collection of objects. They are sometimes

coupled by CALLs, but even this concept is loosened to one of

"messages." COMMON  coupling doesn't exist, and FILE coupling is

minimized by the requirement that each object be responsible for

its own input and output.

As in the procedural paradigm, the action of a subroutine call

(termed a "method invocation" in OO parlance) is noted by its

presence in another method. However, because the coupling of

objects is so loose, the controlling object sometimes will "send

messages" to the rest of the program. Objects that are programmed

to respond to the messages do so, otherwise the message is

ignored. In Fortran, one might approximate message passing as

follows: Let the "message" be the integer variable "MSG". Set its

value to that for a specific action, such as "read quadrilateral

element input". Call every subroutine in the program (!) using

MSG as an argument. The subroutine at which the message is

"aimed" performs the action associated with the variable, and

clears the message to a neutral value.

Of course, Fortran programs "know" where all their subroutines

are, so the subroutine to do the work can be called directly. OO

programs are instead built with "late binding," which allows

messages that were not conceived of at the construction of the

program to be passed around to objects that were not connected to

the original program. That is, even though the program itself is

changed, the "message passing component" of OO programs is

constant. The "efficiency" of knowing where all the subroutines          ________

are is explicitly traded off for the advantage of loose coupling.

5

Although this may appear chaotic, OO programs are easier to

understand because one simply assumes that the message will get

to the right place without having to set up the linkages

personally. The programmer can relax and let the system do the

work. And while source code access is necessary for a PP

programmer to enhance a program, another marvel of the OO

paradigm is that a program may be changed in dramatic ways

without source code access.          _______

OOP programs may be built in many forms; the Application

Framework has been proposed [5] for FE work as immanently

mutable and loosely coupled . The application framework

establishes a common structure into which the programmer fits

code as building blocks. Although in and of themselves

application frameworks do little to nothing, they provide a

re-usable program design.                            _______

To evaluate the OOP concepts, we built TProteus, the PC-based                                                 TProteus

object-oriented Application Framework for Finite Elements. In

order to make TProteus useful, we needed to build a comprehensive

library of objects dealing with engineering computations in

general, and finite elements in particular. The first release of

the system allows the construction and solution of more than 8000

double precision simultaneous equations, and the capability to

quickly develop innovative computational schemes, on a PC. This

provides a rich and flexible environment for finite element

experimentation and analysis for researchers with limited

budgets. A subset of the object library is shown in Table 1 (at

the end of the report).

A BRIDGE FROM THE PROCEDURAL                             A BRIDGE FROM THE PROCEDURAL

TO THE OBJECT-ORIENTED PARADIGM                           TO THE OBJECT-ORIENTED PARADIGM

In this section, we present an approximation of the major OOP

concepts in Fortran with a view to providing an explication of

the paradigm without over-involvement in linguistic training.

Although there is some disagreement about what constitutes "true"

Object Oriented Programming, the technique includes at least data                                                                       ____

encapsulation, inheritance, and polymorphism. While an OOP          _____________  ___________      ____________

language institutionalizes the concepts, they may be approximated

in Fortran only by dint of discipline. We define the ideas below,

giving illustrations in Fortran where possible, and in Borland

Pascal  [6]. For more details of OOP in Fortran, see [7]

and [8].

Data Encapsulation means that the data are concealed from view.          Data Encapsulation

Data encapsulation decouples programs, and therefore makes them

easier to change.

6

For example, suppose that the following COMMON, contained in the

file error.h manages the number of input errors:

common / error / nwarn

integer nwarn          ! # of input warning errors

Fortran programmers might use the include directive to insert the

COMMON wherever error processing is needed. The object-oriented

alternative is to encapsulate the data within a series of

subroutines, perhaps all in the same source file, say, error.ftn.

Then, one constructs a "Fortran error object" like this:

subroutine ierror

c      initialize error object

include error.h

nwarn = 0

end

integer function gwarn()

c      "get" the value of "nwarn"

include error.h

gwarn = nwarn

end

subroutine iwarn

c      increment "nwarn"

include error.h

nwarn = nwarn + 1

end

where the comments indicate the purposes of each subroutine. We

call the routines that access the data, plus the data itself, an

object. The only subroutines that directly access the error data          ______      ____

are the error object routines, so the data are said to "belong"

to the error object. A program using the error object only refers                                                                ____

to the functional interface. Rather than pass COMMON /error/ to

those routines that need error processing, calls are made to

iwarn, as in          _____

if ( X .LE. 0.0 ) then

call iwarn

endif

Hence, the tight COMMON coupling is exchanged for the much looser

CALL coupling. Granted, calling a function or subroutine costs

execution time, but we choose this trade-off explicitly for the

freedom it brings to program modification.

A Borland Pascal implementation of error processing is similar.

In this OO language, the data are stored in fields, and the                                                      ______

procedures that manipulate the data are methods. There are four                                                  _______

steps necessary to the coding of an object: the declaration of

7

the object as a TYPE ("here's what it looks like"), the

definition of the methods ("here's how it's done"), the

declaration of a variable of the object's type ("here's where it

will be, if you make one"), and finally, the use of the object.

Type Declaration:

TYPE

ErrorObjectType = OBJECT

nwarn : INTEGER ;

PROCEDURE Init ;

FUNCTION  HowMany : INTEGER ;

PROCEDURE Increment ;

END ;

Method Definition:

PROCEDURE ErrorObjectType.Init ;

BEGIN

nwarn := 0 ;

END ;

FUNCTION ErrorObjectType.HowMany : INTEGER ;

BEGIN

HowMany := nwarn ;

END ;

PROCEDURE ErrorObjectType.Increment ;

BEGIN

nwarn := nwarn + 1 ;

END ;

Variable declaration:

VAR

eo : ErrorObjectType ;

The only code that deals with the errors is the object eo, in

statements of the form

eo.Init ;

to construct the object,

eo.Increment ;

to bump the error count, and

Writeln( eo.HowMany, ' errors found.' ) ;

to report the current error count. Thus, data encapsulation in                                                   __________________

8

the two paradigms is similar.

Inheritance is that property of an object that allows it to have          Inheritance

all the properties of its ancestor, plus all of its own

properties. For example, we may decide we need to process the

"toxic" error, an error type that eventually causes a program to

die from consuming too much bad data.

Using the Fortran error object, we decide to add another variable

to error.h to monitor toxic errors:

common / error / nwarn, ntoxic

integer nwarn    ! # of input warning errors

integer ntoxic   ! # of toxic errors

Add two new routines to error.ftn

integer function gtoxic()

c      "get" the value of "ntoxic"

include error.h

gtoxic = ntoxic

end

subroutine itoxic

c      increment "ntoxic"

include error.h

ntoxic = ntoxic + 1

end

Finally, add the statement ntoxic = 0 to the ierror subroutine.

By keeping the changes to the error object alone, none of the

routines that already access error processing via calls to iwarn                                                                     _____

need be checked for interference with the new variable ntoxic. If                                                                 ______

the coupling is made via COMMON /error/, however, each routine

that is so coupled must be certified not to have a variable

"clash" with ntoxic.                       ______

The toxic error processing might be said to "inherit" the

properties of the extant error object. By isolating our error

processing, the change requires the editing and compilation of

the error object only (a very small portion of the program), and                           ____

a re-link. However, the change does require modification of                                         ____

verified code (the error object). The enhancement has brought the

previously trusted error object under suspicion, simply because

it has been changed.

9

A more desirable situation prevails in OOP languages, where

inheritance is explicit, and working code need not be modified.                                                         ___

For example, if object B is a descendent of object A, Borland

Pascal would use the form

B = Object( A )

to explicitly declare that B inherits everything from A. Rather

than modifying certified code, we extend it by deriving a new

object, perhaps like this:

BiggerErrorObjectType   = OBJECT( ErrorObjectType )

ntoxic : INTEGER ;

PROCEDURE Init ;

FUNCTION  Toxic : INTEGER ;

FUNCTION  IncrementToxic ;

END ;

and simply change the declaration of eo:

VAR

eo : BiggerErrorObjectType ;

The new eo now has the Init, Toxic, and IncrementToxic methods of

BiggerErrorObject, and inherits the Init, HowMany, and Increment

methods of the parent object type, ErrorObjectType. Although we

still need to write the new methods, the verified code doesn't                                               _________________________

have to be touched. This capability vastly simplifies program          __________________

mutation by limiting the amount of code to be examined. Note also

the re-use of the procedure name Init in the new object                                           ____

declaration: by using consistent naming in derived objects, the

name-space remains small.

Fortran programmers have no direct inheritance facility at the

source code level. Most of us who write Fortran would probably

think of inheritance as "using a subroutine library". But using a

subroutine library occurs at the machine code level rather than

at the source code level, so the inheritance relationship among

Fortran subroutines can sometimes be very difficult to grasp, if

it exists at all.

Polymorphism may be defined as a single action that has a          Polymorphism

different effect on different objects. As implemented,

polymorphism  usually means "an action with the same name" that

acts differently on different objects. This continues the

reasoning that using the same name makes a program's vocabulary

smaller and therefore easier to understand.

10

A Fortran example of polymorphism:

program eg1

integer a, b, c

real  x, y, z

...

a = MAX( b, c )

x = MAX( y, z )

print *, a, x

end

In this fragment, x, y, z are real variables, and a, b, c are

integer variables. Yet, the Fortran print statement, the =

operator, and the generic MAX function can easily handle both

types of data. I.e., print, =, and MAX can be called polymorphic.

Fortran programmers can use the polymorphic aspects of the

language, but they can't directly implement polymorphic

procedures. It can be approximated by providing a flag variable

as a subroutine argument that selects the behavioral variant

desired from a list of possibilities. However, every time a new

flag is added, the modules that know about the flag must be

examined and "fixed," and once again, verified code becomes

suspect.

On the other hand, the major work of the OO programmer is to

define new object types and then use variables (instances) of the

object type to perform the desired calculation. Polymorphism

allows generic code to be written and applied to types that were

not conceived of by the original program author. In Fortran, this

would be equivalent to adding a subroutine that works on, say,

datafromMars, and expecting the MAX function to return the

appropriate value:

subroutine eg2

common / eg2com / a, b, c

datafromMars a, b, c    ! variable type declaration

a = MAX( b, c )

end

POLYMORPHISM: ADDING AN ELEMENT TO A FE PROGRAM          POLYMORPHISM: ADDING AN ELEMENT TO A FE PROGRAM

In this very different kind of patch test, the goal is to compare

the effort necessary to insert a new element (called NewElement)

into an existing finite element program. We'll make the following

assumptions: all the mathematics has been done, and the only work

left is to use the element as part of a larger computational

effort; the target program does 3D static analysis and uses

keyword blocked input data.

11

Inserting NewElement Into A PP Program          Inserting NewElement Into A PP Program

The programmer must examine and add another case to every IF-test

in all parts of the program that have branches on element type.

This suggests modifications of:

the input routines (where a new keyword would be added),

the memory allocation routines, if these are encapsulated

(else, a search across the entire program may be necessary);

the routine that calls for the formation of the stiffness

matrix, and supplying a routine that does it for NewElement;

the routines that call for the calculation of strain and

stress, and the provision of code to do so for the element;

all individual occurrences of COMMON storage that need be

changed (if include is not used);                           _______

intermediate file storage.

The major part of this effort is paleontological: digging up the

old code bones to make sure it isn't bebugged by our

improvements. Certainly, this can be done, and is done, on a

daily basis. But it's expensive, it wastes time and talent, and

it discourages novices who wish to contribute their new ideas to

a working program. The program grows (because nothing is ever

thrown away), every addition makes the next change even more

difficult, and the coupling gets tighter.

Inserting NewElement Into An OOP FE Program          Inserting NewElement Into An OOP FE Program

There is less effort required to develop NewElement in an OOP

environment because many desirable element properties can be

inherited from the element object hierarchy:

TBaseElement

TLinearElasticElement

TLinearElastic2DTruss

TLinearElastic3DTruss

other LE elements

other elements

One simply picks the point in the hierarchy at which new element

properties are desired, and the element inherits everything

"above" that point. Inheritable properties often include the

input methods, and methods to draw the element on graphics

displays, since there are many similarities among these element

"externals". NewElement methods for other computations are

developed by reusing code, following the same process used to

derive BigErrorObject from ErrorObject. New methods will most

12

likely be required to form the element stiffness matrix, to

calculate strain and stress, etc. Because of encapsulation and

inheritance, these methods are contained in the NewElement object

only, rather than being spread across the body of a program. In          ____

true OOP style, the element is not coupled to the program.

The TProteus object library provides the polymorphic element

storage object pElements, which can store any element derived                         _________                  ___

from TBaseElement. It maintains all the elements instantiated for

a particular analysis, and is responsible for "sending messages"

to each element at the appropriate time. As an example, the

formation of the global stiffness matrix from the polymorphic

elements is shown later.

After providing the necessary methods to the NewElement object,

connecting it to a program is accomplished in three steps:

declaration of the new program object, definition of the methods,

and definition of the new program. Following the OOP recipe, we

examine the program hierarchy for a place whence to derive our

new program:

TBaseFEObject

TFE2DObject

TStatic2DObject

static 2d programs

TDynamic2DObject

dynamic 2d programs

other 2D FE programs

TFE3DObject

TStatic3DObject

static 3d programs

TDynamic3DObject

dynamic 3d programs

other 3D FE programs

We choose to inherit from TStatic3DObject, so we declare the

program object type:

TYPE

TNewProg  = OBJECT ( TStatic3DObject ) ;

PROCEDURE HandleKeyWord

( VAR aKeyWord : KeyWordType ); VIRTUAL;

PROCEDURE ReadNewElement ;

END ;

Second, the definition of the object type methods (lines numbered

for convenience):

13

1 PROCEDURE    TNewProg.HandleKeyWord ( VAR aKeyWord :

KeyWordType ) ;

2   BEGIN

3     TStatic3DObject.HandleKeyword(aKeyword) ;

4     IF aKeyWord.What = 'NEWELEMENT' THEN BEGIN

5         ReadNewElement ;

6         ClearKeyWord( aKeyWord ) ;

7     END ;

8   END ;

9

10

11 PROCEDURE    TNewProgram.ReadNewElement ;

12   VAR icode, n : integer ;

13       pNE : PNewElement ;

14   BEGIN

15       pInputObj^.GetInt( n, icode ) ;

16       FOR n := 1 TO n DO BEGIN

17         pNE := New( PNewElement, Read ) ;

18         pElements^.Insert( pNE ) ;

19       END ;

20   END ;

In the HandleKeyWord method, we use all the inherited keywords                 _____________

that the TStatic3DObject recognizes by invoking its HandleKeyWord                                                              _____________

method at line 3. Keywords that are recognized will be "cleared"

by the parent object. We add the new capability to test for the

keyword 'NEWELEMENT' at line 4, and invoke the ReadNewElement                                                         ______________

method at line 5.

In the ReadNewElement method, we use (at line 15) the GetInt                 ______________                                 ______

method of the input parser to read the count n of NewElements to                                                       _

be read. The FOR loop over lines 16 through 19 then uses the                       ___

NewElement Read method to instantiate an element at line 17, and                     ____

the Insert method of the pElements collection to add the element              ______               _________

to the collection.

Finally, the code for the main program:

VAR aProg : TNewProg ;

BEGIN

aProg.Init ;

aProg.Run ;

aProg.Done ;

END .

Thus the work required to link NewElement into a TProteus

derivative is completed with fewer than 40 lines of code, all

within easy mental reach, without interfering with our

colleagues' work. The code enhancement is easy and safe, all

14

changes are obvious and independent of program size, and only the

element hierarchy need be examined for the qualities that

NewElement needs to inherit. The effort to install a new element

in TProteus is insignificant.

UNBREAKABLE SOFTWARE          UNBREAKABLE SOFTWARE

Since TStatic3DObject was not touched during our insertion of

NewElement, and none of the code for other elements was altered,

that code may be considered as constant or unbreakable software.                                         ________    ___________

All the methods of TStatic3DObject are inherited by TNewProg, so

the only code that needs to be supplied is that which is

pertinent to our needs. The program itself is not changed, but

extended.

The mechanism that allows uncoupled objects to do useful work is

provided via the polymorphism of TProteus and its objects.

Suspending some of the Borland Pascal syntax rules for purposes

of this example, the formation of the global stiffness matrix is

done with this code:

1 PROCEDURE TStatic3DObject.BuildModel ; VIRTUAL ;

2      BEGIN

3           {  ...after a few initializations  }

4           pElements^.ForEach( @FormStiffness ) ;

5      END ;

which is inherited from TBaseFEObject. At line 4, ForEach is an                                                            _______

iterator procedure inherited by pElements from its parent,

TCollection. The code in line 4 says: "for each element in the

collection, invoke the procedure FormStiffness."

The FormStiffness procedure is

1 PROCEDURE  FormStiffness( pBE:PBaseElement);FAR;

2    BEGIN

3       pBE^.FormStiff ;

4       MakeLocalToGlobalMap( pBE ) ;

5       AccumulateStiffness ;

6    END ;

This method takes one argument: pBE, a pointer to a TBaseElement

object. PBE will point in turn to each of the elements in the

pElements collection; that is, by the rules of polymorphism, pBE

may point to a TBaseElement or any of its derived descendents.                                      _________________________________

When pBE^ points to a NewElement at line 3, the statement

pBE^.FormStiff tells NewElement to execute its FormStiff method.                                                     ___

Upon completion of the FormStiff procedure, the program maps the

NewElement node numbers to the global degrees of freedom (line

4), and then accumulates the element stiffness matrix into the

global stiffness matrix (line 5). Similar iterators produce the

right hand side, mass and damping matrices, etc.

15

POLYMORPHISM: REPLACING THE SOLVER          POLYMORPHISM: REPLACING THE SOLVER

Although the introduction of new elements to an OOP program is a

simple matter, it is equally easy to replace any other part of

TProteus. For example, at the heart of TProteus is the Run                                                                 ___

method. Schematically, it is

PROCEDURE TProteus.Run ;

BEGIN

ReadInput ;

IF  Valid  THEN BEGIN

Solve ;

END ;

END ;

In order to experiment with a new finite element solution

technique, we add the fields necessary to contain the data, and

override the Solve method by deriving a new TProteus object:                       _____

TYPE

TTryNewTechnique    = OBJECT ( TProteus )

Eulerian : sometype ;

Lagrangian : anothertype ;

PROCEDURE Solve ; VIRTUAL ;

END ;

and then provide the code for the Solve method only. The Run                                            _____        ____      ___

method invokes our new Solve by polymorphism; the remainder of                                 _____

the software remains verified and unbroken.

CONCLUDING REMARKS                                  CONCLUDING REMARKS

We have shown that finite element software is easily mutable if

one builds programs using the object-oriented rather than the

procedural, paradigm. We have also shown that OOP programs are

unbreakable because verified code need not be changed to produce

extensions to a system.

The Object Oriented Paradigm delivers on the promise of open,

plug-in, FE software. OOP facilitates the exchange of good

software, removes collegial "interference," and lowers software

development and maintenance costs.

Innovation can be more reliably included in engineering

analytical efforts because new programs can be developed with

off-the-shelf components, and safe program changes can be made

quickly and cheaply.

Paradigms and software don't exist in a vacuum: the OOP requires

a change of mind, literally. Although many of the structured

programming techniques developed over the last two decades still

16

apply, the structure of OOP programs is much different from the

book-programs. Because of the advantages of OOP, however, we

recommend it as the new institutional model.

REFERENCES                                      REFERENCES

1 O C Zienkiewicz and R L Taylor, THE FINITE ELEMENT METHOD,

McGraw-Hill, 4th ed., 1989.

2 Thomas J R Hughes, THE FINITE ELEMENT METHOD, Prentice-Hall,

1987.

3 Robert D Cook, David S Malkus, and Michael E Plesha, CONCEPTS

AND APPLICATIONS OF FINITE ELEMENT ANALYSIS, John Wiley & Sons,

3rd ed., 1989.

4 Bjarne Stroustrup, THE C++ PROGRAMMING LANGUAGE, 2nd edition,

Addison-Wesley, New York, 1992, p.7.

5 Bruce W R Forde, Ricardo O Foschi, and Siegfried F Steimer,

"Object Oriented Finite Element Analysis", Computers &                                                     ___________

Structures, Vol 34 No 3, pp355-374, 1990.          __________

6 -, Documentation for Borland Pascal with Objects, Borland               _____________________________________________

International, Scotts Valley, California, 1992.

7 Peter N Roth, OBJECT ORIENTED PROGRAMMING IN FORTRAN, The                                                                  ___

Fortran Journal, Vol 4 No 2, March/April 1992.          _______________

8 Peter N Roth, STORAGE MANAGEMENT AND OOP, The Fortran Journal,                                                      ___________________

Vol 4 No 4, July/August 1992.

17

TABLE 1. A PARTIAL LIST OF THE TPROTEUS OBJECT LIBRARY                TABLE 1. A PARTIAL LIST OF THE TPROTEUS OBJECT LIBRARY

Objects                                       Objects

2D & 3D Nodes

2D & 3D Finite Elements

2D & 3D Vectors

Base Element Object

Base Material Object

Base Node Object

Byte, Word, and Floating Point Arrays

Collections

Equation Specifications

Input Parser

Large Matrices (Skyline & Diagonal)

Linear Elastic materials

Matrices

Prescribed Behaviors

Static & Dynamic Solvers

Systems of Equations

Time Functions

Time Integration Clock

Programs                                       Programs

TProteus, the parent program

Base FE Object

FE 2D Object

FE 3D Object

Static 2D FE Program Object

Static 3D FE Program Object

Dynamic 3D FE Program Object

18