SOFTWARE LICENSE AND AGREEMENT

This product is released on the SHAREWARE concept. You may use it for evaluation purposes only. If you find this product useful (and a number of magazines and reviewers have found it to be the best product of its type ever developed) then the developer (me) would like to ask for a donation of $500. Do not use this program for more than 30 days from the time you first activate it, as I will starve to death.

For the $500 registration donation,  you will receive the most recent update to the system, full manuals, audio training tape and 30 days of tech support and a picture of me.

Seriously, this product represents a whole new way of running your entire manufacturing facility. Many man-years of time and several hundred thousands of dollars were spent in its development. I am releasing it as a SHAREWARE product simply because I believe that I can make a real go of it like this. Please don't disappoint me.

This users manual is about 200 pages long, but should definately be read before you run the system. There are certain steps you must perform before other steps are activated. Additionally, this is a very high performance product and has some requirments for operations.

To see a review of the product, read APICS The Performance Advantage Magazine, March 1993 Pages 66-71. This is the most indepth review that has ever been performed on a product by this magazine. The review was produced by Dr. Steve Melnyk, Michigan State University.

Included in this are 5 zip files. You should decompress the 5 files onto individual high-density floppies. The program may then be installed from the first disk. You may also elect to make a directory on your hard drive called TEAMONE and unzip all files directly into that directory. The full program will take up about 5 meg of storage space. It is recommended to have another 5 to 10 meg free for data.

This program can control over 100 million individual parts/processes.

READ THE MANUAL

Included on the 5th zip file is a tutorial program. This will run you through the entire operations of the system, from primary inventory/vendor control, through process development, scheduling and warehousing. Read the tutorial manual fully  TUTOR.TXT.

For more information and registration of the product contact;

Dave Bethany

303-988-5343

163 Zang Court

Lakewood, Colorado   80228

Customized versions of this product are definately available at realistic prices and time frames. I need the work. You need the software.

This product is protected by US and International copyright laws and treaties. This product may not be reverse engineered, disassembled or altered in any way.

This product may not be resold, but may be freely distributed for EVALUATION PURPOSES ONLY.

WARRANTY

Being that you got this free, there is no warranty, expressed or implied and the developer accepts no responsibility for the program being fit for any particular purpose.

All risk of usage and operations lies in the hands of the user.

I am interested in your thoughts or ideas. Over 300 companies supplied ideas over the course of 5 years to bring this product to fruit. Additionally, look for the educational version of this to be incorporated with Dr. Tom  Vollmann's book "Manufacturing Planning and Control Systems" in the near future.

Thank You and happy computing.

TEAM ONE

USERS MANUAL

INTRODUCTION

Congratulations on your obtaining of TEAM ONE. This program is specifically

designed to work with and in all phases of the manufacturing environment.

TEAM ONE is the first program of its type that provides fully integrated

features with a graphic user interface.

TEAM ONE is NOT an MRP or MRPII System. Although TEAM ONE contains most

of the functions of these older systems, TEAM ONE is much more. Therefore,

before you begin program operations we must ask you to clear out all of

those old obsolete thoughts that went along with those systems and to

realize that TEAM ONE is going to redefine the manner in which you run

your manuafacturing operations in the future. TEAM ONE is a totally new

philosophy in which you can use to easily guide your company into the

future. TEAM ONE will allow you to raise productivity and reduce costs.

It will put a real control handle on your inventory usages and production

capacities. It will give you information that you need quickly and efficiently.

TEAM ONE uses a very advanced method of data management, called HYPER-DEX.

This system, which combines HYPER-TEXT with INDEXING, allows for extreme

high speed program operations, while operating in a standard DOS environment.

Although we realize that most people hate to read User's Manuals, due

to the sophistication, functionality and system integration built into

TEAM ONE, it is imperative that you learn the program from this manual.

TEAM ONE provides areas of functionality that were simply not achievable

prior to its development. However, this functional capability can only

be used to its designed capacity if the user fully understands how the

system works. This is not going to be an extremely difficult task, as

the program was specifically designed to assist the user and to work in

a common-sense approach. The more you "Stretch" the data you are developing,

the better the program will operate.

As we go through this tutorial, you will understand fully what we mean

by "Stretching" data. But first, before you even begin this tutorial,

you must compile a basic library of information. This process will take

some time and needs to be extremely accurate. The more accurate and complete

your data is, the more powerful the program becomes. Remember, this system

was designed to FULLY assist you in the day to day operations of your

entire plant and warehouse. So let's begin with the basic system parameters

and data requirements.

TEAM ONE HARDWARE REQUIREMENTS

TEAM ONE is a graphically driven program, but provides you the capability

to run on very inexpensive, basic configured DOS-based machines.

Minimum Hardware Requirements:

1.	100% IBM Compatible 286/386/486 CPU.

2.	EGA(640x350 minimum)/VGA Graphic Card and Monitor

3.	1 Meg. Memory with 520K free conventional memory.

4.	5 1/4", 1.2 Meg. HD Floppy drive OR 3 1/2", 1.44 Meg. HD Floppy drive

5.	40 Megabyte hard drive with at least 6 meg free space.

6.	MicroSoft/Logitech Compatible Mouse or Pointing Device

7.	88-key keyboard

8.	IBM/EPSON/HP compatible Printer, capable of printing 132 columns.

Operating System:

1.	Recommended...Digital Research DOS 6.0 (on 386/486 systems)

2.	MS/PC DOS 3.3 or higher.

Advised Options/System Upgrades:

1.	2 (two) 100 meg. 17ms or faster hard drives, running hardware controlled

disk mirroring.

2.	120 meg. tape backup system running in automatic mode.

3.	2400 baud, Hayes compatible modem.

4.	MS-DOS 5.0 or DR DOS 6.0 with ability to load Btrieve Driver, Network

Driver and Mouse high.

DATA REQUIREMENTS:

This section provides you with the foundation of information necessary

to get started with the system.  TEAM ONE may require more information

then your present system does, but keep in mind that TEAM ONE is fully

integrated and many functions are inter-dependent.

As an example, in a Model T Ford, you didn't have to have a battery, because

you hand-cranked the car, but in a LEXUS 400, you must have the battery.

You also drive faster, safer and in much more comfort. This is the same

with TEAM ONE, the program needs a large amount of initial data because

it can provide you with a massive amount of answers.

TEAM ONE provides you with the ability to streamline your plant and inventory

operations to an extent never before achieved. This streamlining can directly

equate to higher profits and more productivity. But the data needs to

be extensive and exact.

Before we move on further, we will explain how TEAM ONE's various functions

interact with each other.

TEAM ONE automatically links inventory information to vendor data. It

ties product and process development directly to inventory and warehousing

management. TEAM ONE determines what parts are needed to build a product,

it determines how many, when and from whom to order parts. TEAM ONE ties

schedules directly to customer records, vendor ordering records and expected

receipt files for warehousing control. TEAM ONE gives you the ability

to maintain inventories at the most productive and cost effective level.

TEAM ONE provides you the ability to automatically balance your assembly

processes into a smooth flowing sequence and adjust labor force to the

most cost-effective and efficient mode.

Additionally, we will begin to "Stretch" the data. TEAM ONE was designed

to give you extreme flexibility within its functions.

With this understood, lets look at the data we will need to provide to

the system, on a module-by-module basis.

INVENTORY

The Inventory Module provides for Vendor Control, Inventory Control and

Warehousing Control.

IMPORTANT: It is extremely important that data is exact. TEAM ONE is very

flexible, but in some cases that inherent flexibility can cause misunderstandings

in the program. Although few search features are necessary, due to the

HYPERTEXT side of HYPER-TRIEVE, some searches are performed by the program.

These searches are performed on an EXACT basis, but without regard to

case. If you inadvertently enter a `space' at the end of a part number,

and later on enter the part number in another module but do not enter

the `space', the search features will not locate the part number. This

is done, due the fact that some companies may desire spaces in their information,

and the program accommodates that desire.

NEEDED VENDOR INFORMATION<$&vendor[v]>

You will need to obtain the following basic vendor information. This information

needs to be as complete as possible, as this information is what the system

uses when it creates its Purchase Orders.

VENDOR NAME: This is the name of the vendor or their company.

ADDRESS: This is the physical address of the vendor. You may or may not

desire to add this to any "Stretch" data you have developed.

CITY/STATE/ZIP: This is the geographic location of the vendor.

COUNTRY: You may elect to fill out this field or not, it is the country

the vendor is located in.

PHONE: This is the phone number of the vendor. This data can be used by

the system in TEAM ONE's communications module.

FAX: This is simply an informational field, that may be filled out or

not, as you desire.

CONTACT NAME: This is an informational field. This would be the name of

the salesperson you normally speak to.

VENDOR ID#: MUST FILL OUT FIELD...Control to Vendor Indexing System...This

is the most important field on the record. You MUST enter a vendor identification

in this area. This is an alpha-numeric field, so you can enter any information

you desire. However, each vendor must have a unique ID. If you do not

presently use a vendor identification, then now is your chance to create

it.

After you have compiled all of this information, you need to compile your

inventory and warehousing information.

NEEDED INVENTORY INFORMATION<$&inventory[v]>

Now we begin to `crank-up' the capabilities in the system. TEAM ONE has

strived in all areas to reduce repetitive data input as much as possible.

In other words, once primary data is created, the system uses this data

and enters it for you, wherever possible in other modules. TEAM ONE's

Inventory database is very detailed, as it combines both standard inventory

control information with warehousing information. This data is used in

almost all modules in the system. This data must be as complete as possible

and very precise.

If you presently maintain your inventory and warehousing data in a computer

system, then this area will be a breeze. TEAM ONE has the capability to

read either fixed length ASCII text files or dBase III files. It is doubtful

that your database contains all of the information needed in TEAM ONE,

but if you have a present computer system, you can adjust your data to

meet TEAM ONE's demands. It is beyond the scope of this manual to address

all of the ways to bring your data together, but the following information

will tell you what data you need.

NOTE: For conversions of data, please read the CONVERT DATA area in the

INVENTORY chapter.

VENDOR PART NUMBER: MUST FILL OUT FIELD...TEAM ONE allows you to maintain

a cross-check record between vendor part numbers and your part number.

Most companies use an internal part numbering system, in addition to vendor

part numbers. If your company only uses vendor part numbers, then TEAM

ONE will create its own internal part numbering by copying the vendor

part number. This field tells the Vendor Purchase Order module what the

vendor part number is to order and creates a link directly to your internal

part number.

C/C: This is the Customer Code area. This is for informational purposes

only and may or may not be filled out, as you desire. You may use this

as an ABC code, P&IC code or some other coding system that you desire.

YOUR PART NUMBER: This is the control to the Inventory Indexing System...This

is the most important field on the record. As stated in Vendor Part Number,

this provides the cross-reference back to the vendor part number.  If

your company has an internal part numbering system, then this field must

be filled out, as it is the search parameter for the Process Control Module.

If you only use a vendor part number and no separate, internal numbering

system, leave this field blank and TEAM ONE will copy the vendor part

number into this field for you.

DESCRIPTION:This is the description of the part. Although not specifically

required, this field should be filled out. This provides your inventory

personnel the ability to not only look for part numbers, but also to compare

a retrieved part to a description.

QUANTITY ON HAND: This is the number of parts you physically have on hand,

but have not allocated to a product in production; are not presently in

use on the production floor or are not on hold. If your present system

does not allow for being this specific in breakdown, you may enter all

parts presently in inventory here, and allow TEAM ONE to re-allocate the

parts by using the various modules later on.

S/S: This is the Safety Stock. You may elect whether or not to use this

field. The safety stock is a minimum inventory quantity on hand. The Master

Schedule module uses this field to determine when to order parts, if the

user does not want inventory quantities on hand to fall to 0. If maintained

at 0, TEAM ONE ignores the Safety Stock field. If, however, you entered

a quantity of 100 in safety stock for a selected part, the Master Scheduler

would order parts when a product is being placed into production and the

quantity of parts needed would force the quantity of inventory on hand

below 100 units. If Safety Stock data is present then the Master Schedule

will request parts needed for the current order in addition to any quantity

necessary to satisfy the Safety Stock level indicated in this field.

NOTE: The next 5 quantity fields are system conrolled and do not have

to be filled out.

QUANTITY ALLOCATED: This is the number of parts that have been assigned

to production, but are not currently in production. In other words, these

parts physically exist, but can not be used by scheduling functions in

determination of product development. They have already been assigned

to a previous product order and are simply waiting for that order to be

activated in the Warehousing Pick system.

QUANTITY IN USE: This is the number of parts that have been physically

picked by Warehousing and are presently in production of an order.

QUANTITY ORDERED: This is number of parts that the Master Schedule Module

has determined need to be ordered and have been activated and approved

by the Vendor Order Entry system.

QUANTITY BACK ORDERED: This is the number of parts that were determined

to be short on a received shipment. This number is generated by the Warehouse

Receive to Stores Module.

QUANTITY ON HOLD: This is the number of parts that have been received

in Warehousing, but have been placed on a hold status and are not available

for use. This may be due to Q/C checks or other areas. This number is

generated by the Warehouse Receive to Stores Module.

UNIT OF MEASURE:This is an informational field, that allows you to determine

a unit of measure for a part. You may enter up to two characters, such

as EA for Each, or KL for Kilo. This is user definable and is not needed

by any program functions. TEAM ONE does not control or convert the unit

of measure in any manner.

VENDOR IDENTIFICATION:This is an extremely important field and must contain

exact data. This is the Vendor ID that was created in the VENDOR area

for a specific vendor. This links the Inventory record to the Vendor record.

Due to the flexibility of TEAM ONE, you have the ability to maintain an

identical part number for two different vendors and choose between those

vendors during process development.

LEAD TIME: This is the amount of time you have determined it takes to

receive ordered parts from a vendor. The program defaults to days, but

if you are entering information manually or through dBase Conversion,

you may determine the time span by either days, weeks or months. This

automatic function is not available in ASCII conversion; it automatically

defaults to DAYS.

MINIMUM ORDER QUANTITY: This field is used by the Master Schedule Module,

if over 0. It may be used by itself or in conjunction with the Safety

Stock area of the inventory record. If filled out, when the Master Schedule

determines that parts need to be ordered, the Scheduler will order at

least this many parts. However, if the Scheduler determines that more

parts than the minimum is needed, it will order the correct amount, but

never less than the minimum. Additionally, if the scheduler determines

that the system has previously ordered more than the minimum, the Scheduler

will only order what it needs. If a vendor demands each order quantity

must meet a minimum quantity, then the ordered number may be adjusted

in the Vendor Order Module during order activation.

ECONOMIC ORDER QUANTITY: This is an informational field and does not have

to contain data. It is the number of parts that are should be ordered

to provide for best pricing or delivery.

STORES LOCATION: This is the physical location of the inventoried item.

This is an informational field, but it tells the warehousing personnel

where to get a part. This can contain any data that will physically fit

in the field.

COST PER UNIT: This is the individual cost of a part. This number is used

to determine the basic product costing in the Calculate Module. TEAM ONE

does not provide an accounting system, but it allows the user to create

any data they wish and export, through ASCII files using the REPORT WRITER

Module, or by exporting a "," delimited ASCII file from the CONVERT DATA

menu function that will allow conversion to any accounting system that

can read ASCII files. TEAM ONE only calculates product costing on direct

labor and material.

MAKE PART/PURCHASE PART:This area is an informational field, but is used

by the Process Module. It allows the user to detail whether this inventory

item is purchased from an outside vendor or built internally.

"STRETCH"...You may have the identical part number as both a make part

and a purchase part. Whenever, this is desired, simply copy the data to

another inventory record, but change this field.

MULTI-VENDOR: This area is an informational field that is not controlled

by the system.  However, it can be useful if you purchase the same part

from multiple vendors.

NOTE: The above field 2 fields may only be activated in manual input of

data or in dBase conversion. Any fields that are `CHECK BOX' type fields

can not be activated by ASCII conversion, but may be activated later by

using the CHANGE system.

LOT ID: This field tracks lot numbers as assigned to individual parts,

products and sub-assemblies.

SHRINKAGE: This field provides a percentage increase of ordered quantities

for loss factors.

PRODUCT PROCESS PLANNING

This is the most intense area of the program and does not allow any conversion

from existing data. The reasoning behind this is that prior to TEAM ONE,

no other known system provided the capabilities of Product/Process Planning

and Control to the sophistication of TEAM ONE. Many companies have not

developed the data that is needed by TEAM ONE, because prior to TEAM ONE,

it could not be used effectively. Although the creation of this data can

become very time consuming, once developed, it can pay for itself many

times over in direct savings.

TEAM ONE controls PRODUCT/PROCESS data in a three-tiered manner. This

allows the program to prompt the user for data needed, it reduces the

possibility of errors and provides the system the ability to automatically

reuse data that has previously been entered. However, data entry in this

area must be exacting in detail and entry. TEAM ONE accepts the fact that

you know more than it does, and if data is entered incorrectly, it will

operate from that incorrect data.

To begin with, TEAM ONE will ask for an hierarchial structure for a product.

This is the basic make-up of a product and only includes that product's

Parent Part and sub-assemblies. No processes are developed here for parent

parts or sub-assemblies.

Make a block diagram flow chart, with the top block being the main product

name. This can be a top-level assembly, (such as if you are building cars,

it could be CAR) or any level below that (such as engine). TEAM ONE will

track a product structure, and that product can be a parent or any sub-assembly.<$&prodlevl[v]>

Your product structure will need to contain the names of the Parent Part

(or main product, this could be either a top level assembly or sub-assembly,

if it was used as a F.R.U. (field replaceable unit) and was a product

unto itself in addition to being a lower level assembly of a main product).

The structure will need the assembly/sub-assembly name that is maintained,

a line identification for the assembly line it is built on and an assembly

level number.

If sub-assemblies exist under the main product, then create blocks under

it for them. Continue to build the diagram, listing all sub-assemblies

and how they are related. On the side of each level of the diagram, write

in the level number, such as 0 for a top level assembly.

NOTE: This information must match the inventory part# information for

the product or sub-assembly

Create the diagram in the logical order of events that take place in the

building of the product.

Once your basic product structure is designed, now you need to acquire

basic product data for each sub-assembly. Also remember that these products

and sub-assemblies will need to be present in the inventory database.

This basic product data must be:

<$&prodinfo[^]>1) The Product Name. This is the part number you have assigned

to a product or sub-assembly and are maintaining in your inventory records.

This part number can be anything you desire, such as a red car could be

part number 1-123sd-321 or it could be as simple as Car, Red.

2)	The Line ID. TEAM ONE needs to assign assembly line identification

to each product and sub-assembly line. This is necessary, because the

system allows the user to determine where a product is to be built and

where its sub-assemblies are to come from. This also tells the Warehousing

Module where to send picked parts.

3)	You will need to decide if you want TEAM ONE to perform automatic process

balancing for assembly with no input or if you want to determine the number

of workcells TEAM ONE balances your assembly line to. We call these work

cells or work blocks. This figure may be adjusted constantly, so you do

not have to predetermine this data. For the system to do the balancing,

leave the number at zero.  To override the automatic system, place the

number of work cells you would like to balance to in the work blocks field.

Balancing is accomplished by totaling the time of assembly and dividing

by the total processes to achieve the targeted balanced cycle time.  Processes

are then grouped accordingly to meet the time.

4)	You should have a close approximation of what your maximum designed

line capacity is for a given product on a given line during one average

shift of work. Although most of the information used in the second tier

of the Product/Process area is used only for estimational purposes, the

better the data, the better the estimation. TEAM ONE, once fully configured

and operational, will alter this data to real-time values.

5)	You should have a close approximation of the actual number of products

a line is building per shift.

6)	You will need to know the true number of working hours in a shift for

a given line. In many cases this number would be 8 hours per shift. This

field directly affects the cost of a product.

7)	You will need to know the actual number of working hours a given line

is in operation during a shift. If your true number of working hours is

8, but the line stops for 20 minutes for lunch and 10 minutes for breaks,

then the actual working hours would be 7.5 hours per shift. This field

directly affects the number of products that can be built and the time

available in the Manufacturing Calendar.

8)	You will want to have the normal, average number of operating shifts

per day a line operates.

9)	You will want an average number of operating shifts per week

10)	You will want an average number of operating days per month.

NOTE: These previous three areas are for primarily estimational purposes,

but the shifts per day area is used by the Manufacturing Calendar as a

building basis. However this information is changeable in the Calendar

operation on a daily basis.

11)	You need to provide an average wage for all workers on the line. This

is easily provided by simply taking the total wages and dividing by the

number of total workers.

NOTE: Although TEAM ONE does not take into account any overhead costs

in its costing calculations, these costs can be added into both material

cost and labor costs. In other words, if your accounting department has

determined that manufacturing operations adds 25% to labor costs, then

add 25% to your cost of labor. If it takes 12% of your total inventory

cost to run your warehouse, then add 12% to the cost of each part.

12)	You MUST have the total set-up times for a given product line in minutes

for a single shift. This set-up time is the amount of time it takes to

perform the line set-up, if that set-up is a one time operation for one

product during one shift. If set-up times are performed repetitively during

the production process, then the set-up time becomes a process in the

building of a product.

And now to the third tier of data. This information is absolutely essential

to the operations of TEAM ONE and must be exact in nature and detail.

This area DEMANDS a full and accurate time and motion study be performed

on every product, on every line in the entire operation. If you do not

develop precise data for this area, then ALL other program functions will

only be estimations.

It is imperative that when a time and motion study is performed, every

single process or movement, no matter how insignificant, must be taken

into account. Every action that takes time, must be accounted for. We

realize that no worker, machine or process will always operate at the

exact same speed, but if your time and motion study is performed diligently

and is performed several times, at different times of day, on different

days, your average data will be very accurate. The more precise and accurate

you data is in this area, above all others, the more accurate and precise

all TEAM ONE functions will be.

In this third tier of data, you will be working with individual process

steps in the assembly of a product or sub-assembly. For those of you who

are unfamiliar with the term "Process" it is simply "The name or part

number of every single step or part that goes into the building of a product."

These include move steps, test steps, machine steps, assembly steps, painting,

quality control, testing, boxing, waiting, etc.<$&process[v]>

1)	Although you will not have to enter some of the data into the third

tier, as the program will enter existing data for you, when you are entering

inventory part information, you want to have available what vendor you

wish to use for individual parts, and/or the location of the desired parts,

and/or the cost of the desired part. This gives you the ability to assign

specified vendors or locations or part cost to the assembly of specific

products, on specific lines. Or you may elect not to do this, and simply

attach the individual part used to the first inventory record that matches,

exactly, the part # used in a process within a product.

2)	You will need to have a very close approximation of the amount of time

the specific process takes to perform.

3)	You will need the exact quantity of parts used in each individual process

step. This quantity type must match the quantity type entered into your

inventory. As an example, Let's say you purchased 1" pipe in 10' lengths

and entered the quantity of 100 of these pipes in your inventory as 100

and the unit of measure as EA. During your manufacturing process you used

1 foot of this pipe at a specific process, then the quantity would be

0.1 or 1/10th of a pipe. If you bought a thousand pounds of soap powder

and entered it into inventory as 1000, and you only used a cup of powder

in a process, then you would have to figure out what a cup weighs and

make the relationship between the weight of a cup and a pound. The reason

for the need for this exactness is due to the fact that TEAM ONE incorporates

Inventory with Production with Order Entry with Material Depletion and

Warehousing. It would be impossible for TEAM ONE to understand the relationship

between a cup and a pound if you did not set that relationship yourself.

TEAM ONE does not control or convert the unit of measure in the Inventory

(SKU) down to the Process quantity (quantity used).

5)	You may also wish to determine what kind of process each individual

step is. TEAM ONE allows for input of process types such as "VALUE ADDED",

"MACHINE", "MOVE","TEST" and "QC". This is for informational purposes

and does, in fact, affect the calculated product cost when a process is

designated a MACHINE step.  If a MACHINE step is identified, no labor

will be amount will be added to the step.

It is extremely important to stress that when you perform your time and

motion study, you begin at the first process and perform the study in

a sequential method, following the product down the line or through the

processes. TEAM ONE will take your information, apply assembly sequence

numbers to the processes and then during manufacturing calculations will

balance the assembly line and assign individual processes to work centers,

automatically, using the Targeted Balanced Cycle Time as a guide.

That is the basic information you will need to allow TEAM ONE to begin

to bring your plant up to its highest productive and most efficient capability.

Once you have developed this information, you are now ready to begin to

load it. We will begin this loading process where we began the data development,

VENDOR INPUT.

From this point forward, the manual will be broken down in the following

manner:

Main modules are assigned a chapter identification.

All Primary operations inside of a module, with the exception of Warehousing,

which is under the Inventory Module, will be explained as Sub-Systems.

Warehousing, although it is inside of the Inventory structure, constitutes

a Module and is provided a chapter in itself.

You will read in this manual constant references to Hyper-Text and the

Linking process between records of various databases.

The thought behind this linking process (and it is performed many times

throughout the program) is that Hyper-Text acts as a physical link between

records of different databases. This function dramatically reduces the

need for search functions in normal program operations. When the program

reads a record in one database and then needs to access another record

in a different database, instead of having to institute a search function

in the second database every time the program is run,the program only

needs to search the associated database once. When the needed record is

found, TEAM ONE creates an Hyper-Text link between the associated records.

From that point forward, when the data is need, the program simply reads

the Link information held in the Hyper-Text button, opens the needed database

and retrieves the record that has been linked. The beauty of Hyper-Text

links are that they are not affected by sorting databases or deletions

or additions of other records. The Hyper-Text network built into TEAM

ONE is so extensive, that once the full program has been activated and

placed in operation, every database is in one way or another linked to

every other database through the Hyper-Text Matrix.

You may ask, "Why, if Hyper-Text is so fast and so powerful, don't all

programs use it? Is it something very new and untested?" The answers are

simple and economically understandable. Hyper-Text has been around for

over 20 years. The problem with Hyper-Text is not in its operations but

in writing a program that uses it. It is extremely difficult to write

programs that use this method of database management. That is why TEAM

ONE was written in a new programming system. The actual language that

TEAM ONE was written in uses Hyper-Text capabilities extensively and has

allowed us to create a program that makes Hyper-Text program development

very easily accomplished. To write a program in less sophisticated languages

that could use Hyper-Text would take many times as long as it took to

create TEAM ONE and would make the programs extremely expensive, if they

could ever be finalized in any realistic span of time.<$&HELPGRAPHIC[v]>

Hyper-Text and its cousin Hyper-Trieve, are the engines which make TEAM

ONE so effective and fast.

The manual has been written to guide you through all program operations

in the sequence each function should be used.

CHAPTER 1INSTALLATION

Throughout the TEAM ONE program, the developers have attempted to make

all program operations as simple and straight-forward as possible. This

begins with the TEAM ONE Installation Program.

To install TEAM ONE place the TEAM ONE DISK 1 in a floppy drive. Change

your drive path to the floppy that holds this disk.

Type <P12B>INSTALL<P255D>

TEAM ONE's automatic installation process will now begin. The install

program will test your system hardware to make certain that all parameters

needed are able to be met.

Install will first display a screen asking if you desire to continue with

installation. Press the CONTINUE button to continue or CANCEL to end installation.

Next, Install will check all available drives for a minimum of 4 megabytes

of free space. The drives that meet these parameters will be displayed

in the next screen. Place the mouse cursor over the drive you desire to

install TEAM ONE upon and click on it with either mouse button. Now press

the CONTINUE button to continue with the installation process.

Now Install will inform you that it will load all TEAM ONE files into

a directory called \TEAMONE. If this default path is agreeable, simply

click on the \TEAMONE button. If you desire to change the directory name,

enter the name (such as \NEWNAME) and click on the CHANGE button.<$&DIR[v]>

Install will now create the needed directory and copy all files. Install

will display the files being copied and the percentage complete. When

Install prompts you to insert another disk, please do so and press the

CONTINUE button.

When installation is complete Install will detail needed information for

your config.sys file and the path to this file. We purposefully do not

change this file for you, as we do not like programs to alter our own

config.sys files. You may alter this file through any text editor. Please

see your DOS manual for instructions on the creation and alteration of

the Config.sys file.

Installation of TEAM ONE is now complete. Install will inform you as to

where the TEAM ONE program resides.

To activate TEAM ONE, change to the noted directory and type TEAMONE.

NOTE: To activate security, change to the directory that you have loaded

TEAM ONE into, and type the command SECURITY. This will load the Security

files into the system. If you have activated SECURITY on the system, you

will have to enter the following upon activation of the TEAM ONE program

when the Enter Security Information Screen appears:

EMPLOYEE ID:  TEAM (ENTER) _PASSWORD:  ONE (ENTER)

This access may be altered by the system administrator through the Database

Tools Area. Additionally, should you ever lose your ability to enter the

program, TEAM ONE has a hard-code override for the security system. Call

Tech Support for this entry code.

Upon Start-up, the system will automatically load the Btrieve record manager.

If you have previously loaded Btrieve, the system will continue. Also,

it is recommended that if your system has the capability, you should first

load Btrieve into High Memory.

CHAPTER 2INVENTORY MODULE

<$&mainmenu[^]>From the TEAM ONE MAIN MENU, place the mouse over the INVENTORY

button and click on it with either mouse button. This will bring up the

INVENTORY SYSTEM MAIN MENU.

VENDOR INPUT SUB-SYSTEM

<$&1[^]>Place the mouse cursor over the INPUT VENDOR button and click

on it with either mouse button.

This now brings up the Vendor Information Input Screen. This is the primary

information needed for the vendor database.

<$&2[^]>Enter the vendor's name in the VENDOR NAME field.

"STRETCH"...You would also enter line identifications in this field for

all assembly lines that supply sub-assemblies or products to other lines.

Next enter the address, city, state, zip and (if desired) country of the

vendor location. If you are entering line information, you may or may

not wish to fill out these fields.

Enter the phone number of the vendor, if desired. This number can be accessed

in the communications system and will be used in certain other optional

modules that may be released later.

Enter the FAX number, if desired. This number is not accessed by any other

part of the system and is for informational purposes only.

If you desire, you may enter the Contact Name for the person you normally

speak to. This is an informational field only.

Enter the Vendor ID. This is vitally important as it provides for the

link between the inventory record and the associated vendor record. The

program will not allow you to continue until this field has some information

in it. Also remember that this information must be exact and during inventory

input, this information will be accessed by the inventory system.

You are not limited to the number of vendors you may enter into the system,

with the exception of free space available on your hard-drive.

If you desire to continue adding additional vendors, press the CONTINUE

button.

If you have entered all of the vendor data you desire, at this time, you

may press the FINISHED button to end the session.

If you wish to end the session and not save the present record, press

the CANCEL button.

It should be noted that when you begin entering your inventory data and

the system can not locate a desired vendor record, you will have the opportunity

to create a new vendor record at the time of inventory input.

INVENTORY INPUT SUBSYSTEM

Version 2.0 of TEAM ONE now integrates the Novell Btrieve Record Manager

as its indexing system for the Inventory Database. It is advisable to

load this file BTRIEVE.EXE into high memory if at all possible. See your

DOS manual or Memory Manager Manual for instructions on how to do this.

From the TEAM ONE MAIN MENU, place the cursor over the INVENTORY button

and click on it with either mouse button. This will bring up the Inventory

Main Menu.

Place the mouse cursor over the INVENTORY INPUT button and click on it

with either mouse button.

This will bring up the Main Inventory Input Screen. This is the area that

you will <$&3[^]>manually load inventory information into the system.

If you already have inventory information and will only be converting

this data, then you will not load it manually here. However, to become

familiar with the types of information the inventory record maintains,

please follow along through this section.

You will begin by entering the VENDOR PART NUMBER. This is the part number

that your vendor uses. This number is accessed by the Vendor Order Entry

system and is used to order parts. This provides an automatic cross-reference

between part numbering systems that you have generated internally in your

plant and the part numbering structure your vendor would use. This must

be very clear: all areas of the inventory information record, except as

noted, must be filled out. Due to the structure of TEAM ONE, we have strived

to reduce the need for repetitive data input throughout the modules. This

means that various other databases attach to still more databases through

the Hyper-Text links and use data from these other sections. Therefore,

even though we request a vast amount of information for the inventory

record, this will be the only time you need to enter it throughout the

system.

The next field is the C/C field. This is an informational field and does

not need to be filled out. This is the Customer Code field. This field

allows you to enter a two character code such as an ABC code or P&IC code

for a selected part, or any other code that you may have developed internally.

Next you will enter YOUR PART NUMBER. If you use internally developed

part numbers you would enter it here. If, however, you use your vendor's

part number for your own part number, you should leave this field blank.

TEAM ONE will check this field to see if it is blank, and if so, will

copy the vendor part number into this field. This is a very important

field and must contain exact information. TEAM ONE will allow you to enter

spaces into this data field, but remember that when you request a part,

the number must be exact and that includes spaces.

NOTE: You are only allowed to enter spaces in manual input and ASCII Conversion.

DBase does not allow for embedded spaces.  (i.e. Screw_8mm)

Now enter the DESCRIPTION of the part. This field, although not needed

by other areas of the program, should be filled out as it may assist your

warehousing personnel in locating parts.

Next are the Quantity fields. Please read the next few entry areas carefully.

Due to the flexibility that TEAM ONE allows, you have the ability to enter

the same part number many times. You would do this if you desired to locate

the same part in various areas of your warehouse, or if you wanted to

maintain various part costing for the same part or various vendors for

the same part. The quantities are then linked only to the part record

that is created for the specific part, at the specific location, at the

specific cost.

QUANTITY ON HAND...This is the physical number of items you have in inventory

for this part. TEAM ONE makes a distinction between QUANTITIES ON HAND

(as the number of parts that are available, but not allocated to an order,

in use on a line or on hold), QUANTITIES ALLOCATED (parts that have been

allocated to an order, but not picked and placed in production), QUANTITIES

IN USE (parts that have been picked for use in production) and QUANTITIES

ON HOLD (parts that are physically present, but not available due to testing

or some other reason). If your present method of inventory control allows

you this flexibility, then you would break up your inventories similarly.

If not, however, you would enter all part quantities physically present

into the QUANTITY ON HAND field, and then allow TEAM ONE to adjust the

inventories for you through the other modules and sub-systems as the system

is used.

Other QUANTITY areas include QUANTITY ORDERED (the number of parts that

have been ordered). Do NOT fill out this field, as you will be able to

activate this field in the Order Entry System. Remember, this is a fully

integrated system, and thus creates additional databases for information

as it needs them.

Additionally, the program provides a QUANTITY BACK ORDERED field. This

field is activated by the Warehousing Module's Receive to Stores system.

A special quantity area is the S/S or Safety Stock area. This field provides

for a minimum quantity for the inventory item. This field does not have

to be filled out, but if it is provided a number higher than 0, then when

the Master Scheduling Module reads the inventory records for its depletion

and ordering schedules, it will also read this number. If the inventory

Quantity on Hand should fall below this number, then the Master Scheduler

will automatically add a sufficient number of parts to order to bring

the quantity on hand up to this level. Additionally, this field can be

attached to the MINIMUM ORDER QUANTITY FIELD. (See Below).

The UNIT OF MEASURE field is totally user-definable and is an informational

field only. This 2 character designation is for the unit of measure description

of how inventory quantities are controlled. As an example you might enter

EA for each or BX for box. Remember, however, that you must make the correlation

between quantities maintained in inventory and quantities maintained in

manufacturing. As an example, Let's say you purchased 1" pipe in 10' lengths

and entered the quantity of 100 of these pipes in your inventory as 100

and the unit of measure as EA. During your manufacturing process you used

1 foot of this pipe at a specific process, then the quantity would be

0.1 or 1/10th of a pipe. If you bought a thousand pounds of soap powder

and entered it into inventory as 1000, and you only used a cup of powder

in a process, then you would have to figure out what a cup weighs and

make the relationship between the weight of a cup and a pound. The reason

for the need for this precision is due to the fact that TEAM ONE incorporates

Inventory with Production with Order Entry with Material Depletion and

Warehousing. It would be impossible for TEAM ONE to understand the relationship

between a cup and a pound if you did not set that relationship prior to

using the system.

Next is the VENDOR IDENTIFICATION field. This is the Vendor ID for the

company or line that supplies the part. This must be an exact match to

the information loaded in the Vendor database. When you decide to save

the inventory record, the program searches the Vendor database for this

ID. If it finds a corresponding Vendor record, it automatically creates

an Hyper-Text link between the inventory record and the vendor data. If

you did not first enter a vendor record for this part, then the system

will prompt you to create a new vendor record. (For more information,

see the CONTINUE or FINISHED button areas of Inventory Input).

The DATE OF INVENTORY fields are automatically generated by the program.

This is the system date held by your computer. You may elect to change

these fields.

The SHRINKAGE field allows you to set a percentage of shrinkage for the

current inventory item.  This is used by the MASTER SCHEDULE.  When an

order is processed, the net requirement amount in multiplied by the percentage

held in this field and added to the net requirement amount- giving a new

net requirement amount.  (NR * SHRINKAGE = n; NR +n= NEWNR)

The LEAD TIME field is the amount of time it normally takes to receive

a part from a vendor. If this inventory record is for a sub-assembly that

is built by your plant, then you would enter the amount of time it takes

to supply a given quantity of these sub-assemblies (See Minimum Order

Quantity). This number should.d reflect days.

The LEAD TIME CUSHION field holds the amount of time you wish to add to

a Lead Time, just for comfort sake. This number should reflect days.

MINIMUM ORDER QUANTITY...This field is activated by entering a number

larger than 0, then when the Master Scheduling Module determines that

parts need to be ordered, it will order this amount. If you are using

the inventory record to hold data from a sub-assembly line, this would

be the number of products for the line to build as associated to the LEAD

TIME and LEAD TIME CUSHION fields.

The ECONOMIC ORDER QUANTITY field is simply for information only. This

would be the number of parts to order for the best pricing or delivery

scheduling.

STORES LOCATION is the physical location of the inventory item you are

entering. Remember that TEAM ONE allows you to maintain duplicate parts

in various locations or warehouses. Although this is a text field and

thus will allow you to enter any information that fits in the field, if

you desire to run your warehousing system in an efficient manner, then

it would be advisable to grid your warehousing area off. If you plan to

incorporate an Automated Storage and Retrieval System (ASRS) in the future

then the recommended format for entering data in this area would be as

follows:

W1-A1-Z1-B1A-T1

Where	"W1"=the warehouse number

"A1"=the area number

"Z1"=the zone number

"B1A"=the bay assignment

"T1"=the tray assignment

You may also wish to add in a 6th assignment for compartment. The reason

for stressing this gridding information is that TEAM ONE will have an

optional ASRS interface module sometime in the future, and if your data

is already in a format such as this, then conversion is a simple matter.

The COST PER UNIT field provides for the cost of a single unit of the

product. That unit is defined by you in the Unit Of Measure Field (See

Above).

You MUST detail whether this part is a MAKE PART or PURCHASED PART by

clicking on the appropriate check box. This is important due to the fact

that during Product/Process creation the program displays inventory records

that match process part data. If you make a part and also buy the part

and maintain the same part number for both, this field allows you to determine

which part you wish to use in an individual process, within a given product.

The LOT ID field allows you to enter an alpha-numeric Lot ID for tracking

of individual parts, products and sub-assemblies.

The MULTI-VENDOR FIELD is for informational purposes only. You may elect

to activate this or not, but it is not used by any program functions at

this time. This field allows you to perform search and locate features

for all parts that have multiple vendors or not.

The OPERATOR ID field should be the Log-On ID of the person accessing

the Inventory Input area.

The LAST CYCLE COUNT field provides an area for warehousing inventory

tracking of the last time inventories were counted.

Once you have entered all information into these fields you may elect

to click on the CONTINUE button to save the inventory data, have the program

locate the desired vendor data and link the two data records together.

If you do not have a vendor record with an ID that matches EXACTLY the

Vendor Identification entered on this inventory record, the program will

prompt you to create one.

<$&4[^]>If this warning message appears on your screen you may activate

the Vendor Input Sub-System directly by clicking on the CREATE button.

This will bring up a new VENDOR record and allow you to fill out the new

data. Proceed exactly as you did in Vendor Input. Then when you are finished,

click on the FINISHED button on the Vendor Record. The program will save

both the inventory record and vendor record along with the new Hyper-Text

link between the two.

If you do not wish to create a new vendor record, say you know you have

an existing vendor file, but may not be certain you entered the correct

Vendor Identification in the Inventory record, press the CANCEL button

on the warning message. This will return you to the unsaved Inventory

record and allow you a chance to revise the Vendor ID field.

If the program finds a associated vendor record, it will save both the

inventory record and Hyper-Text link information and will then display

a clean inventory record for further entries.

If you are through entering inventory information, click on the FINISHED

button. The operations are similar to the CONTINUE steps, except that

after saving the data, the program will exit back to the TEAM ONE Main

Menu.

If you do not desire to save the inventory information, simply click on

the CANCEL button on the main inventory record. The program will not save

any information that is currently displayed and return you to the TEAM

ONE Main Menu.

NOTE: The inventory database is the only database that combines Hyper-Text

with Btrieve Indexing. The primary inventory information is held in a

database called INVENT.CRD. The indexing of the Inventory Part Number,

Vendor Part Number, Location and Lot ID is held in an indexing database

called INVENT.BTR Never delete the INVENT.BTR file from your storage device

as the system will not operate whenever a module or sub-system needs to

access indexing data. You must also have the Btrieve Record Manager running

before operating TEAM ONE. Additionally, you may elect to delete the databases

that are created during this tutorial, through DOS delete functions. Be

aware that if you delete the INVENT.CRD file you must also delete its

associated .BTR files.  Failure to delete the .BTR file will stop the

search system from operating. It is advisable to use the DELETE FILE command

from the TOOLS/DATABASE TOOLS sub-system.  If the INVENT.CRD is selected

for deletion from this command, TEAM ONE will automatically delete the

.CRD files as well as the .BTR files so there are not any "old" indices

remaining.

INVENTORY AND VENDOR CHANGE PROCEDURES

In an effort to standardize the system across all modules, all CHANGE

procedures in all modules and sub-systems act the same. There are subtle

differences in the Inventory and Vendor Change Modules. We will first

discuss the primary change information and then detail the differences

between Inventory and Vendor Change functions.

To Change Inventory or Vendor Data, enter the Inventory Module from the

TEAM ONE MAIN MENU. Click on either INVENTORY CHANGE or VENDOR CHANGE.

TEAM ONE will now bring up the associated database <$&5[^]>and the Change

Menu.

The operational menu is located at the top of the screen. This menu holds

a series of buttons that allow you to easily move through the database

or work with specific records in the file. It should be noted that in

the CHANGE areas, the user may not add or delete records. We have determined

that deletion procedures should only be accessed by the system administrator.

ADD functions can only be performed in the input areas of the modules.

You may edit information on records.

Under the Change Menu Bar, TEAM ONE details the current file that you

are viewing and at the bottom of the screen, details your position within

the file..

The FIRST button takes you to the first record in the database.

The LAST button takes you to the last record in the database.

The NEXT button takes you to the next record in the database.

The PREV(IOUS) button takes you to the preceding record in the database

and not the previously viewed record.

The FIND button allows you to search the database for primary information

held in the following fields:

VENDOR PART NUMBERINTERNAL PART NUMBERINVENTORY LOCATIONLOT ID

Only in the Inventory Change Find area must you enter EXACT information

and it MUST BE CASE CORRECT. This means that if you entered a small case

"a" in the part number, the search must also be provided on the small

case "a". A large case "A" will not find the correct record.

The MORE button continues the search using the same parameters you previously

entered.

The EDIT button allows you to edit a record. Once you have finished editing

the record, click on the CONTINUE button.

To leave the CHANGE area, click on the EXIT button. This will save any

changes you performed and return you to the TEAM ONE MAIN MENU.

DIFFERENCES IN INVENTORY AND VENDOR CHANGE.

Due to the fact that these two databases are so closely inter-related,

editing of certain information in either of these databases will automatically

institute additional operations that do not occur in any other CHANGE

procedure in any other module.

VENDOR CHANGE...Vendor change operations will not alter from normal change

procedures unless you alter the VENDOR ID field. If this is performed,

TEAM ONE then checks the entire inventory database for all occurrences

of the previous vendor ID and automatically changes the vendor ID on all

associated inventory records.

INVENTORY CHANGE...If you change the vendor ID on an inventory record,

TEAM ONE will search the vendor database for the vendor record that is

now to be associated with the inventory record. If TEAM ONE finds a vendor

record that matches the ID, the program will automatically link the inventory

record to the new vendor record. If TEAM ONE does not find a vendor record

to associate the inventory record to, you will be prompted to create a

new vendor record.<$&chghelp[v]>

INVENTORY CONVERT SUB-SYSTEM

The convert sub-system allows you to use previously developed data from

other formats and to read it directly into TEAM ONE's Inventory system.

The reason TEAM ONE limits you to only being able to import data into

just the inventory records is due to the fact that in other program areas,

TEAM ONE is demanding such a large amount of precise information, few,

if any, existing systems have the capability to supplying all of the information

necessary. If in those areas, such as PRODUCT/PROCESS, you were to only

convert a limited amount of information and then were to go back into

the databases and add the additional needed data, it would take you longer

to perform the entire procedure of getting the data, converting it, and

fixing it than if you simply would have entered it all manually.

However, most of the information needed in the inventory area is readily

attainable from existing sources. As such, we allow the conversion process

from the two most widely used formats available; dBase III and ASCII Record

Line.

Almost all databasing programs have the capability to work with dBase

type files. If yours does not, or you have your data held in a main-frame

type system or in a spreadsheet format, you may export this data by way

of ASCII format.

Unfortunately, it is beyond the scope of this manual to detail how to

extract data from all the different types of programs that exist. You

will just have to read your other operators manuals to achieve this. However,

we will tell you the structure we need to read the data.

Before we continue though, you should have first created your Vendor Information

Database. During the conversion process, TEAM ONE will read your data,

locate the Vendor ID that you have and create the link to our new data

record. Just as in Inventory Input, if a record doesn't exist which EXACTLY

matches your data, the program will stop the conversion process and prompt

you to create one. All Creation features are the same as in Inventory

Input. If your data base that you desire to read does not contain Vendor

Identifications, then you must create them before you begin the conversion

process, otherwise you will spend a massive amount of time cancelling

the Vendor Create, fixing the inventory record and then reactivating the

Vendor linking feature on every record you are converting.

As an additional note, TEAM ONE not only reads the data but allows you

to convert Inventory Information back to the desired database type. When

this function is performed, you will notice that the speed of conversion

from TEAM ONE to either dBase III or ASCII is many times faster than the

conversion from those database types to TEAM ONE. The reasons for this

dramatic speed difference is that when TEAM ONE is reading a database

and creating a new internal database, it is not only creating the database,

it is also creating the index databases and the Hyper-Text links. This

brings conversion speeds down to about 1 record every 2 seconds (assuming

you are converting several thousand records). So if you are converting

a very large database into TEAM ONE (say 20,000 to 100,000 records), this

process could take many hours or even days to complete. We do not recommend

you perform this function unless you have an uninterruptable power supply.

You may also elect to segment your database that you wish to convert into

smaller databases, which will take less computer time per day to convert

(however, it will take the same total amount of time for all the records).

When you are converting from TEAM ONE into these file formats, the program

converts data at speeds running between 10 and 20 records per second on

an average system. This is due to the fact that these database formats

don't use all of the special functions that TEAM ONE uses. However, even

at its slowest, it is still much faster to convert data than to enter

it manually.

As a further note, in TEAM ONE'S Report Writer Module, you have the capability

to generate any type of report structure, from all TEAM ONE databases,

very quickly. The REPORT WRITER allows you to redirect information into

standard ASCII files, so all information in TEAM ONE is available for

use by other types of programs through this feature.

If you have data to convert, please continue through this chapter. If

not, then you may move on to the PRODUCT/PROCESS CHAPTER.

NOTE: Full conversion of all files to DBASE III/IV type files is provided

in the Database Tools Menu option under the CARDFILES menu.

dBase III/IV Conversion

TEAM ONE has the capability to both read properly structured dBase III

type files and also to create these files from the inventory database.

dBase has some limitations on field structures and as such you must name

all fields to be converted exactly as we instruct when you wish to read

the dBase files into TEAM ONE.

It is beyond the scope of this manual to instruct you on how to create

your field structures for reading, you will have to read your dBase manual.

It must be noted that many programs have the capability to write these

files, so the creation routines are different in each of those programs.

However, as long as your data is held in a true dBase III type file, in

the field format we specify(see the Reference Manual for dBase field names),

you will have no trouble in conversion. We have supplied a sample dBase

type file for you to use as a base, if you desire. It is called INVENT.DBF.

WORKING WITH A dBASE III TYPE FILE

To convert an TEAM ONE Inventory file into a dBase III file structure,

simply follow these procedures.

Place the mouse over the INVENTORY button on the TEAM ONE MAIN MENU and

click on it with either button.

You will now see the INVENTORY MAIN MENU. Place the mouse over the CONVERT

DATA button and click on it.

You may elect to click on either the dBase or ASCII button, depending

on which conversion system you desire.

Place the cursor over the DBASE button and click on it with either mouse

button.

Now you have the option of either converting from a dBase III file to

an TEAM ONE CRD type file, or from the TEAM ONE Inventory CRD file to

a dBase III file, to be called INVENT.DBF.<$&final01[v]>

CONVERT TO CRD

NOTE: BEFORE YOU CONVERT DBASE DATA INTO TEAM ONE'S CRD FORMAT, MAKE ABSOLUTELY

CERTAIN YOUR DATA IS PROPERLY CREATED IN DBASE. FOR FULL INFORMATION ON

THE DBASE FILE STRUCTURE, SEE THE DBASE CONVERSION SECTION OF THE REFERENCE

MANUAL.

NOTE: CONVERSION TO A CRD FILE FORMAT ALWAYS ADDS RECORDS TO THE DATABASE

AND DOES NOT OVERWRITE EXISTING DATA!!

NOTE: MAKE CERTAIN THAT YOUR DBASE FILE IS NAMED INVENT.DBF AND IS LOCATED

IN THE DIRECTORY HOLDING YOUR TEAM ONE FILES.

Move into the DBASE area by clicking on the DBASE button.

Select the READ button. TEAM ONE will automatically begin the creation

process for the INVENT.CRD file. If an TEAM ONE inventory file already

exists, the convert program will simply add the conversion records to

the existing file.

During the conversion process you will notice that the conversion speed

is dramatically slower when creating a CRD file than when creating a dBase

file. This is due to the fact that the system is not just creating one

file, but is also adjusting the inventory indexing file, searching the

vendor database for vendor records to link, and linking inventories to

vendors. Just as in Inventory Input, when the system does not locate an

associated vendor record to match the VENDOR ID on the dBase record, the

system will prompt you to create one.

NOTE: During the conversion process you must create these records when

vendor information is not found. You may later adjust any records you

feel were created incorrectly (such as if you put in an incorrect vendor

id into the dbase field) by using the INVENTORY CHANGE procedure.

DBase to CRD conversion will take place at speeds between 1 to 3 seconds

per record. The Conversion process should not be interrupted, so if you

have a very large database, make certain your system has a backup power

supply to insure against any computer system failures during the conversion

process.

However, if you must interrupt conversion, Press the ESCape key. You may

use this to check the quality of the data being converted. It is vitally

important to eliminate all previously converted records from your dBase

file before beginning the conversion process again (if you interrupted

the process.). This is because TEAM ONE's reading from the first record

to the last during conversion, regardless of whether you interrupted the

conversion previously.

HINT...An alternative to stopping conversion would be to create multiple

dBase files, with each file holding different data records. Then you would

convert a file, use the DOS copy command to rename the next file to convert

INVENT.DBF and start the conversion process again.

CONVERT TO DBASE

NOTE: This process will only work if you have an TEAM ONE Inventory Database

to convert. Check program directory for the INVENT.CRD file prior to attempting

a conversion.

Place the mouse cursor over the WRITE button and click on it. The program

automatically loads the TEAM ONE Inventory file and begins the conversion

process.

NOTE: THIS CONVERSION PROCESS CANNOT BE STOPPED ONCE BEGUN.

Conversion to a dBase type file is very quick and operational speed will

reach approximately 20 records per second.

Once conversion is complete the program will automatically return you

to the TEAM ONE MAIN MENU. The program has created a file called INVENT.DBF

in the directory that contains the TEAM ONE system (Usually \TEAMONE).

ASCII CONVERSION

TEAM ONE allows you to read and write standard ASCII format files from

the INVENTORY database. It must also be noted that you may create structured

ASCII files in the REPORT WRITER module by using the Redirect To File

function. ASCII files that are created in the ASCII Write Procedure under

TEAM ONE'S Inventory Conversion System are created as "," (quote-comma-quote)

delimited files. This allows for the use of these files in other programs,

such as spreadsheets and accounting programs.

However, ASCII files that are read into TEAM ONE'S Inventory database

are FIXED RECORD LINE files. This means that the program reads a line

of characters, loads them into a targeted field on the inventory record

and goes to the next line of characters. After a specified number of record

lines have been read and written, the program moves to the next inventory

card and repeats the process. It is imperative that when you are using

this conversion procedure you must STRICTLY adhere to the conventions

of TEAM ONE, otherwise the system will create a useless database.

The definition for the format of the ASCII files to read into TEAM ONE'S

Inventory is detailed below and in the Reference Manual under the ASCII

CONVERT area. Pay close attention to the exact structure of the files,

and to the length of the fields.

LINE #	INFORMATION

1	Vendor Part #2	Internal Part#3	Vendor I.D.4	Description5	Qty.

on Hand6	Unit of Measure7	Lead Time in Days8	Warehouse Location9	Cost

per Unit

TO CONVERT FROM AN ASCII FILE

Enter the Inventory Convert Data system and move into the ASCII Convert

area, as detailed in the above section.

Place the mouse over the READ button and click with either mouse button.

TEAM ONE will now display what is known as a "Load Box". You will see

this feature in several places throughout the program. The "Load Box"

allows you to <$&6[^]>select a file to use.

You may scroll up or down through the file structure by placing the mouse

over the scroll bars to the right of the box and clicking on them. If

you desire to move up or down through directories, you will notice a "File

Folder" ( the ones with 3 arrows under it allow for movement up through

directories, the folders with no arrows are for movement down), click

on the desired folder and the display will change to the files held in

the next desired directory. The directory and path is listed on the top

of the "Load Box". You may also change drives by clicking on the "CHANGE

DRIVE" buttons. The button with the arrow up will take you to the next

drive, the arrow down will take you to the previous drive. This allows

you to load an ASCII file from anywhere in your system. The data is saved

in the directory holding the TEAM ONE program.

Once you have located the file you wish to Read, click on it with the

mouse. TEAM ONE will automatically load the file and begin the conversion

process. You will be shown the inventory records as they are converted.

Carefully check the data on the first few records. If all information

is correct, press CONTINUE to continue with error check. If you do not

desire to verify any further records, press FINISHED. This will begin

full conversion with no further verification.

You may stop conversion once it is started by pressing the <<ESC>>ape

key.

Just as in dBase conversion, the program will link inventory records to

vendor records and will prompt you to create any unknown vendor records.

The conversion process from an ASCII file to an TEAM ONE file will take

between 1 to 3 seconds per record.

Once created or converted, all inventory information is held in the INVENT.CRD

file and all vendor information is held in the VENDORS.CRD file. These

two files work very closely together and changes in one file will be reflected

on it associated records in the other file, automatically.

Note:  If there is an ASCII format that you need to be able to convert

from, please write us or contact us on the BBS so that we may be able

to accommodate your special needs.

TO CONVERT TO AN ASCII FILE

Enter the Inventory Convert Data system as detailed above. Place the mouse

cursor over the ASCII button and click with either mouse button.

TEAM ONE will now query you as to whether you desire to READ an ASCII

file or to WRITE an ASCII file.

Place the cursor over the WRITE button and click with either mouse button.

TEAM ONE will automatically convert all inventory records into the ASCII

delimited file named INVENTOR.TXT. This file is held in the directory

that contains your TEAM ONE files.

CHAPTER 3PRODUCTS AND PROCESSES

The product and process control module of TEAM ONE is extremely powerful,

and it acts as the foundation of most of the information generated the

system. It allows you to create product structures that detail the design

of a product at a sub-assembly level. It allows you to create the main

product information, and it allows you to create all processes that go

into an individual product.  The product control subsystem allows the

program to search through existing products and structures, and when it

finds a product that you need to create and locates an existing product

with the same part number, it will allow you to simply copy the information

from the existing product over into the new one, altering all primary

information, such as parent parts, lead assemblies, and level and line

id's, while maintaining the basic information in the processes, thus reducing

the amount of input time.

We'll begin in the product area by placing the mouse cursor over the PRODUCT

button from the TEAM ONE MAIN MENU.  Click on the product button with

either mouse button, and this will bring up the Product Main Menu.

<$&1[^]>You have the option of inputting a product, changing a product,

changing an hierarchy file or rebuilding a process.

We'll start by creating a product.

Place the cursor over PRODUCT INPUT button, and click with either mouse

button on the PRODUCT INPUT button.  The program will search through its

product structure files (held in the VERTLEVL.CRD) to decide if any are

left open or uncompleted from the last time you entered the program.

If you have no previous products detailed but not entered, the program

will tell you that you must now create a product structure prior to the

creation of your bill of materials and <$&2[^]>product-process structure.

Press CONTINUE if you see the above screen.

Now you will see the product level input screen.  In this area we will

build the <$&3[^]>hierarchial structure of the product.

We will begin by starting at the top level of the product.  This could

be a product such as a CAR being the parent part, and we will build all

sub-assemblies underneath this parent part; or it can also be an individual

sub-assembly itself.  You must start at the top of the structure and create

down, from the far left of the structure to the far right. (See the below

diagram for the proper path through the product structure).

<$&4[^]>Starting at parent part using a car as an example, the parent

part name would be "CAR." So enter "C-A-R." Since we're at the very top

level of the structure, the lead assembly is also car, and the assembly

itself is car.  The line id would be the line that the "CAR" is built

on.  In this case it would be line 1.  Since we're at the top level, our

present level is 0.  You can adjust this number to any number you desire.

Underneath the "CAR" we have three subassemblies: "TRANS", "ENGINE" and

"CHASSIS".  We begin by placing the mouse cursor over the downlevel button

and clicking with either mouse button.  This will save the first information

screen and move to the second.

<$&5[^]>As you will notice, "CAR" is still the parent part and the lead

assembly is still "CAR"; however, the cursor has moved to the assembly

area.  This is the product we will be working on at this point, which

in this case would be "TRANS".  The line id in this case would be line

2, and our present level is level 1.  TEAM ONE is designed to start at

the top of a product and move down on a top-to-bottom, left-to-right fashion.

Now let's say that underneath "TRANS" we have another subassembly called

"GEAR BOX".

We would go downlevel to "GEAR BOX".  You will notice your parent part

is now "CAR" and the lead assembly is "TRANS".  The assembly would be

"GEAR BOX".  The line id would be line 2A; present level is level 2.

Let's assume that also under the "TRANS", in conjunction with the "GEAR

BOX", we also have a "SHAFT".

The "SHAFT" is on the same level as the "GEAR BOX", but is built somewhere

differently.  We would then use levelright, because we are on a horizontal

plane.  <$&6[^]>You will notice that "CAR" is still the parent, lead assembly

is "TRANS", the assembly would now be "SHAFT", the line id would be line

2B, and the present level is still level 2.

We are now down as far in our structure as we desire to go, so we wish

to go back up a level, because we will now build the level next to "TRANS".

Click on the uplevel button.  The program will tell you in an alert screen

that you have selected to move up-level, and for you to check to make

certain that the created level is correct.<$&final03[v]>

The lead assembly in this case would be "CAR", the parent part is still

"CAR", the assembly in this case would be "ENGINE", the line id would

be line 3, and the present level would be level 1.

We will assume that there are no subassemblies under the "ENGINE".

Finally, we would click on the Level Right Button and add in the "CHASSIS".

Once we have created all of our information, we would place our mouse

cursor over the FINISH button and click with either mouse button.  The

product hierarchy file has been created. This file is saved under the

name "VERTLEVL.CRD".

Next TEAM ONE will detail the Continue With Creation Screen.

This will tell you that there are six sub-assemblies, (or however many

subas<$&7[^]>semblies you need to create) and will ask if you desire to

continue in subassembly creation or to leave work until later.

This allows you, during the creation steps of products and processes,

to leave the system and come back, and the system will remember where

you left, even if you lose power.

We will continue with creation. Place the mouse cursor over the CONTINUE

button and click on with either mouse button.

PRODUCT INFORMATION

TEAM ONE will now display the MAIN PRODUCT INFORMATION INPUT SCREEN. TEAM

ONE preloads certain information for you.

NOTE: You must have all products and sub-assemblies loaded into the inventory

database prior to this step.

Throughout the rest of this operation, and the building process of the

"CAR" and all related subassemblies, the program will prompt you for needed

information and drive you through each individual step of creating each

subassembly. TEAM ONE intuitively knows what product should be built and

in what order, due to the hierarchy record you created for the product.

However, it must be noted that the input of product information will be

performed on a level by level basis.

The first product we are going to create is the "CAR". TEAM ONE preloads

the <$&8[^]>product name and line id into the product information record.

The product information record provides an area to create information

that is used in estimational calculations. Once TEAM ONE's full functionality

is activated and run by the user, several of these fields will be updated

by the program. This updating will reflect actual data instead of estimational

data.

Begin at Desired Number of Work Cells.

TEAM ONE has the capability of automatically balancing an assembly line.TEAM

ONE uses a pre-defined formula for balancing structures. This input field

allows you to over-ride that formula and balance a line to your needs.

The Work Cell field allows you a manual override for the number of work

blocks or balanced work cells the program will attempt calculate to.

In this case we'll make it 3 work cells.  This tells TEAM ONE that you

would like to have 3 balanced work centers, if possible.

NOTE: TEAM ONE was designed to operate in the "real-world" environment.

This means that even though you tell the program to balance to 3 work

cells, TEAM ONE's calculation procedure may or may not be able to exactly

meet this desire. TEAM ONE takes into account that most assembly lines

simply cannot be perfectly balanced, due to spikes in the assembly time.

TEAM ONE will, however, bring the number of calculated work cells as close

as possible to your desired number.

If you leave this field at 0, the program will use its automatic calculation

procedures. This adds the total time of all processes and divides the

time by the total number of processes.

100% Line Capacity-This is the maximum design capacity of the assembly

line. In this case we will state that the assembly line can build or was

designed to build 100 cars per shift.  This is a per shift field.

Next we move down to the Actual Production Per Shift.  In actual production

per shift, once again this is estimational purposes and these fields will

be updated throughout the system, the actual production per shift is the

number of products we are really building in a shift. Enter 75 per shift.

This field is used to help calculate true product cost.

True Work Hours Per Shift.  This field relates the total number of hours

in a shift. We will assume that our shift is a standard shift of 8 hours.

Next you will see Actual Hours in each shift; this is the actual working

hours.  This would be the true number of work hours less breaks and lunch.

In this case we'll make it 7.2 hours. This field is used to help calculate

true production capacity.

Operating Shifts Per Day.  Once again, this is only an estimational field.

When TEAM ONE runs its Master Scheduler, TEAM ONE can control process

and production control operations to every 6 minutes of the day.

Operating Shifts Per Day would be 1.

Operating Days Per Week is 5.

Operating Days Per Month is 21.

The Average Hourly Wage (this is the average wage of all workers on this

line) would be $8.00 per hour.

Total Set-up Time for this line allows you to enter the amount of time

that is added to the production process when set-up is performed once

during the entire operation.

If a set-up is performed over and over again, then it is placed into the

process information area.  In this case we will place 12 minutes as a

total set-up time.

PROCESS INPUT

You may now click on either the PROCESS or CONTINUE button, and the program

brings up the process input screen.

As stated in the introduction, you must have performed a complete and

very exacting time and motion study on your line if you desire TEAM ONE

to perform its calculations in the most efficient manner. This will provide

you with routing and process control information that is extremely detailed

and comprehensive. It will also allow you to create a very tight control

over your production operations.

The process record maintains a large amount of information that is normally

not found on a standard bill-of-materials record. The record also links

the individual process directly to inventory information, if the process

is a component.  If the process is a move, test, or QC step, these will

not be linked to Inventory.

<$&9[^]>TEAM ONE preloads information into your process input area.  These

preloads include parent part names, the lead assembly, the product that

you're working on, the level that you're working in, and the line identifier.

Remember that you should enter every single process that is involved in

building a product.  A process can be a move step, a test step, actually

assembling a part, a welding step, a machining step.  Anything that takes

time should be timed and entered.

Our first process would be to retrieve a part from inventory. In this

case we'll enter a part number first, and we'll enter in part number TRANS.

The Description is a description of the step itself, not of the part.

In this case the description should be that we are going to retrieve the

part from inventory.  You do not have to fill in the vendor id, the location,

the individual part cost, or the unit of measure, as the program will

load that information for you when it locates this part in its main inventory

database.

The Time In Process is the amount of time that this process takes.  The

default value is in minutes.

Quantity.  How many of these parts are used in this process?  In this

case we're using one.  If this record were to hold a process instead of

a part, the quantity would be the number of times this process is performed

at this time. In almost all cases this would be 1.

The input date is the system date as the program is running.  You may

alter this information if you so desire.

TEAM ONE also allows you to manually track revision levels and ECOs.

You may enter this information if you desire into this area. This is only

for informational purposes and the system does not manage this information

Kanban Size, Work Cell ID, and Sequence Numbers.

These areas are not adjustable by the user or should not be entered by

the user. TEAM ONE will set these for you.  You can enter information

into these, but the system will override it in the CALCULATE Subsystem

and in Master Schedule.

You may also determine whether this is a make part or a purchase part,

if it's a part.  If it's a process and not a part, then you would not

click on either of these buttons.  If you are not certain whether it is

a make part or a purchase part, when TEAM ONE locates the associated inventory

item, it will inform you if you have made a mistake and correct that mistake

for you.  In this case we will tell it that it is a PURCHASE part, although

it is actually a MAKE part.

We may also identify the process type.  This is primarily for information

only, except that a Machine Step tells the program to adjust necessary

labor and labor costing information.  We have move steps, test steps,

quality control, machines, and value-added steps.  This allows you to

go through a process structure and determine what are wasted steps, such

as move steps.  In this case this is a move step.

We would now press CONTINUE.

This allows the system to search through the inventory and locate the

associated part.  As we said earlier, if you had made a mistake, as we

purposefully did, entering the make part/purchase part area, the program

will notify you of such, and then ask you if this is the inventory card

that you desire.  If it is the inventory <$&11[^]>record that you desire,

you would place the mouse cursor over "THIS CARD" and click with either

button.

If you are maintaining multiple inventory items under the same part number

and in different locations or attached to different vendors, you may click

on the "FIND NEXT" button. This will allow the program to continue searching

the database for any additional entries.  In this case we will accept

the inventory record that has been found by pressing the THIS CARD button.

The program loads in the inventory information, telling you the vendor

ID, the location of the part, the individual part cost, and the unit of

measure.

If this is the only part to build into the process, you would then click

on the FINISH button.

If you desire to add additional processes, then you would click on the

CONTINUE button.

If for some reason you have made an error on this area, you would click

on the CANCEL button.  The CANCEL button will clear the card, move it

back to the original screen, and let you start over at the sequence you

were at.  If you are at sequence 5, it would maintain itself at sequence

5 and only erase the information from that one entered record.

In this case we will tell the program that we are finished.  The program

will save the data information and all hypertext links that were created

between the process and the inventory records.

TEAM ONE will now bring up the Continue With Creation Screen.  TEAM ONE

will inform you that there are several subassemblies left to create, and

do we desire to continue creation or leave work until later.  We may press

the LEAVE WORK button, and that will take us back to the TEAM ONE MAIN

MENU.

If we move back into the product input area (let's say that we've gone

home for the day and come back) and we reactivate product input, the program

will notify us <$&WARNING[v]>that we have created a product structure

but not built all of its associated bill of materials.  TEAM ONE will

remember where we left and what we have left to do.  It will ask us if

we desire to continue subassembly creation or leave work until later.

TEAM ONE PROCESS COPY FUNCTION

<$&COPY[^]>This area allows you to use pre-existing information quickly

and easily. If you have previously developed a product/process relationship

in a sub-assembly or even a parent part, TEAM ONE allows you to use this

information in the building of a new product that holds this sub-assembly

as one of its levels. TEAM ONE also allows this information to be used

in the creation of a new top level product.

During the second tier of Product Input operations, after you have decided

to Continue with Creation, TEAM ONE searches its entire product database

for assemblies that match the product name for the assembly you now desire

to create.

If TEAM ONE locates a matching assembly, it informs you of this. You now

have two choices, you may elect to have TEAM ONE copy the existing process

information into the new structure (TEAM ONE automatically adjusts the

new structure to match the hierarchy, level control and line information)

or you may <$&12[^]>elect to have TEAM ONE run through the existing procedures

and processes one at a time.

The reasoning for this latter capability is so that if you have a product

that contains a process structure that is very similar to one that already

exists, but is slightly different, you may name the sub-assembly the exact

name of the existing structure. You would then have TEAM ONE copy the

pre-existing structure one process at a time, allowing you to edit each

record. After the copying process is complete you would use the VERTICAL

LEVEL CHANGE, PRODUCT CHANGE and PROCESS REBUILD functions to alter the

product name to a new product name. This means that you can copy the main

information quickly and then simply edit small amounts of information

at a later time.

HIERARCHY CHANGE

<$&HIERARCHY[^]>The operational menu is located at the top of the screen.

This menu holds a series of buttons that allow you to easily move through

the database or work with specific records in the database. It should

be noted that in the CHANGE areas, the user may not add or delete records.

We have determined that deletion procedures should only be accessed by

the system administrator. Addition functions can only be performed in

the input areas of the modules. You may edit information on records.

Under the Change Menu Bar, TEAM ONE details the current record that you

are viewing and the total records held in the database.

The FIRST button takes you to the first record in the database.

The LAST button takes you to the last record in the database.

The NEXT button takes you to the next record in the database.

The PREV(IOUS) button takes you to the preceding record in the database

and not the previously viewed record.

The FIND button allows you to search the database for any information

held in any or all fields. The more information you enter into this record

search, the more selective the search will be. However, you must enter

EXACT information.

The MORE button continues the search using the information you previously

entered.

The EDIT button allows you to edit a record. Once you have finished editing

the record, click on the CONTINUE button.

To leave the CHANGE area, click on the EXIT button. This will save any

changes you performed and return you to the TEAM ONE MAIN MENU.

PRODUCT CHANGE

The operational menu is located at the top of the screen. This menu holds

a series of buttons that allow you to easily move through the database

or work with specific records in the database. It should be noted that

in the CHANGE areas, the user may not add or delete records. We have determined

that deletion procedures should only be accessed by the system administrator.

Addition functions can only be performed in the input areas of the modules.

You may edit informa<$&13[^]>tion on records.

Under the Change Menu Bar, TEAM ONE details the current record that you

are viewing and the total records held in the database.

The FIRST button takes you to the first record in the database.

The LAST button takes you to the last record in the database.

The NEXT button takes you to the next record in the database.

The PREV(IOUS) button takes you to the preceding record in the database

and not the previously viewed record.

The FIND button allows you to search the database for any information

held in any or all fields. The more information you enter into this record

search, the more selective the search will be. However, you must enter

EXACT information.

The MORE button continues the search using the information you previously

entered.

The EDIT button allows you to edit a record. Once you have finished editing

the record, click on the CONTINUE button.

To leave the CHANGE area, click on the EXIT button. This will save any

changes you performed and return you to the TEAM ONE MAIN MENU.

PROCESS REBUILD

We have attempted throughout TEAM ONE to make many functions the exact

same, such as all change functions are identical not matter what module

or subsystem you are in.  However, the one variation to this is PROCESS.

You cannot change a process; you have to rebuild the process.  This because

the process structure is so vital to the structure of the program itself.

If you have made a mistake in a process or you need to add, delete, or

edit process information, such as changing a part number or deleting a

process field, you would enter the Process Rebuild area.

From the TEAM ONE MAIN MENU, place the cursor over the product button

and click with either mouse button.  This will bring up the PRODUCT MAIN

MENU SCREEN.

Place the mouse cursor over PROCESS REBUILD and click with either mouse

button.

TEAM ONE will now request the name of the product that you wish to change

<$&15[^]>the processes inside of. In this case we will change the "rear

gear assembly".

NOTE: We have purposefully made all information fields not case sensitive.

The information must be exact, but it becomes a bother to remember what

was capitalized and what was not.

You should also enter the line ID.  This is due to the fact that TEAM

ONE allows you to build multiple products of the same name on different

lines.

Once you have entered the product name and line ID, you would place the

mouse cursor over the CONTINUE button and click with either mouse button.

If you desire to exit this area, you would press the CANCEL button.

Place the cursor over the CONTINUE button and activate with either mouse

button.  TEAM ONE will now bring up the process information area.

You will notice that in the process area the CHANGE procedure is similar

to all other CHANGE functions. However, you now have the option of adding

a <$&16[^]>card, editing a card, deleting a card, moving to the next card,

moving to the previous card, or finishing.

We desire to change the part number on the first card from 7 to L1002.

First we would click on the EDIT button. We would then place the cursor

over the part number/process field. Now we would type in the new part

number, and press the continue button.

TEAM ONE will perform the functions exactly as it did in process input

by locating the associated inventory card and asking if this is the inventory

record you desire.  If it is, you would press "THIS CARD", and TEAM ONE

will load the new information in for you.

The program will ask if you desire to save this card.  Pressing CONTINUE

saves the card and displays it now in the process record.

You may move through the records by clicking on next card or previous

card.

When you come to the last card on the process record, the system will

notify you that you have reached the end of the file.  It will then display

(by pressing NEXT CARD) a blank record.

This is because TEAM ONE will allow you to enter new information at this

point and add it to the end of the file.  Although, if you do not use

the edit procedure and tell it to save this information, this record will

not exist in your process file.

During Process Rebuild, when adding cards or deleting cards, TEAM ONE

will automatically reassign sequencing to the line. You will, however,

have to rerun the calculation procedure again to obtain an updated workcell

relationship.

CHAPTER 4 CALCULATE

Once you have created the primary information in inventory, vendors, and

product and process control, the program will now take this basic information

and provide you with a massive amount of information in all areas of your

manufacturing system and inventory control.

The TEAM ONE CALCULATE SYSTEM provides basic manufacturing calculations.

It provides for line balancing, breaks the individual processes up into

balanced work centers, provides product costing, and other important areas.

Place the mouse cursor over the CALCULATE button from the TEAM ONE MAIN

MENU, and click on either mouse button.  This will bring up the CALCULATE

PRODUCTION MAIN MENU.

You have the option of calculating on a single product (this includes

not just <$&1[^]>parent parts but any sub-assembly) or a product family.

This allows you to perform calculations from any product down through

all levels associated with that product.  This can be a parent part or

any subassembly and all related subassemblies.

We will begin by working with a single product.  Place the mouse cursor

over SINGLE PRODUCT, and click with either mouse button.

This brings up the Product Identification Screen. This screen allows you

to selectively choose a product you wish to work with from a list of products,

or, if you <$&2[^]>know the exact product name, you may simply enter that.

By placing the mouse cursor over the YES button, you would bring up the

product list. By entering the product name and clicking on the CONTINUE

button the program would automatically load that produc<$&3[v]>t.

It is usually best to use the list of products so that you will know the

name is exact.

Place the mouse cursor over the YES button and click with either mouse

button.

TEAM ONE will now list all products that have been placed into the system.

Place the mouse cursor over the desired product.  If you do not see the

product in the list of products to choose from on this screen, you may

place the mouse cursor over the up or down arrow on the scroll bar and

scroll through the additional products. Or you may simply press the Page

Down/Page Up keys.

Once you have found the product you desire to calculate on, place the

cursor over that product and click on either mouse button.  The mouse

cursor will now automatically move down to the CONTINUE button.  Click

with either mouse button on the CONTINUE button, and TEAM ONE will now

display the associated product card for the process products that you

are going to calculate <$&4[^]>on.

This is simply an informational area.  Click on the CONTINUE button with

either mouse button, and TEAM ONE will begin its manufacturing calculations.

This is a very speedy process and usually takes only a few seconds to

perform.

Now you will see the Production Information Screen. This will list the

name of the product that you calculated on, the Line ID for that product,

total number of <$&5[^]>processes involved in that product, the calculated

labor force, and the line efficiency.

This is the amount of production that you are presently performing, versus

the amount of production that the line has the capability of creating

(which was entered into the product information screen back in product

input).

TEAM ONE will detail the 100% line capacity per shift and the actual production

per shift, as you entered them in the product information area under the

product input.

TEAM ONE will calculate total production costs if you are running at 100%

capacity.  This is material and labor only.  It does not account for any

cost overheads.  You may adjust this figure to reflect cost overheads

by simply adding those figures into your original figures either in inventory

or in your labor determination.

TEAM ONE will show you the cost of an individual product as built.  This

is determined by the amount of time that it takes, the labor cost, and

the individual material costs.  Any processes that are identified as machine

steps are not entered into the calculation as labor.  To include machine

costs in the equation, you may either simply add the machine's operating

cost (per operation) to the Cost field on the Process card and identify

the process as a value-added step, or add another process, after the resource

process that contains only the description of the machine operation and

the associated cost of the operation with no time allocated to this process.

This process should be identified with the PROCESS TYPE of MACHINE.  The

process will add cost to the product but will not affect the direct labor

cost.

Calculated balance cycle time.  TEAM ONE has to work in reality.  By this

what we mean is that many instructors in JIT-type training will tell you

that any line can be balanced perfectly.  That is hogwash.  There will

always be spikes and valleys in a line, no matter what you do.  Nothing

can be balanced to perfection.

TEAM ONE will attempt to balance to the best possible cycle time that

it can create.  The calculated balance cycle time is developed automatically

when the manual work block override area is not activated in the product

input screen. Or TEAM ONE will use the number of work cells you desire

to create by using the manual override. This action occurs if a number

greater than 0 is entered in the # Work Blocks area of the product input.

The total production time is the amount of time it takes to build a product

from start to finish.  Remember, the calculated balance cycle time is

the average amount of time that it will take in creating balanced work

centers.

TEAM ONE provides you with a calculation date and time of calculation.

This is for an historical record.  All of these product information records

are saved in a file called PRODINFO.CRD.

You may elect at this time to either continue, which will NOT balance

your processes, or you may elect to have TEAM ONE balance your individual

processes into work centers for you by clicking on the WORKCELLS button.

The reason for the two areas is that you have the ability to change information

in product input and also process rebuild to try to adjust a line by moving

processes or adjusting number of hours or work centers, or costing.

By using the calculate mode of TEAM ONE you have the ability to view the

most efficient line or most productive line.  However, you may have already

told TEAM ONE, through earlier calculations, that you simply want a balanced

figure and you don't want to adjust your process balancing or your physical

layout of your assembly line each time you re-calculate the line.

The only time that TEAM ONE will rebalance a line and reassign individual

processes to individual or newly created work centers is when the WORK

CELLS button is activated.  By activating work cells with either mouse

button, TEAM ONE will now balance all processes and assign those processes

to work centers.

Once again, this is a very quick operation and usually takes only a few

seconds to perform.  If you wish to review how TEAM ONE balanced your

processes, you may go into the PROCESS REBUILD subsystem or use the TOOLS

subsystem.  You may also print a report through the REPORT WRITER on all

processes.<$&calc4[v]>

PRODUCT FAMILY

TEAM ONE's calculate feature also allows you to calculate entire product

families.

Once again place the mouse button over the CALCULATE button.  This will

bring up the CALCULATE PRODUCTION MAIN MENU.  Place the mouse cursor over

the Product Family button and click with either mouse button.

The program will now bring up the Product Family Decision Screen.  In

this area you would enter the name of the product, the highest level you

want to calculate <$&7[^]>from, and its associated Line ID.  Once continue

is clicked, the program will perform the same processes in calculation

that are performed in a single product; however, it will bring up each

individual associated product and subassembly underneath and including

the product you entered into the product family decision screen.

As before, if you desire the program to recalculate work cells each time

it brings up the production information area, you would click on the WORK

CELLS button. Any time you did not want it to recalculate work blocks

you would simply press CONTINUE.  The program will roll through all associated

processes and then return you to the TEAM ONE MAIN MENU.

CHAPTER 5 ORDER ENTRY SYSTEM

TEAM ONE's ORDER ENTRY SYSTEM integrates customer orders with production

scheduling and vendor part ordering.

When you enter a customer order and approve it, the program automatically

generates a master schedule request.  During master schedule operations,

the master schedule determines what parts need to be ordered and from

what vendor and returns this order back to the order entry system under

vendor ordering.

NOTE: In TEAM ONE, no products may be built and no parts ordered, unless

they are entered through the Order Entry System.

To activate order entry, place the mouse cursor button over the ORDER

ENTRY button on the TEAM ONE MAIN MENU and click with either mouse button.<$&main[v]>

Now you will see the ORDER ENTRY SYSTEM MAIN MENU<$&1[v]>.

We will begin by entering a customer order.

Place the mouse cursor over the CUSTOMER button and click.  This brings

up the CUSTOMER MENU. Click on the ADD ORDER Button. The program now brings

up a Customer Selection Screen.

<$&2[^]>If you already have customer information loaded into the database,

filling out any one or more of these fields on the customer selection

screen and then pressing the CONTINUE button will allow the system to

search the customer database, locating the appropriate record and filling

in the remaining information for you.

In other words, if you simply know the Customer ID number, then you would

just fill that in and press CONTINUE, and the program will locate all

other information and fill it in for you.  Or you can put in your client

name, the company, or any other information on the card.

The program then brings up the Customer Information Screen.  This is all

basic client information.  Anything that you need to change at this point,

such as address, <$&3[^]>phone number, or whatever you wish to change

under this ID, you would change at this point.

You would now move down to the list box at PRODUCT ORDERED, place the

arrow into the list box, and click with either mouse button.

You will notice the flashing cursor now appears inside the list box.

Here you would enter the product ordered.  This must be the product number

as used in the Inventory.  However, remember this information must be

exact.

NOTE: TEAM ONE allows you to enter ordered products for products that

do not exist. This is so that you may have a list of products that you

may wish to build, or even accept orders for products that do not presently

exist and then build their structure afterwards. This allows TEAM ONE

to access its optional CONFIGURATOR module that will be available.

Once the information has been entered, click on the PRODUCT link button

to the right of the Product Ordered list, and this brings up the Products

To Order <$&5[^]>Line Item Information Screen.

Information that has been preloaded by the program includes the customer

name, the customer ID, an internally generated control number, and the

product ordered.

The internally generated control number is a combination of the date and

time. If you had entered multiple products to order in the initial screen,

this number will be the same on all orders placed at this time.

The program now expects you to fill out the customer PO. TEAM ONE tracks

both external customer POs and internal order numbers.

You should now enter the Quantity Ordered.

The order date could be today's date or any date that the order is to

go active.

The requested shipping date is the date the customer desires this order

by.

To the right we have the order status, and under order status we have

check boxes.  You may only check on one of these boxes. As the program

performs functions it will automatically update and alter the check box

information. <$&allstat[v]>_

<$&taken[^]>Initially an order is usually TAKEN but not CONFIRMED and

APPROVED.  This means that you have taken the order, you have queued the

customer order, but you have not run it into the master scheduler, you

have not created a master schedule, and the order is just sitting there.

<$&confirmed[v]>

CONFIRMED status means that it has been confirmed by the customer. This

could happen when they supply the P.O. number or terms.<$&approved[v]>

APPROVED means that TEAM ONE will be allowed to create a QUEUED Schedule

operation. <$&queued[v]>

_QUEUED means that it is placed into the master scheduler in a queued,

priority one status.<$&active[v]>SET BY THE SYSTEM

ACTIVE means the scheduler has activated the order. This action is performed

by TEAM ONE.SET BY THE SYSTEM

<$&hold[v]>HOLD allows you to have an approved order but place it on a

hold status.  By clicking on this the master scheduler has specific areas

where it will run hold orders. <$&cancel[v]>SET BY THE SYSTEM

You may CANCEL the order.  This cancels the order, but does not erase

the record. This is one area where cancel still saves information. This

is so that in the future you may go through your customer database and

see how many orders are cancelled for selected products or by customers.<$&built[v]>

The BUILT box is activated by the BACKFLUSH SYSTEM in TEAM ONE, and it

tells when a product has been built. <$&other[^]>_SET BY THE SYSTEM

OTHER is for future use.

If you are the one entering the order, and you have the confirmation from

the customer, and you have the ability to approve it, at this point you

would click on the Approved box.

Next we'll move back to our left to priority level.  Priority 1 orders

take place first, priority 2 orders take place second and automatically

go into a hold status.

Priority 1 sets the customer order and master schedule automatically into

a queued status.

If you forget to click on either one of these boxes, the program will

inform you as such, and will tell you that it automatically determined

the order as a priority 2.

Ship partial order-This is used by the WAREHOUSE SHIPPING function.  If

you select YES, partial Picks of the order are allowed for shipping.

Sales ID-This allows you to enter your sales ID for tracking purposes.

The customer order record is a permanent record, and it allows historical

tracking of all customer information.  This will allow you to see when

orders were made, by whom, for what products, when they were delivered,

when they were built, and how many were built.

At the very bottom of the screen you will see quantity produced and end

production date.  These are filled in by the system as a product is being

built and/or products are taken from inventory stock. The quantity produced

field is updated first by the Master Schedule when it allocates inventory

and then each time the Activation Module updates a production run (if

this order is allowed to be updated). When TEAM ONE determines that the

total quantity ordered has been build, the program will enter the date

of final production and change the order status to BUILT.

Once you have filled out the line item, press CONTINUE.  This brings up

a notification flag, that TEAM ONE is creating a shipping record for this

product.  If you desire to use the same address for shipping as you have

on the customer record, you would press SAME.

If you press CANCEL on any one line item, it will be erased and the next

line item within the PRODUCTS ORDERED list will be displayed.  If the

cancelled record is the only one in the order, then the system will erase

the line item record but maintain the customer information for later orders._

If you desire to enter new information, such as you wish to ship to a

different address for that customer or you are doing drop shipping, then

press on NEW.

By clicking on NEW, we bring up the shipping information input screen.

This lists the customer ID, and then you have fields to fill out for the

name, company, <$&6[^]>address, city, state, zip of where you wish the

product to be shipped.  Authorization is by the client name.

PO number is the customer PO.

Ship date, date shipped, Pre-pay, COD, or PO are all filled out by you

at time of shipping.

At the bottom the record will show you the product shipped and the quantity

shipped.

Pressing CONTINUE here saves this record and brings you up to the sales

confirmation screen.  This gives you an overview of the customer and product

information.  If you had entered more than one product in the product

ordered area, the program would have simply rolled through the customer

line item ordering system until you finished and then returned to this

screen.

You will notice to the right of the product button a shipping button has

now appeared.  These buttons are link buttons, and this tells you a shipping

link has been created between this customer record and a shipping record.

These are simply confirmations that the information has been properly

linked.

Once the shipping information has either been accepted or input and you

press the CONTINUE button, you will return to the CUSTOMER SELECTION SCREEN

to input the next customer order that is received.  If you no longer want

to remain in the CUSTOMER section, press the CANCEL button and you will

return to the TEAM ONE MAIN MENU.

If during the creation process you had decided that you wanted to create

hold orders and then activate all of those hold orders at a later time,

you would use the order activation system.

ACTIVATE CUSTOMER ORDERS<$&OEMM[v]>

The ACTIVATE ORDER option would be used when you have previously created

an order, but did not set it to an APPROVED status. At a later date you

would edit the order, by using the CHANGE CUSTOMER ORDER option in the

CHANGE Menu under Order Entry. After you edited the order and changed

the status to APPROVED, you would then need to activate the order manually.

This activation is performed automatically when an order is initially

APPROVED when creating the order.

Click on the ACTIVATE ORDER option under the CUSTOMER MENU.

The ACTIVATE ORDER area is simple and straight-forward. When the ACTIVATE

ORDER button is pressed, TEAM ONE searches the customer order database

and looks for all approved files. It then runs these files and creates

schedules for these, whether they need to be QUEUED or HOLD status orders,

as determined by the priority level of the order.<$&100[v]>

To bring orders up for the ACTIVATE ORDER system, use the ACTIVATE ORDER

button under the CUSTOMER MENU in the ORDER ENTRY MENU.

Once Activate is run, the system is automatic and upon completion of its

task, informs you that all approved orders have been run.

TEAM ONE is an interactive program between the various modules. Orders

are created here and then moved to the master schedule. The master schedule

then creates production run orders and vendor part orders. These vendor

orders are then moved back to the order entry system. Then the operator

activates the vendor orders.

We will go into definitions in detail of other areas that are happening

in the order entry system, and we will have to refer back and forth between

the master schedule and this area after we move into master schedule

in the next chapter.  So let's move into the vendor system.

VENDOR ORDERING SYSTEM<$&VENDORMM[v]>

Reenter the ORDER ENTRY SYSTEM, if you left it.

Place the mouse cursor over the VENDOR button and click the left or right

mouse button.  This brings up the Vendor Menu.

You have the ability to ACTIVATE Vendor orders, to CREATE a Vendor order

or to RUN P.O.'s.

The ACTIVATE button activates all orders that were created in the master

schedule.  The master schedule, as we have said before, determines what

parts to order, when to order them, and from what vendor. The scheduler

creates a vendor line item which is very similar to the customer line

item except that the vendor line item is for parts while the customer

line item is for products, and sends them back to the order entry system.

Once activated, the orders are Queued for running as P.O.'s.

You then have the ability to run the purchase orders that are created

by the TEAM ONE through your accounting system.

RUNNING P.O.'S

The P.O. system takes all active vendor orders and prints them to either

a standard line printer or to and ASCII file called PURORDR.TXT. The RUN

P.O. system sets the vendor order to a "Run" status, increments inventory

quantities ordered and creates the associated Expected Receipt record

for the part.

If the system detects a fabrication order, it creates a shipping request

for all needed parts associated with the fabricated product to be shipped

to the vendor and generates an expected receipt for the finished product

from the vendor.<$&POINSCREEN[v]>

However, because that is all a transparent operation, we will create a

manual order for parts, so that you can see what the vendor records look

like.

Place the cursor over the CREATE button, and click either mouse button.

You will see the Purchase Order Input screen.  The cursor will be flashing

down at the vendor ID area. Enter an appropriate vendor ID (remember this

has to match an exact vendor ID that was placed up in the vendor input

area).

Now press CONTINUE. The system will locate the appropriate vendor and

load in all information.  In this case, in product ordered, you would

enter in the part number or product that you are ordering from the vendor.

STRETCH: Remember that TEAM ONE holds all sub-assemblies in inventory

and allows the use of a line to be a vendor and a line to be a customer.

At this point you could enter an order for sub-assemblies to be built.

When the scheduler is run and parts are ordered, the scheduler makes no

differentiation between an outside vendor supplying parts and an internal

vendor supplying sub-assemblies. An order is an order.

Clicking on the PRODUCT button, just as you did in customer, now brings

up the Products To Order area.  Products to order area gives you a correlation

between the vendor part number and your part number, shows the vendor

ID, and <$&11[^]>the internally generated purchase order number.

A difference that you will notice between this order number and an internally

generated customer order number is there is an "I" preceding the internal

order number.

You may also enter in the customer order number if you desire to tie a

customer order directly to this part.

The TEAM ONE master scheduler automatically performs this during its automatic

generation of part requests.

Enter the quantity of these parts that you need.

The order date is the default system date.

The need date is the date that you desire the parts/sub-assemblies to

be delivered by.

Under order status we have several boxes.  These are different than in

the customer area.

Approved. Once a vendor order has been approved, the system may activate

it.

Active is activated by the system and tells that this order has been created,

and is ready to be run.

Hold says that we think we want to order this, but not yet.

Fabrication is a special area that we have created which allows us to

take a series of parts from our inventory, ship them out to an outside

fabricator, and the expected receipt that is created will be for a finished

product that will come back into our warehousing.

TEAM ONE has the ability to track outside fabrication using internal parts.

Once parts have been received in the proper quantities, the warehousing

system activates the RECEIVED status box.

The RUN checkbox is activated by the RUN P.O. system, so that when purchase

orders are created this box will be activated and they will not be printed

again.

Cancelling the order--remember that cancelling under here and under customer

order does not erase this record; cancelling merely says that the order

was cancelled, this record is saved and maintained.

Because many vendors have multiple priority levels, we have given four

priority levels here.  This is for informational purposes only.

You may also decide whether the vendor may ship partial or all of a shipment.

In the top right hand corner you will notice under order type "MANUALGEN"

or "AUTOGEN"; in this case it's a manually generated order.

Under ordered by and approved by you would enter the name of the party

ordering and the name of the party approving the order.

In an automatically generated order, out of the master schedule, ordered

by will be the line that desires the part, approved by will say "SYSTEM",

order type will be "AUTOGEN".

It is advisable that when you are filling out the vendor record manually,

click on the approved box and activate it, then at a later date you may

go through and do a batch activation on all orders.  Or you may activate

this order immediately.

Pressing CONTINUE brings you back to the products to order list.  This

is for final confirmation.

Pressing CONTINUE saves the information; pressing cancel erases the information.

By pressing CONTINUE or CANCEL returns you to the ORDER ENTRY SYSTEM MAIN

MENU.

Other operations in the order entry system are driven by other areas of

TEAM ONE, such as the creation and activation of vendor orders and the

final update of the shipping information records after the product has

been built.

ACTIVATE VENDOR ORDERS

To activate vendor orders, move under the vendors menu, place the mouse

cursor over the ACTIVATE button and click on either mouse button.  (Do

not confuse this with the ACTIVATE mode of the CUSTOMER AREA of ORDER

ENTRY.)

The program now requests whether you desire BATCH approval or approval

by INDIVIDUAL part<$&15[v]>.

If you press on BATCH approval, the TEAM ONE will run through all the

vendor parts that are to be placed in an order status or have been placed

in an order status, either by manual generation or automatic generation,

searches for an approved status on these, and activates the order.

Under INDIVIDUAL you are allowed to select the individual orders to activate.

Pressing CANCEL,cancels this operation.

Once you activate either button you have the ability to either automatically

<$&20[^]>generate a purchase order number or to enter a purchase order

number manually.

Automatic generation of the purchase order creates the internal order

number by using system time and date and adding the I in front of it.

Manual allows you to enter your own or an individual purchase order that

you desire to create.

CHANGE

The CHANGE mode for order entry is identical to all basic change procedures.

Enter the Change mode by pressing the CHANGE button from the ORDER ENTRY

MAIN MENU.

You now have the option of changing information in:

Customer Information...this allows changing of the initial customer order

record which contains address and general contact information.

Customer Orders...this allows changing of the individual line items ordered.

Vendor Orders...this allows changing of the individual vendor orders.

PRIMARY CHANGE INFORMATION

The operational menu is located at the top of the screen. This menu holds

a series of buttons that allow you to easily move through the database

or work with specific records in the database. It should be noted that

in the CHANGE areas, the user may not add or delete records. We have determined

that deletion procedures should only be accessed by the system administrator.

Addition functions can only be performed in the input areas of the modules.

You may edit information on records.

Under the Change Menu Bar, TEAM ONE details the current record that you

are viewing and the total records held in the database.

The FIRST button takes you to the first record in the database.

The LAST button takes you to the last record in the database.

The NEXT button takes you to the next record in the database.

The PREV(IOUS) button takes you to the preceding record in the database

and not the previously viewed record.

The FIND button allows you to search the database for any information

held in any or all fields. The more information you enter into this record

search, the more selective the search will be. However, you must enter

EXACT information.

The MORE button continues the search using the information you previously

entered.

The EDIT button allows you to edit a record. Once you have finished editing

the record, click on the CONTINUE button.

To leave the CHANGE area, click on the EXIT button. This will save any

changes you performed and return you to the TEAM ONE MAIN MENU.

ESTIMATOR

The ESTIMATOR provides a very fast process of finding out if and how you

can build a customer order by looking at a specific window of time. The

ESTIMATOR will tell you when production can be completed by and what parts

need to be ordered to build the customer order. Parts and Products that

need to be ordered are saved in the Vendor Orders database, they are set

to a CANCEL status and are ordered by ESTIMATOR.

The ESTIMATOR function is very similar to the Run Queued or Run Hold Orders

function in the Master Scheduler. The main differences are that instead

of the program automatically loading product name information you must

enter the name of the product you wish to estimate on, the line you wish

to run the schedule on, the quantity you desire to build and the production

starting date. The ESTIMATOR looks at the specific window of time for

the calculation and compares desired capacity to present use capacity,

desired inventory to present inventory levels and provides a production

ending date and saves potential vendor orders in the Vendor Order database

for review. The ESTIMATOR is many times faster than the actual scheduling

system, due to the fact that it is not having to save information to all

the databases that the Master Scheduler must save to. Additionally, you

must remember that the calculations performed by the estimator are for

that one moment in time that is has been run. Should you increase or decrease

inventory levels, add or remove operational schedules, then the estimations

will be different.

Enter the ESTIMATOR by the ORDER ENTRY MENU.

Enter the Product/Sub-Assembly you wish to estimate on, the Line ID, the

Quantity Desired and Production Starting Date. Click on the CONTINUE button.

The ESTIMATOR will check present inventory levels and inform you if production

needs to take place to build the quantity desired. If Production does

not need to take place you are returned to the TEAM ONE MAIN MENU. If

production does need to take place, you must enter an allotted capacity

to the order. The system will check all existing schedules, capacities

and needed orders.

For more information on ESTIMATOR functions as they relate to the Master

Scheduler, see RUN QUEUED ORDERS in the Master Schedule Section of this

manual

CHAPTER 6 MASTER SCHEDULER

When the fundamental database information has been built into TEAM ONE

through inventory, vendor, and product information, the order entry system

creates a master schedule card when a customer order is approved with

a Priority of 1.  As we stated before, nothing can be performed in the

program through master scheduling operations of any type unless an order

has been created. Additionally, nothing can be performed in the Master

Scheduler until you have first run primary manufacturing calculations

using the CALCULATE SYSTEM (See Chapter 4).

Once an order creates a master schedule item, you will move into the master

schedule and begin setting up a master schedule and demands upon your

components within your Process database for the ordered product.

From the TEAM ONE MAIN MENU, place the mouse cursor over the master schedule

button and click with either mouse button.  This brings up the <$&1[^]>MASTER

SCHEDULE MAIN MENU.

You must first create a schedule calendar for the product and line. If

a calendar is already created then you may skip this step.

Place the mouse cursor over the CALENDAR button on the MASTER SCHEDULE

MAIN MENU, and click with either mouse button.

The program now displays the primary calendar information card.  In this

area <$&2[^]>you would type in the exact name of the primary product that

you wish to create the manufacturing calendar for and the associated line

ID for that product.

TEAM ONE will default to the present year on the Year To Adjust, starting

with the present system date month.

You have the ability of adjusting the years or months for TEAM ONE to

begin the creation of the shop calendar.  Additionally you have seven

checkboxes for the normal days down.  This allows for global elimination

of any work on those days.  By clicking on any or all of the days, you

can zero out work performed on those days.

When your primary calendar parameters are created, press the CONTINUE

button to bring up the first shop calendar.

The program informs you that it is creating a shop calendar, and brings

up the TEAM ONE shop calendar display.  This details at the top the product

name <$&3[^]>and the line ID for the shop calendar.

Remember this information must be exact on both line ID and product name

or the system will not allow you to create the calendar._

The calendar shows the total operational hours for that month, which month

you are presently working on, and which year.

In the area where the boxes and columns are at, the program first notes

the day of the week in two week blocks.

The DT column is the date column.

Hours is the actual working hours per day, which was entered into the

product information screen back in product process.

The capacity used is an area that is set up by the master scheduler once

it begins to run a queued order.

The scheduler gives you the capability of altering any day of any week

so that, for example, on the 3rd, when the scheduler says we have 8 working

hours, we have the capability to change that to 12.  Or if the 15th it

is a holiday that is off, we have the ability to change the hours to zero.

By pressing CONTINUE brings up the next month, and you would continually

roll through the months.

Pressing the CANCEL button will stop additional month creation, but will

save the months already created.

We advise when first setting up a manufacturing calendar, to set the calendar

at least a five-year time span.  The calendar will allow for correct dating

up to December 31, 2099. If your production runs exceed this date, then

you don't have to worry about it because your product should be obsolete

by then.

If you have no down days, you may simply click on the enter key 12 times

to bring up the add next year decision screen. <$&final06[v]>

This allows you to decide if you wish to add another year or not.  By

clicking on the YES button, this once again brings up the operating day

selection screen and the year to adjust has been altered.

Remember also that the global deletion of working days is only for that

one year, although it will retain any information that you have. You may,

for example, in year 1992 not work on Sundays, so in 1993, Sundays will

be highlighted, but you may click on the Sundays checkbox again and that

will turn Sundays back on.  Or click on Fridays to have Fridays turn off.

An X in the checkbox determines that the day is a down day.  Remember

this is a global elimination process; however, during the month creations

you have the ability to turn individual days back on.

CALCULATE SCHEDULE

Once you have created the calendar, you are ready to calculate the schedule.

Place the mouse cursor over the RUN QUEUED or RUN HOLD button and click

either mouse key.

Queued orders were created in the ORDER ENTRY SYSTEM with the approved

status and a priority level of 1.

Hold orders were created in the ORDER ENTRY SYSTEM and developed under

the hold status using priority 2.  Operations through both areas are the

same.  However, the difference is that on the queued orders there is a

checkbox on the schedule card which denotes queued, and on the hold orders

there is a checkbox on the schedule input card which denotes hold.

Place the mouse cursor over the RUN QUEUED ORDERS button and click with

either mouse button.

This brings up the master schedule selection screen.  This screen will

display the first located queued orders.

<$&8[^]>If the first order that is brought up is the order you desire

to work with (in the red area TEAM ONE is asking "Is this the schedule

card you want?") you would click on the YES button.

If you desire to work on another schedule card, you would click on the

NEXT button and the program would proceed to find the next queued order.

Clicking on the YES button then brings up the enter line to run on and

dates.

The product name is filled in by the order entry system but the line ID

is not.  This is because the order entry system does not know what line

the master <$&9[^]>scheduler desires to build the product on, so you must

at that point enter the line ID.  Remember that the line ID and product

name must match exactly to the A) calendar that you are running and B)

product and production information cards that you are running.

You will see displayed the internal customer order number that was created

by the system in order entry, the customer ID, and the quantity desired.

You will enter the production starting date.  The program will develop

the production ending date.  Once information has been developed in here,

you will notice the order status is queued.  Move the mouse cursor over

the CONTINUE button and click with either mouse button.

First, TEAM ONE will check available inventory quantities for the product

ordered. If there is enough quantity available to fill the order, the

program will inform the user of this. TEAM ONE will then adjust the inventory

quantities on hand and in use, create shipping information, delete the

filled schedule card, update the customer order to a BUILT status and

set the total product and end production fields on the customer order.

If TEAM ONE does not detect enough inventory to totally fill the order,

it will allocate what inventory is available for allocation and reset

the schedule production quantity.

The program will now display the capacity schedule card.  The program

takes a sampling of all days throughout the month that the schedule is

going to <$&10[^]>start on and gives an average remaining capacity.  This

details the product name, the line ID, the present usage of the line,

the maximum production rate of the line, the remaining capacity, and the

desired capacity.

In this area you determine the capacity you wish to allocate to an individual

order.  This allows you to determine a finite capacity for a line.

You also have an area for the maximum desired capacity on the line.  Some

companies always want to have the maximum desired capacity at 100%, while

others never want the line to run at more than 90%.  Additionally, a line

may produce multiple products, and here you could set a maximum capacity

for the line for each product produced.  If you desired infinite scheduling,

you could set this number to a very large number, such as 10,000%. You

have the ability to adjust the capacity of the line.

When capacity information is entered in and the continue button is clicked,

TEAM ONE's master schedule system then activates and begins to determine

the production system and the production demands on inventory.

TEAM ONE checks each part and process in the production of a product and

compares that against inventory items.  TEAM ONE checks the quantities

that are needed, allocates those quantities, and determines if there is

enough inventory to build the number of products you desire.

If there is not sufficient inventory, a vendor request is then generated

automatically by the system and then sent back to the order entry system

for P.O. generation.

A Scheduled for Pick list is generated by the system for the warehousing

people to pick parts for a selected order.

The program will roll through the manufacturing calendar comparing the

capacity desired versus capacity presently in usage.  If the capacity

desired plus the capacity in usage does not exceed the maximum desired

capacity, then the program will add that day to the production schedule.

If the combined capacity does exceed the maximum desired capacity, TEAM

ONE will move on to the next day.

At this point TEAM ONE's master scheduler is assigning capacities to individual

days; it is also setting the master schedule card to active status.

TEAM ONE is setting the customer item cards that are desired to active

status.

However, vendor information for parts to be ordered has not been set to

active. That action must be done in the vendor information area of the

order entry system.

Additionally, the Master Scheduler is searching for sub-assemblies associated

with the product. If and when then assemblies are found, the system automatically

generates another schedule card for the assembly and associated "Customer

and Shipping Cards." The program assigns the Line ID to the customer area

for automatically generated sub-assembly orders.

Once calculations are performed, the program will display once again the

schedule card and the production ending date that was calculated by the

program.  This tells you what day the program has determined that production

will end on.

By pressing the CONTINUE button you save this schedule information and

you save all other database information that was created by the master

scheduler.  During this operation, the scheduler recalculates production

capacities and alters the product and production records to now match

the realities of production and not the estimates that were entered in

the Product and Calculate systems.

The Master Scheduler will continue to display orders matching either the

Queued Status or the Hold Status, depending on what area of calculations

you are in.

The purpose of the Hold Status is to allow you to have a secondary priority

level for customer orders that are able to be inserted into a schedule

when convenient to be run.

Although the user operations are simple, the Master Scheduler is the driving

force for most of the system's information.  It performs many tasks for

you that would normally have to be performed by hand.

To view information and to alter information, you may move into the options

menu, where you have the ability to work through the manufacturing calendar,

the schedule, or the parts that are being ordered.

CHANGE

You have the option to change information in the manufacturing calendar,

schedules or parts to order.

<$&15[^]>The operational menu is located at the top of the screen. This

menu holds a series of buttons that allow you to easily move through the

database or work with specific records in the database. It should be noted

that in the CHANGE areas, the user may not add or delete records. We have

determined that deletion procedures should only be accessed by the system

administrator. Addition functions can only be performed in the input areas

of the modules. You may edit information on records.

Under the Change Menu Bar, TEAM ONE details the current record that you

are viewing and the total records held in the database.

The FIRST button takes you to the first record in the database.

The LAST button takes you to the last record in the database.

The NEXT button takes you to the next record in the database.

The PREV(IOUS) button takes you to the preceding record in the database

and not the previously viewed record.

The FIND button allows you to search the database for any information

held in any or all fields. The more information you enter into this record

search, the more selective the search will be. However, you must enter

EXACT information.

The MORE button continues the search using the information you previously

entered.

The EDIT button allows you to edit a record. Once you have finished editing

the record, click on the CONTINUE button.

To leave the CHANGE area, click on the EXIT button. This will save any

changes you performed and return you to the TEAM ONE MAIN MENU.

CHAPTER 7WAREHOUSING SYSTEM

TEAM ONE provides the natural link between manufacturing and warehousing.

Thus, TEAM ONE provides a fully functioning, conventional warehousing

module.

The warehousing module in TEAM ONE allows for full warehousing control,

linking customer and vendor orders directly to warehousing orders. The

warehousing system knows where to pick parts from, for what customer they

are assigned to, what line-workcell they are to go to and updates the

inventory records accordingly.

The warehousing module tracks expected receipts and determines where they

should go and what customer they were linked to.

Enter the Warehousing Module by clicking on the WAREHOUSING SYSTEM button

under the INVENTORY MENU.

This brings up the WAREHOUSING MAIN MENU. You have the option of <$&1[^]>working

with orders to pick, orders to receive or to work in shipping.

PICK SYSTEM

Access the Pick Area under the WAREHOUSING MAIN MENU. Place the mouse

over the PICK button and click with either mouse button.

Pick Parts are created by the Master Scheduler and only in that module.

Remember, TEAM ONE is an order driven system and no pick procedures can

be <$&2[^]>created without a customer order first being created.

PICK ORDERS

The Pick Orders area allows you to activate all parts that are scheduled

to be picked for a selected order. The database holding the parts to pick

is created by the master scheduler and is called the SCHDPICK.CRD file.

Activate the Pick Orders Option from the Pick Menu.

You would now enter the EXACT control number (generated by <$&3[^]>the

Customer Order) and the quantity to pick.

By pressing the CONTINUE button, TEAM ONE searches the scheduled pick

file for all parts to pick that are associated with this order number

and sets these parts to the PICKMATL.CRD (pick material) file. This sets

the hold status on the pick to "Y"es. TEAM ONE also loads all generated

orders associated by the Master Scheduler. These orders begin with the

MS- prefix.

These parts to pick may be viewed by using the CHOICE button under the

Pick Activate menu option.

PICK INDIVIDUAL

If you do not desire to activate the entire pick for a full order, but

only wish to pick an individual part, you would use the PICK INDIVIDUAL

option under the PICK MENU.

You need only to enter the control number that holds the part to pick<$&4[v]>.

Click on the CONTINUE button.

TEAM ONE will now display all parts that are scheduled to be picked for

the control number that you entered previously.

To accept the part, click on the THIS CARD button, to continue searching

for the part to pick, click on the NEXT CARD button.<$&final09[v]>

Once you have located the part to pick, TEAM ONE displays the pick information

for the part. The Quantity field displays the total quantity of the selected

part that needs to be picked to fulfill the entire order. You may enter

any number to pick here.<$&final10[v]>

Press the CONTINUE button. This activates the inventory linking procedure.

Next you will see a related inventory pick material card that has been

created by the program. This details the present inventory quantity and

other information as it relates to the inventory record.

You now have the choice of QUEUEing the pick or QUEUEing and ENABLING

the pick. Queueing the pick puts the hold status to "Y"es. Queue and <$&5[^]>Enable

sets the hold status for the pick to "N"o.

PICK ACTIVATE

Pick Activate is run when you are actually going to go pick the part.

Click on the PICK ACTIVATE button in the PICK MENU. This brings up the

pick decision screen.

<$&6[^]>Enter the Control Number and Part Number of the part to Pick.

TEAM ONE searches for an EXACT match to this information and if found,

displays the Pick record of the associated part. Pressing the CONTINUE

button sets the hold status to "P"icked.

TEAM ONE now desires to know if you wish to print a PICK TICKET for this

part. By clicking on the YES button, TEAM ONE will automatically print

the preceding information. Clicking on NO will reset the file record to

P for picked.

You may also choose to pick from a list of parts to pick. To perform this

function, click on the CHOICE button on the Pick Activate Screen. This

displays all parts <$&7[^]>to pick, their status, location and associated

control number. This only displays these parts and not parts that have

been confirmed picked, either correct, long or short.

Place the cursor over the part you desire to pick and click on it. As

you click on the parts, the picked part will be loaded into the bottom

list. To end and exit, move to the end of the top list and CLICK ON EXIT.

Press the CONTINUE button. TEAM ONE now displays the Pick Verification

Screen. Press the CONTINUE button to shift the hold status to "P"icked.

TEAM ONE will query you as to whether you desire a pick ticket printed

for the parts you are picking.

CONFIRM PICK

Once a part has been picked, you must now confirm it. This is so that

you can track how many parts were picked, were they picked correctly and

updates are created for the inventory, schedule picks and pick material

files.

Confirm pick is very similar to Pick Activate, except that TEAM ONE now

determines whether the number actually picked was correct, long or short.

Enter the CONFIRM PICK area from the Pick Menu Screen.

You may enter either a part number to confirm or choose from all picked

parts <$&8[^]>through the CHOICE option.

Once the part picked is selected by either manual entry or by clicking

on a part in the part list, (the CHOICE pick list will only display parts

that have been picked or parts that have been picked long or short) you

now see the Pick Confirmation Screen.

If the pick was correct in quantity, click on the Pick Correct button.

TEAM ONE sets the status to "C"orrect.

If the pick was incorrect, click on the PICK INCORRECT button.

Now you enter the quantity that was actually picked. TEAM ONE will compare

this number to the number you were supposed to pick and set the status

to either "S" for short or "L" for long. If you accidentally clicked on

the wrong button and actually picked the correct number just click on

the CONTINUE button and TEAM ONE will set the status to "C"orrect.

You are asked by the program if you desire to print the Pick Confirmation

Ticket. You may elect to do this or not.

RECEIVE TO STORES

The RECEIVE TO STORES menu option is located on the WAREHOUSING MAIN MENU.

Click on the RECEIVE TO STORES button. This brings up the Receive Menu

Screen. Here you have the option of receiving material that was ordered

by the <$&10[^]>Master Scheduler and confirmed by the Vendor Order Entry

system or to receive material that was previously put into a hold status

by quality control and is now ready to be placed in inventory.

Click on the RECEIVE MATERIAL button. This brings a listing screen for

the expected receipts file. Click on the desired Control Number or Part

Number that you desire to receive to stores. TEAM ONE asks if you desire

to print a ticket for the receive. Next TEAM ONE displays the vendor order

for the part to receive. Click on CANCEL to stop the receive function

here.

TEAM ONE will now display the found part information, including the total

number of the selected part ordered through a selected customer order

number.

You must now enter the quantity of these parts you received.

Next you have the choice of sending these parts into a hold status, (by

click on HOLD FOR Q.C.) or to apply these parts to the On Hand area of

the inventory by clicking on CONFIRM STORE.

You are then returned to the TEAM ONE MAIN MENU.

RECEIVE FROM QUALITY CONTROL

This allows you to receive parts into your active inventory from a hold

status.

Click on the RECEIVE FROM QUALITY HOLD button. This brings up the Receive

<$&13[^]>From Q.C. screen.

Enter the part number and quantity to return from Q.C. to active inventory

and press the CONTINUE button.

This displays the appropriate inventory record. Press the CONTINUE button.

TEAM ONE automatically depletes the ON-HOLD quantity by the number entered

and increases the ON-HAND quantity by the same amount.

You are now returned to the TEAM ONE MAIN MENU.

SHIPPING

The SHIPPING area of the TEAM ONE WAREHOUSING SYSTEM allows you to control

your shipping information and to adjust shipping records.

Pressing the SHIPPING button from the Warehousing Main Menu displays a

list of orders currently waiting to be shipped.  To select an order for

shipping, place the mouse cursor over the order and click either mouse

button.

Press CONTINUE to proceed with the order shipment or CANCEL to return

to the TEAM ONE MAIN MENU.<$&final12[v]>

The next screen displays the information generated by the ORDER ENTRY

system such as pick quantity and PARTIAL SHIPMENT information.  If the

order was tagged with a PARTIAL SHIP of NO, then no partial pick will

be allowed.<$&final13[v]>

Press CONTINUE to proceed with the shipment or CANCEL to return to the

TEAM ONE MAIN MENu.<$&final14[v]>

The above screen verifies all final order shipping information such as

quantity, carrier, payment method, and shipping address.  Press CONTINUE

to ship customer order.

Press CANCEL to cancel shipment and return to the TEAM ONE MAIN MENU.

NOTE: When shipping a final product, the shipping system remove the final

product from the "IN USE" field on the INVENTORY database, just as BACKFLUSH

does for all other items. BACKFLUSH does not reset the ordered product,

only shipping does.

CHAPTER 8 BACKFLUSH MODULE

Backflush is what brings TEAM ONE to final operational step.  Backflush

gives you the ability to manually run the system in real time, updating

schedules, order entries, inventories, and orders in a real time mode.

Backflush is what tells TEAM ONE what products have been built and on

what line and can assign production to selected orders.

From the TEAM ONE MAIN MENU, place the mouse cursor over the BACKFLUSH

button and click either mouse button.  This brings up the BACKFLUSH SYSTEM<$&1[v]>

Begin by entering a product, a line ID, and a quantity produced, TEAM

ONE will check all schedules for that product and update all related information

and databases.

To update a product or sub-assembly, type the full name of the product

(and remember this information must be exact).

Under line ID, enter the exact line ID.

Quantity produced is the quantity produced since the last time you updated.

The date of update is automatically loaded by the system, and that is

the date of this update.

The operator ID is the logon ID of the operator.

Once your product, line ID, and quantity produced has been entered, place

the mouse cursor over the CONTINUE button and click with either mouse

button.

If you press CANCEL, you will return to the Main Menu.

This then brings up the product update screen, which is the product information

screen.  By pressing continue at this point, the system will continue

with the up<$&2[^]>dating procedures.

By pressing CANCEL, you return to the UPDATE AND ACTIVATION SYSTEM screen.

Place the mouse cursor over the CONTINUE button and click either mouse

button.  TEAM ONE goes thorough the entire schedule, locates the product

that you have entered for that line.

TEAM ONE now details the total number of that product you have in the

schedule, shows how many you have entered that you wish to produce, and

how <$&3[^]>many orders you have for this product.

Let's say that you have entered that you have built 100 products, and

the first order only needed 25, then it will deduct 25 from the first

order, notify the other modules of the system that this order has now

been completed and built (that includes the order entry system), and then

moves to the next schedule for the next product that matches the information

you entered and updates those schedules.

TEAM ONE continues until it has depleted its number of products that you

had entered that you produced.

Under manual allocation TEAM ONE displays each schedule that is tied to

that product.  Click on manual update screen.

This brings up the activation schedule card, shows the quantity desired

for that product, <$&5[^]>the order status is active.  By clicking on

THIS CARD, tells the system that you wish to accept the update of this

schedule.

By clicking on NEXT CARD will allow the system to search through the schedule

database and locate the next schedule card which matches the product and

line identification.  Click on the THIS CARD button.

This now displays the scheduled update.  TEAM ONE automatically loads

the quantity produced that you entered into this area. You can move into

the quantity produced area and/or you can move into the quantity to update

and enter that number or any number smaller than that.

By entering that number, then pressing CONTINUE, TEAM ONE then displays

all processes that it is updating.  This updates all inventory items,

the schedule itself, and the customer information under the order entry

system.

If you were to move back into the order entry system and use the view

customer orders, you would see that the system is tallying the number

produced and allocated to the customer that you selected.

Once a product order has been completed through BACKFLUSH, the system

will then tell the ORDER ENTRY SYSTEM that this product is completed and

ready for shipment to the customer, if all units ordered have been produced.

TEAM ONE will move the checkbox on the CUSTOMER order status to "built"

and enter the date of completion. Additionally, the BACKFLUSH SYSTEM is

updating the Shipping Pick List in the WAREHOUSING SYSTEM, ship module_

If you looked at your master schedule under the options menu of master

schedule for this item, you would notice that the quantity desired is

decreasing by the number that you placed into the system that you built,

until this reaches 0 and the schedule card is then deleted.

Remember that customer order cards are never eliminated by the system.

Once activation has been completed, the system returns you to the TEAM

ONE MAIN MENU.

CHAPTER 9REPORT WRITER

During the development of TEAM ONE we came to the understanding that nobody

seemed to be happy with the style and content of almost any pre-defined

report that existed in any competitive program. As such, we developed

a very comprehensive and yet easy to use custom report generator.

In the Report Writer we have developed a system that allows the user to

quickly and easily design a report structure and to keep this structure.

The Report Writer is loosely based on desk-top publishing principles,

but is very easy to operate and learn.

We have provided a large number of reports structures in the basic TEAM

ONE package. You may use these just as they are, adjust them to fit your

needs or create new ones.

To access the REPORT WRITER, click on the OPTIONS button on the TEAM ONE

MAIN MENU. Then click on the REPORTS button. This brings up the REPORTS

MAIN MENU <$&1[^]>screen.

Although we have created a large number of reports for you, we will first

go through the creation process for a report.

From the REPORTS MAIN MENU, click on the CREATE REPORT button.

This brings up the list of all databases held in TEAM ONE that provide

informa<$&2[^]>tion needed or desired.

Click on the desired database to work with. You will see the database

name copy to the Chosen Database area. If you are satisfied with this

database, click on the CONTINUE button. If you desire another database,

click on the Chosen Database to erase it and then click on another database

to load. _YOU ARE LIMITED TO ONE DATABASE PER REPORT STYLE.

Now the REPORT WRITER displays all fields that exist on the chosen database

record. We have strived to name the fields exactly what their functions

relate to. You may choose up to 14 fields for any individual report structure

(Maximum of 160 columns wide).

To choose the fields for the report, simply place the mouse over the field

name in the DATABASE FIELDS list and click with either mouse button. You

may <$&3[^]>use the Page Up/Page Down keys to scroll through the fields

to choose.

The order of choice for the fields is set by the order they were chosen

in. You are limited to a maximum number of 14 fields on an individual

report. Additionally, you are limited to a maximum number of 160 columns.

You may erase a field from the Chosen List by placing the mouse over the

field to erase and clicking on it.

When you have loaded all the fields you desire for the report, press the

CONTINUE BUTTON.

This brings up the PRINT CONTROL screen. This screen displays the fields

as <$&4[^]>they were chosen.

To the right of the field name is the LEN(gth) column. This column details

the actual length of the field. You may adjust this number.

Next is the SPCS (SPaCeS) column. This details the number of spaces between

field columns in the report. This number may also be adjusted by the user.

The third column is the CSTRT (Column STaRT) column. This displays the

column number that the field will start on. This number is derived from

the addition of the Column Start, Length and Spaces of the field above

in this screen. This number is calculated by the program and is adjusted

by the program.

The final column is the Column Heading for the field Column. You may enter

any name or title you desire here. This field will be printed exactly

as you enter it.

The 15th field is for program usage. This is for row totals created in

the Advanced functions.

At the bottom of the Print Control Screen is the HEADING TEXT field. This

is what will be printed on the report as the Heading of the report.

Under that field is the REPORT NAME. This is the name of the report as

it will appear in a list of reports for printing or changing.

The first time the Print Control Screen appears during a report creation,

you may press either the CONTINUE, FINISHED or ADVANCED buttons to bring

up the ADVANCED PRINT CONTROLS Screen. After you have entered the advanced

printing functions, you would press the FINISHED button to save the structure

and return to the REPORTS MENU.

The ADVANCED PRINT CONTROL Screen allows you to work with arith<$&5[^]>matic

functions for the numeric based fields. It also allows you to pre-set

certain print controls.

At the top of the ADVANCED PRINT CONTROL Screen you will see the Report

Name, as entered on the Print Control Screen Report Name Field.

Under that you have the ability to set the Top, Bottom, Left and Right

Margins for printing. You may also preset the page length and width.

To the right of these fields you have the option of turning on or off

the Page numbering, time and date functions for the report.

Under the MATH title, the REPORT WRITER displays all the fields that may

use math functions. Think of this area as a very small and basic spreadsheet.

You have control over the four basic mathematic operations (Addition,

Subtraction, Multiplication and Division).

To activate a Row Formula, click on the first field you desire to begin

the calculation on. Report Writer will copy the field name to the first

available Row Formula Box. Then click on the math function desired. Now

click on the field name to perform this function on. The report writer

math function operates in a top down hierarchy.

You will notice that as soon as you begin entering fields to calculate

on, the word TOTAL appears in field 15. This tells you that TEAM ONE is

going to perform extended calculations on some fields.

You are limited to 5 fields to calculate on. This is because that is the

maximum number of calculations that realistically would be performed on

any single database.

If you have made a mistake during formula creation, simply click on the

CLR (Clear) button and this erases the formula fields.

You may elect to sum any or all columns by simply clicking on the SUM

COL(umn) check box next to the field name. This includes the 15th field.

Once you have completed the Advanced Print Control Screen information,

click on the CONTINUE button, this will return you to the PRINT CONTROL

SCREEN.

If you change any column information you will notice a recalculation of

this information.

If all information is appropriate, you may now click on the FINISHED button

to save both the print control records.

If you have failed to name the report, the system will place a default

name on the report. This name will be the word SYSTEM and a numeric value

equal to the number of the report. This name may be altered in the Report

Change area.

PRINT REPORTS

Once you have created a report structure or desire to print an existing

structure, you would use the PRINT REPORTS area to print the report.

Click on the PRINT REPORTS button on the REPORTS MAIN MENU. This brings

up a list of all report structures that have been created in the Report

Writer. The list of reports will hold 500 report names. You may then click

on the desired report. You will see the report name appear in the Active

Report Name field. If this is acceptable, click on the CONTINUE button.

If you desire another report, click on the chosen report name and erase

it from the Active Report field and rechoose a new report to print.

When the report is chosen and CONTINUE is pressed, you will have the option

of changing the format of the report for this print session only.  To

change the format select ALTER REPORT button.  To print the report using

the same format, select the KEEP DESIGN button.<$&final07[v]>

The REPORT WRITER now asks if you desire to print the report to a printer,

to a file or to the screen(with file creations). This allows you to redirect

printing to a standard ASCII format file <$&6[^]>for printing at a later

date, or for usage by other programs.

If you click on PRINTER, the report is immediately sent to the printer

on LPT1. You may use DOS redirect commands to send printer output to other

ports or devices.

If you choose FILE, the REPORT WRITER will query you for the name of the

file to create and redirect printing to.

If you choose SCREEN, the REPORT WRITER will first create the ASCII file

as it does in FILE, but then it displays the file directly to the screen.

Whether you are printing to a printer or to a file, the program allows

you to stop printing at any stage by simply pressing the ESCape key on

the keyboard.

The REPORT WRITER displays the total records to print and the present

record being printed.

When printing is complete, you will be returned to the REPORT WRITER MAIN

MENU.

CHANGE REPORT

The CHANGE REPORT menu allows you to alter the structure of a report.

Click on the CHANGE REPORT button, this brings up a list of reports to

<$&7[^]>choose from. As in the other areas, click on the desired report

to alter.

NOTE: During the change process, you may not add or delete fields. This

would create a totally new report, not an altered one. You may only change

the operational structure of the report, not the basic report itself.

The REPORT WRITER displays the PRINT CONTROL Screen and then the ADVANCED

PRINT CONTROL Screen. This allows you to perform the changes and exit

quickly. (For details, please see the CREATE REPORT section of this chapter.)

VIEW TEXT FILE

The REPORT WRITER VIEW TEXT feature allows you to view the contents of

any text file ending with the DOS extension TXT.

Click on the VIEW TEXT FILE button from the REPORTS MAIN MENU.

You are then provided a load box to choose the file to view. Remember,

only <$&9[^]>files ending with the TXT extension are displayed here.

NOTE: Files of 100k or more are not currently supported within this window.

If your file is too large, use a text editor to view the report.

Click on the desired file to view. The file is then loaded into the Viewing

area. You may scroll up and down or left and right.

<$&10[^]>When you are finished viewing, click on the CONTINUE button.

This returns you to the TEAM ONE MAIN MENU.

CHAPTER 10TOOLS

The TEAM ONE TOOLS option provides for system administrator functions

and communications.

Access the TOOLS MENU by clicking on the OPTIONS MENU and then click on

the TOOLS button.<$&1[v]>

You may elect to work with either DATABASE TOOLS, COMMUNICATIONS, REINDEXING,

ALTERING INVENTORY or PRINTING the On-Line HELP manual.

COMMUNICATIONS

TEAM ONE provides a fully functioning communications system for data transmission

over standard phone lines using modems.

The communications program allows for basic file transfer capabilities

and will display only character based screens. Additionally, file transfer

protocol is limited to the XMODEM/XMODEM-CRC method.

Future optional modules will allow the communications system to automatically

access vendors and customers and provide for other file transfer methods.

Access the COMMUNICATIONS SYSTEM by clicking on the COMMUNICATIONS button

on the TOOLS MENU.

The Communications system allows for keyboard entry of standard modem

com<$&2[^]>mands, (such as all Hayes compatible commands) or for operations

from a Pop-Up menu system.

To activate the menu, press the CTRL-A keys at the same time<$&3[v]>.

You must first set up the communications standards for your system.

Click on the SERIAL CONTROL PANEL button on the Communications Main Menu.<$&4[v]>

WARNING!! AT THIS POINT YOU MUST KNOW WHICH COMMUNICATIONS PORT YOUR MOUSE

IS ATTACHED TO. LOADING THE COMMUNICATIONS PROGRAM TO THE SAME PORT AS

THE MOUSE WILL CAUSE TEAM ONE TO FREEZE AND WILL DEMAND A FULL SYSTEM

RESET. TEAM ONE COMMUNICATIONS DEFAULTS TO COMM2. IF THIS IS THE PORT

THE MOUSE IS PHYSICALLY ACCESSING, CHANGE THE DEFAULT COMM PORT NOW.

To adjust the default settings for communications, click on the appropriate

information area to change, then click on the CLOSE PANEL box.

To dial a number, you may use the DIAL PHONE button. Enter the phone number

to dial and click on whether your phone system is a Tone operated system

or <$&5[^]>the older rotary/pulse system. Dialing with the menu option

will take several seconds to accomplish.

To Transmit a file. Click on the Transmit a File button. Click on the

desired file <$&6[^]>to transmit, TEAM ONE will detail what is being sent

and when transmission is over.

To Receive a File. Click on the Receive a File button. Enter the name

of the file <$&7[^]>to receive and press CONTINUE. You may redirect the

file to any drive or path by using the appropriately named buttons.

To Leave the communications system, click on DISCONTINUE TERMINAL. To

return to the main communications screen, click on the CONTINUE BUTTON.

Other options available for the communicator are to adjust local echo,

colors and dialing directory.

DATABASE TOOLS/SYSTEM ADMINISTRATOR FUNCTION

NOTE TO THE SYSTEM ADMINISTRATOR: REMOVE THIS SUB-CHAPTER FROM THE USER

MANUAL BEFORE ALLOWING USER ACCESS TO THE SYSTEM!!

The DATABASE TOOLS/SYSTEM ADMINISTRATOR functions area is designed for

use only by the system administrator. Operations in this area are similar

to the CHANGE procedures throughout the program, but provide a much higher

capability, such as deletion control.

Additionally, the Database Tools area allow for full access of all databases

and revision of any database area.

With this in mind, this area should be strictly controlled through the

TEAM ONE security system and access should only be granted to selected

individuals.

Information on this area may be obtained through the TEAM ONE REFERENCE

MANUAL. Due to the sensitivity of this area, we are not providing USER

instructions.

It is advisable to maintain a tight control over the REFERENCE MANUAL

due to this area.

REINDEX SYSTEM

Occasionally, the .BTR (Btrieve Index Control) file that controls the

indexing for the INVENTORY.CRD file may be corrupted due to disk errors

or fragmented files that occur on storage devices during everyday use.

If such an error occurs, you can rebuild the indices for the Inventory

file using this utility.  It is very simple to use and can save a lot

of time rebuilding your database for you.

To Rebuild the Index System you will need 3 times the amount of storage

space on your hard drive as your present INVENT.CRD file takes up. To

Check the size of the INVENT.CRD file you may use DOS commands or the

SHOW FILE INFO Menu Option under Database Tools.

Press the REINDEX SYSTEM button from the Tools menu, the REINDEX SYSTEM

window appears.  To rebuild the Inventory index (.BTR), place the mouse

cursor over the INVENTORY button and click either mouse button. The operation

is automatic from this point forward, but may take a fairly long time

to perform. DO NOT STOP THIS OPERATION ONCE IT BEGINS!!!

A confirmation screen will appear that will allow you to either continue

with the rebuilding process or will cancel you out of the current function.

<$&tools2[v]>

Pressing the CONTINUE button rebuilds the database, while pressing CANCEL

returns you to the Tools Main Menu.

ALTER INVENTORY

This allows you to reset your inventory quantities to any desired level

you deem necessary.  This includes minimums and control quantities on

the INVENTORY records.  This function is useful if you are running a test

database on a new or design-stage product.

Select this function by placing the mouse cursor over the ALTER INVENTORY

button in the TOOLS MENU and click either mouse button.<$&tools04[v]>

You may reset any of the quantity fields on the inventory records. To

change a field quantity on a GLOBAL basis, enter the amount to reset by

in the field. To leave a field with its existing values, click on the

corresponding box.  REMEMBER:  This will globally reset all information

in these fields on every Inventory record.

Press CONTINUE to reset the inventory fields or press CANCEL to return

to the TEAM ONE MAIN MENU.

If you press CONTINUE, the system will rapidly display each record with

the reset information in the "inventory" screen.<$&tools03[v]>

PRINT HELP

The PRINT HELP system allows you to print the On-Line help manual, if

you desire. The help manual is a condensed version of the TEAM ONE REFERENCE

MANUAL and will take about 80 pages to print.

To Print the Help manual, click on the PRINT HELP button.

The Screen will change look and a loading box will provide you with the

TEAMONE.HLP file to choose to print. Click on this file and printing will

commence. No automatic redirection of the printing is available for printing

of the help manual to an ASCII file.

CHAPTER 11EXIT PROGRAM

Access the EXIT PROGRAM function by clicking on the OPTIONS button from

the TEAM ONE MAIN MENU<$&1[v]>. Next Click on EXIT PROGRAM button.

You are now returned to DOS.