------------------------------------------------------------------

W H A T 'S   N E W  ? ? ?

------------------------------------------------------------------

------------------------------------------------------------------

Q: why do functions "Water and Steam" and "Saturated Water and Steam"

give different results for the critical point ?

A: "Water and Steam" uses the "IFC 1967 Formulation for Industrial Use"

whereas "Saturated Water and Steam" uses the "IAPS Supplementary

Release on Saturation Porperties of Ordinary Water Substance",

September 1986. Both formulations use different values for properties

at the triple point and at the critical point.

IFC 1967 Formulation :

values at triple point  :   p = 0.006112 [bar]

T = 0.01 [C]

values at critical point :  p = 221.2 [bar]

T = 374.15 [C]

v = 0.00317 [m^^3/kg]

rho = 315.458 [kg/m^^3]

h = 2107.4 [kJ/kg]

s = 4.4429 [kJ/kg/K]

IAPS 1986 Formulation :

values at triple point :    p = 0.00611659 [bar]

T = 0.01 [C]

values at critical point :  p = 220.64 [bar]

T = 373.99 [C]

v = 0.0031056 [m^^3/kg]

rho = 322 [kg/m^^3]

h = 2086 [kJ/kg]

s = 4.41 [kJ/kg/K]

new in Version 1.01 :   help information now explicitly points out

the differences between both formulations.

------------------------------------------------------------------

new in Version 1.02 :   at 374.15 [C] for p > 221.2 [bar] wrong

results were returned.

Fixed in Version 1.02

at 350 [C] in the saturation region sometimes

a floating point error occured.

Fixed in Version 1.02

------------------------------------------------------------------

new in Version 1.03 :   computation of polynomials is accelerated by

a factor of about 2 by replacing power

functions with multiplications and by using

precomputed results

an error in the computation of the Gibbs

enthalpy in the saturation region < 350 [C]

is now fixed.

------------------------------------------------------------------

new in Version 1.04 :   fixed a floating point error which occured at

critical values of temperature or pressure for

combinations of input properties other than

(p,T) and (v,T).

------------------------------------------------------------------

new in Version 1.05 :   fixed an error in computation of differentials

at the boundary between subregion 5 and 6

(saturation at 350 [C]) where some

differentials showed a significant jump.

------------------------------------------------------------------

new in Version 1.06 :   WTHERM and WTHERMPD didn't run properly

in a Novell network environment. This error

should be fixed with Version 1.06. If this

error still occurs with Version 1.06 or later

versions, please enter the following command

after you have installed WTHERM or WTHERMPD

and before anybody tries to start WTHERM or

WTHERMPD :

flag *.* s

This command should be executed only for the

files of the directory where you installed

WTHERM or WTHERMPD.

------------------------------------------------------------------

Q: why does function f (h'') in menue "Saturated Water and Steam" not

return values for the other properties when h'' > 2510 [kJ/kg] ?

A: for values of h'' > 2510 [kJ/kg] there doesn't exist a unique

solution for the pressure p. One solution exists at around 0.01 [bar]

and another solution at around 180 [bar]. Because of this ambiguity

of the solution no values are returned by the steam table functions.

------------------------------------------------------------------

new in Version 1.07 :   WTHERM and WTHERMPD return an error message

when the user input is outside the domain of

the IFC 1967 steamtable or when a solution

is not unique.

------------------------------------------------------------------

new in Version 1.08 :   iteration of specific volume in subregion 3

at boundary to subregion 2 improved.

------------------------------------------------------------------

new in Version 1.09 :   corrected an error in computation of

d2g/dtdp in subregion 1.

corrected an error in computation of

g in region 2.

corrected an error in computation of

d2f/dt2 in region 3.

-------------------------------------------------------------------

new in Version 1.10 :   for certain values of (v,T) in subregion 4

an error was signalled, which was not

justified.

changed postal code in welcome screen.