#

*************************************************************

*                                                           *

*                 P A C K - E T - T E R M                   *

*                                                           *

*                     Version 3.0 ST                        *

*                                                           *

*                     Copyright 1986                        *

*                                                           *

*                By Charles W. Harrington                   *

*                                                           *

*************************************************************

PACK-ET-TERM USER GUIDE

FOR THE ATARI ST

July 15, 1986

COPYRIGHT

Copyright (C) 1986 Charles  W. Harrington, WA4GPF. Rights  to

sell  are reserved.  This program  may  be  freely duplicated  and

distributed,  but  not sold without  the prior written consent  of

the author, WA4GPF,  Charles  W.  Harrington,  5634  Lesser Drive,

Orlando, Florida, 32818.

DISCLAIMER

Charles W. Harrington,  the author, makes  no warranty  as to

to suitability  of  this software  for any purpose.  Further,  the

author  reserves  the right  to revise this material  at any time,

without incurring any obligation to former users. The user assumes

total responsibility  for the use  of the software, and agrees not

to  hold  the author responsible  for any loss connected with it's

use.

SOFTWARE LICENSE

The author grants to the public, a limited license to use and

reproduce this product, providing the terms of the license and the

copyright  are complied with. The  user agrees  that  he will  not

attempt disassembly  of this software, and that  he will not alter

it in any manner, without the prior written consent of the author.

This license may  be  canceled  or  modified  at any time,  at the

discretion of the author.

CONTENTS

1    INTRODUCTION

1.1  Welcome to PACK-ET-TERM

1.2  Description of features

1.3  System Requirements

2.0  Getting PACK-ET-TERM started

2.1  Connecting the ST to the Tnc

2.2  Configuring the ST serial ports

2.3  Configuring the tnc

3.0  Getting to know PACK-ET-TERM

3.1  Help Screen

3.2  Terminal Mode Screen Display

3.3  Transmit Screen Operation

3.4  Receive Screen Operation

4.0  PACK-ET-TERM's Special Features

4.1  Connect State Display

4.2  <F1> - Load CQ Buffer

4.3  <Shift><F1> - Display CQ Buffer

4.4  <F2> - Auto CQ

4.5  <Shift><F2> - Send CQ Buffer

4.6  <F3> - Clear Transmit Buffer

4.7  <Shift><F3> - Clear Receive Buffer

4.8  <F4> - Load Transmit Buffer

4.9  <Shift><F4> - Load Receive Buffer

4.10 <F5> - Save Transmit Buffer

4.11 <Shift><F5> - Save Receive Buffer

4.12 <F6> - Display Transmit Buffer

4.13 <Shift><F6> - Display Receive Buffer

4.14 <F7> - Toggle the Transmit Buffer

4.15 <Shift><F7> - Toggle the Receive Buffer

4.16 <F8> - Send Transmit Buffer

4.17 <SHift><F8> - Send Receive Buffer

4.18 <F9> - Toggle Connect Message

4.19 <Shift><F9> - Toggle Connect Receive Buffer

4.20 <F10> - Load Connect Buffer

4.21 <Shift><F10> - Display Connect Buffer

4.22 <Alternate><c> - Toggle Connect Printer

4.23 <Alternate><d> - Display Bytes in Receive Buffer

4.24 <Alternate><i> - Display Information

4.25 <Alternate><r> - Toggle Receive Printer

4.26 <Alternate><s> - Time Stamps

4.27 <Alternate><u> - Display bytes in Transmit Buffer

5.0  PACK-ET-TERM 3.0 Operating Tips

5.1  Making a Connect Message

5.2  Loading and Use of Connect Message

5.3  Using Save to Receive Buffer if Connected feature

5.4  Using Connect Printer feature

5.5  Using Time Stamps

5.6  Downloading from a BBS

5.7  Uploading to a BBS

5.8  Making a Text File

5.9  Using a Configuration File

6.0  Other Uses for Text Files

6.1  Rig Information

6.2  Routing File

6.3  CQ Messages

1    INTRODUCTION

1.1  WELCOME to PACK-ET-TERM version 3.0

Welcome  to PACK-ET-TERM version 3.0 written by WA4GPF, Chuck

Harrington.  Because  the author believes  in Packet Radio and the

Atari ST, you have received  at no cost,  a piece of software that

the author believes is of commercial quality.  Please feel free to

give your friends  a copy  of PACK-ET-TERM 3.0,  or upload  it  to

favorite telephone BBS system.

1.2  PACK-ET-TERM version 3.0 features

PACK-ET-TERM has  a  combination of features that you may not

have seen  in a  packet radio terminal before.  It takes advantage

of some of the Atari ST's best features to give the average user a

more  enjoyable packeting  experience.  This  program  features  a

triple  split  screen display, with transmit  and  receive screens

divided, and connect status and time displayed.  The status of the

various  buffers  and  program features  are  also  displayed,  to

remind the user of what the software is currently doing.

PACK-ET-TERM  3.0 supports  your  printer with  a  "print  if

connected" feature; this is a great way to save only your connects

to the printer.  It  is especially useful  as  a mailbox, allowing

a station  to  connect and leave you  a message. You may also save

connections to  a buffer, for the same purpose.  A connect message

is  also  provided, which  may  be  used  to prompt the connecting

station  to leave a message and disconnect, or may be used to send

a greeting automatically when someone connects.

PACK-ET-TERM  3.0 also supports text file transfers, and uses

several buffers to make packeting easy for you. The receive buffer

is 196K bytes  in  size, adequate for "all day" receive buffering,

or  the largest file transfers.  A  CQ buffer holds the text which

is automatically sent, at  the intervals you specify. The Connect

buffer, holds  the messages you want sent upon connection, and the

transmit buffer allows you to enter from the keyboard, up to 8K of

of text, to send or save to disk.

PACK-ET-TERM 3.0 also  has  a handy time stamp feature, which

supports the screen display, receive buffer, and the printer.  The

CONNECTED  and DISCONNECTED times, are also printed  in the status

area of the screen.

Best of all, PACK-ET-TERM 3.0  is designed  to work with most

TNCs, and provide all  or most  of the features discribed  in this

document, with no special interfaces or cables.

The author hopes you enjoy packeting with your  Atari St, and

that you enjoy the features of PACK-ET-TERM.

1.3  System Requirements

PACK-ET-TERM requires an Atari 520 or 1040 ST equiped with at

least one disk drive and  a Black & White or Color monitor.  Since

the software  is  written for the MEDIUM RESOLUTION mode  (meaning

an 80 charactor line!), the use of  a TV set will not be possible.

In addition, the program will not run on the early 520 STs without

TOS  in rom, unless they have  the one megabyte upgrade.  If there

is any interest, the author will make available  a version  of the

the software with smaller buffers, making  it compatable with  the

early TOS on disk 520 ST units.

PACK-ET-TERM  is designed  to work with your TNC. The program

was developed with  a TAPR type TNC-1.  The software provides some

features like the "Connect Message", which may not  be required in

some newer TNC units such  as  the TNC-2 types, which already have

similiar features implemented in the TNC itself. The software will

work with all TNCs, but the features utilized may vary.

2   GETTING PACK-ET-TERM STARTED

2.1 Connecting the ST to the TNC

It  is recommended that the RS-232 cable be prepared with the

the following pins utilized on the DB-25 connectors:

Pin 1     -    Protective ground

Pin 2     -    Transmitted data

Pin 3     -    Received data

Pin 4     -    Request to send

Pin 5     -    Clear to send

Pin 7     -    Signal ground

Pin 8     -    Data carrier detect

Pin 20    -    Data Terminal Ready

* You may use the mass terminated type DB25 connectors and connect

all pins if you wish.

2.2 STEP TWO - CONFIGURING THE SERIAL PORTS

(A) Establishing communication with the TNC

The first thing that must done, is to configure the ST's VT-52

terminal emulator; this is done BEFORE you enter PACK-ET-TERM.  We

will establish communication between  the VT-52 desk accessory and

the tnc, and then save our preferences for future use.

Select the VT-52 desk accessory from the DESK accessory menu;

this puts you  in  the VT-52 terminal mode. Turn  on your TNC; and

check for the sign on message:

EXAMPLE:

TAPR packet radio

ram length is 2000

cmd:

* Your sign  on message will vary, depending  on your type of tnc.

If you  do not get the sign  on  message, then most likey the

serial port  of the ST  is not configured the same way as the port

of your tnc.  To reconfigure the  ST  serial port, simply hit your

help   key  from   the  VT-52  terminal   mode,  or  select  RS232

configuration, from the DESK accessory menu. Now that you have the

RS232 PORT CONFIGURATION menu,  you may easily alter the  STs port

settings as necessary to establish communication with the TNC. You

will  find  additional information  on this procedure  in your TNC

and ST owners manuals.

(B) SELECTION the FINAL BAUD RATE

PACK-ET-TERM supports  300, 1200, 4800, 9600 baud rates.  You

should select a baud rate that is comfortable for you. AT 300 baud

you may get tired  of watching the letters come  to the screen one

at a time. At 9600 baud, lines may be pushed off the screen before

you have  a chance to read them! Either 1200 or 4800,  is probably

the best baud  rate  for most operators; experiment  with this and

determine which baud rate is best for you.

(C) SELECTION of PARITY

Parity must be set to the same type as your tnc.

(D) DUPLEX

Duplex must be set to OFF for split screen operation.

(E) Bits/Char

Either 7 or 8 bits may be selected, depending on your current

tnc "AWlen" (word length) setting.

(F) STRIP BIT

Strip bit should be set to off.

(G) FLOW CONTROL - Xon/Xoff

Xon/Xoff should be set to the ON position.

(H) FLOW CONTROL - Rts/Cts

Rts/Cts must  be set to on, or to off, depending on the type

of tnc which  is being used.  Improper setting of this parameter,

will prevent the tnc from sending any charactors to your ST!

(G) SAVE ST computers RS-232 settings

Once your RS-232 settings  are established,  you should save

them  to  your PACK-ET-TERM disk by selecting "Save DESKTOP" from

the desktop menu.  Your  custom settings  will  be saved  in  the

DESKTOP.INF file and loaded automatically at bootup.

2.3 CONFIGURING YOUR TNC FOR PACK-ET-TERM

For proper operation  of PACK-ET-TERM,  it is important that

the tnc  is properly configured.  The parameters  of interest are

listed below.

AUtolf - Auto linefeed

AUTOLF should be set to ON; your ST requires a linefeed from

the tnc at the end of each line.

BKondel - Delete on

BKONDEL should  be set  to ON if you wish to use <Backspace>

to delete characters and off if you wish to use the <Delete> key.

CANline - Cancel line input charactor

Canline should be set to $18,  or Control X for proper Clr/

home operation. The Clr/Home key sends a Control X to the tnc.

COMmand - Enter Command mode

COMmand usually has  a default setting of $03, or <CTRL-C>.

It is recommended that you change this  to $1b,  or <Escape> for

use with the ST. Use of <CTRL-C> on the ST will sometimes result

in  the unexpected exit from PACK-ET-TERM because  of quirks  in

the ST's TOS operating system.

Echo ON/OFF

Echo must normally  be set  to OFF, for proper split screen

operation.  If Echo  is ON,  the characters you type will appear

both in the transmit screen and the receive screen! You may wish

to turn Echo on at times, to save  a complete record of your QSO

to the Receive buffer  or to the printer, but normally, leave it

off.

Flow ON/OFF

Flow ON will halt output  to the screen any time you are in

the process  of typing, except when the transmit buffer is open;

this feature is helpful to those of us that are a little slow at

typing.  When FLOW is OFF, lines of text may move off the screen

and drop into the bit bucket,  before you have  a chance to read

them. The author normally runs this parameter in the ON setting.

SEndpac - Select send packet character

Sendpac should be set  to the default value of $0d or <cr>.

Screenl - Screen length

Screenl should  be set  to  0,  for proper PACK-ET-TERM 3.0

printing to the receive screen. Since PACK-ET-TERM already keeps

track  of the length of your screen lines, any other setting may

result  in unwanted linefeeds!   Lines will either be broken, or

unwanted blank lines may be added to the receive screen.

Xflow ON/OFF - Flow control

Xflow should  be ON,  for proper operation  of PACK-ET-TERM.

In addition, Xoff and Xon, should be at their default settings of

$13 and $11 respectively.

**  A tnc configuration file may  be  designed and save  to  disk

for later use;  this feature  is  explained  in the file transfer

portion of the documentation.

3.0 GETTING TO KNOW PACK-ET-TERM

3.1 STARTING PACK-ET-TERM

After communication has  been established  and  the  tnc properly

configured for PACK-ET-TERM,  you  are finally ready  to give the

program  a trial run.  Make sure  the time and date have been set

with the CONTROL PANEL before running PACK-ET-TERM.  Double click

PACKET.TOS to start PACK-ET-TERM.

3.2 <HELP> SCREEN - COMMAND MENU

The first screen you see after program execution, is PACK-ET

-TERM's <HELP> menu. Look the menu over before hitting any key to

enter into  the terminal mode.  You  may see this help screen  by

touching  the <HELP>  key;  touching  any  key when  the menu  is

displayed, will return you to terminal mode.

3.3 TERMINAL MODE SCREEN DISPLAY

In terminal mode,  you will  see PACK-ET-TERM's triple split

screen.  The top line  of the screen is used for "connect status"

and current time display. The center portion of the screen is the

receive area,  where received packets  and messages from your tnc

are   displayed.  The  lower portion  of the screen,  or transmit

screeen,  displays   the  characters  that   are input  from  the

keyboard.  Notice  the shaded  line which  separates  the receive

screen from the transmit screen;  this is known   as the "command

line".  PACK-ET-TERM will display errors  and various prompts  in

the center of this command line. Also located on the command line

are  six status slots, three either  side  of center; these slots

indicate  the status  of  the buffers, and some  of the program's

special functions.   These  status messages  are  printed  on the

command line in unreversed video when active, forming slots.  All

slots are closed when until opened by the user.

3.4 UNBUFFERED KEYBOARD ENTRY - TRANSMIT SCREEN

The cursor, which  is an underline charactor,  is located in

the transmit screen  area.  When  the transmit buffer  is closed,

ascii charactors are sent  to the tnc one at  a  time as they are

typed from the keyboard.  Holding  a key down, will result  in an

automatic repeat of the charactor.  Also, the "key click" will be

heard, assuming that  it has not been turned off  at  the control

panel,  or that the volume  is not turned down low. Typing errors

may  be corrected with the <DEL> key, assuming  a carriage return

has not been hit yet;  once the return key is hit, no changes are

possible,  and  the  tnc will act upon what you have entered.  If

charactors entered from  the  keyboard, appear  on  the  recieive

screen as they are typed, either the ECHO parameter in the tnc is

turned   on,  or  the ST's RS-232  is configured  for FULL DUPLEX

rather than HALF; echo  to  the receive screen  is  not desirable

during split screen operation. <CLR/HOME> will clear the transmit

screen and send  the cursor home, and send a Control-x to the tnc

to cancel any characters typed since the last <Return>.

3.5 RECEIVE SCREEN OPERATION

All ascii charactors sent by your tnc to the ST over the RS-

232 interface, will be displayed in the center or receive portion

of the screen.  You may pause the printing  of new charactors  to

the receive screen by, using the <CTRL><s> key sequence; printing

is restarted  using  the <CTRL><q> sequence.  You   may pause the

printing   of  new  characters   to  the  receive  screen  during

unbuffered keyboard entry,  by setting the FLOW parameter in your

tnc to ON.  It should also  be noted that the speed at which text

is printed  to the receive screen,  may  be altered  for operator

comfort by adjusting the RS-232 baud rate in the ST and tnc. PACK

-ET-TERM will function  at 9600 baud,  but some people may not be

able to read the text before it scrolls off the screen!  Also, if

AUtolf  is not set  to ON in the tnc,  lines will print on top of

each other in the receive screen.

4.0 PACK-ET-TERM's SPECIAL FEATURES

4.1 CONNECT STATE

The CONNECT  STATE display feature  is  one  of  the  unique

features of PACK-ET-TERM. When you become connected to a station,

the connect message,  along with the time will be printed  in the

upper left hand corner of the screen.  In addition, the ST's bell

will also ring  to alert  you  to the fact that  a connection has

just been made. A typical connect message on a tnc-1 would be:

*** Connected to WA4GPF at 18:04

On the tnc-2 types, the path will also be displayed:

*** Connected to WA4GPF via ORL at 18:04

Either *** Connected or *** Disconnected messages will be printed

by PACK-ET-TERM 3.0  to  the "Connect state" area  of the screen,

all the way  to the messages line feed!  please note that  in the

case  of  multiple connects,  the  last connect  or  disconnected

message received,  will  be the one printed  in the Connect state

area.  Support  for multiple connects  is under consideration  in

future versions  of  this software , but  is  not implemented  in

version 3.0.

4.2 <F1> - LOAD CQ BUFFER

The CQ buffer may be loaded from disk only.  CQ messages are

typed into  the transmit buffer and then saved  to disk.  Hitting

the <F1> key will bring the following prompt at the center of the

command line:

Load CQ Filename?

You must now enter the filename of your CQ message. You may abort

from entering  a filename  by hitting the <Esc> key.  Please note

that  the CQ buffer will not hold messages longer than 160 bytes;

very long  CQ messages sent automatically would cause unnecessary

QRM! Should you attempt to load a longer file into the CQ buffer,

you will receive  a "File  >  CQ buffer" message.  If the file is

successfully loaded,  you will receive "n bytes in CQ buffer"  at

the center of the command line.

4.3 <Shift><F1> - Display CQ buffer

The contents  of the CQ buffer may  be displayed  by hitting

the  <Shift><F1>  key sequence.  If the buffer is empty, you will

you will see  a  "CQ Buffer Empty"  message  at the center of the

the command line.

4.4 <F2> - AUTO CQ

AUTO CQ may  be turned on by hitting function key <F2>.  You

will be prompted at the center of the command line:

1 to 9 minutes?

You must enter  a number between   1 and 9 minutes.  If you enter

<5>,  AUTOcq5 will  be printed  on  the command line  in  reverse

at  the AUTOcq  slot;  AUTOcq  is  now  turned  on.   Every  five

minutes,  the contents  of the CQ buffer will be sent to the tnc;

assuming no characters have been printed to the receive screen in

5 minutes! For example, lets say I have loaded my CQ message into

the CQ buffer, and it is:

CQ: This is Chuck in Orlando....k

PACK-ET-TERM  will  send  this message  to  the  tnc  every  five

minutes,  provided that no characters are received  at the RS-232

port!  AUTOcq  is  sent after  5 minutes  of  no activity  on the

packet channel.  Your tnc must be  in converse mode,  and your CQ

will  be sent  as set  in your UNPROTO parameter  in the tnc.  So

every  five  minutes  that elapses without channel activity,  the

above message would be sent:

WA4GPF>CQ: CQ This is Chuck in Orlando....k

Hitting   the  <F2>  key  again  will  reverse  the  AUTOcq  slot

printout, and disable the AUTOcq feature.

4.5 <Shift><F2> - Send CQ Buffer

The contents  of CQ buffer will  be sent  to the tnc without

delay,  when  the  <Shift><F2>  key sequence  is hit.  If  the CQ

buffer is empty,  a "CQ buffer empty" message will appear  at the

center of the command line.

Functions F3 thru F8 concern various transmit buffer commands. It

should be noted that shifting F3 thru F8, will do the same thing,

but to the receive buffer.

4.6 <F3> - Clear Transmit buffer

Touching <F3> will clear the contents of the transmit buffer.

4.7 <Shift><F3> - Clear Receive buffer

Touching <F4> will clear the contents of the receive buffer.

4.8 <F4> - Load Transmit buffer

You may load a file from disk into the transmit buffer, with

the <F4> key.  You will  be prompted at the center of the command

line for a filename.  This buffer is 8K in size;  should you need

to load  a larger file,  you must use the receive buffer.  If the

file  is too large  or does not exist, you will receive  an error

message at the center of the command line. If the file is loaded,

"n bytes in TR buffer" will be printed. PACK-ET-TERM will attempt

to load the file from the same drive that was active when PACK-ET

-TERM was loaded; disk IO to other drives is not supported.

4.9 <Shift><F4> - Load Receive buffer

You may load a file from disk into the receive buffer,  with

the <Shift><F4> key sequence.  You will be prompted for filename,

at the center  of the command line;  <Escape> will abort the file

load  at this point.  The receive buffer is 196K in size.  If the

load is successful,  the  "n bytes in Receive buffer"  will print

on the command line.

4.10 <F5> - Save Transmit buffer

The transmit buffer may be saved to disk by hitting the <F5>

key.  You will  be prompted  at the center of the command line to

enter  the filename.  If the file already exists,  you will write

the new file over it, and all previous data will be lost.  If the

file does not exist, it will be created.  Before saving the file,

you should make sure that certain that the disk has adequate room

for the file, and that the disk  is not write protected.  You may

obtain the number  of bytes currently  in the transmit buffer, by

using the <Alternate><u> key sequence.  After the file  is saved,

the number  of bytes saved will be displayed at the center of the

command line;  this  of course be the same as the number of bytes

that were in the buffer at the time of the save.

4.11 <Shift><F5> - Save Receive buffer

The Receive buffer may be saved to disk by using the <Shift>

<F5> sewuence. You will be prompted  at the center of the command

line to enter the filename.  If the file already exists, you will

copy the new file over it, and all previous data will be lost. If

the file does not exist,  it will be created.   Before saving the

file,  make certain that the disk has adequate room for the file,

and that  the disk  is  not write protected.  You may obtain  the

number  of bytes currently  in the transmit buffer,  by using the

<Alternate><d> key sequence. After the file  is saved, the number

of bytes saved will  be displayed  at  the center  of the command

line;  this  of course should  be the same as the number of bytes

that were in the buffer at the time of the save.

4.12 <F6> - Display the transmit buffer

You may display the contents  of the transmit buffer  to the

receive screen with the <F6> key. Files in the transmit buffer do

not normally have linefeeds at the end of each line,  and so this

function generates a linefeed for proper display. Generally lines

in this buffer are terminated with only a <cr>, because they were

entered from the keyboard.

4.13 <Shift><F6> - Display the receive buffer

You may display  the contents  of the receive buffer  to the

receive screen  by using  the <shift><F6>  key sequence.  If  the

buffer  is too large  to be displayed  on the screen at one time,

hitting any key will take you forward to the next screen.  Unlike

the display transmit buffer function, display receive buffer does

not generate  a linefeed  and expects  to have each line  in  the

buffer terminated with a line feed for proper display.

4.14 <F7> - Toggle the Transmit buffer

The transmit buffer  is really  a  keyboard buffer.  You may

open or close it, by touching the <F7> key.  The maximum capacity

of the transmit buffer is 8,192 bytes.

4.15 <Shift><F7> - Toggle the Receive buffer

The Receive buffer may be opened or closed using the <Shift>

<F7>  key sequence.  Maximum capacity  of  the Receive buffer  is

196,602 bytes.

4.16 <F8> - Send the Contents of the transmit buffer

If  you touch  the <F8> key,  the contents  of  the transmit

buffer will be sent to the tnc.

4.17 <Shift><F8> - Send the Contents of the Receive buffer

If you use the <Shift><F8> key sequence, the contents of the

Receive buffer will be sent to the tnc.

4.18 <F9> - Toggle Connect Message

Touching the <F9> key will enable the connect message if the

connect buffer  is not empty.  CONmess will be reverse printed on

the command line,  indicating that the connect message feature is

turned on.  Hitting  the <F9> key  a second time will disable the

connect message function.

4.19 <Shift><F9> - Toggle Connect receive buffer

Using  the <Shift><F9> key sequence will enable  the CONbuff

feature. When enabled, the receive buffer is automatically opened

when a connect message is received. CONbuff is printed in reverse

on the command line.  Hitting the <Shift><F9> sequence a 2nd time

will turn it back off.

4.20 <F10> - Load Connect buffer

Touching the <F10> key will result in a prompt at the center

of  the command  line  for filename;  enter filename  of  Connect

message.  Keep  in mind that this buffer will accept files of 512

characters  or  less;  attempts  to  load longer files  into  the

Connect buffer will result in a "File > Connect buffer" error.

4.21 <Shift><F10> - Display Connect Buffer

The key sequence <Shift><F10> will send the contents  of the

Connect buffer to the receive screen.

4.22 <Alternate><c> - Toggle Connect Printer

The key sequence <Alternate><c> enables the CONprnt feature.

When enabled,  received text will be sent to the printer when the

tnc is connected. Using the <Alternate><c> sequence a second time

will turn the feature back off.

4.23 <Alternate><d> - Display Bytes in Receive Buffer

Use  of  the <Alternate><d> key sequence will result  in the

number  of bytes currently in the receive buffer being printed at

the center of the command line.

4.24 <Alternate><i> - Display Information

Use  of  the <Alternate><i>  key sequence  will display  the

author's copyright information on the receive screen.

4.25 <Alternate><r> - Toggle the Receive Printer

Use  of  the <Alternate><r>  key sequence  will reverse  the

RECprnt message on the command line and send all received text to

the printer. Using the key sequence again will disable RECprnt.

4.26 <Alternate><s> - TIME STAMPS

TIME STAMP  may  be toggled  on  or  off  by using  the key

sequence <Alter><s>.  Either Time Stamp ON or Time Stamp OFF will

be printed  to  the center  of  the command line;  although these

status messages are not protected.  When Time Stamp is ON, a time

message will  be printed at the beginning of any line sent to the

receive screen,  any line saved  in  the receive buffer,  and any

line sent to the printer. Example:

18:04>WA4GPF>FADCA: Try out PACK-ET-TERM 3.0 soon

4.27 <Alternate><u> - Display Bytes in Transmit Buffer

The <Alternate><u> key sequence will display  the number  of

characters currently  in the Transmit buffer at the center of the

command line.

5.0  PACK-ET-TERM OPERATING TIPS

5.1 Making a Connect Message

You may make a connect message with this procedure:

1. Touch  the <F3> key  to clear the transmit buffer;  you should

see  a "Transmit buffer cleared"  message at the center of the

command line.

2. Use the <F7> key  to open the transmit buffer  for typing from

the keyboard;  TR buff should  be printed  on the command line

in reverse now.

3. Enter your connect message;  end with the return key.  Keep in

mind that the maximum size of the connect buffer is 512 bytes;

you will be able  to type  up  to 8192 bytes into the transmit

buffer,  but attempting  to load more than 512 bytes into  the

connect buffer  will result  in  an  error.  Use backspace  or

delete to make corrections.

4. Use the <F7> key again to close the transmit buffer;  you will

see the TR buff slow on the command line unreverse, indicating

that the Transmit buffer is closed.

5. You  may display  the contents  of the transmit buffer  to the

receive screen with the <F6> key.

6. You may add to your connect message by repeating steps  2 thru

5,  keeping  in mind  the 512 character limit!  If you use the

<Alternate><u> sequence, the number of characters currently in

the Transmit buffer will  be displayed  at  the center  of the

command line.

7. If  you  wish  to  change anything entered  into  the transmit

buffer after the <Return> key has been hit,  you will have  to

repeat steps  1 thru 5, retyping  your message.   PACK-ET-TERM

version 3.0 does  not have editing features beyond backspace /

delete.

8. Save the message to disk by touching the <F5> key; you will be

prompted  for  a  filename.  Type  in  a  filename  that means

something, Example:  greet.txt.  Your message will be saved to

disk with the number of bytes saved printed  to the center  of

the command line.

You now have your connect message stored for later use.  For

greater editing ability,  use first word   to create or edit your

connect message;  make sure the WP mode is off when you save your

message to disk.

5.2 Loading and use of Connect Message

You  may load  a previously created connect message into the

connect buffer  to  be  sent  upon connection  with  this feature.

1. Touch  the <F10> key; you will  be promted for  a filename  to

load  the connect buffer.  Enter greet.txt, from  the previous

example.  PACK-ET-TERM  loads  the  file  from  the  disk  and

displays  the number  of bytes loaded  at  the center  of  the

command line.

2. While holding  the <Shift>  key  down,  touch  the  <F10> key;

your connect message will be printed to the receive screen.

3. Touching the <F9> key will enable the CONmess feature; CONmess

will  be  printed  in reverse  on  the command  line.  When  a

connection occurrs, PACK-ET-TERM will send the message that is

in  the connect buffer;  your TNC MUST BE IN CONVERSE MODE for

this to work properly!

4. Touching the <F9> key again will turn the CONmess feature back

off  and reverse  the printing  of   the CONmess slot  on  the

command line.

Note: There is never a need to clear the connect buffer;  loading

a  new connect message,  will  erase  the contents  of  the

previous message.

5.3 Using the Save to Receive Buffer if connected feature.

You  may  automaically  open  the  Receive  buffer  when   a

connection occurrs; to do this:

1. Hold the <shift> key down and touch the <F9> key. You will see

CONbuff reversed on the command line now, and connects will be

saved  to  the receive buffer.  Later,  you  may read thru the

receive buffer,  or save the contents  of the buffer  to disk.

This feature along with the connect message, is a handy way to

create a mailbox.

2. Use the <Shift><F9> key sequence again  to disable the CONbuff

feature.  CONbuff will become unreversed  on the command line,

indicating that it is now turned off.

5.4 Using the Connect Printer feature.

You may automatically save receive text to your printer with

the CONprnt feature.

1. Hold  the <alternate> key down and touch the <c> key.  CONprnt

will  be printed  on  the command line  in reversed video,  to

remind you that it is turned on.  This feature, along with the

connect message, is a handy way to create a mailbox.

2. Use the <Alternate><c> key sequence again  to turn the CONprnt

option back off. CONprnt will become unreversed on the command

line, indicating that is disabled.

5.5 Using Time Stamps

Note:  You must  set the correct time  of day from the control

panel before entering PACK-ET-TERM.

Time stamps may  be added  to  the beginning  of received

lines with this feature.

1. Hold the <Alternate> key down and touch the <s> key;  you will

receive  a Time Stamp ON message  at the center of the command

line.  All received lines  sent  to  the printer,  the receive

screen,  or  the receive buffer  will have  the time stamp  at

their beginning.  This feature  is very useful  for unattended

monitoring of packet activity.

2. Use of the <Alternate><s> sequence a second time, will disable

the feature and send a Time Stamp OFF message to the center of

the command line.

5.6 Downloading from a BBS

Incoming text  is always saved  in the receive buffer.  Lets

say that we are connected to a bbs that has a file named tnc.doc,

that we wish to download and save to disk; here is the procedure,

a step at a time.

1. Clear  the  contents  of  the receive  buffer  by holding  the

<shift> key down and touching the <F3> key.  You should get  a

Receive buffer cleared message  at  the center  of the command

line. Make sure Time Stamping is off!

2. Type  in  the command that starts the BBS sending the file but

do not hit <return> yet!

In the case of a W0RLI bbs it might be: D tnc.doc

3. While holding the <shift> key down, touch the <F7> key to open

the Receive buffer;  RECbuff should now  be highlighted on the

command line.

4. Now,  hit the <Return> key.  Your D tnc.doc is now sent to the

BBS;  your receive buffer is open!  The bbs will send the text

file, and you will see the file come to your receive screen.

5. After the file has been received from the bbs, use the <Shift>

<F7> sequence to close the Receive buffer; RECbuff will become

unhighlighted on the command line.

6. To find out how many bytes are in the receive buffer,  use the

<alternate><d> key sequence; the number will be printed at the

center of the command line.

7. To save  the receive buffer  to disk,  use the <Shift><F5> key

sequence.  You will  be prompted  for  a file name;  enter the

filename: tnc.doc

The file will be saved to disk; the number of bytes saved will

be written to the center of the command line. If the number is

less than the number  of bytes in the buffer displayed in step

5,  there was not enough room  on your disk to save the entire

file and you should insert another disk and resave the file.

8. You may print  the downloaded file  to the receive screen with

the <shift><F6> key sequence.  By hitting any key except <ESC>

you may page through the file.  If it is a long file,  you may

exit back to the terminal mode, by hitting the <ESC> key.

5.7 Uploading to a BBS

You may send  a file to a BBS with this procedure.  The file

may  be  in either the Receive  or  the Transmit buffers for  an

upload,  assuming  that  the file  does  not exceed  the maximum

buffer size.(196K for the Receive buffer and 8K for the transmit

buffer.)   The following example loads the file into the receive

buffer:

1. Look at the command line where it says RECbuff; if the RECbuff

is printed in reverse,  the receive buffer is open and must be

closed with the <Shift><F7> key sequence.

2. While holding  the <Shift> key down,  touch the <F4> key;  you

will now be prompted at the center of the command line for the

filename. Enter the name of the file on the disk that you wish

to transfer  and  hit <Return>.  When the file has been loaded

the number  of bytes  in the Receive buffer will be printed at

the center of the command line.

3. You  may  use the <Shift><F6> key sequence to see the file now

in the receive buffer.  If  it  is  a long file and you do not

care  to page thru the whole thing,  you may use the <Esc> key

to exit.

4. Send whatever  the BBS needs  to upload a file,  somthing like

U tnc.doc. You will be told to send your file and terminate it

with some character, <Control><z> on the W0RLI bbs systems.

5. Use the <Shift><F8> key sequence  to send your file;  you must

be in converse mode! When all of the file has been sent to the

tnc by PACK-ET-TERM, you will see  a "Receive Buffer Sent!" at

the center of the command line.

6. Send the end  of file character,  to let the BBS know that all

of the file has been sent; the <Control><z> key sequence and a

<Return> for a W0RLI bbs system.

5.8 Making Text file.

You  may easily create special files with PACK-ET-TERM which

will make your packeting much more enjoyable. Lets review the use

of the transmit buffer to create a text file:

1. Touch the <F3> key to clear the transmit buffer.

2. Use the <F7> key to open the transmit buffer.

3. Type in each line; ending each line with the <Return> key.

4. When your file  is complete, reclose the transmit buffer using

the <F7> key again.

5. Use the <F5> key to save your file to disk; give it a familiar

name so that you may quickly recall it in the future.

5.9 Using a configuration file.

1. With the procedure given  in section 5.8  we have created  the

following short configuration file and saved it under the file

name of TNC1.FIG :

MYCALL WA4GPF

UNPROTO CQ VIA ORL

AUTOLF OFF

BK ON

2. You load the TNC1.FIG into the transmit buffer using  the <F4>

key.

3. Use the <Esc> key  to return the tnc to CMD mode if necessary.

4. Transmit the file to the tnc by touching the <F8> key; the tnc

should answer back  as if you had typed the new paramters from

the keyboard.

6.0  Other uses for text files

6.1 Rig Information

1. Using  the method given  in section 5.8,  we have created  the

following file and saved it under the filename  RIG

Rig hr is  a Kenwood TS-780 transceiver to a Ringo at 25 feet off

the ground.  I  am also using  the Atari 520 ST with PACK-ET-TERM

software. Tnc is the TAPR-1.

2. Using the <F4> key we load  rig  into the transmit buffer.

3. Touch the <F8> key to send your rig information!

6.2 Routing file

1. Using the method given in section 5.8,  we have made a routing

file and save it under the name of UNJAX:

UNPROTO CQ VIA ORL, OCF, GNV, JAX

2. We load UNJAX into the transmit buffer with the <F4> key.

3. With the tnc in CMD mode,  we hit the <F8> key sending our new

routing information to the tnc.

6.3 CQ messages

1. Using the method given is section 5.8,  we have made a CQ file

and save it under the filename of CQJAX:

CQ CQ: This is Chuck in Orlando via ORL, OCF, GNV, JAX.....k

2. We load this message into the CQ buffer with by using the <F1>

key.

3. The CQ to JAX may now be sent using the <F2> key.

It is handy to create routing and CQ files ahead of time so that

they might  be quickly recalled  and utilized  when needed.  You

may also create  a variety  of Connect Messages  for setting  up

mailboxes, greetings, etc. Build a library of routine text files

and you will reduce your typing during packeting!

#######################################################################################################################