C-play - Tomcat Softworks

Date: 16Jul89

A set of free executeable audio & utility software programs

NEXT.EXE  - This file starts a disc in the audio play mode if

stopped.

id: 0   Tracks: 1-99

If playing, advances to the next track.

No text displayed.

BACK.EXE  - This file starts a disc in the audio play mode if

stopped.

id: 0   Tracks: 1-99

If playing, retreats and plays prior track.

No text displayed.

MODE.EXE  - This file displays the Drive Status and lock status.

Will display Cstat if not zero.

Will display the Track and Times if playing.

MODE4.EXE - Same as mode, EXCEPT reports on drives 0-3 and does

not show play time.

ALLPLAY.EXE - Plays Tracks 1-99 on drive id: 0-3.

The above files are for Port address 300, and Hitachi PC bus

interface cards with the Hitachi CDR-1503S, CDR-3500  or

CDR-3600.

PLAY360.EXE - Allplay at Port address 360H

PLAY340.EXE -                         340H

See also Lock, Unlock and Nap files and documentation.