================================================================================

DEBUGNXT.SYS :  Ver. 1.0, Copyright (1992) by Philip B. Gardner

DEBUGN86.SYS (8086):  Ver. 1.0, Copyright (1992) by Philip B. Gardner

================================================================================

DEBUGNXT.SYS is a companion device driver to WRAPPER.SYS which enables

debugging of the intialization phase of device drivers.  It complements

WRAPPER's special support for loading debuggers, but may also be used

independently of WRAPPER.  DEBUGNXT triggers the debug interrupt (INT 03)

in such away as to trap a debugger at the first instruction of the

the next device following DEBUGNXT in CONFIG.SYS.  DEBUGNXT will operate

only once for each instance that it is loaded, but may be loaded multiple

times in the CONFIG.SYS file.  High memory users may load WRAPPER and/or

DEBUGNXT into high memory with a device loader, or using MS-DOS 5.0's

DEVICEHIGH command.

================================================================================

(Requirements)::

* 186 processor or better (WRAPP86.SYS and DEBUGN86.SYS are

included to support 8086/8088 processors)

* Requires MS-DOS Version 3.1 or better

================================================================================

(Installation)::

(CONFIG.SYS)::

Install DEBUGNXT.SYS in your CONFIG.SYS as a DEVICE

DEVICE=DEBUGNXT.SYS [options]

An example of how to use DEBUGNXT.SYS to debug MYDEVICE.SYS follows.

The debugger is MS-DOS's DEBUG (loaded by WRAPPER).

DEVICE=WRAPPER.SYS /C-/D/M32 C:\DEBUG.EXE

DEVICE=DEBUGNXT.SYS

DEVICE=MYDEVICE.SYS

(Display at boot-up):

WRAPPER.SYS :  Ver. 1.0, Copyright (1992) by Philip B. Gardner

:  User interrupt 60h hooked

(Debug's prompt):

-a 100                               ; assemble at 0100

1234:0100 int 60                     ; the instruction "int 60"

-g                                   ; go

DEBUGNXT.SYS :  Ver. 1.0, Copyright (1992) by Philip B. Gardner

(Stops at first instruction of MYDEVICE's strategy routine):

AX=0000 BX=03E6 CX=0000 DX=0000 SP=05A6 BP=0000 SI=0000 DI=0000

DS=0C96 ES=9087 SS=9087 CS=0C96 IP=0016 NV UP EI PL NZ NA PO NC

0C96:0016 2E            CS:

0C96:0017 891E1200      MOV     [0012],BX          CS:0012=0000

-

All switch options and possible messages for DEBUGNXT are detailed

below:

================================================================================

(Syntax)::

The only thing to remember about syntax for the command line is

that just about anything works.  What this means is that you

should be able to type in the options on the command line the

way they make sense to you.

To illustrate this point, the following command line example

is mutated several different ways.

DEVICE=DEBUGNXT.SYS /R- /P-

DEVICE=DEBUGNXT.SYS -R- -P-

DEVICE=DEBUGNXT.SYS -R--P-

... and so on.

(Switch Options)::

(REPORT)::      R[+ or -]       (toggle:  default +)

Use /R- to disable messages from WRAPPER.SYS.

(PAUSE)::       P[+ or -]       (toggle:  default +)

Use /P+ to enable the pause after the any report messages.

(NOPAUSE)::     N[+ or -]       (toggle:  default -)

Use /N+ to never pause after the report, even if there

are errors.  This switch can be used if your configuration

generates a warning or error message which you choose to ignore.

*  everything in square brackets "[]" is optional

(CONFIG.SYS Line)::

DEVICE=DEBUGNXT.SYS [/R-][/P+][/N-]

device default:  /R+/P-/N-

*  Report status messages

*  Don't Pause after report

*  Pause on errors or warnings

================================================================================

(Report Format)::

DEBUGNXT.SYS :  Ver. 1.0, Copyright (1992) by Philip B. Gardner

ERROR :  (Error Messages)

WARNING :  (Warning Messages)

ADVICE :  (Advisory Messages)

:  (Report Messages)

:  (Allocation Status)

PAUSE :  Press any key to continue...

================================================================================

(Error Messages)::

MS-DOS version 3.10 or above is required

DOS version is not compatible

An 80186 processor or better is required

Bad or missing program specification

User Interrupts 60h - 67h are in use

================================================================================

(Warning Messages)::

Unknown option in command line

Extra characters on command line

================================================================================

(Advisory Messages)::

================================================================================

(Report Messages)::

User interrupt 00h hooked

Invalid Parameter for /D 00

================================================================================

(Allocation Status)::

================================================================================

(Changes)::

================================================================================