------------------------------ setup.doc -------------------------------

Wintab Digitizer Services and Drivers -- installation instructions.

Copyright  LCS/Telegraphics, 1991-1995.  All Rights Reserved.

--------------------------------------------------------------------------

WINDOWS 95 INSTALLATION

--------------------------------------------------------------------------

* Prepare the Device

Make sure your device is properly connected and ready for use according to

manufacturer's instructions.

* Install the Wintab Driver

To install the Wintab driver, use the Windows Control Panel "Add New Hardware"

Wizard. When it asks to search for your new hardware, choose "No". From the

list of hardware types, choose "Other devices".  Choose the "Have Disk..."

button and select the path of the Wintab driver disk or directory. Select your

device model from the list, then follow the prompts.

* Removing the Driver

To remove the driver, use the Device Manager, found under the Windows Control

Panel "System" icon. Select your device from the list, then select "Remove".

The driver will be removed the next time you restart Windows.

* Wintab Drivers and Windows 95 Pen Services

If you install Windows 95 Pen Services after installing the Wintab driver, you

will need to remove and re-install to the Wintab driver for it to provide pen

features.

--------------------------------------------------------------------------

WINDOWS 3.x INSTALLATION

--------------------------------------------------------------------------

* Prepare the Device

Make sure your device is properly connected and ready for use according to

manufacturer's instructions.

* Install the Device Driver

The Wintab driver normally coexists with a conventional mouse driver.  If you

wish to use Wintab without also using a mouse (for example, if you only have

one COM port), you should first install AYEMOUSE.DRV, then install the Wintab

Driver as described below.

* Installing AYEMOUSE.DRV

To use a tablet as your only pointing device, you must install

AYEMOUSE.DRV to keep Windows pointing enabled.  This driver is

provided with your setup kit.  Use Windows' SETUP.EXE, either from

the DOS prompt in your Windows directory, or from within Windows.

Follow the directions to change your mouse driver, and choose 'Other'

from the end of the list of mouse drivers.  When asked for a disk or

pathname, enter the path of your Wintab driver disk or directory.

* Installing a Windows 3.1 Wintab Driver

To install an installable driver, use the Windows Control Panel "Drivers"

Applet.  Choose "Unlisted or Updated Driver" and select the path of

the Wintab driver disk or directory.

* If you have "Windows for Pen Computing"

Use the Drivers Applet in the Windows Control Panel to install the Wintab Pen

Interface, WTPEN.DRV.  This pen driver makes any Wintab-compatible pointing

device pen-compatible.

* Wintab Drivers and Upgrading to Windows 95

If you have installed a Wintab driver and you wish to upgrade your

configuration to Windows 95, you must remove the Wintab drivers before

upgrading and re-install them once the upgrade is complete.

--------------------------------------------------------------------------

SYSTEM.INI STRINGS USED

--------------------------------------------------------------------------

* AYEMOUSE.DRV

The AYEMOUSE.DRV driver changes the following strings in SYSTEM.INI:

[boot]

mouse.drv=ayemouse.drv

[boot.description]

mouse.drv=Enable Windows pointing without a mouse.

* Installable Drivers

Installable drivers use an "alias" in performing their SYSTEM.INI

modifications.  The alias is usually the same as the driver file name,

minus the extension.  They will add the alias to the following line:

[boot]

drivers=mmsystem.dll <other aliases may appear here> <alias>

Drivers will then add the following line:

[drivers]

<alias>=<driver file name>

* 386 Enhanced Mode Virtual Device Drivers:

If you are using the installable driver, find the line in the [drivers]

section of SYSTEM.INI that contains the filename of the driver you are

using.  The portion to the left of the equals sign is the driver's "alias".

At the end of SYSTEM.INI, add the lines:

[vsertd.386]

<Alias>=<X>

where Alias is the driver's alias discovered from the [drivers] section,

and X is the com port number, as above.  For example, if you found this:

[drivers]

wtmmkye=wtmmkye.drv

add this:

[vsertd.386]

wtmmkye=1

--------------------------------------------------------------------------

ADVANCED INSTALLATION OPTIONS

--------------------------------------------------------------------------

* Nonstandard Com Ports

If you have a non-standard com port IRQ line or IO port base address, add the

following lines to the [386Enh] section of SYSTEM.INI:

COM<X>IRQ=<I>

COM<X>BASE=<B>

where I is an IRQ line number from 2 through 15, B is a base address in

hexadecimal, and X is the com port number.  For example:

COM3IRQ=5

COM3BASE=338

--------------------------------------------------------------------------

Copyright  LCS/Telegraphics, 1991-1995.  All Rights Reserved.