Rebooting:

40:72h holds 1234h for warm boot, and 4321h (or != 1234h) for cold boot

jmp to F000:FFF0h

-----------------------------------------------------------------------------

Speaker:

Port 61h  read  (ISA, EISA)

Bit [7] = Parity check (parity error)

Bit [6] = Channel check (ISA parity error)

Bit [5] = Timer 2 output

Bit [4] = Toggles with each refresh request

Bit [3] = Channel check enable (enable ISA parity check)

Bit [2] = Parity check enable (enable parity check)

Bit [1] = Speaker data enable (Timer 2 output enable)

Bit [0] = Speaker timer enable (Timer 2 gate enable)

Port 61h  write  (ISA, EISA)

Bit [7-4] = Reserved

Bit [3] = Channel check enable (enable ISA parity check)

Bit [2] = Parity check enable (enable parity check)

Bit [1] = Speaker data enable (Timer 2 output enable)

Bit [0] = Speaker timer enable (Timer 2 gate enable)

Port 61h  write  (XT only)

Bit [7] = 1 Clear keyboard

Bit [6] = 0 Hold keyboard clock low

Bit [5] = 0 I/O check enable

Bit [4] = 0 RAM parity check enable

Bit [3] = 0 Read low switches

Bit [2] = Reserved

Bit [1] = 1 Speaker data enable (Timer 2 output enable)

Bit [0] = 1 Speaker timer enable (Timer 2 gate enable)

-----------------------------------------------------------------------------

8254 Programmable Interval Timer (PIT)

Counter 0:  System timer, Always on, 1.193MHz, IRQ0

Counter 1:  Refresh request, Always on, 1.193MHz, Request refresh

Counter 2:  Speaker frequency, enable speaker, 1.193MHz, Speaker signal

Port 40h  Read/Write    Counter 0

Port 41h  Read/Write    Counter 1

Port 42h  Read/Write    Counter 2

Port 43h  Write/Only    Control register

Bits [7..6]     Select counter

00          Counter 0

01          Counter 1

10          Counter 2

11          Read back command

Bits [5..4]     Operation

00          Latch counter (bits[3..1] are don't care)

01          Read/Write LSB only

10          Read/Write MSB only

11          Read/Write LSB, then MSB

Bits [3..1]     Operating mode selection

000         Mode 0:  interrupt on terminal count

001         Mode 1:  hardware triggered one-shot

x10         Mode 2:  rate generator

x11         Mode 3:  square-wave generator

100         Mode 4:  software triggered strobe

101         Mode 5:  hardware triggered strobe

xxx         For latch counter operation

Bit [0]         Binary or BCD count down format

0           Binary (16-bit) count down

1           BCD count down (four-decades)

x           For latch counter operation

Port 43h  Write/Only    Control Register (read-back command)

Bits [7..6]

11          Read back command

Bits [5..4]

0x          latches the state of the CE in COL and COH

x0          latches status of selected counters into the

status register.

Bits [3..0]

1xx0        Selects counter 2

x1x0        Selects counter 1

xx10        Selects counter 0

If status is latched, status is read first through counter read/write

register.

If count is latched, the count is read back through counter read/write

retister, one or two reads depending on how programmed.

Status Byte Latched

Bit [7]

0           OUT signal 0 (low)

1           OUT signal 1 (high)

Bit [6]

0           Counter loaded from the counter input registers,

count can be read.

1           Write to the congtrol register or the counter, but

the new value has not been loaded into CE.

Bits [5..4]

00          Reserved

01          Read/Write LSByte

10          Read/Write MSByte

11          Read/Write LSByte then MSByte

Bits [3..1]

000         Mode 0

001         Mode 1

010         Mode 2

011         Mode 3

100         Mode 4

101         Mode 5

Bit [0]

0           Binary (16-bit) count down

1           BCD count down (four decades)

1.19318MHz = 4.77273MHz/4.  Typically Counter 0 is set for 0000h (max

count) 54.925ms.  There are 1573040 tics per day.

This is an example code segment that shows how to speed up the timer

resolution.  I had no problem with this piece of code on a 486DX2-66,

on slower machines you may need to slow it down.

---- cut ----

.model tiny

.code

org 100h

entry:

jmp over

old08vec dd ?

keeptime dw 0000h

isr08h:

inc word ptr cs:[keeptime]

; at this point, you have a sped up interrupt

; be careful not overload the processor

cmp cs:[_counter],0FFFFh

jz notenabled

notenabled:

cmp word ptr cs:[keeptime],1000h

jz handoff

push ax

mov al,20h

out 20h,al

pop ax

iret

handoff:

mov word ptr cs:[keeptime],0000h

jmp dword ptr cs:[old08vec]

over:

mov ax,3508h

int 21h

mov word ptr cs:[old08vec],bx

mov word ptr cs:[old08vec+2],es

mov ax,2508h

mov dx,offset isr08h

;tiny no ds

int 21h

mov al,10h

out 40h,al

mov al,00h

out 40h,al

mov ah,00h

int 16h

xor al,al

out 40h,al

out 40h,al

mov ax,2508h

mov dx,word ptr cs:[old08vec]

mov ds,word ptr cs:[old08vec+2]

int 21h

mov ax,4C00h

int 21h

end entry

---- cut ----

The speaker timer (counter 2) can also be used for high resolution

timing for periods less than 55ms total, using this timer avoids

the potential harm to the system like Counter 0 or 1.  One

implementation might be:

Enable timer 2 gate, disable timer 2 output:

out(61h,01h)

Set Counter 2 for interrupt on terminal count:

out(43h,B0h)

Set MSB to start count and halt counting:

out(42h,FFh)

Set LSB to start count and start counting:

out(42h,FFh)

Perform task to be timed (with no speaker access)

Latch count:

out(43h,C0h)

Read count:

LSByte=in(42h)

MSByte=in(42h)

-----------------------------------------------------------------------------

8259 Programmable Interrupt Controllers (PICs)

Port 20h    write a 20h to clear interrupts (during isr)

can write a 60h+irq, but that is not necessary if

the PIC is configured for modes that preserve the fully

nested structure (which is how you will find it normally)

Port 21h    read/write, enables and disables irqs 0-7,

0 = enabled, 1 = disabled

Bit [7]  irq 7

Bit [6]  irq 6

Bit [5]  irq 5

Bit [4]  irq 4

Bit [3]  irq 3

Bit [2]  irq 2

Bit [1]  irq 1

Bit [0]  irq 0

Port A0h    write a 20h to clear interrupts (during isr)

can write 60h+irq-8, see above (Port 20h)

Port A1h    read/write, enables and disables irqs 8-15,

0 = enable, 1 = disable

Bit [7]  irq 15

Bit [6]  irq 14

Bit [5]  irq 13

Bit [4]  irq 12

Bit [3]  irq 11

Bit [2]  irq 10

Bit [1]  irq  9

Bit [0]  irq  8

--------------------------------------------------------------------------------

8237 DMA Controller (DMAC)

Port 0000h  DMA Channel 0, Memory address register (r/w)

16-bit, LSByte first then MSByte

Port 0001h  DMA Channel 0, Transfer count register (r/w)

16-bit, LSByte first then MSByte

Port 0002h  DMA Channel 1, Memory address register (r/w)

Port 0003h  DMA Channel 1, Transfer count register (r/w)

Port 0004h  DMA Channel 2, Memory address register (r/w)

Port 0005h  DMA Channel 2, Transfer count register (r/w)

Port 0006h  DMA Channel 3, Memory address register (r/w)

Port 0007h  DMA Channel 3, Transfer count register (r/w)

Port 0008h  DMA Status register (r)

bit [7] = 1 Channel 3 request

bit [6] = 1 Channel 2 request

bit [5] = 1 Channel 1 request

bit [4] = 1 Channel 0 request

bit [3] = 1 Terminal count on channel 3

bit [2] = 1 Terminal count on channel 2

bit [1] = 1 Terminal count on channel 1

bit [0] = 1 Terminal count on channel 0

Port 000Ah  DMA Channel 0-3 Mask register (r/w)

bits [7..3] = 0 reserved

bit [2] = 0 Clear mask bit

1 Set mask bit

bits [1..0] = 00 Select channel 0

01 Select channel 1

10 Select channel 2

11 Select channel 3

Port 000Bh  DMA Channel 0-3 Mode register (r/w)

bits [7..6] = 00 Demand mode

01 Single mode

10 Block mode

11 Cascade mode

bit [5] = 0 Address increment select

bit [4] = 0 Autoinitialized

1 Non-autoinitialized

bits [3..2] = 00 Verify operation

01 Write operation

10 Read operation

11 Reserved

bits [1..0] = 00 Select channel 0

01 Select channel 1

10 Select channel 2

11 Select channel 3

Port 000Ch  DMA Clear Byte Pointer (w)

Port 000Dh  DMA Master Clear (w)

Port 000Eh  DMA Clear Mask register (w)

Port 000Fh  DMA DMA Channel 0-3 Write Mask register (w)

bits [7..4] = 0 reserved

bit [3] = 0 Unmask channel 3 mask bit

1 Set channel 3 mask bit

0 Unmask channel 2 mask bit

1 Set channel 2 mask bit

0 Unmask channel 1 mask bit

1 Set channel 1 mask bit

0 Unmask channel 0 mask bit

1 Set channel 0 mask bit

Port 0019h  DMA Scratch register (r/w)

Port 0081h  DMA channel 2, page table address register (r/w)

Port 0082h  DMA channel 3, page table address register (r/w)

Port 0083h  DMA channel 1, page table address register (r/w)

Port 0087h  DMA channel 0, page table address register (r/w)

Notes:

First off, DMA can only access the lower meg (some channels can access

16M on ATs) which translates to a 20-bit address.  Secondly, DMA cannot

read/write across a physical page boundary (address with lower 16-bits = 0).

This also means that DMA cannot transfer more than 64K (actually 65535

bytes) bytes at a time.  To guarantee no boundary conflicts, many

programmers allocate 128KBytes, and find the first page boundary within

that memory space.  Here is an example in Borland 'C' to get 65000 bytes:

---- cut ----

unsigned char *data;

unsigned char *aligned;

unsigned long aligned_physical;

...

unsigned long physical;

data=farmalloc(131000L);

if(data==NULL)

{

printf("Memory Allocation Error\n");

exit(1);

}

physical=((unsigned long)FP_OFF(data))+

(((unsigned long)FP_SEG(data))<<4);

aligned_physical=physical+0x0FFFFL;

aligned_physical&=0xF0000L;

aligned=MK_FP((aligned_physical>>4)&0xFFFF,0);

---- cut ----

to have a device read len bytes from the above memory (*aligned) (DMA

channel 1):

---- cut ----

len--;

outportb(0x0A,0x05);

outportb(0x0C,0x00);

outportb(0x0B,0x49);

outportb(0x02,0);

outportb(0x02,0);

outportb(0x83,(aligned_physical>>16)&15);

outportb(0x03,(unsigned char)(len&0xFF));

outportb(0x03,(unsigned char)((len>>8)&0xFF));

outportb(0x0A,0x01);

---- cut ----

to have a device write len bytes from the above memory (*aligned) (DMA

channel 1):

---- cut ----

len--;

outportb(0x0A,0x05);

outportb(0x0C,0x00);

outportb(0x0B,0x45);

outportb(0x02,0);

outportb(0x02,0);

outportb(0x83,(aligned_physical>>16)&15);

outportb(0x03,(unsigned char)(len&0xFF));

outportb(0x03,(unsigned char)((len>>8)&0xFF));

outportb(0x0A,0x01);

---- cut ----