TimeCalc V1.0

1993 Paul Huxham

paulh@wavefrm.DIALix.oz.au

PO Box 875

Morley,

Perth,

Western Australia 6062

Well, I never found a decent timecode calculator, so here is one that works

correctly  (For  me  at  any rate) with EBU and SMPTE timecodes (SMPTE Drop

Frame  is not currently supported).  It has clever string gadgets that make

timecode entry a breeze.

The timecode entry routines are part of the "Diamond Edit" package which is

a  single  frame  recorder  for the PAL environment (soon for NTSC) that is

a  very  powerful  and  cost  effective  means  of  recording  hires  24bit

animations   to  video  tape  for  playback  at  a  full  frame  rate  (for

professional VTRs only).

Please  direct  all  comments  and  criticisms (all welcome) to the authors

interNET address above.

===========================================================================

OverView

Timecode  is entered into Timecode Entry Gadgets.  At first sight they look

very  much  like standard string gadgets, except they have three colons ':'

inside of them.

When  you  have  clicked  in  a  timecode entry gadget, there are shortcuts

available that make for quick and easy timecode entry.

You  can enter an IN and OUT time and TimeCalc will show you the difference

between  them, or you can enter an IN time and a DURATION and TimeCalc will

generate the corresponding OUT time automatically.

TimeCalc  will  also  show  the total number of frames to the right of each

gadget.

Pull  down menus enable you to select a timecode entry mode, as well as get

infor about the author and quit the program.

---------------------------------------------------------------------------

CLI interface

-------------

There are 5 options available when executing TimeCalc from a CLI/Shell:

1)  FILM     - 24 Frames per second

2)  EBU      - 25 Frames per second    (default)

3)  SMPTEDF  - 29.97 Frames per second (Not implemented yet)

4)  SMPTE    - 30 Frames per second

5)  PUBSCREEN=<Public screen name>

Entering  any  of  the first 4 options, will set the timecode entry mode in

TimeCalc to that mode.

The  PUBSCREEN  can  be  used to force TimeCalc to open on a public screen.

This  is best when called as a hotkey from Tool Manager, using "GetPubName"

to retrieve the frontmost public screens' name.

Workbench interface

-------------------

Currently there is only one tooltype:

MODE=<FILM | EBU | SMPTEDF | SMPTE>

This is used to preset the timecode entry mode for TimeCalc.

---------------------------------------------------------------------------

Clicking in an Entry Gadget

When  you  click  in an entry gadget do not bother worrying about where you

click.  Timecode entry will always start at the right and move to the left,

just  like  a  calculator.  The cursor will snap over to the right when you

start  typing.   So,  to enter 1 second and 5 frames, you only need to type

105<return>.

The  separator  colons  ':'  are not entered by you, they are placed by the

gadget  automatically.   The  only  part  of the timecode you need to worry

about is the numbers themselves.

---------------------------------------------------------------------------

Movement between gadgets

Pressing  <return>  in the IN gadget moves you automatically to the OUT and

then on to the DURATION gadget and then deselects all gadgets.

You  can also TAB between the gadgets or use the up and down cursor keys to

move between gadgets.

---------------------------------------------------------------------------

Entering digits

You  do  not  have  to enter all of the hours, minutes, seconds and frames.

Unentered  digits always assume the value of zero.  This means if you enter

312<return>,   you   have  entered  3  seconds  and  12  frames.   Entering

450622<return> means 45 minutes, 6 seconds and 22 frames.

If  you  do  not enter enough digits to reach hours, the gadget will assume

you mean no hours.

You  will  be unable to enter any more than 8 digits into any entry gadget.

This  is  because  there are only 2 digits available for each of the hours,

minutes, seconds and frames.

Pressing the backspace key will erase the last entered digit.  You can move

the  cursor  across  to  other  digits, but it will always snap back to the

right  when  you  enter  more digits.  If you have entered 4 digits and you

find  that the first digit is incorrect, you will have to delete all of the

entered digits and retype them.

---------------------------------------------------------------------------

Incorrect timecode entry

Timecode  that is not correct i.e.  28 frames; will be corrected by summing

the  total number of frames in the entered timecode and then splitting this

value up into hours, minutes, seconds and frames.

With  EBU timecode selected, entering 250<return> would appear to be 2 secs

and  50 frames.  However you can't have 50 frames so this will be summed to

a  value  of  100  frames  and  then  split into a value of 4 seconds and 0

frames.  (Because there are 25 frames in one second in EBU).

---------------------------------------------------------------------------

Wrap around

Timecode that runs over 23 hours, 59 minutes, 59 seconds and 24 frames will

turn over to 0 hours, 0 minutes, 0 seconds and 0 frames.

---------------------------------------------------------------------------

Aborting timecode entry

Pressing escape will cancel the current timecode entry operation and return

the timecode previously entered into the gadget before you started changing

it.

---------------------------------------------------------------------------

Entering two zeros

Pressing  the  fullstop key on the numeric keypad or main keyboard enters a

double zero '00'.

It  is  a  quick  way  of  entering  say 2 minutes, which would normally be

entered as 20000<return>.  Type 2..<c r> instead.

To  delete  a double zero, you will need to press backspace twice as double

zeros are just two normal zero digits.

---------------------------------------------------------------------------

Entering a number of frames

If  you  need to enter a number of frames less than 99, you just enter them

and  press  <return>.  They will be converted to the correct timecode value

for you.

If  however  you  need  to enter say 1000 frames, enter 1000 and instead of

pressing <return>, press 'f'.  This will enter 100 frames and convert it to

its correct timecode value.

---------------------------------------------------------------------------

Plus Mode

Pressing  + on the numeric keypad or = on the main keyboard enters the plus

mode  where  the  timecode  you  enter  next  will be added to whatever was

displayed in the gadget before you entered plus mode.  A + sign will appear

in the left end of the entry gadget indicating plus mode is active.

This  is  an  easy  way  to add one or two frames or minutes to the current

timecode value.

---------------------------------------------------------------------------

Minus Mode

Pressing  -  either on the numeric keypad or main keyboard enters the minus

mode where the timecode you enter next will be taken away from whatever was

displayed  in  the  gadget  before  you  entered minus mode.  A - sign will

appear in the left end of the entry gadget indicating minus mode is active.

This  is  an  easy  way  to  subtract one or two frames or minutes from the

current timecode value.

---------------------------------------------------------------------------

Sum Mode

Pressing  *  on the numeric keypad or \ on the main keyboard will resolve a

plus  or  minus  operation  currently being entered and then continue entry

until you press <return>.  You must press return to store your entry.

After  using  this  function  you can continue entry as though you had just

clicked in the entry gadget with the mouse.

This  is  extremely  useful when you are adding to an existing timecode, to

see a plus or minus result before accepting a final value.