SIMPOUTR.DOC       VERS:- 01.00 DATE:- 09/26/86 TIME:- 10:02:54 PM

description of output from the simplex fitting programs

By J. A. Rupley, Tucson, Arizona

DESCRIPTION OF OUTPUT FROM THE FITTING PROGRAM

LDHFIT.COM (C-LANGUAGE)

A. RESULTS

At various stages of the fitting, determined by the control

values prt_cycle and quad_test, the computer program displays on

the crt terminal and stores on disk the parameter and data arrays

and other results.

The interpretation of this output is explained here.  The

description is for the output from LDHFIT, but it holds, except

for some special values given at the end of the data display, for

any function fit by use of the SIMPxxxx routines.

(1) SIMPLEX: for each vertex of the simplex, the function value

is displayed, then two lines of values (four values per line)

give in order the values of the 7 parameters. Note that

parameters (1) through (6) are different for each vertex, ie they

are free to be optimized, and that parameter (7) is fixed at the

small value 1E-10. The number of vertices is one greater than the

number of free parameters ( 6 + 1 = 7 ).

NOTE: in some of the displays, the count of vertices or

parameters starts at 0. For example, in the above display of the

simplex, the first vertex is vertex(0) and the 7th is vertex(6).

(2) CENTROID OF THE SIMPLEX: the function value is given, then

two lines of values (four values per line) give in order the

values of the 7 parameters. The centroid is the average of the

vertices of the simplex and is the preferred set of estimates of

the best-fit parameter values. These estimates are used in the

calculation of YCALC, of the points for construction of

reciprocal plots, and of the values of the kinetic constants,

given later in the output.

(3) EXIT TESTING: immediately following the centroid listing,

one line gives the mean of the function values, the root mean

square deviation of the function values from the mean, and the

test value = rms deviation/mean. When the test value is less than

the control value exit_test, generally set at 0.00000001, the

fitting is completed.

(4) RMS ERROR: the root mean square error of the fit is the

square root of the quantity: the least squares function value at

the centroid divided by the number of data points less the number

of free parameters ( RMS ERROR = SQRT(CENTROID SSQ/(# DATA - #

FREE PARAMETERS). The rms error is useful as a test for the

quality of the fit (ie, of the deviations between observed and

calculated values). The rms error should be no greater than the

1

average expected uncertainty in the data. If it is greater than

expectation, the fit may be unsatisfactory. The rms error can

also be used as a test for flyers among the data points. A

deviation of 3 x rms error between the observed and calculated

values is sufficient to allow dropping of a data point from the

fitting, or to cause you to check your calculation of vo.

(5) DATA AND CALCULATED VALUES: for each data point are listed

in order the values of: Yobserved(i), which is the initial

reaction rate you measured and entered into the program as data;

Ycalculated(i) = Ymodel(i), which is the initial reaction rate

calculated with the model equation and the best-fit parameter

values of the centroid; Yobserved(i) - Ycalculated(i), which is

the deviation between observation and calculation; the

statistical weight of the data point, here set at unity; and the

values of the independent variables for the data point, which are

in order the concentrations of NADH, pyruvate, and lactate (a, b,

and p).

(6) VALUES CALCULATED FROM THE BEST-FIT PARAMETERS: following

the display of the data, various other values are displayed: (a)

the coordinates of points that can be used to draw the lines of

the primary and secondary reciprocal plots used in steady-state

kinetic analysis; (b) values of the parameters, a variation of

the listing under CENTROID.

(7) QUADRATIC FIT: at intervals determined by the control value

quad_test, quadratic fitting of the function surface is carried

out, to reconstruct the simplex and to obtain standard deviations

of the free parameters. There is considerable screen display that

allows tracking of the process. The handout "Notes on the fitting

program", subsection on standard deviations, serves as an

explanation of this output. The information you are likely to be

most interested in is the last group of values, under "STANDARD

DEVIATIONS OF THE FREE PARAMETERS." Look especially for failure

of the quadratic fit to give proper standard deviations. If the

quadratic fit fails, the values of the Q MATRIX DIAGONAL and of

the B MATRIX DIAGONAL can be helpful in deciding what to do, eg.

whether to fix a parameter.

(8) STANDARD DEVIATIONS FOR THE FREE PARAMETERS: after

completion of the quadratic fit, a table is displayed which

includes estimates of the standard deviations of the free

parameters. Following this table, the vertices of the

reconstructed simplex are given. Further minimization starts with

this simplex. If y-pmin is less than yzero = ycentroid, the

parameter values at pmin, an improved estimate, are inserted as

one of the vertices of the reconstructed simplex. This procedure

can speed up convergence.

2

B. MINIMIZATION

There is output during minimization. In each cycle of the

Nelder-Mead simplex method, the vertex with the highest least

squares function value is adjusted to give a lower value. The

output displayed at each cycle allows one to track this process.

(1) One line giving in order: the number of the cycle, the

values of the least squares function before adjustment and after

adjustment, and the adjustment operation.

(2) One or more lines giving the parameter values of the vertex

before adjustment.

(3) One or more lines giving the parameter values after

adjustment.

(4) One or more lines giving the parameter values of the

centroid of the simplex.

(5) One line giving the mean and rms deviation of the function

values of all the vertices of the simplex after the adjustment,

and the test value rms deviation/mean.

(6) One line giving the rms error of the current best fit to the

data.

(7) One line describing results of the last quadratic fit.

3