GRAD Programmers' Reference Manual

Table Of Content

1   Introduction . . . . . . . . . . . . . . . . . . . . . .    1

2   Basic Philosophy . . . . . . . . . . . . . . . . . . . .    2

3   calcaddr and plottype  . . . . . . . . . . . . . . . . .    3

3.1  calcaddr . . . . . . . . . . . . . . . . . . . . . .    3

3.2  plottype . . . . . . . . . . . . . . . . . . . . . .    3

4   Frame Table  . . . . . . . . . . . . . . . . . . . . . .    4

5   Font Table . . . . . . . . . . . . . . . . . . . . . . .    5

6   HorzLine, VertLine and Line  . . . . . . . . . . . . . .    6

6.1  HorzLine . . . . . . . . . . . . . . . . . . . . . .    6

6.2  VertLine . . . . . . . . . . . . . . . . . . . . . .    6

6.3  Line . . . . . . . . . . . . . . . . . . . . . . . .    7

7   Circle, Ellipse and Arc  . . . . . . . . . . . . . . . .    9

7.1  Circle . . . . . . . . . . . . . . . . . . . . . . .    9

7.2  Ellipse  . . . . . . . . . . . . . . . . . . . . . .   10

7.3  Arc  . . . . . . . . . . . . . . . . . . . . . . . .   11

8   Text Writing . . . . . . . . . . . . . . . . . . . . . .   12

9   Region Filling . . . . . . . . . . . . . . . . . . . . .   13

10  Block Copy, Save and Load  . . . . . . . . . . . . . . .   15

11  Writing Graphics Display Adapter Driver  . . . . . . . .   16

12  Writing Printer Driver . . . . . . . . . . . . . . . . .   17

13  Hints on Porting to Other C Compiler or Other Machine  .   18

GRAD Programmers' Reference Manual

1   Introduction

The source code of the GRAD library are coyrighted. However

you  are  free  to modify and re-distribute the source files for

non-commerical  use  and  you indiciate the files are originated

from the GRAD library.

If you find and bugs in the GRAD routines, please inform me.

Your help will be appreciated.

1

GRAD Programmers' Reference Manual

2   Basic Philosophy

The idea of GRAD  system come to  me about 1  year ago (1986

summer). At that time, I want to write a program that can produce

high quality,  high speed full page  text and  graphics print out

suitable for report on a dot matrix printer.

In order to  produce high  quality text  output,  bit mapped

character must be able to be loaded and included in the print out

as  graphics  data.  However printing a full  page  of bit mapped

graphics data on a dot matrix printer may take  5  to 15 minutes.

So  all normal  text  should be printed in  NLQ text  mode of the

printer instead of in graphics mode.

Furthermore 2  types of graphics data may be included in the

print out.  First type is that the graphics  data  are already in

bit mapped format.  Another type is that the graphics data are in

high level command format  such  as circles,  lines and dots. The

advantages of second type are that the graphics data file size is

relatively small unless the drawing is very complicated. Further-

more and the drawings may be changed easily.

Very  quickly,  I realized  that  this  program is  not that

simple.  As the first phase of implementation, a graphics library

is written which is the GRAD system. The sample program MPrint is

a simplified version of my original idea.

In GRAD,  you may divide the routines into different layers.

Layer  0  contains routines that directly deal with  the hardware

(including  memory).   Hence  layer   0   routines  are  hardware

dependent.  The  number  of routines in  this layer is kept  to a

minimum.   Layer  1  routines  accepts  physical  coordinates  as

parameters. No external GRAD functions are in this layer. Most of

this routines  are written in  assembly  language. GRAD functions

usually  spend most  of  them  in  this  layer of  codes. Layer 2

routines handles translation of  logical coordinates  to physical

coordinates and also clipping  of drawings under  current defined

window.  Most  of  these routines are  written in C.  Most of the

layer  2  routines call layer  1  routines to do actual drawings.

However there are some exception cases that layer 2 routines call

layer 0  routines directly.  But none of the routines  in layer 1

and layer 2 access hardware directly.

There are also  some  functions  that  do  not  requires any

coordinates in the parameter  such as PlotType and ResetWin. They

are called system functions and do not belong  to any  one of the

above layers.

2

GRAD Programmers' Reference Manual

3   calcaddr and plottype

3.1  calcaddr

$calc and downln  are the two main  routines  in  the module

calcaddr. There are also some hardware dependent routines such as

settext  and setgraph  but they  would  not be discussed  in this

manual.

$calc accepts x and y coordinates  in  register  AX  and BX.

Segment and offset values are returned in AX and BX respectively.

The  return  address a far address that  means the  segment value

should not be changed because the segment value may  be a logical

number only  and not a actual  segment number.  The  offset value

must be a even number and it must  be set  up in such  a way that

the  whole line  containing  the point  x,y  can  be  accessed by

changing the offset value only.

downln accepts  the address of  a frame in  ES:BX and return

the address of the next line with  the same number of  words from

the beginning  of the line.  This is used to speed up drawings in

vertical direction such as VertLine.

3.2  plottype

The module plottype contain routines  that  access the frame

memory using the address returned  by $calc and  downln. There is

nothing fancy so the comment in the module should be sufficient.

3

GRAD Programmers' Reference Manual

4   Frame Table

In  the frame table,  it  contains  all information  about a

frame.  Information include starting address of  the frame, frame

width in byte,  horizontal and vertical size,  logical origin and

the window defined in  that frame.

The  number of entries in  frame table controls  the maximum

number of frame that can be existed in GRAD system. The number of

entry in the  frame table is  control by the  value NFRAME during

compilation time. The default value of NFRAME is 10.

The structure of  the frame table may be  changed  in future

version.  So do not make any assumption about  the relative order

of the variables inside the structure in your routines.

4

GRAD Programmers' Reference Manual

5   Font Table

Font  table contains  information  about  each  font loaded.

Information  include the font  type (fix size  or variable size),

lowest code and highest  code number,  the default character code

and  also  pointers  to  the  pattern  and  individual  character

information.

Individual  character  information  include  width,  height,

inkwidth,  cellheight,  leftmargin and topline. This is one entry

for fix size font and one entry per character  for  variable size

font.

There  is also an  additional array of pointers  pointing to

pattern of individual characters for variable size font. The font

table contains the pointer to the array of pointers.

5

GRAD Programmers' Reference Manual

6   HorzLine, VertLine and Line

6.1  HorzLine

The C code  can be  used  to  replace  the  assembly routine

horzl. It uses the same algorithm as horzl.

horzl(x,y,length)

int x,y,length;

{

int far *addr;

unsigned int pword;

int len,loop;

addr=calcaddr(x,y);

len = (x & 0x0f) + length;

if (len <= 16) {

pword=LEFTWORD[x & 0x0f] ^ LEFTWORD[len];

fr_write(addr,pword);

return;

}

fr_write(addr,LEFTWORD[x & 0x0f]);

addr++;

for (loop=len >> 4; loop > 1; loop--,addr++)

fr_write(addr,0xffff);

fr_write(addr,RIGHTWORD[len & 0x0f]);

}

The program should not be difficult to understand.

6.2  VertLine

The C routine below is  functional equivalent to  vert1l. It

only draws a line 1 pixel wide.

vertline(x,y,length)

int x,y,length;

{

int far *addr, far *downline();

unsigned int pword;

int loop;

addr=calcaddr(x,y);

LASTY=y;

pword=DOTVALUE[x & 0x0f];

while (length-- > 0) {

fr_write(addr,pword);

addr=downline(addr);

}

}

To draw a vertical line of 1 pixel wide is quite simple. But

when the width can be  any number  then  the routine  become much

more complicated.  The  routine  vertl  combines  both  horzl and

vert1l together.  The  downline  routine is not  included in GRAD

library so the code is listed below.

6

GRAD Programmers' Reference Manual

_downline proc  near

push    bp

mov     bp,sp

push    si

les     bx,[bp+smo]

call    downln

mov     dx,es           ; result return in DX:AX

mov     ax,bx

pop     si

pop     bp

ret

_downline endp

6.3  Line

The algorithm used in drawing line is explained in  the book

"Fundamental of Interactive  Computer Graphics"  by Foley and Van

Dam p.433-436.  The C routine below can draw a line but there are

a lot of difference between this routine and the routine  in GRAD

library. So the one below is for reference only.

line(x1,y1,x2,y2)

int x1,y1,x2,y2;

{

int dx,dy,d,x,y,xend,yend,len,incr1,incr2,xdir,ydir,dir;

xdir=ydir=0;

if ((dx=x2-x1) < 0) { dx = -dx; xdir=(-1); }

if ((dy=y2-y1) < 0) { dy = -dy; ydir=(-1); }

if (dx == 0) {

if (xdir)

vertline(x1,y2,++dy);

else

vertline(x1,y1,++dy);

}

if (dy == 0) {

if (ydir)

horzline(x1,y2,++dx);

else

horzline(x1,y1,++dx);

}

if ((xdir==(-1)) && (ydir==(-1)))

dir=1;

else if (xdir || ydir)

dir=xdir+ydir;

else

dir=1;

if (dy <= dx) {

d = (dy << 1) - dx;

incr1 = dy << 1;

incr2 = (dy - dx) << 1;

if (xdir) {

x=x2; y=y2; xend=x1;

}

else {

x=x1; y=y1; xend=x2;

7

GRAD Programmers' Reference Manual

}

len=1;

while (x++<xend) {

if (d<0) {

d+=incr1;

len++;

}

else {

horzline(x-len,y,len);

len=1;

d+=incr2;

y+=dir;

}

}

horzline(x-len,y,len);

}

else {

d = (dx << 1) - dy;

incr1 = dx << 1;

incr2 = (dx - dy) << 1;

if (ydir) {

x=x2; y=y2; yend=y1;

}

else {

x=x1; y=y1; yend=y2;

}

len=1;

while (y++<yend) {

if (d<0) {

d+=incr1;

len++;

}

else {

vertline(x,y-len,len);

len=1;

d+=incr2;

x+=dir;

}

}

vertline(x,y-len,len);

}

}

8

GRAD Programmers' Reference Manual

7   Circle, Ellipse and Arc

7.1  Circle

Because a circle is symmetrical so we only need to calculate

the  value  for  one-eight  of  a  circle  and  by  making simple

transformation of  the value  calculated,  we  may  draw  a whole

circle.  The program for drawing circle  is  very  simple but the

deriving process is not simple. You may find the algorithm in the

book "Fundamentals of Interactive Computer Graphics"  by Foley on

page 442-446.

circle_pt(ctrx,ctry,x,y)

int ctrx,ctry,x,y;

{

dot(x+ctrx,y+ctry);

dot(y+ctrx,x+ctry);

dot(y+ctrx,-x+ctry);

dot(x+ctrx,-y+ctry);

dot(-x+ctrx,-y+ctry);

dot(-y+ctrx,-x+ctry);

dot(-y+ctrx,x+ctry);

dot(-x+ctrx,y+ctry);

}

circle(centerx,centery,radius)

int centerx,centery,radius;

{

register int x, d;

int y, incr1, incr2;

x=0;

y=radius;

incr1=6;

incr2=10-(radius<<2);

d=3-(radius<<1);

while (x < y) {

circle_pt(centerx,centery,x++,y);

if (d<0) {

d+=incr1;

incr2+=4;

} else {

d+=incr2;

incr2+=8;

y--;

}

incr1+=4;

}

if (x==y) circle_pt(centerx,centery,x,y);

}

The program below is  equivalent to the circle  above except

it is shorter but slower.

circle(centerx,centery,radius)

int centerx,centery,radius;

{

int x,y,d;

9

GRAD Programmers' Reference Manual

x=0;

y=radius;

d=3-(radius<<1);

while     (x < y) {

circle_pt(centerx,centery,x++,y);

d += (d < 0) ? (x << 2) + 6     : ((x - y--) << 2) + 10;

}

if (x==y) circle_pt(centerx,centery,x,y);

}

7.2  Ellipse

The algorithm for ellipse is the much more  complicated than

circle. The equation is derived by myself using method similar to

that of the circle. The program below is a simplified version.

ellipse_pt(ctrx,ctry,x,y)

int ctrx,ctry,x,y;

{

dot(x+ctrx,y+ctry);

dot(x+ctrx,-y+ctry);

dot(-x+ctrx,-y+ctry);

dot(-x+ctrx,y+ctry);

}

ellipse(ctrx,ctry,a,b)

int ctrx,ctry,a,b;

{

long incr1,incr2,a2,b2,d,step1,step2,ptx,pty;

int x,y;

x=0;

y=b;

a2= (long) a * a;

b2= (long) b * b;

step1 = a2 << 2;

step2 = b2 << 2;

d= ((b2 - (a2 * b)) << 1) + a2;

incr1= step2 + (b2 << 1);

incr2= step1 + step2 + (b2 << 1) - ((a2 * b) << 2);

ptx=0;

pty=a2 * y;

while (ptx < pty) {

ellipse_pt(ctrx,ctry,x++,y);

if (d<0)

d+=incr1;

else {

d+=incr2;

incr2+=step1;

y--;

pty-=a2;

}

ptx+=b2;

incr1+=step2;

incr2+=step2;

}

10

GRAD Programmers' Reference Manual

if (ptx == pty) ellipse_pt(ctrx,ctry,x,y);

y=0;

x=a;

d= ((a2 - (b2 * a)) << 1) + b2;

incr1= step1 + (a2 << 1);

incr2= step2 + step1 + (a2 << 1) - ((b2 * a) << 2);

pty=0;

ptx=b2 * x;

while (pty < ptx) {

ellipse_pt(ctrx,ctry,x,y++);

if (d<0)

d+=incr1;

else {

d+=incr2;

incr2+=step2;

x--;

ptx-=b2;

}

pty+=a2;

incr1+=step1;

incr2+=step1;

}

if (ptx==pty) ellipse_pt(ctrx,ctry,x,y);

}

7.3  Arc

Program for Arc1  and Earc1 can be obtained by making simple

modification of the program  circle and ellipse  in the preceding

sections.  Arc2  and Earc2 are much more complicated. I use table

instead of using the sine and tangent function in  order to avoid

using  floating  arithmetic.  Moreover the  situation  is further

complicated by the fact that I do  not know  the exact coordinate

of the starting  and ending  point.  There are a  lot  of special

cases in the processing.  The consequence is that I use  a lot of

time to debug  the routines.  Therefore do not try to  modify the

Arc2 and Earc2 routines unless you know what you are doing.

11

GRAD Programmers' Reference Manual

8   Text Writing

WriteStr  is  quite simple  so I will concentrate  on writec

here.

In GRAD system,  there are two types of font  can be loaded.

They are fix size  character and  variable  size  characters. The

font table and the program  are  structure  in  such  a  way that

differences in processing are kept to a minimum. This is achieved

by using pointers to pointer to the desire information instead of

direct referencing.

I am  not going to  describe  the  clipping  and  copying of

patterns to  the frame because  I  have  already  written  a more

efficient routine to do both of them at the same time. It will be

included in the next release in the near future.

12

GRAD Programmers' Reference Manual

9   Region Filling

The routine below will  extend  from the given  point to the

left until it  encounters  a pixel  which  value  is  opposite to

blankv or left edge  of the frame  is reached.  The line is drawn

using  the style given in  pattern.  blankv  can either  be  0 or

0xffff but not any other value.

xleft(x,y,blankv,pattern)

int x,y,blankv,pattern;

{

int *xyloc;

int loop;

unsigned int word, word2, temp, tempdot;

unsigned int fr_read();

xyloc=calcaddr(x,y);

word2=(word=fr_read(xyloc)) ^ blankv;

if (word2 & DOTVALUE[x & 0x0f]) return;

word2=word2 & RIGHTWORD[(x & 0x0f) + 1];

if (word2) {

tempdot=exchange(DOTVALUE[x & 0x0f]);

word2=exchange(word2);

for (loop=(x & 0x0f)+1; !(word2 & tempdot);

loop--, word2>>=1) ;

temp=LEFTWORD[loop] & RIGHTWORD[(x & 0x0f) + 1];

word2=(pattern & temp) & (word & (~temp));

fr_write(xyloc,word2);

return;

}

word2=(pattern & RIGHTWORD[(x & 0x0f)+1]) |

(word & LEFTWORD[(x & 0x0f)+1]);

fr_write(xyloc,word2);

xyloc--; x-=16;

while(!((word=fr_read(xyloc))^blankv) && (x >= 0)) {

fr_write(xyloc,pattern);

x-=16;

xyloc--;

}

if (x>=0) {

word2=exchange(word ^ blankv);

for(loop=16; !(word2 & 0x01); loop--, word2>>=1) ;

word2=(pattern & LEFTWORD[loop]) |

(word & RIGHTWORD[loop]);

fr_write(xyloc,word2);

}

}

The XHLine in GRAD system will extend in both  direction and

the coordinates of  the end points  are returned.  XHLine  is the

basis for region filling. The algorithm for SolidFill is:

1. execute XHLine using the starting coordinate.

2. if nothing can be drawn, return

3. otherwise put the left most coordinate and length on the

queue

13

GRAD Programmers' Reference Manual

4. fetch the first item on queue. the coordinates fetched is

the current location and the line starting from current

location is called the current line

5. go up 1 line (decrement y coordinate) and search in the

left direction

6. if a pixel in background state is found, XHLine is

executed and the left most coordinate and the length is

put on queue

7. the search continue until it is above the right most point

of current line.

8. go down 1 line from current location (increment y

coordinate by 1) and search to the left.

9. if a pixel in background state is found, XHLine is

executed and the left most coordinate and the length is

put on queue

10. the search continue until it is below the right most

point of current line.

11. if there is any item on queue, repeat process 4 to 11

Note that I use queue instead of  stack so  that  the region

filling routine gives a illusion of  filling in  all direction at

the  same time.  The memory for  the queue is  allocated from the

stack so it won't 'waste'  any memory when filling  functions are

not executed.

14

GRAD Programmers' Reference Manual

10  Block Copy, Save and Load

Block file  structure is  already  explained  in  the Users'

manual so  it  would not  be  repeated  here.  A  new  version of

BlockCopy, BlockSave and BlockLoad is written such that block can

be copied or loaded  to any  location.  They would be included in

the next release in the near future.

15

GRAD Programmers' Reference Manual

11  Writing Graphics Display Adapter Driver

One basic requirement for a graphics adapter  in GRAD system

is that the memory for each line must be in consecutive locations

and the most significant bit of each byte is the left  most pixel

of that  byte  when displayed.  The byte with lower  address in a

line is  displayed on the  left.  Furthermore, the number of byte

for each line must be even and the memory  in the  same line must

be in  the same  bank if the graphics  display memory  is divided

into banks.

I shall discuss the writing of graphics adapter according to

different cases.

Case I:   All  graphics  display memory  can be  accessed without

changing any page  register  or  toggling  any software

switch.

The modification  required  is quite  simple. Firstly change

the  location  defined for _SCREEN in  calcaddr.asm.  Then change

$calc  and downln  for  your  graphics  display  card.  Since the

address  return  by  $calc is a  far address so  the offset value

should be small enough such  that every  byte in the line  can be

accessed without changing the segment value.

Case II:  Graphics memory  is  divided into banks  but memory for

each line is in the same bank.

Same as Case I and then change $or,  $and and $xor such that

they  can change  to  appropriate bank  using information  in the

segment value. phyaddr routine has to be written too. JLASER is a

example for this case.

Then you may need to  rewrite some  supporting routines such

as settext and setgraph.

16

GRAD Programmers' Reference Manual

12  Writing Printer Driver

Information given in the users'  manual should be sufficient

for  writing printer drivers  for  all  printer  providing  8 bit

graphics mode.  In the section below, I shall describe the way to

write drivers for 7 bit and 24 bit graphics mode printer.

Firstly,   you  should  define  the  control   sequences  in

prtcntl.c as described in the users' manual.

For 7 bit graphics mode, change ndot in pframe.c from 8 to 7

and then change prt in prtgc.asm so that the data can be send out

properly.

If you use a  24-pin  printer,  you  need  to  do  some more

things. Firstly, change ndot to 24. You need to change the prt in

prtgc such that 3  bytes in a row are generated. Inside prt,  you

may use any registers except si and di unless you save them first

and then restore them  before return.  Furthermore, if the length

of data  graphics  are required  to  be  included  in the control

sequence,  you may have  to  modify prtpc and  optprt in pframe.c

and rmv0  in  prt.asm.  It is because the length  in  the control

sequence should be the number of columns of graphics data instead

of  the number  of  bytes.  You cannot divide the number  of data

byte by  a factor  using the standard  features.  Furthermore the

rmv0 only look for the last non-zero byte in the output data. You

need to change either  rmv0  or optprt to ensure the length  is a

multiple of 3.

After making the changes, recompile all modified files.

17

GRAD Programmers' Reference Manual

13  Hints on Porting to Other C Compiler or Other Machine

To port GRAD  to other C  compiler or other system,  I think

the most difficult part is the use far and huge  pointers and the

large number of assembly routines.

If the C compiler does not have  the far or  huge pointer or

the architecture  of  the object  machine does  not need  to have

different kinds of pointer,  the first thing you need to do is do

eliminate  all those far and huge key words and the none standard

function calls halloc and hfree.

After that  rewrite calcaddr.asm  and plottype.asm  and then

use the C routines supplied  in the documentation  to implement a

simple system first.

Rewriting  the assemble  routines will be  difficult but not

impossible.  But in the initial implementation,  you should avoid

the following functions:  Arc2, Earc2 and related portion in circ

and Ell, SolidFill, PatternFill and XHLine, Line (use the version

given in this manual instead), writec and WriteStr.

Good Luck !

18

###########################################################