Introduction

This guide is designed to help you get Backup Exec for NetWare up and running

as quickly as possible.  It is intended for administrators familiar with

installing NLMs on NetWare 3.x and 4.x fileservers.

Installation Overview

Backup Exec's installation is a five-step process:

1.  Installation of Backup Exec on the backup server.  During installation,

files required for workstation client and agent installation are also

copied to the backup server.

2.  Upgrading other NetWare fileservers that are to have backup and restore

operations performed on them with Backup Exec (applies only to

Enterprise Edition).

3.  (Optional) Modifying the AUTOEXEC.NCF file with the addition of the

BESTART.NCF file.  (The BESTART.NCF file loads the Job Manager and the

resources it requires.)

4.  Installation of the tape hardware (controller and tape drive) in the

backup server.

5.  Rebooting the server.

Before you proceed with the installation, please make sure that your server

and workstations meet Backup Exec's minimum system requirements and take a

moment to read the pre-installation notes included in this guide.

Installation Requirements

Servers and workstations on your network must meet the following requirements

to run the Backup Exec Job Manager, clients and agents.

Module                             Requirements

Job Manager and NetWare client   * NetWare version 3.1x or 4.x

386 processor with at least 2 MB of

RAM over the server's normal

operating requirements.

* Compatible tape device and

controller. (A compatibility

listing can be found in the

HARDWARE.DOC located in the directory

where you extracted the Backup Exec

installation files.)

Windows client and agent         * Microsoft Windows version 3.1

running in enhanced mode or Windows

for Workgroups version 3.11

DOS client and agent             * MS-DOS version 3.31 or later

Novell DOS version 6.0 or 7.0

OS/2 client and agent            * OS/2 version 1.3 (agent only), and

(Enterprise Edition only)          versions 2.0 or 2.1(client and agent)

Windows NT agent                 * Windows NT version 3.1

(Enterprise Edition only)        * Windows NT Server version 3.1

UNIX agent                       * SCO UNIX 3.2.4, Interactive 3.2,

(Enterprise Edition only)          Sun OS 4.1.3, UnixWare 1.0, ISC 4.2

Macintosh agent                  * Macintosh System 7.x

(Enterprise Edition only)

Pre-Installation Notes

For a trouble-free installation, please take a moment to read the following

notes.

Make sure your controller and tape device are listed in Backup Exec's

compatibility matrix (refer to the HARDWARE.DOC file located in the directory

where you extracted the Backup Exec installation files).

Check to see that the tape drive is properly connected, and be sure the SCSI

bus is addressed and terminated properly.

If you are using an ASPI-compatible controller, verify with the controller's

manufacturer that you are loading the latest drivers

(e.g., AHA1540.DSK, etc.).

Backup Exec Installation

1. From the directory where you extracted the Backup Exec installation

files, type: load [volume]:[directory]\beinstl (e.g., load

sys1:\install\beinstl).  Please take a moment to read the README file.

This document contains the latest release information about Backup Exec.

During installation, Backup Exec checks the versions of some of the

server's NetWare system files (NLMs).  Outdated versions of these files

can be updated during the installation procedure.  Updating these files

is recommended.

2. After all the files are installed, the next step is to upgrade other

servers targeted for backup and restore operations (applies only to

Enterprise Edition).

3. Next, update the AUTOEXEC.NCF file to remove any lines associated with

the outdated files listed in the following table.

Outdated NLM System Files

tsa_311.nlm     patch311.nlm

tsa_400.nlm     smdr311.nlm

clibp002.nlm    tsa_nds.nlm

tsa_dos.nlm     tsa_os2.nlm

ws_man.nlm

4. Add the BESTART command as the last line in your backup server's

AUTOEXEC.NCF file if you want to have Backup Exec's Job Manager loaded

upon server startup.  This step is optional.  BESTART.NCF is a file

that contains commands used to load the Backup Exec Job Manager.

Note: If you are using Backup Exec for NetWare - Enterprise Edition,

BESTART.NCF files are also installed on remote servers.  These files load

the necessary TSAs to allow the backup server to see the remote servers.

Make sure none of the outdated files listed in the previous table are

included in the AUTOEXEC.NCF file on remote servers. Adding the BESTART

command to AUTOEXEC.NCF files on remote servers is recommended.

5. Install the tape device and controller in the backup server (if it is

not already installed).

6. Reboot the backup server and all updated remote servers.

7. After the servers are rebooted, continue with the workstation agent and

client installation or simply load the NetWare client to submit your

first backup.  To load the NetWare client, type load be at the system ":"

prompt.

Installing Workstation Agents and Clients

Once the backup server is up and running, you can proceed with installing

workstation agents and clients.

Backup Exec Component Installation Commands

DOS client/agent

Location: Drive mapped to: [backup server]/[sys:bkupexec]\ dos_ws

Command:  install

Windows client/agent

Location: Drive mapped to: [backup server]/[sys:bkupexec]\ win_ws

Command:  setup (Use File/Run under the Windows Program Manager)

Windows NT agent (Enterprise Edition only)

Location: Drive mapped to: [backup server]/[sys:bkupexec]\ win_nt\intel

Command:  setup (Use File/Run under the Windows NT Program Manager)

OS/2 client/agent (Enterprise Edition only)

Location: Drive mapped to: [backup server]/[sys:bkupexec]\ os2_ws

Command:  install

UNIX agent (Enterprise Edition only)

Location: Backup Exec UNIX Agent Installation Diskette

Command:  From a temporary directory, type: tar xvf /dev/rfd0.  Then, run

the installation script by typing: #./INSTALL

Macintosh agent (Enterprise Edition only)

Location: Backup Exec Macintosh Agent Installation Diskette

Command:  Drag both files from the Installation diskette into the System

Folder.  Open the Backup Exec Agent Control Panel.

Upgrading From Earlier Versions of Backup Exec

If you are upgrading from Backup Exec version 4.x, make sure that you fully

install all of the components of this version of Backup Exec.  The Job

Manager, client and agent modules from Backup Exec version 4.x cannot be

used with this version of Backup Exec.

To upgrade from a previous version of Backup Exec for NetWare:

1. Install this version of Backup Exec for NetWare.  Make sure you down

and restart your server after the files are installed.

2. After the server finishes booting, load the job manager (type: BESTART)

if it is not already running.  This creates a new job queue (if one

has not yet been created).

3. Run the upgrade program by typing: LOAD BEUPGRAD.  The upgrade program

copies jobs from the old job queue to the new one.  It also modifies

any jobs that have old agents listed to reference the new agents.

4. Upgrade workstations running Backup Exec agents from a previous

version.  When upgrading these agents, you will probably want to keep the

previous  agent names and passwords.  If you change either the name or

the password on an agent, any jobs in the updated queue that reference

that agent will have to be deleted and re-submitted.