The facsimile (or just FAX, for short) mode of communications

continues to increase in use on the HF bands.  A number of

additional government and commercial stations transmit a variety

of information ranging from weather satellite to press photo's.

Something interesting to copy can be found most hours of the day

on many frequencies.

My earlier programs for the  Atari 800/800XL/130XE and the

Commodore 64 and 128 computers ( December 1985 and October 1986 73

Amateur Radio, respectively) work very well, but can receive only

one chart at a time and can not automatically save charts to disk.

One thing FAX isn't is fast. Normal FAX charts can take 10 to 15

minutes to receive.  Initially it's fun to watch the charts be

displayed on the computer but after awhile one wishes the charts

could be recorded over a period of time (say a day or more) and

reviewed at leisure later.

AUTOFAX TO THE RESCUE

My latest computer program, AUTOFAX, solves these problems.  It

provides numerous features to enhance receiving FAX signals,

including:

COMPUTER CONTROLLED UNATTENDED RECEPTION OF CHARTS.  The user

can construct a program schedule containing up to 20 steps

including date, time, disk file name, and all other parameters

associated with receiving a chart.

OPTIONAL CONTROL OF RECEIVER FREQUENCY.  If you are using a

Kenwood TS440 or R5000 receiver and connect it to the

computer's serial port as described in the article "Control

Your Rig from a PC" in the August 1989 issue of 73, AUTOFAX

will control the receive frequency as well.

FLEXIBLE FAX RECEIVE PARAMETERS.  AUTOFAX will receive FAX

signals using speeds of 60, 120, and 240 lines per minute.

Charts may be 320, 640, or 1280 pixels wide and 1600 lines long

(at 320 pixels per line).

CONVENIENT VIEWING OF CHARTS.  The received charts are viewed

on the computer's monitor.  The operator has full control over

the display by course and fine horizontal and vertical

scrolling of the display.  Both the color monitor (in medium

resolution) and the superb monochrome monitors are supported.

CHARTS SAVED TO DISK FILES.  Charts may be saved to disk files

in two different formats.  One format is compatible with the

popular drawing program DEGAS.  DEGAS will allow additional

processing of the chart including adding color, animation, and

hardcopy on numerous printers.

SIMPLE INTERFACE CIRCUIT.  A simple two IC circuit  is required

to connect your receiver to the Atari ST's serial port.  All

circuit components are easily obtained and any normal

construction technique may be used.

SIMPLE OPERATION.  AUTOFAX uses the Atari ST's MacIntosh like

mode of operation to good advantage.  Most operations are

controlled via 'pull down' menu's.  See Figure 4 for an

example.  No complicated commands need be learned.  The user

simply uses the computer's mouse to point at an option and

'click' the mouse button to select the option.

SYSTEM COMPONENTS

The components of this system are shown in the block diagram in

Figure 1 and consist of a suitable HF receiver, a tone detector,

an Atari ST computer, and the program AUTOFAX.  Also shown is an

optional RS232 to TTL converter in case you wish to have the

computer control the frequency of your Kenwood receiver.

RECEIVER REQUIREMENTS  The receiver you use should be a stable,

good quality, general coverage receiver with SSB capability.  If

your receiver provides acceptable  ease of tuning and frequency

stability for SSB voice signals, it should be suitable for

facsimile reception.  I primarily use a Kenwood TS440S

transceiver, but have also used a Sony ICF2010 receiver.

TONE DETECTOR.  The tone detector is a simple circuit that

connects between the receiver audio output and the computer's

RS232 serial port.   The circuit, shown in Figure 2, is based on

two IC's.  The NE565 phased locked loop IC functions as a simple

tone detector. The 50k potentiometer (marked 'TUNE') is used to

adjust the PLL center frequency.  The LM324 quad op-amp IC is used

as a low pass filter and level convertor to provide a 12 and 5

volt output signals.  We use the 12 volt output here to drive the

Atari ST computer.  The 5 volt output is suitable for computers

requiring a TTL level signal (Commodore 64/128, Atari

800/800XL/130XE, etc.) and is not used with the Atari ST.  The LED

is used for tuning purposes.  Note that two op-amps within the

LM324 are not used in this circuit.  They could be used for

additional filtering or other functions.

The circuit uses only simple, commonly available components.  Your

friendly local Radio Shack or other suppliers should be able to

provide all of the components.  The resistors are all 1/8 watt and

10% tolerance components.  All capacitors are ceramic disks and

are in microfarads, except for the two electrolytics (shown with

the polarity '+' marker) which are rated for 20 volts.

Since only low frequencies are used, any construction technique

may be used to put the circuit together ... a circuit board,

point-to-point, etc.

ATARI ST COMPUTER.  The Atari ST computer and the program AUTOFAX

are the heart of the system and control all aspects of its

operation.  The ATARI ST is a very capable computer system that

uses the Motorola 68000 microprocessor and may have from .5 to 4

megabytes of memory.  The ST features a mode of operation very

similar to the MacIntosh's thus it is very 'user friendly' for

this application.

All connections to the ST are via the RS232 serial port.   The

tone detector circuit's output is connected to pin 22 and ground

to pin 7. Additional pins of this connector are used if you use

the optional RS-232 to TTL converter.

RS232 TO TTL CONVERTER.  This optional circuit (see Figure 3) is

used to convert the computer's RS232 serial port's voltages to the

TTL levels required by the Kenwood TS440 transceiver and R5000

receiver.  The connections to the Atari are via pins 1 through 7

of the serial port.  Radio connections are via the 6 pin DIN

connector, ACC1.  The circuit was originally presented in the

August 1989 issue of 73 Amateur Radio.  Please refer to that

article for more information.  Note that this circuit is only

required if you wish to have the ST able to control your receive

frequency.

AUTOFAX PROGRAM OPERATION

AUTOFAX uses the graphical user interface capabilities totally to

result in a sophisticated program that does not require a PHD in

computer science to operate!  All operations are via the ST's pull

down menus.  Figures 4 to 7 are sample print outs of the ST's

screen with each of the pull down menus highlighted.  The ST's

mouse is used to point at a desired menu and operation from the

menu.  'Clicking' the mouse button will then invoke the operation.

The following is a brief discussion of major functions of the

menus.

DATA FILES.  These menu options (Figure 4) provide capabilities

related to the storage and retrieval of received FAX charts.  The

first two options (Load and Save File) are used to load and save

FAX buffer contents to and from the attached disk drive.  The FAX

buffer is 128,000 bytes long ... suitable for receiving charts up

to 1280 pixels wide and 1600 lines long! The Load and Save DEGAS

File options are used to load and save a single screens worth of a

chart to or from a disk file that is compatible with the popular

drawing program named DEGAS.  DEGAS provides the ability to touch

up charts, add color, add animation, and print a hardcopy to

dozens of supported printers.

The last three options provide disk file management utility

functions. Set Path is used to set the default disk drive to be

used for all disk file operations.  This option will be especially

useful if you happen to have a hard disk drive.

The Directory option will provide a listing of the names of the

files currently on disk, followed by the amount of free space on

the drive.

The Delete File option allows the permanent removal of a disk file

that is no longer required.

SET UP MENU (Figure 5).

The SET UP menu provides the ability to specify the lines per

minute (LPM) receiving rate, pixels per line, and other options

for the Manual FAX mode of reception. To choose the desired LPM

receive rate, simply use the mouse to pull down the SET UP menu,

then high light the desired rate (60, 120, and 240 LPM are

available), then click the left mouse button.  The next time you

pull down this menu the selected rate will have a little check

mark to its left to indicate the present rate.

In a similar manner, three chart widths are available: Normal -

640 pixels per line, Wide - 1280 pixels per line, and Narrow - 320

pixels per line.  Selection of the desired width is the same as

described above.

The Line Skip option may be used to specify how many lines are to

be skipped between lines that are displayed.  It is used to

control the aspect ratio of received charts.  Its value will vary

with the chart being received and the present width setting.  I

usually find that a skip value of 1 or 2 is correct for a normal

width.  When this option is chosen , the computer will ask you to

enter a value of 0 to 9, and then press Return.  The next time the

SET UP menu is pulled down, the skip value will be displayed.

The last SET UP menu option is the Clear Buffer option which, when

selected, will clear the memory chart buffer.  The buffer has room

to for the following lines of chart:

Receive minutes at 120 LPM vs. skip

Width     Lines      0   1   2   3   4   5   6   7   8   9

-------   -----     --- --- --- --- --- --- --- --- --- ---

Normal    1600       13  26  40  53  66  79  93 106 119 133

Wide      800         6  13  20  26  33  40  46  53  60  66

Narrow    3200       26  53  80 106 133 160 186 213 240 266

Also shown in the above table is the number of minutes of

reception that the buffer can hold for the various skip count

values.  For example, the buffer can hold over two hours of chart

using a width of narrow and a skip count of four!  This is great

for unattended monitoring a frequency to look for FAX

transmissions.

MAN. FAX MENU (Figure 6)

This menu provides various options for the control of the manual

mode of FAX reception.

The Start option will start the process of the reception of a

chart by the computer using the parameters as set up by the SET UP

menu.  The chart is stored in the memory buffer starting at the

beginning of the buffer, overwriting any existing chart.

The Stop option will halt the receive process until the next Stop

or Continue option.  While halted, the present synchronization

state and memory buffer position are maintained.

The Continue option will resume the reception of a FAX chart from

the point where the Stop option was selected.

The Sync option may be used to synchronize the computer reception

of a chart with the station transmitting the chart.  After

starting the reception during the first part of a chart, select

this option.  The computer will display the received chart and

direct you to point the mouse to the desired left margin of the

chart (usually a white vertical bar during the first portion of

each chart) and click the left mouse button.  The remainder of

this chart will be received properly centered on the computer's

display.  The Reset option may be used to reset to the top of the

memory buffer without altering the synchronization.

Finally, the View Chart option may be selected to view the

contents of the memory buffer on the computer's display.  The

chart will be displayed until the left mouse button is clicked,

which will return to the menu mode of operation.  While the chart

is being displayed, extensive options are available to control the

portion of the chart to be displayed.  The options are controlled

by the following keys:

KEYS USED          RESULTING ACTION

------------------ -------------------------------------------

Up arrow           Move up in the buffer 1/4 inch.

Down arrow         Move down in the buffer 1/4 inch.

Right arrow        Move to right in buffer 1/4 inch.

Left arrow         Move to left in buffer 1/4 inch.

Clr-Home           Move to top of current window of buffer.

Shift-Up arrow     Move to next window up in buffer.

Shift-Down arrow   Move to next window down in buffer.

Shift-Right arrow  Move to right side of a wide width buffer.

Shift-Left arrow   Move to left side of a wide width buffer.

Shift-Clr-Home     Move to top of memory buffer.

Left mouse button  Discontinue chart display.  Return to menu.

Use these chart scrolling options to select the portion of a chart

to be saved during the next Save Degas File option from the DATA

FILES menu.  The Save Degas File option will save exactly the

contents of the computer display.

AUTO FAX MENU (Figure 7)

This final menu provides the powerful, automatic FAX reception

features for which this programmed is named.  AUTOFAX is centered

around the concept of a reception schedule that may be up to 20

steps long with each step specifying the following items:

Frequency, in khz., that the attached Kenwood TS440 or R5000

receiver is to be set to.  If a receiver is not directly

connected to the computer, this is simply a memo field.

Mode of receiver operation.  The choices are U=USB, L=LSB,

A=AM.

Comment field to be used as desired.  Examples are the call and

location of the station to be received.

Date to start reception.  Date formats are xx/xx/19xx.  The

special date value of TODAY my be used to start reception

regardless of the actual date.

Time to start reception.  Time formats ar xx:xx.  The special

time format of NOW may be used to start reception regardless

of the actual time.

Minutes to record a chart.  Values may be from 001 to 999,

although the longest practical length is the length of time

that can be accommodated in the memory buffer using the

following Width, LPM, and Skip values.  See the chart presented

in the SET UP menu discussion.

Width of chart.  The values may be Normal, Narrow, or Wide.

LPM receive rate.  The values may be 60, 120, or 240.

S or desired Skip value.  The skip value can be 0 to 9.

File Name.  The disk file will be used to hold the contents of

the memory buffer after the specified number of minutes of chart

are received.

The Load Schedule and Save Schedule AUTO FAX menu options will

load and save a receive schedule to and from a disk file,

respectively.  Any number of receive schedules may be saved on

disk.

The Edit Schedule invokes a full screen editor to allow the entry

and/or edit of a the current receive schedule.  The editor

displays the schedule headings and all 20 schedule steps.  The

arrow keys are used to highlight the schedule step to be modified.

Then the right arrow will signal you wish to change this line.

Now all keyboard keys are active to allow the line to be changed

or entered.  Pressing the Return key signals that you are finished

with the line.  The Control-Insert and Control-Delete keys may be

used to insert or delete a line in the middle of a schedule.  The

schedule items are position dependent ... they must be lined up

exactly under the headings shown.  When all changes are completed,

pressing Control-Z will return to menu mode operation.

The Start Schedule will display the receive schedule and start up

the Auto FAX mode of operation.  The computer will sequentially

search through the schedule for the first entry that is in the

future and will then wait for that step's starting date and time.

The step is highlighted and the present date and time are

displayed.  If the computer is controlling your receiver's

frequency and mode, then those values are set appropriately per

the schedule step values. The computer will do each step in the

schedule, until the end is reached, where the computer will simply

continue at the top of the schedule.  The user may halt Auto FAX

operation at any time by simply entering Control-Z.

The View Files option of the AUTO FAX menu may be used to direct

the computer to sequentially step through each step of the

schedule, load in the data file holding the received FAX chart,

and display the chart until the left mouse button is clicked.

This provides a quick way to examine the charts after collection.

The Terminal Emulator provides a method to send characters

directly from your computer's keyboard to your Kenwood receiver.

This feature is handy for checking out your interface circuit or

or using other receiver remote control features that are not

supported by AUTOFAX.

Finally, the Set Date/Time option allows those of us who do not

have a built in date and time chip to set the date and time after

first running AUTOFAX.

The example receive schedule shown samples weather info from the

Pacific ocean from NPM (Pearl Harbor), the Gulf area from WLO

(Slidell, La.), Canada from CFH (Halifax, N.S.), continental US

form the Air Force Weather Service (Elkhorn, Ne.), and several

GOES satellite pictures from NAM (Norfolk, Va.)!  This entire

24 hour  schedule is conducted unattended by the computer, with

all received charts stored on disk for later viewing.

USING THE SYSTEM

Turn on your computer then double click on the file GFABASRO.PRG.

This file is the run only version of the program GFA BASIC (which

is what AUTOFAX is written in) and select file AUTOFAX.BAS when

the file selector box appears.  AUTOFAX will then load off disk

and run.

Connect the tone detector circuit to your receiver's audio output

and to the computers serial port.  Also connect the RS-232 to TTL

interface circuit to the receiver and computer if you are using

the computer control of receive frequency option.  Power up the

receiver and the tone detector and tune in a strong FAX signal

until the characteristic screech-screech sound is of medium pitch.

A moderate level of receiver audio output will be required.  Now

adjust the Tune control until the LED blinks in time with the

received audio.  Using the mouse, select the MAN. FAX menu item

and the Start option.  Now again select the MAN. FAX menu item and

select the View Chart option.  You should see the chart being

displayed horizontally from top to bottom on the computers

display.  Fine tune the receiver and/or the tone detector for the

sharpest copy.

WHERE TO TUNE

By far the best FAX signals at my location are from NAM out of

Norfolk, Va.  Current frequencies are 3357, 8080, 10865, 16410,

and 20225 khz.  Weather charts and satellite photographs of all

types are broadcast nearly continuously.

The Air Force Weather Service broadcasts a variety of weather

charts also nearly continuously from Offutt AFB at Elk Horn, Ne.

on 3231, 5096, 6904, 10576, 11120, and 19326 khz.

Coastal station WLO from Slidell, La. provides weather information

pertinent to the gulf coastal area and surrounding states.

Frequencies are 6852, 9157, and 11145 khz.

Finally, CFH from Halifax, N.S., Canada, provides charts during

the first 15 to 30 minutes of each hour on 4271, 6330, 10536, and

13520 khz.  As you might expect, their information centers on

Canada or Canadian waters.

GETTING THE PROGRAM

The program is essentially share-ware at this time.  It has been

uploaded to GENIE and is available there.  The tone detector

circuit diagram and other figures mentioned above are available

for $5 from:

Gary Sargent

4227 Willow Run Dr.

Dayton, Ohio  45430

For an extra $10 (or $15 total), I will include a 3.5 diskette

containing all of the program and other files, saving you the

download expense.

I can not accept CODs or credit cards (US funds only, please).

Personal checks or postal money orders in U.S. funds are

acceptable.

Autofax is an outgrowth of the earlier VISIFAX programs for the 8

bit Atari and the Commodore 64 computers.  Visifax packages

(software diskette, 73 Amateur Radio magazine article reprint, and

other info) is still available at the address above for $15 each.

Be sure to specify the computer.