GVP-IO Patch 1.0

================

1993 by Marc Heuler (Data Design)

marc@aargh.incubus.sub.org

LEGAL

~~~~~

This  is  freeware.   Distribute  it  whatever way you like, but care about

these rules:

Don't change anything, especially the Copyright notices must stay

intact.

Spread the program in whole (documentation included).

Especially  GVP and DTM Germany are allowed to include it with their GVP IO

Extender.

There's no warranty, use at your own risk.  If you feel necessity to donate

something to the author, just do it.

REQUIREMENTS

~~~~~~~~~~~~

Using  GVP-IO  Patch  requires  Kickstart  2.04  or  higher, and the GVP IO

Extender  board  with  software  version 1.7 installed.  It might work with

other versions, although I didn't test.

WHAT IS IT?

~~~~~~~~~~~

This  program is useful in conjunction with the GVP IO Extender board only.

The  current software (version 1.7) is lacking a nice feature, and contains

a  minor  bug.   GVP-IO Patch is intended to solve these problems.  The new

(added by this program) functionality of your GVP IO Extender board is:

CPU load during high speed transfers is decreased considerable.

Software relying on EOFMODE (such as common Voice Mail software

for ZyXEL modems) that does not work with GVP IO Extender might

work now.

INSTALLATION

~~~~~~~~~~~~

Drag  the  icon  into  your SYS:PREFS drawer.  The GVP IO Extender software

must  be  installed  correctly  (Binddrivers  command in startup-sequence),

however, it probably already is.

USAGE

~~~~~

You  can start GVP-IO Patch from Shell or Workbench.  It will open a window

with those choices:

IRQ....	This  describes  after  how  many  bytes received the board

should  issue  an  Interrupt  Request to the computer.  The

FIFO  buffer  can hold up to 16 bytes - if it is filled up,

further   data  will  be  lost.   On  the  other  hand,  if

interrupts  occur  often (one interrupt per one or very few

bytes), the CPU is quite busy answering all those requests.

GVP  IO  Extender software is delivered with this value set

to  1.   An ideal value is around 8, because it reduces CPU

load  during  high  speed  transfers significant, but still

offers an 8 byte safety margin to not lose any data.

You can choose between 1 and 8.

EOF....	The  EOFMODE  is  used  by  some  not  very common software

packages such as many Voice Mail programs for ZyXEL modems.

The   GVP  IO  Extender  software  rejects  SDCMP_SETPARAMS

requests with EOFMODE set to ON.  This causes some programs

to fail.

If you set this option to YES, the software described might

work  better with your GVP IO Extender board.  Set it to NO

for default behaviour, although I don't recommend this.

SAVE...	Save your changes permanently.

USE....	Save your changes temporarily (until you reboot the Amiga).

CANCEL.	Exit the program without patching.

In the menu strip, it'll present those functions:

ABOUT..................	Show information about GVP-IO Patch.

QUIT...................	Exit the program without patching.

RESET TO DEFAULTS......	Load default values, just like a brand-new

GVP IO Extender board comes with.

LAST SAVED.............	Re-load the last configuration you saved.

RESTORE................	Restores the settings as they were before

you began to play around with the gadgets.

If  the  program  can't identify the GVP IO Extender software on disk or in

memory,  it  will  complain.   If  it  does  not, you can be pretty sure it

patches  correctly,  even if you use another software version than 1.7 (but

be careful though).

THANKS

~~~~~~

Thanks  to Ralph Babel (maker of GVP IO Extender software) who explained me

where to actually patch the software.

AUTHOR

~~~~~~

To contact me for donating or some other reason, here's my address:

Mail:	Marc Heuler

Stephanstr. 6

97070 Wuerzburg

GERMANY

Phone:	+49 931-53295 (voice)

EMail:	marc@aargh.incubus.sub.org