giga.device, version 1.71, 26.6.1995, Copyright by Christoph Dworzak

--------------------------------------------------------------------

(I don't like to write docs, so this will be short...)

Introduction

------------

Some time ago, I bought a Seagate Elite Harddisk with 9GB (actually

it has only ~8.5GB, but MarketingPeople tend to _miscalculate_ a

little bit. In this Document one GigaByte equals 2^32 Byte).

After connecting this Drive, I started using it, but I got some

strange things going. Some data disappeared, a lot of files became

corrupt....

Some closer examinations revealed that the AmigaOS can only handle

Drives up to 4GB.

My pride as a ComputerFreak did not allow me to give up, until I

could fully use this drive :-) So I wrote giga.device.

Whats this 4GB-limit?

---------------------

The Amiga uses device-drivers to access the Hardware. One of these

dirvers is scsi.device (other Manufacturers than Commodore call it

differently, e.g. warpdrive.device, oktagon.device,....).

Now to access this scsi.device a program has to specify what to

read or write. And exactly there is the problem.

The Location is defined as a ByteOffset to de beginning of the

drive. This Offset is a ulong, that means the largest Offset is

2^32 Byte. So you can not access more than 4GB.

It doesn't matter if you make many Pertitions or not. This Offset

is on the device-level.

Also a new FileSystem cannot change this (except doing something

like giga.device does).

Amiga Technologies (Commodore) should define new calls with 64

Bit_Pointer as soon as possible. Until this happens you may live

with giga.device.

There is another thing to say: In the early days of the AmigaOS,

this Offset was defined as a long. That would allow +/- 2GB (but

whats a negative HarddiskSize anyway?).

So some programs, which use this old definition, have some troubles

with sizes above 2GB.

For example, see how "info" works...

******************************************************************

Ram Disk:T> info Big0:

Mounted disks:

Unit      Size    Used    Free Full Errs   Status   Name

E10:    -2049K 8310650   73860  99%   0  Read/Write BIG0

Volumes available:

BIG0 [Mounted]

Ram Disk:T>

******************************************************************

The Partition E10 is 4GB big... (but it still works, it just looks

a _little_ strange :-)

How does it work?

-----------------

giga.device _sits on top_ of the scsi.device. It divides the Harddisk

into several smaller Disks.

Instead of a 9GB Harddisk, I have now two Disks with 4GB each and one

drive with the rest.

Above I told you, that it is not possible to access more than 4GB,

but now i tell you that giga.device does exactly this?

Now, it bypasses the scsi.device by using scsi-direct-calls. I am

not going to explain this. You just have to make shure, that your

scsi.device understand these calls.

I don't know if every scsi.device is compatible with giga.device.

So far I tested only the scsi.device from Commodore on a A3000T

and the warpdrive.device from the Warpengine on a A4000.

Installation

------------

-First of all make a Backup of all your data. As you know Murphy,

something _will_ go wrong...

-For the demo-version you have to set the big Harddisk as Unit 0!

In the registered Version, you may choose any Unitnumber you want.

(Question to the professionals out there: how do I pass Parameters

to a device?)

-Copy giga.device into devs:

-Then add the following ToolType to the HDToolBox (DeviceNames are

case sensitive!):

SCSI_DEVICE_NAME=giga.device

-Then start HDToolBox. Now you should see some new drives. You may

make Partitions exactly the same as always (rtfm).

There is one thing to know: Unit 0 from giga.device is the same

as you would see with the scsi.device, just limited in size.

As giga.device is not loaded at startup, you cannot boot from giga-

device. But you can still boot from scsi.device.

(ok, I know, it's quiet confuseing. Just try it).

To mount the drive, you may use something like ScsiMounter

(AmiNet:DISK/MISC/SCSIMOUNTER203.LHA by Martin A. Blatter).

My S:User-Startup looks something like this:

******************************************************************

scsimounter >nil: d elite0.device 1

scsimounter >nil: d elite0.device 2

******************************************************************

You don't have to mount Unit 0, as this was already mounted at

startup via scsi.device.

Shareware

---------

To get your own unlimited giga.device you have to pay me something

(don't tell me, that you are a _poor student_. You just bought a

drive with more than 4GB :-)

The demo-version of giga.device is freely distributable. You may use

it as long as you want (but i still like to hear about bugs and so).

I suggest 5$/GB. But as this is the first release of giga.device,

I am open for discussions.

To get a registered Version you have to fill in the following form

(you may write more, just in case I forgot something):

******************************************************************

GigaDeviceRegistration

----------------------

Name:

Address:

E-Mail:

ScsiName (scsi.device, warpdrive.device, oktagon.device,...):

ScsiUnit (0-7):

HarddiskSize (in GB):

Max. simulated HarddiskSize you like to have (2GB should work

everywere. 4GB may give some strange results, but allows for

bigger Partitions...):

How did you pay? (cash,...):

How do you get yor giga.device (eMail, snailMail,...? Please

add appropriate postage):

******************************************************************

Legals

------

I hate these things...

giga.device is copyright 1994-1995 by Christoph Dworzak

The demo-version with the 6GB-Limit is free.

Any other Version have to be paid to me and may only be used on one

Computer.

The software is provided as is. There is no warranty.

I _may_ correct Bugs or make Enhancements, but I don't have to!

(If you ask nicely, I will probably do it. but my time is limited,

so it may take a while... :-)#

If any Distributor likes to sell giga.device -> contact me. I am quiet

shure that we will find an agreement.

Author

------

Christoph Dworzak

Molzgasse 10

2502 Biel

Switzerland

Tel.     ++41-32-501911

BBS/Fax  ++41-32-501915

BBS/ISDN ++41-32-501919

eMail: dworz@link-ch1.limmat.net.ch

SwissAmiNet: ZyXEL 44:8010/409.0

ISDN  44:8010/410.0

AmigaNet:          39:110/409.0

PC-Konto (Swiss PTT): 30-96030-7

Bank: Swiss Bank Corporation

48-106,203.0

Bye

dworz

PS: Thats probably the longest Doc I ever wrote. If anybody likes to do it

better -> email me. You may get giga.device in Retrun for your work.