TTY: General Serial Port Management Library rev 1.0

(c) Francois Bergeon 1987

This set of function allow MS-C programmers to manage serial ports

thru a buffered input/IRQ method. The header tty.h has to be included

in the program source code. Two 2048 bytes buffers are reserved for

COM1 and COM2.

NOTE: The <fd> value (return value from a tty_open call) is an

internal index. It cannot be used as an argument to standard

library functions such as lseek(), fstat(), etc...

GENERAL FUNCTIONS

-----------------

int tty_open(comport, param)

int tty_write(fd, buffer, lenght)

int tty_read(fd, buffer, lenght)

int tty_eof(fd)

int tty_close(fd)

LOW LEVEL FUNCTIONS

-------------------

void _combrk(fd)

void _comflush(fd)

TTY_OPEN

SYNOPSYS:

#include <tty.h>

int tty_open(comport, param)

char *comport;

int param;

DESCRIPTION:

Open <comport> an initializes it with <param>. Return port descriptor,

or -1 if <comport> is not valid (errno is set to EBADF).

ARGUMENTS:

<comport> is either "COM1" or "COM2". <param> is the initialization

parameter:

B0              Hang-up the line by resetting DTR

B110              110 bauds

B150              150 bauds

B300              300 bauds

B600              600 bauds

B1200            1200 bauds

B2400            2400 bauds

B4800            4800 bauds

B9600            9600 bauds

B19200          19200 bauds

ORed with

CS7             7 bits

CS8             8 bits

eventually ORed with

CSTOPB          2 stop bits (default is 1)

eventually ORed with

PARENB          EVEN parity

PARODD          ODD  parity (default is NONE)

EXAMPLE:

if ((fd = tty_open("COM1", B1200 | CS7 | PARENB)) < 0)

fprintf(stderr, "Error on COM1");

NOTE:

to hang-up line and close COMx port you can use:

tty_close(tty_open("COMx", B0));

TTY_WRITE

SYNOPSYS:

int tty_write(fd, buffer, lenght)

int fd;

char *buffer;

int lenght;

DESCRIPTION:

Write <lenght> bytes from <buffer> to <fd>. <fd> is the return value

of a previously issued tty_open call. Return the number of bytes sent,

or -1 if <fd> is not valid (errno is set to EBADF).

EXAMPLE:

tty_write(fd, &c, 1);

TTY_EOF

SYNOPSYS:

int tty_eof(fd)

int fd;

DESCRIPTION:

<fd> is the return value of a previously issued tty_open call. Return

-1 if <fd> is not valid (errno is set to EBADF), 1 if no characters

are available, otherwise 0.

EXAMPLE:

if (tty_eof(fd))

tty_read(fd, &c, 1);

TTY_READ

SYNOPSYS:

int tty_read(fd, buffer, lenght)

int fd;

char *buffer;

int lenght;

DESCRIPTION:

Read <lenght> bytes from <fd> to <buffer>. <fd> is the  return value

of a previously issued tty_open call. Return the number of bytes received,

or -1 if <fd> is not valid (errno is set to EBADF), 0 if no characters

are available.

EXAMPLE:

#include <time.h>

.

.

.

time_t start;

.

.

.

time(start);

while ((bytes_received = tty_write(fd, buffer, lenght)) == 0 &&

(int)difftime(time(NULL), start) < timeout)

{;}

TTY_CLOSE

SYNOPSYS:

int tty_close(fd)

int fd;

DESCRIPTION:

Close <fd>. <fd> is the return value of a previously issued tty_open

call. When exiting the program, all COM ports must be closed. Return

-1 if <fd> is not valid (errno is set to EBADF)

EXAMPLE:

tty_close(fd);

_COMBRK

SYNOPSYS:

_combrk(fd)

int fd;

DESCRIPTION:

Send a BREAK to <fd>. <fd> is the  return value of a previously

issued tty_open call. No checks are made on the integrity of <fd>.

EXAMPLE:

_combrk(fd);

_COMFLUSH

SYNOPSYS:

_comflush(fd)

int fd;

DESCRIPTION:

Flush <fd> input buffer. <fd> is the  return value of a previously

issued tty_open call. No checks are made on the integrity of <fd>.

EXAMPLE:

_comflush(fd);

######################################################################