Just a short note for those wanting to build this.

The Amiga horizontal and vertical sync lines are used as inputs when

interfacing a genlock. If these lines are loaded too much, the amiga

will think that it has a genlock connected, and won't provide sync

signals. By buffering these lines, you will be able to hook up any

monitor without problems. I chose a CMOS schmitt-trigger inverter

to buffer these lines to give the signals a nice sharp rise & fall

time, as well as providing for sync inversion if necessary.

The VGA interface is somewhat vague about which pins are used as grounds.

Different manufacturers use different pins, however, they all use pins

6, 7 & 8 (return grounds for Red, Green, & Blue). The diagram provided

will work on 99% of the monitors out there without problems, and should

be used first. Otherwise consult the pinouts for your particular monitor.

When I built my interface, I put the 74HCT14 in the hood for the DB-23.

There was more than enough space for the IC mounted on a piece of perf-

board along with the wires & capacitor. If you do not have a 74HCT14,

almost any buffer or gate will do, but best results will be had with this

chip.  It is best to run all the ground wires seperately between the two

connectors, but it won't hurt it to combime them if you do not have enough

conductors in the cable you are using.  Be sure to double check your

connections!!!  If you wire this up incorrectly, you could permanently

damage your Amiga. Good Luck.

-Mark Spankus

(Mark @ Shorty.CS.wisc.edu)