N E U T E X E C

Technical character generator for CP/M PLUS and Epson-compatible

dot-matrix printers

Version 1.0

USER MANUAL

(C) Copyright Walter D. Neumann, 1986

All rights reserved

Walter D. Neumann

1F Southway

Greenbelt MD 20770

(301) 345-7017

USER SUPPORTED SOFTWARE:  SUGGESTED CONTRIBUTION $20

SEE APPENDIX FOR DETAILS

NEUTEXEC USER GUIDE  Page 1

1.   General description of the program.

NEUTEXEC is also available for IBM PC compatible computers under

the name NEUTECH.

NEUTEXEC is a memory-resident technical character generator for

CP/M Plus computers and for Epson printers or compatible equipment

(IBM Graphics printer etc.).  It is designed to be invisible to

applications that do not specifically try to use it.  Its effect is to

add a set of technical or other user-defined symbols to the set of fonts

of your printer and to make them easily accessible to application

programs (word-processors, etc.) which do wish to use them.

Individual characters or complete character sets can also be

"downloaded" to the program from the document being printed or from the

operating system.

The built-in character set consists of a Greek alphabet plus a set

of Gothic capitals plus a set of mathematical symbols, each one in a

regular (pica) size and in a compressed size.  This character set can be

replaced (in whole or part) by the user.  NEUTEXEC keeps track of whether

the printer is printing in compressed and/or wide mode and adjusts its

characters accordingly.  Several copies of NEUTEXEC with different symbol

sets can be loaded and accessed at once.  Each copy takes up about 3K of

memory.

The NEUTEXEC character set can be printed for reference by installing

NEUTEXEC and printing the file NEUCHARS.DOC.

NEUTEXEC works by watching the character stream sent to the printer

for its own one-character "attention command."  When it sees its

attention command, it replaces the next character sent to the printer by

the corresponding character from an internal alphabet, using graphics

mode.  A secondary one-character command is used to initiate downloading

a character to NEUTEXEC.  The attention command and secondary command

both default to ^Q ("control-Q" or ASCII 11h) but can be changed by the

user (see section 2; this is necessary for using multiple copies of

NEUTEXEC or using it with programs that cannot send a "^Q" to the

printer).  Thus, using the defaults, "^Qa" would print the NEUTEXEC

character corresponding to "a" (a greek "alpha") and "^Q^Qa" initiates

downloading a new character to NEUTEXEC to be printed by "^Qa".

NEUTEXEC must keep track of all printer commands being sent, since

if the NEUTEXEC attention command occurs as part of an Epson printer

command, it must be ignored by NEUTEXEC.  For this reason the program is

relatively printer-specific.  It is designed to work with any printer

that uses the Epson FX80 printer protocol or a subset of it (e.g.

earlier Epson printers, the IBM graphics printer, Centronics GLP, and

many others).  NEUTEXEC will work with any printer that uses Epson

graphics commands, so long as the software never uses a non-FX printer

command containing a NEUTEXEC command character.

In the following, "^a" and "^s" will denote the NEUTEXEC attention

command character and secondary command character respectively.  As

already described, the default is ^a = ^s = ^Q (ASCII 11h).

NEUTEXEC USER GUIDE  Page 2

2.   Loading and unloading the program.

NEUTEXEC is loaded into memory by running the program NEUTEXEC.EXE

(enter "NEUTEXEC" at the CP/M command prompt, or include it in your

PROFILE.SUB file).  Once loaded, it remains resident in memory, taking

about 3K of memory space and can be unloaded only by rebooting.

The two NEUTEXEC command characters can be set to other values than

the default by adding a parameter on the command line when loading

NEUTEXEC .  The format is "NEUTEXEC XXYY" where:

XX is the hex representation of the NEUTEXEC "attention character"

^a (default is XX = 11);

YY is the hex representation of the NEUTEXEC "secondary command

character" ^s (default is YY = 11).

NEUTEXEC will accept ANY value of XX and YY for ^a and ^s; if ^a is

also a printer command it will then always be intercepted by NEUTEXEC and

will be unavailable to the printer, so changes to the default should be

chosen with care (see Section 7).

3.   Accessing NEUTEXEC.

Once NEUTEXEC is loaded, printing "^ax" will print the current

NEUTEXEC character corresponding to "x".  Here "x" represents any

printable character other than <space>, ^a, or ^s.  (If neither ^a or ^s

has been installed to a printable character, then only <space> is

excluded).

The character printed by "^ax" will be compressed and/or wide

according as the the printer is in compressed and/or wide mode.

(At present NEUTEXEC only changes character width on seeing one of

the width-change commands that is common to all Epson printers, namely

one of the 1-character commands or the <esc>W commands.  Most software

uses only these, to ensure compatibility.  The FX and later printers

have an additional command which should be used only if constant

NEUTEXEC character width is desired).

Printing "^a^sx" followed by a sequence of 12 hex bytes (7 bytes in

compressed mode) will download the character represented by this byte-

sequence to the NEUTEXEC character corresponding to "x"; see Section 4

for details.  Again, "x" should be a printable character other than a

space, ^a, or ^s.

NEUTEXEC USER GUIDE  Page 3

4.   Downloading to Neutech

Printing "^a^sx" followed by a sequence of 12 hex bytes will send

nothing to the printer but will download the character represented by

this sequence to the NEUTEXEC character corresponding to "x".  For

example, when using the defaults ^a = ^Q and ^s = ^Q, then printing

"^Q^Q/,FF,81,02,04,08,10,08,04,02,81,FF,00"

would download an extra-large "W" to the NEUTEXEC character corresponding

to "/".  Characters other than 0,1,2,3,4,5,6,7,8,9,A,B,C,D,E,F are

ignored until all 12 hex bytes have been received, so puncuation and

comments can be included for clarity.  Thus the following download

commands have the same effect as the above example:

"^Q^Q/FF810204081008040281FF00"

"^Q^Q/: large W FF810204081008040281FF00" .

If NEUTEXEC is in compressed mode, then the character being

downloaded will be sent to NEUTEXEC's compressed symbol set, so only 7

hex bytes should be included in the command.

See your printer manual for an explanation of how a sequence of

bytes is interpreted as a sequence of vertical columns of eight dot

positions each to form a character.

5.   Installing user fonts.

A file of "^a^sx" download commands can be printed at any time to

change NEUTEXEC's character set.  For example, a command

PIP LST:=<filename>

can be included in a PROFILE.SUB file to change NEUTEXEC's default set

on start-up.  Two files, "NEUCHREG.QQ" and "NEUCHCMP.QQ," which can be

printed to restore the regular and compressed default character sets in

a default installation of NEUTEXEC, are included and can be edited as

desired.  They can also be combined into one file which restores both

character sets at once.

6.   Using multiple copies of NEUTEXEC.

Several copies of NEUTEXEC can be loaded and used simultaneously

with one printer by defining different attention characters for each of

them.  Each can be loaded with a different symbol set as described in

Section 5.

Two (or more) copies of NEUTEXEC can also share the SAME attention

character if the secondary command ^s is NOT equal to ^a.  In this case,

the copy loaded last acts as usual, but printing "^a^a" sends a single

"^a" to the previously installed NEUTEXEC which will therefore act as if

"^a^a" is its attention command (if there are three copies then the

first loaded one would have attention command "^a^a^a^a", and so on).

NEUTEXEC USER GUIDE  Page 4

This technique is particularly useful if NEUTEXEC is to be used with

a word-processor or other user program which limits the number of

different non-printable characters that can be embedded in text to be

printed.  A typical installation of two copies of NEUTEXEC might use ^a =

^Q and ^s = | (ASCII 7Ch) in both.  Then the character corresponding to

"|" is unavailable in both copies of NEUTEXEC, but the only non-printing

symbol that need be sent by user software is "^Q".  If user software

cannot send any non-printing symbols, one might replace "^Q" by "\"

(ASCII 5Ch) in this installation.

7.   Recommended installation.

It is recommended that the default installation be used unless

there is good reason to do otherwise.  As already pointed out,

installing a value for ^a which is a printer command (e.g. ^a = <esc> =

1Bh, ^a = <SI> = 0Fh, etc.) will cause problems.  Choosing a printable

value for ^a is necessary if your software cannot send non-printing

characters in text, but it disables the corresponding NEUTEXEC character

(the character ^a itself can still be sent to the printer by using

"^a^a", unless ^a = ^s).  If only one copy of NEUTEXEC is used at once,

then any choice of ^s is safe, but choosing a printable value for ^s

disables the corresponding NEUTEXEC character.  If more than one copy of

NEUTEXEC will be used at once, then ^s should NOT be a printer command

since this would confuse the subsequently installed copies of NEUTEXEC.

The default ^Q for ^a was chosen for two reasons:

(i)  ^Q is the first of the Wordstar "user commands" and thus gives

particularly easy compatibility with Wordstar.  Install the ^Q user

command (USER1) in Wordstar to send a ^Q to the printer.  ASCII files

containing ^Q commands will then be compatible between Wordstar and

other print utilities (e.g. "PIP LST:=<filename>").  Wordstar counts ^Q

as being of zero length, so line-length count and formatting by Wordstar

remains correct.

(ii)  ^Q  IS  in fact a printer command on later Epson printers than

the MX -- it is the "Printer enable" command.  Since there is no reason

ever to DISable the printer, this command is never needed and never used

by standard software.  Since it is unlikely to be used for other

purposes in future printer upgrades, future compatibility is probable.

Currently unused characters such as ASCII 00h to 06h and 15h to 19h are

also possible choices for ^a but have less certain future compatibility.

8.   Using NEUTEXEC with Wordstar.

I originally wrote NEUTEXEC for my own use with Wordstar.  I was

encouraged by friends who found it useful to distribute it more widely.

The following is the installation of Wordstar which I use with NEUTEXEC

and an Epson MX80 or a Centronics GLP.  This installation may be done

from the standard menus in Micropro's installation program Winstall

which comes with Wordstar.  A simpler installation, if your printer

allows it, is to install Wordstar to use reverse and forward half line

feeds to enable sub- and superscripting.

NEUTEXEC USER GUIDE  Page 5

I install user key ^Q to send a ^Q (ASCII 11 hex or 17 decimal),

the user characters ^W to do "wide print to end of line" (ASCII 0Eh), ^E

as Escape (1Bh), ^R as 00h.  I install the width commands ^A and ^N to

start and end compressed mode (0Fh and 12h) and the ribbon color toggle

^Y to start and end italics (1Bh,04h and 1Bh,05h respectively).

Finally, Wordstar's sub- and super-scripts are enabled by installing

half line feed as 0Dh,0Ah and full line feed as 0Dh,0Ah,0Dh,0Ah and

installing the printer initialisation sequence of 1Bh,41h,06h to set

half line feed at 1/12 inches.  A termination sequence of 1Bh,40h should

then be installed to reset the printer after use.  I double space normal

text and use single spacing for special effects.

This installation permits sending most Epson commands to the

printer.  The one I use most often is ^EE to start emphasized print.

Line height can be changed to some extent using ^E3-commands;  for

example, with the above installation of ^A and ^N, printing

"^A^E3^N^A^N" returns to the default line height of 1/12 inch per half

line (the extra ^A's and ^N are needed to make sure Wordstar sends the

"^N" after the "3";  Wordstar is smart enough not to act on a normal

width command "^N" if it thinks it is not in alternate width).  Epson's

built in sub- and super-scripts have vertical alignment problems on

early Epson printers and are superfluous with this set-up, though they

are available for use if desired (e.g. ^ES^R for subscripts).

Wordstar is clever at knowing how to handle control characters in

conjunction with sub- and super-scripting but it has trouble with

backspacing at the same time.  This is not often a problem, but it will

surface if you want simultaneous sub- and super-scripts using special

characters (e.g. to create a large integral sign using the NEUTEXEC

characters for ;, |, and :).  However, it can be avoided in several

ways, for instance by installing "overprinting" in the installation menu

to use backspacing (08h -- not available on the IBM graphics printer).

Wordstar then uses backspacing also for underlining and boldface, which

is a bit slow, but no matter.  The Epson MX has a bug:  it cannot

backspace back over anything printed in graphics mode.  Again rarely a

problem, and usually avoidable by interchanging the order of characters

being overprinted.  This bug appears to be fixed in later printers.

If you have modified Wordstar with ANYCHAR -- a modification to

allow sending any code to the printer, then ^R must be set to send FFh.

9.   Error handling.

NEUTEXEC makes certain assumptions to recover from user errors:  if

"^a" or "^a^s" precedes a non-printing character, then the "^a" or

"^a^s" is ignored; the same is true if "^a" or "^a^s" precedes a "^a"

(unless ^a = ^s).

If NEUTEXEC appears to behave erratically, check the file that you

are printing:  the most common cause is an accidental "^a^sx", making

NEUTEXEC swallow everything until the download it thinks you wanted is

completed.  For example, editing in Wordstar with control-character

display off can put control characters in unusual places: make sure

control-character display is on if you are inserting or deleting text or

doing a search-and-replace.

NEUTEXEC USER GUIDE  Page 6

10.  Incompatibilities.

Some Epson printers with NLQ capability seem unable to correctly

mix NLQ printing and graphics on one line.  Since NEUTEXEC prints in

graphics mode, NEUTEXEC characters cannot mix with NLQ mode in such

cases.  The Centronics GLP does not have this problem.

If a memory-resident print spooler is used, it should be loaded

after NEUTEXEC.  This is for two reasons:  it optimizes use of the

spooler's buffer, and some spoolers will not work correctly if any other

memory resident program which captures the printer interrupt is loaded

over them.

No incompatibility of NEUTEXEC with other software is known to date;

if you find any, please inform me, and I will try to fix it.  However,

programs such as Microsoft Basic (CP/M version), which do not use standard

operating system calls to access the printer, will bypass NEUTEXEC.

11.  Design considerations.

NEUTEXEC is designed for printing occasional special characters

in regular text.  It puts the printer into graphics mode for each

such character.  If several special characters occur in sequence,

this is much slower than if NEUTEXEC buffered them and then printed

them all at once in graphics mode.  A buffered version will be made

available if there is sufficient demand.

NEUTEXEC intentionally does not use the download capability of the

FX and other Epson printers.  Such use would make NEUTEXEC incompatible

with other programs which simultaniously use this feature.  Moreover,

downloaded characters have marginally lower quality than NEUTEXEC

characters, since they may not have adjacent horizontal dots, and they

are lost if the printer is turned off.

NEUTEXEC USER GUIDE  Page 7

APPENDIX: USER SUPPORTED SOFTWARE

Permission to copy:

Individuals may copy this software and documentation for their own

use or to share with others, so long as no price is charged.  Computer

clubs and other non-profit organizations may copy this software and

documentation and share it with their members so long the software and

documentation are distributed unmodified and together, no price is

charged other than cost of distribution, and members are encouraged to

support the user-supported concept with their donations.

If you find this program useful you are encouraged to make a $20

contribution.  You will thereby become a registered user.  This will

entitle you to notice of updates and other information.  Moreover, if enough

users contribute useful NEUTEXEC character sets, I will distribute them

to registered users for a small charge to cover materials, handling, and

modest honorariums to the contributors.

If you send a character set, please do so in the format of the

files NEUCHREG.QQ and NEUCHCMP.QQ and include a printout of the file

NEUCHARS.DOC with your characters installed.

Disclaimer:

In no event will the author be liable to you for any damages of any

kind arising out of the use or inability to use this program.

The original version of NEUTEXEC is a MS-DOS program called NEUTECH

designed for the IBM PC and compatibles.  It is available on request.

Walter Neumann                                     January, 1986

1F Southway

Greenbelt,  MD 20770

(301) 345-7017

##############################################