18E USER'S DOCUMENTATION

Copyright 1988

by

Damon Gibson

8201 Castleton Boulevard

Indianapolis, Indiana  46256

(317) 842-7128

TABLE OF CONTENTS

Section                                        Page

I.  INTRODUCTION                                       1

II.  FEATURES AND FIXES                                 1

III.  INSTALLATION                                       2

IV.  NEW OR MODIFIED COMMANDS                           3

A       Assemble                                 4

J       Animated Full Screen Debugging           4

S       Single Step                              4

X       Display Machine State                    4

Z       Disassemble                              4

N       Output to I/O Port without Pre-read      4

Q       Query I/O Ports                          4

T       Select Debug Terminal (new command)      5

V.  APPENDICES                                         6

APPENDIX A - Hitachi HD64180 Instructions        7

APPENDIX B - System Memory Map                   8

APPENDIX C - 18E Command Summary                 9

18E - Z80/HD64180 DEBUG MONITOR                    Damon R. Gibson

I.  INTRODUCTION

18E is  an outstanding debug  tool for use  in developing software

for Zilog Z80 and Hitachi HD64180 microprocessors.

This  document describes the feature, installation and operational

differences between 18E  and Z8E and  is meant as  a supplement to

Z8E.DOC which very adeptly defines the Z8E debug monitor.

The  original  monitor  (Z8E)  and  its  derivative(s)  (18E)  are

available in the public domain in source and object code forms and

may  be downloaded from a number of Z-node bulletin board systems.

These software packages may be freely used and distributed for any

non-commercial purpose.

If you find that 18E saves you so much time that you don't see how

you ever got along  without  it,  we  would  appreciate  your  $50

donation (or less if  you're  poor  like  me)  for  its  continued

support (feel free to send  your  upgrade  suggestions  with  your

check).

Contributions suggested to:

Developer of 18E                Original author of Z8E

=======================         ======================

Damon R. Gibson                 Richard Surwillo

8201 Castleton Blvd.            330 Calvin Ct.

Indianapolis, IN  46256         Wycoff, NJ  07481

II.  FEATURES AND FIXES:

In addition to all  of the  capabilities of  the Z8E  monitor, 18E

provides the following new features and fixes:

- HD64180  assembly/dissambly support via modified

A, J, S, X and Z commands

- HD64180 I/O register access support via modified

N and Q commands

- Auxiliary debug terminal support (accessable via

the  new  T  command)  to  facilitate  debugging

software which makes use of the console keyboard

and display

- May use  RUBOUT  as well as  BACKSPACE  for line

entry character delete

- Fixed  cursor address  bug which  occurred while

tracing  an EXX or EX AF,AF' instruction via the

J command

With both HD64180  and auxiliary  terminal support  activated (see

installation  section below), the 18E  monitor still occupies less

than 9.5K bytes.

1

18E - Z80/HD64180 DEBUG MONITOR                    Damon R. Gibson

III.  INSTALLATION:

Installing 18E into your CP/M (or ZRDOS) system is very similar to

doing so with Z8E as described in Z8E.DOC.

First tell 18E to load itself and its symbol file as follows:

A>18E 18E.COM 18E.SYM

The  symbol file  18E.SYM contains  the name  and address  of each

parameter which may need to be modified for your system.

The CASE, MAXLEN and INIT  symbols  correspond  exactly  to  those

included   with  the  Z8E  monitor  (in  Z8E.SYM)  and  should  be

configured according to the instuctions for them in Z8E.DOC.

However, since 18E supports two terminals, there are two terminal-

related symbols in 18E.SYM for each one in Z8E.SYM as shown in the

following chart:

Z8E Symbol  |  18E Symbol     |  18E Symbol

|  Main Terminal  |  Auxiliary Terminal

--------------+-----------------+----------------------

MXYCP       |  MXYSTR         |  AXYSTR

ROWB4?      |  MROWB4         |  AROWB4

ROW         |  MROW           |  AROW

COLUMN      |  MCOL           |  ACOL

TTYQ        |  MTTYQ          |  ATTYQ

TTYI        |  MTTYI          |  ATTYI

TTYO        |  MTTYO          |  ATTYO

XYCP        |  MXYPRG         |  AXYPRG

To configure 18E for your  main  and  auxiliary  terminal,  follow

Z8E.DOC's  instructions for  configuring the  symbols in  column 1

above while substituting the symbols in columns 2 and 3 above.

The default main  terminal  configuration  uses  Soroc,  Wyse  and

Televideo compatable  cursor  addressing  and  accesses  the  CP/M

console device via BDOS calls (just like Z8E).

The   default  auxiliary  terminal  configuration  uses  DEC  VT52

compatable cursor addressing and  accesses  the  CP/M  reader  and

punch  devices via BDOS calls.  Note since CP/M makes no provision

for  determining the status  of the reader  device without waiting

forever for a  character, ATTYQ  is written  to read  the reader's

status  port directly and defaults to serial port 0 of the HD64180

(this is the reader port for the SB180).

In  addition to all this object level patching, 18E is implemented

at  the source code level to allow the user to optionally activate

or deactivate the two new  major  features  (Hitachi  HD64180  and

auxiliary terminal support).  This allows the user to tailor 18E

for his needs while minimizing its size (and thus maximizing the

available TPA space).  To do this, the user should obtain a copy

of 18E.Z80 from the 18E-SRC.LBR library file.

2

18E - Z80/HD64180 DEBUG MONITOR                    Damon R. Gibson

IV.  NEW OR MODIFIED COMMANDS:

All  18E  commands work just  as  defined  in  Z8E.DOC  except  as

described in the following pages:

3

18E - Z80/HD64180 DEBUG MONITOR                    Damon R. Gibson

A       Assemble

__________________________________________________________________

18E's A command performs just as it does in Z8E except that it now

accepts  Hitachi HD64180-specific  instructions from  the operator

for assembly (see Appendix B for a list of these instructions).

J       Animated Full Screen Debugging

S       Single Step

X       Display Machine State

Z       Disassemble

__________________________________________________________________

18E's J, S, X and Z commands perform just as they do in Z8E except

that  they now  successfully disassemble  Hitachi HD64180-specific

instructions (see Appendix B for a list of these instructions).

N       Output to I/O Port without Pre-read

Q       Query I/O Ports

__________________________________________________________________

18E's N and Q commands perform just as they do in Z8E except that

they now support access to the HD64180's internal I/O registers.

When  reading an I/O port, 18E first clears the B register, places

the  port number in the  C register and then  executes an IN A,(C)

instruction.

When writing  to an  I/O port,  18E first  clears the  B register,

places  the port number in  the C register, points  HL to a buffer

internal to 18E  containing  the  data  to  be  written  and  then

executes an OUTI instruction.

This  method ensures  that A8  to A15  are at  0 when  the N  or Q

commands are accessing an I/O port.

4

18E - Z80/HD64180 DEBUG MONITOR                    Damon R. Gibson

T       Select Debug Terminal (new command)

__________________________________________________________________

The T command does not exist in Z8E and is used to implement 18E's

auxiliary debug terminal feature.

When 18E is  first invoked, the  main terminal is  enabled.  Then,

the T command may be used to enable an auxiliary terminal.

The enabled terminal  displays all 18E  output and is  used by the

operator to enter commands and data to 18E.  In this way, enabling

the auxiliary  terminal allows the  user to debug  a program which

makes use of  the main terminal  as parts of  its function without

fear  that the debugger will "walk"  on his nice clean application

screen.   (Of course, the user must HAVE a second terminal to take

advantage of this.)

The format for the T command is as follows:

*T  ARG1 <cr>

where ARG1 is the letter A to enable the auxiliary ter-

minal or the letter M to re-enable the main terminal.

Any other letter will result in an error and will cause

two question marks to be displayed  (terminal selection

does not change).

Note that if the auxiliary terminal is enabled, and the user types

a  CTRL-C on the  auxiliary  terminal, a warm boot  (sure cure for

cold feet)  will be  performed and the main terminal  will receive

the  CP/M  prompt  (no  need to transfer control back  to the main

terminal via "T M" first).

* * *  CAUTION  * * *

------------------------------------------------------------------

The  user must  have already  configured 18E's  auxiliary terminal

parameters  and must have a terminal (set to the proper baud rate)

connected  to his auxiliary port before executing a "T A" command.

Failure to do this  will result  in a  "locked up"  computer which

must be re-booted.

------------------------------------------------------------------

5

18E - Z80/HD64180 DEBUG MONITOR                    Damon R. Gibson

V.  APPENDICES

6

18E - Z80/HD64180 DEBUG MONITOR                    Damon R. Gibson

APPENDIX A - Hitachi HD64180 Instructions

In addition to the 67 Z80 mnemonics and the 5 pseudo-ops supported

by   Z8E,  18E  also  supports  the  10  Hitachi  HD64180-specific

mnemonics.

The Z80 mnemonics  are all either 2, 3 or 4 characters long  while

the 5  pseudo-ops are  either 3 or 4 characters long.  The Hitachi

HD64180-specific  mnemonics  do not quite  fit this mold  in  that

three of them (only the Japanese know why) are 5 characters long.

After careful deliberation, it  was  decided  to  implement  these

three "rogues"  as 4 character  mnemonics for 18E.   The following

table shows all  10 Hitachi  HD64180-specific mnemonics  and their

18E equivalents.

HITACHI  |  18E  |  USAGE

-----------+-------+------------------------------

IN0    |  IN0  |  IN0 g,(m)

OUT0   |  OUT0 |  OUT0 (m),g

TST    |  TST  |  TST g or TST (HL) or TST m

SLP    |  SLP  |  SLP

MLT    |  MLT  |  MLT ww

OTIM   |  OTIM |  OTIM

OTDM   |  OTDM |  OTDM

OTIMR  |  OIMR |  OIMR      \   5 character

OTDMR  |  ODMR |  ODMR       >  mnemonics

TSTIO  |  TSIO |  TSIO m    /   changed to 4

Key to parameter symbols in USAGE column:

g       register A, B, C, D, E, H or L

ww      register pair BC, DE, HL or SP

(m)     8 bit I/O port address

m       8 bit immediate data

7

18E - Z80/HD64180 DEBUG MONITOR                    Damon R. Gibson

APPENDIX B - System Memory Map

||||||||||||||||||||||||||||||||||||     FFFF

||                                ||

||                                ||

||      CP/M (BDOS and BIOS)      ||

||                                ||

||                                ||

||||||||||||||||||||||||||||||||||||

||                                ||

||    18E (9.0k to 9.5k BYTES)    ||

||                                ||

||||||||||||||||||||||||||||||||||||

||                                ||

||     OPTIONAL SYMBOL TABLE      ||

||                                ||

||||||||||||||||||||||||||||||||||||

||                                ||

||                                ||

||                                ||

||                                ||

||                                ||

||                                ||

||              TPA               ||

||                                ||

||                                ||

||                                ||

||                                ||

||                                ||

||                                ||

||                                ||

||||||||||||||||||||||||||||||||||||  --- 0100

||                                ||

||      PAGE ZERO  RESERVED       ||

||                                ||

||||||||||||||||||||||||||||||||||||  ___ 0000

The  size of 18E is dependent on how the H64180 and AUXPRT equates

are set in the source file 18E.Z80.  If both equates are set to NO

(neither HD64180 or auxiliary terminal support activated) 18E will

use 9k.  Each  of the  equates will  add 1/4k  (256 bytes)  to the

length of 18E if it is set to YES for a maximum size of 9.5k.

8

18E - Z80/HD64180 DEBUG MONITOR                    Damon R. Gibson

APPENDIX C - 18E Command Summary

CMD  Description                  Arguments

A   Inline  Assembly             StartAddr

B   Set Breakpoint               Addr1[,Pass Count]  [Addr2..AddrN]

C   Clear Breakpoint             Addr1               [Addr2..AddrN]

D   Dump Memory                  [StartAddr]         [End/Count]

E   Examine Memory               StartAddr

F   Find                         StartAddr           MatchData

G   Go                           ExecutionAddr

H   Display Symbol Table         [FirstSymbol]

I   Input File                   FileName            [,Load Address]

J   Full Screen/Animated Debug   [/] [*] [Addr]      [Timeout]

K   Set Memory Window            StartAddr           [Size]

M   Move Memory                  SourceStart   SourceEnd   DestStart

N   Output to Port NO Pre-Read   [(] PortAddr [)]

O   Output Current Breakpoints

P   Exam/Modify PSW (Flag Reg)

Q   Query I/O Port               [(] PortAddr [)]

R   Examine/Modify Registers     RegSpecifier

S   Single-Step                  [/]  [Count]

T   Select Debug Terminal        A or T

U   Write Symbol Table To Disk   FileName

V   Verify Memory                SourceStart   SourceEnd   DestStart

W   Write to Disk                FileName      [StartAddr] [EndAddr]

X   Examine Machine State

Y   Fill Memory                  FromAddr      ToAddr      Data

Z   Disassemble                  StartAddr     End/Count   FileName

[]        Denotes Optional Argument

[/]       Do Not Trace Subroutine

[*]       Do Not Trace BDOS Call

[(] [)]   I/O Port Monitor Mode

9

###########################