MINIPROP (TM)

Ionospheric Propagation Predictions

Version 2.0

A User-Supported Program

Copyright 1985, 1987 by Sheldon C. Shallon, W6EL

Protected by Federal Copyright Law.

Permission granted for non-commercial use and distribution.

Not to be used for commercial advantage.  All rights reserved.

This document contains 34 pages.

TABLE OF CONTENTS

1. Introduction                                                   4

Background                                                   4

Fundamentals of Ionospheric Propagation                      5

Disclaimer                                                   7

Distribution and Use of MINIPROP                             8

2. Starting MINIPROP                                              9

3. The Master Menu                                               10

4. Running a MINIPROP Prediction (Master Menu Choice 1)          11

Latitudes and Longitudes                                    11

Entering Terminal A Data                                    12

Entering Terminal B Data                                    13

Entering the Date                                           13

Entering the Sunspot Number or Solar Flux                   13

An Example                                                  14

The Command Line                                            19

5. DX Compass (Master Menu Choice 2)                             21

Entering Data                                               21

An Example                                                  21

6. Changing Prediction Parameters (Master Menu Choice 3)         23

Change Frequencies Used in Signal Level Predictions         23

Change Antenna Minimum Radiation Angle Used in Predictions  24

Suppress Output of Low Signal Levels                        25

Suppress Output of Signal Levels at Frequencies

Higher than Predicted FMUF                               25

Change Path (Short or Long) to be Computed First            26

Restore Original Set of MINIPROP Defaults                   26

7. MINIPROP Utilities (Master Menu Choice 4)                     28

8. Create or Modify Terminal A Default

(MINIPROP Utilities Choice 2)                                 28

9. Display Entries in MINIPROP Atlas

(MINIPROP Utilities Choice 3)                                 29

10. Add, Modify or Delete Entry in MINIPROP Atlas

(MINIPROP Utilities Choice 4)                                 29

Adding an Atlas Entry                                       29

Modifying an Atlas Entry                                    30

Deleting an Atlas Entry                                     30

11. Display MINIPROP Documentation on Screen

(MINIPROP Utilities Choice 5)                                 30

12. Send MINIPROP Documentation to Printer

(MINIPROP Utilities Choice 6)                                 31

-2-

Table of Contents

13. Print Table of Great-Circle Bearings

(MINIPROP Utilities Choice 7)                                 31

14. Convert Version 1 MINIPROP.ATL to Version 2 MP.ATL

(MINIPROP Utilities Choice 8)                                 32

15. References                                                    33

-3-

1. Introduction

---------------

MINIPROP is a program for predicting ionospheric (skywave) propagation on

frequencies between 3 and 30 MHz.

Background

----------

I wrote my first propagation prediction program in 1966 to help me work

DX.  That first program, a computerized version of the cook-book procedure in

NBS Circular 462 (1), was written in BASIC on a GE time-sharing computer.

During the next several years the program was revised several times to take

advantage of improved prediction algorithms contained in several government

publications, primarily NBS Monograph 80 (2), ESSA Technical Report ITSA-1 (3),

and ESSA Technical Report ERL 110-ITS 78 (4).  Additional information was

obtained through personal correspondence with one of the authors of ERL 110-

ITS-78.  Additional program versions were written in BASIC for several

computers, and versions were written in FORTRAN and RPN.

All of these programs predicted only signal strength.  MUF was predicted

manually, using the two-control-point method with a control point near each end

of the path, from world maps with superimposed predicted iso-MUF(ZERO)F2 and

iso-MUF(4000)F2 curves (5)(6).

In December, 1982, MINIMUF-3.5 (7)(8), developed by the U.S. Naval Ocean

Systems Center, appeared in QST (9).  Although it used an incorrect algorithm

for control points that are in midnight sun conditions (10)(11) and for some

control points that are in polar night conditions, MINIMUF made practical the

prediction of F-layer MUF on microcomputers.

MINIPROP version 1, released August 19, 1985, for CP/M computers, combined

the prediction of signal levels, F-layer MUF (using an improvement on the

MINIMUF-3.5 model with corrections for the midnight sun and polar night

conditions), and E-layer cutoff frequency.  The MS-DOS version was released the

following October.  MINIPROP was a compiled program written in the Pascal

programming language.

MINIPROP was the most comprehensive propagation prediction program for

microcomputers that was available to the radio amateur, and copies of version 1

spread quickly worldwide.  As usage expanded, an increasing number of requests

were made for additional features.  At the same time, improved ionospheric

propagation models were acquired from several sources.  In particular, a

superior F-layer model (12) and other prediction techniques (13) used by the

BBC were made available through published literature and personal

correspondence, and an improved method for predicting E-layer cutoff frequency

was obtained from a report of the ITU International Radio Consultative

Committee (CCIR) (14).  The additional features and improved propagation models

have been combined in version 2 of MINIPROP.  The improved models are more

complex than those in the original MINIPROP and so have a longer running time,

but the additional time is well worth the improved prediction accuracy.

-4-

Fundamentals of Ionospheric Propagation

---------------------------------------

The ionosphere is that region of the earth's atmosphere in which free ions

and electrons exist in sufficient abundance to affect the properties of

electromagnetic waves that are propagated within and through it.  Ions are

produced in the atmosphere partly by cosmic rays but mostly by solar radiation.

The latter include ultraviolet light, x rays, and particle radiation (during

storm periods).  These radiations are selectively absorbed by the several

gaseous constituents of the atmosphere, ion-electron pairs being produced in

the process.  For practical purposes, the ionosphere can usually be assumed to

extend from about 50 to roughly 2000 km above the earth's surface.  The

structure of the ionosphere is highly variable, and this variability is

imparted onto signals propagated via the ionosphere.  The ionosphere is divided

into three vertical regions -- D, E, and F -- which increase in altitude and in

electron density.

The D region has an altitude range from 50 to 90 km.  The electron density

in the region has large diurnal variations highly dependent upon solar zenith

angle.  The electron density is maximum near local noon, is higher in summer

than in winter, and is lowest at night.

The E region spans the altitude range from about 90 to 130 km.  The

maximum density occurs near 100 km, although this height varies with local

time.  The diurnal and seasonal variations of electron density are similar to

those of the D region.  Collisions between electrons and neutral particles,

while important in the E region, are not as numerous as in the D region.  The E

region acts principally as a reflector of hf waves, particularly during

daylight hours.

Embedded within the E region is the so-called sporadic-E layer.  This

layer is an anomalous ionization layer that assumes different forms --

irregular and patchy, smooth and disklike -- and has little direct bearing to

solar radiation.  The causes of sporadic-E ionization are not fully known.  The

properties of the sporadic-E layer vary substantially with location and are

markedly different at equatorial, temperate, and high latitudes.  "Short-skip"

openings, sometimes on an otherwise dead band, are often a result of one-hop

sporadic-E ionization.  When sporadic-E ionization is sufficiently widespread,

multi-hop propagation is possible.  MINIPROP predictions do not include the

effects of sporadic-E ionization.

The highest ionospheric region is termed the F region.  The lower part of

the F region, from 130 to 200 km, is termed the F1 region, and the part above

200 km is termed the F2 region.

The F2 region is the highest ionospheric region, usually has the highest

electron density, and is the region of greatest value in long-distance hf

ionospheric propagation.  The region exhibits large variability in both time

and space in response to neutral winds and electrodynamic drifts in the

presence of the earth's magnetic field.  The maximum electron density generally

occurs well after noon, sometimes in the evening hours.  The height of the

maximum ranges from 250 to 350 km at midlatitudes to 350 to 500 km at

equatorial latitudes.  At midlatitudes, the height of the maximum electron

density is higher at night than in the daytime.  At equatorial latitudes, the

opposite behavior occurs.

-5-

The F1 region, like the E region, is under strong solar control.  It

reaches a maximum ionization level about one hour after local noon.  At night

and during the winter the F1 and F2 regions merge and are termed simply "F

region".

Electromagnetic waves are refracted when passing through an ionized

medium, the refraction increasing with increased electron density and

decreasing with increase of frequency.  If the refraction is large enough, a

wave reaching the ionosphere is bent back toward earth as though it had been

reflected, thereby permitting reception of the wave at a large distance from

the transmitter.  The F2 layer is the most important in this regard because of

its height and its high electron density.  The maximum earth distance traversed

in one F2-layer "hop" is about 4000 km.  Round-the-world communication can

occur via multiple hops.

If the frequency is too high, the wave is not refracted sufficiently to

return to earth.  The maximum frequency for which a wave will propagate between

two points is called the maximum usable frequency (MUF).  Frequencies higher

than the existing MUF at any given time are not supported, no matter how much

power is used.  However, because of the large variability that exists in the

electron density of the F2 region, predicted MUFs are not absolute limits, but

are statistical in nature.  The actual MUF at any given time may be higher or

lower than the predicted MUF.  Predicted MUFs are intended to be median values;

i.e., the actual MUF will exceed the predicted MUF 50 percent of the time, and

will be less than the predicted FMUF 50 percent of the time.  The predicted

frequency that will be supported only 10 percent of the time is a frequency

higher than the predicted MUF called the highest probable frequency (HPF), but

even higher frequencies are possible 10 percent of the time.  The predicted

frequency that will be supported 90 percent of the time is a frequency lower

than the predicted MUF called the optimum traffic frequency (FOT).  Curves of

predicted HPF, MUF, and FOT for several paths appear each month in QST.  In

MINIPROP, the predicted F2-layer maximum usable frequency is referred to as

FMUF.

Signals on their way to or from the F2 layer must pass through the E

region of the ionosphere.  The E layer is also capable of "reflecting" hf

signals, and if the E-layer MUF is too high, the signals to or from the F2

layer are blocked -- or cut off -- by the E layer.  In MINIPROP, the E-layer

cutoff frequency is referred to as ECOF.  Signals at frequencies below the ECOF

will not pass through the E layer.  Signals can propagate between two points on

earth via the E layer in the same manner as they do via the F2 layer, but the

maximum earth distance traversed in one E-layer hop is only about 2000 km, so a

significantly greater number of hops is usually required on DX paths.  E-layer

propagation modes are not predicted by MINIPROP.

The D region of the ionosphere must be traversed by signals on their way

to and from the F2 or E layers.  Electron densities in the D region are not

large enough to cause hf signals to be returned to earth, but the high

collision frequency between the electrons and neutral particles in the D region

gives rise to absorption of signals passing through it.  The reduction of

signal strength can be substantial, particularly in daytime on the lower hf

frequencies.  Antenna installations that provide low radiation (take-off)

angles can minimize the number of hops required between two stations, thereby

reducing the number of passes through the D region and the amount of signal

absorption.

-6-

Electron density in the ionosphere increases with increased solar

activity.  Therefore, MUFs and signal absorption both increase as solar

activity increases.  The Zurich smoothed mean sunspot number has been used

extensively as an index of solar activity and the one with which propagation

data has been correlated over the years.  Therefore, most propagation

prediction models require that the user specify the sunspot number to be used

in making a prediction.  The 2800-MHz (10.7-cm) solar noise flux is generally

considered a more accurate measure of solar activity, but with a smaller base

of propagation observations.  Since the two indices are highly correlated,

either index may be used.  MINIPROP allows you to specify either sunspot number

or solar flux as a measure of solar activity.

Ionospheric propagation is susceptible to several kinds of short-term

disturbance that are usually associated with solar flares.  Depending upon the

nature of the disturbance, they are called sudden ionospheric disturbances,

polar cap absorption events, or ionospheric storms.  These disturbances upset

the electron configuration in the ionosphere, and consequently affect

propagation.  Propagation is also affected by changes in the earth's magnetic

field.  The magnetic field is constantly fluctuating, but the fluctuation

occurs over much wider limits during magnetic storms that accompany ionospheric

storms.  Ionospheric and magnetic storms are also often accompanied by visible

aurora.

Except for the tendency of these disturbances to recur in synchronism with

the 27-day rotation period of the sun, they are difficult for the amateur (as

well as the professional) to predict and to quantify.  The severity of magnetic

disturbances is indicated by A and K indices that are broadcast by WWV at 18

minutes past each hour.  The A index is a daily measure of geomagnetic field

activity on a scale of 0 to 400.  The K index is a measure, for a 3-hour

period, of variation or disturbance in the geomagnetic field on a scale of 0 to

9.  In general, MUFs decrease and signal absorption increases as geomagnetic

field activity increases, although MUFs sometimes increase in equatorial

regions.

Propagation only along great-circle paths is considered in MINIPROP.

Backscatter modes and bent-path modes are not considered.

More information about ionospheric propagation can be found in "The

Shortwave Propagation Handbook" by W3ASK and N4XX (15), and in Chapter 22 of

the "The 1987 ARRL Handbook" (16).

Disclaimer

----------

Sheldon C. Shallon shall have no liability or responsibility to user or

any other person or entity with respect to any loss or damage caused or alleged

to be caused directly or indirectly through the use of this documentation or

the MINIPROP program.

-7-

Distribution and Use of MINIPROP

--------------------------------

MINIPROP is a copyrighted program.  All rights are retained by the author.

MINIPROP is not to be sold; nor is it to be used for commercial purposes.  You

may freely distribute MINIPROP, and you are encouraged to do so, but you may

not charge for doing so.  The following files should be included in any

distribution:

MP.COM

MP1.000

MP1.CHN

MP2.CHN

MP3.CHN

MP4.CHN

MP.DOC

MP.ATL

READ.ME

MINIPROP is a user-supported program.  This means that if you find

MINIPROP useful you should consider sending a contribution in the suggested

amount of $25.00 to the author:

Sheldon C. Shallon

11058 Queensland St.

Los Angeles, CA  90034-3029

Your comments and suggestions regarding MINIPROP will be welcome.  Please

enclose a self-addressed stamped envelope if you wish a reply.

Good DXing,

Shel, W6EL

2-24-87

-8-

2. Starting MINIPROP

--------------------

If you have not already done so, please make a working copy of MINIPROP.

The disk on which you received MINIPROP should be put away for safekeeping, and

the working copy used when you want to run MINIPROP.  Do not write protect the

working copy because MINIPROP has to able to write to the disk.

Your working disk should contain the following files:

MP.COM   (Required to start MINIPROP.)

MP1.000  (You cannot run a MINIPROP prediction without this file.)

MP1.CHN  (Required to start MINIPROP.)

MP2.CHN  (Necessary for the DX Compass feature only.)

MP3.CHN  (You will not be able to change several of the

built-in prediction parameters without this file.)

MP4.CHN  (You will not be able to use the MINIPROP utilities

without this file.)

MP.DOC   (This MINIPROP documentation.)

MP.ATL   (Atlas of latitudes and longitudes.  Not essential,

but very helpful.)

READ.ME  (Help in starting MINIPROP.)

In addition, a MP.DEF file will be created automatically the first time

you run MINIPROP.  This file will hold any changes you make to several of the

values and settings used in the MINIPROP predictions, as well as the latitude

and longitude of your location.  Be sure there is room on the disk and in the

disk directory for this short file.

All of the MINIPROP files should be on the logged disk drive and in the

current directory.

To run MINIPROP, type "MP" following the > prompt from your computer's

operating system.  (In this documentation, everything you are to enter on your

computer keyboard appears in quotes.  You should type everything between the

quotes exactly as shown, but do not type the quotes.  You may always use either

upper or lower case letters.)  Then press the ENTER or RETURN key, depending on

the keyboard of your computer.  When running MINIPROP, you will see this key

referred to on the screen as <ENTER>, <RETURN>, or <RTN>.

After MINIPROP has been loaded from disk into RAM, an opening message and

the MINIPROP master menu will appear on the screen.

-9-

3. The Master Menu

------------------

The MINIPROP master menu will appear on your screen as follows:

MASTER MENU

0. Exit from MINIPROP (Quit)

1. Run a MINIPROP prediction (you may also press <ENTER>)

2. DX Compass (directions of band openings)

3. Change prediction parameter defaults

4. MINIPROP utilities

Your choice :

If you do not wish to continue, enter "0" to terminate MINIPROP and return

to the operating system.  You may also enter "Q" or "X".

Use choice 1 to predict propagation between two locations (terminals).

This is the choice you will probably use most often.  You may also press just

<ENTER> or <RETURN> to invoke this choice.  See Section 4 below.

Choice 2 will give you a picture of the highest frequencies that are most

likely to be open to DX in several directions from a specified location

(terminal) at a given time of day.  See Section 5 below.

Choice 3 gives you the option to change certain prediction parameters from

their default values.  These options are:

Change frequencies used in signal level predictions

Change antenna minimum radiation angle used in predictions

Suppress output of low signal levels

Suppress output of signal levels at frequencies higher than predicted FMUF

Change path (short or long) to be computed first

Restore original set of MINIPROP defaults

More information on these options can be found in Section 6 of this

documentation.

Choice 4 gives you access to the MINIPROP utilities.  With these utilities

you can:

Create or modify Terminal A default

Display entries in MINIPROP atlas

Add, modify, or delete entry in MINIPROP atlas

Display MINIPROP documentation on screen

Send MINIPROP documentation to printer

Print table of great-circle bearings

Convert version 1 MINIPROP.ATL to version 2 MP.ATL

See Sections 7 through 14 of this documentation for more information about the

MINIPROP utilities.

-10-

4. Running a MINIPROP Prediction (Master Menu Choice 1)

-------------------------------------------------------

Latitudes and Longitudes

------------------------

To compute a propagation prediction between two terminals, MINIPROP needs

to know the locations of those two terminals.  A location is defined in terms

of its latitude and longitude.  You will be asked for the latitude and

longitude of the terminals at the two ends of the propagation path.  One of the

terminals is called Terminal A; the other is Terminal B.  It makes no

difference which terminal is A and which is B.

Latitude is a measure of angular distance north or south of the earth's

equator.  It is measured in degrees.  The latitude of the equator is 0 degrees.

The north pole is at 90 degrees north latitude, and the south pole is at 90

degrees south latitude.  Latitudes can never exceed 90 degrees in magnitude.

To distinguish between north and south latitudes, the latter must be preceded

by a minus (-) sign.  For example, 40 degrees north latitude would be entered

as 40; 40 degrees south latitude would be entered as -40.  Do not place a plus

(+) sign in front of north latitudes.

Longitude is a measure of angular distance east or west of the prime

(Greenwich) meridian.  It is measured in degrees.  The longitude of the prime

meridian is 0 degrees.  Longitudes to the west of the prime meridian (e.g., in

the western hemisphere) are west longitudes.  Longitudes to the east of the

prime meridian (e.g., in the eastern hemisphere) are east longitudes.  The

meridian half way around the world from the prime meridian is at longitude 180

degrees; its longitude can be given as either 180 degrees east or 180 degrees

west (180 or -180).  Longitudes usually have magnitudes between 0 and 180

degrees.  However, any particular meridian of longitude can be reached by

travelling either east or west from the prime meridian.  Therefore, a longitude

of 90 degrees west is equivalent to a longitude of 270 degrees east, for

example.  MINIPROP will accept longitudes with magnitudes between 0 and 360

degrees.  To distinguish between west and east longitudes, the latter must be

preceded by a minus (-) sign.  For example, 100 degrees west longitude would be

entered as 100; 100 degrees east longitude would be entered as -100.  Do not

place a plus (+) sign in front of west longitudes.

An atlas containing latitudes and longitudes of more than 350 locations is

supplied with MINIPROP, but you will probably have need for your own latitude

and longitude and for latitudes and longitudes of other locations that are not

in the MINIPROP atlas.  You can generally find these in atlases that are

available in bookstores and libraries, or you may scale them from maps or

charts.  A note of caution is in order with respect to latitudes and longitudes

that are tabulated in atlases.  They are usually shown as decimal numbers, but

are often in a degree.minute format.  Thus, 40.54 might mean 40.54 degrees, but

it usually means 40 degrees, 54 minutes.  If the latter is meant, the

equivalent decimal value is 40.9 degrees.

MINIPROP will accept latitudes and longitudes as decimals or in a degree

minute second format.  If you wish to enter a latitude or longitude in the

latter format, leave one blank space between the degrees and minutes, and

another between the minutes and seconds, if any.  Leading and trailing blank

spaces are invalid.  Here are some examples.  All of the values in each column

-11-

are equivalent.

"40.9"         "0.5"         "-28 10 30"    "-135" (east longitude)

"40 54"        "00 30"       "-28 10.5"     "225"  (west longitude)

"40 54 0"      "00 30 0"     "-28.175"

"40 54 00"     "00 30 00"

The following entries are invalid.

"33 60"      (Minutes and seconds must be less than 60.)

"118  25"    (Too many blank spaces.)

"-34 -25"    (Put minus sign in front only.)

" 42"        (Leading blanks not allowed.)

"42 "        (Trailing blanks not allowed.

"85.5 25"    (Decimal allowed only in last part of entry.)

"91"         (If a latitude, may not exceed 90.)

"361"        (Longitudes may not exceed 360.)

"+34"        (Leading + sign not needed or allowed.)

Entering Terminal A Data

------------------------

When you run a MINIPROP prediction you will see the following prompt on

the screen.

Enter Terminal A latitude (or <RTN> for default), or prefix, or <?> :

You have four choices:  (1) You may enter the latitude of Terminal A.

(2) you may press <RTN> to use the Terminal A default location.  (Remember,

<RTN> means you should press the RETURN or ENTER key.)  (3) you may enter the

country prefix (or other unique identifier) if you want to use a MINIPROP atlas

entry.  (4) you may press <?> followed by <RTN> if you want to be reminded of

the prefixes that are available in the atlas.

If you enter a latitude in response to the latitude prompt, you will next

be prompted for the longitude of Terminal A.  Enter the longitude.  Then you

will be asked for the name of Terminal A.  You may want to enter a call sign,

city name, or other identifier.  The name you enter will appear in the

prediction heading as an identifier for your use.  MINIPROP makes no other use

of the name.  The name may contain up to 17 characters and blanks.  You may

press only <RTN> if you do not want to enter a name.

If you press only <RTN>, MINIPROP will look in the MP.DEF disk file for

the default latitude, longitude, and name of Terminal A.  This saves you the

trouble of entering your own latitude, longitude, and terminal name each time

you want a prediction on a path from your own station.  If you have not already

created a Terminal A default for your own location, one of the MINIPROP

utilities (Master Menu choice 4) will help you do so.

If you enter a prefix in response to the latitude prompt, MINIPROP will

search the MP.ATL atlas file to find the entry with that prefix.  The contents

of the atlas entry will appear on the screen and the displayed values will be

used in the prediction.

If you need to be reminded of the prefixes in the MINIPROP atlas, press

-12-

<?>.  All of the prefixes in the atlas will appear on the screen, followed

again by the above prompt.

Entering Terminal B Data

------------------------

The following prompt will appear next.

Enter Terminal B latitude, or prefix, or <?> :

Except for the Terminal A default, your choices are the same as for

Terminal A.  Proceed as for Terminal A.  You may not use the Terminal A default

location for Terminal B.

Entering the Date

-----------------

Next you will be asked for the date to be used in the prediction.

MINIPROP needs the date to compute the position of the sun, the times of

sunrise and sunset, and the directions of the gray lines at the path terminii.

Enter the date as a six-digit number with no leading, trailing, or embedded

spaces and no punctuation.  Examples:

122585 is December 25, 1985

010187 is January 1, 1987

070410 is July 4, 2010

MINIPROP will convert the last two digits to a year between 1945 and 2044,

inclusive.  This 100-year span allows you to obtain current predictions, to

compare MINIPROP predictions with actual propagation records back to the

beginning of the post-WW II DXCC, and to predict propagation well into the

future.  Invalid dates (e.g., 022987) will not be accepted.

Entering the Sunspot Number or Solar Flux

-----------------------------------------

You have to enter one more item of information.  You will see the

following prompt on the screen.

Sunspot number (or solar flux, e.g., F70) for above date?

If you want to enter a sunspot number, just enter the number and press

<RTN>.  If you want to enter a solar flux value, first enter "F" or "f",

immediately followed (no blanks) by the value and press <RTN>.  For example, a

sunspot number of 45 would be entered as "45"; a solar flux of 70 would be

entered as "F70" or "f70".  MINIPROP will do the conversion for you.

The sunspot number and solar flux are both measures of solar activity.

They are highly, but not 100-percent, correlated with each other.  Therefore, a

range of sunspot numbers can correspond to a particular value of solar flux,

and vice versa.  For example, the sunspot number was 0 on eight days in

November, 1986; on those days, the solar flux value ranged from 71 to 80.  On

six days when the value of the solar flux was 76, the sunspot number ranged

from 0 to 23.  Various formulas to approximate the relationship between sunspot

number and solar flux have appeared in the literature.  MINIPROP uses the

-13-

following formula published by the Institute for Telecommunication Sciences

(6):

solar flux = 63.75 + 0.728 ssn + 0.00089 ssn^2

In this formula, ssn is the 12-month moving average Zurich sunspot number, and

ssn^2 is the square of that number.  The solar flux is in units of 10^-22 watts

per square meter per Hz.

MINIPROP will accept sunspot numbers between 0 and 390, and solar flux

values between 60 and 483.  However, if you input a solar flux value between 60

and 63.75, MINIPROP will substitute a solar flux value of 63.75 corresponding

to a sunspot number of 0.  Solar flux values less than 60 will not be accepted.

Predicted sunspot numbers are published monthly in conjunction with the

propagation charts in QST, and in George Jacobs' propagation column in CQ.  QST

also predicts the solar flux value, but the formula relating QST's predicted

sunspot number and solar flux value differs somewhat from the above formula

used in MINIPROP.  Daily values of solar flux are broadcast by WWV at 18

minutes past each hour.  In addition, the weekly KH6BZF Reports contains up-to-

date sunspot numbers, solar flux values, A and K indices, propagation

conditions, and other useful information.

MUFs do not seem to follow day-to-day variations in solar activity, so I

prefer to use monthly predicted sunspot numbers.  However, a change in solar

activity, when sustained for a period of time (several days, say), does

influence MUFs.  Therefore, you may want to use solar flux values representing

the average of the values broadcast by WWV for the past several days.

Absorption in the D region reacts more quickly to solar activity, but

significant changes in predicted signal strengths would not be expected except

for very large changes in solar activity, and then only on daytime paths at the

lower frequencies.

MINIPROP will start computing the propagation prediction as soon as you

have entered a sunspot number or solar flux value.

An Example

----------

As an example, let's predict what propagation will be like on the path

from Omaha, Nebraska, to Agalega (3B6) on January 25, 1990, if the predicted

smoothed sunspot number is 114.  I have chosen this example because it

demonstrates the several elements of a MINIPROP prediction and illustrates how

band openings are predicted on the 10- through 80-meter bands.

Start MINIPROP by entering "MP" following the > prompt from your

computer's operating system.  When the MINIPROP master menu appears on the

screen, enter the number "1" or press the ENTER or RETURN key.

We will assume that the latitude and longitude of Omaha have not been

entered as the Terminal A default and are not in the MINIPROP atlas, so we have

to look them up and enter them manually.  My Rand McNally atlas lists the

coordinates as 41 deg 15 min N, 95 deg 56 min W.  In response to the prompt for

the latitude of Terminal A, enter "41 15".  In response to the longitude

prompt, enter "95 56".  In response to the name prompt, we can enter any string

-14-

of up to 17 characters.  Let's use only the city name; enter "Omaha".

Let's use the MINIPROP atlas entry for Agalega.  In response to the prompt

for the latitude of Terminal B, enter "3B6".

For the date enter "012590".  For the sunspot number enter "114".

After a brief delay, the following display will appear on the screen while

MINIPROP continues to compute the prediction.

MINIPROP

01-25-1990

Sunspot Number : 114.0   Flux : 158.3   Min. Radiation Angle :  1.5 deg

TERMINAL A : Omaha              TERMINAL B : Agalega

Latitude :  41.25 N             Latitude :  10.42 S

Longitude :  95.93 W            Longitude :  56.65 E

Sunrise : 1346 UTC              Sunrise : 0211 UTC

Gray line : 26/206 deg          Gray line : 19/199 deg

Sunset : 2326 UTC               Sunset : 1440 UTC

Gray line : 334/154 deg         Gray line : 341/161 deg

SHORT-PATH  Bearing from A to B :  45.8 deg

Bearing from B to A : 326.7 deg

Path Length : 15651 km    9726 mi (U.S.)

LONG-PATH   Bearing from A to B : 225.8 deg

Bearing from B to A : 146.7 deg

Path Length : 24349 km   15131 mi (U.S.)

The second line shows the date you specified in expanded form showing the

four-digit year.  On the next line are the sunspot number you entered and the

equivalent solar flux value.  If you had entered the solar flux value, MINIPROP

would have computed and displayed the equivalent sunspot number.  The third

line also shows the antenna minimum radiation (take-off) angle that is being

used in computing the prediction.  The default (initial setting) for this

parameter is 1.5 degrees.  If your antenna installation or horizon obstructions

preclude achievement of this low an angle, Master Menu choice 3 allows you to

increase the value.  (See Section 6 below.)  The value may also be decreased if

appropriate to your installation.  For this example, we will use the default

1.5 degrees.

The next seven lines contain two columns of information, one for each of

the terminals.  For each terminal, the name, latitude, and longitude are

displayed.  In this example the latitudes and longitudes were specified in

degree-minute format, but MINIPROP has converted them to decimals.  Although

shown to two decimal places, latitudes and longitudes are carried to greater

accuracy within MINIPROP.  Instead of a plus (implied) or minus sign, a letter

(N, S, E, or W) indicating the direction from the equator or the prime meridian

is shown.

-15-

Sunrise and sunset times and directions of the gray lines are also shown

for each terminal.  There are several definitions of sunrise and sunset (e.g.,

civil, nautical, astronomical), and times can differ by several minutes between

the different definitions.  MINIPROP defines sunrise and sunset as the times

when the center of the sun is on the celestial horizon.  The gray line is the

fuzzy band around the earth separating daylight and darkness.  Many DXers point

their antennas along the gray line at sunrise and sunset to look for DX.  For

each sunrise and sunset, the first gray line direction shown is the most

northerly direction and the second direction is the most southerly;  the two

directions are 180 degrees apart.  In this example, sunrise at Omaha is at 1346

UTC.  Around that time you would point your antenna at 26 or 206 degrees east

of north to point along the gray line.

Next, the bearings and path lengths are shown for both the short great-

circle path and the long great-circle path between the two terminii.  You will

note that on neither path do the bearings from one terminal to the other differ

by 180 degrees (e.g., 45.8 and 326.7 degrees on the short path).  This point is

mentioned because some people are surprised that this is true.  Except for some

unique geometries, the bearings will not be 180 degrees apart.  Of course, the

short-path and long-path bearings from any one terminal are 180 degrees apart

(e.g., 45.8 and 225.8 degrees).

By the time you have read this far, MINIPROP will probably have finished

computing the propagation prediction for the short path.  MINIPROP displays its

progress at the bottom of the screen while it is computing the prediction.  It

displays a period (.) periodically while it is computing absorption along the

path, a lower case f while computing F-layer MUF, and a lower case e while

computing the E-layer cutoff frequency.  When MINIPROP has finished computing

the prediction, "Press any key when ready for predictions." appears at the

bottom of the screen.  Press any key when you are ready.

The table of predictions on the next page will appear on your screen.

The information displayed at the top of the screen is similar to the

information in the previous display, but somewhat abbreviated.  Bearings and

path length are shown only for one path, in this case the short path.  MINIPROP

has computed and displayed the minimum number of F hops corresponding to the

minimum radiation angle (1.5 degrees) that appeared on the previous display.

In this example, 5 F hops are required.  The resulting predicted radiation

angle is shown to be 5 degrees.

The predicted F-layer MUF (FMUF), E-layer cutoff frequency (ECOF), and

received signal levels are shown for every two hours UTC.

The predicted signal levels are shown for frequencies in the five primary

hf bands.  You may change these built-in frequencies to any five frequencies of

your choice between 3 and 30 MHz.  (See Section 6 below.)  The signal levels

are in dB with respect to 0.5 microvolt "behind" 50 ohms, and assume a matched

antenna load.  On my receiver 0.5 microvolt produces an S3 signal.  So 0 dB

would be S3.  At 5 dB per S unit, 10 dB would be S5, 30 dB would be S9, etc.

The signal levels assume 100 watts radiated power with half-wave dipole

antennas in free space at both ends of the path, and with the antennas oriented

for maximum gain along the path.  Add or subtract whatever number of dB you

think appropriate for the antennas and power levels in use.

-16-

Predicted signal levels are computed independently of the FMUF and ECOF

under the assumption that the number of F hops (in this case 5) shown in the

header are present at all times and for all frequencies, and that there is no

E-layer cutoff.  (To take E-layer reflections into account in computing signal

levels would be time prohibitive.)  This is important to remember: SIGNAL LEVEL

PREDICTIONS ASSUME THAT A REFLECTING F LAYER IS ALWAYS PRESENT AND THAT THERE

IS NO REFLECTING E LAYER.

MINIPROP (TM) SHORT-PATH PREDICTIONS     01-25-1990     Path Length : 15651 km

Sunspot Number : 114.0   Flux : 158.3   F Hops :  5   Radiation Angle :  5 deg

TERMINAL A : 41.25 N    95.93 W   Omaha               Bearing to B :  45.8 deg

TERMINAL B : 10.42 S    56.65 E   Agalega             Bearing to A : 326.7 deg

Terminal A Sunrise/Set : 1346/2326 UTC  Terminal B Sunrise/Set : 0211/1440 UTC

----------- SIGNAL LEVELS ABOVE 0.5 uV ------------

UTC      FMUF     ECOF     3.6 MHZ    7.1 MHZ   14.1 MHZ   21.2 MHZ   28.3 MHZ

0000     15.9      5.8      40.5 a     35.5 A     29.2 A     24.9

0200     12.5      4.6      39.3 a     35.1 A     29.1 B

0400     14.0     13.8       8.4 a     22.4 a     25.1 B

0600     14.8     17.8                 -0.9 a     17.7 a     19.5

0800     14.2     19.3                            10.7 a     16.2

1000     11.3     19.1                             6.6

1200     11.5     16.9                             6.9

1400     17.3     11.6                            10.8 A     16.3

1600     26.1     14.1                  2.7 a     18.9 A     20.1 A     18.9 B

1800     30.8     15.3     -12.8 a     13.7 a     22.3 a     21.7 A     19.9 A

2000     29.5     14.4       6.9 a     21.8 a     24.9 a     22.9 A     20.6 A

2200     23.3     10.8      28.0 a     30.4 a     27.6 A     24.2 A     21.3

<G>raph, <T>able, <P>rint table, <Q>uit, <M>enu, or <L>ong path ?

The "A", "a", "B", and "b" flags are included in the predictions to flag

predicted band openings and to aid in interpreting the predicted signal levels.

The table on the next page shows how the flags indicate the relationship that

exists between the frequency at the top of a  column and the FMUF and ECOF, and

also shows the probability that propagation on the path can be supported by the

F layer.

"A"s or "a"s in a particular frequency column flag the times that openings

are most probable on that frequency.  At these times, the frequency at the top

of the column is less than the predicted FMUF.  Therefore, it is expected that

the probability of F-layer propagation is greater than 50 percent.  The

difference between "A" and "a" is that the frequency is above the ECOF in the

first case, but below the ECOF in the second.  (The "A" flag has the same

meaning as the "+" flag in MINIPROP version 1.0.)  When the "A" flag is

present, the predicted signal levels apply.  When the "a" flag is present,

usually when one or both of the two terminii is in daylight, E hops are added

to, or substituted for, F hops.  This causes additional absorption loss, and

possibly additional ground reflection loss, that results in weaker signals than

the predicted values.  The reduction of received signal level can be very

large, particularly at the lower frequencies.  Transmitter power and high

antenna gains may sometimes be used to overcome the increased signal loss.

-17-

This allows you to hear foreign broadcast stations in the middle of the day on

the 41 meter band, but this approach usually is not practical for radio

amateurs.

Frequency at Top     !   Flag   !   Probability of

of Column         !          !   F-Layer Support

========================================================

Below FMUF           !          !

---------------------!----------!

and above ECOF    !    A     !    More than 50%

---------------------!----------!

and below ECOF    !    a     !

========================================================

Between FMUF and HPF !          !

---------------------!----------!

and above ECOF    !    B     !       10 - 50%

---------------------!----------!

and below ECOF    !    b     !

========================================================

Above HPF            !   None   !    Less than 10%

========================================================

"B"s or "b"s in a particular frequency column flag the times that the

frequency at the head of the column is greater than the predicted FMUF, but is

predicted to be less than the F-layer HPF.  Therefore, the probability of F-

layer propagation is expected to be less than 50 percent, but greater than 10

percent.  When this possible opening actually occurs, and if the flag is "b",

signal levels are expected to be weaker than shown because E-layer cutoff is to

be expected; the predicted signal levels apply when the opening occurs and the

flag is "B".

Propagation may occur, with low probability, on frequencies even higher

than the HPF, so predicted signal levels are shown for frequencies up to 150

percent of the predicted FMUF.  The display of signal levels is normally

suppressed for frequencies greater than 150 percent of the FMUF, but this

suppression can be cancelled if you wish; see Section 6 below.  Of course, the

signal levels that are shown at the higher frequencies are only applicable when

the actual MUF is high enough to permit propagation at these frequencies.

The display of signal levels below -20 dB is normally suppressed because

weaker signals are generally unusable by radio amateurs, even after allowance

is made for transmitter power and antenna gains.  The blanks in the 3.6- and

7.1-MHz columns are a result of this suppression.  The suppression threshold

can be changed; see Section 6 below.

In this example, 10 meters will most probably be open with good signals

from 1800 to 2000 UTC, as indicated by the "A" flag in the 28.3-MHz column.

The "B" flag in this column indicates that there is a probability between 10

and 50 percent that the band will open as early as 1600.  If the band does not

open early, the signal level shown at 1600 has no meaning.

15 meters will be open from 1600 to 2200 with a probability in excess of

50 percent.

-18-

20 meters will probably open around 1400 UTC with only fair signals.

Signals will build until 1600, drop some around 1800 to 2000 as a result of E-

layer cutoff, and then build to strong signals around 0000 UTC.  The band will

probably go out on this path from 0200 to 0400; predicted signal levels

opposite the "B" flags apply only if the band stays open, and should be

disregarded otherwise.  20 meters will be open again from 0600 to 0800, but

signals will be weaker than shown because of E-layer cutoff.

On 40 meters, signals will be good from 0000 to 0200.  E-layer cutoff will

reduce signal levels, probably below usable levels because of the low

frequency, during most of the remainder of the day.

E-layer cutoff will reduce all signal levels shown for 80 meters, but

signals will be best around 0000 to 0200 UTC.

The Command Line

----------------

At the bottom of the screen is a line containing several command choices.

Make your choice by pressing the initial letter of the command; either upper or

lower case may be used.

<G>raph.  This choice causes the following screen display to appear.

SHORT-PATH PREDICTIONS               01-25-1990               Omaha TO Agalega

Sunspot Number : 114.0   Flux : 158.3   F Hops :  5   Radiation Angle :  5 deg

MHZ            X = FMUF         O = ECOF            MHZ

30 -----------------------------------XX+XXX-------- 30

28                                  XX      X        28

26                                 X         XX      26

24                                X            X     24

22                               X              X    22

20 ---------------OOOOOO--------X----------------X-- 20

18            OOOO      OOO    X                  X  18

16 X        OO             OO X       OOOO         X 16

14  X    XXXXXXXXXXXX        OO   OOOO    OOOO       14

12   XXXX O          XXXXXXXX  OOO            O      12

10 ------O-------------------------------------OO--- 10

8      O                                        O    8

6 O                                              OO  6

4  OOOO                                              4

2                                                    2

0 -------------------------------------------------  0

0   0   0   0   0   1   1   1   1   1   2   2   0 U

0   2   4   6   8   0   2   4   6   8   0   2   0 T

<G>raph, <T>able, <P>rint table, <Q>uit, <M>enu, or <L>ong path ?

The graph uses the letters X and O to plot FMUF and ECOF versus time.  The time

resolution is 30 minutes instead of the 2-hour resolution in the previous

prediction table.  However, the limited screen height limits the frequency

resolution to 2 MHz.  When both an X and an O should appear in the same

location on the graph, the one that is predicted to be at the highest frequency

-19-

appears.  At 0400 UTC in this example, FMUF and ECOF are both 14 MHz to the

nearest 2 MHz; an X appears because the FMUF of 14.0 MHz exceeds the ECOF of

13.8 MHz.  At 1300, only an O appears at 14 MHz because the ECOF is higher than

the FMUF, with both rounding to 14 MHz.  (The predicted ECOF of 14.9 MHz

exceeds the predicted FMUF of 13.5 MHz.)  The plus (+) sign at 1830 indicates

that the predicted FMUF exceeds 31 MHz.  The command line is repeated below the

graph.

<T>able.  This choice causes the prediction table to reappear.

<P>rint table.  This choice sends the prediction table to your printer.

The longer page length as compared to the screen height allows predictions to

be printed for every 30 minutes instead of every 2 hours.  Otherwise, the table

is the same as seen on the screen.  57 lines will be printed, so make sure the

paper is adjusted in your printer so that printing can start within 10 lines

from the top of the page.  It is assumed that your page length is 66 lines.

<Q>uit.  This choice terminates MINIPROP.

<M>enu.  This choice causes the MINIPROP master menu to reappear on the

screen.  From the master menu you can choose to run another prediction, or you

can select one of the other master menu choices.

<L>ong path.  This choice is available only after the short-path

prediction has been displayed first, and causes MINIPROP to compute a

propagation prediction for the long great-circle path between terminals A and

B.  Both table and graph displays are available as in the above example.  Long-

path propagation is difficult to predict reliably; predicted openings often do

not occur, particularly on the higher frequencies.

<S>hort path.  This choice is available only after the long-path

prediction has been displayed first, and causes MINIPROP to compute a

propagation prediction for the short great-circle path between terminals A and

B.

-20-

5. DX Compass (Master Menu Choice 2)

------------------------------------

The DX Compass is a screen display showing the highest FMUF you can expect

on twelve bearings separated at intervals of 30 degrees from a specified

location for Terminal A and at any specified time of day.  This feature is

especially useful for planning your operations during DX contests.

Entering Data

-------------

Data entry is similar to the data entry described above in Section 4 for

MINIPROP predictions.  Only one terminal is used in the DX Compass, so you will

not be asked for Terminal B data.  Entry of Terminal A data, the date, and the

sunspot number or solar flux are the same as in Section 4.

One new piece of data must be entered.  The following prompt will appear

on the screen after the sunspot number or solar flux is entered.

DX Compass for what UTC (e.g., 0215) (or <RTN> to quit)?

As indicated in the prompt, a four-digit UTC in 24-hour format should be

entered.  Valid times are from 0000 to 2400.  Press <RTN> if you want to quit

the DX Compass and return to the MINIPROP master menu.

An Example

----------

As an example, let's use the same data we used in the prediction example

in Section 4, except that there is no Terminal B.  Terminal A is Omaha,

Nebraska.  The date is January 25, 1990.  We assume a predicted smoothed

sunspot number of 114.

Start MINIPROP by entering "MP" following the > prompt from your

computer's operating system.  When the MINIPROP master menu appears on the

screen, enter the number "2".

In response to the prompt for the latitude of Terminal A, enter "41 15".

In response to the longitude prompt, enter "95 56".  For the name of Terminal

A, enter "Omaha".

For the date enter "012590".  For the sunspot number enter "114".

For UTC, let's use the time of sunrise.  From the example in Section 4,

we know this to be 1346; so enter "1346".

When MINIPROP has finished computing the DX Compass, the display at the

top of the next page will appear on the screen.

The time you specified is shown in the center of the display together with

the specified latitude, longitude, and date.  The FMUF values are arranged in a

"circle" around the center of the display.  Immediately below "FMUF" in the top

line is the FMUF that applies on a bearing of 0 degrees (due north) from

Dallas.  The other FMUFs around the DX Compass are for bearings in steps of 30

degrees.

-21-

Sunrise: 1346 UTC                     FMUF           Radiation Angle:  1.5 deg

Sunset:  2326 UTC                                        Sunspot Number: 114.0

Omaha                                 13.2                   Solar Flux: 158.3

12.1                    15.9

11.9                              19.8

1346 UTC

13.3         41.25 N   95.93 W         25.3

01-25-1990

15.6                              26.9

18.6                    24.9

22.2

DX Compass for what UTC (e.g., 0215) (or <RTN> to quit)?

The displayed values of FMUF are an indication of the highest-frequency

bands that can be expected to be open to DX on either a short or long great-

circle path in the different directions around the compass from Terminal A.  In

this example, 10 meters is not yet open in any direction at the time of

sunrise.  15 meters would be expected to be open, with a probability exceeding

50 percent, to DX on bearings between 90 and 180 degrees from Omaha.  20 meters

would be expected to be open on bearings of 30 degrees, through south, to 240

degrees.  20 meters would not yet be open from 300 through 360 degrees, but 40

meters (and 30 meters) would be expected to be open in these directions.  Keep

in mind that the predicted values of only the FMUF are shown, so you cannot

tell, from the DX Compass alone, how strong the received signals might be.

However, the first band lower in frequency than the indicated FMUF will

probably be usable for working DX.

A different interpretation of the compass display applies when Terminal A

is at either the north or south pole.  Instead of compass bearings, the twelve

positions around the "circle" represent meridian longitudes measured west from

the prime meridian.  For example, the FMUF shown in the 30-degrees position

would be the maximum FMUF to be expected on a path from the pole along the 30-

degree west meridian.

The time prompt appears again at the bottom of the screen.  You can enter

additional times to see how the expected band openings shift during the day.

Press only <ENTER> or <RETURN> when you want to quit and return to the MINIPROP

master menu.

-22-

6. Changing Prediction Parameters (Master Menu Choice 3)

--------------------------------------------------------

There are several default (preset) values and settings (e.g., frequencies)

used in the MINIPROP predictions.  Five of these can be changed by the user.

In addition, you can restore the original set of MINIPROP default values and

settings.  If you make any changes, you will be given the chance to save them

in the MP.DEF file of default settings upon returning to the master menu.  Your

settings will then be in effect each time you use MINIPROP.  If you do not save

your changes, they will remain in effect only during the current MINIPROP

session.

To change any of the defaults, first start MINIPROP by entering "MP"

following the > prompt from your computer's operating system.  When the

MINIPROP master menu appears on the screen, enter the number "3".  The

following Defaults Change Menu will appear on the screen.

DEFAULTS CHANGE MENU

0. Return to MASTER MENU

1. Change frequencies used in signal level predictions

2. Change antenna minimum radiation angle used in predictions

3. Suppress output of low signal levels

4. Suppress output of signal levels at frequencies

higher than predicted FMUF

5. Change path (short or long) to be computed first

6. Restore original set of MINIPROP defaults

Your choice :

To make your choice, enter a number, 0 to 6.  Choice 0 will return you to the

MINIPROP Master Menu.  Choices 1 through 6 are explained in the following

sections.

Change Frequencies Used in Signal Level Predictions

---------------------------------------------------

In the MINIPROP propagation predictions described above in Section 4,

signal levels are predicted for frequencies of 3.6, 7.1, 14.1, 21.2, and 28.3

MHz.  You may change these frequencies to any combination of five frequencies

between 3 and 30 MHz.  This will allow you to obtain signal level predictions

for frequencies in the 12-, 17-, or 30-meter bands or in the shortwave

broadcast bands, for example.

The frequencies currently in effect will be displayed.  You will then be

asked if you want to change the frequencies.  If you answer "N" (without the

quotes), the Defaults Change Menu will reappear.  If you enter "Y", the

following (top of next page) will appear.

-23-

You may enter 0 to 5 frequencies between 3 and 30 MHz.

If you do not enter at least one frequency, signal levels

will not be included in the predictions.

Enter your frequencies, if any, one at a time.

Press only <ENTER> or <RETURN> when finished.

Frequency 1 :

Press only <ENTER> or <RETURN> if you do not want signal levels to appear

in the predictions; this will save time when running predictions if you are

only interested in the FMUF and ECOF predictions.  If you do want signal levels

to appear, enter one to five frequencies, one at a time.  You may enter them in

any order; MINIPROP will place them in numerical order and will round them to

the nearest 0.1 MHz.  Duplicate frequencies will be rejected.  If you want to

enter less than five frequencies, press only <ENTER> or <RETURN> when asked for

the next frequency.

MINIPROP will display the new frequencies that are now in effect and will

ask you if you want to change them.  If you are not satisfied with the

frequencies, enter "Y" and you will again be asked to enter your frequencies.

When you are satisfied with the displayed frequencies, enter "N" and the

Defaults Change Menu will reappear.  You may select other default parameters

that you want to change, or you may choose to return to the MINIPROP Master

Menu.  If you make the latter choice, and you have changed any of the defaults,

you will be given the opportunity to save your changes in the MP.DEF defaults

file before the MINIPROP Master Menu is reloaded.

Change Antenna Minimum Radiation Angle Used in Predictions

----------------------------------------------------------

The minimum usable antenna radiation (take-off) angle depends primarily

upon antenna heights and horizon obstructions.  In both the propagation

predictions and the DX Compass, MINIPROP assumes this minimum angle to be 1.5

degrees unless you change it.  You may change the antenna minimum radiation

angle to any value between 0 and 45 degrees.  MINIPROP uses this value to

determine the minimum number of F hops required on a given path, and then

computes the actual antenna radiation angle from the resulting path geometry.

If you specify too great a radiation angle, the number of hops on a given path

may be excessive.  In this case, a screen message will advise you that the

prediction computation has been aborted.

The antenna minimum radiation angle currently in effect will be displayed.

You will then be asked if you want to change the angle.  If you answer "N", the

Defaults Change Menu will reappear.  If you enter "Y", you will be asked to

enter an angle between 0 and 45 degrees.

MINIPROP will display the new antenna minimum radiation angle that is now

in effect and will ask you if you want to change it.  If you are not satisfied

with the value, enter "Y" and you will again be asked to enter an angle.  When

you are satisfied with the displayed angle, enter "N" and the Defaults Change

Menu will reappear.  You may select other default parameters that you want to

change, or you may choose to return to the MINIPROP Master Menu.  If you make

the latter choice, and you have changed any of the defaults, you will be given

the opportunity to save your changes in the MP.DEF defaults file before the

-24-

MINIPROP Master Menu is reloaded.

Suppress Output of Low Signal Levels

------------------------------------

In the discussion of the prediction example in Section 4, it was stated

that signal levels below -20 dB with respect to 0.5 microvolt are normally not

displayed or sent to the printer because weaker signals are generally unusable

by radio amateurs.  However, listeners to shortwave broadcasting stations that

use high power and high-gain antennas might find it desirable to use a lower

threshold which, after correction for the power and antenna gain, might result

in a usable received signal strength.  You may set the suppression threshold to

0, -20, -40, or -60 dB according to your needs, or you may completely eliminate

suppression of the output of low signal levels.

The suppression level currently in effect will be displayed.  You will

then be asked if you want to change the suppression threshold.  If you answer

"N", the Defaults Change Menu will reappear.  If you enter "Y", the following

will appear.

A  Do not show signal levels that are below 0 dB

with respect to 0.5 microvolt.

B  Do not show signal levels that are below -20 dB

with respect to 0.5 microvolt. (Default)

C  Do not show signal levels that are below -40 dB

with respect to 0.5 microvolt.

D  Do not show signal levels that are below -60 dB

with respect to 0.5 microvolt.

E  Do not suppress output of low signal levels.

Your choice :

To make your choice, enter a letter, A through E.

MINIPROP will display the new suppression threshold that is now in effect

and will ask you if you want to change it.  If you are not satisfied with the

threshold, enter "Y" and you will again be asked to enter your choice of A

through E.  When you are satisfied with the new threshold, enter "N" and the

Defaults Change Menu will reappear.  You may select other default parameters

that you want to change, or you may choose to return to the MINIPROP Master

Menu.  If you make the latter choice, and you have changed any of the defaults,

you will be given the opportunity to save your changes in the MP.DEF defaults

file before the MINIPROP Master Menu is reloaded.

Suppress Output of Signal Levels at Frequencies Higher than Predicted FMUF

--------------------------------------------------------------------------

Signal level predictions are normally suppressed for frequencies greater

than 150 percent of the predicted FMUF, but you may cancel this suppression.

This is a toggle; the suppression is either on or off.

-25-

The  current setting of the toggle will be displayed.  You will then be

asked if you want to change the setting to the other state.  If you answer "N",

the Defaults Change Menu will reappear.  If you enter "Y", the setting will be

reversed.

MINIPROP will display the new toggle setting that is now in effect and

will ask you if you want to change it.  If you are not satisfied with the

setting, enter "Y" and the setting will again be reversed.  When you are

satisfied with the toggle setting, enter "N" and the Defaults Change Menu will

reappear.  You may select other default parameters that you want to change, or

you may choose to return to the MINIPROP Master Menu.  If you make the latter

choice, and you have changed any of the defaults, you will be given the

opportunity to save your changes in the MP.DEF defaults file before the

MINIPROP Master Menu is reloaded.

Change Path (Short or Long) to be Computed First

------------------------------------------------

Short-path predictions are normally computed first, long-path second.  You

may reverse this order.  This is a time saver if you need only the long-path

prediction.  This a toggle.

The  current setting of the toggle will be displayed.  You will then be

asked if you want to reverse the setting.  If you answer "N", the Defaults

Change Menu will reappear.  If you enter "Y", the setting will be reversed.

MINIPROP will display the new toggle setting that is now in effect and

will ask you if you want to change it.  If you are not satisfied with the

setting, enter "Y" and the setting will again be reversed.  When you are

satisfied with the toggle setting, enter "N" and the Defaults Change Menu will

reappear.  You may select other default parameters that you want to change, or

you may choose to return to the MINIPROP Master Menu.  If you make the latter

choice, and you have changed any of the defaults, you will be given the

opportunity to save your changes in the MP.DEF defaults file before the

MINIPROP Master Menu is reloaded.

Restore Original Set of MINIPROP Defaults

-----------------------------------------

The original set of defaults built into MINIPROP are:

Prediction frequencies of 3.6, 7.1, 14.1, 21.2, and 28.3 MHz

Antenna minimum elevation angle of 1.5 degrees

Suppression threshold for low signal levels at -20 dB

Output of signal levels suppressed for frequencies higher

than 150% of FMUF

Short-path predictions computed before long-path predictions

You will be asked if you want to restore this set of defaults.  If you

answer "N", the Defaults Change Menu will reappear.  You may select other

default parameters that you want to change, or you may choose to return to the

-26-

MINIPROP Master Menu.  If you enter "Y", the original settings will be restored

for the current MINIPROP session, and you will be asked to press any key before

returning to the Defaults Change Menu.  You may then select other default

parameters that you want to change, or you may choose to return to the MINIPROP

Master Menu at which time you will be given the opportunity to save your

changes in the MP.DEF defaults file before the MINIPROP Master Menu is

reloaded.

-27-

7. MINIPROP Utilities (Master Menu Choice 4)

--------------------------------------------

When you select Master Menu choice 4, the following MINIPROP Utilities

menu will appear.

MINIPROP (TM) UTILITIES

0.  Exit from MINIPROP (Quit)

1.  Return to MASTER MENU

2.  Create or modify Terminal A default

3.  Display entries in MINIPROP atlas

4.  Add, modify, or delete entry in MINIPROP atlas

5.  Display MINIPROP documentation on screen

6.  Send MINIPROP documentation to printer

7.  Print table of great-circle bearings

8.  Convert version 1 MINIPROP.ATL to version 2 MP.ATL

Your choice :

Use choice 0 to quit MINIPROP without first returning to the Master Menu.

To return to the MINIPROP Master Menu, use choice 1.  Choices 2 through 8 are

discussed in the following sections.

8. Create or Modify Terminal A Default (MINIPROP Utilities Choice 2)

--------------------------------------------------------------------

There is probably a location you will use most often as one of the

terminals when running a MINIPROP prediction.  This utility allows you to save

the latitude, longitude, and name of this location as the Terminal A default.

Then when you want to use this location in a MINIPROP prediction or in the DX

Compass, it will only be necessary to press <ENTER> or <RETURN> when you are

prompted for the latitude of Terminal A; you will not have to enter the

latitude, longitude, and name  each time.  The Terminal A Default is saved on

your disk in the MP.DEF file (the same file that holds the default values of

several of the prediction parameters).

When you select choice 2 on the MINIPROP Utilities menu, the latitude,

longitude, and name of any existing Terminal A default location will be

displayed.  Then you will be directed to enter your desired Terminal A default.

Enter the latitude, longitude and name the same as you would enter them

manually when running a MINIPROP prediction (Section 4 above).  When you have

finished making these entries, they will be repeated back on the screen, and

you will be asked if they are correct.  If you want to make any corrections,

answer "N".  In this case, you will again be asked to enter the latitude,

longitude, and name.  When the displayed entries are correct, answer "Y".  You

will then be asked to confirm that you want the entries saved in the MP.DEF

defaults file.  If you do want them saved, answer "Y"; but if you want to abort

the creation or change of the Terminal A default location, answer "N".

You will be directed to press any key to continue, and the MINIPROP

Utilities menu will reappear.

-28-

9. Display Entries in MINIPROP Atlas (MINIPROP Utilities Choice 3)

------------------------------------------------------------------

This choice allows you to view all of the entries in the MINIPROP atlas.

The entries will appear in alphabetical order of the prefix.  (ASCII

alphabetization is used.  Numerals precede letters.  The numeral 0 precedes the

numeral 1.)  The prefix, latitude, longitude, and name will be displayed for

each entry.  The scrolling of the display will stop after every 20 entries; to

continue, press any key.  The total number of atlas entries will be displayed

after the last atlas entry.  If you want to terminate the atlas display before

the last entry is reached, press Control-X (Hold down the CONTROL key; then

type "X".).

10. Add, Modify or Delete Entry in MINIPROP Atlas (MINIPROP Utilities Choice 4)

-------------------------------------------------------------------------------

You may add, modify, or delete entries in the MINIPROP atlas.  The only

limit on the number of atlas entries is the capacity of your disk.  When you

select this utility from the MINIPROP Utilities menu, the following message and

list of edit choices will appear on your screen.

You will be given a chance to abort before any change is made

in the atlas.

<A>dd an entry

<M>odify an entry

<D>elete an entry

<R>eturn to utilities menu

Your choice :

Enter "A", "M", or "D" to add, modify, or delete an atlas entry.  The list

of available edit choices will reappear after each addition, modification, or

deletion.  Enter "R" to return to the MINIPROP Utilities menu.

Adding an Atlas Entry

---------------------

You will first be asked for the prefix to be added to the atlas.  Prefixes

may contain from one to six characters.  Leading, trailing, and embedded spaces

are not allowed, and you will not be allowed to enter an all-numeric prefix.

Country prefixes, call area prefixes, station call signs, or any other unique

identifier may be used.  Letters may be entered in either upper or lower case;

MINIPROP will convert all letters of a prefix to upper case before the entry is

made in the atlas.

MINIPROP will check to make sure the new prefix is not already in the

atlas; if it is not, you will be asked to enter the latitude, longitude, and

name for the new entry.  Enter these in the same manner as they are manually

entered when you run a MINIPROP prediction (Section 4 above).  The latitude and

longitude may be entered as a decimal or in deg min sec format.  North

latitudes and west longitudes are positive (with no plus sign); south and east

are negative (with a minus sign).  Latitudes greater than 90 degrees in

magnitude, and longitudes greater than 360 degrees in magnitude, will not be

accepted.  Names may not exceed 17 characters in length, and may contain any

-29-

combination of characters.  You might want to look at some examples in the

atlas; use choice 4 in the MINIPROP Utilities menu.

After you have entered the name, your new atlas entry will be repeated

back on the screen, and you will be asked if it is correct.  If you want to

make any corrections, answer "N".  In this case, you will again be asked to

enter the prefix, latitude, longitude, and name.  When the displayed entry is

correct, answer "Y".  You will then be asked to confirm that you want the entry

put in the MINIPROP atlas file.  If you do want the entry put in the atlas

file, answer "Y"; but if you want to abort the addition to the atlas, answer

"N".  You will then be given the choice of adding, modifying, or deleting

another atlas entry or returning to the MINIPROP Utilities menu.

Modifying an Atlas Entry

------------------------

You will be asked for the prefix of the atlas entry that you want to

modify.  MINIPROP will then display the current contents of that entry, and you

will be asked to enter your changes.  If you do not want to change an item

(prefix, latitude, longitude, or name) within an entry, just press <ENTER> or

<RETURN> and the current contents for that item will appear on the screen.

When you do want to change an item, enter your desired contents for that item.

After the name is entered, the modified atlas entry will be repeated back on

the screen, and you will be asked if it is correct.  If you answer "N", the

modification will be aborted.  If the displayed entry is correct, answer "Y".

You will then be asked to confirm that you want the modified entry put in the

MINIPROP atlas file.  If you do want the modified entry put in the atlas file,

answer "Y"; but if you want to abort the modification, answer "N".  You will

then be given the choice of adding, modifying, or deleting another atlas entry

or returning to the MINIPROP Utilities menu.

Deleting an Atlas Entry

-----------------------

You will be asked for the prefix of the atlas entry that you want to

delete.  MINIPROP will then display the contents of that entry, and you will be

asked to confirm that you want the entry deleted.  If you do want the entry

deleted from the atlas file, answer "Y"; but if you want to abort the deletion,

answer "N".  You will then be given the choice of adding, modifying, or

deleting another atlas entry or returning to the MINIPROP Utilities menu.

11. Display MINIPROP Documentation on Screen (MINIPROP Utilities Choice 5)

--------------------------------------------------------------------------

Use this choice in the MINIPROP Utilities menu if you want to read this

documentation on the screen.  The page number at the bottom of each page will

not appear on the screen, and where the documentation contains multiple

adjacent blank lines, only one blank line will appear on the screen.  The last

line of documentation on each screen will be repeated at the top of the next

screen.  The scrolling of the display will stop after every 22 lines; to

continue, press any key.  This documentation is lengthy, so it will take a

while to read it all.  If you want to terminate before the end of the

documentation is reached, press Control-X (Hold down the CONTROL key; then type

"X".).

-30-

12. Send MINIPROP Documentation to Printer (MINIPROP Utilities Choice 6)

------------------------------------------------------------------------

When you make this choice in the MINIPROP Utilities menu, the following

instructions will appear on the screen.

Please turn on your printer and set it for 80 characters per

line and 66 lines per page.  Adjust the paper so printing

can start within 10 lines from the top of the page.  There

will be as many as 57 printed lines on each page.

Press any key when printer is ready.

Just follow these instructions.  The documentation is lengthy, so be sure

there is plenty of paper in your printer.  (The total number of pages will

appear at the bottom of the first page.)  Printing will begin when you press

any key.  You may interrupt printing (and return to the MINIPROP Utilities

menu) by pressing Control-X.

13. Print Table of Great-Circle Bearings (MINIPROP Utilities Choice 7)

----------------------------------------------------------------------

This choice in the MINIPROP Utilities menu allows you to print a

customized table of great-circle bearings (beam headings) from any location

(Terminal A) to all of the locations in the MINIPROP atlas.

If you have saved a Terminal A default location in the MP.DEF defaults

file, you will be asked if you want the bearings to be from the default

location.  If you answer "N", or if their is no default location in the

defaults file, you will be asked to enter the latitude, longitude, and name of

Terminal A.  Enter these as described in Section 4.

You will now be asked for information needed by MINIPROP to optimize the

printing of the table by your printer.  You will be asked first for the number

of lines per page (form length) for your printer with the paper you will be

using.  (66 lines per page is standard in the U.S.)  You will be asked next for

the maximum number of characters that your printer will print on one line.

Then you will be given the opportunity to send any control codes that may be

required to configure your printer.  But first turn on your printer and adjust

the paper so printing can start 4 to 6 lines from the top of the page.  Press

any key when ready.

Now enter the number of lines per page (form length) for your printer with

the paper you will be using.  This value must be an integer between 48 and 126.

The U.S. standard of 66 lines per page will allow 52 bearings to be printed in

each column.

The heading and first few entries of a column in the printed table might

look like this:

PREFIX LOCATION          DEG

1A0    S.M.O. of Malta    34

1S     Spratly Is.       302

3A     Monaco             36

-31-

The number of pages required to hold the table of great-circle bearings depends

upon the number of characters that your printer can print on one line.  A

larger number of characters per line will allow a larger number of columns on a

page, and fewer pages will be needed to hold the complete table.  A table will

appear on the screen showing how the maximum number of characters per line

affects the number of columns of great-circle bearings that will be printed on

each page and the number of pages that will be required.  If your printer

prints 80 characters on a line, only two columns will fit on a page.  If your

printer has more than one character width, you will probably want to specify

the number of characters corresponding to the narrowest character width.  For

example, an OKIDATA ML92 printer can print 80 characters per line at 10

characters per inch (cpi), 96 at 12 cpi, and 136 at 17.1 cpi.  The best choice

is 136; this allows four columns to be printed on each page.  Be sure to

specify an exact, instead of approximate, number of characters per line so

MINIPROP can center the columns correctly on the page.

You will next be asked if you want to send control codes to your printer.

Control codes are special non-printable codes -- usually numbered 1 through 31

(decimal) -- that tell your printer to change the way it prints.  For example,

control code 29 must be sent to the ML92 printer in the above example to cause

it to switch to 136 characters per line. Control code 27 is often combined with

other codes into what are called "escape sequences".  Your printer user's

manual will tell you what control codes or escape sequences, if any, are needed

to set up your printer for the number of characters per line you have

specified, to choose a type style, or for another purpose.  If you answer "N"

(you do not want to send control codes), printing will begin.  If you answer

"Y" (you want to send control codes), you will be directed to enter the first

control code followed by <ENTER> or <RETURN>.  Enter the integer decimal value

of the first code you want sent.  MINIPROP will send the code to your printer

and will ask you for another control code.  Enter as many codes as are required

by your printer.  Press only <ENTER> or <RETURN> to tell MINIPROP you are

finished.  Printing will begin.  You may interrupt printing (and return to the

MINIPROP Utilities menu) by pressing Control-X.

The entries in the printed table will appear in alphabetical order of the

prefix.  (ASCII alphabetization is used.  Numerals precede letters.  The

numeral 0 precedes the numeral 1.)  The bearings are measured in degrees east

of north, and are in the range 0 through 359 degrees.  However, if Terminal A

is at either the north or south pole, the west longitudes (0 through 359

degrees) of  the atlas entries are printed in the DEG column.  Three asterisks

(***) are printed in the DEG column when Terminal A and an atlas entry are co-

located or when Terminal A and the atlas entry are antipodes.

14. Convert Version 1 MINIPROP.ATL to Version 2 MP.ATL

------------------------------------------------------

(MINIPROP Utilities Choice 8)

-----------------------------

An atlas file containing latitudes and longitudes for all DXCC countries

and several additional geographic locations is supplied with this version of

MINIPROP.  If you already have such an atlas, named MINIPROP.ATL, that you used

with MINIPROP version 1, you can use it with version 2 but its internal format

must be changed before it can be read by the new version of MINIPROP.  This

utility will do the conversion for you.

-32-

MINIPROP will look for your MINIPROP.ATL file on the logged disk drive

and in the current directory that should also contain the MINIPROP version 2

files. The new atlas file will be created in the same directory and will be

named MP.ATL.  The new MP.ATL will occupy somewhat more disk space than does

your MINIPROP.ATL file; be sure there is enough space for it on your disk.  Any

existing file named MP.ATL will be renamed to MPBACKUP.ATL, and any existing

file named MPBACKUP.ATL will be erased. Your MINIPROP.ATL file will remain

intact.  The conversion will abort if your atlas contains more than 800 entries

or an invalid latitude or longitude, or if a prefix contains leading, trailing,

or embedded spaces, is longer than six characters, or is all-numeric.

When you select this utility, you will be asked if you want to proceed

with the conversion.  Answer "N" if you want to abort the conversion.  The

atlas will be converted if you answer "Y".  The conversion may take a while if

you have a large atlas or a slow disk drive.

15. References

--------------

1. National Bureau of Standards, "Ionospheric Radio Propagation", Circular 462,

1948.

2. Davies, K., "Ionospheric Radio Propagation", National Bureau of Standards

Monograph 80, 1965.

3. Lucas, D.L., and Haydon, G.W., "Predicting Statistical Performance Indexes

for High Frequency Ionospheric Telecommunication Systems", Technical Report

ITSA-1, Institute for Telecommunication Sciences and Aeronomy, Environmental

Science Services Administration, 1966.

4. Barghausen, A.F., Finney, J.W., Proctor, L.L., and Schultz, L.D.,

"Predicting Long-Term Operational Parameters of High-Frequency Sky-Wave

Telecommunication Systems", Technical Report ERL 110-ITS 78, Institute for

Telecommunication Sciences, Environmental Science Services Administration,

1969.

5. National Bureau of Standards, "Handbook for CRPL Ionospheric Predictions

Based on Numerical Methods of Mapping", Handbook 90, 1962.

6. Leftin, M., "The Estimation of Maximum Usable Frequencies from World Maps of

MUF(ZERO)F2, MUF(4000)F2 and MUF(2000)E", Telecommunications Research and

Engineering Report 13, Institute for Telecommunication Sciences, Office of

Telecommunications, 1971.

7. Rose, R.B., Martin, J.N., and Levine, P.H., "MINIMUF-3: A Simplified HF MUF

Prediction Algorithm", Technical Report 186, Naval Ocean Systems Center, 1978.

8. Rose, R.B., and Martin, J.N., "MINIMUF-3.5: Improved Version of MINIMUF-3, A

Simplified HF MUF Prediction Algorithm", Technical Document 201, Naval Ocean

Systems Center, 1978.

9. Rose, R.B., "MINIMUF: A Simplified MUF-Predicting Program for

Microcomputers", pp. 36-38, QST, December, 1982.

-33-

10. Shallon, S.C., "MINIMUF for Polar Paths", p. 48 (Technical Correspondence),

QST, October, 1983.

11. Rose, R.B., "MINIMUF Revisited", p. 46 (Technical Correspondence), QST,

March, 1984.

12. Fricker, R., "A Microcomputer Program for the Critical Frequency and Height

of the F Layer of the Ionosphere", pp. 546-550, Fourth International Conference

on Antennas and Propagation, Institution of Electrical Engineers (UK), 1985.

13. Fricker, R., "A Microcomputer Method for Predicting the Field Strengths of

HF Broadcast Transmissions", to be published.

14. International Radio Consultative Committee, "CCIR Atlas of ionospheric

characteristics", Report 340, International Telecommunication Union, Geneva,

1983.

15. Jacobs, G. and Cohen, T.J., "The Shortwave Propagation Handbook", 2nd ed.,

CQ Publishing Inc., 1982.

16. "The 1987 ARRL Handbook for the Radio Amateur",  64th ed., American Radio

Relay League, 1986.

-34-

###############################################r, 1983.

11. Rose, R.B., "MINIMUF Revisited", p. 46 (Technical Correspondence), QST,

March, 1984.

12. Fricker, R., "A Microcomputer Program for the Critical Frequency and Height

of the F Layer of the Ionosphere", pp. 546-550, Fourth International Conference

on Antennas and Propagation, Institution of Electrical Engineers (UK), 1985.

13. Fricker, R., "A Microcomputer Method for Predicting the Field Strengths of

HF Broadcast Transmissions", to be published.

14. International Radio Consultative Committee, "CCIR Atlas of ionospheric

characteristics", Report 340, International Telecommunication Union, Geneva,

1983.

1