BAUDSET.DOC  from BAUDSET.LBR

SUBJECT: BAUDSET.COM, baudrate setting program created using

IMP overlays

IMP is Irv Hoff's modem communication program.	IMP overlays can be

found on most public RCP/M systems.  BAUDSET uses the overlays, not

the IMP program itself.

BAUDSET is a program that can be used to set baudrates for a wide

variety of CP/M computer systems.  It requires that the IMP overlay

specific for your computer system be overlaid on the BAUDSET.COM

distributed with this .LBR file.  Procedure for overlayment is

described in the header of each IMP overlay source file, and

consists of steps (3) and (4) in the instructions below contained

in the section entitled "TO ASSEMBLE AND OVERLAY".

BAUDSET achieves its great generality by virtue of ability to use

any standard IMP overlay.  This will allow setting of baudrates

to whatever settings are allowed in the overlay.

If this program fails in any way to function properly with any

sound IMP overlay, please let me know.	I will do what I can to

fix BAUDSET so all who are interested in the program and what it

does can benefit.

26-Sep-87	Jerry Levy

1129 Dundee Drive

Dresher, PA 19025

215 657-0898 (voice)

BAUDSET identifies your system by making use of the sign-on

string in the overlay.	Remember to look over the IMP overlay and

set any equates neded to customize for your setup before

assembling.  It is suggested you customize even though most

settings are not required by the few subroutines used by us.

Some things, however, like slot number, must be set correctly if

your system uses them.

I thought this approach was a neat way to arrive at a fairly

general baudrate setting program even if most IMP inserts don't

allow for the complete range of standard baudrates.  It's easy to

add new ones.  As for me, I needed something to reset baudrates

for non-modem use of my serial I/O port.  In my case I added all

other baudrates I needed and I also added 7E1 and 8N1 data-word-

format selection which additionally I had a need for.

OVERLAY SIZE:  The standard IMP overlay, for use with IMP itself,

cannot extend beyond 3FFH (or is it 400H).  It loads from 0100H-3FFH.

BAUDSET can accept larger overlays (that run from 0100H-059FH).

This allows for addition of more baudrates, etc., to the overlay.

Not unreasonable since we are using it as a baudrate setter.

Please contact me if you encounter any problem with this program

for any IMP overlay.  I will adjust this program as necessary.

TO ASSEMBLE AND OVERLAY

You need MAC and RMAC (DRI)

LINK	 (DRI)

MLOAD	 (NightOwl Software)

SYSLIB.REL (Echelon)

Edit the IMP overlay exactly as you would to install IMP.	Then:

(1) RMAC BAUDSET $PZSZ

(2) LINK BAUDSET,SYSLIB[S]

(3) MAC IMPOVLY.ASM $PZSZ	Use whatever overlay applies

(4) MLOAD BAUDSET.COM=BAUDSET.COM,IMPOVLY.HEX

After overlayment, affords a ready-to-use overlaid file named

BAUDSET.COM.

Any overlaid file may be re-overlaid with another IMP overlay by

doing steps (3) and (4) without repeating steps (1) and (2).

FILES IN BAUDSET.LBR

BAUDSET.DOC	This File

BAUDSET.ASM	Source for the main program

BAUDSET.COM	Baudsetting program customized for ALS CP/M Card on

Apple ][ with Apple Super Serial Card.	The overlay

used for this is an elaborated version of the I2AP-10

overlay that pertains to a whole host of Apple ][

CP/M cards and serial I/O cards.  You should overlay

this with the IMP overlay .HEX file for your computer

to customize for your particular computer setup.

IMP-OVL.HEX	From I2AP-10, but after modified by adding baudrates,

etc.  For Apple ][ with ALS CP/M Card and Apple SSC.

As large as we can be and still be usable as IMP

overlay.  You can use this with IMP or BAUDSET.

BDSETOVL.HEX	Same, but has more settings.  Too large for IMP but

OK to use with BAUDSET.COM.  This is what was used

as overlay to arrive at BAUDSET.COM, above.

MLOAD24.COM	To use for doing overlayment.

-JL

############