Following is the documentation provided by Dysan Corporation for

use with their Digital Diagnostic Diskette (DDD).

Using the DDD, it is possible to align most floppy drives

with no test equipment - other than your computer and a

screwdriver or two.

The program DDD.ASM is a generic one, and is useless until

modified for a specific computer or controller board.  We will

be maintaining copies of all of these "specific" versions on

Technical CBBS ( (313) 846-6127 ), so if you modify this program

for your own use, please pass a copy along to us there, so that

others may also benefit from Dysan's contribution of this

program to the public.

-Dave Hardy, CDP Corp., Technical CBBS

<><><><><><><><><><><><><><><><><><><><><><><><><><><><><><><><>

Requirements for using Dysan's Diagnostic Program

DYSAN CORPORATION

CE DIVISION

DRIVE DIAGNOSTIC PROGRAM

for CP/M-80

The Drive Diagnostic Program is a sample program showing how

the Digital Diagnostic Diskette (DDD) can be used for Floppy

Drive checking.

The program requires the following hardware to run:

CRT with cursor positioning and clear screen commands.

(The program is currently configured for a Hazeltine 1500.)

CP/M-80 system with at least 32k of memory.

At least one 8" or 5.25" floppy disk drive.

This source program is provided on an 8" single density, one-

sided diskette that is compatible with the CP/M-80 operating

system.  The program provided will not run "as-is"; it must

be modified by someone experienced in 8080 Assembly language,

CP/M-80 and disk drive controller interfacing.

In order to make the program run with the target system, there

are four assembly language subroutines that must be modified

to interface with the system's disk controller/drive(s):

1) "SELECT" - Selects and Homes the drive.

2) "TRACK"  - Seeks to the track number found in register A.

3) "HOME"   - Restores the selected drive's head to track 0.

4) "READ"   - Reads sector in register "A" and exits with

error status.

The above routines must be coded within the Diagnostic Program,

instead of calling the routines through the BIOS, because the

BIOS may interfere with disk I/O operation (either by trapping

disk errors or deblocking sector calls, i.e. where sector sizes

are greater than 128 bytes).

The sample program was coded for use with the Western Digital

FD-1793 floppy disk controller IC.  The program will work with

other controllers (i.e. NEC uPD765 and Intel 8272 FDCs) by

modifying the source program, as indicated above.  Because some

disk controllers don't allow programmed access to the drive's

INDEX signal (i.e. NEC and Intel), "Index Timing" and "Spindle

Speed" testing are not possible with the current program.

#################################################################################