UNLOAD22

UNLOAD22 is derived from UNLOAD21, a venerable P.D. program that

converts .COM files to .HEX files, the opposite of Digital Research's

LOAD.COM.  This is useful if you want to send the data to a PROM

programmer, send it to a 7 bit device like a paper tape punch, (in

1987?) or down a serial link where the .HEX file's built in checksum

gives some degree of confidence in an ASCII transfer.

Having used UNLOAD21 on a regular basis, I quickly found some

deficiencies.  The most annoying was the fact that UNLOAD21 forced the

input file to be a .COM file.  Not defaulted, FORCED.  As I was often

UNLOADing files that weren't even 8080/Z-80 code, I didn't want these

non-executable files called .COM files on my CP/M disk.  Makes for

confusion, I prefer to call these files .CIM for Core IMage.  The next

problem I found was in the EOF record.  UNLOAD21 output an EOF record

(End Of File record) consisting of ten (10) ASCII zeros.  This is also

the EOF record output by ASM.COM, MAC.COM, & maybe others.  This type of

EOF record does not meet the Intel specification, and caused at least

one PROM programmer I was working with to "choke".  The third problem

was mainly one of preference.  A previous programmer had changed

UNLOAD21 to output 32 byte data records, to conserve disk space.  This

is perfectly legal, however it does reduce the reliability of the

checksum.  I prefer the more standard 16 byte data record.

All the above problems have been fixed with UNLOAD22!  The program

defaults to a .COM filetype for the input file, but allows .CIM or other

input filetypes as well.  The EOF record now meets the Intel

specification.  The user now has the option of setting the output data

record size.  (Defaults to 16)

Invoke as follows:

UNLOAD22 FILENAME<.EXT> <ADDR></X>

Where:

FILENAME  is the name of the file you wish to convert

<.EXT>  is the extension, if not given, assumes .COM

<ADDR>  is the load address, if not given defaults to 0100H

</X>  is the data record size, if given after an address

parameter, there must be no spaces!  Where X:

/S   Small, 8 bytes per data record, for max reliability

(Within reason) of checksum.

/M   Medium, 16 bytes per data record, the old standby.

/L   Large, 32 bytes per data record, some prefer it because

the resultant file is smaller.

/E   Extra large, 64 bytes per data record.  Produces even

smaller files.

Notes by Mark D. Pickerill, SYSOP Hacker Heaven BBS, (408) 646-3547

9/5/87

##################################################################################################