MORE THAN TWO SYSTEM TRACKS FOR CP/M 2.2 - A PRACTICAL SOLUTION

By Donald E. Killen

For those of you out there in CP/M-land who want three (or more) system

tracks but are tired of asking "HOW", here is one way.  It works in

conjunction with the program "SGEN.COM" which I have provided the Group

with.

For openers, when I found myself running out of BIOS space (CP/M 2.2 is

two sectors WORSE than good ol' 1.4 on this count), I asked myself,

"Self, how are you going to GET all the nice stuff you write in

assembler code ON a floppy, using Digital Research's SYSGEN (or a

"GETSYS/ PUTSYS" combination of some sort)?" And I answered - "with

difficulty".

Next, I found myself musing about compatibility with the "two-tracked"

world once I did manage to build 3 or more system tracks and use the

thing successfully.  So again I said, "Self, how are you going to get

out of THAT one?" And I answered "With difficulty".

Just to show you how unreliable first impressions can be, I looked

closer at the documentation on CP/M 2.2 (As an avid non-reader of most

documentation, I did what I usually did, search for the pictures, found

none of those, and started from the back, reading towards the front.)

About three one morning I said "AHA" - I know what I'll do - I'll fool

the system by defining (in 2.2) TWO sets of logical disk drives, and

"fold" one set over the other such that there is one set of physical

drives, each drive of which has two logical definitions, either of which

can be specified at run time.

I defined eight physical drives, 0 through 7.  Drives 0, 1, 2, and 3 are

defined just like the others except that three (3) tracks are "reserved"

- i.e. used for the "system", which includes my gigantic BIOS.  Drives

4, 5, 6, and 7 use two (2) system tracks.  Selecting drive "A" in my

system gets a 3-track drive, selecting "E" gets you the SAME physical

drive, only it is known (to CP/M) to have only two system tracks.  Thus

I can be compatible with certain 2-tracked worlds with only one drive,

if I must.  Or I can use up to four drives and mix them any way I want.

One EASY way to tell if you've forgotten and selected a two-track drive

which has a 3-track floppy in it is that DIR gets you a lot of nulls or

"NO FILE", depending upon whims, etc.This works the other way around

also.

Physical Drive  0       1       2       3

----------------------------------------------

3-track drive:  A       B       C       D

2-track drive:  E       F       G       H

----------------------------------------------

The chart above is a handy reference and serves to illustrate the

functionality of this scheme.  However, one further point has to be

mentioned.  Your average (e.g. Tarbell) Floppy Disk Interface can

select one of a maximum of FOUR drives.  Therefore, I had to perform

minor surgery on the SELDSK routine in BIOS.  Just at the point that

SELDSK is about to ship off two bits specifying which of four drives you

want, the third (hi-order, or bit 2) bit gets discarded.  Up to that

point, and for that matter, in RAM until changed, are three bits

defining the LOGICAL drive requested.  That accesses the tables (in CP/M

2.2) which define the disk parameters.  Then, for the purposes of

limited controllers (like the Tarbell Single Density controller), only

the least significant two bits are used - hence a 4 looks like a 0, a 5

like a 1, and so-forth.

If anyone is really interested in the gory details, Charlie Foster has a

disk with a copy of my BIOS (KBIOS.MAC/PRN) - which might convey the

details in more depth.

Once CP/M has this capability, I next needed to refine the program I

call SGEN.  SGEN.COM will do anything Digital Research's SYSGEN will do,

plus it is an "n-track" system in that it asks you up front how many

tracks (system tracks) you want to copy.  It works with 1-9 tracks

(NOTE: If anyone comes up with a one-track system, let me know, o.k.).

It makes you say "y" (or "Y") to make doubly sure, then copies from

memory or disk to memory or disk - just like SYSGEN.  You'll also need

the upgraded versions of MACRO.LIB, which, after converting nearly

everything to Zilog notation, is called MACROZ.MAC, and I believe

CLIB.MAC is also used.  The latter is a clutter of miscellany which I've

put together for assembly development work.

I have no idea what one would do with regard to defining drives "sort

of" like the above only for CP/M 1.4 - but I'd like to hear any comments

anyone has.

EDITOR'S NOTE: Don Killen is willing to discuss this (or any other

items of interest) with anyone but prefers a note in the mail

however he doesn't really mind the phone at all.  So----

Donald Killen

2101 Hickory Drive  Home: (214) 242-4968

Carrollton, Tx 75006

#####################################################################################################################