***************

* C B I O S R *

***************

COPYRIGHT

=========

CBIOSR is copyright (c) 1986 Michael Liddle.  It is released to

the public domain for non commercial usage and may be freely copied and

used by anyone who wishes to do so providing this is not done for

commercial or other gain.

INTRODUCTION

============

CBIOSR is a BIOS (Baisc Input Output System) for use in

conjunction with CP/M 2.2 on pre 84 series Kaypro computers.  It provides

all the facilities of the standard BIOS plus supporting a 191K RAM DISK as

drive C.  In order to use CBIOSR you need to replace your 64K RAM chips

with 256K RAM chips and add extra address decoding and refresh as described

by me in Micro Cornucopia.

Use of CBIOSR entails a loss of 1K of the transient program area.

This is unlikely to be a problem.

USAGE

=====

Once installed and saved on the sytem tracks of a floppy disk

the RAM DISK routines are loaded at cold boot along with the rest of the

operating system.  The RAM disk can be accessed as drive C: in the same way

as drives A: and B: are used.  However the RAM disc is a lot faster and is

an ideal place to put overlay and swap files.

Data on the RAM disk is volatile and will be lost when the

computer is powered down.  Therefore it is essential to save any files to a

floppy at the end of each session.

Rebooting the computer using the reset button does not affect

data on the RAM disK.  This is because the cold boot routine does not

initialise the RAMDRIVE.  As a result if a program crashes and you have to

reboot you will usually find all your data intact (rarely a rogue program

may corrupt the RAM disk).  However this means that before use the disk

must be initialised by a special program call RAMINIT.COM.  This is done by

typing RAMINIT in response to the A> prompt from CP/M.  RAMINIT can also be

used in place of ERA C:*.* to erase all files on the disk.

THEORY OF OPERATION

===================

Most 8 bit processors are limited to an address range of 64K.

This is because they have 16 bit internal registers for addresses and 16

address lines (2^16=64K).  A few of the more modern 8 bit processors have

extra address lines to allow RAM addresses greater than 64K.  Likewise the

8088 series (arguably also 8 bit) implement 64K segments.

One way to allow processors such as the Z80, 8080, 6502, etc to

address more than 64K is through bank switching.  Bank switching controls

the system memory (RAM and ROM) in such a way that different 'banks` of

memory can be switched into and out of the address map so that although

only 64K of memory is addressable at any one time the amount of memory in

the system can be far more than this limit.  Perhaps the best example is

that of the system ROM in many machines which is switched in at address

zero on cold boot, reads a bootstrap into RAM and then jumps to that

bootstrap which disables the ROM replacing it by RAM.

Just as most 8 bit processors were designed to address a maximum

of 64K so most 8 bit operating systems and application programs are limited

to this.  One exception is CP/M plus (version 3) which can use extra RAM

banks as buffers.  However CP/M+ has disadvantages not least being cost and

compatibility.  Therefore to make use of extra RAM it is usually necessary

to modify the hardware dependent BIOS.  Extra RAM may be used by the BIOS

in a number of ways.  For peripheral buffers to speed I/O response e.g.

printer buffer, track buffering on disks, etc.  One easy to implement and

worthwhile use for extra ram is to simulate a very very fast disk drive.

CBIOSR is a fairly standard CP/M BIOS.  However the disk related

routines (Home, SetTrk, Read, etc) contain code to fool the CP/M BDOS into

thinking tyhere is a floppy disk called drive 'C' when in fact this is five

32 kilobyte banks of RAM.

SetTrk, SetSec and Home are used to position a pointer to the

part of the RAM containing data rather than positioning a read/write head

over a physical floppy disk.  Read and write then transfer data from and to

the area of the TPA selected by the application program via the SetDMA

routine.

A 128 byte 'sector' buffer in the BIOS is used as an intermediate

storage for data written to or read from drive C:.  This is necessary since

the lower area of the TPA (usually containing the users disk buffer at

080H) is switched out during accesses to the RAM disk, the top 32K of RAM

is allways selected.

INSTALLATION

============

On this disk are the files CBIOSR.Z80 and CBIOSR.HEX which

contain the source and object code of the RAM DISK BIOS.  To install CBIOSR

you will need a system disk containing the following files:

CBIOSR.HEX

DDT.COM

SYSGEN.COM

MOVCPM.COM

Below is a step by step guide.  Lines begining with the A> prompt

are those typed at operating system (CP/M) level whilst the rest are

responses to the various programs used.  Output from these programs is not

shown.

o Step 1).  First obtain a copy of CP/M using MOVCPM and saving it as follows:

A>MOVCPM 63 *

A>SAVE 34 CPM63.COM

o Step 2).  Now overlay it with CBIOSR.HEX using DDT and save the patched

version:

A>DDT CPM63.COM

S906

34

.

ICBIOSR.HEX

R2980

G0

A>SAVE 34 CPM63.COM

o Step 3).  Finally install it on a disc with SYSGEN.

A>SYSGEN CPM63.COM

##################################