Z3BIOS4G

This is the BIOS I use day in day out.  It supports ZCPR3, a

27.5K print buffer, a 159K RAM disk and it buffers the BDOS and CCP making

warm boots faster and independant of having system tracks on the disk in

drive A:.

This uses an initialisation program RAMDINITD and the ramdrive is

D: NOT C:. (D for DRAM!).

Version 1.4g is different from previous versions in that the RAM

DISK is now handled by the ordinary BIOS routines Home, SetSector, etc

rather than having two sets of routines and swapping part of the BIOS jump

table.  I did this to make the system more compatible with SPOOL, UNIFORM,

etc which also modify the jump table.  It is slower than my original BIOS

but only by a fraction of a percent!!!

This BIOS was never meant to be seen by anyone except me.  As a

result it is messy and hard to follow in places as part of the documentation

is stored in HAM (human access memory).  However anyone intending to implement

ZCPR3 may find it interesting.

Z3BIOS4G is big because it contains an IOP, an ENV and a TCP as a

result it needs a patched version of Sysgen called sysgen8 which copies

more sectors.#nd independant of having system tracks on the disk in

drive A:.