*** NOTE ***   12/11/84   DEVICE.DOC

The  December  issue of BYTE magazine contains an  excellent

article  by Steve Ciarcia (as usual) on using micros  to  control

the real world.

--J.F.

*** NOTE ***   11/25/84

I   just  glanced  through  the  December/January  issue  of

Profiles  and saw a "home control system" for your Kaypro in  the

"New Products" column.   While the DEVICE setup described in this

file  is no where near as complex as the system described in  the

magazine,  it doesn't carry a $1500.00 price tag either.  I think

that every computer owner,  at one time or another,  dreams of  a

"computerized" household. DEVICE is just my way of automating the

troublesome  task  of  hooking  up  various  printers  and  other

peripherals  while,  in a rather small fashion,  fulfilling  this

dream.   If  you  are of the more ambitious sort and  plan  on  a

similar,  but  more  extensive  system,  I  hope DEVICE can be of

some  small help to you,  possibly as a starting point.   In  any

case, good luck and happy computing!

--J.F.

DEVICE.DOC -- Documentation for DEVICE vers 1.1

November 21, 1984

(for use with pre-'84 Kaypro IIs and 4s only)

The longer that I own a computer,  the more gizmos, gadgets,

and do-hickeys I accumulate for it.  Of course, every time I want

to  use one of these assorted goodies,  I have to swap a  zillion

cables and extension cords.

I looked at various RS-232 switchers, etc,  but none of them

seemed to satisfy my (or my wallet's) requirements.  Some digging

around (my favorite pastime!) showed that my Kaypro 4 had two Z80

PIOs,   but   only used one side of each.   I decided that one of

these would satisfy my "switching" needs nicely.

HARDWARE

--------

I selected the unused B side of the general purpose PIO (the

A  side  is used for the Centronics printer port) as  my  victim.

Bits 0-7 of this spare parallel port are available at solder pads

E7-E14 on the main board (next to U54) of older (pre-'84) Kaypros

(see  the  table below),  and can be brought to the back of  your

Kaypro,  along with +5 volts D.C.  and supply ground,  using some

ribbon cable and a small "D" type connector.

bit #  - solder pad

-----    ----------

0         E14

1         E13

2         E12

3         E11

4         E10

5         E9

6         E8

7         E7

Each  of  these eight lines can be used to  control  various

peripherals,  etc.  in a variety of ways.  The following circuit,

for  example,  allows  your Kaypro to control the  flow  of  A.C.

current to a given load.

inverter        +5              ----------+------->120

|    ______    |         |M2   __>VAC

|\            ---1| MOC  |6---         |     |

| \    200 ohm    | 3010 |           TRIAC   |

cntrl line---|  >O--/\/\/\----2|      |   r2*   G / |     |

| /               |      |4--/\/\/\-/  |M1   |

|/                 ------              |     |

* - value depends on triac's      -LOAD--

gate current

The  inverter  allows the PIO to drive the  LED  inside  the

opto-coupler,  which, by the way, provides a very comforting 7500

volts  of isolation between your computer and the 120 volts A.C..

You could substitute a relay for the triac or use the circuit  to

control  a  lower A.C.  voltage (for low voltage  lamps,  motors,

etc.).

The  following  two examples show how to control  either  an

analog or digital signal buss.

-----

8 digital lines in->|     |>-8 digital lines out (buffered)

|     |

e2|     |

|\       ----|     |

| \      | e1|     |  74LS244

cntrl line---|  >O----+---|     |               Digital "Switch"

| /           -----

|/ inverter

----------

4 lines analog  |          |  4 lines analog

data in-------->|          |>-data out

|          |

|\        ----------  4066

| \      A| B| C| D|           Analog "Switch"

cntrl line---|  >------+--+--+--+

| /

|/ buffer

By using variations of the above techniques,  I have set  up

my  work  station  so  that I can manipulate  or  enable  various

peripherals,  including my  printers,  an  EPROM  programmer,   a

modem, and a speech synthesizer without leaving the keyboard.

SOFTWARE

--------

Using  the  spare parallel port is quite  straight  forward.

The data address is 0AH.  The control address is 0BH.  If you are

not  familiar  with programming the Z80 PIO I would suggest  that

you  pick  up a copy of the Mostek data book,  it seems to  be  a

little easier to decipher than the one available from Zilog.  The

following   partial   Z80  listing  sets  up  the  PIO   in   the

bidirectional mode and then sets up all 8 bits as outputs.

PORT    EQU     0AH    ;SPARE parallel port

CPORT   EQU     0BH    ;PIO control port

CWORD   EQU     0CFH   ;control word to set spare port to

;                       bidirectional mode.  the PIO expects

;                       another word after this one to set the

;                       individual bits to either INPUT or OUTPUT

;                              0 in bit pos. = OUTPUT

;                              1 in bit pos. = INPUT

;    initialization

LD      A,CWORD     ;cntrl word in A

OUT     (CPORT),A   ;output to cntrl port

LD      A,00H       ;ALL outputs

OUT     (CPORT),A

;

;    output info to port

LD      A,byte 1        ;first byte to output

OUT     (PORT),A

LD      A,byte 2        ;second byte to output

OUT     (PORT),A

.        .

.        .

.        .

and so on

DEVICE.COM is a simple  menu driven program  that allows you

to  toggle  "devices" 1 thru 8 ON or  OFF (bits 0 thru 7  of  the

spare  port  HI  or  LOW)  from  the  keyboard.    Simply  enter:

DEVICE<cr>  to enter the program and either ^C or <ESC> to  exit.

Any  settings  that  you make using DEVICE  will  remain  in

effect until you either change them with DEVICE,  press the reset

button (Cold Boot), or turn your computer OFF.

You can strike any key to cancel the pause after the sign-on

message  (striking  any key also cancels the pause  after  DEVICE

informs  you of an improper  entry).   You can also remove either

the sign-on message or the improper entry message (or both)  with

the patches listed at the end of this file.

I  have  been using the same basic setup for close to  eight

months now,  and have had no problems with it.   For me at least,

the  convenience provided has been well worth the effort it  took

to get everything "up and running".   I would enjoy hearing  from

you  if  you set up a similar system or are using your  micro  in

some other type of "real world" control.  Just leave me a message

on the Downey RCPM  (213) 806-2226.

ENJOY!

Joe Fitzpatrick

DEVICE.COM PATCHES

---------- -------

Remove sign-on message:

change  0169 from 11H to 18H

016A from CAH to 07H

Remove improper entry message:

change  020E from 11H to 18H

020F from 07H to 0EH

If  you  simply want to change the pause length (default  is

approximately  2 seconds on a 5 Mhz Kaypro) change the hex  value

at 027A (currently 07H) to a lower value to shorten the pause, or

a higher value to lengthen it.

Note about DEVICE on PRO8 systems:

If you are using the PRO8 monitor ROM from Micro  Cornucopia

(a great magazine!),  DEVICE changes  your cursor  to   invisible

(SPACE) until you exit the program,  at which time your cursor is

restored.   This  operation should have no nasty side effects  on

non-PRO8  systems.  If  one  should  turn  up  however,  you  may

remove this feature by changing the ten bytes from  015F  to 0168

to 00H and the seven bytes from 023B to 0241 to 00H.#6.