COPYRIGHT (C) 1984 BY GENE HEAD  ALL RIGHTS RESERVED

SCHEDULED TO APPEAR IN THE AUGUST ISSUE OF DDJ

More dBASE II Programming Techniques	Gene Head    6-13-84

In my last article (DDJ #92 - JUNE '84) I described a

machine language subroutine called ZIP-CHK.ASM that partially

validated zip codes and could be used by dBASE II using the

LOAD FILE, SET CALL TO ADDRESS, and CALL VARIABLE commands.

I now realize that older versions of dBASE II do not support the

LOAD FILE command that facilitate the loading of machine language

modules from assembled HEX files.  However, most early versions of

dBASE II (pre 2.4) WILL support the SET CALL TO ADDRESS and CALL

VARIABLE even though these commands may be undocumented in the

user's manual.

In this article I will explain these and some other undocumented

features of early versions of dBASE II.  I only have access to versions

2.3 and 2.4 so other versions are untested and should be checked

thoroughly to determine the feature actually works as I will

describe it.  I would appreciate feed back to my home address or phone

number of any results you may have on versions prior to 2.3.

I will catalog your findings and pass them on in future articles.

In addition to the undocumented features I have included a

description of the HEX file format and a dBASE II command file that

will simulate the LOAD FILE command IF you can get the undocumented

PEEK and POKE to work.  After reviewing the format of the HEX file

you should be able to write a LOAD simulator in any high level

language using LOAD-HEX.CMD as a guideline.

PRE-2.4 UNDOCUMENTED COMMANDS

-----------------------------

PEEK(aaaaa)        will return the contents of a decimal address.

POKE aaaaa,nnn     will fill a decimal address with a decimal value.

SET CALL TO aaaaa  will set the call address to a decimal value.

CALL VARIABLE      will call the address as defined in the SET CALL TO

command and pass the string VARIABLE as follows:

Upon entry of the subroutine the HL register pair will point

to the location of the passed variable.  The variable

is stored in the format LENGTH BYTE followed by the actual

string characters.  For example if the variable ZIP represents

the string '97330' then upon CALLing a subroutine HL would

point to a series of locations with the following data bytes

in hex:  05, 39, 37, 33, 33, 30.  The HL pair points to the

05 and indicates there are five bytes in this string.  The next

39, 37, 33, 33, 30 are the hex bytes that represent the ASCII

string '97330'.  You can modify the string but you must NOT

increase the length of the variable.  If the string becomes

shorter you should fill it with trailing blanks.

TEST(variable)     will test the variable and return the following:

-6  if the value is NUMERIC type

0  if the variable does not exist

1  if the variable is LOGICAL type

nn  (a number between 1 and 255) to indicate the length

of a STRING type variable.

LOADING A HEX FILE

------------------

The HEX file is simply an ASCII text file that holds information

about what bytes go where.  The LOAD.COM program usually is used with

the HEX file to create an executable COM file.  The format of any HEX

file is as follows:

:10A470002356235E22F0A401080021F2A47EFE07E9    <-- line 1

:10A48000CACFA4BACA8BA409C37DA4237EBBCA9633    <-- line 2

(30 lines were deleted in this example)

:10A672005756323437323638574935333035343914    <-- line 33

:09A68200575938323038333100E9		       <-- line 34

:0000000000				       <-- line 35

In a HEX file every line ends with a carriage return followed by

a line feed and begins with a colon.  In the above example in line

one the next two bytes following the colon, '10', are two ASCII characters

that represent one HEX value in the range 00-FF (0 to 255 decimal).

I will call this value the NUMBER OF BYTE-PAIRS value.  We will get

back to it in just a bit.  (No pun intended.)

The next four bytes 'A470' are ASCII characters that represent a

HEX value in the range of 0000-FFFF (00000 to 65,535 decimal).  I will

call this value the FIRST LOAD ADDRESS.

The next two bytes '00' are ASCII characters that are reserved.

That means I don't know what they are for but in every HEX file I

have ever looked at these two bytes are always '00' and for the scope

of this article I don't think it is necessary to define them exactly.

The same can be said for the last two bytes on each line ('E9' in

line 1).  A checksum maybe?

The following bytes 02, 35, 62, 35 etc. should be viewed as

byte-pairs.  Remember the NUMBER OF BYTE-PAIRS mentioned earlier?

Well that is exactly the number of these byte-pairs we will load into

consecutive memory starting at the FIRST LOAD ADDRESS.  Each of these

byte-pairs are ASCII characters that represent a HEX value in the

range of 00-FF (0 to 255 decimal).

From the example of line 1 the address locations should be

filled with the corresponding byte values as follows:

A470	23			A478	08

A471	56			A479	00

A472	23			A47A	21

A473	5E			A47B	F2

A474	22			A47C	A4

A475	F0			A47D	7E

A476	A4			A47E	FE

A477	01			A47F	07

If your high level language does not support a LOAD function

you can simulate one if you can read a text file and PEEK at and

POKE into memory.  The command file LOAD-HEX.CMD was written for

dBASE II.  dBASE II only understands decimal numbers and

most of the code is converting two ASCII bytes to a decimal number

to be POKED into memory.  Understanding the format of the HEX file

as outlined should enable you to write your own HEX loader simulator

in any language.

Using LOAD-HEX.CMD as guide, process line 1 of the HEX file.

Extract the load address (position 4-7 in the string), the number of

BYTE-PAIRS to POKE (position 2-3) and the individual BYTE-PAIRS

(beginning at position 10).  POKE each BYTE-PAIR then process the next

line of the HEX file.  Continue processing sequential lines until the

NUMBER OF BYTE-PAIRS is equal to zero.  The HEX file is now loaded

into memory and ready to CALL as a machine language subroutine.

< < < <    end of manuscript    program listing follows   > > > >

*	LOAD-HEX.CMD

*

*	File to simulate the LOAD feature of dBASE II 2.4 in

*	earlier versions that do not support this feature.

*

*	Verify that PEEK and POKE commands work before typing

*	in this file and watch the syntax especially '(' & ')'.

*

*	From:	Head Quarters

*		2860 NW Skyline Drive

*		Corvallis, Oregon  97330

*		(503)  758-0279

*

*	Copyright 1984 by Gene Head - All rights reserved

*	For private, non-commercial use only.

*

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

SET TALK OFF

* --->	First time initialization

*

* The following need be done only once

*

* First CREATE a file named HEXDATA.DBF with ONE

* field NAMED DATA of CHARACTER type, forty-four

* (44)  characters in length.

*

* APPEND FROM the SUBROUTINE HEX file using the SDF option.

* If you were going to use the Subroutine ZIP-CHK.HEX file then:

*

*	.

*	.USE HEXLOAD

*	.APPEND FROM ZIP-CHK.HEX SDF

*	.

*

* <---	End of first time initialization

*  Each line of HEX data will be in the field named DATA

USE HEXDATA

*   POSITION IN THIS STRING IS THE DECIMAL VALUE OF THE HEX CHARACTER

*   (SEARCHING FOR A '0' RETURNS 0)

STORE '123456789ABCDEF' TO HEX

* --->  COMPUTE THE BASE LOCATION OF THIS LOAD (THIS FUNCTION IS OPTIONAL)

STORE STR(((@($(DATA,4,1),HEX)*16 + @($(DATA,5,1),HEX)) * 256) +;

(@($(DATA,6,1),HEX)*16 + @($(DATA,7,1),HEX)),5) TO CALL:ADR

SET CALL TO &CALL:ADR

* <---  END OF LOCATION OPTION

DO WHILE $(DATA,2,2) <> '00'

* Compute how many bytes to POKE from this line of the HEX file

STORE (@($(DATA,2,1),HEX)*16 + @($(DATA,3,1),HEX)) TO COUNT

*  Get the starting POKE address

STORE ((@($(DATA,4,1),HEX)*16 + @($(DATA,5,1),HEX)) * 256);

+ (@($(DATA,6,1),HEX)*16 + @($(DATA,7,1),HEX)) TO ADDRESS

* We POKE the last BYTE-PAIR on the line of the HEX file

* and work our way back to the first BYTE-PAIR on the line.

DO WHILE COUNT

STORE STR(ADDRESS+COUNT-1,5)+','+STR(@($(DATA,COUNT*2+8,1),;

HEX)*16+@($(DATA,COUNT*2+9,1),HEX),3) TO BYTE

POKE &BYTE

STORE COUNT -1 TO COUNT

ENDDO WHILE COUNT

* GET NEXT LINE OF HEX DATA

SKIP

ENDDO WHILE $(DATA,2,2) <> '00'

USE

###################################################################################################################