; 15.4.86, 25.4.86

SCC notes  (very brief)

Part 1 ---- Imp

Part 2 ---- Construction

Part 3 ---- Telcom

Part 1 IMP

------------------------------------------------------------------------

| Note IMPORTANT                                                       |

|                                                                      |

| for use on some boards such as the earlier 'ROS' boards use 'RX'     |

| and 'TX' disabling 1k block request otherwise you will hang on a     |

| transfer of more than 128 blocks                                     |

|                                                                      |

------------------------------------------------------------------------

I2B-8530.ASM - MicroBee 128k computer with 8530 (SCC) hardware

installed as per MicroBee drawing 8432-4-01

test version started 25/02/87 by Tony Ellis VK2ZNH (234-5274 bus hours)

polling 8530, no autocall, for use with BeeModem

having 300 1200/75 and 75/1200 baud options

to implement the split rates pin 3 must be connected to pin 6 on Port A

DTR does not have an RS232 driver, so RTS is toggled at the same time and

can be used instead of DTR for that function ( it does have a driver )

high-speed option at 9600 baud provided for direct bee-to-another micro link

imp244.obj patched - replaced 110 bps option with 75 bps option

operates in full duplex with split baud rates using port A

(that is the one nearest to the DIN power connector)

NB this is different to Telcom which uses Port B

Split rates

Selecting speed of 75 bps gives Tx = 1200 bps Rx = 75 bps

Selecting speed of 1200 bps gives Tx = 75 bps Rx = 1200 bps (normal use)

-----------------------------------------------------------------------------

SUGGESTED PIN CONNECTIONS FOR ADAPTOR CABLE for IMP (Port A)

15 pin male connector                  25 pin female connector

+10.5 volts for BeeModem

pin 1 -------------------------------- pin 9

DTR/RTS

pin 2 -------------+------------------ pin 4

|

------------------- pin 20

pin 3 ------- TRxC

|

pin 6 ------- RTxC

TxD

pin 9 -------------------------------- pin 2

CTS

pin 11 ------------------------------- pin 5

RxD

pin 13 ------------------------------- pin 3

GND

pin 15 ------------------------------- pin 7

Part 2      Construction Notes

Parts list

G2      Crystal 3.6864 MHz   source Geoff Wood in Lane Cove

X3      D15 right angle female  type 15SCA Port B

X4      D15 right angle female  type 15SCA Port A

IC42    Zilog 8530 SCC source George Brown on Paramatta Road

IC43    74LS04

IC44    74LS164

IC45    1489

IC46    74LS74

IC47    1488

IC48    74LS32

IC50    7660

Sockets to suit (Makes fault finding much easier - use good quality dual-wipe)

C44-51  .1 mfd chip monolithic

C53-55  .1 mfd chip monolithic

C56     10 mfd Tantalum

C57     10 mfd Tantalum

R8      4.7k resistor

D3      OA91 or similar

D4      IN914/IN4148 or similar

D5      IN914/IN4148 or similar

A Larger Heatsink for the two 5 volt Rectifiers is needed for reliability.

The one (or two on some early machines) are just not effective enough.

N.B. HARD-DISK owners. (only, and not applicable to all harddisk machines)

1. The W3 123 pads beside the SCC need changing to put the SCC second priority

in the Interrupt daisy chain. On the none-component side cut the link between

W3-2 and W3-3 and make a connection between W3-1 and W3-2. This ensures the

hard-disk is still the highest priority.

2. The 4Mhz oscillator is needed for the SCC clock. Therefore the following

parts must all be installed. Fit any that are missing.

IC40    74LS04

G1      4 MHz crystal

C39     .01 mfd

C40     220pf

R4,R5   1k resistor

Connection Details for Port A and Port B

pin 1   +10.5 volts

pin 2   RTS  RS232 level

pin 3   TRxC TTL level

pin 4   DTR  TTL level

pin 5   RTS  TTL level

pin 6   RTxC

pin 7   NC

pin 8   DCD  TTL level

pin 9   TxD  RS232 level

pin 10  TxD  TTL level

pin 11  CTS  RS232 level

pin 12  -9 volts

pin 13  RxD  RS232 level

pin 14  +5 volts

pin 15  GND

Note: there is an internal connection between pin 3 Port A and pin 3 Port B

on the PC board.

Part 3         Telcom Notes

To use BeeModem with Telcom 2.3 use Port B (i.e. one nearest disk cable)

Make a D15 male to D25 pin female adaptor cable as follows

15 pin connector          25 pin Connector

pin  1                    pin  9                +10.5 volts

pin  9                    pin  2                TxData

pin 11                    pin  5                CTS

pin 13                    pin  3                RxData

pin 15                    pin  7                GND

Select port 2 on the Telcom Menu and off you go!

The information here is correct to the best of my knowledge. Please leave a

message describing any omissions or needed corrections which will go into the

next version.

Tony Ellis VK2ZNH 02.05.86

##############################################################################################