CONTENTS

1.  INTENDED AUDIENCE................................................ 2

2.  ACKNOWLEDGMENTS.................................................. 2

3.  RELATED DOCUMENTS................................................ 2

4.  ROSETTA STONE.................................................... 2

5.  WHY DEVELOP ZMODEM?.............................................. 3

6.  ZMODEM Protocol Design Criteria.................................. 5

6.1    Ease of Use............................................... 5

6.2    Throughput................................................ 5

6.3    Integrity and Robustness.................................. 6

6.4    Ease of Implementation.................................... 6

7.  ZMODEM BASICS.................................................... 6

7.1    Packetization............................................. 7

7.2    Link Escape Encoding...................................... 7

7.3    Header Packet Information................................. 8

7.4    Binary Header Packet...................................... 9

7.5    HEX Header Packet......................................... 9

7.6    Binary Data Packets.......................................  10

7.7    ASCII Encoded Data Packet.................................  10

8.  PROTOCOL TRANSACTION OVERVIEW....................................  10

8.1    Session Cancel Packet.....................................  13

9.  ZMODEM STREAMING TECHNIQUES......................................  14

9.1    Full Streaming with Sampling..............................  14

9.2    Full Streaming with Interrupt.............................  14

9.3    Full Streaming with a Sliding Window......................  15

9.4    No Streaming..............................................  15

10.  ATTENTION SEQUENCE...............................................  15

11.  PACKET/FRAME TYPES...............................................  15

11.1   ZRQINIT...................................................  16

11.2   ZRINIT....................................................  16

11.3   ZSINIT....................................................  16

11.4   ZACK......................................................  16

11.5   ZFILE.....................................................  16

11.6   ZSKIP.....................................................  18

11.7   ZNAK......................................................  18

11.8   ZABORT....................................................  18

11.9   ZFIN......................................................  18

11.10  ZRPOS.....................................................  18

11.11  ZDATA.....................................................  18

11.12  ZEOF......................................................  18

11.13  ZFERR.....................................................  18

11.14  ZCRC......................................................  18

11.15  ZCHALLENGE................................................  19

11.16  ZCOMPL....................................................  19

11.17  ZCAN......................................................  19

11.18  ZFREECNT..................................................  19

11.19  ZCOMMAND..................................................  19

12.  TRANSACTION EXAMPLE..............................................  20

12.1   A simple file transfer....................................  20

12.2   Challenge and Command Download............................  20

13.  ZFILE FRAME FILE INFORMATION.....................................  21

14.  PERFORMANCE RESULTS..............................................  23

14.1   Throughput................................................  23

14.2   Error Recovery............................................  23

15.  PACKET SWITCHED NETWORK CONSIDERATIONS...........................  24

16.  PERFORMANCE COMPARISION TABLES...................................  25

17.  MORE INFORMATION.................................................  26

18.  ZMODEM PROGRAMS..................................................  27

19.  YMODEM PROGRAMS..................................................  27

LIST OF FIGURES

Figure 1.  Order of Bytes in Header Packet............................  8

Figure 2.  Binary Header Packet.......................................  9

Figure 3.  HEX Header Packet..........................................  10

Figure 4.  Flow Control Compatibility.................................  24

Figure 5.  Protocol Overhead Information..............................  25

Figure 6.  Transmission Time Comparision..............................  26

Figure 7.  Y/ZMODEM Header Information usage..........................  26

The ZMODEM Asynchronous Inter Application File Transfer Protocol

Chuck Forsberg

Omen Technology Inc

ABSTRACT

The ZMODEM file transfer protocol greatly simplifies file transfers

compared to XMODEM.  In addition to supporting a transparent user

interface, ZMODEM provides Personal Computer and other users an efficient,

accurate, robust file transfer method.

ZMODEM provides especially efficient file transfers with timesharing

systems, satellite relays, and wide area packet switched networks.  A

choice of buffering and windowing modes allow ZMODEM to operate

efficiently on systems that cannot support some other streaming protocols.

ZMODEM provides advanced file management features including AutoDownload,

remote file compare, aborted transfer recovery, selective file transfers,

and security verified command downloading.

The ZMODEM Asynchronous Inter Application File Transfer Protocol

Chuck Forsberg

Omen Technology Inc

17505-V Northwest Sauvie Island Road

Portland Oregon 97231

Voice: 503-621-3406

Modem (Telegodzilla): 503-621-3746 Speed 1200,300

Compuserve: 70715,131

UUCP: ...!tektronix!reed!omen!caf

1.  INTENDED AUDIENCE

This document is intended for systems programmers and other technically

qualified people who choose and implement asynchronous file transfer

protocols over dial-up networks and related environments.

2.  ACKNOWLEDGMENTS

Encouragement and suggestions by Stuart Mathison, Thomas Buck, John Wales,

Ward Christensen, and Irv Hoff are gratefully acknowledged.

3.  RELATED DOCUMENTS

The following files should be available for reference while studying this

document:

YMODEM.DOC Describes the XMODEM and YMODEM file transfer protocols

ZMODEM.H Provides definitions for the manifest constants referenced

herein.

rz.c, sz.c, rbsb.c Unix source code for operating ZMODEM programs.

rz.1, sz.1 Manual pages for rz and sz.

zm.c, zmodem.h Operating system independent ZMODEM subroutines, header

file.

4.  ROSETTA STONE

Here are some definitions which reflect the current vernacular in the

computer media.  The attempt here is identify the file transfer protocol

rather than specific programs.

Frame A ZMODEM frame consists of a header packet and 0 or more data packets.

XMODEM  refers to the original 1979 file transfer etiquette introduced by

Ward Christensen's 1979 MODEM2 program.  It's also called the

XMODEM or MODEM2 protocol.  Some who are unaware of MODEM7's

unusual batch file mode call it MODEM7.  Other aliases include

"CP/M Users's Group" and "TERM II FTP 3".  This protocol is

supported by every serious communications program because of its

universality, simplicity, and reasonable performance.

XMODEM/CRC replaces XMODEM's 1 byte checksum with a two byte Cyclical

Redundancy Check (CRC-16), giving modern error detection

protection.

YMODEM  refers to the XMODEM/CRC protocol with the throughput and/or batch

transmission enhancements described in YMODEM.DOC.

ZMODEM  Zmodem is a second generation streaming protocol for text and

binary file transmission between applications running on

microcomputers and mainframes.

5.  WHY DEVELOP ZMODEM?

Since its development half a decade ago, the Ward Christensen MODEM

protocol has enabled a wide variety of computer systems to interchange

data.  There is hardly a communications program that doesn't at least

claim to support this protocol, now called XMODEM.

Advances in computing, modems and networking have spread the XMODEM

protocol far beyond the micro to micro environment for which it was

designed.  These application have exposed some weaknesses:

+ The user interface is suitable for computer hobbyists.  Three or four

commands must be keyboarded to transfer each file.

+ The short block length causes throughput to suffer when used with

timesharing systems, packet switched networks, satellite circuits,

and buffered (error correcting) modems.

+ The 8 bit checksum and unprotected transactions allow undetected

errors and disrupted file transfers.

+ Only one file can be sent per command.  The file name has to be given

twice, first to the sending program and then again to the receiving

program.

+ The transmitted file accumulates as many as 127 extraneous bytes.

+ The modification date and other file attributes are lost.

+ XMODEM requires complete 8 bit transparency, all 256 codes.  XMODEM

will not operate over some networks that need flow control.

A number of other protocols have been developed over the years, but none

have displaced XMODEM to date.

+ Lack of public domain documentation and example programs have kept

proprietary protocols such as MNP, Blast, and others tightly bound to

the fortunes of their suppliers.

+ Hardware and/or software complexity discourages the widespread

application of BISYNC, SDLC, HDLC, X.25, and X.PC protocols.

+ Link level protocols such as X.25, X.PC, and MNP do not manage

application to application file transfers.

+ The Kermit protocol was developed to allow file transfers in

environments hostile to XMODEM.  The performance compromises

necessary to accomodate non transparent environments limit Kermit's

efficiency.  Even with completely transparent channels, Kermit

control character quoting limits the efficiency of binary file

transfers to about 75 per cent.@1\

Kermit Sliding Windows ("SuperKermit") improves throughput over

networks at the cost of increased complexity.  SuperKermit state

transitions are encoded in a special language "wart" which requires a

C compiler.  The SuperKermit C code requires full duplex

communications and the ability to check for the presence of

characters in the input queue, precluding its implementation on some

operating systems.

A number of submodes are used in various Kermit programs, including

different methods of transferring binary files.  Two Kermit programs

will mysteriously fail to operate with each other if these submodes

are not matched.

A number of extensions to the XMODEM protocol have been made under the

collective name YMODEM.

+ YMODEM-k uses 1024 byte blocks to reduce the overhead from transmission

delays by 87 per cent compared to XMODEM, but network delays can still

degrade performance.  Some networks may not be transmit the 1024 byte

packets unmodified.

+ The handling of files that are not a multiple of 1024 or 128 bytes is

awkward, especially if the file length is not known, or changes during

transmission.

+ YMODEM-g provides efficient batch file transfers, preserving the exact

file length and file modification date.  YMODEM-g is essentially

insensitive to network delays.  Because it does not support error

recovery, YMODEM-g is usually used hardwired or with a reliable link

level protocol.  IF YMODEM-g detects a CRC error, data transfers are

aborted.  YMODEM-g is easy to implement because it closely resembles

XMODEM-CRC.

Another XMODEM "extension" is protocol cheating, referred to as "Turbo

Download" and OverThruster.  (2) These sometimes improve XMODEM throughput

1. Some Kermit programs support run length encoding.

at the expense of error recovery.

The ZMODEM Protocol is proposed as a means of addressing the weaknesses

described above while maintaining as much of XMODEM's simplicity and prior

art as possible.

6.  ZMODEM Protocol Design Criteria

The design of a file transfer protocol is an engineering compromise

between conflicting requirements:

6.1  Ease of Use

+ ZMODEM allows either program to initiate file transfers, passing

commands and/or modifiers to the other program.

+ File names need be entered only once, menu selections are possible.

+ Wild Card names may be used with batch transfers.

+ Minimum keystrokes required to initiate transfers.

+ ZRQINIT packet sent by sending program can trigger automatic downloads.

+ ZMODEM can step down to YMODEM if the other end does not support

ZMODEM.(1)

6.2  Throughput

ZMODEM is designed for optimum performance with minimum degradation caused

by delays introduced by packet switched networks and timesharing systems.

ZMODEM is optimized for best throughput when line hits occur infrequently.

This assumption markedly reduces code complexity and memory requirements.

ZMODEM protocol features enhance rapid error recovery compared to network

compatible XMODEM implementations.

It is assumed that many transfers will originate from a timesharing system

connected to a packet switched network.  ZMODEM provides features to allow

for simple, efficient implementation on timesharing hosts.

2. Omen Technology Trademark

1. Provided the transmission medium accomodates YMODEM.

File transfers begin immediately regardless of which program is started

first, without the 10 second delay associated with XMODEM.

6.3  Integrity and Robustness

All packets are protected with 16 bit CRC.  Proprietary alogrithyms (2) are

not needed for reliable transfers.

A security challenge guards againgst Trojan Horse messages.

6.4  Ease of Implementation

ZMODEM accomodates a wide variety of systems:

+ Microcomputers that cannot overlap disk and serial i/o

+ Microcomputers that cannot overlap serial send and receive

+ Computers and/or networks requiring XON/XOFF flow control

+ Computers that cannot check the serial input queue for the presence of

data without having to wait for the data to arrive.

Although ZMODEM provides "hooks" for multiple "threads", ZMODEM is not

intended to replace link level protocols such as X.25.

ZMODEM accomodates network and timesharing system delays by continuously

transmitting data unless the receiver interrupts the sender to request

retransmission of garbled data.  ZMODEM in effect uses the entire file as

a window.(3)

ZMODEM provides a general purpose application to application file transfer

protocol which may be used directly or with with reliable link level

protocols such as X.25, MNP, Fastlink, etc.

7.  ZMODEM BASICS

2. Unique to Professional-YAM, PowerCom, etc.

3. Streaming strategey is discussed in a coming chapter.

7.1  Packetization

ZMODEM frames somewhat different from X/YMODEM blocks.  X/YMODEM blocks

are not used for the following reasons:

+ Block numbers are limited to 256

+ No provision for variable length blocks

+ Line hits corrupt protocol signals, causing failed file transfers.  In

particular, modem errors sometimes generate false block numbers, false

EOTs and false ACKs.  False ACKs are the most troublesome as they cause

the sender to lose synchronization with the receiver.

State of the art X/YMODEM programs such as Professional-YAM and

PowerCom overcome some of these weaknesses with clever proprietary

code, but a stronger protocol is desired.

+ It is difficult to determine the beginning and ends of X/YMODEM blocks

when line hits cause a loss of synchronization.  This precludes rapid

error recovery.

7.2  Link Escape Encoding

ZMODEM acheives data transparency by extending the 8 bit character set

(256 codes) with escape sequences based on the ZMODEM data link escape

character ZDLE.(1)

Link Escape coding permits variable length data packets without the

overhead of a separate byte count.  It allows the beginning of frames to

be detected without special timing techniques, facilitating rapid error

recovery.

Link Escape coding does add some overhead.  The worst case, a file

consisting entirely of ZDLE characters, would incur a 50% overhead.

The ZDLE character is special.  ZDLE represents a control sequence of some

sort.  If a ZDLE character appears in the data sent within a binary

packet, it is prefixed with ZDLE, then sent as ZDLEE.

The value for ZDLE is octal 030 (ASCII CAN).  This particular value was

chosen to allow a string of CAN characters to abort a ZMODEM session,

compatible with X/YMODEM session abort.

1. This and other constants are defined in the zmodem.h include file.

Please note that constants with a leading 0 are octal constants in C.

Since CAN is not used for normal terminal operations, communications

programs can monitor the data flow for ZDLE.  The following characters can

be scanned to detect the ZRQINIT packet, the invitation to automatically

download commands or files.

Two successive CAN characters will abort a ZMODEM session.  Experience

with YMODEM file transfers suggests that this does not impair the

robustness of the protocol.  A minimum of 8 CAN are sent, so the ZMODEM

subroutines can be modified to require more successive CAN characters to

signal an abort.

The receiving program will decode any sequence of ZDLE followed by a byte

with bit 6 set and bit 5 reset (upper case letter, either parity) to the

equivalent control character by inverting bit 6.  This allows the

transmitter to escape any control character that cannot be sent by the

communications medium.  The ZMODEM software currently escapes ZDLE, 021,

0221, 023, and 0223.  In addition, the receiver recognizes escapes for

0177 and 0377 should these characters need to be escaped.

7.3  Header Packet Information

All ZMODEM frames begin with a header packet which may be sent in binary

or HEX form.  ZMODEM uses a single routine to recognize binary and hex

header packets.  Either form of the header packet contains the same raw

information:

+ A type byte(2) Future extensions to ZMODEM may use the high order bits

of the type byte to indicate thread selection.

+ Four bytes of data indicating flags and/or numeric quantities depending

on the packet type

Figure 1.  Order of Bytes in Header Packet

TYPE:  packet Type

F0: Flags least significant byte

P0: file Position least significant

P3: file Position most significant

TYPE  F3 F2 F1 F0

--------------

TYPE  P0 P1 P2 P3

2. The packet types are cardinal numbers beginning with 0 to minimize

state transition table memory requirements.

7.4  Binary Header Packet

A binary header packet is only sent by the sending program to the

receiving program.

A binary header packet begins with the sequence ZPAD, ZDLE, ZBIN.

The frame type byte is ZDLE encoded.

The four position/flags bytes are ZDLE encoded.

A two byte CRC of the frame type and position/flag bytes is ZDLE encoded.

0 or more binary data packets will follow depending on the frame type.

The function zsbhdr transmits a binary header packet.  The function

zgethdr receives a binary or hex header packet.

Figure 2.  Binary Header Packet

* * ZDLE TYPE F3/P0 F2/P1 F1/P2 F0/P3 CRC-1 CRC-2

7.5  HEX Header Packet

The receiver sends responses in hex header packets.  The sender also uses

hex header packets when they are not followed by binary data packets.

Hex encoding is required to support XON/XOFF flow control.  The hex header

receiving routine ignores flow control characters.

Use of Kermit style encoding for control and paritied characters was

considered and rejected because of increased possibility of interacting

with some timesharing systems's line edit functions.  Use of HEX packets

from the receiving program allows control characters to be used to

interrupt the sender when errors are detected.  Except for header packet

types that imply data packets to follow, a HEX header packet may be used

in place of a binary header packet.

A hex header packet begins with the sequence ZPAD, ZPAD, ZDLE, ZHEX.  The

zgethdr routine synchronizes in the ZPAD-ZDELE sequence.  The extra ZPAD

allows other parts of the program to detect a ZMODEM packet and then call

zgethdr to receive the packet.

The type byte, the four position/flag bytes, and the CRC thereof are sent

in hex using the character set 01234567890abcdef.  Upper case hex digits

are not allowed; they false trigger X/YMODEM programs.

A carriage return, line feed, and XON are appended to the HEX header

packet but are not considered to be part of it.  The CR/LF aids debugging

from printouts.  The XON releases the sender from spurious XOFF flow

control characters generated by line noise, a common occurrence.

0 or more ASCII Encoded data packets will follow depending on the frame

type.

The function zshhdr sends a hex header packet.

Figure 3.  HEX Header Packet

* * ZDLE TYPE F3/P0 F2/P1 F1/P2 F0/P3 CRC-1 CRC-2 CR LF XON

(TYPE, F3...F0, CRC-1, and CRC-2 are each sent as two hex digits.)

7.6  Binary Data Packets

Binary data packets immediately follow the associated binary header

packet.  A binary data packet contains 0 to 1024 bytes of data.

Recommended length values are 256 bytes below 4800 bps, 1024 above 4800

bps or when the data link is known to be relatively error free.

No padding is used with binary data packets.  The data bytes are ZDLE

encoded and transmitted.  A ZDLE and frameend are then sent, followed by

two ZDLE encoded CRC bytes.  The CRC accumulates the data bytes and

frameend.

The function zsbdata sends a binary data packet.  The function zrbdata

receives a binary data packet.

7.7  ASCII Encoded Data Packet

The format of ASCII Encoded data packets is not currently specified.

These would be used for server commands, etc.

8.  PROTOCOL TRANSACTION OVERVIEW

As with the XMODEM recommendation, ZMODEM timing is receiver driven.  The

transmitter should not time out at all, except to abort the program if no

packets are received for an extended period of time, say one minute.(1)

To start a ZMODEM file transfer session, the sending program is called

with the names of the desired file(s) and option(s).

The sending program sends the string "rz[r" to invoke the receiving

program from a possible command mode.  The "rz" followed by carriage

return activates a ZMODEM receive program or command if it were not

already active.

1. Special considerations apply when sending commands.

The sender may then display a message intended for human consumption, such

as a list of the files requested, etc.

Then the sender sends a ZRQINIT packet.  The ZRQINIT packet causes a

previously started receive program to send its ZRINIT packet without

delay.

In an interactive or conversational mode, the receiving application may

monitor the data stream for ZDLE.  The following characters may be scanned

for B000000 indicating a ZRQINIT packet, a command to download a command

or data.

The sending program awaits a command from the receiving program to start

file transfers.  If a "C", "G", or NAK is received, an XMODEM or YMODEM

file transfer is indicated, and file transfer(s) use the X/YMODEM

protocol.  Note: With ZMODEM and YMODEM Batch, the sending program

provides the file name, but not with XMODEM.

When the ZMODEM receive program starts, it immediately sends a ZRINIT

packet to initiate ZMODEM file transfers, or a ZCHALLENGE packet to verify

the sending program.  The receive program resends its packet at repsonse

time intervals for a suitable period of time (40 seconds typical) before

falling back to X/YMODEM protocol.  If the receiving program receives a

ZRQINIT packet, it resends the ZRINIT packet.  If the sending program

receives the ZCHALLENGE packet, it places the data in ZP0...ZP3 in an

answering ZACK packet.

If the receiving program receives a ZRINIT packet, it is an echo

indicating that the sending program is not operational.

Eventually the sending program correctly receives the ZRINIT packet.

The sender may then respond with an optional ZSINIT frame to set the

receiving program's Attention string.  The receiver sends a ZACK packet in

response, containing the serial number of the receiving program, or 0.

The sender then sends a ZFILE header with ZMODEM Conversion, Management,

and Transport options@2\ followed by a ZCRCW data packet containing the

file name, file length, modification date, and other information identical

to that used by YMODEM Batch.

The receiving program should insure the pathname and options are

compatible with its operating environment and local security requirements.

2. See below, under ZFILE packet type.

If the receiver has a file with the same name and length,

it may respond with a ZCRC packet, which requires the

sender to permorm a 16 bit CRC on the file and transmit the

CRC in ZP0...ZP1 of a ZCRC packet.  The receiver uses this

information to determine whether to accept the file or skip

it.  This sequence is triggered by the ZMCRC Management

Option.

The receiver may then respond with a ZSKIP packet, which causes the

sender to process the next file (if any) in the batch.

A ZRPOS packet from the receiver initiates transmission of the file data

starting at the offset in the file specified in the ZRPOS packet.

Normally the receiver specifies the data transfer begin begin at offset 0

in the file.

The receiver may start the transfer further down in the

file.  This allows a file transfer interrupted by a loss

or carrier or system crash to be completed on the next

connection without requiring the entire file to be

retransmitted.@3\ If downloading a file from a timesharing

system that becomes sluggish, the transfer can be

interrupted and resumed later with no loss of data.

The sender sends a ZDATA binary header packet (with file position)

followed by one or more data packets.

The receiver compares the file position in the ZDATA header with the

number of characters successfully received to the file.  If they do not

agree, a ZRPOS error response is generated to force the sender to the

right position within the file.(4)

A data packet terminated by ZCRCGO and CRC does not elicit a response

unless an error is detected; more data packet(s) follow immediately.

ZCRCQ data packets expect a ZACK response (with the file

offset) if no error, otherwise a ZRPOS response (with the

last good file offset).  Another data packet continues

immediately.  ZCRCQ packets are not used if the receiver

does not indicate FDX ability with the CANFDX bit.

ZCRCW data packets expect a response before the next frame is sent.  If

the receiver does not indicate overlapped I/O capability with the

3. This does not apply to files that have been translated.

4. If the ZMSPARS option is used, the receiver instead seeks to position

in the ZDATA packet.

CANOVIO bit, or by setting a buffer size, the sender uses the ZCRCW to

allow the receiver to write its buffer before sending more data.

A zero length data frame may be used as a sending idle

packet to prevent the receiver from timing out in case

data is not immediately available to the sender.

In the absence of fatal error, the sender eventually encounters end of

file.  If the end of file is encountered within a frame, the frame is

closed with a ZCRCE data packet which does not elicit a response

except in case of error.

The sender sends a ZEOF packet with the file ending offset equal to

the number of characters in the file.  The receiver compares this

number with the number of characters received.  If the receiver has

received all of the file, it closes the file.  If the file close was

satisfactory, the receiver responds with ZRINIT.  If the receiver has

not received all the bytes of the file, the receiver sends ZRPOS with

the current file offset, forcing the sender to resend the missing

data.  (If the receiver cannot properly close the file, a ZFERR packet

is sent.)

After all files are processed, any further protocol

errors should not prevent the sending program from

returning with a success status.

The sender closes the session with a ZEXIT header packet.  The

receiver acknowledges this with its own ZEXIT packet.

When the sender receives the acknowledging packet, it sends two

characters, "OO" (Over and Out) and exits to the operating system or

application that invoked it.  The receiver waits briefly for the "O"

characters, then exits whether they were received or not.

8.1  Session Cancel Packet

The Cancel packet consists of two ZPAD characters, eight CAN

characters, and an optional ten backspace characters.  First, the

Attn sequence is executed if the receiving program has been receiving

data in streaming mode.  The ZPAD characters allow sending programs

that sample the reverse data stream to check for a single character

code indicating a packet from the receiver.  The trailing backspace

characters attempt to erase the effects of the other characters if

they are received by a command interpreter.

static char canistr() =

(ZPAD,ZPAD,24,24,24,24,24,24,24,24,8,8,8,8,8,8,8,8,8,8,0);

9.  ZMODEM STREAMING TECHNIQUES

ZMODEM allows choices of several data streaming methods selected

according to the limitations of the sending environment, receiving

environment, and transmission channel(s).

9.1  Full Streaming with Sampling

If the computers can overlap serial I/O with disk I/O, and if the

sender can sample the reverse channel for the presence of data

without having to wait, full streaming can be used with no Attn

sequence required.  The sender begins data transmission with a ZDATA

header and continuous ZCRCG data packets.  When the receiver detects

an error, it executes the Attn sequence and then sends a ZRPOS packet

to force the sender back to the correct position within the file.  At

the end of each transmitted packet, the sender checks for the

presence of an error packet from the receiver.  To do this, the

sender may sample the reverse data stream for the presence of a ZPAD

character.

Such a program would sample the reverse channel for ZPAD.  If seen,

an empty ZCRCW data packet is sent (in case the receiver was still

reading packets) and then the receiver's response packet is read and

acted upon.  The code fragment in sz.c beginning at NOTDEF^DOS would

perform this function.

9.2  Full Streaming with Interrupt

The method above cannot be used if if the reverse data stream cannot

be sampled without entering an I/O wait.  An alternate method is to

instruct the receiver to interrupt the sending program when an error

is detected.

The receiver can interrupt the sender with a control character, break

signal, or combination thereof, as specified in the ZSINIT frame sent

by the sending program.

When the sending program "catches" this interrupt, it reads a HEX

packet (normally ZRPOS) from the receiver and takes appropriate

action.  The Unix sb.c program uses a setjmp/longjmp call and the

getinsync() function to read the receiver's error packet and take

appropriate action.

9.3  Full Streaming with a Sliding Window

If none of the above methods is applicable, hope is not yet lost.  If

the sender can buffer responses from the receiver, the sender can use

ZCRCQ packets to get ACKs from the receiver without interrupting the

transmission of data.  After a sufficient number of ZCRCQ packets

have been sent, the sender can read one of the one or more packets

that should have arrived in it's receive interrupt buffer.

A problem with this method is the probability of wasting an excessive

amount of time responding to the receiver's error packet.

9.4  No Streaming

If the receiver cannot overlap serial and disk I/O, it uses the

ZRINIT frame to specify a buffer length which the sender will not

overflow.  The sending program sends a ZCRCW packet and waits for an

ZACK packet before sending the next segment of the file.

If the sending program supports reverse data stream sampling or

interrupt, error recovery will be faster (on average) than a protocol

(such as YMODEM) that sends "monolithic" blocks.

10.  ATTENTION SEQUENCE

The receiving program sends the Attn sequence whenever it detects an

error and needs to interrupt the sending program.

The default Attn string value is empty (no Attn sequence).  The

receiving program resets Attn to the empty default before each

transfer session.

The sender speficies the Attn sequence in its optional ZSINIT frame.

The Attn string is terminated with a null.

Two meta-characters perform special functions:

+ [335 (octal) Sends a break signal

+ [336 (octal) Pauses one second

11.  PACKET/FRAME TYPES

The numeric values for the values shown in boldface are given in

zmodem.h.

11.1  ZRQINIT

Sent once by the sending program, to trigger the receiving program to

send its ZRINIT packet.  This aviods the aggravatimg startup delay

associated with XMODEM and Kermit transfers.

ZF0 contains ZCOMMAND if the program is attempting to send a command,

0 otherwise.

11.2  ZRINIT

Sent by the receiving program.  ZF0 and ZF1 contain the  bitwise or

of the receiver capability flags:

#define CANFDX  1 /* Rx can send and receive FDX */

#define CANOVIO 2 /* Rx can receive during disk I/O */

#define CANBRK  4 /* Rx can send a break signal */

#define CANCRY  8 /* Receiver can decrypt */

ZP0 and ZP1 contain the size of the receiver's buffer in bytes, or 0

if nonstop I/O is allowed.

11.3  ZSINIT

Sender sends capability flags (currently all 0) (none currently

defined) followed by a binary data packet terminated with ZCRCW.  The

data packet contains the null terminated Attn sequence, maximum

length 32 bytes including the terminating null.

11.4  ZACK

Acknowedgement to ZSINIT header packet, ZCHALLENGE header packet, or

ZCRCW data packet.  ZP0 to ZP3 contain file offset.  Response to

ZCHALLENGE contains the same 32 bits as received.

11.5  ZFILE

This packet denotes the beginning of a file transmission attempt.

ZF0, ZF1, and ZF2 may contain options.  A value of 0 in each of these

bytes implies no special treatment.  If options are specified to the

reciever, they override options specified to the sender with the

exception of the ZCBIN option, which overrides any other Conversion

Option.

11.5.1  ZF0: Conversion Option

If the receiver does not recognize the Conversion Option, an

application dependent default conversion may apply.

ZCBIN "Binary" transfer - inhibit conversion unconditionally

ZCNL Convert received end of line to local end of line convention.

The suported end line conventions are CR/LF (most ASCII based

operating systems except Unix and Macintosh), and NL (Unix).

Neither of these two end of line conventions violate the

permissible ASCII definitions for Carriage Return and Line

Feed/New Line.

ZCRECOV Recover interrupted file transfer; start transfer at location

corresponding to the receiver's end of file.  This option does

not apply if the source file is shorter.  Files that have been

converted (e.g., ZCNL) or subject to a single ended Transport

Option cannot have their transfers recovered.

11.5.2  ZF1: Management Option

If the receiver does not recognize the Management Option, the file

should be transferred normally.

ZMNEW Compare the source and destination files.  Transfer file if

source newer or different length

ZMCRC Compare the source and destination files.  Transfer if

different file length or CRC

ZMAPND Append source file contents to existing destination file (if

any)

ZMCLOB Replace existing destination file (if any)

ZTSPARS Special processing for sparse file; each file segment is

transmitted as a separate frame, where the frames are not

necessarily contiguous.

11.5.3

I Management Option, the file should be transferred normally.

ZMNEW Compare the source and destination files.  Transfer file if

source newer or different length

ZMCRC Compare the source and destination files.  Transfer if

different file length or CRC

ZMAPND Append source file contents to existing destination file (if

any)

ZMCLOB Replace existing destination file (if any)

ZTSPARS Special processing for sparse file; each file segment is

transmitted as a separate frame, where the frames are not

necessarily contiguous.

11.5.3  ZF2: Transport Option

If the receiver does not implement the particular transport option,

the file is copied without conversion for later processing.

ZTLZW Lempel-Ziv compression.  Transmitted data will be identical to

that produced by compress 4.0 operating on a computer with VAX

byte ordering, using 12 bit encoding.

ZTCRYPT Encryption.  An initial null terminated string identifies the

key.  Details to be determined.

ZTRLE Run Length encoding Details to be determined.

A ZCRCW data packet follows with file name, file length, modification

date, and other information described in a later chapter.

11.6  ZSKIP

Sent by the receiver in response to ZFILE, makes the sender skip to

the next file.

11.7  ZNAK

Indicates last packet header was garbled.  (See also ZRPOS).

11.8  ZABORT

Sent by receiver to terminate batch file transfers when requested by

the user.  Sender initiates a ZFIN sequence.@1\

11.9  ZFIN

Sent by sending program to terminate a ZMODEM session.  Receiver

responds with ZFIN.

11.10  ZRPOS

Sent by receiver to force file transfer to resume at file offset

given in ZP0...ZP3.

11.11  ZDATA

ZP0...ZP3 contain file offset.  One or more data packets follow.

11.12  ZEOF

Sender reports End of File.  ZP0...ZP3 contain the ending file

offset.

11.13  ZFERR

Error in reading or writing file, protocol equivalent to ZABORT.

11.14  ZCRC

Request (receiver) and response (sender) for file CRC.  ZP0 and ZP1

contain 16 bit file CRC.

1. Or ZCOMPL in case of server mode.

11.15  ZCHALLENGE

Request to echo a random number in ZP0...ZP3 in a ZACK frame.  Sent

by the receiving program to the sending program to verify that it is

connected to an operating program, and was not activated by spurious

data or a Trojan Horse message.

11.16  ZCOMPL

Request now completed.

11.17  ZCAN

This is a pseudo frame type returned by gethdr() in response to a

Cancel sequence.

11.18  ZFREECNT

Sending program requests a ZACK frame with ZP0...ZP3 containing the

number of free bytes on the current file system.  A value of 0

represents an indefinite amount of free space.

11.19  ZCOMMAND

ZCOMMAND is only sent as a binary header packet.  ZP0...ZP2 contain a

unique cardinal number to differentiate this command from other

commands(2).  ZF0 contains 0 or ZCACK1.

A ZCRCW data packet follows, with the ASCII text command string

terminated with a NULL character.  If the command is intended to be

executed by the operating system hosting the receiving program (e.g.,

"shell escape"), it must have "!" as the first character.  Otherwise

the command is meant to be executed by the application program which

received the command.

If ZF0 contained ZCACK1, the receiver immediately responds with a

ZCOMPL header.  Otherwise, the receiver responds with a ZCOMPL header

when the operation is completed.

The exit status of the completed command is stored in ZP0...ZP3 (0 if

ZCACK1).  A 0 exit status implies nominal completion of the command.

If the command caused a file to be transmitted, the command sender

will see a ZRQINIT frame from the other computer attempting to send

2. Currently unused.

data.

The sender examines ZF0 of the received ZRQINIT packet to determine

it is not an echo of its own ZRQINIT packet.  It is illegal for the

sending program to command the receiving program to send a command.

12.  TRANSACTION EXAMPLE

12.1  A simple file transfer

A simple transaction, one file, no errors, no CHALLENGE, overlapped

I/O:

Sender  Receiver

"rz[r"

ZRQINIT(0)

ZRINIT

ZFILE

ZRPOS

ZDATA data ...

ZEOF

ZRINIT

ZFIN

ZFIN

OO

12.2  Challenge and Command Download

Sender  Receiver

"rz[r"

ZRQINIT(ZCOMMAND)

ZCHALLENGE(rnd)

ZACK(same-rnd)

ZRINIT

ZCOMMAND

(Perform Command)

ZCOMPL

ZFIN

ZFIN

OO

13.  ZFILE FRAME FILE INFORMATION

ZMODEM sends the same file information with the ZFILE frame data that

YMODEM Batch sends in its block 0.

N.B.: Only the pathname (file name) part is required for batch

transfers.

Pathname The pathname (conventionally, the file name) is sent as a

null terminated ASCII string.  This is the filename format used

by the handle oriented MSDOS(TM) functions and C library fopen

functions.  An assembly language example follows:

DB   'foo.bar',0

No spaces are included in the pathname.  Normally only the file

name stem (no directory prefix) is transmitted unless the sender

has selected YAM's f option to send the full relative pathname.

The source drive designator (A:, B:, etc.) is not sent.

Filename Considerations:

+ File names should be translated to lower case unless the

sending system supports upper/lower case file names.  This

is a convenience for users of systems (such as Unix) which

store filenames in upper and lower case.

+ The receiver should accommodate file names in lower and

upper case.

+ The rb(1) program on Unix systems normally translates the

filename to lower case unless one or more letters in the

filename are already in lower case.

+ When transmitting files between different operating

systems, file names must be acceptable to both the sender

and receiving operating systems.

If directories are included, they are delimited by /; i.e.,

"subdir/foo" is acceptable, "subdir[foo" is not.

Length The file length and each of the succeeding fields are

optional.(1) The length field is stored as a decimal string

counting the number of data bytes in the file.

With ZMODEM, the receiver uses the file length only for display

(progress reporting) purposes; the actual length is determined

1. Fields may not be skipped.

by the data transfer.

Modification Date A single space separates the modification date from

the file length.

The mod date is optional, and the filename and length may be

sent without requiring the mod date to be sent.

The mod date is sent as an octal number giving the time the

contents of the file were last changed measured in seconds from

Jan 1 1970 Universal Coordinated Time (GMT).  A date of 0

implies the modification date is unknown and should be left as

the date the file is received.

This standard format was chosen to eliminate ambiguities arising

from transfers between different time zones.

Two Microsoft blunders complicate the use of modification dates

in file transfers with MSDOS(TM) systems.  The first is the lack

of timezone standardization in MS-DOS.  A file's creation time

can not be known unless the timezone of the system that wrote

the file (2) is known.  Unix solved this problem (for planet

Earth, anyway) by stamping files with Universal Time (GMT).

Microsoft would have to include the timezone of origin in the

directory entries, but does not.  Professional-YAM gets around

this problem by using the z parameter which is set to the number

of minutes local time lags GMT.  For files known to originate

from a different timezone, the -zT option may be used to specify

T as the timezone for an individual file transfer.

The second problem is the lack of a separate file creation date

in DOS.  Since some backup schemes used with DOS rely on the

file creation date to select files to be copied to the archive,

back-dating the file modification date could interfere with the

safety of the transferred files.  For this reason,

Professional-YAM does not modify the date of received files with

the header information unless the d parameter is non zero.

Mode A single space separates the file mode from the modification

date.  The file mode is stored as an octal string.  Unless the

file originated from a Unix system, the file mode is set to 0.

rb(1) checks the file mode for the 0x8000 bit which indicates a

Unix type regular file.  Files with the 0x8000 bit set are

assumed to have been sent from another Unix (or similar) system

2. Not necessarily that of the transmitting system!

which uses the same file conventions.  Such files are not

translated in any way.

Serial Number A single space separates the serial number from the

file mode.  The serial number of the transmitting program is

stored as an octal string.  Programs which do not have a serial

number should omit this field, or set it to 0.  The receiver's

use of this field is optional.

The file information is terminated by a null.  If only the pathname

is sent, the pathname will be terminated by two nulls.  The length of

the file information packet, including the trailing null, must not

exceed 1024 bytes; a typical length is less than 64 bytes.

14.  PERFORMANCE RESULTS

14.1  Throughput

Between two single task PC-XT computrers, on a Telenet link through

the local Telenet, SuperKermit gave 72 ch/sec throughput at 1200

baud.  YMODEM-k yielded 85 chars/sec, and ZMODEM provided 113 chat

sec.  ZMODEM was not measured, but would have given much less.

14.2  Error Recovery

Some tests of ZMODEM protocol performance have been made.  A PC-AT

with SCO SYS V Xenix or DOS 3.1 was connected to a PC with DOS 2.1

either directly at 9600 bps or with unbuffered dial-up 1200 bps

modems.  The ZMODEM software was configured to use 1024 byte packet

lengths above 2400 bps, 256 otherwise.

Because no time delays are necessary in normal file transfers, per

file negotiations are much faster than with YMODEM, the only observed

impidiment being the time required by the program(s) to update

logging files.

During a file transfer, a short line hit seen by the receiver usually

induces a CRC error.  The interrupt packet is usually seen by the

sender before the next packet is sent, and the resultant loss of data

throughput averages about half a packet.  At 1200 bps this is would

be about .75 second lost per hit.  At 10-5 error rate, this would

degrade throughput by about 9 per cent.  The throughput degradation

increases with the channel delay, as the packets in transit through

the channel are discarded on error.

A longer noise burst that affects both the receiver and the sender's

reception of the interrupt packet usually causes the sender to remain

silent until the receiver times out in 10 seconds.  If the round trip

channel delay exceeds the receiver's 10 second timeout, recovery from

this type of error may become difficult.

Noise affecting only the sender is usually ignored, with one common

exception.  Spurious XOFF characters generated by noise stop the

sender until the receiver times out and sends an interrupt packet

which concludes with an XON.

In summation, ZMODEM performance in the presence of errors resembles

that of X.PC and SuperKermit.  Short bursts cause minimuml data loss.

Long bursts (such as pulse dialing noises) often require a timeout

error to restore the flow of data.

15.  PACKET SWITCHED NETWORK CONSIDERATIONS

Flow control is necessary for printing messages and directories, and

for streaming file transfer protocols including Kermit Sliding

Windows and ZMODEM.  A non transparent flow control is incompatible

with XMODEM and YMODEM transfers.  XMODEM and YMODEM protocols

require complete transparency of all 256 8 bit codes to operate

properly.

The most desireable flow control (when X.25 or hardware CTS is

unavailable) does not "eat" any characters at all.  When the PAD's

buffer almost fills up, an XOFF should be emitted.  When the buffer

is no longer nearly full, send an XON.  Otherwise, the network should

neither generate nor eat XON or XOFF control characters.

On Telenet, this can be met by setting CCIT X3 5:1 and 12:0 at both

ends of the network.  For best throughput, parameter 64 (advance ACK)

should be set to something like 4.  Packets should be sent when the

packet is a full 128 bytes, or after a moderate delay (3:0,4:10,6:0).

For YMODEM, PAD buffering should guarantee that a minimum of 1040

characters can be sent in a burst without loss of data or generation

of flow control characters.  Failure to provide this buffering will

generate excessive retries with YMODEM.

Figure 4.  Flow Control Compatibility

Connectivity              Interactive     XMODEM   KERMIT   ZMODEM

Direct Connection               YES        YES       YES     YES

Network, no flow control        NO         YES       (1)     (1)

Network, transparent f.c.       YES        YES       YES     YES

Network, semi-transparent f.c.  YES        NO        YES     YES

Network, 7 bit                  YES        NO        YES(2)  NO(3)

(1) Cannot operate in streaming mode.  Kermit is very slow because of

96 byte max packet size.  ZMODEM can adjust burst length to maximum

for faster transfers.

(2) Parity bits must be encoded, slowing binary transfers.

(3) Extension possible for encoding data to 7 bits.

16.  PERFORMANCE COMPARISION TABLES

"Round Trip Delay Time" includes the time for the last byte in a

packet to propagate through the operating systems and network to the

receiver, plus the time for the receiver's response to that packet to

propogate back to the sender.

The figures shown below are calculated for round trip delay times of

40 milliseconds and 5 seconds.  Shift registers in the two computers

and a pair of 212 modems generate a round trip delay time on the

order of 40 milliseconds.  Operation with busy timesharing computers

and networks can easily generate round trip delays of five seconds.

Because the round trip delays cause visible interruptions of data

transfer when using XMODEM protocol, the subjective effect of these

delays is greatly exaggerated, especially when the user is paying for

connect time.

A 102400 byte binary file with randomly distributed codes is sent at

1200 bps 8 data bits, 1 stop bit.  The calculations assume no

transmission errors.  For each of the protocols, only the per file

functions are considered.  Processor and I/O overhead are not

included.  YM-k refers to YMODEM with 1024 byte packets.  YM-g refers

to the YMODEM "g" option.  ZMODEM uses 256 byte packets for this

example.  SuperKermit uses maximum packet size, 8 bit transparent

transmission, no run length compression.

For comparison, a straight "dump" of the file contents with no file

management or error checking takes 853 seconds.

Figure 5.  Protocol Overhead Information

Protocol                       XMODEM  YM-k  YM-g ZMODEM  S-Kermit

Protocol Round Trips              803  103    5      5        5

Trip Time at 40ms                  32s  4s    0      0        0

Trip Time at 5s                  4015s  515s  25s     25s     25s

Overhead Characters              4803   603   503    3600   38280

Transfer Time at 0s               893s  858s  857s   883s   1172s

Transfer Time at 40ms             925s  862s  857s   883s   1172s

Transfer Time at 5s              5761s  1373s 882s   918s   1197s

Figure 6.  Transmission Time Comparision

(5 Second Round Trip)

************************************************** XMODEM

************ YMODEM-K

********** SuperKermit (Sliding Windows)

******* YMODEM-G

******* ZMODEM

Figure 7.  Y/ZMODEM Header Information usage

Program     Batch   Length   Date   Mode   S/N   YMODEM-g   ZMODEM

Unix rb/sb   yes     yes      yes    yes    no    sb only    no

Unix rz/sz   yes     yes      yes    yes    no    sz only    yes

VMS rb/sb    yes     yes      no     no     no    no         no

Pro-YAM      yes     yes      yes    no     yes   yes        yes

CP/M YAM     yes     no       no     no     no    no         no

KMD/IMP      yes     yes-     no     no     no    no         no

MEX          no      no       no     no     no    no         no

17.  MORE INFORMATION

More information may be obtained by calling Telegodzilla at

503-621-3746.

UUCP sites can obtain the nroff/troff source to this file with

uucp omen!/usr/caf/public/zmodem.mm /tmp

A continually updated list of available files is stored in

/usr/spool/uucppublic/FILES.

The following L.sys line calls site "omen" yia UUCP.  Telegodzilla

uses Pro-YAM in host operation.

In response to "Name Please:" uucico gives the Pro-YAM "link" command

as a user name.  The password (Giznoid) controls access to the Xenix

system connected to the IBM PC's other serial port.  Communications

between Pro-YAM and Xenix use 9600 bps; YAM converts this to the

caller's speed.

Finally, the calling uucico logs in as uucp.

omen Any ACU 1200 1-503-621-3746 e:--e: link d: Giznoid n:--n: uucp

18.  ZMODEM PROGRAMS

A demonstration version of Professional-YAM is available as

YAMDEMO.ARC on TeleGodzilla..  This file must be unpacked with the

"ARC" program, version 5 or later.  A copy of ARC is available as

"ARC.EXE" or "ARC510.COM" on TeleGodzilla.

This may be used to test ZMODEM and YMODEM implementations.  A

flash-up tree structured help file and processor are provided in

YAMHELP.LQR.

19.  YMODEM PROGRAMS

Unix programs supporting the YMODEM protocol are available on

Telegodzilla in the "upgrade" subdirectory as RBSB.SHQ (a SQueezed

shell archive).  Most Unix like systems are supported, including V7,

Sys III, 4.2 BSD, SYS V, Idris, Coherent, and Regulus.

A version for VAX-VMS is available in VRBSB.SHQ, in the same

directory.

Irv Hoff has added YMODEM 1k packets and YMODEM batch transfers to

the KMD and IMP series programs, which replace the XMODEM and

MODEM7/MDM7xx series respectively.  Overlays are available for a wide

variety of CP/M systems.

Many other programs, including MEX and MEX-PC also support some of

the YMODEM extensions.

Questions about YMODEM, the Professional-YAM communications program,

and requests for evaluation copies may be directed to:

Chuck Forsberg

Omen Technology Inc

17505-V Sauvie Island Road

Portland Oregon 97231

Voice: 503-621-3406

Modem (Telegodzilla): 503-621-3746

Usenet: ...!tektronix!reed!omen!caf

Compuserve: 70715,131

Source: TCE022

#######################################################################################################################