ComLink V0.98

(C) 1996-2000 H�rsfalvi Levente

The pack you opened is a slightly improved version of ComLink 0.96�,

the small communications + linker software I introduced in the first days of

1996. It is still intended for cross-development purposes, but it can be

also useful for you if you, for example, just need a fast transfer routine

for simply downloading one filer programs from a PC to a C64.

Copyright notice

------------------

This program is free software; you can redistribute it and/or

modify it under the terms of the GNU General Public License as

published by the Free Software Foundation; either version 2 of the

License, or (at your option) any later version.

This program is distributed in the hope that it will be useful,

but WITHOUT ANY WARRANTY; without even the implied warranty of

MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.  See the

GNU General Public License for more details.

You should have received a copy of the GNU General Public License

along with this program; if not, write to the Free Software

Foundation, Inc., 59 Temple Place, Suite 330, Boston, MA

02111-1307  USA

All names and trademarks in this document are trademarks of their

respective owners.

Table of contents

-------------------

1.) Introduction

2.) Files included

3.) System requirements

4.) Installing

4.a) Software

4.b) Connecting the devices

4.c) Setting up the communication

5.) Switches and parameters

6.) Transferring one filers

7.) Cross development using ComLink

7.a) Including external binary files

7.b) Relocating codes

7.c) Tips and tricks

8.) Cables

8.a) X1541

8.b) Disk64E

8.c) Parallel

8.d) Parallel port for the C-16 / C-116

9.) Inside the program

11.) Contact

12.) Thanks and respects

Appendixes

A.) Some measurements

1.) Introduction

-----------------

As I said, from one side, this is a small filetransfer package, with which

you can download datas and/or code from a PC to your Commodore. Well, ComLink

is not unique in this subject; a lot of programs exist nowadays with such

capability. Then, why to write another one? Well. Because my goal was slightly

different when I created it. I wrote this program because I needed something

that helps me in cross-development (using PC for editing and compiling

Commodore asm sources, and then running the code on an original Commodore).

The advantages of this method are convincing: you can edit huge sources with

less trouble, you can do this on (in worst case) 80x25 displays, as well as

compile them fast and easily with cross assemblers - all much much easier than

on a real Commodore micro. Developing programs this way can also help saying

byebye to the well known "damn it, this ***** routine filled up the memory

before I saved my source" scream. All in all, be your PC even a vintage XT

(Commodore PC-1 ?!), the development process can reveal far less troubles

if you develop this way.

If you're just familiar with cross-development and you rather use a

C64 emulator to first-test your code (as I also do...), ComLink can still be

useful for you since it has other, rather linking-oriented features. Besides

the transfer capability, it can read and link standard hexadecimal .OBJ

format (Intel HEX, for the moment) as well as usual .BIN files. With it you

can simplify the code generation process; using the binary data linking-in

feature, you can list and set all parts of your program easily, and ComLink

links the parts together either into a PC binary file or to the Commodore

computers memory (linking through a cable).

In fact, with this release (0.98) ComLink can help you working around two

common problems with the code generation. The first one is that most cross

assemblers, including my favourite (Table driven assembler) lack the

relocatable origin. I could best describe this problem as follows: you have

a subroutine in your code that you have to place to a special area of the

memory (typical example is when you want a digiplay routine to reside on the

zeropage, since it can be coded to run slightly faster this way). Obviously,

you have to tell the assembler to start this code at this particular

address (example, $0010), for right code generation. But also obviously, you

can't keep the routine at this place at the end of code generation, since you

couldn't load it to here directly from the disk. Rather, you have to store

this routine somewhere and copy it to the zeropage in the initialization of

your program. ...Now, the problem is: the assembler happily assembles your

code to the _original origin, this is no problem; but you need this code to be

at the place from where it will be _copyed to the original origin (where it

will run) in the initialization. Most assemblers are unable to handle this

(Commodore (not just 6502) relative crossasms tend to do it, however).

A bit simpler problem is, that it's quite difficult to link something

external into a program, like graphics drawn in editors, music, data tables

and so on. You can, of course, link the data parts together somehow by hand

but then you're stuck with absolute (static) locations of the data areas;

whatever you do, you must take care of locating everything right if you don't

want your code and tables bite each other ;-). This is no problem in certain

cases, since graphics and music usually have to be linked to static

addresses anyway. But ComLink can also help you a lot in linking the other

group, since if you tell it to do so, it can collect the appropriate addresses

together from the assembly export file and link the external binary files

in to the addresses that were generated by the assembler in the compiling

pass.

As speaking of the other part, the communication capabilities, ComLink

supports 'standard' cables to connect the Commodore computer to the PC

(currently X1541 and Disk64E cables are supported (serial transfer), and

a parallel cable that is much likely an X1541 and 8 parallel lines in one).

The transfer is quite optimized for speed; it is about 3.3K/s and 19K/s with

a normal C-64 and a PC, via serial and parallel connection. Both the

transmitter and receiver routines were written in machine language.

2.) Files included

-------------------

- comlink.exe      The compiled and executable program

- comlink.pas      The source, written in Turbo Pascal 7.0

- comlink.asm      Common source for various receiver routines (C= asm)

- comlink.doc      Documentation, you're just reading it.

- clxxyy.prg       Assembled receiver routines in binary format.

3.) System requirements

------------------------

- IBM PC, PC/XT, PC/AT or compatible computer

- 640K RAM (some 150-200K of free base mem should be well enough)

- 30-40K free HDD or even FDD

- one free standard parallel port

- a standard X1541, Disk64E or parallel cable, described later

- DOS 3.3 or above

The program was written / compiled to 8086 code (no 286+ instructions).

On the other side:

- Commodore 16,116,Plus/4 or 64 computer

- some storage, preferably a floppy drive.

At the time only Plus/4 and compatibles (16,116, ...CBM 364 :-) ) and C-64

are supported; although it would be good to support other systems too.

Please contact me if you can help me converting the receiver routines.

4.) Installing

----------------

4.a) Software

--------------

You don't have to do much installing this package; simply, unpack it - I

guess, you did it already.

From the other hand, installing the receiver routines on the Commodore

side is not so obvious like the above one is. I'd suggest to use a 1541

handler (Star Commander is the best I know currently) and copy the appropriate

receiver routine to a disk from which you can later load it easily whenever

it's neccessary.

4.b) Connecting the devices

----------------------------

(You can skip this and the next section if you don't plan to use the

communications features).

You must have either cable the pack supports (X1541, Disk64E or the

parallel thing, described later). Having the cable, connect the units together.

Since the program uses the same cable as the floppy drive uses, if you want to

use a floppy drive, connect your floppy to the Commodore, and connect the PC

to the drive with the X1541/Disk64E cable. It's possible to do it because of

the OC driver architecture of the bus. If you have the parallel cable, then

connect that to the user port besides of connecting to the floppy. Take care

about connecting the user port connector WITH THE APPROPRIATE SIDE ON THE TOP

AND ON THE BOTTOM, else you'll burn the LPT of your PC.

(You can read more about the cables in the appropriate section.)

Important. Try to avoid voltage peaks when your system is connected

together. Commodore never really took care about protection against peaks...

Just keep in mind that, at least in Plus/4, the input serial lines go directly

to the processors own I/O port. If you give a peak to the thing, maybe, you'll

have to (try to) obtain a replacement 8501.

4.c) Setting up the communication

----------------------------------

After connecting the units together, you should try if the system works.

If you have a floppy drive, you can first save the receiver routines to a

1541 disk. Thus, execute for example, Star Commander. You should get its

usual user interface. If not, then your Commodore holds a line and you should

tell it to leave it. C-64s hold mostly their CLK line. Set CIA2, 0. reg as

$DD00 to 7, this way, the line OC output-driver will open. From Basic,

Poke 56576,7

Maybe, you've just got the right user-interface. Transfer the used receivers

to the disk. O.K.

(Maybe not, but then it is by another problem and around Star Commander).

Then exit the 1541 handler and try to load a receiver from the disk to the

Commodore. Use LOAD"NAME",8,1 as the receivers have no Basic boot. If the

computer seems to hang, it's no problem - your PC is holding the lines.

Execute

comlink -i

on the PC to fix this. After loading, execute the receiver, either with

SYS 1525

on Plus/4 or

SYS 52736

on 64.

Using the normal versions, you'll see their message 'waiting for transfer'.

Then, you can transfer anything from the PC.

5.) Switches and parameters

----------------------------

ComLink has no config file or real user interface. Parameters can be given

only in DOS command line. Let's have a look at the DOS parameter-list. It

looks something like this:

COMLINK [filename] [-pxxx] [-cx] [-tx] [-sxxxx] [-rxxxx]

Where:

filename    :  The name of the file you want to transfer. It may be either in

raw binary or object format.

Default: '', so ComLink will give an error-message and exit

when no filename was given.

-pXXX       :  The port-address of your LPT, in hexadecimal format. Only the

first 3 digits. Either $378 (LPT1),$278 (LPT2) or $3BC (LPT3)

is valid.

Default: -P378

-cX         :  Type of the used cable. Currently X1541, Disk64E and Parallel

cables are supported, so valid switches are 'X','D' and 'P'.

You can also use 'F', since ComLink can link the pieces

together into a PC binary file with no transfer activity at all.

Default: -CF

-tXX        :  Type of input-file. It may be either 'B'inary or 'H'exadecimal.

Use binary for all filetypes, except for .OBJ's generated by

assemblers. Using 'H', the input-file will be translated into

binary (separate pieces if it contains data in incoherent

data-areas). The blocks are transferred (or linked) then as

binary.

You can issue the 'H' setting together with 'X' (eXport). If

you do, ComLink will look for the export file generated by

the cross assembler (filename.EXP) and tries to parse the

rows. You have to issue this switch whenever you use ComLinks

linking features (block relocating and binary block linking)

since these are controlled by the export file (see 'Cross

development with ComLink').

Default: -TB

-sXXXX      :  Start address of the block. ComLink will link the block to

this start address, either into the PC binary file (linking)

or to the other computers memory.

You can however, use a special value: If you give $FFFF in

the parameter-line, ComLink will pick the first 2 bytes of the

linked block and use that as start address. (It's useable for

transferring simple one filers.)

When transferring objects, the -s option will not affect the

start address of the transferred blocks. They will be rather

picked from the objectfile.

Default: -SFFFF

-rXXXX      :  ComLink copies this value to the transferring block's 2-3

bytes. You have to enter it as hex, of course. After

transferring, the receiver routine will execute a JSR to this

address. It's no matter that it's not a simple JMP - but if

you develop a new stuff, this gives you the chance to RTS to

the receiver and waiting for transfer again if your code ran

down successfully.

There are 2 special values with different meaning. When using

$FFFF, the receiver will loop to its start and waits again.

In case $FFFE it executes an RTS, so if you called the

receiver with a SYS command from BASIC, your computer will

be back to the prompt.

When linking to file, this option has no effect.

Default: -RFFFE. It is temporarily set to -RFFFF when you

link a bunch of blocks of an object file (and also static

binary blocks, included in the export file) through a cable

(thus, the run address will be effective at the _very last

transferred block only).

There's another way of executing ComLink:

COMLINK -i [pXXX]

The program only sets the LPTs used 'open collector' I/O lines

to high state, giving the possibility for other units to

communicate on the serial bus.

Note that no additional option is needed except port-address;

extra parameters are ignored.

6.) Transferring one filers

----------------------------

The simplest task to use ComLink for is to download one filer programs,

demos, games and so on to the Commodore. Although it may seem to be a bit

strange, since there are a lot of other programs for this task, this use

seem to have a real point. If you have a lot of stuff on CD or on your HDD,

and you want just to browse around the files and try if one or another

program works, you just want to play around and load another game if you

played for two or three minutes, ComLink could be the solution since its

receiver routine is small, fast and doesn't need extra cables that a lot of

other programs do. With this, you avoid waiting for copying to disk, loading

from disk (1541 handlers); and in opposition to the use of a 1541 emulator,

you avoid loading with sloooow normal serial IEC speed (there is currently no

1541 emulator software supporting turbo load).

To download such an one-filer to the Commodore, first load and start the

appropriate receiver routine on the Commodore. After this, start the data

transfer with

comlink your_file_name -c? -p???

After the -c switch, place your cables type switch (x, d or p, see

switches), and after the -p the port address of the LPT (you can most

probably leave this latter alone, since most machines have the LPT at

address $378 which is the default in ComLink). So,

comlink nebulus.prg -cx

will hopefully transfer Nebulus to the C64 through an X1541 cable if there

is a C64 on the other end, and then the computer will be back to the Basic

interpreter so you can start the program with 'run'.

IMPORTANT NOTICE. When you use serial connection (e.g. X1541 or Disk64E),

please SWITCH OFF YOUR DRIVE before you start transferring a program to the

Commodore computer. This is a bug that I could only avoid by selecting

different communications method and with lowering the serial transfer speed.

A drive that is switched on on the line will definitely result in corrupted

transfer. (It doesn't apply for parallel connection, however).

7.) Cross development using ComLink

------------------------------------

As I should have mentioned, ComLink's linking capabilities make that group

that I started coding this whole program for. There are a lot of utilities

out there that can transfer data blocks to (and a lot of them even _from) a

Commodore computer, but I don't really know such one that would allow

linking more blocks together, even just statically. Way back in 1995 when I

started to move my coding activities to cross-platform base, I needed

something that saves me these linking tasks (since the cross assemblers I

knew were not able to do this process). BTW, if speaking of the past, I

should also mention that the transfer capabilities were implemented exactly

for the same (cross-development) reason; way back then were neither good

Commodore emulators (with debugging capabilities) available to first-check

the code, nor were 1541 handlers so fast and popular that I could seriously

think on using them for such a task.

For a cross-development one usually needs a cross assembler, a text

editor, some (rather self-coded) utils occassionally (sometimes you have to

generate constant tables by external programs - the same as you'd, for

example, generate a sine table in Basic). In the subject of Commodore, a good

Commodore emulator can also be a handy tool. The critical part is the cross

assembler; good news, you can find really a lot of them nowadays, for whatever

processor you want your code compiled for. Commodore people most probably need

a 6502 cross assembler - you can bet there are also a lot of them around the

Net. The bests include DASM, by Matthew Dillon and slightly modified by Olaf

Seibert, Taboo ASM that was created by people from Taboo (Polish C64 demo group)

and is intended to be 100% compatible with the C64 Turbo Assembler. There are

also a lot that can compile the extended 6502 command set (65c02, 65816 etc.)

or can even compile the illegal commands of the stock 6502 serie. (Just see

the Simtel archives for such programs).

Me personally decided with the shareware Table Driven Assembler by Speech

Technology Inc. This program contains just the 'raw' compiling engine, and

the actual command sets with the appropriate infos are stored in external

data files. This way, TASM can compile code for dozens of processors by

default, and _whatever processor you make a command set table for. That's

why I like it so much - besides these, it understands a lot of (rather

standard) directives that one can remember either from his Commodore ASM or C

language programming experiences :-). Macros can also be used, of course.

One drawback of the program is, that I don't know if it was ever ported to

other systems than the good old PC/DOS. And since it is shareware, I also

don't know if it will be ever ported at all. For the moment, I still rather

use the DOS command line for such development (this document was also written

in DosNavigator), but I understand that a lot of people will refuse to use such

an assembler (and BTW, also ComLink, since it's still DOS only) because of

such drawbacks.

After talking this over, let me talk about the fact that ComLinks linking

capabilities are (rather) strongly associated with TASM's features. The raw

linking capability (to link the pieces of the compiled code you generated

with an assembler) depends on one feature only: whatever assembler you use,

it must support the Intel HEX object format as output. ...But, else in case

of quick first-testing of your code by transferring it through a cable right

after compiling, this doesn't make much sense, since you can link your

program easier by telling your assembler to generate raw binary output

instead of generating whatever HEX object file. (If you generated such a raw

binary output with your assembler, you can of course use ComLink to

quick-transfer it to the Commodore, but this involves no linking features of

ComLink to be used).

The other features depend on TASM's export file generating capability.

This feature makes it possible to put certain labels in the assembly source

to be collected into an external textfile (in labelname EQU address form,

one label in each row). The dependence comes from the fact that ComLink

picks certain labelnames from here during the linking process and decides

what to do with the appropriate blocks.

As a consequence, you can use ComLinks extra capabilities as long as your

assembler is capable of generating an 'export file'; a file that contains

one 'label  .EQU value' record in each rows for all appropriate labels.

7.a)  Including external binary files

--------------------------------------

The process is controlled by the labels you write and export into / from

your source code. Let's see, how to include some static located external

data into your program (the example below was cut from the SIDwinder F1.22

editor source).

;-----

;        .                 ;Sum other code...

;        .

;        .

i_chars  = $0800

.export i_chars   ;Load the characterset, please...

i_masks  = $d000

.export i_masks   ;Screen masks

i_tracks = $e000

.export i_tracks  ;Default track tables

i_sectors = $7000

.export i_sectors ;Default sector data

;        .

;        .

;----

You can see, how it works. First, somehow, you give a value to a label,

whose name starts with 'i_' and _continues with the _filename of the raw

file to be loaded. Then, you tell TASM to list the labels into the export

file (if you issue at least one .export directive in the source code, the

sourcename.EXP file will be generated). The filename must be the name you

wrote after the 'i_' _plus a .BIN extension.

Then, after you assembled your code with 'tasm -65 sourcename', you'll also

get a sourcename.EXT file besides the sourcename.OBJ (and the sourcename.LST

but ComLink doesn't use that file). Now, for example, if you want your whole

program to be linked into a raw binary file, execute

comlink sourcename.OBJ -thx

You'll get a sourcename.BIN raw binary image, that has no Commodore-style

load address at the start. (However, the start address of the raw binary image

should be the start address of the block that had the lowest start address

in the whole linking process). If, in the above example, CHARS.BIN,

MASKS.BIN, TRACKS.BIN and SECTORS.BIN were available, they get linked into

the image (else, ComLink complains that the file couldn't be found).

The method makes it also possible that you can include such binaries if you

actually don't know their starting address in the memory, 'cos these

addresses become known in the assembling process. Let's have a look at an

example, how to tell ComLink to link such block into your program! (This

example was cut from LaserMax, the disco-laser scanner controller program I

also wrote back in '96)

-----

;        .

;        .

i_log1

.export i_log1

.block 256

i_log2

.export i_log2

.block 256

i_logsin

.export i_logsin

.block 256

;        .

;        .

----

As result, the external files log1.bin, log2.bin and logsin.bin will be

inserted into the binary image, to the addresses pointed by their

appropriate labels. The .block directives (that skip _parameter bytes) reserve

place for the inserting block; from this, you see, you have to know the exact

length of the appropriate files, since you _have to reserve enough space for

them in the source file.

Since this works exactly the same way as the above one does (you just gave

value for the labels in a different way), you have to link it just the same

way to that one.

Of course, you're not stuck with linking to the raw binary file; if you

want, you can tell ComLink to link them 'on the fly' to a Commodore's

memory, through the cable. For example, issuing

comlink editor.obj -cx -thx -r0c00

will for me transfer all parts of the SIDwinder editor through an X1541

cable to my C64 and start the code immediately after finishing with the last

block's transferring.

7.b)  Relocating codes

-----------------------

Like in the introduction, the problem is as follows. You want something to

be at a special address range. For example, let this be a digital play

routine that you want to run at the zeropage. Because of the assembler, you

must tell the assembler the real start address of this portion (like,

*= $0002 ). You then write your player code. Now, you probably want this part

to be linked somewhere into your program, from where it will be copied to

its place in the initialization part. How to achieve this?

This example was also cut from the LaserMax stuff, where the lowlevel

player routine was designed to run on the zeropage.

-----

;        .

;        .                 Some code

;        .

*= $0002

_irqs

retrace .byte 0

_nmi    pha

tya

pha

_incnt  = *+1

ldy #0

dey

adc #0

sta _qhcnt

;        .                 Some code

;        .

;        .

;        .

pla

tay

pla

bit $dd0d

rti

_irqe

*=$0900           ;Important that the relocating block must be

;disjunct from the other code blocks

;        .                 Some code

;        .

;        .

;        .

r_0002

.export r_0002

.block _irqe-_irqs

;        .

;        .

----

As it is seen, the special code is compiled to the original address.

_irqs and _irqe were set to the start and the end of the code - from these,

you can later calculate the length of the block. The data block to be relocated

must be _completely disjuct from your other code - thus, issue a *= directive

both at the start and at the end of the code part, to make the assembler

generate a separate data block in the object file. Later, in the main program,

define somewhere the special label that controls the relocation. In the

example, R_0002 is 'Relocate block found at the address '0002' to the

address given by the value of this label'. And, as you know from the previous

example, you have to reserve an appropriate area for the relocated code. (You

can also relocate to a static address, BTW). For ComLink, the block to be

relocated is known from the label name, and it will relocate the whole

continuous data block it finds with this start address in the compiled code -

regardless to what you intended to be the end if you haven't closed the area

properly, so please take care of this. The address the block is to be

relocated is determined by the value of the label.

The linking goes exactly the same way as in the previous section (giving

the -thx switch is important only).

7.c)  Tips and tricks

---------------------

Well, not much to tell, just some words. As I said, if you use a Commodore

emulator, be it C64 or Plus/4, it can make things go faster significantly.

Personally, I use VICE and Minus4 - because both have built-in debugger and

monitor (and because VICE is the best C64 emulator I've seen so far, not to

mention its capability of emulating a VIC-20 or a PET ;-) ). If you rather

decide with testing your code via linking and transferring it 'on the fly'

to a C64 or a Plus/4, ComLink's receiver can also help you since it executes

the code with JSR instead of JMP. Because of this, if you take care of

re-initializing everything in your program, you don't have to reset your

machine and restart the receiver everytime you tested your code. Rather, let

your program re-initialize the system variables, the chips etc. and simply

execute an RTS, with which your Commodore will be back to the waiting state

for another transfer...

8.) The cables

------------------------

8/a.) Serial cables

-------------------

As I said, no special cables are needed for the data transfer. If you

fiddled with PC-1541 disk-handlers recently, the cable you already have will

most likely do. At this time, both X1541 and Disk64E cables can be used.

You'll probably have the X1541 type one (I've seen no Disk64E cables since

years).

Also, at this time no special serial and parallel cables (XE1541, XP1541,

XH1541 and so on) are supported. The main reason, I don't have these. The

other reason: XE1541 cables were introduced because newer LPT ports don't

behave exactly as regular SPP LPT ports did, so these cables were made as a

work-around. They're also quite difficult to build, due to the fact that

they need Schottky diodes that are both expensive and hard to obtain. In

case of having bad (new) LPT port on your computer, buy a standard ISA LPT

card in a garage shop (maybe, for cents or even for free) and use that. As

speaking of the XP1541 and XH1541 cable: the XP1541 is exactly the same as

ComLink's parallel cable's parallel part, but is assembled different (with

SPP, it doesn't work bi-directionally, and with an EPP port, the X1541 part

refuses to work - thus regular XP1541s used to be assembled as a separate

_8_parallel_lines_only cable from an LPT to the parallel lines of such an

1541 drive). ...I should probably support it, it would just need to allow two

LPT ports to be specified. The XH1541 is also a work-around and thus I don't

really want to support it (it is a 4-bits-to-both-directions cable for SPP

parallel ports, achieving noticeable speedup for 1541 handlers).

You need the following parts to build a serial cable (I'd strongly suggest

to build the X1541 first):

DIN 6 pin plug (the same type is used as video-cable plugs.)

Canon DB-25 connector (male), as LPT plug.

DB-25 connector shell

4 or 5 wire cable

solder, soldering iron

Canon DB-25 (male, from the back)

.------------------------------.

13\ ************************** /1

\ ************************ /

25`------------------------'14

DIN (male,from the back)

_     _

/   \_/   \

/ 5*     *1 \

I      *6     I

\ 4*     *2 /

\    *3   /

` --- '

The connections:

X1541

-----------------------------------------------------------

Signal name    DB-25 pin       Signal name  DIN-6 pin

-----------------------------------------------------------

STROBE'        1               ATN             3

AUTOFD'        14              CLK             4

{INIT'          16              RESET'          6}

SEL_IN'        17              DATA            5

GROUND         18-25           GROUND          2

-----------------------------------------------------------

Disk64E

-----------------------------------------------------------

Signal name    DB-25 pin       Signal name  DIN-6 pin

-----------------------------------------------------------

STROBE'        1               CLK             4

AUTOFD'        14              DATA            5

{INIT'          16              RESET'          6}

SEL_IN'        17              ATN             3

GROUND         18-25           GROUND          2

-----------------------------------------------------------

I'd suggest _not to connect INIT' and RESET'. By the way, ComLink never sets

INIT' to 0, to reset external devices.

8/b.) Parallel cable

---------------------

Now, some about the other cable that gives better transfer rate. It's just

an X1541 cable, extended with 8 data-lines that connect the LPT data bits

to the user port's data bits.

Since the user port does not have the same layout on all Commodore micros,

this means that parallel cables depend on the Commodore computer you use.

Usually, you'll need a specific user port connector besides the things

listed above, and additional 8 wires of cable. The best would be a ribbon

cable in one, with 8+4(5) = 12 (13) wires. When assembling it, you should put

a common DB-25 connector to one and two separate (DIN-6 and the user-port)

connectors to the another end. Be sure that you connected right wires to right

pins in the DB-25, before you try to separate the other end of the cable! :-)

C-64

-----

Signal name    DB-25 pin       Signal name     User port   DIN-6 (serial)

STROBE'        1               CLK             -           3

D0             2               PB0             C

D1             3               PB1             D

D2             4               PB2             E

D3             5               PB3             F

D4             6               PB4             H

D5             7               PB5             J

D6             8               PB6             K

D7             9               PB7             L

AUTOFD'        14              DATA            -           4

{INIT'          16              RESET'          -           6}

SEL_IN'        17              ATN             -           5

GROUND         18-25           GROUND          -           2

C-64 User port connector:

1    2  3  4  5  6  7  8  9 10   11 12

**   ** ** ** ** ** ** ** ** **   ** **

---   --------------------------   ------

**   ** ** ** ** ** ** ** ** **   ** **

A    B  C  D  E  F  H  J  K  L    M  N

Note that 'G' and 'I' letters are missing from the layout.

Plus/4

-------

Signal name    DB-25 pin       Signal name     User port   DIN-6 (serial)

STROBE'        1               CLK             -           3

D0             2               P0              B

D1             3               P1              K

D2             4               P2              4

D3             5               P3              5

D4             6               P4              6

D5             7               P5              7

D6             8               P6              J

D7             9               P7              F

AUTOFD'        14              DATA            -           4

{INIT'          16              RESET'          -           6}

SEL_IN'        17              ATN             -           5

GROUND         18-25           GROUND          -           2

Plus/4 user port connector:

1  2  3  4  5  6  7  8  9 10 11 12

** ** ** ** ** ** ** ** ** ** ** **

-------------------------------------

** ** ** ** ** ** ** ** ** ** ** **

A  B  C  D  E  F  H  J  K  L  M  N

Note that 'G' and 'I' letters are missing from the layout.

Fortunately, the size of the Plus/4 user port connector is the same as

the C64's one; if you can obtain one for the 64, you can also obtain one

for the +4.

8.c)  Parallel port for C-16 / C-116

-------------------------------------

This is hard, in first approach. These computers have no user port. It seems

to be difficult to present parallel connection with them. Maybe one could

fiddle a bit with the joystick ports, but anyway, I don't believe that it

could be made with no modifications to the computer (...except with an

expansion card, of course, but it means the same complexity).

After thinking it over, I'd suggest not to touch the motherboard at all if

you only aim to get ComLink to work in parallel mode (too much trouble for

such a reason, rather stay with the serial transfer or get a Plus/4).

If you still consider the modification neccessary, in first step I suggest

to use an original MOS 6529B I/O chip, also found in Plus/4. The only

problem is, it is hard to obtain (if at all), and it is expensive. Still, it

_is the best solution since it will save you a lot of trouble, fiddling and

time.

If obtaining a 6529B is impossible, you can make an own board substituting

it. Well, it depends on your demands. If you want to have a simple input or

output port, you can use either 74373,74573,74245 - mostly, from the LS-TTL

or CMOS HCT series as 74LS373, 74HCT573 etc. - as input latch and 74373,74573

as output. As speaking of the output, you have one more feature to implement

- the 6529B-like open collector output driving. This can be done by a lot of

OC line driver gates (7407, two pieces) or a good tip could be the 74757,

since it is an 8-bit OC bus driver in one. For full compatibility, you'll have

to use both an input and an output latch, this latter with OC outputs. With

short calculation, this means at least 3 IC's and 8 resistors of some Kohms

(these resistors are pullups). It also seems to mean that you have to design

and produce a PCB (too much components to piggy back them together).

AS I SAID ALREADY, I ACCEPT NO RESPONSIBILITY FOR POSSIBLE DAMAGES OCCURING

WHEN INSTALLING MODIFICATIONS DESCRIBED HERE. AND I CAN'T GUARANTEE THAT YOU

CAN DO IT IF I DID. IF YOU TRY IT, YOU DO IT AT YOUR OWN RISK!

INSTALLING INTERNAL THINGS IN YOUR COMPUTER WILL VOID WARRANTY!

(...If any, 16 years after releasing the C16... )

Using a termostate soldering iron (a Weller, preferably) is a damn good idea,

speaking in other words, simply forget touching the board with an instant

soldering iron (it would most probably burn a lot of chips on the board by its

peaks).

I think, the circuit can be installed most easily by mounting it directly to

the top of an existing 6529B chip on the board (the keyboard latch). Well,

this is not an industry standard way but it works and you don't have to make

an extra board.

First, pull out all cables from the computer, and open it, removing the

screws. Then, disconnect the keyboard and the LED. Open the shielding paper

and remove screws from the mainboard. Take the board out of the cover. You'll

see a heatsink and a 7805 voltage- stabilizer IC with three pins, and just

below it the U13 6529B. Bend out pins 2,3,4,...,9 and 19 on the new 6529B and

solder it to the keyport latch 6529B chip with the remaining pins. Then solder

a wire to the bended p19 and solder the another end to pin 16 of U16, 7700

FPLA. ...The FPLA might have different number, not 7700. You'll most probably

have to connect the wire to the bottom side of the mainboard, as the FPLA is

in socket. Be sure not to short extra fields on the board! Then, locate U11,

74LS125 and cut the field from its pin8. Solder a wire to pin 9 and connect

its another end to pin4 of the newly inserted 6529B. This line comes from the

CST SENSE line of the tape and it's used for detecting the tape-keys. In

Plus/4, the User port's 6529B is also used for this task, whereas in C-16 and

C-116, the 74LS125 bus-driver is used. (We've just cut it out...). O.K. Then,

solder a 9 wire ribbon cable to pins 2-9 of 6529B and solder them to the pins

of a connector. Solder the 9th wire to GROUND (pin 10 of 6529B is good). This

connector may be either a Canon DB-9 or a Canon DB-15 or any type of

connectors you want. ...Well, it will not be ("shape-") compatible with the

original edge connector, only after building an interface. If you're good in

making boards, maybe you can do it but it involves quite a lot of trouble I

guess.

After you've soldered everything, then try your work out. Be sure that you

turned off the power switch before connecting power... Connect RF cable, turn

on the monitor and then, the mainboard. If you see the usual message

'Commodore Basic V3.5 12277 (or 60671) bytes free' and the prompt, the unit

passed the first test. Type 'LOAD' and if it answers 'press play on tape'

and doesn't blank the screen immediately, the second pass is also O.K.

Type 'MONITOR' and write '>FD10 FF'. Measure the levels on the new portbits,

you should get high level on all lines. Type '>FD10 00', then try again -

the portbits should be on ground. Type '>FD10 FF' again and short a line to

ground. Then typing '>FD10' should give a value with 0 bit at the appropriate

place.

If the unit doesn't seem to work, there are several possibilities. If it

starts to boot up but it hangs before could print '12277' or '60671',

probably you shorted few traces around the FPLA. Well, if you did no brutal

movement, this should be some short-circuit or loose wire. ...If you burnt

something, that's bad luck since you'll most probably have to purchase

another black beast... :-(  (don't seriously count on obtaining a TED or a

8501 CPU, it's almost impossible to do).

Well, if everything seems to be O.K. then assemble the parts together.

Finally, this is the pin-out of the 6529B.

.-----\_/-----.

R/W' -I 1        20 I- VCC

P0 -I 2        19 Io CS'

P1 -I 3        18 I- D0

P2 -I 4   MOS  17 I- D1

P3 -I 5        16 I- D2

P4 -I 6  6529B 15 I- D3

P5 -I 7        14 I- D4

P6 -I 8        13 I- D5

P7 -I 9        12 I- D6

GND -I 10       11 I- D7

`-------------'

And it's a sketch of a possible substitution of the 6529B

1K

.-----\_/-----.         7407  /-\/\/-> +5V

GND<--  OC'oI 1        20 I- VCC    I\    I

D0<--  D1 -I 2        19 I- Q1 --- I  > -+----> P0

D1<--  D2 -I 3        18 I- Q2     I/

.  D3 -I 4        17 I- Q3  .

.  D4 -I 5        16 I- Q4  .

.  D5 -I 6  74573 15 I- Q5  .           1K

.  D6 -I 7        14 I- Q6  .  7407  /-\/\/-> +5V

.  D7 -I 8        13 I- Q7  .  I\    I

D7<--  D8 -I 9        12 I- Q8 --- I  > -+----> P7

GND -I 10       11 I- CE -\  I/

`-------------'      I    /I 7404

CS' I\  7404                          \-o<  I <------ CS'

--> I  >o-\                                \I

I/    I      .-----\_/-----.

\- OC'oI 1        20 I- VCC

P0 --> D1 -I 2        19 I- Q1 --> D0

P1 --> D2 -I 3        18 I- Q2 --> D1

. D3 -I 4        17 I- Q3 .

. D4 -I 5        16 I- Q4 .

. D5 -I 6  74573 15 I- Q5 .

. D6 -I 7        14 I- Q6 .

. D7 -I 8        13 I- Q7 .

P7 --> D8 -I 9        12 I- Q8 --> D7

GND -I 10       11 I- CE --> VCC

`-------------'

You can most probably substitute the bunch of 7407 drivers by one 74757,

8 bit open collector bus driver (not tested).

Some comments. I think you should select LS or HCT TTL's for less

dissipation and TTL level compatibility. The circuit is mostly based on

two independent 74573's with some additional things but I left

a lot of lines because of its visibility. When designing the board,

please connect all lines signed as P0-P7, D0-D7 and CS' in the separate

parts together. And connect all unused inputs of 7407 / 7404 to GND.

For a partial implementation, you can use just the upper or the lower part

for output or input. By the way, it's well enough to have the lower part for

the requirements of ComLink.

9.) Inside the program

-----------------------

On PC, all tasks are done by one single executable, ComLink.exe. This one

was developed in Turbo Pascal by Borland and includes a lot of inline ASM.

Most of the code was written way back in 1996, and rather in the 'quick

and dirty' style (since Pascal fits so much for this style - almost as a

good tool for DOS as Perl is for *nix ;-) ). I don't think if anybody with

TPC couldn't easily compile it again, but on the other hand, I have serious

doubts if it could be ported to other systems without a lot of modifications.

BTW, writing it portable was not my intention, I just wanted something that

does such tasks. ...I haven't used much features of Borlands 'self-compatible'

Pascal, rather just a lot of ASM that needs to be rewritten entirely in Pascal

in order to have a small chance to compile the thing with Free Pascal Compiler

or HiSoft Pascal (if you have Linux/PC or an Amiga, for example).

Since I have quite a lot of better things to do, I don't seriously plan

implementing such modifications into the program. It fits for me for the

moment. ...But if you consider it worth the work for you, you're welcome to

do so (...see, this program was released under the GPL for everyone's

pleasure ;-) ). If you did such work, please contact me, I'll check your

results for sure!

As speaking of the program functions inside... since the linking to file

feature was added later, the program builds around the block transmitter

routine (if linking to file, the appropriate block is just copied into

somewhere a 64k block). You'll also find a parameter parser, a file loader

(both binary and hex) and small routines around these supporting the

relocating / including features. ...Quite simple code, I guess, and anyway,

it should be quite readable, even if it's not well portable at all.

The block transmitter is written in such a way that it transmits blocks to

whatever computer hooked to the another end. It is no matter if this computer

is a C64 or a Plus/4, since the protocol is conform (the routine just needs

info about the cable type and the LPT port address, but nothing more).

In opposition to the PC side, where you find only one executable file for

all functions, the used receiver routine depends on the type of your computer

and the type of the cable; from the other hand, the very same code is used

in all receivers but they're compiled with different addresses and options.

The receiver routines were written with small memory usage and high speed in

mind (that's why you can compile the receiver source with 'quiet' option,

thus the receiver will not log the activity on the screen). All receivers can

load a block to anywhere in the RAM, except their own location. (If this

latter becomes a problem in your particular project, simply change the start

address of the receiver routine in the source, then compile it again - that's

so simple).

For compiling the receiver source, you'll need TASM (mentioned above).

Issuing buldall.bat compiles all possible receiver versions. Since the

receiver source was written utilizing TASM's conditional compiling, the

batch just defines some macros in the command line and TASM then compiles

the appropriate version (see the batch and the source on the macros and

about defining them if you need a particular version).

By default, on C-64, the receiver routines compile to $CE00. On Plus/4,

all routines start on $05F5, they compile to the place reserved for the

built-in user software package's boot routines.

The names of the compiled receivers (after issuing buildall.bat) should be

denoted as something like this:

- 'CL'          - ComLink

- '64','P4'     - Type of the computer, as C-64 or Plus/4. If you have a

funny C-16 or C-116, you should also use these latter

routines.

- 'P','S'       - Connection type, Parallel or Serial. You should use the

'P' version with parallel cable and 'S' using serial

cables. Both X1541 and Disk64E cables are serial, and

there's no need for different receivers using them; use

the same receiver for both.

- 'N','Q'       - Normal or Quiet version. Using normal, you'll have the

usual messages of the receiver, printed on the screen.

Quiet versions don't write anything on the screen.

All versions update $2B-$2C, $2D-$2E after receiving a block. So, the

computer will have the right start-end BASIC pointers when you transfer

something (this counts, if you use the program for example for transferring

an one filer game).

Now some about the algorithm...

Transferring datas is not else than sending a set of BLOCKS through the

cable. Each blocks have three words (16 bit) in their begin, start address,

data-size and an address for executing the code in the block, if any. Then,

the block continues with the data-bytes. There's no checksum; bad data

transmit used to reveal things that you won't miss if they occurs, and

implementing checksums would be simply wasting memory and speed. The receiver

keeps on watching the lines and waiting for transfer until a block doesn't

have a real executing address; this way, it's possible to transfer more

blocks in a loop, and when all data was transferred, give an execute command.

Object files are transferred as separate blocks if they contain data in

separate memory-areas.

The maximum block-size is 65526 bytes, it comes from the largest block-size

allocated by GetMem procedure of Pascal. I think it's quite enough this time.

By the way, there's no limitation to the size of raw hex object-files, since

they're converted to binary just before transferring, and if you have a block

larger than 63K in an object, ComLink transmits it in more steps (although

currently this doesn't seem to have much sense. At the moment, no memory

mapping and only 16 bit addresses are supported).

Some about the header of the blocks.

- The first word (L/H) is always the block's size in bytes. Theoretically,

it could be anything between 0 and $FFFF, but as I said, ComLink doesn't send

blocks larger than 65526 bytes; since 'size' counts only the data-bytes, it

can be max. 65520.

- Second word: run-address. ComLink tries to run the transferred thing

with a JSR to this address after finishing the transfer. It tries, except if

you use one of the two special values - giving $FFFF will cause a jump back to

the begin of the receiver, so it will keep on waiting for more blocks. And

giving $FFFE, the receiver will return to BASIC or wherever from you

called it with a JSR, as it will simply execute an RTS.

- Third word: Start-address in the memory. The receiver will place the

received data-bytes from this address continuously. There's no limitation for

its value, even $FFFF is valid.

At the physical level of the communication, the block-transmitter uses 2

handshake lines to detect if a data was received by the another computer, and

to note the valid data on the output. These lines are CLK and DATA when using

parallel and CLK and ATN when serial connection. The data lines are the D0-D7

lines of the parallel port, and DATA signal when using serial connection.

(Using ATN is the reason of broken data transfers if you leave the floppy

on while transferring a block in serial data transfer mode. Right in the

moment when one of the computers pull ATN low, the drive notices it and

pulls its data line low as answer - thus breaking the process. The only

possible way to remedy this problem is not using the ATN line in the process

at all. This involves a 2-line-only serial data transfer protocol to be used

between the computers. This would be possible to implement, even in such

synchronous mode ComLink currently uses - as Marko proved with his Prlink

serial routines, this is fully possible to do. From the other hand - it

would be significantly slower than the current serial method is. And I'm

also quite lazy to implement it to the program :-/. Sorry, C-128D users out

there... )

ComLink utilitizes a timer function, to detect if no computer is hooked up

or no receiver is running on the other side or if the connection is bad. It

waits for one second for a response from the other computer. It sets this

'watchdog' function every time waiting for a handshake-bit. (By the way,

it uses the 0. timer of the PC, that is used for the DOS-clock. Thus the

system timer may be late after transferring some blocks. Of course, this

doesn't affect the date and the time of the CMOS setup.)

10.) Contact

-------------

If you have questions, thoughts, suggests etc. contact me via

email at

Levente@Terrasoft.HU

Or by snail mail:

H�rsfalvi Levente

7200 Domb�v�r

Gorkij 33.

- Hungary -

(A short note regarding my name. Surname, firstname. Hungarian name order.

Did you know?... )

You can most probably find this program at the known Commodore FTP sites -

for example, in the FTP.FUNET.FI Commodore archive, somewhere under

FTP://ftp.funet.fi/pub/cbm/crossplatform/transfer/CBM-to-PC/

11.) Thanks and respects

--------------------------

- To Marko M�kel�, for his support and his idea of making one common

receiver source file with conditional asm.

- Borland Inc.

- Speech Technology Inc.

- RIT Research Lab. (DosNavigator)

- To all people who contacted me with questions and thoughts

during the few last years!

APPENDIX

A.) Some measurements

-----------------------

I used my old 486DLC-40/256K cache (ISA) PC clone with my C-16/60K and C-64,

and measured all receiver-versions both in normal and in blanked screen mode.

All results are in kbytes/seconds.

NORMAL      BLANK

C64

CL64PN   18.9        19.8

CL64PQ   21.32       22.48

CL64SN   3.3         3.52

CL64SQ   3.4         3.6

PLUS/4

CLP4PN   29.7        43.77

CLP4PQ   36.16       51.98

CLP4SN   4.62        6.93

CLP4SQ   4.72        7.04

(The Plus/4 is faster by its higher clock-rate and easier memory mapping.)