;	Commodore 128 RAM Memory Map

;	V1.3b	18 Nov 1994

C128 Memory Map

This is as complete as possible compilation of two separate memory maps,

and a few random sources which naturally do differ from each other.

C128 RAM

All Banks

0000		I/O direction register

0001		I/O port, similar to C64

0002		Token 'SEARCH' looks for, Bank Number, Jump to SYS Address

0003-0004	SYS address, MLM register PC

0005-0009	SYS and MLM register save (SR, A, X, Y, SP)

0005		SR Temp

0006		AC Temp

0007		XR Temp

0008		YR Temp

0009		SP Temp, Integer

000a		Scan-quotes flag

000b		TAB column save

000c		Flag: 0 = LOAD, 1 = VERIFY

000d		Input buffer pointer / number of subscripts

000e		Default DIM flag

000f		Data Type: FF = string, 00 = numeric

0010		Data Type: 80 = integer, 00 = floating point

0011		DATA scan / LIST quote / memory flag

0012		Subscript / FNxx flag

0013		Flag: 0 = INPUT, $40 = GET, $98 = READ

0014		ATN sign / Comparison evaluation flag

0015		Current I/O prompt flag

0016-0017	Integer value

0018		Pointer: temporary string stack

0019-0023	Stack for temporary strings

0024-0027	Utility pointer area

0028-002c	Product area for multiplication

002d-002e	Pointer: Start-of-BASIC (bank 0) [1C01]

002f-0030	Pointer: Start-of-variables (bank 1) [0400]

0031-0032	Pointer: Start-of-arrays

0033-0034	Pointer: End-of-arrays

0035-0036	Pointer string-storage (moving down)

0037-0038	Utility string pointer

0039-003a	Pointer: Limit-of-memory (bank 1) [FF00]

003b-003c	Current BASIC line number

003d-003e	Textpointer: BASIC work point (chrget)

003f-0040	Utility Pointer

0041-0042	Current DATA line number

0043-0044	Current DATA address

0045-0046	Input vector

0047-0048	Current variable name

0049-004a	Current variable address

004b-004c	Variable pointer for FOR/NEXT

004d-004e	Y-save, op-save, BASIC pointer save

004f		Comparison symbol accumulator

0050-0055	Miscellaneous work area, pointers, and so on

0056-0058	Jump vector for functions

0059-0062	Miscellaneous numeric work area

0060-0062	MLM Address 0

0063-0065	MLM Address 1

0066-0068	MLM Address 2

0063		Accum #1 exponent

0064-0067	Accum #1 mantissa

0068		Accum #1 sign

0069		Series evaluation constant pointer

006a-006f	Accum #2 exponent, and so on

0070		Sign comparison Acc #1 versus #2

0071		Accum #1 lo-order (rounding)

0072-0073	Cassette buffer len / Series pointer

0074-0075	Auto line number increment

0076		Graphics flag: FF = Graphics allocated, 00 = Not allocated

0077		Color source number

0078-0079	Temporary counters

007a-007c	DS$ descriptor

007d-007e	BASIC pseudo-stack pointer

007f		Flag: 0 = direct mode

0080-0081	DOS, USING work flags

0082		Stack pointer save for errors

0083		Graphic color source

0084		Multicolor 1 (1)

0085		Multicolor 2 (2)

0086		Graphic foreground color (13)

0087-008a	Graphic scale factors, X & Y

008b-008f	Graphic work values

0090		Status word ST

0091		Keyswitch 1A: STOP and RVS flags

7	STOP

6	Q

5	C=

4	Space

3	2

2	CTRL

1	<-

0	1

0092		Timing constant for tape ($80)

0093		Work value, monitor, LOAD / SAVE: 0 = LOAD, 1 = VERIFY

0094		Serial output, deferred character flag

0095		Serial deferred character

0096		Cassette work value

0097		Register save

0098		How many open files

0099		Input device, normally 0

009a		Output CMD device, normally 3

009b-009c	Tape parity, output-received flag

009d		I/O messages: 192 = all, 128 = commands, 64 = errors, 0 = nil

009e-009f	Tape error pointers

00a0-00a2	Jiffy clock HML

00a3-00a4	Temp data area

00a5-00a6	I/O work bytes (tape)

00a7-00ab	RS-232C Receive, Misc Work Values

00a7		RS-232 Input Bit Storage, Cassette Short Count

00a8		RS-232 Bit Count In, Cassette Read Error

00a9		RS-232 Flag For Start Bit Check, Cassette Reading Zeroes

00aa		RS-232 Byte Buffer, Cassette Read Mode

00ab		RS-232 Parity Storage, Cassette Short Cnt

00ac-00ad	Pointer for tape buffer and screen scrolling

00ae-00af	Tape end address / End of program

00b0-00b1	Tape timing constants

00b2-00b3	Pointer: Start of tape buffer

00b4-00b6	RS-232 Send, Misc work values

00b4		RS-232 Bit Count

00b5		RS-232 Next Bit To Be Sent

00b6		RS-232 Byte Buffer

00b7		Number of characters in file name

00b8		Current logical file

00b9		Current secondary address

00ba		Current device

00bb-00bc	Pointer to file name

00bd		RS-232 TRNS Parity Buffer

00be		Cassette Read Block Count

00bf		Serial Word Buffer

00c0		Cassette Manual/Cntrolled Switch (Updated during IRQ)

00c1-00c2	I/O Start Address

00c3-00c4	Cassette LOAD Temps (2 bytes)

00c5		Tape Read/Write Data

00c6-00c7	BANKS: I/O data, filename

00c8-00c9	RS-232 input buffer addresses  [0c00]

00ca-00cb	RS-232 output buffer addresses [0d00]

00cc-00cd	Keyboard decode pointer (bank 15) [fa80]

00ce-00cf	Print string work pointer

00d0		Number of characters in keyboard buffer

00d1		Number of programmed chars waiting

00d2		Programmed key character index

00d3		Key shift flag:

0	No shift

1	Shift

2	C=

4	Control

8	Alt

15	Caps Lock

00d4		Current key code: 88 = no key

00d5		Previous key code: 88 = no key

00d6		Input from screen / from keyboard

00d7		40 / 80 columns: 0 = 40 column screen

00d8		Graphics mode code

000	0	Graphic 0

001	1		1

011	2		2

101	3		3

111	4		4

00d9		Character base: 0 = ROM, 4 = RAM

00da-00df	Misc Editor work area

00e0-00e1	Pointer to screen line / cursor

00e2-00e3	Color line pointer

00e4		Current screen bottom margin

00e5		Current screen top margin

00e6		Current screen left margin

00e7		Current screen right margin

00e8-00e9	Input cursor log (row, column)

00ea		End-of-line for input pointer

00eb		Row where cursor lives

00ec		Position of cursor on screen line

00ed		Maximum screen lines (24)

00ee		Maximum screen columns (39)

00ef		Current I/O character

00f0		Previous character printed

00f1		Character color

00f2		Temporary color save

00f3		Screen reverse flag

00f4		Quotes flag (Editor)

;		0 = direct cursor, else programmed

00f5		Number of INSERTs outstanding

00f6		255 = Auto Insert enabled

00f7		Text mode lockout (SHFT-C=): 0 = enabled, 128 = disabled

00f8		Scrolling: 0 = enabled, 128 = disabled

00f9		Bell (CTRL-G):  0= enable, 128 = disable

00fa-00fe	Not used

00ff		Basic Scratch

0100-01ff	Processor Stack Area

0100-013e	Tape error log

0100-0124	DOS work area

0100		Tape Read Errors, Area to build filename in (16 bytes)

0110		DOS Loop Counter

0111		DOS Filename 1 Len

0112		DOS Disk Drive 1

0113		DOS Filename 2 Len

0114		DOS Disk Drive 2

0115		DOS Filename 2 Addr

0117		BLOAD/BSAVE Starting Address

0119		BLOAD/BSAVE Ending Address

011B		DOS Logical Addr [00]

011C		DOS Phys Addr [08]

011D		DOS Sec. Addr [6F]

011E		DOS Record Length

0120		DOS Disk ID

0122		DOS DSK ID FLG SPACE Used by PRINT USING

0123		Pointer to Begin. NO.

0124		Pointer to End NO.

0125-0138	PRINT / USING work area

0125		Dollar Flag

0126		Comma Flag, PLAY: VOXTUM flag

0127		Counter

0128		Sign Exponent

0129		Pointer to Exponent

012A		# Of Digits Before Decimal Point

012B		Justify Flag

012C		# Of Pos Before Decimal Point (Field)

012D		# Of Pos After Decimal Point (Field)

012E		+/- Flag (Field)

012F		Exponent Flag (Field)

0130		Switch

0131		Char Counter (Field)

0132		Sign No

0133		Blank/Star Flag

0134		Pointer to Begin of Field

0135		Length of Format

0136		Pointer to End Of Field

0137		System Stack ($0137-$01FF)

0200-03ff	Vectors, buffers, Kernal RAM

0200-02a1	BASIC & Monitor input buffer

02a2-02ae	674-686	Kernal RAM

02a2-02ae	Bank Peek Subroutine

02af-02bd	Bank Poke Subroutine

02be-02cc	Bank Compare Subroutine

02cd-02e2	JSR to Another Bank

02e3-02fb	JMP to Another Bank

02fc-02fd	Function Execute Hook	[4c78]

0300-0301	Error Message Link	[4D3F]

0302-0303	BASIC Warm Start Link	[4DC6]

0304-0305	Crunch BASIC Tokens Link [430D]

0306-0307	Print Tokens Link	[5151]

0308-0309	Start New BASIC Code Link [4AA2]

030a-030b	Get Arithmetic Element Link [78DA]

030c-030d	Crunch FE Hook		[4321]

030e-030f	List FE Hook		[51CD]

0310-0311	Execute FE Hook 	[4BA9]

0312-0313	Unused

0314-0315	IRQ Vector		[FA65]

0316-0317	Break Interrupt Vector	[B003]

0318-0319	NMI Interrupt Vector	[FA40]

031a-031b	OPEN Vector		[EFBD]

031c-031d	CLOSE Vector		[F188]

031e-031f	Set Input Vector	[F106]

0320-0321	Set Output Vector	[F14C]

0322-0323	Restore I/O Vector	[F226]

0324-0325	Input Vector		[EF06]

0326-0327	Output Vector		[EF79]

0328-0329	Test STOP Key		[F66E]

032a-03ab	GET Vector		[EEEB]

032c-032d	Abort I/O Vector	[F222]

032e-032f	Machine Lang Monitor Link [B006]

0330-0331	LOAD Link		[F26C]

0332-0333	SAVE Link		[F54E]

;	I/O Vectors

0334-0335	Print Control Code Link [C7B9]

0336-0337	Print High ASCII Code Link [C805]

0338-0339	Print ESC Sequence Link [C9C1]

033a-033b	Keyscan Link		[C5E1]

033c-033d	Store Key		[C6AD]

033e-033f	Pointer to KBD Decoding Table: Unshifted [FA80 / FD29]

0340-0341	Pointer to KBD Decoding Table: Shifted   [FAD9 / FD82]

0342-0343	Pointer to KBD Decoding Table: Commodore [FB32 / FDDB]

0344-0345	Pointer to KBD Decoding Table: Control   [FB8B / FE34]  1)

0346-0347	Pointer to KBD Decoding Table: Alt       [FA80 / FD29]

0348-0349	Pointer to KBD Decoding Table: Ascii/DIN [FB4E / FD29]

1) The value FE34 is used in German version, it doesn't change in others.

034a-0353	IRQ Keyboard Buffer (10 Bytes)  FF = No key

0354-035d	Bitmap Of TAB Stops (10 Bytes)

035e-0361	Bitmap Of Line Wraps

0362-036b	Logical File Number Table

036c-0375	Device Number Table

0376-037f	Secondary Addresse Table

0380-039e	CHRGET Subroutine

0386		CHRGOT Entry

039f-03aa	Fetch From RAM Bank 0

03ab-03b6	Fetch From RAM Bank 1

03b7-03bf	Index1 Indirect Fetch From RAM Bank 1

03c0-03c8	Index2 Indirect Fetch From RAM Bank 0

03c9-03d1	Txtptr Fetch From RAM Bank 0

03d2-03d4	Numeric Constant For BASIC

03d5		Current Bank For SYS, POKE, PEEK

03d6-03d9	INSTR Work Values

03da		Bank Pointer For String/Number CONVERT RTN

03db-03dd	Sprite: Work bytes for SSHAPE

03df		FAC#1 Overflow

03e0-03e1	Sprite: Work bytes for SPRSAV

03e2		Graphic Foreground/ Background Color Nybbles

03e3		Graphic Foreground/ Multicolor 1 Color Nybbles

03f0-03f6	DMA Link Code

ff00		MMU Configuration Register

ff01		MMU LCR: Bank 0

ff02		MMU LCR: Bank 1

ff03		MMU LCR: Bank 14

ff04		MMU LCR: Bank 14 Over RAM 1

Page Four & Higher Declarations (Beginning Of Bankable RAM)

Bank 0

0400-07e7	VIC 40-Column Text Screen

07e8-07ff	Sprite Identity Pointers For Text Mode

0800-09ff	BASIC Pseudo Stack (gosub and loop addresses and commands)

0a00-0a01	Vector to Restart System (BASIC Warm) [4003]

0a02		KERNAL Warm/Cold Init'n Status Byte

0a03		PAL/NTSC System Flag

0a04		Flags RESET vs. NMI Status for init'n rtns

0a05-0a06	Ptr to Bottom of Avail. Memory in System Bank

0a07-0a08	Ptr to Top of Available Memory in System Bank

0a09		Tape Handler preserves IRQ Indirect here

0a0b		TOD Sense during tape operations

0a0c		CIA 1 Interrupt Log

0a0d		CIA 1 Timer Enabled

0a0f-0a17	RS-232 Work Values

0a0f		RS-232 Enables

0a10		RS-232 Control Register

0a11		RS-232 Command Register

0a12		RS-232 User Baud Rate

0a14		RS-232 Status Register

0a15		RS-232 Number of Bits To Send

0a16		RS-232 Baud Rate Full Bit Time (Created by OPEN)

0a18		RS-232 Receive Pointer

0a19		RS-232 Input Pointer

0a1a		RS-232 Transmit Pointer

0a1b		RS-232 Send Pointer

0a1d-0a1f	Sleep Countdown, FFFF = disable

0a20		Keyboard Buffer Size (10)

0a21		Screen Freeze Flag

0a22		Key Repeat: 128 = all, 64 = none

0a23		Key Repeat Timing

0a24		Key Repeat Pause

0a25		Graphics / Text Toggle Latch

0a26		40-Col Cursor Mode

0a27-0a2a	40-Col Blink Values

0a2b		80-Col Cursor Mode

0a2c		40-Col Video $D018 Image

0a2d		VIC Bit-Map Base Pointer

0a2e-0a2f	80-Col Pages-Screen, Color

0a40-0a5a	40/80 Pointer Swap (to E0-FA)

0a60-0a6d	40/80 Data Swap (0354-0361)

0a80    	Compare Buffer (32 bytes)

0aa0-0aa1	MLM

0aab		ASM/DIS

0aac		For Assembler

0aaf		Byte Temp used all over

0ab0		Byte Temp used all over

0ab1		Byte Temp for Assembler

0ab2		Save .X here during Indirect Subroutine Calls

0ab3		Direction Indicator For 'TRANSFER'

0ab4		Parse Number Conversion

0ab5		Parse Number Conversion

0ab7		Parse Number Conversion

0ac0		PAT Counter

0ac1-0ac4	ROM Physical Address Table

0ac5		Flag: KBD

0b00-0bbf	Cassette Buffer

0bc0-0bff	?

0c00-0dff	RS-232 Input, Output Buffers

0e00-0fff	System Sprites (56-63)

1000-1009	Programmed Key Lenghts

100a-10ff	Programmed Key Definitions

1100-1130	DOS Command Staging Area

1131-116e	Graphics Work Area

1131		Current X Position

1133		Current Y Position

1135		X-Coordinate Destination

1137		Y-Coordinate Destination

1139		Line Drawing Variables

1149		Sign Of Angle

114a		Sine Of Value Of Angle

114c		Cosine of Value of Angle

114e		Temps For Angle Distance Routines

1150		CIRCLE Center, X Coordinate, BOX POINT 1 X-Coord.

1152		CIRCLE Center, Y Coordinate, BOX POINT 1 Y-Coord.

1154		X Radius, BOX Rotation Angle

1156		Y Radius

1158		CIRCLE Rotation Angle

115a		BOX: Length of a side

115c		Arc Angle Start

115e		Arc Angle End, Char's Col. Counter

1160		X Radius * COS(Rotation Angle)

1162		Y Radius * SIN(Rotation Angle)

1164		X Radius * SIN(Rotation Angle)

1166		Y Radius * COS(Rotation Angle)

1168		HIGH BYTE: ADDR OF CHARROM For 'CHAR' CMD.

1169		Temp For GSHAPE

116a		SCALE Mode Flag

116b		Double Width Flag

116c		Box Fill Flag

116d		Temp For Bit Mask

116f		Trace Mode: FF = on

1170-1173	Renumbering Pointers

1174-1177	Directory Work Pointers

1178-1197	Graphics Index

117a-117b	Float-fixed Vector [849F]

117c-117d	Fixed-float Vector [793C]

117e-11d5	Sprite Motion Tables (8 x 11 bytes)

11d6-11e5	Sprite X/Y Positions

11e6		Sprite X-High Positions

11e7-11e8	Sprite Bumb Masks (sprite - backgnd)

11e9-11ea	Light Pen Values, X and Y

11eb		CHRGEN ROM Page, Text Mode [D8]

11ec		CHRGEN ROM Page, Graphics Mode [D0]

11ed		Secondary Address For RECORD

11ee-11ff	Unused

1200-1201	Previous BASIC Line

1202-1203	Pointer: BASIC Statement for CONTINUE

1204		PRINT USING Fill Symbol

1205		PRINT USING Comma Symbol

1206		PRINT USING D.P. Symbol

1207		Print Using Monetary Symbol

1208		Used by Error Trapping Routine - Last Err No

1209-120a	Line # of Last Error. FFFF if No Error

120b-120c	TRAP Address, FFFF = none

120d		Hold Trap # of Tempor.

1210-1211	End of Basic, Bank 0

1212-1213	Basic Program Limit [FF00]

1214-1217	DO Work Pointers

1218-121a	USR Program Jump [7D28]

121b-121f	RND Seed Value

1220		Degrees Per CIRCLE Segment

1221		'Cold' or 'Warm' Reset Status

1222		Sound Tempo

1223-1224	Remaining Note Length LO/HI, Voice 1

1225-1226	Remaining Note Length LO/HI, Voice 2

1227-1228	Remaining Note Length LO/HI, Voice 3

1229-122a	Note Length LO/HI

122b		Octave

122c		Flag: 01 = Sharp, FF = Flat

122d-122e	Pitch

122f		Music Sequencer (Voice Number)

1230		Wave

1233		Flag: Play Dotted Note

1234-1237	Note Image

1239-123e	Current Envelope Pattern

123f-1270	Envelope Tables

123f-1248	AD(SR) Pattern

1249-1252	(AD)SR Pattern

1253-125c	Waveform Pattern

125d-1266	Pulse Width Lo Pattern

1267-1270	Pulse Width Hi Pattern

1271-1274	Note: xx, xx, volume

1275		Previous Volume Image

1276-1278	Collision IRQ Task Table

1279-127e	Collision IRQ Address Tables

127f		Collision Mask

1280		Collision Work Value

1281		SOUND Voice

1282		SOUND Time LO

1285-1287	SOUND Time HI

1288-128a	SOUND Max LO

128b		SOUND Max HI

128e		SOUND Min LO

1291		SOUND Min HI

1294		SOUND Direction

1297		SOUND Step LO

129a		SOUND Step HI

129d		SOUND Freq LO

12a0		SOUND Freq HI

12a3		Temp Time LO

12a4		Temp Time HI

12a5		Temp Max LO

12a6		Temp Max HI

12a7		Temp Min LO

12a8		Temp Min HI

12a9		Temp Direction

12aa		Temp Step LO

12ab		Temp Step HI

12ac		Temp Freq LO

12ad		Temp Freq HI

12ae		Temp Pulse LO

12af		Temp Pulse HI

12b0		Temp Waveform

12b1-12b2	PEN/POT Work Values

12b7		Used BY SPRDEF & SAVSPR

12fa		Used BY SPRDEF & SAVSPR

12fb		Used BY SPRDEF & SAVSPR

12fc		Sprite Number. Used BY SPRDEF & SAVSPR

12fd		Used by BASIC IRQ to block all but one IRQ call

12ff		?

1300-17ff	Unused

1800-1bff	Reserved for Key Functions

1c00-fbff	BASIC RAM Memory (Text)

1c00-1ff7	Video Color Matrix For Graphics Mode

1ff8-1fff	Sprite Identity Pointers For Graphics Mode

2000-3fff	Screen Memory For Graphics Mode

4000-fbff	BASIC RAM Memory (with graphics enabled)

Bank 1

0400-fbff	BASIC Variables, Arrays, And Strings

Bank 14

The same as Bank 15, except:

d000-dfff	Character Generator ROM

Bank 15

4000-cfff	BASIC ROM

d000-d030	VIC, 40 Col Video Chip 8564 (NTSC) or 8566 (PAL)

d400-d41c	SID Sound Chip 6581

d500-d50b	MMU, Memory Management Unit 8722

d600-d601	VDC, 80 Col Video Controller 8563

d800-d8e7	Color Nybbles For VIC Text Screen

dc00-dc0f	CIA 1 (IRQ), 6526

dd00-dd0f	CIA 2 (NMI), 6526

df00-df0a	DMA Slot

e000-ffff	KERNAL ROM

ff00-ff04	MMU

ff05-ffff	Transfer, Jump Table

*****

502 COMMODORE 128

This chapter provides the memory maps for both C128 and C64 modes. A memory

map tells you exactly how memory is laid out internally in both RAM and ROM.

It tells you exactly what resides in each memory location. The memory map

directs you in finding address vectors for routines and entry points and provi-

des information about the general layout of the computer. The memory map is

probably the most vital programming tool.

Refer to the memory map whenever you need directions throughout the memory of

your Commodore 128. Addresses listed with more than one address label are used

for more than one purpose. To BASIC, the variable has one purpose; to the Mac-

hine Language Monitor, it may have another.

The conventions used for the memory maps are as follows:

Column 1	Column 2	Column3		Column 4

MEMORY

ADDRESS		HEXADECIMAL	DECIMAL

LABEL		ADDRESS		ADDRESS		DESCRIPTION

See Appendix K for the Z80 memory map for CP/M on the Commodore 128.

C128 MEMORY MAP

MEMORY ADDRESS	  ADDRESS

LABEL		HEX	DEC	DESCRIPTION

D6510		0000	0	6510 Data Direction Register

R6S10		0001	1	6510 Data Register

BANK		0002	2	Token 'SEARCH' looks for, or Bank #

PC_HI		0003	3	Address for BASIC SYS command or MONITOR and

LONG CALL/JUMP Routines

PC_LO		0004	4	Address

S_REG		0005	5	STATUS REG Temp

A_REG		0006	6	.A REG Temp

X_REG		0007	7	.X REG Temp

Y_REG		0008	8	.Y REG Temp

STKPTR		0009	9	Stack Pointer Temp

BASIC ZERO PAGE STORAGE

INTEGR		0009	9

CHARAC				Search Character

ENDCHR		000A	10	Flag: Scan For Quote at End Of String

TRMPOS		000B	11	Screen Column from Last Tab

VERCIC		000C	12	Flag: 0 = LOAD, 1 = VERIFY

COUNT		000D	13	Input Buf.ptr / # of Subscripts

DIMFLG		000E	14	Flag: Default Array Dimension

VALTYP		000F	15	DATA Type: $FF=String, $00 = Numeric

INTFLG		0010	16	DATA Type: $00=Float.Pt, $80 = Integer

GARBFL		0011	17	Flag: DATA Scan/ LIST Quote/ Garbage Collection

DORES

SUBFLG		0012	18	Flag: Subscript Ref. / User Func. Call

INPFLG		0013	19	Flag: $00=INPUT, $40=GET, $98 = READ

DOMASK		0014	20

TANSGN				Flag: TAN Sign / Comparison Result

CHANNL		0015	21

POKER		0016	22

LINNUM				Temp Integer Value

TEMPPT		0018	24	Pointer: Temp String Stack

LASTPT		0019	25	Last Temp String Address

TEMPST		001B	27	Stack For Temp Strings

INDEX		0024	36	Utility Pointer Area

INDEX1

INDEX2		0026	38

RESHO		0028	40	Floating Pt. Product of Multiply

RESMOH		0029	41

ADDEND		002A	42

RESMO

RESLO		002B	43

TXTTAB		002D	45	Pointer: Start Of BASIC Text

VARTAB		002F	47	Pointer: Start Of BASIC Variables

ARYTAB		0031	49	Pointer: Start Of BASIC Arrays

STREND		0033	51	Pointer: End Of BASIC Arrays + 1

FRETOP		0035	53	Pointer: Bottom of String Storage

FRESPC		0037	55	Utility String Pointer

MAX_MEM_1	0039	57	Top of String/Variable Bank (Bank 1)

CURLIN		003B	59	Current BASIC Line Number

TXTPTR		003D	61	Pointer to BASIC Text used by CHRGET,etc.

FORM		003F	63	Used by PRINT USING

FNDPNT				Pointer to Item Found By Search

DATLIN		0041	65	Current DATA Line Number

DATPTR		0043	67	Current DATA Item Address

INPPTR		0045	69	Vector: INPUT Routine

VARNAM		0047	71	Current BASIC Variable Name

FDECPT		0049	73

VARPNT				Pointer: Current BASIC Variable DATA

LSTPNT		004B	75

FORPNT				Pointer: Index Variable For FOR/NEXT

ANDMSK

EORMSK		004C	76

VARTXT		004D	77

OPPTR

OPMASK		004F	79

GRBPNT		0050	80

TEMPF3

DEFPNT

DSCPNT		0052	82

0054	84

HELPER		0055	85	Flags 'HELP' or 'LIST'

JMPER		0056	86

0057	87

OLDOV		0058	88

TEMPF1		0059	89

PTARG1				Multiply defined for INSTR

PTARG2		005B	91

STRl		005D	93

STR2		0060	96

POSITN		0063	99

MATCH		0064	100

ARYPNT		005A	90

HIGHDS

HIGHTR		005C	92

TEMPF2		005E	94

DECCNT		005F	95	Number of Digits after the Decimal Point

TENEXP		0060	96

T0				ML MONITOR Z.P. Storage in FAC

GRBTOP		0061	97

DPTFLG				Decimal Point Flag

LOWTR

EXPSGN		0062	98

FAC		0063	99

DSCTMP

LEFT_FLAG			Paint-Left Flag

FACEXP				FAC#1 Exponent

Tl				MONITOR Z.P. Storage in FAC

RIGHT_FLAG	0064	100	Paint-Right Flag

FACHO				FAC#1 Mantissa

FACMOH		0065	101

INDICE		0066	102

FACMO

T2				MONITOR Z.P. Storage in FAC

FACLO		0067	103

FACSGN		0068	104	FAC#1 Sign

DEGREE		0069	105

SGNFLG				Pointer: Series-Eval. Constant

ARGEXP		006A	106	FAC#2 Exponent

ARGHO		006B	107	FAC#2 Mantissa

ARGMOH		006C	108

INIT_AS_0			Just a count for INIT

ARGMO		006D	109

ARGLO		006E	110

ARGSGN		006F	111	FAC#2 Sign

STRNGI		0070	112

ARISGN				Sign Comparison Result: FAC#1 vs #2

FACOV		0071	113	FAC#1 Low-Order (Rounding)

STRNG2		0072	114

POLYPT

CURTOL

FBUFPT				Pointer: Cassette Buffer

AUTINC		0074	116	Inc. Val for AUTO (0 = Off)

MVDFLG		0076	118	Flag If 10K HIRES Allocated

Z_P_TEMP_1	0077	119	PRINT USING's Leading Zero Counter

MOVSPR & Sprite Temporary

MID$ Temporary

HULP		0078	120	Counter

KEYSIZ

SYNTMP		0079	121	Used as Temp For Indirect

DSDESC		007A	122	Descriptor for DS$

TXTPTR				MONITOR Z.P. Storage

TOS		007D	125	Top of Run Time Stack

RUNMOD		007F	127	Flags Run/Direct Mode

PARSTS		0080	128	DOS Parser Status Word

POINT				USING's Pointer to Dec.Pt

PARSTX		0081	129

OLDSTK		0082	130

BASIC Z-P STORAGE FOR GRAPHIC COMMANDS

COLSEL		0083	131	Current COLOR Selected

MULTICOLOR	0084	132

MULTICOLOR_2	0085	133

FOREGROUND	0086	134

SCALE_X		0087	135	Scale Factor In X

SCALE_Y		0089	137	Scale Factor In Y

STOPNB		008B	139	Stoppaint if not Background/Not same color

GRAPNT		008C	140

VTEMPI		008E	142

VTEMP2		008F	143

KERNAL/EDITOR STORAGE

STATUS		0090	144	I/O Operation Status Byte

STKEY		0091	145	STOP KEY Flag

SVXT		0092	146	Tape Temporary

VERCK		0093	147	LOAD or VERIFY Flag

C3P0		0094	148	Serial Buffered Char Flag

BSOUR		0095	149	Char Buffer For Serial

SYNO		0096	150	Cassette Sync #

XSAV		0097	151	Temp For BASIN

LDTND		0098	152	Index to Logical File

DFLTN		0099	153	Default Input Device #

DFLTO		009A	154	Default Output Device #

PRTY		009B	155	Cassette Parity

DPSW		009C	156	Cassette Dipole Switch

MSGFLG		009D	157	OS Message Flag

PTR1		009E	158	Cassette Error Pass1

T1				Temporary 1

PTR2		009F	159	Cassette Error Pass2

T2				Temporary 2

TIME		00A0	160	24 Hour Clock In 1/60th Seconds

R2D2		00A3	163	Serial Bus Usage

PCNTR				Cassette

BSOUR1		00A4	164	Temp used by Serial Routine

FIRT

COUNT		00A5	165	Temp used by Serial Routine

CNTDN				Cassette Sync Countdown

BUFPT		00A6	166	Cassette Buffer Pointer

INBIT		00A7	167	RS-232 RCVR Input Bit Storage

SHCNL				Cassette Short Count

BITCI		00A8	168	RS-232 RCVR Bit Count In

RER				Cassette Read Error

RINONE		00A9	169	RS-232 RCVR Flag For Start Bit Check

REZ				Cassette Reading Zeroes

RIDATA		00AA	170	RS-232 RCVR Byte Buffer

RDFLG				Cassette Read Mode

RIPRTY		00AB	171	RS-232 RCVR Parity Storage

SHCNH				Cassette Short Cnt

SAL		00AC	172	Pointer: Tape Buffer / Screen Scrolling

SAH		00AD	173

EAL		00AE	174	Tape End Addresses / End of Program

EAH		00AF	175

CMP0		00B0	176	Tape Timing Constants

TEMP		00B1	177

TAPE1		00B2	178	Address of Tape Buffer

BITTS		00B4	180	RS-232 TRNS Bit Count

SNSW1

NXTBIT		00B5	181	RS-232 TRNS Next Bit To Be Sent

DIFF

RODATA		00B6	182	RS-232 TRNS Byte Buffer

PRP

FNLEN		00B7	183	LENGTH Current File N Str

LA		00B8	184	Current File Logical Addr

SA		00B9	185	Current File 2nd Addr

FA		00BA	186	Current File Primary Addr

FNADR		00BB	187	Addr Current File Name Str

ROPRTY		00BD	189	RS-232 TRNS Parity Buffer

OCHAR

FSBLK		00BE	190	Cassette Read Block Count

DRIVE		00BF	191

MYCH				Serial Word Buffer

CAS1		00C0	192	Cassette Manual/Cntrled Switch (Updated during IRQ)

TRACK		00C1	193

STAL				I/O Start Address (LO)

SECTOR		00C2	194

STAH				I/O Start Address (HI)

MEMUSS		00C3	195	Cassette LOAD Temps (2 bytes)

DATA		00C5	197	Tape Read/Write Data

BA		00C6	198	Bank For Current LOAD/SAVE/VERIFY Operation

FNBANK		00C7	199	Bank Where Current FN is found (at 'FNADR')

RIBUF		00C8	200	RS-232 Input Buffer Pointer

ROBUF		00CA	202	RS-232 Output Buffer Pointer

GLOBAL SCREEN EDITOR VARIABLES

KEYTAB		00CC	204	Keyscan Table Pointer

IMPARM		00CE	206	PRIMM Utility String Pointer

NDX		00D0	208	Index to Keyboard Queue

KYNDX		00D1	209	Pending Function Key Flag

KEYIDX		00D2	210	Index Into Pending Function Key String

SHFLAG		00D3	211	Keyscan Shift Key Status

SFDX		00D4	212	Keyscan Current Key Index

LSTX		00D5	213	Keyscan Last Key Index

CRSW		00D6	214	<CR> Input Flag

MODE		00D7	215	40/80 Column Mode Flag

GRAPHM		00D8	216	Text/Graphic Mode Flag

CHAREN		00D9	217	RAM/ROM VIC Character Fetch Flag (Bit-2)

THE FOLLOWING LOCATIONS ARE SHARED BY SEVERAL EDITOR ROUTINES.

SEDSAL		00DA	218	Pointers For MOVLIN

BITMSK		00DA	218	Temporary For TAB & LINE WRAP Routines

SAVER		00DB	219	Another Temporary Place To Save A Reg.

SEDEAL		00DC	220

SEDT1		00DE	222	SAVPOS

SEDT2		00DF	223

KEYSIZ		00DA	218	Programmable Key Variables

KEYLEN		00DB	219

KEYNUM		00DC	220

KEYNXT		00DD	221

KEYBNK		00DE	222

KEYTMP		00DF	223

LOCAL SCREEN EDITOR VARIABLES.

These are swapped out to $0a40 when screen (40/80) mode changes.

PNT		00E0	224	Pointer to Current Line (Text)

USER		00E2	226	Pointer to Current Line (Attribute)

SCBOT		00E4	228	Window Lower Limit

SCTOP		00E5	229	Window Upper Limit

SCLF		00E6	230	Window Left Margin

SCRT		00E7	231	Window Right Margin

LSXP		00E8	232	Current Input Column Start

LSTP		00E9	233	Current Input Line Start

INDX		00EA	234	Current Input Line End

TBLX		00EB	235	Current Cursor Line

PNTR		00EC	236	Current Cursor Column

LINES		00ED	237	Maximum Number of Screen Lines

COLUMNS		00EE	238	Maximum Number of Screen Columns

DATAX		00EF	239	Current Character to Print

LSTCHR		00F0	240	Previous Char Printed (For <ESC> Test)

COLOR		00F1	241	Curr Attribute to Print (Default FGND Color)

TCOLOR		00F2	242	Saved Attrib to Print ('INSERT' & 'DELETE')

RVS		00F3	243	Reverse Mode Flag

QTSW		00F4	244	Quote Mode Flag

INSRT		00F5	245	Insert Mode Flag

INSFLG		00F6	246	Auto-Insert Mode Flag

LOCKS		00F7	247	Disables <Shift><C*>, <CTRL> S

SCROLL		00F8	248	Disables Screen Scroll, Line Linker

BEEPER		00F9	249	Disables <CTRL> G

FREKZP		00FA	250	Free Zero Page reserved for applications

software ($FA-$FE)

LOFBUF		00FF	255

BASIC/DOS INTERFACE VARS

BAD		0100	256	Tape Read Errors

FBUFFR				Area to build filename in (16 bytes)

XCNT		0110	272	DOS Loop Counter

DOSF1L		0111	273	DOS Filename 1 Len

DOSDS1		0112	274	DOS Disk Drive 1

DOSF2L		0113	275	DOS Filename 2 Len

DOSDS2		0114	276	DOS Disk Drive 2

DOSF2A		0115	277	DOS Filename 2 Addr

DOSOFL		0117	279	BLOAD/BSAVE Starting Address

DOSOFH		0119	281	....	and Ending Address

DOSLA		011B	283	DOS Logical Addr

DOSFA		011C	284	DOS Phys Addr

DOSSA		011D	285	DOS Sec. Addr

DOSRCL		011E	286	DOS Record Length

DOSBNK		011F	287

DOSDID		0120	288	DOS Disk ID

DIDCHK		0122	290	DOS DSK ID FLG SPACE Used by PRINT USING

BNR		0123	291	Pointer to Begin. NO.

ENR		0124	292	Pointer to End NO.

DOLR		0125	293	Dollar Flag

FLAG		0126	294	Comma Flag

SWE		0127	295	Counter

USGN		0128	296	Sign Exponent

UEXP		0129	297	Pointer to Exponent

VN		012A	298	# Of Digits Before Decimal Point

CHSN		012B	299	Justify Flag

VF		012C	300	# Of Pos Before Decimal Point (Field)

NF		012D	301	# Of Pos After Decimal Point (Field)

POSP		012E	302	+/- Flag (Field)

FESP		012F	303	Exponent Flag (Field)

ETOF		0130	304	Switch

CFORM		0131	305	Char Counter (Field)

SNO		0132	306	Sign No

BLFD		0133	307	Blank/Star Flag

BEGFD		0134	308	Pointer to Begin of Field

LFOR		0135	309	Length of Format

ENDFD		0136	310	Pointer to End Of Field

SYSTK		0137	311	System Stack ($0137-$01FF)

BUF		0200	512	Input Buffer: BASIC & MONITOR

FETCH		02A2	674	LDA(-),Y From Any Bank

FETCHVEC	02AA	682

STASH		02AF	687	STA(-),Y to Any Bank

STAVEC		02B9	697

CMPARE		02C8	702	CMP(-),Y to Any Bank

CMPVEC		02C8	712

JSRFAR		02CD	716	JSR XXXX to Any Bank & Return

JMPFAR		02E3	739	JMP XXXX to Any Bank

VECTORS

ESC_FN_VEC	02FC	764	Vector For Additional Function Routines

K EC		02FE	766	Vector For Function Cart.

IERROR		0300		Users

0302		(ERR IN .X)

ICRNCH		0304	772	Vector to Crunch

774	Vector to List BASIC Text

776	Vector to Gone (BASIC Char

778	Vector to BASIC Token

780	Vector to Escape-Token

030E	782	LIST

IESCEX		0310	784	... And Execute.

IIRO		0314	788	IRQ RAM Vector

IBRK		0316	790	BRK INSTR RAM Vector

INMI		0318	792	NMI Vector

IOPEN		031A	794	KERNAL OPEN Routine Vector

796	KERNAL CLOSE Routine Vector

798	KERNAL CHKIN Routine Vector

800	KERNAL CHKOUT Routine

CLRCH		0322	Vector

VECTOR

IBASIN		0324	804	KERNAL CHRIN Routine Vector

IBSOUT		0326	806	KERNAL CHROUT Routine Vector

ISTOP		0328	808	KERNAL STOP Routine Vector

IGETIN		032A	810	KERNAL GETIN Routine Vector

ICLALL		032C	812	KERNAL CLALL Routine Vector

EXMON		032E	814	Monitor Command Vector

ILOAD		0330	816	KERNAL LOAD Routine Vector

ISAVE		0332	818	KERNAL SAVE Routine Vector

EDITOR INDIRECT VECTORS

CTLVEC		0334	820	Editor: PRINT 'CONTRL' Indirect

SHFVEC		0336	822	Editor: PRINT 'SHIFTD' Indirect

ESCVEC		0338	824	Editor: PRINT 'ESCAPE' Indirect

KEYVEC		033A	826	Editor: Keyscan Logic Indirect

KEYCHK		033C	828	Editor: Store Key Indirect

DECODE		033E	830	Vectors to Keyboard Matrix Decode Tables

KEYD		034A	842	IRQ Keyboard Buffer (10 Bytes)

TABMAP		0354	852	Bitmap Of TAB Stops (10 Bytes, $0354-D)

BITABL		035E	862	Bitmap Of Line Wraps

TABMAP and BITABL get swapped to $0A60 when

screen 40/80 mode is changed.

LAT		0362	866	Logical File Numbers

FAT		036C	876	Primary Device Numbers

SAT		0376	886	Secondary Addresses

CHRGET		0380	896

CHRGOT		0386	902

QNUM		0390	912

INDIRECT LOAD SUBROUTINE AREA

INDSUB_RAM0	039F	927	Shared ROM Fetch Sub

INDSUB_RAM1	03AB	939	Shared ROM Fetch Sub

INDIN1_RAM1	03B7	950	Index1 Indirect Fetch

INDIN2		03C0	959	Index2 Indirect Fetch

INDIXT		03C9	968	Txtptr

ZERO		03D2	977	Numeric Constant For BASIC

CURRENT_BANK	03D5	979	Context For SYS,POKE,PEEK From Bank Cmmd

TMPDES		03D6	980	Temp For INSTR

FIN_BANK	03DA	984	Bank Pointer For String/Number CONVERT RTN

SAVSIZ		03DB	985	Temp Work Locations for SSHAPE

BITS		03DF	989	FAC#1 OVERFLOW DIGIT

SPRTMP_1	03E0	990	Temp For SPRSAV

SPRTMP_2	03E1	991

FG_BG		03E2	992	Packed Foreground/ Background Color Nybbles

FG_MC1		03E3	993	Packed Foreground/ Multicolor 1 Color Nybbles

Page Four & Higher Declarations (Beginning Of Bankable RAM)

VICSCN		0400	1024	VIDEO MATRIX #1: VIC 40-Column Text Screen

$0400-$07FF

0800	2048	BASIC RUN-TIME STACK (512 BYTES)

$0800-$09FF

ABSOLUTE KERNAL VARIABLES

SYSTEM_VECTOR	0A00	2560	Vector to Restart System (BASIC Warm)

DEJAVU		0A02	2562	KERNAL Warm/Cold Init'n Status Byte

PALNTS		0A03	2563	PAL/NTSC System Flag

INIT_STATUS	0A04	2564	Flags RESET vs. NMI Status for init'n rtns

MEMSTR		0A05	2565	Ptr to Bottom of Avail. Memory in System Bank

MEMSIZ		0A07	2567	Ptr to Top of Available Memory in System Bank

IRQTMP		0A09	2569	Tape Handler preserves IRQ Indirect here

CASTON		0AOB	2571	TOD Sense during tape operations

KIKA26		0AOC	2572	Tape Read Temporary

STUPID		0AOD	2573	Tape Read D1IRQ Indicator

TIMOUT		0AOE	2574	Fast Serial Timeout Flag

ENABL		0A0F	2575	RS-232 Enables

M51CTR		0A10	2576	RS-232 Control Register

MSICDR		0A11	2577	RS-232 Command Register

MSIAJB		0A12	2578	RS-232 User Baud Rate

RSSTAT		0A14	2580	RS-232 Status Register

BITNUM		0A15	2581	RS-232 Number of Bits To Send

BAUDOF		0A16	2582	RS-232 Baud Rate Full Bit Time (Created by OPEN)

RIDBE		0A18	2584	RS-232 Input Buffer Index to End

RIDBS		0A19	2585	RS-232 Input Buffer Index to Start

RODBS		0A1A	2586	RS-232 Output Buffer Index to Start

RODBE		0A1B	2587	RS-232 Output Buffer Index to End

SERIAL		0A1C	2588	Fast Serial Internal/External Flag

TIMER		0A1D	2589	Decrementing Jiffie Register

GLOBAL ABSOLUTE SCREEN EDITOR DECLARATIONS

XMAX		0A20	2592	Keyboard Queue Maximum Size

PAUSE		0A21	2593	CTRL-S Flag

RPTFLG		0A22	2594	Enable Key Repeats

KOUNT		0A23	2595	Delay Between Key Repeats

DELAY		0A24	2596	Delay Before a Key Starts Repeating

LSTSHF		0A25	2597	Delay Between <C=><SHFT> Toggles

BLNON		0A26	2598	VIC Cursor Mode (Blinking, Solid)

BLNSW		0A27	2599	VIC Cursor Disable

BLNCT		0A28	2600	VIC Cursor Blink Counter

GDBLN		0A29	2601	VIC Cursor Character Before Blink

GDCOL		0A2A	2602	VIC Cursor Color Before Blink

CURMOD		0A2B	2603	VDC Cursor Mode (when enabled)

VM1		0A2C	2604	VIC Text Screen/Character Base Pointer

VM2		0A2D	2605	VIC Bit-Map Base Pointer

VM3		0A2E	2606	VDC Text Screen Base

VM4		0A2F	2607	VDC Attribute Base

LINTMP		0A30	2608	Temporary Pointer to Last Line For LOOP4

SAV80A		0A31	2609	Temporary For 80-Col Routines

SAV80B		0A32	2610	Temporary For 80-Col Routines

CURCOL		0A33	2611	VDC Cursor Color Before Blink

SPLIT		0A34	2612	VIC Split Screen Raster Value

FNADRX		0A35	2613	Save .X During Bank Operations

PALCNT		0A36	2614	Counter for PAL Systems (Jiffie adjustment)

SPEED		0A37	2615	Save System Speed During Tape and Serial Ops

SPRITES		0A38	2616	Save Sprite Enables During Tape and Serial Ops

BLANKING	0A39	2617	Save Blanking Status During Tape Ops

HOLD_OFF	0A3A	2618	Flag set by user to resrv full control of VIC

LDTBI_SA	0A3B	2619	Hi byte:SA Of VIC Scrn (Use W/VMI to move scrn)

CLR_EA_LO	0A3C	2620	8563 Block Fill

CLR_EA_HI	0A3D	2621	8563 Block Fill

0A40	2624	$0A40-$0A7F Reserved Swap Area For Screen

Variables When (40/80) Mode Changes

MONITOR's domain

XCNT		0A80	2688	Compare Buffer (32 bytes)

HULP		0AA0	2720

FORMAT		0AAA	2730

LENGTH		0AAB	2731	ASM/DIS

MSAL		0AAC	2732	For Assembler

SXREG		0AAF	2735	1 Byte Temp used all over

SYREG		0AB0	2736	1 Byte Temp used all over

WRAP		0AB1	2737	1 Byte Temp for Assembler

XSAVE		0AB2	2738	Save .X here during Indirect Subroutine Calls

DIRECTION	0AB3	2739	Direction Indicator For 'TRANSFER'

COUNT		0AB4	2740	Parse Number Conversion

NUMBER		0AB5	2741	Parse Number Conversion

TEMPS		0AB7	2743	Parse Number Conversion

FUNCTION KEY ROM CARD TABLES

CURBNK		0AC0	2752	Current Function Key ROM Bank Being Polled

PAT		0AC1	2753	Physical Address Table (IDS OF LOGGED-IN CARDS)

DK_FLAG		0AC5	2757	Reserved For Foreign Screen Editors

0AC6	2758	$0AC6-$0AFF Reserved For System

TBUFFR		0B00	2810	Cassette Buffer (192 BYTES)

$0B00-$0BC0, This page also used as a buffer

for the disk auto-boot.

RS232I		0C00	3072	RS-232 Input Buffer

RS2320		0D00	3328	RS-232 Output Buffer

0E00	3584	Sprite Definition Area (Must be below $1000)

$0E00-$0FFF, 512 Bytes

PKYBUF		1000	4096	Programmable Function Key Lengths Table For

10 Keys (F1-F8, <SHIFT RUN>, HELP)

PKYDEF		100A	4106	Programmable Function Key Strings

DOS/VSP AREA

DOSSTR		1100	4352	DOS OUTPUT STR. BUF

48 Bytes to Build DOS String

VWORK		1131	4401	Graphics Vars

XYPOS		1131	4401

XPOS		1131	4401	Current X Position

YPOS		1133	4403	Current Y Position

XDEST		1135	4405	X-Coordinate Destination

YDEST		1137	4407	Y-Coordinate Destination

XYABS		1139	4409	Line Drawing Variables

XABS		1139	4409

YABS		113B	4411

XYSGN		113D	4413

XSGN		113D	4413

YSGN		113F	4415

FCT		1141	4417

ERRVAL		1145	4421

LESSER		1147	4423

GREATR		1148	4424

ANGLE ROUTINE VARIABLES

ANGSGN		1149	4425	Sign Of Angle

SINVAL		114A	4426	Sine Of Value Of Angle

DOS/VSP AREA

COSVAL		114C	4428	Cosine of Value of Angle

ANGCNT		114E	4430	Temps For Angle Distance Routines

BASIC GRAPHIC VARIABLES.

The following 24 bytes are multiply defined.

CIRCLE DRAWING VARIABLES

XCIRCL		1150	4432	CIRCLE Center, X Coordinate

YCIRCL		1152	4434	CIRCLE Center, Y Coordinate

XRADUS		1154	4436	X Radius

YRADUS		1156	4438	Y Radius

ROTANG		1158	4440	Rotation Angle

ANGBEG		115C	4444	Arc Angle Start

ANGEND		115E	4446	Arc Angle End

XRCOS		1160	4448	X Radius * COS(Rotation Angle)

YRSIN		1162	4450	Y Radius * SIN(Rotation Angle)

XRSIN		1164	4452	X Radius * SIN(Rotation Angle)

YRCOS		1166	4454	Y Radius * COS(Rotation Angle)

BASIC GENERAL USE PARAMETERS

XCENTR		1150	4432

YCENTR		1152	4434

XDIST1		1154	4436

YDIST1		1156	4438

XDIST2		1158	4440

YDIST2		115A	4442

DISEEND		115C	4444	Placeholder

COLCNT		115E	4446	Char's Col. Counter

ROWCNT		115F	4447

STRCNT		1160	4448

BOX-DRAWING VARIABLES

XCORD1		1150	4432	POINT 1 X-Coord.

YCORD1		1152	4434	POINT 1 Y-Coord.

BOXANG		1154	4436	Rotation Angle

XCOUNT		1156	4438

YCOUNT		1158	4440

BXLENG		115A	4442	Length of a side

XCORD2		115C	4444

YCORD2		115E	4446

SHAPE AND MOVE-SHAPE VARIABLES

KEYLEN		1151	4433

KEYNXT		1152	4434

STRSZ		1153	4435	String Len

GETTYP		1154	4436	Replace Shape Mode

STRPTR		1155	4437	String Pos'n Counter

OLDBYT		1156	4438	Old Bit Map byte

NEWBYT		1157	4439	New String or Bit Map Byte

1158	4440	Placeholder

XSIZE		1159	4441	SHAPE Column Length

YSIZE		115B	4443	SHAPE Row Length

XSAVE		115D	4445	Temp For Column Length

STRADR		115F	4447	Save SHAPE String Descriptor

BITIDX		1161	4449	Bit Index Into Byte

BASIC GRAPHIC VARIABLES

CHRPAG		1168	4456	HIGH BYTE: ADDR OF CHARROM For 'CHAR' CMD.

BITCNT		1169	4457	Temp For GSHAPE

SCALEM		116A	4458	SCALE Mode Flag

WIDTH		116B	4459	Double Width Flag

FILFLG		116C	4460	Box Fill Flag

BITMSK		116D	4461	Temp For Bit Mask

NUMCNT		116E	4462

TRCFLG		116F	4463	Flags Trace Mode

RENUM_TMP_1	1170	4464	A Temp For Renumber

RENUM_TMP_2	1172	4466	A Temp For Renumber

T3		1174	4468

T4		1175	4469

VTEMP3		1177	4471	Graphic Temp Storage

VTEMP4		1178	4472

VTEMP5		1179	4473

ADRAY1		117A	4474	Ptr to routine: Convert Float -> Integer

ADRAY2		117C	4476	Ptr to routine: Convert Integer -> FLOAT

SPRITE_DATA	117E	4478	Sprite Speed/Direction Tables ($117E-D5)

VIC_SAVE	11D6	4566	Copy of VIC Reg's, Used to update chip during

retrace (21 bytes, $11D6-EA)

UPPER_LOWER	11EB	4587	Pointer to Upper/Lower Char Set for CHAR

UPPER_GRAPHIC	11EC	4588	Ptr. to Upper/Graphic Char Set

DOSSA		11ED	4589	Temp storage for file SA during RECORD cmd

BASIC GENERAL NON-ZP STORAGE

OLDTXT		1200	4608	Previous BASIC Line

1202	4610	Pointer: BASIC Statement for CONTINUE

PRINT USING DECLARATIONS

PUCHRS		1204	4612

PUFILL		1204	4612	PRINT USING Fill Symbol

PUCOMA		1205	4613	PRINT USING Comma Symbol

PUDOT		1206	4614	PRINT USING D.P. Symbol

PUMONY		1207	4614	Print Using Monetary Symbol

ERRNUM		1208	4616	Used by Error Trapping Routine - Last Err No

ERRLIN		1209	4617	Line # of Last Error. $FFFF if No Error

TRAPNO		120B	4619	Line to go to on error - $FFFF if none set

TMPTRP		120D	4621	Hold Trap # of Tempor.

ERRTXT		120E	4622

TEXT_TOP	1210	4624	Top of Text Pointer

MAX_MEM_0	1212	4626	Highest Address Available to BASIC in RAM 0

TMPTXT		1214	4628	Used by DO-LOOP. Could be Mult. Assigned

TMPLIN		1216	4630

USRPOK		1218	4632

RNDX		121B	4635

CIRCLE_SEGMENT	1220	4640	Degrees Per CIRCLE 51:6MENT

DEJAVU		1221	4641	'Cold' or 'Warm' Reset Status

BASIC STORAGE FOR MUSIC VECTORS

TEMPO_RATE	1222	4642

VOICES		1223	4643

NTIME		1229	4649

OCTAVE		122B	4651

SHARP		122C	4652

PITCH		122D	4653

VOICE		122F	4655

WAVE0		1230	4656

DNOTE		1233	4659

FLTSAV		1234	4660

FLTFLG		1238	4664

NIBBLE		1239	4665

TONNUM		123A	4666

TONVAL		123B	4667

PARCNT		123E	4668

ATKTAB		123F	4669

SUSTAB		1249	4681

WAVTAB		1253	4691

PULSLW		125D	4701

PULSHI		1267	4711

FILTERS		1271	4721

INTERRUPT VECTORS

INT_TRIP_FLAG	1276	4726

INT_ADR_LO	1279	4729

INT_ADR_HI	127C	4732

INTVAL		127F	4735

COLTYP		1280	4736

BASIC SOUND COMMAND VARS

SOUND_VOICE	1281	4737

SOUND_TIME_LO	1282	4738

SOUND_TIME_HI	1285	4741

SOUND_MAX_LO	1288	4744

SOUND_MAX_HI	128B	4747

SOUND_MIN_LO	128E	4750

SOUND_MIN_HI	1291	4753

SOUND_DIRECTION	1294	4756

SOUND_STEP_LO	1297	4759

SOUND_STEP_HI	129A	4762

SOUND_FREQ_LO	129D	4765

SOUND_FREQ_HI	12A0	4768

TEMP_TIME_LO	12A3	4771

TEMP_TIME_HI	12A4	4772

TEMP_MAX_LO	12A5	4773

TEMP_MAX_HI	12A6	4774

TEMP_MIN_LO	12A7	4775

TEMP_MIN_HI	12A8	4776

TEMP_DIRECTION	12A9	4777

TEMP_STEP_LO	12AA	4778

TEMP_STEP_HI	12AB	4779

TEMP_FREQ_LO	12AC	4780

TEMP_FREQ_HI	12AD	4781

TEMP_PULSE_LO	12AE	4782

TEMP_PULSE_HI	12AF	4783

TEMP_WAVEFORM	12B0	4784

POT_TEMP_1	12B1	4785	Temporaries for 'POT' Function

POT_TEMP_2	12B2	4786

WINDOW_TEMP	12B3	4787

SAVRAM		12B7	4791	Used BY SPRDEF & SAVSPR

DEFMOD		12FA	4858	Used BY SPRDEF & SAVSPR

LINCNT		12FB	4859	Used BY SPRDEF & SAVSPR

SPRITE_NUMBER	12FC	4860	Used BY SPRDEF & SAVSPR

IRQ_WRAP_FLAG	12FD	4861	Used BY BASIC IRQ to BLOCK

All BUT ONE IRQ CALL

1300	4864	Application Program Area $1300-$1BFF

RAMBOT		1C00	7168	Start Of Basic Text $1C00-$EFFF

(KERNAL sets MEMBOT here)

1C00	7168	Video Matrix #2 (1KB of colors for

Bitmap, if allocated) $1C00-$1FFF

2000	8192	VIC BITMAP (8KB, if allocated)

$2000-$3FFF

BEGINNING OF ROM OVER RAM

4000	16384	C128 BASIC LO ROM

Start of BASIC Text if Bit Map

is allocated (RAM) $4000-$EFFF

8000	32768	C128 BASIC Hl ROM (or Function ROM) $8000-$BFFF

BASIC JUMP TABLE

BASIC ENTRY

JMP HARD_RESET	4000	16384	COLD ENTRY

JMP SOFT_RESET	4003	16387	WARM ENTRY

JMP BASIC_IRQ	4006	16390	IRQ ENTRY

FORMAT CONVERSIONS

JMP AYINT	AF00	44800	Convert F.P. to Integer

JMP GIVAYF	AF03	44803	Convert Integer to F.P.

JMP FOUT	AF06	44806	Convert F.P. to ASCII String

JMP VAL_l	AF09	44809	Convert ASCII String to F.P.

JMP GETADR	AF0C	44812	Convert F.P. to an Address

JMP FLOATC	AF0F	44815	Convert Address to F.P.

MATH FUNCTIONS

JMP FSUB	AF12	44818	MEM - FACC

JMP FSUBT	AF15	44821	ARG - FACC

JMP FADD	AF18	44824	MEM + FACC

JMP FADDT	AF1B	44827	ARG - FACC

JMP FMULT	AF1E	44830	MEM * FACC

JMP FMULTT	AF21	44833	ARG * FACC

JMP FDIV	AF24	44836	MEM / FACC

JMP FDIVT	AF27	44839	ARG / FACC

JMP LOG		AF2A	44842	Compute Natural LOG Of FACC

JMP INT		AF2D	44845	Perform BASIC INT On FACC

JMP SQR		AF30	44848	Compute Square Root OF FACC

JMP NEGOP	AF33	44851	Negate FACC

JMP FPWR	AF36	44854	Raise ARG to The Mem Power

JMP FPWRT	AF39	44857	Raise ARG to The FACC Power

JMP EXP		AF3C	44860	Compute EXP Of FACC

JMP COS		AF3F	44863	Compute COS Of FACC

JMP SIN		AF42	44866	Compute SIN Of FACC

JMP TAN		AF45	44869	Compute TAN Of FACC

JMP ATN		AF48	44872	Compute ATN Of FACC

JMP ROUND	AF4B	44875	Round FACC

JMP ABS		AF4E	44878	Absolute Value Of FACC

JMP SIGN	AF51	44881	Test Sign Of FACC

JMP FCOMP	AF54	44884	Compare FACC With Memory

JMP RND 0	AF57	44887	Generate Random F.P. Number

MOVEMENT

JMP CONUPK	AF5A	44890	Move RAM MEM to ARG

JMP ROMUPK	AF5D	44893	Move ROM MEM to ARG

JMP MOVFRM	AF60	44896	Move RAM MEM to FACC

JMP MOVFM	AF63	44899	Move ROM MEM to FACC

JMP MOVMF	AF66	44902	Move FACC to MEM

JMP MOVFA	AF69	44905	Move ARG to FACC

JMP MOVAF	AF6C	44908	Move FACC to ARG

OTHER BASIC ROUTINES

JMP OPTAB	AF6F	44911

JMP DRAWLN	AF72	44914

MEMORY

ADDRESS		HEXADECIMAL DECIMAL

LABEL		ADDRESS ADDRESS DESCRIPTION

OTHER BASIC ROUTINES

JMP GPLOT	AF75	44917

JMP CIRSUB	AF78	44920

JMP RUN		AF7B	44923

JMP RUNC	AF7E	44926

JMP CLEAR	AF81	44929

JMP NEW		AF84	44932

JMP LNKPRG	AF87	44935

JMP CRUNCH	AF8A	44938

JMP FNDLIN	AF8D	44941

JMP NEWSTT	AF90	44944

JMP EVAL	AF93	44947

JMP FRMEVL	AF96	44950

JMP RUN_A_PROGRAM

AF99	44953

JMP SETEXC	AF9C	44956

JMP LINGET	AF9F	44959

JMP GARBA2	AFA2	44962

JMP EXECUTE_A_LINE

AFA5	44965

MONITOR ENTRY

JMP CALL	B000	45056	MONITOR Call Entry

JMP BREAK	B003	45059	MONITOR Break Entry

JMP MONCMD	B006	45062	MONITOR Command Parser Entry

C000	49152	KERNAL (or Function) ROM $C000-$FFFF

EDITOR JUMP TABLE

JMP CINT	C000	491S2	Initialize Editor & Screen

JMP DISPLY	C003	49155	Display Charac in .A, Color

JMP LP2		C006	49158	Get Key From IRQ Buffer

JMP LOOP5	C009		Into A

JMP PRINT	C00C	49164	Print Character In .A

JMP SCRORG	C00F	49167	Get # of Scrn Rows, Cols Into X & Y

JMP SCNKEY	C012	49170	Scan Keyboard Subroutine

JMP REPEAT	C015	49173	Handle Repeat Key & Store Decoded Key

JMP PLOT	C018	49176	Read Or Set CRSR Position In X, Y

JMP CURSOR	C01B	49179	Move 8563 Cursor Subroutine

JMP ESCAPE	C01E	49182	Execute ESC Function using chr in .A

JMP KEYSET	C021	49185	Redefine A Programmable Func'n Key

JMP IRQ		C024	49188	IRQ Entry

JMP INIT80	C027	49191	Initialize 80-Column Character Set

JMP SWAPPER	C02A	49194	Swap Editor Locals (in 40/80 change)

JMP WINDOW	C02D	49197	Set Top-Left or Bot-Right of Window

D000	53248	VIC Character ROM ($D000-$DFFF)

VIC CHIP REGISTERS

VICREG0		D000	53248	Sprite 0, X-Location

VICREG1		D001	53249	Sprite 0, Y-Location

VICREG2		D002	53250	Sprite 1, X-Location

VICREG3		D003	53251	Sprite 1, Y-Location

VICREG4		D004	53252	Sprite 2, X-Location

VICREG5		D005	53253	Sprite 2, Y-Location

VICREG6		D006	53254	Sprite 3, X-Location

VICREG7		D007	53255	Sprite 3, Y-Location

VICREG8		D008	53256	Sprite 4, X-Location

VICREG9		D009	53257	Sprite 4, Y-Location

VICREG10	D00A	53258	Sprite 5, X-Location

VICREG11	D00B	53259	Sprite 5, Y-Location

VICREG12	D00C	53260	Sprite 6, X-Location

VICREG13	D00D	53261	Sprite 6, Y-Location

VICREG14	D00E	53262	Sprite 7, X-Location

VICREG15	D00F	53263	Sprite 7, Y-Location

VICREG16	D010	53264	M.S. Bit of X-Location for Sprites 0 - 7

THE COMMODORE 128 AND COMMODORE 64 MEMORY MAPS

KERNAL JUMP TABLE

New Entries For C128

JMP SPIN SPOUT	FF47		65351	Set up Fast Serial Port for I/O

JMP CLOSE ALL	FF4A		65354	Close All Logical Files for a device

JMP C64MODE	FF4D		65357	Reconfigure System as a C64 (no return)

JMP DMA CALL	FF50		65360	Initiate DMA Request to External RAM

Expansion, Send command to DMA Device

JMP BOOT CALL	FF53		65363	Boot Load Program From Disk

JMP PHOENIX	FF56		65366	Call All Function Cards' Cold Start

Routines, Initialize

JMP LKUPLA	FF59		65369	Search Tables For Given LA

JMP LKUPSA	FF5C		65372	Search Tables For Given SA

JMP SWAPPER	FF5F		65375	Switch Between 40 and 80 Columns (Editor)

JMP DLCHR	FF62		65378	Init 80-Col Character RAM (Editor)

JMP PFKEY	FF65		65381	Program Function Key (Editor)

JMP SETBNK	FF68		65384	SET Bank For I/O Operations

JMP GETCFG	FF6B		65387	Lookup MMU Data For Given Bank

JMP JSRFAR	FF6E		65390	JSR to Any Bank, RTS to Calling Bank

JMP JMPFAR	FF71		65393	JMP to Any Bank

JMP INDFET	FF74		65396	LDA (FETVEC),Y FROM Any Bank

JMP INDSTA	FF77		65499	STA (STAVEC),Y to Any Bank

JMP INDCMP	FF7A		65402	CMP (CMPVEC),Y to Any Bank

JMP PRIMM	FF7D		65405	Print Immediate Utility

(Always JSR to this routine)

STANDARD KERNAL JUMP TABLE

FF80		65408	Release Number Of KERNAL

JMP CINT	FF81		65409	Init Editor & Display

JMP IOINIT	FF84		65412	Init I/O Devices (ports, timers, etc.)

JMP RAMTAS	FF87		6S415	Initialize RAM And Buffers For System

JMP RESTOR	FF8A		65418	Restore Vectors to Initial System

JMP VECTOR	FF8D		65421	Change Vectors For USER

JMP SETMSG	FF90		65424	Control O.S. Message

JMP SECND	FF93		65427	Send SA After LISTEN

JMP TKSA	FF96		65430	Send SA After TALK

JMP MEMTOP	FF99		65433	Set/Read Top Of System RAM

JMP MEMBOT	FF9C		65436	Set/Read Bottom Of System RAM

JMP KEY		FF9F		65439	Scan Keyboard (Editor)

JMP SETTMO	FFA2		65442	Set Timeout In IEEE (reserved)

JMP ACPTR	FFA5		65445	Handshake Serial Byte In

JMP CIOUT	FFA8		65448	Handshake Serial Byte Out

JMP UNTLK	FFAB		65451	Send UNTALK Out Serial

JMP UNLSN	FFAE		65454	Send UNLISTEN Out Serial

JMP LISTN	FFB1		65457	Send LISTEN Out Serial

JMP TALK	FFB4		65460	Send TALK Out Serial

JMP READSS	FFB7		65463	Return I/O Status Byte

JMP SETLFS	FFBA		65460	Set LA, FA, SA

JMP SETNAM	FFBD		65469	Set Length And File Name Address

JMP (IOPEN)	FFC0	OPEN	65472	OPEN Logical File

JMP (ICLOSE)	FFC3	CLOSE	65475	CLOSE Logical File

JMP (ICHKIN)	FFC6	CHKIN	65478	Set Channel In

JMP (ICKOUT)	FFC9	CKOUT	65481	Set Channel Out

JMP (ICLRCH)	FFCC	CLRCH	65484	Restore Default I/O Channel

JMP (IBASIN)	FFCF	BASIN	65487	INPUT From Channel

JMP (IBSOUT)	FFD2	BSOUT	65490	OUTPUT To Channel

JMP LOADSP	FFD5		65493	LOAD From File

JMP SAVESP	FFD8		65496	SAVE to File

JMP SETTIM	FFDB		65599	Set Internal Clock

JMP RDTIM	FFDE		65502	Read Internal Clock

JMP (ISTOP)	FFE1	STOP	65505	Scan STOP Key

JMP (IGETIN)	FFE4	GETIN	65508	Read Buffered Data

JMP (ICLALL)	FFE7	CLALL	65511	Close All Files And Channels

JMP UDTIM	FFEA	CLOCK	65514	Increment Internal Clock

JMP SCRORG	FFED		65517	Return Screen Window Size (Editor)

JMP PLOT	FFF0		65520	Read/Set X,Y Cursor Coord (Editor)

JMP IOBASE	FFF3		65523	Return I/O Base

SYSTEM		FFF8		65528	Operating System Vector (RAM1)

NMI		FFFA		65530	Processor NMI Vector

RESET		FFFC		65532	Processor RESET Vector

IRQ		FFFE		65534	Processor IRQ/BRK Vector

KERNAL/EDITOR FLAGS AND SHADOW REGISTERS

The following symbols are used by the C128 Editor. Note that the Editor IRQ

VIC screen handler depends upon them. In most cases the contents of these

locations will be placed directly into the appropriate register and should be

used instead of the actual register. For example, to change the location of the

character set used by VIC, use VM1 ($0A2C) instead of VIC register 24 ($D018).

VM1 will be used by the editor to update VIC register 24.

ADDRESS NAME	EXPLANATION

$00D8/GRAPHM	See below. IF = $FF THEN EDITOR LEAVES VIC ALONE.

$00D9/CHAREN	MASK For 8502 /CHAREN BIT.

$0A2C/VM1	VIC Text Mode Video Matrix & Character Base Pointer.

$0A2D/VM2	VIC Graphic Mode Video Matrix & Bit Map Pointer.

$0A2E/VM3	8563 TEXT DISPLAY BASE Address.

$0A2F/VM4	8S63 ATTRIBUTE BASE Address.

$0A34/SPLIT	IN SPLIT SCREEN MODE, CONTAINS VALUE For MIDDLE RASTER IRQ.

$0A2B/CURMOD	8563 CURSQR Mode.

$0A21/PAUSE	CONTROL S Flag (In effect = $13)

540 COMMODORE 128

EXPLANATION OF VARIOUS KERNAL/EDITOR FLAG BYTES, ETC.

ADDRESS SYMBOL			DESCRIPTION

7	6	5	4	3	2	1	0

0000	D6510	-	(IN)	(OUT)	(IN)	(OUT)	(OUT)	(OUT)	(OUT)

0001	R6510	-	CAPKEY	CASMTR	CASSEN	CASWRT	CHAREN	HIRAM	LORAM

00F7	LOCKS	CASE	CTL S	-	-	-	-	-	-

00F8	SCROLL	OFF	LINKER

00D3	SHFLAG	-	-	-	ALT	ALPHA	CTRL		SHIFT

OA22	RPTFLG	ALL	NONE	-	-	-	-	-	-

OA26	BLNON	ON	BLNK	-	-	-	-	-	-

00F9	BEEPER	ON	-	-	-	-	-	-	-

00D8	GRAPHM	MCM	SPLIT	BMM	-	-	-	-	-

00D7	MODE	40/80	-	-	-	-	-	-	-

OA04	INIT_	CHRSET	CINT	-	-	-	-	-	BASIC

STATUS

Notes on Kernal Symbols:

Init_Status. See also above. Lets system know what has been initialized and

what hasn't. Set to $00 by a reset but untouched by NMI.

System_Vector. Where the Kernal goes when it has to go somewhere. It's set

to BASIC cold at reset. BASIC itself sets it to BASIC warm after it has

initialized. The monitor respects it too.

System. Vector in RAM1 at $FFF8. Set at power-up to C128MODE, user may redirect

it to his code. Taken at reset always providing user with control (protection)

from reset.

<end>