;	Commodore 1541 / OC-118 Disk Drive ROM Map

;	V1.1	18 Jan 1995

; Note: This Memory map is collected from the 1541 diassembly

;   which was made by Peter Weighill.

;

; Data types in headers (for reassembler):

;

;	DATA		Misc data

;	TEXT		String terminated with 00

;	WORD		Vectors in LO/HI byte pairs

;	CHIP		I/O Area

;	EMPTY		ROM containing AA's or FF's

;

c000	Unused					EMPTY

;c100	File Manager Routines

c100	Turn LED on for current drive

c118	Turn LED on

c123	Clear error flags

c12c	Prepare for LED flash after error

c146	Interpret command from computer

c194	Prepare error msg after executing command

c1bd	Erase input buffer

c1c8	Output error msg (track and sector 0)

c1d1	Check input line

c1e5	Check ':' on input line

c1ee	Check input line

c268	Search character in input buffer

c2b3	Check line length

c2dc	Clear flags for command input

c312	Preserve drive number

c33c	Search for drive number

c368	Get drive number

c38f	Reverse drive number

c398	Check given file type

c3bd	Check given drive number

c3ca	Verify drive number

c440	Flags for drive check			DATA

c44f	Search for file in directory

c48b	?

c4d8	?

c5ac	?

c63d	Test and initalise drive

c66e	Name of file in directory buffer

c688	Copy filename to work buffer

c6a6	Search for end of name in command

c6ce	?

c6de	?

c7ac	Clear Directory Output Buffer

c7b7	Create header with disk name

c806	Print 'blocks free.'

c817	'Blocks free.'				DATA

c823	Perform [S] - Scratch command

c87d	Erase file

c8b6	Erase dir entry

c8c1	Perform [D] - Backup command (Unused)

c8c6	Format disk

c8f0	Perform [C] - Copy command

c9a1	?

c9a7	?

c9d5	?

c9fa	?

ca35	?

ca88	Perform [R] - Rename command

cacc	Check if file present

caf8	Perform [M] - Memory command

cb20	M-R memory read

cb50	M-W memory write

cb5c	Perform [U] - User command

cb84	Open direct access channel, number

cc1b	Perform [B] - Block/Buffer command

cc5d	Block commands "AFRWEP"			DATA

cc63	Block command vectors			WORD

cc6f	Get parameters form block commands

ccf2	Decimal values  1, 10, 100		DATA

ccf5	B-F block free

ccfe	?

cd03	B-A block allocate

cd36	Read block to buffer

cd3c	Get byte from buffer

cd42	Read block from disk

cd56	B-R block read

cd5f	U1, Block read without changing buffer pointer

cd73	B-W block write

cd97	U2, Block write without changing buffer pointer

cda3	B-E block execute

cdbd	B-P block pointer

cdd2	Open channel

cdf2	Check buffer number and open channel

ce0e	Set pointer for REL file

ce2c	?

ce6e	Divide by 254

ce71	Divide by 120

ced9	Erase work storage

cee2	Left shift 3-byte register twice

cee5	Left shift 3-byte register once

ceed	Add 3-byte registers

cefa	?

cf09	?

cf1e	?

cf8c	Change buffer

cf9b	Write data in buffer

cff1	Write data byte in buffer

d005	Perform [I] - Initalise command

d00e	Read BAM from disk

d042	Load BAM

d075	Calculate blocks free

d09b	?

d0af	?

d0c3	Read block

d0c7	Write block

d0eb	Open channel for reading

d107	Open channel for writing

d125	Check for file type REL

d12f	Get buffer and channel numbers

d137	Get a byte from buffer

d156	Get byte and read next block

d19d	Write byte in buffer and block

d1c6	Increment buffer pointer

d1d3	Get drive number

d1df	Find write channel and buffer

d1e2	Find read channel and buffer

d227	Close channel

d25a	Free buffer

d28e	Find buffer

d2ba	?

d2da	?

d307	Close all channels

d313	Close all channels of other drives

d339	?

d37f	Find channel and allocate

d39b	Get byte for output

d44d	Read next block

d460	Read block

d464	Write block

d475	Allocate buffer and read block

d486	Allocate new block

d48d	Write dir block

d4c8	Set buffer pointer

d4da	Close internal channel

d4e8	Set buffer pointer

d4f6	Get byte from buffer

d506	Check track and sector numbers

d552	Get track and sector numbers for current job

d55f	Check for vaild track and sector numbers

d572	DOS mismatch error

d586	Read block

d58a	Write block

d599	Verify execution

d5c6	Additional attempts for read errors

d676	Move head by half a track

d693	Move head one track in or out

d6a6	Attempt command execution multiple times

d6d0	Transmit param to disk controller

d6e4	Enter file in dir

d7b4	OPEN command, secondary addr 15

d7c7	-Check '*' Last file

d7f3	-Check '$' Directory

d815	-Check '#' Channel

d8f5	Open a file with overwriting (@)

d940	?

d9a0	Open file for reading

d9e3	Open file for writing

da09	Check file type and control mode

da2a	Preparation for append

da55	Open directory

dac0	Close routine

db02	Close file

db62	Write last block

dba5	Directory entry

dc46	Read block, allocate buffer

dcb6	Reset pointer

dcda	Construct a new block

dd8d	Write byte in side-sector block

dd95	Manipulate flags

dda6	?

ddab	Verify command code for writing

ddb7	?

ddf1	Write a block of a REL file

ddfd	Write bytes for following track

de0c	Get following track and sector numbers

de19	Following track for last block

de2b	buffer pointer to zero

de3b	Get track and sector

de50	Write (?)

de57	R

de5e	W

de65	R

de6c	W

de73	R

de95	Get following track and sector from buffer

dea5	Copy buffer contents

dec1	Erase buffer Y

ded2	Get side-sector number

dedc	Set buffer pointer to side-sector

dee9	Buffer pointer for side-sector

def8	Get side sector and buffer pointer

df1b	Read side-sector

df21	Write side-sector

df45	Set buffer pointer in side-sector

df4c	Calculate number of blocks in a REL file

df66	Verify side-sector in buffer

df93	Get buffer number

df9e	?

dfb7	?

dfc2	?

dfd0	Get next record iin REL file

e03c	Write block and read next block

e07c	Write a byte in a record

e0ab	Write byte in REL file

e0f3	Fill record with 0s

e105	Write buffer number in table

e120	Get byte from REL file

e1cb	Get last side-sector

e207	Perform [P] - Position command

e2e2	Divide data blocks into records

e304	Set pointer to next record

e31c	Expand side-sector

e44e	Write side-sector and allocate new

e4fc	Table of error messages			DATA

;	(with start and end letters +$80)  If value AND $7F less

;	 than $10 then look up text in pointer table.

;

;	Format: Error numbers, Description

;

;e4fc	00,	Ok

;e500	20,21,22,23,24,27, Read error

;e50b	52,	File too large

;e517	50,	Record not present

;e522	51,	Overflow in record

;e52f	25,28,	Write error

;e533	26,	Write protect on

;e540	29,	Disk id mismatch

;e546	30,31,32,33,34, Syntax error

;e552	60,	Write file open

;e556	63,	File exists

;e55f	64,	File type mismatch

;e567	65,	No block

;e570	66,67,	Illegal track or sector

;e589	61,	File not open

;e58d	39,62,	File not found

;e592	01,	Files scratched

;e59f	70,	No channel

;e5aa	71,	Dir error

;e5af	72,	Disk full

;e5b6	73,	Cbm dos v2.6 1541

;e5c8	74,	Drive not ready

;

e5d5	Indexed words				DATA

;	Format: Index number  Description

;

;e5d5	09 Error

;e5db	0A Write

;e5e1   03 File

;e5e6   04 Open

;e5eb   05 Mismatch

;e5f4   06 Not

;e5f8   07 Found

;e5fe   08 Disk

;e603   0B Record

;

e60a	Prepare error number and message

e645	Print error message into error buffer

e680	TALK

e688	LISTEN

e69b	Convert BIN to 2-Ascii (error message buffer)

e6ab	Convert BCD to 2-Ascii (error message buffer)

e6bc	Write OK in buffer

e6c1	Print error on track 00,00 to error buffer

e6c7	Print error on current track to error buffer

e706	Write error message string to buffer

e754	Get character and in buffer

e767	Get a char of the error message

e775	Increment pointer

e77f	Dummy subroutine

e780	Check for auto start - removed

e7a3	Perform [&] - USR file execute command

e84b	Generate checksum

e853	IRQ routine for serial bus

e85b	Service the serial bus

e909	Send data

e99c	DATA OUT lo

e9a5	DATA OUT hi

e9ae	CLOCK OUT hi

e9b7	CLOCK OUT lo

e9c0	Read IEEE port

e9c9	Get data byte from bus

e9f2	Accept byte with EOI

ea2e	Accept data from serial bus

ea59	Test for ATN

ea6e	Flash LED for hardware defects, self-test

eaa0	Power-up RESET routine

ebff	Wait loop

ec9e	Load dir

ed23	?

ed59	Transmit dir line

ed67	Get byte from buffer

ed84	Perform [V] - Validate command

edb3	?

ede5	Allocate file blocks in BAM

ee0d	Perform [N] - New (Format) command

eeb7	Create BAM

eef4	Write BAM if needed

ef3a	Set buffer pointer for BAM

ef4d	Get number of free blocks for dir

ef5c	Mark block as free

ef88	Set flag for BAM changed

ef90	Mark block as allocated

efcf	Erase bit for sector in BAM entry

efe9	Powers of 2				DATA

eff1	Write BAM after change

f005	Erase BAM buffer

f011	?

f05b	?

f0d1	Crear BAM				?

f0df	?

f10f	Get buffer number for BAM

f119	Buffer number for BAM

f11e	Find and allocate free block

f1a9	Find free sector and allocate

f1fa	Find free sectors in current track

f220	Verify number of free blocks in BAM

f24b	Establish number of sectors per track

f258	Dummy subroutine

;

;f259	Disk Controller Routines

;

f259	Initialise disk controller

f2b0	IRQ routine for disk controller

f2f9	Head transport

f36e	Execute program in buffer

f37c	Bump, find track 1 (head at stop)

f393	Initialise pointer in buffer

f3b1	Read block header, verify ID

f410	Preserve block header

f418	Work Return value 01 (OK) into queue

f41b	Work Return value 0B (READ ERROR) into queue

f41e	Work Return value 09 (READ ERROR) into queue

f423	Job optimisation

f4ca	Test command code further

f4d1	Read sector

f50a	Find start of data block

f510	Read block header

f556	Wait for SYNC

f56e	Test command code further

f575	Write data block to disk

f5e9	Calculate parity for data buffer

f5f2	Convert buffer of GCR data into binary

f691	Test command code further

f698	Compare written data with data on disk

f6ca	Command code for find sector

f6d0	Convert 4 binary bytes to 5 GCR bytes

f77f	GCR (5-bit) nybble table		DATA

f78f	Convert 260 bytes to 325 bytes group code

f7e6	Convert 5 GCR bytes to 4 binary bytes

f8a0	Conversion table GCR to binary - high nybble

;	$FF means invalid

f8c0	Conversion table GCR to binary - low nybble

;	$FF means invalid

f8e0	Decode 69 GCR bytes

f934	Convert block header to GCR code

f969	Error entry disk controller

f97e	Turn drive motor on

f98f	Turn drive motor off

f99c	Job loop disk controller

f9cb	?

f9d9	?

fa05	Move head to next track

fa1c	Calculate number of head steps

fa3b	Move stepper motor short distance

fa4e	Load head

fa63	?

fa7b	Prepare fast head movement

fa97	Fast head movement

faa5	Prepare slow head movement

fac7	Formatting

fda3	Write SYNC 10240 times, erase track

fdc3	Read/write ($621/$622) times

fdd3	Attempt counter for formatting

fde5	?

fdf5	Copy data from overflow buffer

fe00	Switch to reading

fe0e	Write $55 10240 times

fe30	Convert header in buffer 0 to GCR code

fe67	Interrupt routine

;fe85	Constands for disk format

fe85	Directory track				DATA

fe86	Start of bam				DATA

fe87	Length of bam per track			DATA

fe88	End of bam				DATA

fe89	Table of command words			DATA

;	"VIDMBUP&CRSN"

;	Validate, Initialise, D(Backup unused),

;	Memory, Block, User,

;	Position, &and;USR execute, Copy,

;	Rename, Scratch, New (Format).

fe95	Low  byte of command addresses		DATA

fea1	High byte of command addresses		DATA

fead	Bytes for syntax check			DATA

feb2	File control methods "RWAM"		DATA

feb6	File types "DSPUL"			DATA

febb	Names of file types			DATA

febb	1st letter "DSPUR"			DATA

fec0	2nd letter "EERSE"			DATA

fec5	3rd letter "LQGRL"			DATA

feca	Error LED bit value			DATA

fecd	Masks for bit command			DATA

fed1	Number of sectors per track		DATA

fed5	Constands for disk format		DATA

fed5	'A' marker for 1541 format		DATA

fed6	4 track ranges				DATA

fed7	Number of tracks			DATA

fed8	Tracks on which sector numbers change	DATA

fedb	Control bytes for head postion		DATA

fee0	Buffer address high bytes		DATA

fee6	ROM checksum				DATA

fee7	From UI command $EB22, to reset

;	Without RAM/ROM test

feea	Patch for diagnostic routine from $EA7A

fef3	Delay loop for serial bus in 1541 mode, from $E97D

fefb	Patch for data output to serial bus, from $E980

ff01	U9 vector, switch 1540/1541

ff10	Patch for reset routine, from $EAA4

ff20	Patch for listen to serial bus, from $E9DC

ff2f	Unused					EMPTY

ffe6	Format	[C8C6]				WORD

ffe8	Turn motor off [F98F]			WORD

ffea	USER vectors

ffea	UA, U1	[CD5F]				WORD

ffec	UB, U2	[CD97]				WORD

ffee	UC, U3	[0500]				WORD

fff0	UD, U4	[0503]				WORD

fff2	UE, U5	[0506]				WORD

fff4	UF, U6	[0509]				WORD

fff6	UG, U7	[050C]				WORD

fff8	UH, U8	[050F]				WORD

fffa	UI, U9	[FF01]				WORD

fffc	Hardware vectors

fffc	RESET [EAA0]				WORD

fffe	IRQ   [FE67]				WORD