From: rhialto@mbfys.kun.nl (Olaf Seibert)

Subject: PIA and VIA info (PET)

Organization: University of Nijmegen, The Netherlands

Date: Tue, 13 Dec 1994 11:13:11 GMT

PIA 6520 and VIA 6522

---------------------

Summarised by Olaf Seibert, from "Programming the PET/CBM" (by Raeto Collin

West) chapter 14 and The Transactor, Volume 4 Issue 05.

In the PET, you find the two PIAs at E810 and E820, and the VIA at E840.

The PIA

-------

The PIA has two 8-bit I/O ports, A and B, which are mostly identical in

function.  All 8 bits can be set to input (1 in the DDR) or output (0 in

the DDR) independently. Bit 2 in CRx determines whether the DDRx or Px are

accessed.  Each port has 2 control lines: CA1, CA2, CB1, CB2. The Cx1 are

input only, the Cx2 can be input or output.

PIAs have two interrupt lines: IRQA and IRQB. They may go low on a change

on the inputs of the Cxy lines. These interrupts may be enabled on either

an 1->0 or an 0->1 transition. The selected transition is called the

"active" transition. The flags which register that an active transition has

occurred are reset by reading the appropriate PORT register.

Register map:

E810  PORT A or DDR A: Data Direction Register A

E811  CRA: Control Register A

E812  PORT B or DDR B: Data Direction Register B

E813  CRB: Control Register B

Control registers:

CRA:

bit   meaning

---   -------

7    CA1 active transition flag. 1= 0->1, 0= 1->0

6    CA2 active transition flag. 1= 0->1, 0= 1->0

5    CA2 direction           1 = out        | 0 = in

4    CA2 control   Handshake=0 | Manual=1   | Active: High=1 Low=0

3    CA2 control   On Read=0   | CA2 High=1 | IRQ on=1, IRQ off=0

Pulse  =1   | CA2 Low=0  |

2    Port A control: DDRA = 0, IORA = 1

1    CA1 control: Active High = 1, Low = 0

0    CA1 control: IRQ on=1, off = 0

CRB works identical for CB1 and CB2, except for the differences in

handshaking.

The Cx2 handshake is not identical between ports. For port A, the handshake

is on reading the PORT A register, for CB2 the handshake is sent on writing

the PORT B register.

On the listening side:

BIT 3 LOW with CA2: CA2 is now controlled by two events:

(i) CA1 active transition sets it high ("Data Valid")

(ii) a READ operation sets it low ("Data Accepted").

On the talking side:

BIT 3 LOW with CB2: CB2 is now controlled by two events:

(i) CB1 active transition sets it high ("Ready For Data")

(ii) a WRITE operation sets it low ("Data Valid").

Bit 2 HIGH: Causes pulse output, CA2 or CB2 going low for one cycle only

after a read or write operation. This pulse may be too short for some uses.

For operation with a sending and a receiving PIA one would connect

talker      listener

------      --------

Port B   -> Port A

CB2      -> CA1 with active = 1->0 (data valid)

CB1     <-  CA2 with active = 1->0 (data accepted and ready for more)

Use of PIA signals in a PET:

PIA 1

E810	PORT A	7   Diagnostic sense (pin 5 on the user port)

6   IEEE EOI in

5   Cassette sense #2

4   Cassette sense #1

3-0 Keyboard row select (through 4->10 decoder)

E811	CA2         output to blank the screen (old PETs only)

IEEE EOI out

CA1         cassette #1 read line

E812	PORT B	7-0 Contents of keyboard row

Usually all or all but one bits set.

E813	CB2         output to cassette #1 motor: 1=on, 0=off

CB1         screen retrace detection in

PIA 2

E820	PORT A      Input buffer for IEEE data lines

E821	CA2         IEEE NDAC out

CA1         IEEE ATN in

E822	PORT B      Output buffer for IEEE data lines

E823	CB2         IEEE DAV out

CB1         IEEE SRQ in

The VIA

-------

The VIA is a superset of the PIA. Many of the principles apply here as

well, though the organisation is slightly different.

The VIA has two ports, PA and PB, 4 control lines C[AB][12], an 8-bit shift

register SR and 2 timers TA and TB.

Like the PIA, the Cx1 lines are input only, the Cx2 lines are I or O.

Port A has two registers. One register causes handshaking with CA1 to

happen, the other doesn't. Port B occurs in memory before port A.

There are control registers CRA, CRB, ACR, PCR, and interrupt registers IFR

and IER. The data direction registers DDRA and DDRB have their own

addresses, unlike the PIA.

I'll only describe the extra or different features.

Timers. The VIA has two 16-bit timers T1 and T2. When written to the timers

start counting down on each clock cycle, and when reaching 0000 will flag

and may cause an interrupt or other special action. When the low byte of

the timer is read, the interrupt flag is cleared. When writing to the high

byte clears the flag and starts the timer counting.

T1 has a latch register. This is a place to store the timer value before it

will be used. When T1 reaches 0, the latch value is moved into the timer so

that the countdown may begin all over again, if so enabled.

Ports. PA and PB may be latched, so that on an active transition of CA1 the

value in the PA register is retained indefinitely (or until the next active

transition on that pin), and similarly with CB1 and PB.

The shift register. This 8-bit register is connected to CB2. On command the

SR performs 8 shifts, moving 8 bits to or from CB2 one at a time. The most

significant bit is moved first (the register shifts left). The SR can be

timed by T2 and at the same rate as the 6202, using the phase 2 clock phi2.

Alternatively an external clock may time it.

It seems there are bugs in the shift register. For instance, when doing

tape I/O, the CB2 sound, which uses the SR, must be turned off.

The ACR controls the timers, shift register and latch status of PA and PB.

(T1 has effects on pin PB7, which are not described in the book, but I

suspect they are similar to the features of the CIA in this respect. -Olaf)

ACR E84B

7-6 Timer 1 control

0 = PB7 unused

0 0 = one shot

0 1 = continuous, i.e. on underflow timer restarts at latch value.

(The following 3 lines are guessed, based on the Amiga's CIA description)

1 = PB7 used

1 0 = T1 underflow toggles PB7

1 1 = T1 underflow pulses PB7

5   Timer 2 control

0 = one shot

1 = count set no. of PB6 pulses

4-2 Shift register control

000 = shift reg disabled

001 = shift in by timer 2

010 = shift in by phi2

011 = shift in by external clock (PB6?)

100 = free run by timer 2 (setting for sound)

this means keep shifting the same byte out over and over.

T2 is hereby set to continuous mode.

101 = shift out by timer 2

110 = shift out by phi2

111 = shift out by external clock (PB6?)

1   Port B latch

0 = disabled

1 = enabled on CB1 transition (in/out)

0   Port A latch

0 = disabled

1 = enabled on CA1 transition (in)

The Periheral Control Register PCR  controls the operating modes of the

control lines CA1-CB2.

PCR E84C

7-5 CB2 control

7  direction

1 = output

1 0 = do handshake

1 0 0 = on write

1 0 1 = pulse?

1 1 = manual CB2 control

1 1 0 = CB1 low

1 1 1 = CB1 high

0 = input

0 0 x = Active high: 0->1

0 1 x = Active low:  1->0

0 x 0 = Clear IFR/ORB (don't ask me what this means -Olaf)

0 x 1 = Clear IFR

4   CB1 control

0 = active transition High

1 = active transition Low

3-1 CA2 control (similar to CB2 control)

3  direction

1 = output

1 0 = do handshake

1 0 0 = on read

1 0 1 = pulse

1 1 = manual CA2 control

1 1 0 = CA1 low

1 1 1 = CA1 high

0 = input

0 0 x = Active high: 0->1

0 1 x = Active low:  1->0

0 x 0 = Clear IFR/ORB (don't ask me what this means -Olaf)

0 x 1 = Clear IFR

The handshaking is not specifically described in the book, so I presume it

is identical to that of the PIA.

Interrupt Flag Register IFR and Interrupt Enable Register IER. These

registers are symmetrical wrt each other. The IER enables specific

interrupts (i.e., allows an event to trigger an interrupt), and the IFR

flags if the event took place.

When writing to the IER bit 7 controls the meaning of the 1 bits in the

data: when set each 1-bit sets the interrupt enable bit, when cleared it

resets that interrupt enable bit.

IFR E84D

IER E84E

7   IFR: master flag bit: 1 when any of the other bits are set

IER: 0 disables, 1 enables interrupts

6   Timer 1 underflow

5   Timer 2 underflow

4   CB1 active transition

3   CB2 active transition

2   Shift register full/empty

1   CA1 active transition

0   CA2 active transition

Use of VIA signals in a PET

---------------------------

E840	PORT B	7   DAV in

6   NRFD in

5   Video retrace in

4   Tape #2 motor (note 2)

3   Tape data out

2   ATN out

1   NRFD out

0   NDAC out

E841	PORT A	    USER PORT with CA2 handshake (note 1)

E842	DDRB	7-0 normal bits set to 0 0 0 1 1 1 1 0

E843	DDRA	7-0 USER PORT data direction register

E844	Timer 1 LO

E845	Timer 1 HI  (set to $FF on system power-on)

E846	Timer 1 latch LO

E847	Timer 1 latch HI

E848	Timer 2 LO

E849	Timer 2 HI

E84A	Shift register

E84B	ACR	Aux. control register; set to $00 at power on

7-6 timer 1 control

5   timer 2 control

4-2 shift register control

1   port B latch

0   port A latch

E84C	PCR	Peripheral Control Register; set to $0C or $0E at power on

7-5 CB2 control (user port pin M) (note 3)

4   CB1 control (note 3)

3-1 CA2 control (graphics mode) (note 3)

0   CA1 control (note 3)

E84D	IFR	Intertrupt Flag Register; set to $00 at power on

7   IRQ on/off

6   T1 interrupt flagged

5   T2 interrupt flagged

4   CB1 interrupt flagged

3   CB2 interrupt flagged

2   shift register interrupt flagged

1   CA1 interrupt flagged

0   CA2 interrupt flagged

E84E	IER	Interrupt Enable Register; set to $80 at power on

7   1=enable, 0=disable

6-0 enable interrupts; same bits as in IFR.

E84F	PORTA	    USER PORT without CA2 handshake

Note 1: E84F is the preferred user port register, since CA2 controls screen

graphics.

Note 2: The motor is on when this line is low, and off when it is high.

Note 3: CA1 is connected to pin B of the user port. Pins B-L correspond to

port A, which is invaiably E84E. CB2 (connected to the shift register) also

connects to pin M of the user port; square wave tones (see chapter 9 of

"Programming the PET/CBM") use these facts. CB1 signals input from cassette

#2. CA2 controls screen graphics: it is configured for output, and, when

low, gives lower case characters and others. When high, the mode is upper

case and graphics.

IEEE Port Pinouts

-----------------

This info taken from Transactor Volume 4, Issue 05.

rear view for IEEE and User port:

1   2   3   4   5   6   7   8   9   10  11  12

--- --- --- --- --- --- --- --- --- --- --- ---

===================================================

--- --- --- --- --- --- --- --- --- --- --- ---

A   B   C   D   E   F   H   J   K   L   M   N

The User port is the second port from the left, the IEEE-488 port is

the third port.

PET   IEEE

Pin#  Pin# Mnemonic Definition

-----------------------------------

1     1   DIO1	    Data Input/Output Line #1

2     2   DIO2	    Data Input/Output Line #2

3     3   DIO3	    Data Input/Output Line #3

4     4   DIO4	    Data Input/Output Line #4

5     5   EOI      End Or Identify (PIA1 PA6 in, PIA1 CA2 out)

6     6   DAV      Data Valid

7     7   NRFD     Not Ready For Data

8     8   NDAC     Not Data Accepted

9     9   IFC      Interface Clear

10    10   SRQ      Service Request (PIA2 CB1)

11    11   ATN      Attention (VIA PB2 out, PIA2 CB1)

12    12   GND      Chassis Ground (IEEE cable shield)

A    13   DIO5     Data Input/Output Line #5

B    14   DIO6     Data Input/Output Line #6

C    15   DIO7     Data Input/Output Line #7

D    16   DIO8     Data Input/Output Line #8

E    17   REN      Remote Enable

F    18   GND      DAV Ground

H    19   GND      NRFD Ground

J    20   GND      NDAC Ground

K    21   GND      IFC Ground

L    22   GND      SRQ Ground

M    23   GND      ATN Ground

N    24   GND      Data Ground (DIO1-8)

Pin# Function Description

-------------------------

1   Ground   System Ground

2   TV Video Video Out for external displays

3   SRQ      Connected to IEEE-488 SRQ (PIA2 CB1)

4   EOI      Connected to IEEE-488 EOI (PIA1 PA6 in, PIA1 CA2 out)

5   Diagnostic Sense (PIA1 PA7)

Held low causes power up to diagnostic routines or monitor

6   Read 1   Connected to cassette #1 read line (PIA1 CA1)

7   Read 2   Connected to cassette #2 read line (VIA CB1)

8   Write    Diagnostic tape write verify

9   Vert     TV Vertical for external displays

10   Horiz    TV Horizontal for external displays

11   GND

12   GND

A   GND

B   CA1      Edge sensitive input for 6522 VIA

C   PB0      PB0-7 are independently programmable

D   PB1      for Input or Output

E   PB2

F   PB3

H   PB4

J   PB5

K   PB6

L   PB7

M   CB2      Special IO pin of VIA, connected to shift register

N   GND      Digital ground

Cassette port

-------------

1   2   3   4   5   6

--- --- --- --- --- ---

==========================

--- --- --- --- --- ---

A   B   C   D   E   F

Pin# Name  Description

----------------------

A-1  GND   Digital Ground

B-2  +5V   +5 Volts to operate cassette circuitry only

C-3  Motor Computer controlled +6V for cassette motor

D-4  Read  Read line from cassette

E-5  Write Write line to cassette

F-6  Sense Monitors closure of any locking type cassette switch

-Olaf.

--

___ Olaf 'Rhialto' Seibert    rhialto@mbfys.kun.nl     Ooey-Gooey-Fluffy-Barfie

\X/ I'm not weird, I'm differently perceived. D787B44DFC896063 4CBB95A5BD1DAA96