Newsgroups: comp.sys.cbm

From: rhialto@mbfys.kun.nl (Olaf Seibert)

Subject: PET RAM memory map

Organization: University of Nijmegen, The Netherlands

Date: Wed, 14 Dec 1994 19:22:02 GMT

This could be improved upon - not all locations have labels.

I must note I've never seen a PET memory map with labels, so in that

respect this is a first. I could add the locations for the Basic 1

ROMs in a next version. Corrections welcome. Enjoy.

;	Commodore PET RAM Memory Map

;	for Basic 2 and 4 (40 and 80 columns)

;	V1.1	13 dec 1994

;

;	Note that the labels in this list are matched up against C-64

;	memory maps. All attempts were made to secure plausibility of

;	placement.

;	Locations marked with (64#..) are found by taking the label given

;	for the C-64 and comparing ROM disassemblies. Many of these

;	locations have RS-232 specific names.

LABEL	HEX		DECIMAL

ADDRESS		LOCATION	DESCRIPTION

USRPOK	0000		0  	USR Function Jump Instr (4C)

USRADD	0001-0002	1-2    	USR Address Low	Byte / High Byte

CHARAC	0003		3	Search Character

ENDCHR	0004		4	Flag: Scan for Quote at End of String

COUNT	0005		5	Input Buffer Pointer / No. of Subscripts

DIMFLG	0006		6	Flag: Default Array DiMension / array name

initial / AND, OR flag

VAUYP	0007		7	Data Type: $FF = String, $00 = Numeric

INTFLG	0008		8	Data Type: $80 = Integer, $00 = Floating

GARBFL	0009		9	Flag: DATA scan/LlST quote/Garbage Coll

SUBFLG	000A		10	Flag: Subscript Ref / User Function Call

INPFLG	000B		11	Flag: $00 = INPUT, $40 = GET, $98 = READ

TANSGN	000C		12	Flag TAN sign / Comparison Result

000D		13	3: Flag to suppress PRINT or PRINT# when -ve

000E		14	3: File# of current I/O device (as 0010)

000F		15	3: terminal width (unused-carried over from TTY)

0010		16	3: width of source (unused - from TTY)

000D-000F	13-15	4: Disk Status DS$ descriptor

0010		16	4: File# of current I/O device (when non-zero

suppresses INPUT prompt etc)

LINNUM	0011-0012	17-18	Temp: Integer Value

TEMPPT	0013		18	Pointer Temporary String

LASTPT	0014-0015	19-21	Last Temp String Address

TEMPST	0016-001E  	22-30	Stack for Temporary Strings

INDEX	001F-0022	31-34	Utility Pointer Area

RESHO	0023-0027	35-39	Floating-Point Product of Multiply

TXTTAB	0028-0029	40-41	Pointer: Start of BASIC	Text

VARTAB	002A-002B	42-43	Pointer: Start of BASIC	Variables

ARYTAB	002C-002D	44-45	Pointer: Start of BASIC Arrays

STREND	002E-002F	46-47	Pointer End of BASIC Arrays (+1)

FRETOP	0030-0031	48-49	Pointer: Bottom of String Storage

FRESPC	0032-0033	50-51	Utility String Pointer

MEMSIZ	0034-0035	52-53	Pointer: Highest Address Used by BASIC

CURLIN	0036-0037	54-55	Current BASIC Line Number

OLDLIN	0038-0039	56-67	Previous BASIC Line Number

OLDTXT	003A-003B	58-59	Pointer: BASIC Statement for CONT

DATLIN	003C-003D	60-61	Current DATA Line Number

DATPTR	003E-003F	62-63	Pointer: Current DATA Item Address

INPPTR	0040-0041	64-65	Vector: INPUT Routine

VARNAM	0042-0043	66-67	Current BASIC Variable Name

VARPNT	0044-0045	68-69	Pointer: Current BASIC Variable Data

FORPNT	0046-0047	70-71	Pointer: Index Variable for FOR/NEXT

0048-0049	72-73	Y-save; op-save; BASIC pointer save

004A		74	Comparison symbol accumulator: bits 0,1,2

are <, =, >.

004B-004C	75-76	Pointer to temp storage in RAM for FN DEF,

TAN, &c

004D-0050	77-80	Pointer to string, length and garbage

collect constant

TEMPF1	0054-0058		Temporary storage for FLPT value.

TEMPF2	0059-005D		Temporary storage for FLPT value.

FACEXP	005E		94	Floating-Point Accumulator #1: Exponent

FACHO	005F-0062	95-98	Floating Accum.	#1: Mantissa

FACSGN	0063		99	Floating Accum.	#1: Sign

SGNFLG	0064		100	Pointer: Series Evaluation Constant Pointer

BITS	0065		101	Floating -accum. #1: Overflow Digit

ARGEXP	0066		102	Floating-Point Accumulator #2: Exponent

ARGHO	0067-006A	103-106	Floating Accum. #2: Mantissa

ARGSGN	006B		107	Floating Accum. #2: Sign

ARISGN	006C		108	Sign Comparison Result: Accum. # 1 vs #2

FACOV	006D		109	Floating Accum. #1. Low-Order (Rounding)

FBUFPT	006E-006F	110-111	Pointer: Cassette Buffer

CHRGET	0070-0087	112-135	Subroutine: Get Next Byte of BASIC Text

CHRGOT	0076		118	Entry to Get Same Byte of Text Again

TXTPTR	0077-0078	119-120	Pointer: Current Byte of BASIC Text

0070			INC $77

BNE $0076

0076			LDA $xxxx

CMP #$3A

BCS $0087

CMP #$20

BEQ $0070

SEC

SBC #$30

SEC

SBC #$D0

0087			RTS

RNDX	0088-008C	136-140	Floating RND Function Seed Value

TIME	008D-008F	141-143	Real-Time Jiffy Clock (approx) 1/60 Sec

CINV	0090-0091	144-145	Vector: Hardware Interrupt

CBINV	0092-0093	146-147	Vector: BRK Instr. Interrupt

NMINV	0094-0095	148-149	Vector: Non-Maskable Interrupt

STATUS	0096		150	Kernal I/O Status Word: ST

LSTX	0097		151	Current Key Pressed: 255 = No Key

SFDX	0098		   	Flag: Print Shifted Chars.

0099-009A               Jiffy clock correction: 623rd 1/60 sec

does not increment time

STKEY	009B		155	Flag: STOP key / RVS key

SVXT	009C		156	Timing Constant for Tape

VERCK	009D		157	Flag: 0 = Load, 1 = Verify (Kernel)

NDX	009E		158	No. of Chars. in Keyboard Buffer (Queue)

RVS	009F		159	Flag: Print Reverse Chars. -1=Yes, 0=No Used

C3PO	00A0		160	Flag: Serial Bus-Output Char. Buffered

INDX	00A1		161	Pointer: End of Logical	Line for INPUT

00A2		162	Not Used

LXSP	00A3-00A4	163-164	Cursor Y-X Pos. at Start of INPUT

BSOUR	00A5		165	Buffered Character for IEEE Bus

00A6		166	Key Image

BLNSW	00A7		167	Cursor Blink enable: 0 = Flash Cursor

BLNCT	00A8		168	Timer: Countdown to Toggle Cursor

GDBLN	00A9		169	Character Under Cursor

BLNON	00AA		170	Flag: Last Cursor Blink	On/Off

?SYNO	00AB		171	Cassette Sync No. (64#0096)

?NXTBIT	00AB		171	Tape EOT Flag: EOT received from tape

CRSW	00AC		172	Flag: INPUT or GET from	Keyboard

00AD		173	X save in tape handling (saves cassette #)

LDTND	00AE		174	No. of Open Files / Index to File Table

DFLTN	00AF		175	Default Input Device (0)

DFLTO	00B0		176	Default Output (CMD) Device (3)

PRTY	00B1		177	Tape Character Parity

DPSW	00B2		178	Flag: Tape Byte-Received

PSW=DPSW

00B3		179	Temporary save eg. logical address or DOS

wedge

00B7		183	Temp Data Area (64#00A3)

00B9		185	Temp Data Area (64#00A4)

BUFPNT	00BB		   	Pointer: Tape I/O Buffer #1

00BC		   	Pointer: Tape I/O Buffer #2

INBIT	00BD		189	Cassette Temp (64#00A7)

BITCI	00BE		190	Cassette Temp (64#00A8)

RINONE	00BF		191	RS-232 Flag: Check for Start Bit (64#00A9)

FNMIDX 00C0            192     Index to Cassette File name/Header ID for

Tape write.

PTR1	00C0		192	Tape Pass 1 Error Log

PTR2	00C1		193	Tape Pass 2 Error Log

RIDATA	00C2		194	Cassette Temp (64#00AA) read flags: 0=scan,

1-15=count, $40=load, $80=end of tape marker

RIPRTY	00C3		195	Cassette Short Cnt (64#00AB): counter of seconds

before tape write / checksum

PNT	00C4-00C5	196-197	Pointer: Current Screen	Line Address

PNTR	00C6		198	Cursor Column on Current Line

SAL	00C7-00C8	199-200	Pointer: Tape Buffer/ Screen Scrolling

EAL	00C9-00CA	201-202	Tape End Addresses/End of Program

CMP0	00CB-00CC	203-204	Tape Timing Constants

QTSW	00CD		205	Flag: Editor in Quote Mode, $00 = NO

BITTS	00CE		206	Cassette Temp (64#00B4): Tape read timer flag

=IRQ enabled for Timer 1

00CF		207	End of tape read

00D0		208	Read character error

FNLEN	00D1		209	Length of Current File Name

LA	00D2		210	Current Logical File Number

SA     00D3            211     Current Secondary Address

FA	00D4		212	Current Device Number

LNMX	00D5		213	Physical Screen Line Length

00D5            213     4.80: right side of window

TAPE1	00D6-00D7	214-215	Pointer: Start of Tape Buffer

TBLX	00D8		216	Current Cursor Physical Line Number

DATAX  00D9            217     Current Character to Print

FNADR	00DA-00DB	218-219	Pointer: Current File Name

INSRT	00DC		220	Flag: Insert Mode, >0 = # INSTs

?ROPRTY	00DD		121	Cassette Temp

FSBLK	00DE		222	Cassette Read / Write Block Count

MYCH	00DF		223	Serial Word Buffer

LDTB1	00E0-00F8	224-248	3+4.40: Screen Line Link Table / Editor Temps

SCTOP  00E0            224     4.80: first line of window

SCBOT  00E1            225     4.80: last line of window

SCLF   00E2            226     4.80: first column of window

XMAX	00E3		227	4.80: Size of Keyboard Buffer

XMAX	03EB		1003	4.40

RPTFLG	00E4		228	4.80: Flag: REPEAT Key Used, $80 = Repeat

$40 = disable

RPTFLG	03EE		1006	4.40

KOUNT	00E5		651	4.80: Repeat Speed Counter

KOUNT	03EA		1002	4.40

DELAY	00E6	        230	4.80: Repeat Delay Counter

DELAY	03E9		1001	4.40

00E7		231	4.80: Chime Time

03EC		1004	4.40: Chime Time

00E8		232	4.80: Home Count

00E9-00EA       233-234	4.80: input from screen vector (from E006)

00EB-00EC       235-236	4.80: print to screen vector (from E009)

00ED-00F7	237-247	4.80: not used

00F8		248	4.80: Counter to speed TI by 6/5

03ED		1005	4.40: Counter to speed TI by 6/5

CAS1	00F9		249	Tape Motor Interlock #1

CAS2	00FA		250	Tape Motor Interlock #2

STAL	00FB-00FC	251-252	I/O Start Address

MEMUSS	00FD-00FE	253-254	Tape Load Temps

00FF		255	Not used

0100-01FF	256-511	Micro-Processor System Stack Area

0100-010A	256-266	Floating to String Work Area

BAD	0100-013E	256-318	Tape Input Error Log

BUF	0200-0250	512-592	System INPUT Buffer

LAT	0251-025A	593-602	KERNAL Table: Active Logical File No's.

FAT	025B-0264	603-612	KERNAL Table: Device No. for Each File

SAT	0265-0270	613-623	KERNAL Table: Second Address Each File

KEYD	0270-027A	624-633	Keyboard Buffer Queue (FIFO)

TBUFFR	027A-0329	634-825	  Tape I/O Buffer #1

027A			  Type of tape file:

1=program header for SAVE "",1,0

2=data block

3=absolute load SAVE "",1,1 (VIC-20 and later)

4=data file header

5=end of tape block: SAVE "",1,2

027B-027C		  Start address for load

027D-027E		  End address for load

027F-028E		  File name

TBUFFR	033A-03F9	826-1017  Tape I/O Buffer #2

033A			  4: DOS byte parameter in RECORD / char ptr

033B			  4: DOS drive 1 number

033C			  4: DOS drive 2 number

033D			  4: DOS length / write flag

033E			  4: DOS 8-bit syntax checking flag

033F-0340		  4: DOS diskette ID

0341			  4: Length of DOS command string

0342-0352		  4: Buffer for filename

0353-0380		  4: Full DOS command string buffer

03EE-037F		  4.80: Table of 80 bits to set TABs

DELAY	03E9		1001	4.40

KOUNT	03EA		1002	4.40

XMAX	03EB		1003	4.40

03EC		1004	4.40: Chime Time

03ED		1005	4.40: Counter to speed TI by 6/5

RPTFLG	03EE		1006	4.40

03F0-03F9		  4.40: Table of 80 bits to set TABs

03FA-03FF	1018-1023	Unused

TIMOUT	03FC		1020	4: Flag: Kernal Variable for IEEE Timeout

0400-8000	1024-32767      Basic program area

0400			0 byte at start of Basic program

0401-0402		first link to next Basic line

0403-0404		first line number

0405-			tokenized basic line, terminated with 00

followed by next link

8000-83E7	32768-33767 40 column screen memory

8000-87EF	32768-34767 80 column screen memory

A000-AFFF	free space for 4K EPROM

B000-BFFF	3: free space for 4K EPROM

4         3

B000-DFFF C000-DFFF	Basic keywords and operators, and general processing

E000-EFFF E000-EFFF	Mostly screen editor functions

F000-FFFF F000-FFFF	Kernel: tape processing, IEEE-488, jump table.

-Olaf.

--

___ Olaf 'Rhialto' Seibert    rhialto@mbfys.kun.nl         What's the use of

\X/  racism if you can't even see if a person belongs to your abhorred kind?