m_vireneigenschaften

48

A virus can belong to one of the

following classes:

- STEALTH

The virus takes measures not to be

detected (like manipulation of the

DIR-command in such a way that an

increase in file size  after

infection will not be shown by the

DIR-command).

- ENCRYPTION

The virus code is not detectable

by fixed byte sequences, it keeps

encrypting itself.

- RESIDENT

The virus will stay resident in

memory after execution of its

program code. Thus it can repli-

cate and manipulate without

depending on any program being

started. Resident viruses are

often RESET-resistent.

- COMMAND_COM

The virus infects the

COMMAND.COM file. Since this

file is always loaded by the

operating system,

early activation of the virus

is guaranteed.

- COM_FILES

The virus infects COM-files.

- EXE_FILES

The virus infects EXE-files.

- OVERLAY_FILES

The virus infects overlay files.

- DISK_BOOT

The boot sector of the floppy disk

will be infected.

These viruses often install

in the main memory and from there

can multiply and manipulate

without being dependent on the

execution of a certain program.

- FIXED_DISK_BOOT

The boot sector on the hard disk

is infected.

- FIXED_DISK_PART

The partition table of the hard

disk is infected.

m_warmstart

5

- Since the removal of a TSR

virus from the memory during a

warm boot (reset of computer) is

not guaranteed, all TSR viruses

survive a warm boot in VIRLAB.

m_virusschutz

71

What to do if a virus has infected

your PC?

- switch your computer off, wait a

few seconds, switch it on again,

- insert a write-protected original

floppy disk containing the system

software into the disk drive and

boot

- check if the virus symptoms occur

again (in this case, the virus is

either resident or the boot

floppy disk has been infected)

- delete all the files you do not

need badly along with all those

files you don't know the origin of

from the hard disk. For security,

copy the remaining files on

newly formatted floppy disks,

- format the hard disk (using an

original system floppy disk),

- install the system from the

original floppy disk

- if possible, install an anti-

virus program with which you can

supervise any file access and

other activities

- copy only those programs from

original floppy disks which you

really need.

How to prevent viral infections:

- make backup copies of original

floppy disks as early as

possible; thereafter, work only

with these

- save a hard-copy of the contents

of important directories. This may

allow you to tell whether a file

has been infected by comparing

file sizes

- refrain from using unlicenced

software, be careful when using

floppy disks other than your own

- be especially careful with game

programs! Very often they are

infected.

- check both hard disk and floppy

disks regularly by using

anti-virus programs.

- run backups regularly.

- Use original floppy disks only

with write protection whenever

possible

Possible symptoms of a viral

infection:

- increase of file size of program

files

- change of date or time of program

files

- unknown files are added or files

are deleted

- files are undergoing changes

without the user having any

control over it,

- more storage space is needed,

- slow-down of program loading or

execution

- unexpected screen effects

- unexpected disk access