Contens

1.            Instructions   . . . . . . . . . . . . . . .  11

1.1.          Arithmetic Instructions    . . . . . . . . .  11

1.1.1.        ADD    . . . . . . . . . . . . . . . . . . .  12

1.1.2.        AddLong    . . . . . . . . . . . . . . . . .  12

1.1.3.        ADC    . . . . . . . . . . . . . . . . . . .  13

1.1.4.        INC    . . . . . . . . . . . . . . . . . . .  14

1.1.5.        SUB    . . . . . . . . . . . . . . . . . . .  14

1.1.6.        SBB    . . . . . . . . . . . . . . . . . . .  15

1.1.7.        SubLong    . . . . . . . . . . . . . . . . .  16

1.1.8.        DEC    . . . . . . . . . . . . . . . . . . .  16

1.1.9.        NEG    . . . . . . . . . . . . . . . . . . .  17

1.1.10.       IMUL   . . . . . . . . . . . . . . . . . . .  17

1.1.11.       MUL    . . . . . . . . . . . . . . . . . . .  18

1.1.12.       IDIV   . . . . . . . . . . . . . . . . . . .  19

1.1.13.       DIV    . . . . . . . . . . . . . . . . . . .  19

1.2.          BCD Adjust Instructions    . . . . . . . . .  20

1.2.1.        AAA    . . . . . . . . . . . . . . . . . . .  20

1.2.2.        AAS    . . . . . . . . . . . . . . . . . . .  21

1.2.3.        AAS    . . . . . . . . . . . . . . . . . . .  21

1.2.4.        AAM    . . . . . . . . . . . . . . . . . . .  21

1.2.5.        IntToAsc   . . . . . . . . . . . . . . . . .  21

1.2.6.        AAD    . . . . . . . . . . . . . . . . . . .  22

1.2.7.        DAA    . . . . . . . . . . . . . . . . . . .  22

1.2.8.        DAS    . . . . . . . . . . . . . . . . . . .  23

1.3.          Bit Shifting Instructions    . . . . . . . .  23

1.3.1.        RCL/RCR/ROL/ROR    . . . . . . . . . . . . .  24

1.3.2.        SAL/SAR/SHL/SHR    . . . . . . . . . . . . .  25

1.4.          Compare Instructions   . . . . . . . . . . .  26

1.4.1.        CMP    . . . . . . . . . . . . . . . . . . .  26

1.4.2.        SetLineMode    . . . . . . . . . . . . . . .  27

1.4.3.        CMPS/CMPSB/CMPSW   . . . . . . . . . . . . .  29

1.4.4.        StrCompare   . . . . . . . . . . . . . . . .  30

1.4.5.        TEST   . . . . . . . . . . . . . . . . . . .  30

1.5.          Control-Flow Instructions    . . . . . . . .  31

1.5.1.        JMP    . . . . . . . . . . . . . . . . . . .  32

1.5.2.        JUMP.ASM   . . . . . . . . . . . . . . . . .  33

1.5.3.        JCXZ/JECXZ   . . . . . . . . . . . . . . . .  34

1.5.4.        Jcondition   . . . . . . . . . . . . . . . .  34

1.5.5.        JCOND.ASM    . . . . . . . . . . . . . . . .  35

1.5.6.        LOOP   . . . . . . . . . . . . . . . . . . .  37

1.5.7.        WriteTTY   . . . . . . . . . . . . . . . . .  37

1.5.8.        LOOPcondition    . . . . . . . . . . . . . .  38

1.5.9.        CALL   . . . . . . . . . . . . . . . . . . .  38

1.5.10.       HELLOM.ASM   . . . . . . . . . . . . . . . .  39

1.5.11.       RET/RETN/RETF    . . . . . . . . . . . . . .  40

1.5.12.       INT    . . . . . . . . . . . . . . . . . . .  41

1.5.13.       IRET   . . . . . . . . . . . . . . . . . . .  42

1.5.14.       NewBreak   . . . . . . . . . . . . . . . . .  42

1.5.15.       ENTER    . . . . . . . . . . . . . . . . . .  43

1.5.16.       LEAVE    . . . . . . . . . . . . . . . . . .  43

1.6.          Data Transfer Instructions   . . . . . . . .  44

1.6.1.        MOV    . . . . . . . . . . . . . . . . . . .  44

1.6.2.        MOVS/MOVSB/MOVSW   . . . . . . . . . . . . .  45

1.6.3.        WinOpen    . . . . . . . . . . . . . . . . .  46

1.6.4.        XCHG   . . . . . . . . . . . . . . . . . . .  48

1.6.5.        IntToAsc   . . . . . . . . . . . . . . . . .  48

1.6.6.        LODS/LODSB/LODSW   . . . . . . . . . . . . .  49

1.6.7.        WinClose   . . . . . . . . . . . . . . . . .  49

1.6.8.        STOS/STOSB/STOSW   . . . . . . . . . . . . .  50

- 2 -

MASM 5.1

1.6.9.        LEA    . . . . . . . . . . . . . . . . . . .  51

1.6.10.       VeriAnsi   . . . . . . . . . . . . . . . . .  51

1.6.11.       LDS/LES    . . . . . . . . . . . . . . . . .  52

1.6.12.       LoadPtr    . . . . . . . . . . . . . . . . .  52

1.6.13.       XLAT/XLATB   . . . . . . . . . . . . . . . .  53

1.6.14.       BinToHex   . . . . . . . . . . . . . . . . .  54

1.7.          Flag-Setting Instructions    . . . . . . . .  54

1.7.1.        CLC    . . . . . . . . . . . . . . . . . . .  55

1.7.2.        CopyFile   . . . . . . . . . . . . . . . . .  55

1.7.3.        CLD    . . . . . . . . . . . . . . . . . . .  57

1.7.4.        StrFindChar    . . . . . . . . . . . . . . .  57

1.7.5.        CLI    . . . . . . . . . . . . . . . . . . .  58

1.7.6.        DisableCGA   . . . . . . . . . . . . . . . .  59

1.7.7.        CMC    . . . . . . . . . . . . . . . . . . .  59

1.7.8.        STC    . . . . . . . . . . . . . . . . . . .  60

1.7.9.        STD    . . . . . . . . . . . . . . . . . . .  60

1.7.10.       STI    . . . . . . . . . . . . . . . . . . .  60

1.7.11.       POPF   . . . . . . . . . . . . . . . . . . .  61

1.7.12.       PUSHF    . . . . . . . . . . . . . . . . . .  61

1.7.13.       FindFirst    . . . . . . . . . . . . . . . .  61

1.7.14.       LAHF   . . . . . . . . . . . . . . . . . . .  62

1.7.15.       SAHF   . . . . . . . . . . . . . . . . . . .  63

1.7.16.       Quadratic    . . . . . . . . . . . . . . . .  63

1.8.          Logic Instructions   . . . . . . . . . . . .  65

1.8.1.        AND    . . . . . . . . . . . . . . . . . . .  65

1.8.2.        Colors   . . . . . . . . . . . . . . . . . .  66

1.8.3.        OR   . . . . . . . . . . . . . . . . . . . .  67

1.8.4.        XOR    . . . . . . . . . . . . . . . . . . .  68

1.8.5.        NOT    . . . . . . . . . . . . . . . . . . .  69

1.9.          Port I/O Instructions    . . . . . . . . . .  70

1.9.1.        IN   . . . . . . . . . . . . . . . . . . . .  70

1.9.2.        Sound    . . . . . . . . . . . . . . . . . .  70

1.9.3.        INS/INSB/INSW    . . . . . . . . . . . . . .  71

1.9.4.        OUT    . . . . . . . . . . . . . . . . . . .  72

1.9.5.        OUTS/OUTSB/OUTSW   . . . . . . . . . . . . .  72

1.10.         Processor-Control Instructions   . . . . . .  73

1.10.1.       NOP    . . . . . . . . . . . . . . . . . . .  73

1.10.2.       ESC    . . . . . . . . . . . . . . . . . . .  73

1.10.3.       WAIT   . . . . . . . . . . . . . . . . . . .  74

1.10.4.       LOCK   . . . . . . . . . . . . . . . . . . .  74

1.10.5.       HLT    . . . . . . . . . . . . . . . . . . .  75

1.11.         Stack-Oriented Instructions    . . . . . . .  75

1.11.1.       PUSH   . . . . . . . . . . . . . . . . . . .  75

1.11.2.       GetMem   . . . . . . . . . . . . . . . . . .  76

1.11.3.       PUSHA    . . . . . . . . . . . . . . . . . .  77

1.11.4.       POP    . . . . . . . . . . . . . . . . . . .  77

1.11.5.       POPA   . . . . . . . . . . . . . . . . . . .  78

1.11.6.       PopAll   . . . . . . . . . . . . . . . . . .  78

1.12.         String Operations    . . . . . . . . . . . .  78

1.12.1.       MOVS/MOVSB/MOVSW   . . . . . . . . . . . . .  79

1.12.2.       LODS/LODSB/LODSW   . . . . . . . . . . . . .  80

1.12.3.       STOS/STOSB/STOSW   . . . . . . . . . . . . .  80

1.12.4.       SCAS/SCASB/SCASW   . . . . . . . . . . . . .  81

1.12.5.       CMPS/CMPSB/CMPSW   . . . . . . . . . . . . .  81

1.12.6.       REP    . . . . . . . . . . . . . . . . . . .  82

1.12.7.       REPcondition   . . . . . . . . . . . . . . .  83

1.13.         Type-Conversion Instructions   . . . . . . .  84

1.13.1.       CBW    . . . . . . . . . . . . . . . . . . .  84

1.13.2.       GetCurDisk   . . . . . . . . . . . . . . . .  85

1.13.3.       CWD    . . . . . . . . . . . . . . . . . . .  85

2.            Directive Summary    . . . . . . . . . . . .  86

2.1.          Conditional-Assembly Directives    . . . . .  86

- 3 -

MASM 5.1

2.1.1.        IF block syntax    . . . . . . . . . . . . .  87

2.1.2.        LoadPtr    . . . . . . . . . . . . . . . . .  87

2.1.3.        IF   . . . . . . . . . . . . . . . . . . . .  88

2.1.4.        IF1, IF2   . . . . . . . . . . . . . . . . .  88

2.1.5.        IFB    . . . . . . . . . . . . . . . . . . .  89

2.1.6.        IFDEF    . . . . . . . . . . . . . . . . . .  89

2.1.7.        IFDIF    . . . . . . . . . . . . . . . . . .  90

2.2.          Conditional-Error Directives   . . . . . . .  90

2.2.1.        .ERR   . . . . . . . . . . . . . . . . . . .  91

2.2.2.        .ERR1/.ERR2    . . . . . . . . . . . . . . .  91

2.2.3.        .ERRE/.ERRNZ   . . . . . . . . . . . . . . .  91

2.2.4.        .ERRB/.ERRNB   . . . . . . . . . . . . . . .  92

2.2.5.        .ERRDEF/.ERRNDEF   . . . . . . . . . . . . .  92

2.2.6.        .ERRDIF/.ERRIDN    . . . . . . . . . . . . .  92

2.3.          Code-Label Directives    . . . . . . . . . .  93

2.3.1.        PROC   . . . . . . . . . . . . . . . . . . .  93

2.3.2.        PUTSTR.ASM   . . . . . . . . . . . . . . . .  94

2.3.3.        ENDP   . . . . . . . . . . . . . . . . . . .  94

2.3.4.        LABEL    . . . . . . . . . . . . . . . . . .  95

2.3.5.        ALIGN    . . . . . . . . . . . . . . . . . .  95

2.3.6.        EVEN   . . . . . . . . . . . . . . . . . . .  95

2.3.7.        ORG    . . . . . . . . . . . . . . . . . . .  96

2.4.          Data-Allocation Directives   . . . . . . . .  96

2.4.1.        DB   . . . . . . . . . . . . . . . . . . . .  96

2.4.2.        data.asm   . . . . . . . . . . . . . . . . .  97

2.4.3.        DW   . . . . . . . . . . . . . . . . . . . .  98

2.4.4.        DD   . . . . . . . . . . . . . . . . . . . .  98

2.4.5.        DQ   . . . . . . . . . . . . . . . . . . . .  98

2.4.6.        DT   . . . . . . . . . . . . . . . . . . . .  99

2.5.          Listing-File Directives    . . . . . . . . .  99

2.5.1.        .LIST    . . . . . . . . . . . . . . . . . . 100

2.5.2.        .XLIST   . . . . . . . . . . . . . . . . . . 100

2.5.3.        .LFCOND    . . . . . . . . . . . . . . . . . 100

2.5.4.        .SFCOND    . . . . . . . . . . . . . . . . . 100

2.5.5.        .TFCOND    . . . . . . . . . . . . . . . . . 101

2.5.6.        .LALL    . . . . . . . . . . . . . . . . . . 101

2.5.7.        .SALL    . . . . . . . . . . . . . . . . . . 101

2.5.8.        .XALL    . . . . . . . . . . . . . . . . . . 101

2.5.9.        TITLE    . . . . . . . . . . . . . . . . . . 102

2.5.10.       SUBTTL   . . . . . . . . . . . . . . . . . . 102

2.5.11.       PAGE   . . . . . . . . . . . . . . . . . . . 102

2.6.          Macro    . . . . . . . . . . . . . . . . . . 103

2.6.1.        MACRO    . . . . . . . . . . . . . . . . . . 103

2.6.2.        ENDM   . . . . . . . . . . . . . . . . . . . 103

2.6.3.        EXITM    . . . . . . . . . . . . . . . . . . 104

2.6.4.        LOCAL (in macros)    . . . . . . . . . . . . 104

2.6.5.        PURGE    . . . . . . . . . . . . . . . . . . 104

2.6.6.        REPT   . . . . . . . . . . . . . . . . . . . 105

2.6.7.        IRP    . . . . . . . . . . . . . . . . . . . 105

2.6.8.        IRPC   . . . . . . . . . . . . . . . . . . . 105

2.7.          Miscellaneous Directives   . . . . . . . . . 106

2.7.1.        COMMENT    . . . . . . . . . . . . . . . . . 106

2.7.2.        END    . . . . . . . . . . . . . . . . . . . 107

2.7.3.        INCLUDE    . . . . . . . . . . . . . . . . . 107

2.7.4.        INCLUDELIB   . . . . . . . . . . . . . . . . 107

2.7.5.        %OUT Directive   . . . . . . . . . . . . . . 107

2.7.6.        .RADIX   . . . . . . . . . . . . . . . . . . 108

2.8.          Multimodule Directives   . . . . . . . . . . 108

2.8.1.        PUBLIC   . . . . . . . . . . . . . . . . . . 109

2.8.2.        EXTRN    . . . . . . . . . . . . . . . . . . 109

2.8.3.        COMM   . . . . . . . . . . . . . . . . . . . 109

2.8.4.        INCLUDELIB   . . . . . . . . . . . . . . . . 110

- 4 -

MASM 5.1

2.9.          Processor Directives   . . . . . . . . . . . 110

2.9.1.        .186   . . . . . . . . . . . . . . . . . . . 111

2.9.2.        .286   . . . . . . . . . . . . . . . . . . . 111

2.9.3.        .287   . . . . . . . . . . . . . . . . . . . 111

2.9.4.        .8086    . . . . . . . . . . . . . . . . . . 111

2.9.5.        .8087    . . . . . . . . . . . . . . . . . . 112

2.10.         Segment Directives   . . . . . . . . . . . . 112

2.10.1.       SEGMENT    . . . . . . . . . . . . . . . . . 112

2.10.2.       HELLOF.ASM   . . . . . . . . . . . . . . . . 113

2.10.3.       ENDS   . . . . . . . . . . . . . . . . . . . 114

2.10.4.       GROUP    . . . . . . . . . . . . . . . . . . 114

2.10.5.       ASSUME   . . . . . . . . . . . . . . . . . . 114

2.10.6.       HELLOC.ASM   . . . . . . . . . . . . . . . . 115

2.10.7.       DOSSEG   . . . . . . . . . . . . . . . . . . 115

2.10.8.       HELLO.ASM    . . . . . . . . . . . . . . . . 115

2.10.9.       .ALPHA   . . . . . . . . . . . . . . . . . . 116

2.10.10.      .SEQ   . . . . . . . . . . . . . . . . . . . 116

2.11.         Simplified Segment Directives    . . . . . . 117

2.11.1.       .MODEL   . . . . . . . . . . . . . . . . . . 117

2.11.2.       .STARTUP   . . . . . . . . . . . . . . . . . 118

2.11.3.       EXIT   . . . . . . . . . . . . . . . . . . . 118

2.11.4.       .CODE    . . . . . . . . . . . . . . . . . . 118

2.11.5.       .STACK   . . . . . . . . . . . . . . . . . . 119

2.11.6.       .DATA    . . . . . . . . . . . . . . . . . . 119

2.11.7.       .DATA?   . . . . . . . . . . . . . . . . . . 120

2.11.8.       .CONST   . . . . . . . . . . . . . . . . . . 120

2.11.9.       .FARDATA   . . . . . . . . . . . . . . . . . 120

2.11.10.      .FARDATA?    . . . . . . . . . . . . . . . . 121

2.12.         Record and Structure Directives    . . . . . 121

2.12.1.       RECORD   . . . . . . . . . . . . . . . . . . 121

2.12.2.       STRUC    . . . . . . . . . . . . . . . . . . 122

2.12.3.       ENDS   . . . . . . . . . . . . . . . . . . . 122

3.            Operator Summary   . . . . . . . . . . . . . 123

3.1.          Arithmetic Operator Summary    . . . . . . . 123

3.1.1.        + (Plus, Binary)   . . . . . . . . . . . . . 123

3.1.2.        + (Plus, Unary)    . . . . . . . . . . . . . 124

3.1.3.        - (Minus, Binary)    . . . . . . . . . . . . 124

3.1.4.        - (Minus, Unary)   . . . . . . . . . . . . . 124

3.1.5.        * (Multiplication)   . . . . . . . . . . . . 125

3.1.6.        / (Division)   . . . . . . . . . . . . . . . 125

3.1.7.        MOD    . . . . . . . . . . . . . . . . . . . 125

3.1.8.        . (Structure-Field Reference)    . . . . . . 126

3.1.9.        [] (Index Operator)    . . . . . . . . . . . 126

3.2.          Macro Operator Summary   . . . . . . . . . . 126

3.2.1.        <> (Literal Text Operator)   . . . . . . . . 127

3.2.2.        ! (Literal Character Operator)   . . . . . . 127

3.2.3.        ;; (Macro Comment Delimeter)   . . . . . . . 127

3.2.4.        % (Expression Operator)    . . . . . . . . . 128

3.2.5.        & (Substitute Operator)    . . . . . . . . . 128

3.3.          Logic/Shift Operator Summary   . . . . . . . 128

3.3.1.        AND    . . . . . . . . . . . . . . . . . . . 129

3.3.2.        OR   . . . . . . . . . . . . . . . . . . . . 129

3.3.3.        XOR    . . . . . . . . . . . . . . . . . . . 130

3.3.4.        NOT    . . . . . . . . . . . . . . . . . . . 131

3.3.5.        SAL/SAR/SHL/SHR    . . . . . . . . . . . . . 131

3.4.          Record Operator Summary    . . . . . . . . . 133

3.4.1.        MASK   . . . . . . . . . . . . . . . . . . . 133

3.4.2.        WIDTH    . . . . . . . . . . . . . . . . . . 134

3.5.          Type Operator Summary    . . . . . . . . . . 134

3.5.1.        HIGH   . . . . . . . . . . . . . . . . . . . 134

3.5.2.        LOW    . . . . . . . . . . . . . . . . . . . 135

3.5.3.        PTR    . . . . . . . . . . . . . . . . . . . 135

MASM 5.1

3.5.4.        SHORT    . . . . . . . . . . . . . . . . . . 135

3.5.5.        SIZE   . . . . . . . . . . . . . . . . . . . 136

3.5.6.        THIS   . . . . . . . . . . . . . . . . . . . 136

3.5.7.        TYPE   . . . . . . . . . . . . . . . . . . . 137

3.5.8.        .TYPE    . . . . . . . . . . . . . . . . . . 137

3.6.          Address Operator Summary   . . . . . . . . . 138

3.6.1.        SEG    . . . . . . . . . . . . . . . . . . . 138

3.6.2.        OFFSET   . . . . . . . . . . . . . . . . . . 138

3.7.          Relational Operator Summary    . . . . . . . 139

3.7.1.        EQ   . . . . . . . . . . . . . . . . . . . . 139

3.7.2.        NE   . . . . . . . . . . . . . . . . . . . . 139

3.7.3.        GT   . . . . . . . . . . . . . . . . . . . . 140

3.7.4.        GE   . . . . . . . . . . . . . . . . . . . . 140

3.7.5.        LT   . . . . . . . . . . . . . . . . . . . . 140

3.7.6.        LE   . . . . . . . . . . . . . . . . . . . . 141

3.8.          Miscellaneous Operator Summary   . . . . . . 141

3.8.1.        ; (Comment Delimeter)    . . . . . . . . . . 141

3.8.2.        DUP    . . . . . . . . . . . . . . . . . . . 142

3.8.3.        ? (Uninitialized Value)    . . . . . . . . . 142

3.8.4.        \ (Line Continutation)   . . . . . . . . . . 142

4.            Predefined Equates   . . . . . . . . . . . . 143

4.1.          @code    . . . . . . . . . . . . . . . . . . 143

4.2.          Vector   . . . . . . . . . . . . . . . . . . 143

4.3.          @CurSeg    . . . . . . . . . . . . . . . . . 144

4.4.          @FileName    . . . . . . . . . . . . . . . . 144

4.5.          @WordSize    . . . . . . . . . . . . . . . . 145

4.6.          @CodeSize    . . . . . . . . . . . . . . . . 145

4.7.          @data    . . . . . . . . . . . . . . . . . . 145

4.8.          @Model   . . . . . . . . . . . . . . . . . . 145

4.9.          @Cpu   . . . . . . . . . . . . . . . . . . . 146

4.10.         @DataSize    . . . . . . . . . . . . . . . . 146

4.11.         @Version   . . . . . . . . . . . . . . . . . 146

5.            Specifying Radixes   . . . . . . . . . . . . 147

6.            Useful Tables    . . . . . . . . . . . . . . 148

6.1.          Regular Expressions    . . . . . . . . . . . 148

6.2.          Data Types   . . . . . . . . . . . . . . . . 149

6.3.          Register Summary   . . . . . . . . . . . . . 149

6.4.          Flags Summary    . . . . . . . . . . . . . . 149

6.5.          Register Setup   . . . . . . . . . . . . . . 150

6.6.          Keyboard Scan Codes    . . . . . . . . . . . 150

6.7.          Program Segment Prefix   . . . . . . . . . . 151

6.8.          Color Display Attributes   . . . . . . . . . 151

7.            Interrupt 21H Summary (DOS)    . . . . . . . 153

7.1.          Interrupt 21H, Character-Oriented Functions  153

7.1.1.        Int 21H Function 01H   . . . . . . . . . . . 153

7.1.2.        Int 21H Function 02H   . . . . . . . . . . . 154

7.1.3.        PUTSTR.ASM   . . . . . . . . . . . . . . . . 155

7.1.4.        Int 21H Function 03H   . . . . . . . . . . . 156

7.1.5.        Int 21H Function 04H   . . . . . . . . . . . 156

7.1.6.        Int 21H Function 05H   . . . . . . . . . . . 157

7.1.7.        Int 21H Function 06H   . . . . . . . . . . . 157

7.1.8.        Int 21H Function 07H   . . . . . . . . . . . 158

7.1.9.        Int 21H Function 08H   . . . . . . . . . . . 159

7.1.10.       Int 21H Function 09H   . . . . . . . . . . . 159

7.1.11.       Int 21H Function 0AH   . . . . . . . . . . . 160

7.1.12.       Int 21H Function 0BH   . . . . . . . . . . . 161

7.1.13.       Int 21H Function 0CH   . . . . . . . . . . . 161

7.2.          Interrupt 21H, Directory-Control Functions   162

7.2.1.        Int 21H Function 39H   . . . . . . . . . . . 162

7.2.2.        Int 21H Function 3AH   . . . . . . . . . . . 163

7.2.3.        Int 21H Function 3BH   . . . . . . . . . . . 163

7.2.4.        Int 21H Function 47H   . . . . . . . . . . . 164

MASM 5.1

7.3.          Interrupt 21H, Disk-Management Functions   . 164

7.3.1.        Int 21H Function 0DH   . . . . . . . . . . . 165

7.3.2.        Int 21H Function 0EH   . . . . . . . . . . . 165

7.3.3.        Int 21H Function 19H   . . . . . . . . . . . 166

7.3.4.        Int 21H Function 1BH   . . . . . . . . . . . 166

7.3.5.        Int 21H Function 1CH   . . . . . . . . . . . 167

7.3.6.        Int 21H Function 2EH   . . . . . . . . . . . 168

7.3.7.        Int 21H Function 36H   . . . . . . . . . . . 168

7.3.8.        Int 21H Function 54H   . . . . . . . . . . . 169

7.4.          Interrupt 21H, File Operations   . . . . . . 169

7.4.1.        Int 21H Function 3CH   . . . . . . . . . . . 170

7.4.2.        CreateFile   . . . . . . . . . . . . . . . . 170

7.4.3.        Int 21H Function 3DH   . . . . . . . . . . . 171

7.4.4.        Int 21H Function 3EH   . . . . . . . . . . . 171

7.4.5.        Int 21H Function 41H   . . . . . . . . . . . 172

7.4.6.        Int 21H Function 43H   . . . . . . . . . . . 173

7.4.7.        Int 21H Function 45H   . . . . . . . . . . . 173

7.4.8.        Int 21H Function 45H   . . . . . . . . . . . 174

7.4.9.        Int 21H Function 46H   . . . . . . . . . . . 174

7.4.10.       Int 21H Function 4EH   . . . . . . . . . . . 175

7.4.11.       Int 21H Function 4FH   . . . . . . . . . . . 176

7.4.12.       FindNext   . . . . . . . . . . . . . . . . . 177

7.4.13.       Int 21H Function 56H   . . . . . . . . . . . 178

7.4.14.       Int 21H Function 57H   . . . . . . . . . . . 179

7.4.15.       Int 21H Function 5AH   . . . . . . . . . . . 179

7.4.16.       Int 21H Function 5BH   . . . . . . . . . . . 180

7.4.17.       CreateNewFile    . . . . . . . . . . . . . . 181

7.4.18.       Int 21H Function 67H   . . . . . . . . . . . 182

7.5.          Interrupt 21H, File Operations, FCB    . . . 182

7.5.1.        Int 21H Function 0FH   . . . . . . . . . . . 183

7.5.2.        Int 21H Function 10H   . . . . . . . . . . . 183

7.5.3.        Int 21H Function 11H   . . . . . . . . . . . 184

7.5.4.        Int 21H Function 12H   . . . . . . . . . . . 185

7.5.5.        Int 21H Function 13H   . . . . . . . . . . . 185

7.5.6.        Int 21H Function 16H   . . . . . . . . . . . 186

7.5.7.        Int 21H Function 17H   . . . . . . . . . . . 187

7.5.8.        Int 21H Function 23H   . . . . . . . . . . . 187

7.5.9.        Int 21H Function 29H   . . . . . . . . . . . 188

7.6.          Interrupt 21H, Memory-Allocation Functions   189

7.6.1.        Int 21H Function 48H   . . . . . . . . . . . 190

7.6.2.        Int 21H Function 49H   . . . . . . . . . . . 190

7.6.3.        Int 21H Function 4AH   . . . . . . . . . . . 191

7.6.4.        Int 21H Function 58H   . . . . . . . . . . . 191

7.7.          Interrupt 21H, Miscellaneous System

Functions    . . . . . . . . . . . . . . . . 192

7.7.1.        Int 21H Function 25H   . . . . . . . . . . . 192

7.7.2.        Exec   . . . . . . . . . . . . . . . . . . . 193

7.7.3.        Int 21H Function 30H   . . . . . . . . . . . 195

7.7.4.        Int 21H Function 33H   . . . . . . . . . . . 195

7.7.5.        Int 21H Function 35H   . . . . . . . . . . . 196

7.7.6.        Int 21H Function 38H   . . . . . . . . . . . 196

7.7.7.        Int 21H Function 44H   . . . . . . . . . . . 198

7.7.8.        Int 21H Function 59H   . . . . . . . . . . . 199

7.7.9.        Interrupt 21H, Function 5EH

(Machine/Printer)    . . . . . . . . . . . . 201

7.7.9.1.      Int 21H Function 5EH Subfunction 00H   . . . 201

7.7.9.2.      Int 21H Function 5EH Subfunction 02H   . . . 201

7.7.9.3.      Int 21H Function 5EH Subfunction 03H   . . . 202

7.7.10.       Interrupt 21H, Function 5FH (Device

Redirection)   . . . . . . . . . . . . . . . 203

7.7.10.1.     Int 21H Function 5FH Subfunction 02H   . . . 203

7.7.10.2.     Int 21H Function 5FH Subfunction 03H   . . . 204

MASM 5.1

7.7.10.3.     Int 21H Function 5FH Subfunction 04H   . . . 204

7.7.11.       Int 21H Function 63H   . . . . . . . . . . . 205

7.7.12.       Int 21H Function 65H   . . . . . . . . . . . 206

7.7.13.       Int 21H Function 66H   . . . . . . . . . . . 208

7.8.          Interrupt 21H, Process-Control Functions   . 209

7.8.1.        Int 21H Function 00H   . . . . . . . . . . . 209

7.8.2.        Int 21H Function 26H   . . . . . . . . . . . 210

7.8.3.        Int 21H Function 31H   . . . . . . . . . . . 210

7.8.4.        Int 21H Function 4BH   . . . . . . . . . . . 212

7.8.5.        Int 21H Function 4CH   . . . . . . . . . . . 213

7.8.6.        Int 21H Function 4DH   . . . . . . . . . . . 214

7.8.7.        Int 21H Function 62H   . . . . . . . . . . . 215

7.9.          Interrupt 21H, Record Functions    . . . . . 215

7.9.1.        Int 21H Function 1AH   . . . . . . . . . . . 215

7.9.2.        SetDTA   . . . . . . . . . . . . . . . . . . 216

7.9.3.        Int 21H Function 2FH   . . . . . . . . . . . 216

7.9.4.        Int 21H Function 3FH   . . . . . . . . . . . 217

7.9.5.        ReadFile   . . . . . . . . . . . . . . . . . 217

7.9.6.        Int 21H Function 40H   . . . . . . . . . . . 218

7.9.7.        Int 21H Function 42H   . . . . . . . . . . . 219

7.9.8.        Int 21H Function 5CH   . . . . . . . . . . . 219

7.9.9.        Int 21H Function 5CH   . . . . . . . . . . . 220

7.9.10.       Int 21H Function 68H   . . . . . . . . . . . 221

7.10.         Interrupt 21H, Record Functions, FCB   . . . 222

7.10.1.       Int 21H Function 14H   . . . . . . . . . . . 222

7.10.2.       Int 21H Function 15H   . . . . . . . . . . . 223

7.10.3.       Int 21H Function 21H   . . . . . . . . . . . 224

7.10.4.       Int 21H Function 22H   . . . . . . . . . . . 224

7.10.5.       Int 21H Function 24H   . . . . . . . . . . . 225

7.10.6.       Int 21H Function 24H   . . . . . . . . . . . 226

7.10.7.       Int 21H Function 27H   . . . . . . . . . . . 226

7.10.8.       Int 21H Function 28H   . . . . . . . . . . . 227

7.11.         Interrupt 21H, Time and Date Functions   . . 228

7.11.1.       Int 21H Function 2AH   . . . . . . . . . . . 228

7.11.2.       Int 21H Function 2BH   . . . . . . . . . . . 229

7.11.3.       Int 21H Function 2CH   . . . . . . . . . . . 229

7.11.4.       Int 21H Function 2DH   . . . . . . . . . . . 229

8.            Miscellaneous DOS Interrupts   . . . . . . . 231

8.1.          Interrupt 20H    . . . . . . . . . . . . . . 231

8.2.          Interrupt 25H    . . . . . . . . . . . . . . 232

8.3.          Interrupt 26H    . . . . . . . . . . . . . . 233

8.4.          Interrupt 27H    . . . . . . . . . . . . . . 234

8.5.          Interrupt 2FH    . . . . . . . . . . . . . . 235

9.            BIOS Interrupts    . . . . . . . . . . . . . 236

9.1.          Interrupt 10H (All Monitors)   . . . . . . . 236

9.1.1.        Int 10H Function 00H   . . . . . . . . . . . 236

9.1.2.        GetVidConfig   . . . . . . . . . . . . . . . 237

9.1.3.        Int 10H Function 01H   . . . . . . . . . . . 239

9.1.4.        SetCurSize   . . . . . . . . . . . . . . . . 240

9.1.5.        Int 10H Function 02H   . . . . . . . . . . . 240

9.1.6.        SetCurPos    . . . . . . . . . . . . . . . . 241

9.1.7.        Int 10H Function 03H   . . . . . . . . . . . 242

9.1.8.        Int 10H Function 04H   . . . . . . . . . . . 242

9.1.9.        Int 10H Function 05H   . . . . . . . . . . . 243

9.1.10.       Int 10H Function 06H   . . . . . . . . . . . 243

9.1.11.       Int 10H Function 07H   . . . . . . . . . . . 244

9.1.12.       Int 10H Function 08H   . . . . . . . . . . . 244

9.1.13.       ReadCharAttr   . . . . . . . . . . . . . . . 245

9.1.14.       Int 10H Function 09H   . . . . . . . . . . . 245

9.1.15.       Int 10H Function 0AH   . . . . . . . . . . . 246

9.1.16.       Int 10H Function 0EH   . . . . . . . . . . . 247

9.1.17.       Int 10H Function 0FH   . . . . . . . . . . . 247

MASM 5.1

9.1.18.       Int 10H Function 13H   . . . . . . . . . . . 248

9.2.          Interrupt 10H (Graphics Monitors)    . . . . 249

9.2.1.        Int 10H Function 0BH   . . . . . . . . . . . 249

9.2.2.        Int 10H Function 0CH   . . . . . . . . . . . 250

9.2.3.        Int 10H Function 0DH   . . . . . . . . . . . 250

9.2.4.        Int 10H Function 1AH   . . . . . . . . . . . 251

9.2.5.        Int 10H Function 1BH   . . . . . . . . . . . 251

9.2.6.        Int 10H Function 1CH   . . . . . . . . . . . 255

9.3.          Interrupt 10H, Function 10H (Colors)   . . . 256

9.3.1.        Int 10H Function 10H Subfunction 00H   . . . 256

9.3.2.        Int 10H Function 10H Subfunction 01H   . . . 257

9.3.3.        Int 10H Function 10H Subfunction 02H   . . . 257

9.3.4.        Int 10H Function 10H Subfunction 03H   . . . 258

9.3.5.        Int 10H Function 10H Subfunction 07H   . . . 258

9.3.6.        Int 10H Function 10H Subfunction 08H   . . . 259

9.3.7.        Int 10H Function 10H Subfunction 09H   . . . 259

9.3.8.        Int 10H Function 10H Subfunction 10H   . . . 259

9.3.9.        Int 10H Function 10H Subfunction 10H   . . . 260

9.3.10.       Int 10H Function 10H Subfunction 12H   . . . 260

9.3.11.       Int 10H Function 10H Subfunction 13H   . . . 260

9.3.12.       Int 10H Function 10H Subfunction 15H   . . . 261

9.3.13.       Int 10H Function 10H Subfunction 17H   . . . 261

9.3.14.       Int 10H Function 10H Subfunction 1AH   . . . 262

9.3.15.       Int 10H Function 10H Subfunction 1BH   . . . 262

9.4.          Interrupt 10H, Function 11H (Fonts)    . . . 262

9.4.1.        Int 10H Function 11H Subfunction 00H/10H   . 263

9.4.2.        SetLineMode    . . . . . . . . . . . . . . . 264

9.4.3.        Int 10H Function 11H Subfunction 01H/11H   . 265

9.4.4.        Int 10H Function 11H Subfunction 02H/12H   . 266

9.4.5.        Int 10H Function 11H Subfunction 03H   . . . 267

9.4.6.        Int 10H Function 11H Subfunction 04H   . . . 267

9.4.7.        Int 10H Function 11H Subfunction 14H   . . . 268

9.4.8.        Int 10H Function 11H Subfunction 20H   . . . 269

9.4.9.        Int 10H Function 11H Subfunction 21H   . . . 269

9.4.10.       Int 10H Function 11H Subfunction 22H   . . . 270

9.4.11.       Int 10H Function 11H Subfunction 23H   . . . 270

9.4.12.       Int 10H Function 11H Subfunction 24H   . . . 271

9.4.13.       Int 10H Function 11H Subfunction 30H   . . . 271

9.5.          Interrupt 10H, Function 12H (Video Config.)  272

9.5.1.        Int 10H Function 12H Subfunction 10H   . . . 272

9.5.2.        Int 10H Function 12H Subfunction 20H   . . . 273

9.5.3.        Int 10H Function 12H Subfunction 30H   . . . 273

9.5.4.        Int 10H Function 12H Subfunction 31H   . . . 273

9.5.5.        Int 10H Function 12H Subfunction 32H   . . . 274

9.5.6.        Int 10H Function 12H Subfunction 33H   . . . 274

9.5.7.        Int 10H Function 12H Subfunction 34H   . . . 274

9.5.8.        Int 10H Function 12H Subfunction 35H   . . . 275

9.5.9.        Int 10H Function 12H Subfunction 36H   . . . 275

9.6.          Function 13H (Disk I/O)    . . . . . . . . . 276

9.6.1.        Int 13H Function 00H   . . . . . . . . . . . 277

9.6.2.        Int 13H Function 01H   . . . . . . . . . . . 277

9.6.3.        Int 13H Function 02H   . . . . . . . . . . . 278

9.6.4.        Int 13H Function 03H   . . . . . . . . . . . 279

9.6.5.        Int 13H Function 04H   . . . . . . . . . . . 279

9.6.6.        Int 13H Function 05H   . . . . . . . . . . . 280

9.6.7.        Int 13H Function 06H   . . . . . . . . . . . 281

9.6.8.        Int 13H Function 07H   . . . . . . . . . . . 281

9.6.9.        Int 13H Function 08H   . . . . . . . . . . . 282

9.6.10.       Int 13H Function 09H   . . . . . . . . . . . 282

9.6.11.       Int 13H Function 0AH   . . . . . . . . . . . 284

9.6.12.       Int 13H Function 0BH   . . . . . . . . . . . 284

9.6.13.       Int 13H Function 0CH   . . . . . . . . . . . 285

MASM 5.1

9.6.14.       Int 13H Function 0DH   . . . . . . . . . . . 285

9.6.15.       Int 13H Function 0EH   . . . . . . . . . . . 285

9.6.16.       Int 13H Function 0FH   . . . . . . . . . . . 286

9.6.17.       Int 13H Function 10H   . . . . . . . . . . . 286

9.6.18.       Int 13H Function 11H   . . . . . . . . . . . 287

9.6.19.       Int 13H Function 12H   . . . . . . . . . . . 287

9.6.20.       Int 13H Function 13H   . . . . . . . . . . . 288

9.6.21.       Int 13H Function 14H   . . . . . . . . . . . 288

9.6.22.       Int 13H Function 15H   . . . . . . . . . . . 288

9.6.23.       Int 13H Function 16H   . . . . . . . . . . . 289

9.6.24.       Int 13H Function 17H   . . . . . . . . . . . 289

9.6.25.       Int 13H Function 18H   . . . . . . . . . . . 290

9.6.26.       Int 13H Function 19H   . . . . . . . . . . . 290

9.6.27.       Int 13H Function 1AH   . . . . . . . . . . . 291

9.7.          Interrupt 14H (COM Port)   . . . . . . . . . 292

9.7.1.        Int 14H Function 00H   . . . . . . . . . . . 292

9.7.2.        Int 14H Function 01H   . . . . . . . . . . . 293

9.7.3.        Int 14H Function 02H   . . . . . . . . . . . 294

9.7.4.        Int 14H Function 03H   . . . . . . . . . . . 294

9.7.5.        Int 14H Function 04H   . . . . . . . . . . . 295

9.7.6.        Int 14H Function 05H   . . . . . . . . . . . 296

9.8.          Interrupt 15H (I/O Subsystem)    . . . . . . 296

9.8.1.        Int 15H Function 00H   . . . . . . . . . . . 297

9.8.2.        Int 15H Function 01H   . . . . . . . . . . . 297

9.8.3.        Int 15H Function 02H   . . . . . . . . . . . 298

9.8.4.        Int 15H Function 03H   . . . . . . . . . . . 298

9.8.5.        Int 15H Function 0FH   . . . . . . . . . . . 299

9.8.6.        Int 15H Function 21H Subfunction 00H   . . . 299

9.8.7.        Int 15H Function 21H Subfunction 01H   . . . 300

9.8.8.        Int 15H Function 4FH   . . . . . . . . . . . 300

9.8.9.        Int 15H Function 80H   . . . . . . . . . . . 301

9.8.10.       Int 15H Function 81H   . . . . . . . . . . . 301

9.8.11.       Int 15H Function 82H   . . . . . . . . . . . 302

9.8.12.       Int 15H Function 83H   . . . . . . . . . . . 302

9.8.13.       Int 15H Function 84H   . . . . . . . . . . . 303

9.8.14.       Int 15H Function 85H   . . . . . . . . . . . 304

9.8.15.       Int 15H Function 86H   . . . . . . . . . . . 304

9.8.16.       Int 15H Function 87H   . . . . . . . . . . . 305

9.8.17.       Int 15H Function 88H   . . . . . . . . . . . 306

9.8.18.       Int 15H Function 89H   . . . . . . . . . . . 306

9.8.19.       Int 15H Function 90H   . . . . . . . . . . . 307

9.8.20.       Int 15H Function 91H   . . . . . . . . . . . 308

9.8.21.       Int 15H Function C0H   . . . . . . . . . . . 308

9.8.22.       Int 15H Function C1H   . . . . . . . . . . . 309

9.8.23.       Int 15H Function C3H   . . . . . . . . . . . 310

9.8.24.       Int 15H Function C4H   . . . . . . . . . . . 310

9.8.25.       Interrupt 15H (Mouse Pointer)    . . . . . . 311

9.8.25.1.     Int 15H Function C2H Subfunction 00H   . . . 311

9.8.25.2.     Int 15H Function C2H Subfunction 01H   . . . 312

9.8.25.3.     Int 15H Function C2H Subfunction 02H   . . . 312

9.8.25.4.     Int 15H Function C2H Subfunction 03H   . . . 313

9.8.25.5.     Int 15H Function C2H Subfunction 04H   . . . 313

9.8.25.6.     Int 15H Function C2H Subfunction 05H   . . . 313

9.8.25.7.     Int 15H Function C2H Subfunction 06H   . . . 314

9.8.25.8.     Int 15H Function C2H Subfunction 07H   . . . 315

9.9.          Interrupt 16H (Keyboard)   . . . . . . . . . 316

9.9.1.        Int 16H Function 00H   . . . . . . . . . . . 316

9.9.2.        GetKeyClock    . . . . . . . . . . . . . . . 316

9.9.3.        Int 16H Function 01H   . . . . . . . . . . . 318

9.9.4.        Int 16H Function 02H   . . . . . . . . . . . 318

9.9.5.        Int 16H Function 03H   . . . . . . . . . . . 319

9.9.6.        Int 16H Function 04H   . . . . . . . . . . . 320

MASM 5.1

9.9.7.        Int 16H Function 05H   . . . . . . . . . . . 320

9.9.8.        Int 16H Function 10H   . . . . . . . . . . . 321

9.9.9.        Int 16H Function 11H   . . . . . . . . . . . 321

9.9.10.       Int 16H Function 12H   . . . . . . . . . . . 321

9.10.         Interrupt 17H (Parallel Printer Port)    . . 322

9.10.1.       Int 17H Function 00H   . . . . . . . . . . . 322

9.10.2.       Int 17H Function 01H   . . . . . . . . . . . 323

9.10.3.       Int 17H Function 02H   . . . . . . . . . . . 323

9.10.4.       VeriPrint    . . . . . . . . . . . . . . . . 323

9.11.         Interrupt 1AH (Clock)    . . . . . . . . . . 324

9.11.1.       Int 1AH Function 00H   . . . . . . . . . . . 324

9.11.2.       Pause    . . . . . . . . . . . . . . . . . . 325

9.11.3.       Int 1AH Function 01H   . . . . . . . . . . . 326

9.11.4.       Int 1AH Function 02H   . . . . . . . . . . . 326

9.11.5.       Int 1AH Function 03H   . . . . . . . . . . . 326

9.11.6.       Int 1AH Function 04H   . . . . . . . . . . . 327

9.11.7.       Int 1AH Function 05H   . . . . . . . . . . . 327

9.11.8.       Int 1AH Function 06H   . . . . . . . . . . . 328

9.11.9.       Int 1AH Function 07H   . . . . . . . . . . . 328

9.11.10.      Int 1AH Function 0AH   . . . . . . . . . . . 328

9.11.11.      Int 1AH Function 0BH   . . . . . . . . . . . 329

9.11.12.      Int 1AH Function 80H   . . . . . . . . . . . 329

9.12.         Int 11H    . . . . . . . . . . . . . . . . . 329

9.13.         VeriCop    . . . . . . . . . . . . . . . . . 330

9.14.         Int 12H    . . . . . . . . . . . . . . . . . 331

9.15.         GetMem   . . . . . . . . . . . . . . . . . . 331

9.16.         Int 18H    . . . . . . . . . . . . . . . . . 332

9.17.         Int 19H    . . . . . . . . . . . . . . . . . 332

I n d e x    . . . . . . . . . . . . . . . . . . . . . . . 333

Instructions by Topic

1. Instructions

Category

Description

Arithmetic

Arithmetic calculations: add, subtract, etc.

BCD Conversion

Adjust result of Binary Coded Decimal operation

Bit Shifting

Shift or rotate bits left or right

Compare

Compare or test, for upcoming conditional jump

Control Flow

Jump or execute procedure

Data Transfer

Load or store values in registers

Flags

Clear or set processor flags

Logic

Logical operations: AND, OR, etc.

Port Input/Output

Read or write data to hardware ports

Processor Control

Control basic operation of the processor

Stack Operations

Push or pop values from stack

String Operations

Perform block operation (move, store, load)

Type Conversion

Cast data to new size while preserving sign

1.1. Arithmetic Instructions

Mnemonic

Name

ADD

Add

ADC

Add with Carry (add values in multiple registers)

INC

Increment (add 1)

SUB

Subtract

SBB

Subtract with Borrow (subtract values  in  multiple

registers)

DEC

Decrement (subtract 1)

NEG

Two's Complement Negation

IMUL

Signed Multiply

MUL

Unsigned Multiply

IDIV

Signed Division

DIV

Unsigned Division

Arithmetic Instructions

1.1.1. ADD

ADD                                 Flags:  O D I T S Z A P C

-----------------

Syntax:  ADD dest,src                       *       * * * * *

Adds the source and destination operands and puts the sum in

the destination operand.

000000dw     mod,reg,r/m       disp (0 or 2)

-------------------+---------------------------+---------------

ADD  reg,reg      | add   ax,bx               | 88/86 3

|                           | 286   2

|                           | 386   2

-------------------+---------------------------+---------------

ADD  mem,reg      | add   total,cx            | 88/86 16+EA

|                           | (W88=24+EA)

| add   array[bx+di],dx     | 286   7

|                           | 386   7

-------------------+---------------------------+---------------

ADD  reg,mem      | add   cx,incr             | 88/86 9+EA

|                           | (W88=13+EA)

| add   dx,[bp+6]           | 286   7

|                           | 386   6

-------------------+---------------------------+---------------

100000sw     mod,000,r/m       disp (0 or 2)     data (1 or 2)

-------------------+---------------------------+---------------

ADD  reg,immed    | add   bx,6                | 88/86 4

|                           | 286   3

|                           | 386   2

-------------------+---------------------------+---------------

ADD  mem,immed    | add   amount,27           | 88/86 17+EA

|                           | (W88=23+EA)

| add   pointers[bx][si],6  | 286   7

|                           | 386   7

-------------------+---------------------------+---------------

0000010w     data (1 or 2)

-------------------+---------------------------+---------------

ADD  accum,immed  | add   ax,10               | 88/86 4

|                           | 286   3

|                           | 386   2

-------------------+---------------------------+---------------

1.1.2. AddLong

;* AddLong - Adds two double-word (long) integers.

;*

;* Shows:   Instructions - add     adc

;*

;* Params:  long1 - First integer

;*          long2 - Second integer

;*

;* Return:  Sum as long integer

AddLong PROC \

long1:DWORD, long2:DWORD

mov     ax, WORD PTR long1[0] ; AX = low word, long1

Arithmetic Instructions

mov     dx, WORD PTR long1[2] ; DX = high word, long1

add     ax, WORD PTR long2[0] ; Add low word, long2

adc     dx, WORD PTR long2[2] ; Add high word, long2

ret                           ; Result returned as DX:A

AddLong ENDP

1.1.3. ADC

Add with Carry                       Flags:  O D I T S Z A P C

-----------------

Syntax:  ADC dest,src                        *       * * * * *

Adds the source operand, the destination  operand,  and  the

value of  the  carry  flag.  The  result  is  assigned  to  the

destination operand. This instruction is used to add  the  more

significant portions of numbers that must be added in  multiple

registers.

000100dw    mod,reg,r/m    disp(0 or 2)

---------------------------------------------------------------

ADC  reg,reg      | adc   dx,cx               | 88/86 3

|                           | 286   2

|                           | 386   2

---------------------------------------------------------------

ADC  mem,reg      | adc   WORD PTR m32[2],dx  | 88/86 16+EA

|                           | (W88=24+EA)

|                           | 286   7

|                           | 386   7

---------------------------------------------------------------

ADC  reg,mem      | adc   dx,WORD PTR m32[2]  | 88/86 9+EA

|                           | (W88=13+EA)

|                           | 286   7

|                           | 386   6

---------------------------------------------------------------

100000sw    mod,010,r/m    disp (0 or 2)     data (1 or 2)

---------------------------------------------------------------

ADC  reg,immed    | adc   dx,12              | 88/86 4

|                          | 286   3

|                          | 386   2

---------------------------------------------------------------

ADC  mem,immed    | adc   WORD PTR m32[2],16 | 88/86 17+EA

|                          | (W88=23+EA)

|                          | 286   7

|                          | 386   7

---------------------------------------------------------------

0001010w    data (1 or 2)

---------------------------------------------------------------

ADC  accum,immed  |  adc   ax,5              | 88/86 4

|                          | 286   3

|                          | 386   2

---------------------------------------------------------------

Arithmetic Instructions

1.1.4. INC

Increment                            Flags:  O D I T S Z A P C

Syntax:  INC dest                            -----------------

*       * * * *

Adds 1 to the destination operand. Because  the  operand  is

treated as an unsigned integer, the INC  instruction  does  not

affect the carry flag. If a signed  carry  requires  detection,

use the ADD instruction.

1111111w     mod,000,r/m      disp (0 or 2)

---------------------------------------------------------------

INC  reg8        | inc   cl                | 88/86 3

|                         | (W88=23+EA)

|                         | 286   2

|                         | 386   2

---------------------------------------------------------------

INC  mem         | inc   vpage             | 88/86 15+EA

|                         | 286   7

|                         | 386   6

---------------------------------------------------------------

01000 reg

---------------------------------------------------------------

INC  reg16       | inc   bx                | 88/86 3

|                         | 286   2

|                         | 386   2

---------------------------------------------------------------

1.1.5. SUB

Subtract                             Flags:  O D I T S Z A P C

-----------------

Syntax:  SUB dest,src                        *       * * * * *

Subtracts the source operand from  the  destination  operand

and stores the result in the destination operand.

001010dw    mod,reg,r/m      disp (0 or 2)

---------------------------------------------------------------

SUB  reg,reg      | sub    ax,bx              | 88/86 3

| sub    bh,dh              | 286   2

|                           | 386   2

---------------------------------------------------------------

SUB  mem,reg      | sub    tally,bx           | 88/86 16+EA

|                           | (W88=24+EA)

| sub    array[di],bl       | 286   7

|                           | 386   6

---------------------------------------------------------------

SUB  reg,mem      | sub    cx,discard         | 88/86 9+EA

|                           | (W88=13+EA)

| sub    al,[bx]            | 286   7

|                           | 386   7

---------------------------------------------------------------

Arithmetic Instructions

100000sw    mod,101,r/m      disp (0 or 2)     data (1 or 2)

---------------------------------------------------------------

SUB  reg,immed    | sub    dx,45              | 88/86 4

| sub    bl,7               | 286   3

|                           | 386   2

---------------------------------------------------------------

SUB  mem,immed    | sub    total,4000         | 88/86 17+EA

|                           | (W88=25+EA)

| sub    BYTE PTR [bx+di],2 | 286   7

|                           | 386   7

---------------------------------------------------------------

0010110w    data (1 or 2)

---------------------------------------------------------------

SUB  accum,immed  | sub    ax,32000           | 88/86 4

|                           | 286   3

|                           | 386   2

---------------------------------------------------------------

1.1.6. SBB

Subtract with Borrow                 Flags:  O D I T S Z A P C

-----------------

Syntax:  SBB dest,src                        *       * * * * *

Subtracts the source from the  destination,  then  subtracts

the value of the carry flag from the  result.  This  result  is

assigned to the destination. SBB is used to subtract the  least

significant portions of  numbers  that  must  be  processed  in

multiple registers.

000110dw     mod,reg,r/m      disp (0 or 2)

---------------------------------------------------------------

SBB  reg,reg       | sbb    dx,cx              | 88/86 3

|                           | 286   2

|                           | 386   2

---------------------------------------------------------------

SBB  mem,reg       | sbb    WORD PTR m32[2],dx | 88/86 16+EA

|                           | (W88=24+EA)

|                           | 286   7

|                           | 386   6

---------------------------------------------------------------

SBB  reg,mem       | sbb    dx,WORD PTR m32[2] | 88/86 9+EA

|                           | (W88=13+EA)

|                           | 286   7

|                           | 386   7

---------------------------------------------------------------

100000sw     mod,011,r/m      disp (0 or 2)     data (1 or 2)

---------------------------------------------------------------

SBB  reg,immed     | sbb    dx,45              | 88/86 4

|                           | 286   3

|                           | 386   2

---------------------------------------------------------------

SBB  mem,immed     | sbb    WORD PTR m32[2],40 | 88/86 17+EA

|                           | (W88=25+EA)

|                           | 286   7

|                           | 386   7

---------------------------------------------------------------

Arithmetic Instructions

0001110w     data (1 or 2)

---------------------------------------------------------------

SBB  accum,immed   | sb     ax,320             | 88/86 4

|                           | 286   3

|                           | 386   2

---------------------------------------------------------------

1.1.7. SubLong

;* SubLong - Subtracts a double-word (long) integer from anothe

;*

;* Shows:   Instructions -  sub     sbb

;*

;* Params:  long1 - First integer

;*          long2 - Second integer

;*

;* Return:  Difference as long integer

SubLong PROC \

long1:DWORD, long2:DWORD

mov     ax, WORD PTR long1[0]   ; AX = low word, long1

mov     dx, WORD PTR long1[2]   ; DX = high word, long1

sub     ax, WORD PTR long2[0]   ; Subtract low word, long2

sbb     dx, WORD PTR long2[2]   ; Subtract high word, long2

ret                             ; Result returned as DX:AX

SubLong ENDP

1.1.8. DEC

Decrement                            Flags:  O D I T S Z A P C

Syntax:  DEC dest                            -----------------

*       * * * *

Subtracts  1  from  the  destination  operand.  Because  the

operand is treated as an unsigned integer, the DEC  instruction

does not affect the carry flag. If  a  signed  borrow  requires

detection, use the SUB instruction.

1111111w    mod,001,r/m      disp (0 or 2)

---------------------------------------------------------------

DEC  reg8    | dec   cl            | 88/86 3

|                     | 286   2

|                     | 386   2

---------------------------------------------------------------

DEC  mem     | dec   counter       | 88/86 15+EA (W88=23+EA)

|                     | 286   7

|                     | 386   6

---------------------------------------------------------------

Arithmetic Instructions

01001,reg

---------------------------------------------------------------

DEC  reg16   | dec   ax            | 88/86 3

|                     | 286   2

|                     | 386   2

---------------------------------------------------------------

1.1.9. NEG

Two's Complement Negation            Flags:  O D I T S Z A P C

-----------------

*       * * * * *

Syntax:  NEG dest

Replaces the operand with its  two's  complement.  NEG  does

this by subtracting the operand from 0. If the  operand  is  0,

the carry flag is cleared. Otherwise the carry flag is set.  If

the operand contains the maximum possible negative value  (-128

for 8-bit operands or -32768 for 16-bit  operands),  the  value

does not change, but the overflow and carry flags are set.

1111011w    mod, 011,r/m      disp (0 or 2)

---------------------------------------------------------------

NEG  reg     | neg   ax            | 88/86 3

|                     | 286   2

|                     | 386   2

---------------------------------------------------------------

NEG  mem     | neg   balance       | 88/86 16+EA (W88=24+EA)

|                     | 286   7

|                     | 386   6

---------------------------------------------------------------

1.1.10. IMUL

Signed Multiply                      Flags:  O D I T S Z A P C

Syntax:  IMUL src                            -----------------

*       ? ? ? ? *

Multiplies an implied destination  operand  by  a  specified

source operand. Both operands are treated as signed numbers. If

a single 16-bit operand is given, the implied destination is AX

and the product goes into the DX:AX register pair. If a  single

8-bit operand is given, the implied destination is AL  and  the

product goes into AX. On the 80386, if the operand is EAX,  the

product goes into the EDX:EAX  register  pair.  The  carry  and

overflow flags are set if the product is sign extended into  DX

for 16-bit operands, into AH for 8-bit operands,  or  into  EDX

for 32-bit operands.

Two additional syntaxes are  available  on  the  80186-80386

processors. In the two-operand form, a  16-bit  register  gives

one of the factors  and  serves  as  the  destination  for  the

result; a source constant specifies the other  factor.  In  the

three-operand form, the first  operand  is  a  16-bit  register

where the result  will  be  stored,  the  second  is  a  16-bit

register or memory operand containing one of the  factors,  and

Arithmetic Instructions

the third is a constant representing  the  other  factor.  With

both variations, the overflow and carry flags are  set  if  the

result  is  too  large  to  fit  into  the  16-bit  destination

register. Since the low 16 bits of the product are the same for

both signed and unsigned multiplication, these syntaxes can  be

used for either signed or unsigned numbers.

1111011w     mod,101,r/m      disp (0 or 2)

---------------------------------------------------------------

IMUL  reg    | imul  dx       | 88/86 b=80-98,w=128-154

|                | 286   b=13,w=21

|                | 386   b=9-14,w=9-22,d=9-38*

---------------------------------------------------------------

IMUL  mem    | imul  factor   | 88/86 (b=86-104,w=134-160)+EA*

|                | 286   b=16,w=24

|                | 386   b=12-17,w=12-25,d=12-41*

---------------------------------------------------------------

* The  80386  has  an  early-out  multiplication  algorithm.

Therefore multiplying an 8-bit or 16-bit value in EAX takes the

same time as multiplying the value in AL or AX.

** Word  memory  operands  on  the  8088  take  (138-164)+EA

clocks.

011010s1     mod,reg,r/m      disp (0 or 2)     data (1 or 2)

---------------------------------------------------------------

IMUL  reg16,immed       | imul  cx,25    | 88/86 -

|                | 286   21

|                | 386

|                | b=9-14,w=9-22,d=9-3

---------------------------------------------------------------

IMUL  reg16,reg16,immed | imul  dx,ax,18 | 88/86 -

|                | 286   21

|                | 386

|                | b=9-14,w=9-22,d=9-3

---------------------------------------------------------------

IMUL  reg16,mem16,immed |imul  bx,[si],60| 88/86 -

|                | 286   24

|                | 386

|                |    b=12-17,

|                |    w=12-25,d=12-41

---------------------------------------------------------------

1.1.11. MUL

Unsigned Multiply                    Flags:  O D I T S Z A P C

Syntax:  MUL src                             -----------------

*       ? ? ? ? *

Multiplies an implied destination  operand  by  a  specified

source operand. Both operands are treated as unsigned  numbers.

If a single 16-bit operand is given, the implied destination is

AX and the product goes into the  DX:AX  register  pair.  If  a

single 8-bit operand is given, the implied  destination  is  AL

and the product goes into AX. The carry and overflow flags  are

set if DX is not 0 for 16-bit operands or if AH is not zero for

8-bit operands.

Arithmetic Instructions

1111011w    mod, 100,r/m      disp (0 or 2)

---------------------------------------------------------------

MUL  reg        | mul   bx        | 88/86 b=70-77,w=118-113

| mul   dl        | 286   b=13,w=21

|                 | 386   b=9-14,w=9-22,d=9-38

---------------------------------------------------------------

MUL  mem        | mul   factor  | 88/86 (b=76-83,w=124-139)+EA

| mul   WORD PTR [bx]    | 286   b=16,w=24

|               | 386   b=12-17,w=12-25,d=12-4

---------------------------------------------------------------

*  Word memory operands on the 8088 take (128-143)+EA clocks.

1.1.12. IDIV

Signed Divide                        Flags:  O D I T S Z A P C

Syntax:  IDIV src                            -----------------

?       ? ? ? ? ?

Divides an implied destination operand by a specified source

operand. Both operands are treated as signed  numbers.  If  the

source (divisor) is 16 bits wide, then the implied  destination

(dividend) is the DX:AX register pair. The quotient  goes  into

AX and the remainder into DX. If the source is 8 bits wide, the

implied destination is AX. The quotient goes into  AL  and  the

remainder into AH.

1111011w     mod,111,r/m      disp (0 or 2)

---------------------------------------------------------------

IDIV  reg    | idiv  bx        | 88/86 b=101-112,w=165-184

| div   dl        | 286   b=17,w=25

|                 | 386   b=19,w=27,d=43

---------------------------------------------------------------

IDIV  mem    | idiv  itemp     | 88/86 (b=107-118,w=171-190)+E

|                 | 286   b=20,w=28

|                 | 386   b=22,w=30,d=46

---------------------------------------------------------------

* Word memory operands on the 8088 take (175-194)+EA clocks.

1.1.13. DIV

Unsigned Divide                      Flags:  O D I T S Z A P C

-----------------

Syntax:  DIV src                             ?       ? ? ? ? ?

Divides an implied destination operand by a specified source

operand. Both operands are treated as unsigned numbers. If  the

source (divisor) is 16 bits wide, then the implied  destination

(dividend) is the DX:AX register pair. The quotient  goes  into

AX and the remainder into DX. If the source is 8 bits wide, the

implied destination operand is AX. The quotient  goes  into  AL

and the remainder into AH.

Arithmetic Instructions

1111011w    mod,110,r/m      disp (0 or 2)

-----------+---------------------+-----------------------------

DIV  reg  | div   cx            | 88/86 b=80-90,w=144-162

| div   dl            | 286   b=14,w=22

|                     | 386   b=14,w=22,w=38

-----------+---------------------+-----------------------------

DIV  mem  | div   [bx]          | 88/86 (b=86-96,w=150-168)+EA

| div   fsize         | 286   b=17,w=25

|                     | 386   b=17,w=25,d=41

-----------+---------------------+-----------------------------

* Word memory operands on the 8088 take (158-176)+EA clocks.

1.2. BCD Adjust Instructions

The ASCII instructions assume unpacked BCD values--one value

per  word.  The  DECIMAL   instructions   assume   packed   BCD

values--one value per byte.

Mnemonic

Name

AAA

ASCII Adjust After Addition

AAS

ASCII Adjust After Subtraction

AAM

ASCII Adjust After Multiply

AAD

ASCII Adjust Before Division

DAA

Decimal Adjust After Division

DAS

Decimal Adjust After Subtraction

1.2.1. AAA

ASCII Adjust After Addition          Flags:  O D I T S Z A P C

-----------------

Syntax:  AAA                                 ?       ? ? * ? *

Adjusts the result of an addition to a decimal digit  (0-9).

The previous addition instruction should place its 8-bit sum in

AL. If the sum is greater than 9h, AH is  incremented  and  the

carry and auxiliary carry flags are set. Otherwise,  the  carry

and auxiliary carry flags are cleared.

00110111

----------+------------------------+---------------------------

AAA      | aaa                    | 88/86  8

|                        | 286    3

|                        | 386    4

----------+------------------------+---------------------------

BCD Adjust Instructions

1.2.2. AAS

ASCII Adjust After Subtraction       Flags:  O D I T S Z A P C

-----------------

Syntax:  AAS                                 ?       ? ? * ? *

Adjusts the result of  a  subtraction  to  a  decimal  digit

(0-9). The previous subtraction instruction  should  place  its

8-bit result in AL. If the result is greater than 9h,  then  AH

is decremented and the carry and auxiliary carry flags are set.

Otherwise, the carry and auxiliary carry flags are cleared.

1.2.3. AAS

00111111

--------------+--------------------+---------------------------

AAS          | aas                | 88/86 8

|                    | 286   3

|                    | 386   4

--------------+--------------------+---------------------------

1.2.4. AAM

ASCII Adjust After Multiply          Flags:  O D I T S Z A P C

-----------------

Syntax:  AAM                                 ?       * * ? * ?

Converts an 8-bit binary number less than 100 decimal in  AL

to an unpacked BCD number in AX.  The  most  significant  digit

goes in AH and the least significant in AL. This instruction is

often used to adjust the product after a MUL  instruction  that

multiplies unpacked BCD digits in AH and AL. It is also used to

adjust the quotient after a  DIV  instruction  that  divides  a

binary number less than 100 decimal in AX by  an  unpacked  BCD

number.

11010100     00001010

--------------+--------------------+---------------------------

AAM          | aam                | 88/86 83

|                    | 286   16

|                    | 386   17

--------------+--------------------+---------------------------

1.2.5. IntToAsc

BCD Adjust Instructions

;* IntToAsc - Converts integer to ASCII string.

;* This procedure is useful only for assembly language,

;* and is not intended to be C-callable.

;*

;* Shows:   Instructions - cwd     aam     xchg

;*

;* Entry:   AX = integer (9999 max)

;*

;* Return:  DX:AX = 4-digit ASCII number

IntToAsc PROC

cwd                     ; Zero DX register

mov     cx, 100         ; Divide AX by 100, yields

div     cx              ;   AX=quotient, DX=remainder

aam                     ; Make digits unpacked BCD

or      ax, '00'        ; Convert to ASCII

xchg    ax, dx          ; Do same thing for DX

aam

or      ax, '00'

ret                     ; Return DX:AX = ASCII number

IntToAsc ENDP

1.2.6. AAD

ASCII Adjust Before Division         Flags:  O D I T S Z A P C

-----------------

Syntax:  AAD                                 ?       * * ? * ?

Converts unpacked BCD digits in AH (most significant  digit)

and AL (least significant digit) to a binary number in AX.  The

instruction is often used to prepare an unpacked BCD number  in

AX for division by an unpacked BCD digit in an 8-bit  register.

11010101     00001010

----------+------------------------+---------------------------

AAD      | aad                    | 88/86  60

|                        | 286    14

|                        | 386    19

----------+------------------------+---------------------------

1.2.7. DAA

Decimal Adjust After Addition        Flags:  O D I T S Z A P C

-----------------

Syntax:  DAA                                 ?       * * * * *

Adjusts the result of an addition to  a  packed  BCD  number

(less than 100  decimal).  The  previous  addition  instruction

should place its 8-bit binary sum  in  AL.  DAA  converts  this

binary sum to packed BCD  format  with  the  least  significant

decimal digit in the lower four bits and the  most  significant

digit in the upper four bits. If the sum is  greater  than  99h

after adjustment, then the carry and auxiliary carry flags  are

BCD Adjust Instructions

set.  Otherwise,  the  carry  and  auxiliary  carry  flags  are

cleared.

00100111

--------------+---------------------+--------------------------

DAA          | daa                 | 88/86 4

|                     | 286   3

|                     | 386   4

--------------+---------------------+--------------------------

1.2.8. DAS

Decimal Adjust after Subtraction     Flags:  O D I T S Z A P C

-----------------

Syntax:  DAS                                 ?       * * * * *

Adjusts the result of a subtraction to a packed  BCD  number

(less than 100 decimal). The previous  subtraction  instruction

should place its 8-bit binary result in AL. DAS  converts  this

binary sum to packed BCD  format  with  the  least  significant

decimal digit in the lower four bits and the  most  significant

digit in the upper four bits. If the sum is  greater  than  99h

after adjustment, then the carry and auxiliary carry flags  are

set. Otherwise, carry and auxiliary carry flags are cleared.

00101111

--------------+---------------------+--------------------------

DAS          | das                 | 88/86 4

|                     | 286   3

|                     | 386   4

--------------+---------------------+--------------------------

1.3. Bit Shifting Instructions

The ROTATE instructions cycle bit values through a  register

if done repeatedly. The SHIFT instructions do not recycle  bits

shifted out.

Mnemonic

Name

RCL

Rotate Through Carry Left

RCR

Rotate Through Carry Right

ROL

Rotate Left

ROR

Rotate Right

SAL

Shift Arithmetic Left (identical to SHL)

SAR

Shift Arithmetic Right (preserves sign)

SHL

Shift Left

SHR

Bit Shifting Instructions

Shift Right (does not preserve sign)

1.3.1. RCL/RCR/ROL/ROR

Rotate                               Flags:  O D I T S Z A P C

-----------------

*               *

Syntax:  RCL dest,1

RCL dest,CL

RCL dest,number (80186/286/386 Only)

Rotates the bits in the destination operand  the  number  of

times specified in the source operand. RCL and ROL  rotate  the

bits left; RCR and ROR rotate right.

ROL and ROR rotate the number of bits in  the  operand.  For

each rotation, the leftmost or rightmost bit is copied  to  the

carry flag as well as rotated. RCL and RCR rotate  through  the

carry flag. The carry flag becomes an extension of the  operand

so that a 9-bit rotation is  done  for  8-bit  operands,  or  a

17-bit rotation for 16-bit operands.

On the 8088 and 8086, the source operand can be either CL or

1. On the 80186-80386, the source operand can be CL or an 8-bit

constant. On the 80186-80386, rotate counts larger than 31  are

masked off, but on the 8088 and 8086, larger rotate counts  are

performed despite the inefficiency involved. The overflow  flag

is only modified by single-bit variations of  the  instruction;

for multiple-bit variations it is undefined.

1101000w    mod,TTT*,r/m     disp (0 or 2)

*  TTT represents one of the following bit codes: 000 for ROL,

001 for ROR, 010 for RCL, or 011 for RCR.

-------------+-------------------------+-----------------------

ROL reg,1   | ror   ax,1              | 88/86 2

ROR reg,1   | rol   dl,1              | 286   2

|                         | 386   3

-------------+-------------------------+-----------------------

RCL reg,1   | rcl   dx,1              | 88/86 2

RCR reg,1   | rcr   bl,1              | 286   2

|                         | 386   9

-------------+-------------------------+-----------------------

ROL mem,1   | ror   bits,1            | 88/86 15+EA (W88=23+EA

ROR mem,1   | rol   WORD PTR [bx],1   | 286   7

|                         | 386   7

-------------+-------------------------+-----------------------

RCL mem,1   | rcl   WORD PTR [si],1   | 88/86 15+EA (W88=23+EA

RCR mem,1   | rcr   WORD PTR m32[0],1 | 286   7

|                         | 386   10

-------------+-------------------------+-----------------------

Bit Shifting Instructions

1101001w    mod,TTT*,r/m      disp (0 or 2)

*  TTT represents one of the following bit codes: 000 for ROL,

001 for ROR, 010 for RCL, or 011 for RCR.

-------------+-------------------------+-----------------------

ROL  reg,CL | ror   ax,cl             | 88/86 8+4n

ROR  reg,CL | rol   dx,cl             | 286   5+n

|                         | 386   3

-------------+-------------------------+-----------------------

RCL  reg,CL | rcl   dx,cl             | 88/86 8+4n

RCR  reg,CL | rcr   bl,cl             | 286   5+n

|                         | 386   9

-------------+-------------------------+-----------------------

ROL  mem,CL | ror   color,cl          | 88/86 20+EA+4n

|                         | (W88=28+EA+4n)

ROR  mem,CL | rol   WORD PTR [bp+6],cl| 286   8+n

|                         | 386   7

-------------+-------------------------+-----------------------

RCL  mem,CL | rcr  WORD PTR [bx+di],cl| 88/86 20+EA+4n

|                         | (W88=28+EA+4n)

RCR  mem,CL | rcl  masker             | 286   8+n

|                         | 386   10

-------------+-------------------------+-----------------------

1100000w    mod,TTT*,r/m       disp (0 or 2)     data (1)

*  TTT represents one of the following bit codes: 000 for ROL,

001 for ROR, 010 for RCL, or 011 for RCR.

------------------+-------------------------+------------------

ROL  reg,immed8  | rol   ax,13             | 88/86 -

ROR  reg,immed8  | ror   bl,3              | 286   5+n

|                         | 386   3

------------------+-------------------------+------------------

RCL  reg,immed8  | rcl   bx,5              | 88/86 -

RCR  reg,immed8  | rcr   si,9              | 286   5+n

|                         | 386   9

------------------+-------------------------+------------------

ROL  mem,immed8  | rol   BYTE PTR [bx],10  | 88/86 -

ROR  mem,immed8  | ror   bits,6            | 286   8+n

|                         | 386   7

------------------+-------------------------+------------------

RCL  mem,immed8  | rcl   WORD PTR [bp+8],5 | 88/86 -

RCR  mem,immed8  | rcr   masker,3          | 286   8+n

|                         | 386   10

------------------+-------------------------+------------------

1.3.2. SAL/SAR/SHL/SHR

Shift                                Flags:  O D I T S Z A P C

-----------------

Syntax:  SHL dest,1                          *       * * ? * *

SHL dest,CL

SHL dest,number (80186/286/386 Only)

Shifts the bits in the destination  operand  the  number  of

times specified by the source operand. SAL and  SHL  shift  the

bits left; SAR and SHR shift right.

With SHL, SAL, and SHR, the bit shifted off the end  of  the

operand is copied into the  carry  flag  and  the  leftmost  or

rightmost bit opened by the shift is set to 0.  With  SAR,  the

Bit Shifting Instructions

bit shifted off the end of the operand is copied into the carry

flag and the leftmost bit  opened  by  the  shift  retains  its

previous value (thus preserving the sign of the  operand).  SAL

and SHL are synonyms; they have the same effect.

On the 8088 and 8086, the source operand can be either CL or

1. On the 80186-80386 processors, the source operand can be  CL

or an 8-bit constant.  On  the  80186-80386  processors,  shift

counts larger than 31 are masked off, but on the 8088 and 8086,

larger shift counts  are  performed  despite  the  inefficiency

involved. The overflow flag  is  only  modified  by  single-bit

variations of the instruction; for multiple-bit  variations  it

is undefined.

1.4. Compare Instructions

The compare instructions are similar to arithmetic and logic

instructions, except that the  result  is  not  stored  in  the

destination  operand.  Like  arithmetic   instructions,   these

instructions set processor flags and can set up a  test  for  a

subsequent conditional jump.

Mnemonic

Name

CMP

Compare (same as SUB but result is ignored)

CMPS

Compare String

TEST

Logical Compare (same as AND but result is ignored)

1.4.1. CMP

Compare Two Operands                 Flags:  O D I T S Z A P C

Syntax:  CMP dest,src                        -----------------

*       * * * * *

Compares two operands as a test for a subsequent conditional

jump or set instruction.  CMP  does  this  by  subtracting  the

source operand from the destination  operand  and  setting  the

flags according to the result. CMP  is  the  same  as  the  SUB

instruction, except that the result is not stored.

Compare Instructions

001110dw    mod,reg,r/m       disp (0 or 2)

-------------+---------------------+---------------------------

CMP  reg,reg| cmp   di,bx         | 88/86 3

| cmp   dl,cl         | 286   2

|                     | 386   2

-------------+---------------------+---------------------------

CMP  mem,reg| cmp   maximum,dx    | 88/86 9+EA (W88=13+EA)

| cmp   array[si],bl  | 286   7

|                     | 386   5

-------------+---------------------+---------------------------

CMP  reg,mem| cmp   dx,minimum    | 88/86 9+EA (W88=13+EA)

| cmp   bh,array[si]  | 286   6

|                     | 386   6

-------------+---------------------+---------------------------

100000sw    mod,111,r/m       disp (0 or 2)     data (1 or 2)

-----------------+---------------------+-----------------------

CMP  reg,immed  | cmp  ax,24          | 88/86 4

|                     | 286   3

|                     | 386   2

-----------------+---------------------+-----------------------

CMP  mem,immed  | cmp  WORD PTR [di],4| 88/86 10+EA (W88=14+EA

| cmp  tester,4000    | 286   6

|                     | 386   5

-----------------+---------------------+-----------------------

0011110w    data (1 or 2)

-----------------+-------------------------+-------------------

CMP  accum,immed| cmp   ax,1000           | 88/86 4

|                         | 286   3

|                         | 386   2

-----------------+-------------------------+-------------------

1.4.2. SetLineMode

;* SetLineMode - Sets line mode for EGA or VGA.

;*

;* Shows:   BIOS Interrupt - 10h, Function 11h

;*                            (Character Generator Interface)

;*                           10h, Function 12h

;*                            (Video Subsystem Configuration)

;*          Instruction - cmp

;*

;* Uses:    vconfig - Video configuration structure, declared i

;*          the DEMO.INC include file. The structure must first

;*          be initialized by calling the GetVidConfig procedur

;*

;* Params:  line - Requested line mode (25, 43, or 50)

;*

;* Return:  Short integer with error code

;*          0 if successful

;*          1 if error

SetLineMode PROC \

line:WORD

cmp     vconfig.adapter, EGA    ; EGA or VGA?

jge     @F              ; Yes?  Continue

jmp     e_exit          ; No?  Exit with error

Compare Instructions

@@:     mov     ax, line        ; Check for valid parameter

cmp     al, 25

je      line25

cmp     al, 43

je      line43

cmp     al, 50

je      line50

jmp     SHORT e_exit

; If not 25, 43, or 50, exit w/ error

line25: mov     al, 11h         ; Set for EGA 25-line mode

cmp     vconfig.adapter, EGA    ; EGA?

je      lmode           ; Yes?  Continue

mov     ax, 1202h       ; No?  Function 12h for VGA

mov     bl, 30h         ; AL = 2 for 400 scan lines

int     10h             ; Reset to 400 scan lines

mov     ax, 0003        ; Reset mode (Function 0)

int     10h             ;   to mode 3 (80-col text)

mov     al, 14h         ; Request 8x16 char matrix

jmp     SHORT lmode

line43: mov     al, 12h         ; Set for EGA 43-line mode

cmp     vconfig.adapter, EGA    ; EGA?

je      lmode           ; Yes?  Continue

mov     ax, 1201h       ; No?  Function 12h for VGA

mov     bl, 30h         ; AL = 1 for 350 scan lines

int     10h             ; Reset to 350 scan lines

mov     ax, 0003        ; Reset mode (Function 0)

int     10h             ;   to mode 3 (80-col text)

mov     al, 12h         ; Request 8x8 character matrix

jmp     SHORT lmode

line50: cmp     vconfig.adapter, VGA    ; VGA?

jne     e_exit          ; No?  Exit with error

mov     ax, 1202h       ; Yes?  Function 12h

mov     bl, 30h         ; AL = 2 for 400 scan lines

int     10h             ; Reset to 400 scan lines

mov     ax, 0003        ; Reset mode (Function 0)

int     10h             ;   to mode 3 (80-col text)

mov     al, 12h         ; Request 8x8 character matrix

lmode:  sub     bl, bl          ; Use table 0

mov     ah, 11h         ; Request Function 11h

int     10h             ; Set new line mode

mov     ah, 12h         ; Select alternate print

mov     bl, 20h         ;    screen for EGA and VGA

int     10h

cmp     vconfig.adapter, VGA    ; VGA?

je      exit            ; Yes?  Then exit

cmp     line, 12h       ; If EGA 43-line mode, set

je      port            ;   cursor through port to

;   avoid cursor emulation bug

mov     al, 7           ; Else use BIOS to set cursor

push    ax              ; Pass bottom scan line

mov     al, 6

push    ax              ; Pass top scan line

call    SetCurSize      ; Set normal cursor

add     sp, 4           ; Clean stack

jmp     SHORT exit      ; Exit

Compare Instructions

port:   mov     dx, 03D4h       ; Video controller address

mov     ax, 060Ah       ; Set AH = 06h (cursor start)

;     AL = 0Ah (register #)

out     dx, ax          ; Update port

mov     ax, 000Bh       ; Set AH = 00h (cursor end)

;     AL = 0Bh (register #)

out     dx, ax          ; Update port

jmp     SHORT exit      ; Normal exit

e_exit: mov     ax, 1           ; Set error code

jmp     SHORT @F

exit:   sub     ax, ax          ; Clear error code

@@:     ret

SetLineMode ENDP

1.4.3. CMPS/CMPSB/CMPSW

Compare String                       Flags:  O D I T S Z A P C

-----------------

*       * * * * *

Syntax:  CMPS dest,src

CMPSB

CMPSW

Compares two strings. DS:SI must point to the source  string

and ES:DI  must  point  to  the  destination  string  (even  if

operands are  given).  For  each  comparison,  the  destination

element is subtracted from the source element and the flags are

updated to reflect the  result  (although  the  result  is  not

stored). DI and SI are adjusted according to the  size  of  the

operands and  the  status  of  the  direction  flag.  They  are

increased if the direction flag has been cleared  with  CLD  or

decreased if the direction flag has been set with STD.

If the CMPS form of the instruction is used,  operands  must

be provided to indicate the size of the  data  elements  to  be

processed. A segment override can be given for the source  (but

not for the destination). If CMPSB (bytes) or CMPSW (words)  is

used, the instruction determines the size of the data  elements

to be processed. Operands are not allowed with CMPSB or  CMPSW.

CMPS  and  its  variations  are  usually  used  with  repeat

prefixes. REPNE (or REPNZ) is used  to  find  the  first  match

between two strings. REPE (or REPZ) is used to find  the  first

nonmatch. Before the comparison, CX should contain the  maximum

number of elements to compare. After a  REPNE  CMPS,  the  zero

flag will be cleared if no match was found. After a REPE  CMPS,

the zero flag will be set if no nonmatch was  found.  Otherwise

SI and DI will point to the element after the  first  match  or

nonmatch.

1010011w

------------------------------+--------------------+-----------

CMPS  [segreg:]src,[ES:]dest |cmps  source,es:dest| 88/86 22 (

CMPSB                        |repne cmpsw         | 286   8

CMPSW                        |repe  cmpsb         | 386   10

------------------------------+--------------------+-----------

Compare Instructions

1.4.4. StrCompare

;* StrCompare - Compares two strings for equality.

;* See StrWrite, StrFindChar, WinOpen, and WinClose

;* procedures for other examples of string instructions.

;*

;* Shows:   Instructions - cmpsb   cmpsw   repe   test   jcxz

;*

;* Params:  str1 - Pointer to first string

;*          str2 - Pointer to second string

;*          len  - Length in bytes for comparison.

;*                  Strings need not be of equal length;

;*                  however if len is an even number,

;*                  comparison is made on a word-by-word

;*                  basis and thus is more efficient.

;*

;* Return:  Null pointer if strings match;

;*                  else pointer to string #1 where match faile

StrCompare PROC \

USES ds di si, \

str1:PTR BYTE, str2:PTR BYTE, len:WORD

LoadPtr es, di, str1    ; ES:DI points to string #1

LoadPtr ds, si, str2    ; DS:SI points to string #2

mov     cx, len         ; Length of search in bytes

and     al, 0           ; Set ZR flag in case CX = 0

jcxz    nullp           ; Assume success if length = 0

test    cl, 1           ; Even number?

jz      wrdcmp          ; Yes?  Compare word-by-word

repe    cmpsb           ; No?  Compare byte-by-byte

jmp     SHORT nullp

wrdcmp: shr     cx, 1           ; Decrease count by half

repe    cmpsw           ; Compare word-by-word

sub     di, 2           ; Back up 2 characters

sub     si, 2

cmpsb                   ; Match?

jne     nullp           ; No?  Then failure

cmpsb                   ; Compare last characters

nullp:  mov     ax, 0           ; Set null pointer without

mov     dx, 0           ;   disturbing flags

je      exit            ; If strings match, exit

dec     di              ; Else point to failure

mov     ax, di

mov     dx, es

exit:   ret

StrCompare ENDP

1.4.5. TEST

Logical Compare                      Flags:  O D I T S Z A P C

-----------------

Syntax:  TEST dest,src                       0       * * ? * 0

Compare Instructions

Tests specified bits of an operand and sets the flags for  a

subsequent conditional jump or  set  instruction.  One  of  the

operands contains the value to be tested. The other contains  a

bit mask indicating the bits to be tested. TEST works by  doing

a logical bitwise AND on the source and  destination  operands.

The flags  are  modified  according  to  the  result,  but  the

destination operand is not changed.  This  instruction  is  the

same as the AND instruction, except  that  the  result  is  not

stored.

1000011w    mod,reg,r/m      disp (0 or 2)

---------------+-----------------------+-----------------------

TEST  reg,reg | test   dx,bx          | 88/86 3

| test   bl,ch          | 286   2

|                       | 386   2

---------------+-----------------------+-----------------------

TEST  mem,reg*| test   dx,flags       | 88/86 9+EA (W88=13+EA)

TEST  reg,mem | test   bl,bitarray[bx]| 286   6

|                       | 386   5

---------------+-----------------------+-----------------------

*The assembler transposes TEST mem,reg so that it is encoded

as TEST reg,mem

1111011w    mod,000,r/m      disp (0 or 2)     data (1 or 2)

-------------------+--------------------------+----------------

TEST  reg,immed   | test   cx,30h            | 88/86 5

| test   cl,1011b          | 286   3

|                          | 386   2

-------------------+--------------------------+----------------

TEST  mem,immed   | test   masker,1          | 88/86 11+EA

| test   BYTE PTR [bx],03h | 286   6

|                          | 386   5

-------------------+--------------------------+----------------

1010100s    data (1 or 2)

-------------------+--------------------------+----------------

TEST  accum,immed | test   ax,90h            | 88/86 4

|                          | 286   3

|                          | 386   2

-------------------+--------------------------+----------------

1.5. Control-Flow Instructions

Mnemonic

Name

JMP

Jump Unconditionally

JCXZ/JECXZ

Jump If CX is Zero

Jcond

Jump conditionally

LOOP

Loop (decrement CX, then jump if CX is not 0)

LOOPE/LOOPZ

Loop If (loop while condition is met and CX is  not

0)

CALL

Call Procedure

RET

Return from Procedure

Control-Flow Instructions

RETN/RETF

Return from Procedure (near or far)

INT

Interrupt (call interrupt procedure)

IRET

Interrupt Return

ENTER

Make Stack Frame (80186/286/386 Only)

LEAVE

High Level Procedure Exit (80186/286/386 Only)

1.5.1. JMP

Jump Unconditionally                 Flags:  O D I T S Z A P C

-----------------

Syntax:  JMP label

Transfers program execution to the address specified by  the

destination operand. By default, jumps are near (between -32768

and 32767 bytes from the instruction following the  jump),  but

you can use an override to make them short  (between  -128  and

127 bytes) or far (in a different code segment). With near  and

short jumps, the operand specifies a new IP address.  With  far

jumps, the operand specifies new IP and CS addresses.

11101011       disp (1)

------------------+-------------------------+------------------

JMP  label       | jmp   SHORT exit        | 88/86 15

|                         | 286   7+m

|                         | 386   7+m

------------------+-------------------------+------------------

11101001    disp (2)

------------------+-------------------------+------------------

JMP  label       | jmp   close             | 88/86 15

| jmp   NEAR PTR distant  | 286   7+m

|                         | 386   7+m

------------------+-------------------------+------------------

11101010    disp (4)

-----------------+-------------------------+-------------------

JMP  label      | jmp   FAR PTR close     | 88/86 15

| jmp   distant           | 286   11+m*

|                         | 386   12+m*

-----------------+-------------------------+-------------------

* Timings for jumps through  call  or  task  gates  are  not

shown, since they are normally used only in operating  systems.

11111111    mod,100,r/m      disp(0 or 2)

------------------+-------------------------+------------------

JMP  reg16       | jmp   ax                | 88/86 11

|                         | 286   7+m

|                         | 386   7+m

------------------+-------------------------+------------------

JMP  mem16       | jmp   WORD [bx]         | 88/86 18+EA

| jmp   table[di]         | 286   11+m

| jmp   DWORD [si]        | 386   10+m

------------------+-------------------------+------------------

Control-Flow Instructions

11111111    mod,101,r/m      disp(4)

------------------+-------------------------+------------------

JMP  mem32       | jmp   fpointer[si]      | 88/86 24+EA

|                         | 286   15+m

|                         | 386   12+m

------------------+-------------------------+------------------

1.5.2. JUMP.ASM

;* JUMP.ASM (unconditional jump) - The following code illustrat

;* how to transfer execution to another statement with the JMP

;* instruction. The code is arranged as a program so that you

;* can trace through it with a debugger.

;* See JCOND.ASM for examples of conditional jump instructions.

;*

;* Shows:   jmp

.MODEL small

.STACK

.CODE

.STARTUP

; Jump to a destination (LBL2) within 128 bytes. Use SHORT oper

mov     ax, 1           ; Make AX = 1

jmp     SHORT lbl2      ; Skip short

lbl1:   mov     ax, 2           ; Never executed

lbl2:                           ; SHORT destination

; Jump to a "near" destination (between 128 and 32,767 bytes aw

mov     bx, 2

jmp     label3          ; Jump near over the next secti

;       .                       ; Represents more than

;       .                       ;   128 bytes

;       .                       ;   of code

mov     bx, 5           ; Never executed

label3:                         ; Near destination

; A dispatch table (jump table in this case).

; See the MISCDEMO.ASM example program for a similar call table

; Dispatch tables are arrays of addresses.

; An input value (such as a keystroke) is used to calculate

; the index of the destination address.

.DATA

table   DW      dst4            ; Offset for dst4 (index 0)

DW      dst5            ; Offset for dst5 (index 2)

DW      dst6            ; Offset for dst6 (index 4)

prompt  DB      13, 10, 'Type a number between 4 and 6 . . .  $

disp    DB      13, 10, 13, 10, 'You chose '

num     DB      ?, 13, 10, '$'

.CODE

mov     ah, 9       ; Request string display

mov     dx, OFFSET prompt

int     21h         ; Display String

cons:   mov     ah, 7       ; Request console input without ech

int     21h         ; Get Console Input

Control-Flow Instructions

cmp     al, '4'     ; Make sure it's between 4 and 6

jb      cons

cmp     al, '6'

ja      cons

sub     al, '4'     ; Convert to binary number

shl     al, 1       ; Convert byte index to word index

sub     ah, ah      ; Clear AX high byte

mov     bx, ax      ; Copy to BX for index

jmp     table[bx]   ; Jump to table address

; with correct label

dst4:   mov     num, '4'    ; Copy 4 to string

jmp     SHORT exit

dst5:   mov     num, '5'    ; Copy 5 to string

jmp     SHORT exit

dst6:   mov     num, '6'    ; Copy 6 to string

exit:   mov     ah, 9   ; Request $-terminated string display

mov     dx, OFFSET disp

int     21h     ; Display String

.EXIT   num     ; Terminate program

; with num as return code

END

1.5.3. JCXZ/JECXZ

Jump if CX is Zero                   Flags:  O D I T S Z A P C

-----------------

Syntax:  JCXZ label

Transfers program execution to the specified label if CX  is

0. If the count register is not 0, execution continues  at  the

next instruction. The label given as the operand must be  short

(between -128 and 127 bytes from the instruction following  the

jump).

11100011       disp (1)

------------------+-------------------------+------------------

JCXZ  label      | jcxz  notfound          | 88/86 18,noj=6

|                         | 286   8+m,noj=4

|                         | 386   9+m,noj=5

------------------+-------------------------+------------------

1.5.4. Jcondition

Jump Conditionally                   Flags:  O D I T S Z A P C

-----------------

Transfers execution to the  specified  label  if  the  flags

condition is true. The condition  is  tested  by  checking  the

flags shown  in  the  table  on  the  following  page.  If  the

Control-Flow Instructions

condition is false, then no jump is taken and program execution

continues  at  the  next   instruction.   On   the   8088-80286

processors, the label  given  as  the  operand  must  be  short

(between -128 and 127 bytes from the instruction following  the

jump).

0111cond       disp (1)

---------------------+-------------------------+---------------

| jg    bigger            | 88/86 16,noj=4

Jcondition  label   | jo    SHORT too_big     | 286   7+m,noj=

| jpe   p_even            | 386   7+m,noj=

---------------------+-------------------------+---------------

00001111     1000cond       disp (2)

---------------------+-------------------------+---------------

| je    next              | 88/86 -

Jcondition  label   | jnae  lesser            | 286   -

| js    negative          | 386   7+m,noj=

---------------------+-------------------------+---------------

JUMP CONDITIONS

Opcode      Mnemonic  Flags Checked   Description

---------------------------------------------------------------

size 0010   JB/JNAE   CF=1      Jump if below/not above or equa

size 0011   JAE/JNB   CF=0      Jump if above or equal/not belo

size 0110   JBE/JNA   CF=1 or

ZF=1      Jump if below or equal/not abov

size 0111   JA/JNBE   CF=0 and

ZF=0      Jump if above/not below or equa

size 0100   JE/JZ     ZF=1      Jump if equal (zero)

size 0101   JNE/JNZ   ZF=0      Jump if not equal (not zero)

size 1100   JL/JNGE   SF<>OF    Jump if less/not greater or equ

size 1101   JGE/JNL   SF=OF     Jump if greater or equal/not le

size 1110   JLE/JNG   ZF=1 or

SF<>OF    Jump if less or equal/not great

size 1111   JG/JNLE   ZF=0 or

SF=OF     Jump if greater/not less or equ

size 1000   JS        SF=1      Jump if sign

size 1001   JNS       SF=0      Jump if not sign

size 0010   JC        CF=1      Jump if carry

size 0011   JNC       CF=0      Jump if not carry

size 0000   JO        OF=1      Jump if overflow

size 0001   JNO       OF=0      Jump if not overflow

size 1010   JP/JPE    PF=1      Jump if parity/parity even

size 1011   JNP/JPO   PF=0      Jump if no parity/parity odd

Note: The size bits are 0111 for short jumps.

1.5.5. JCOND.ASM

;* JCOND.ASM (conditional jump) shows how to transfer execution

;* conditionally with jump instructions. The code is arranged

;* as a program so that you can trace through it and change

;* registers to test alternate results. Many (but not

;* all) of the conditional jump instructions are illustrated.

Control-Flow Instructions

;*

;* See JUMP.ASM for an example of the related JMP instruction

;* and StrCompare for an example of JCXZ.

;*

;* Shows: jb/jnae  jae/jnb  jbe/jna  ja/jnbe  (unsigned)

;*        jl/jnge  jge/jnl  jle/jng  jg/jnle  (signed)

;*        je/jz    js       jns             (equality and sign)

;*        jc       jnc      jo       jno   (carry and overflow)

;*        jp/jpe   jnp/jpo                          (parity)

.MODEL small

.STACK

.CODE

.STARTUP

mov     ax, 1   ; Initialize arbitrary registers

; for comparisons

mov     bx, 2

mov     cx, 3

cmp     ax, bx  ; Compare unsigned relative values

jb      below   ; Jump if unsigned AX

;  is below unsigned BX

je      equal

above:  add     cx, 2   ; If not below and not equal,

;  must be above

equal:  add     cx, 2   ; Jump to here if equal

below:  add     cx, 2   ; Jump here if below

cmp     ax, bx  ; Compare signed relative values

jl      less    ; Jump if signed AX

jg      more    ;  is less than signed BX

add     cx, 2   ; If not less or more, must be equal

less:   add     cx, 2   ; Jump here if less

more:   add     cx, 2   ; Jump here if more

sub     ax, 2   ; Set flags for sign test

js      sign    ; Jump if sign flag is set

nosign: add     cx, 1   ; Sign not set, so positive number

sign:   add     cx, -1  ; Sign set for negative.

sub     bx, 3   ; Set flags for carry test (make BX = -

jc      carry   ; Jump if carry flag is set

nocarry:add     cx, 1   ; Subtraction causes carry

carry:  add     cx, -1  ; Subtraction doesn't cause carry

mov     al, 254 ; Prepare for multiply

mul     bl      ; Set flags with multiply of AL x BL

jo      over    ; Jump if overflow flag is set

ok:     add     cx, 2   ; No overflow

over:   add     cx, 2   ; Jump not taken,

;  so this line is executed

cmp     ax, bx  ; Set flags for parity check

jp      peven   ; Jump if parity flag is set

;  (parity even)

podd:   add     cx, 2   ; Odd number of set bits in

;  low byte of result

peven:  add     cx, 2   ; Even number of set bits in

;  low byte of result

exit:   .EXIT           ; Terminate program

Control-Flow Instructions

END

1.5.6. LOOP

Loop                                 Flags:  O D I T S Z A P C

-----------------

Syntax:  LOOP label

Loops repeatedly to a specified label.  LOOP  decrements  CX

(without changing any flags)  and  if  the  result  is  not  0,

transfers execution to the address specified by the operand. If

CX is 0 after being decremented,  execution  continues  at  the

next instruction.  The  operand  must  specify  a  short  label

(between -128 and 127 bytes from the instruction following  the

LOOP instruction).

11100010       disp (1)

------------------+-------------------------+------------------

LOOP  label      | loop  wend              | 88/86 17,noj=5

|                         | 286   8+m,noj=4

|                         | 386   11+m

------------------+-------------------------+------------------

1.5.7. WriteTTY

;* WriteTTY - Displays ASCIIZ string at cursor position,

;* in either text or graphics mode.

;*

;* Shows:   BIOS Interrupt - 10h, Function 0Eh

;*           (Write Character in TTY mode)

;*          Instruction - loop

;*

;* Uses:    vconfig - Video configuration structure, declared

;*          in the DEMO.INC include file.

;*          The structure must first be initialized

;*          by calling the GetVidConfig procedure.

;*

;* Params:  str - Pointer to ASCIIZ string

;*          icolor - Color index (for graphics mode only)

;*

;* Return:  None

WriteTTY PROC \

USES ds si, \

str:PTR BYTE, icolor:WORD

mov     bx, icolor          ; BL = color index

mov     bh, vconfig.dpage   ; BH = current display page

LoadPtr ds, si, str

mov     cx, -1          ; Set loop counter to maximum

mov     ah, 14          ; Function 14

loop1:  lodsb                   ; Get character from string

or      al, al          ; NULL string terminator?

jz      exit            ; Yes?  Exit

int     10h             ; No?  Display, advance cursor

Control-Flow Instructions

loop    loop1           ; Loop to get next character

exit:   ret

WriteTTY ENDP

1.5.8. LOOPcondition

Loop If                              Flags:  O D I T S Z A P C

-----------------

Syntax:  LOOPE label

LOOPNE label

Loops repeatedly to a specified label if  condition  is  met

and if CX is not 0.  The  instruction  decrements  CX  (without

changing any flags) and tests to see if the zero flag  was  set

by a previous instruction (such as CMP). With LOOPE  and  LOOPZ

(they are synonyms), execution is transferred to the  label  if

the zero flag is set and CX is not 0. With  LOOPNE  and  LOOPNZ

(they are synonyms), execution is transferred to the  label  if

the zero flag is cleared and CX is not 0.  Execution  continues

at the next instruction if the condition  is  not  met.  Before

entering the loop, CX should be set to the  maximum  number  of

repetitions desired.

11100001       disp (1)

-----------------+-------------------------+-------------------

LOOPE label     | loopz again             | 88/86     18,noj=6

LOOPZ label     |                         | 286       8+m,noj=

|                         | 386       11+m

-----------------+-------------------------+-------------------

11100000       disp (1)

-----------------+-------------------------+-------------------

LOOPNE label    | loopnzfor_next          | 88/86     19,noj=5

LOOPNZ label    |                         |   286     8,noj=4

|                         |   386     11+m

-----------------+-------------------------+-------------------

1.5.9. CALL

Call Procedure                       Flags:  O D I T S Z A P C

-----------------

Syntax:  CALL label

Calls a procedure. The instruction does this by pushing  the

address of the next instruction onto the stack and transferring

to the address specified by the operand. For NEAR calls, SP  is

decreased by 2, the offset (IP) is pushed, and the  new  offset

is loaded into IP.

For FAR calls, SP is decreased by 2,  the  segment  (CS)  is

pushed, and the new segment is  loaded  into  CS.  Then  SP  is

decreased by 2 again, the offset (IP) is pushed,  and  the  new

offset is loaded into IP. A subsequent RET instruction can  pop

the address so that execution continues  with  the  instruction

following the call.

Control-Flow Instructions

11101000       disp (2)

-----------------+-------------------------+-------------------

CALL  label     | call  upcase            | 88/86     19 (88=2

|                         | 286       7+m

|                         | 386       7+m

-----------------+-------------------------+-------------------

10011010       disp (4)

-------------+---------------------+---------------------------

CALL  label | call  FAR PTR job   | 88/86     28 (88=36)

| call  distant       | 286       13+m,pm=26+m

|                     | 386       17+m,pm=34+m

-------------+---------------------+---------------------------

11111111       mod,010,r/m      disp(0 or 2)

-------------+---------------------+---------------------------

CALL  reg   | call  ax            | 88/86     16 (88=20)

|                     | 286       7+m

|                     | 386       7+m

-------------+---------------------+---------------------------

CALL  mem16 | call  pointer       | 88/86     21+EA (88=29+EA)

| call  [bx]          | 286       11+m

|                     | 386       10+m

-------------+---------------------+---------------------------

11111111       mod,011,r/m      disp(4)

-------------+---------------------+--------------------------

CALL  mem32 | call  far_table[di] | 88/86     37+EA (88=53+EA)

| call  DWORD PTR [bx]| 286       16+m,pm=29+m

|                     | 386       22+m,pm=38+m

-------------+---------------------+--------------------------

1.5.10. HELLOM.ASM

Control-Flow Instructions

;* HELLOM.ASM illustrates multiple-modules and memory model

;* independent techniques. To create the program, paste this

;* module and PUTSTR.ASM (get help on PUTSTR.ASM to open it)

;* into separate program files. In the environment, you must

;* also create a program list for the program and define a mode

;* in the Defines field of the Assembler Flags dialog box.

;* For example, use "model=large". Alternately, you could assem

;* outside the environment with the following command line:

;*      qcl /Dmodel=large hellom.asm putstr.asm

;*

;* Shows:   Instruction - call

;*          Directives - EXTRN

;*          Equates - @DataSize     @CodeSize

;*          Operators - %

; Model is symbol passed from command line or environment.

; Note that the % operator is required to receive

; external constant symbols.

%       .MODEL  model, c

DOSSEG

.STACK  100h

.DATA

msg     DB "Hello, world.",13,10,0  ; Null-terminated string

.CODE

.STARTUP         ; Initialize data and stack segments

; Tell assembler that PutStr is elsewhere (in another module).

EXTRN   PutStr:PROC     ; PROC evalutes to NEAR or FAR

;   code depending on model

IF      @DataSize       ; If data is far (compact, larg

push    ds              ;   or huge), push data segment

ENDIF

mov     ax, OFFSET msg  ; Always push the offset

push    ax

call    PutStr          ; Call external procedure

.EXIT                   ; Exit program

END

1.5.11. RET/RETN/RETF

Return from Procedure                Flags:  O D I T S Z A P C

-----------------

Syntax:  RET [number]

Returns from a  procedure  by  transferring  control  to  an

address popped from the top of the stack.  A  constant  operand

can be given indicating  the  number  of  additional  bytes  to

release. The constant is normally used to adjust the stack  for

arguments pushed before the procedure was called. The size of a

return (near or far) is the size of the procedure in which  the

RET is defined with the PROC directive.

RETN can be used to specify a near return; RETF can  specify

a far return. A near return works by popping a word into IP.  A

Control-Flow Instructions

far return works by popping a word into IP and then  popping  a

word into CS. After the return, the number of  bytes  given  in

the operand (if any) is added to SP.

11000011

------------------+-------------------------+------------------

RET              | ret                     | 88/86 16 (88=20)

RETN             | retn                    | 286   11+m

|                         | 386   10+m

------------------+-------------------------+------------------

11000010    data (2)

------------------+-------------------------+------------------

RET  immed16     | ret   2                 | 88/86 20 (88=24)

RETN  immed16    | retn  8                 | 286   11+m

|                         | 386   10+m

------------------+-------------------------+------------------

11001011

------------------+-------------------------+------------------

RET              | ret                     | 88/86 26 (88=34)

RETF             | retf                    | 286   15+m

|                         | 386   18+m

------------------+-------------------------+------------------

11001010    data (2)

------------------+-------------------------+------------------

RET  immed16     | ret   8                 | 88/86 25 (88=33)

RETF  immed16    | retf  32                | 286   15+m

|                         | 386   18+m

------------------+-------------------------+------------------

1.5.12. INT

Interrupt                            Flags:  O D I T S Z A P C

-----------------

0 0

Syntax:  INT number

Generates a software interrupt. An 8-bit constant operand (0

to 255) specifies the interrupt procedure  to  be  called.  The

call  is  made  by  indexing  the  interrupt  number  into  the

Interrupt Descriptor Table (IDT) starting at segment 0,  offset

0. The IDT contains 4-byte pointers  to  interrupt  procedures.

When an interrupt is called, the flags CS  and  IP  are  pushed

onto the stack (in that order) and the trap and interrupt flags

are cleared. STI can be used to restore interrupts.  To  return

from an interrupt, use the IRET instruction.

Control-Flow Instructions

11001101       data (1)

------------------+-------------------------+------------------

INT  immed8      | int   25h               | 88/86 51 (88=71)

|                         | 286   23+m

|                         | 386   37

------------------+-------------------------+------------------

11001100

------------------+-------------------------+------------------

INT  3           | int   3                 | 88/86 52 (88=72)

|                         | 286   23+m

|                         | 386   33

------------------+-------------------------+------------------

1.5.13. IRET

Interrupt Return                     Flags:  O D I T S Z A P C

Syntax:  IRET                                -----------------

* * * * * * * * *

Returns  control  from  an  interrupt   procedure   to   the

interrupted code. The IRET instruction pops  IP,  CS,  and  the

flags (in that order) and resumes execution.

11001111

------------------+------------------+-------------------------

IRET             | iret             | 88/86 32 (88=44)

|                  | 286   17+m

|                  | 386   22

------------------+------------------+-------------------------

1.5.14. NewBreak

;* NewBreak - A primitive handler for Interrupt 1Bh that can ta

;* control when the Ctrl-Break keyboard sequence is pressed.

;* For an example of how to use such a handler and install its

;* address into the interrupt vector table,

;* see the MiscDemo program and the Exec example procedure.

;*

;* Shows:   BIOS Interrupt - 1Bh (Ctrl-Break Handler Address)

;*          Instruction - iret

NewBreak PROC

sti                 ; Reenable interrupts

push    ax          ; Preserve AX register

mov     al, 20h     ; Send end-of-interrupt signal

out     20h, al     ;   to interrupt controller

pop     ax          ; Recover AX register

iret                ; Return from handler

;   without taking action

NewBreak ENDP

Control-Flow Instructions

1.5.15. ENTER

Make Stack Frame                     Flags:  O D I T S Z A P C

80186/286/386 Only                           -----------------

Syntax:  ENTER bytes,nesting

Creates  a  stack  frame  for  a  procedure  that   receives

parameters passed on the stack. The BP register is  pushed  and

BP is set as the stack frame through which parameters and local

variables can be accessed.  The  first  operand  of  the  ENTER

instruction specifies the number of bytes to reserve for  local

variables. The second operand specifies the nesting  level  for

the procedure. The nesting level should be 0 for languages that

do  not  allow  access  to  local  variables  of  higher  level

procedures  (such  as  C,  BASIC,   and   FORTRAN).   See   the

complementary instruction LEAVE for a method of exiting from  a

procedure.

11001000       data (2)     data (1)

------------------------+----------------------+---------------

ENTER  immed16,0       | enter 4,0            | 88/86 -

|                      | 286   11

|                      | 386   10

------------------------+----------------------+---------------

ENTER  immed16,1       | enter 0,1            | 88/86 -

|                      | 286   15

|                      | 386   12

------------------------+----------------------+---------------

ENTER  immed16,immed8  | enter 6,4            | 88/86 -

|                      | 286   12+4(n-1

|                      | 386   15+4(n-1

------------------------+----------------------+---------------

1.5.16. LEAVE

High Level Procedure Exit            Flags:  O D I T S Z A P C

80186/286/386 Only                           -----------------

Syntax:  LEAVE

Terminates the stack frame of a  procedure.  LEAVE  reverses

the action of a previous ENTER instruction by restoring SP  and

BP to the values they had before the procedure stack frame  was

initialized.

11001001

------------------+-------------------------+------------------

LEAVE            | leave                   | 88/86 -

|                         | 286   5

|                         | 386   4

------------------+-------------------------+------------------

Data Transfer Instructions

1.6. Data Transfer Instructions

Mnemonic

Name

MOV

Move Data

MOVS

Move String Data

XCHG

Exchange (switch values between two operands)

LODS

Load String Data

STOS

Store String Data

LEA

Load Effective Address (calculate address of memory

operand)

LES

Load Far Pointer to Extra Segment (ES)

LDS

Load Far Pointer to Data Segment (DS)

XLAT/XLATB

Translate (translate a value by using  a  table  in

memory)

1.6.1. MOV

Move Data

Flags:  O D I T S Z A P C

Syntax:  MOV dest,src                        -----------------

Copies the value in the source  operand  to  the

destination operand. If the destination operand  is

SS, then interrupts are  disabled  until  the  next

instruction is executed (except on  early  versions

of the 8088 and 8086).

100010dw       mod, reg, r/m     disp (0 or 2)

-------------+-------------------------+-----------------------

MOV  reg,reg| mov   dh,bh             | 88/86 2

| mov   dx,cx             | 286   2

| mov   bp,sp             | 386   2

-------------+-------------------------+-----------------------

MOV  mem,reg| mov   array[di],bx      | 88/86 9+EA (W88=13+EA)

| mov   count,cx          | 286   3

|                         | 386   2

-------------+-------------------------+-----------------------

MOV  reg,mem| mov   bx,pointer        | 88/86 8+EA (W88=12+EA)

| mov   dx,matrix[bx+di]  | 286   5

|                         | 386   4

-------------+-------------------------+-----------------------

1100011w       mod, 000,r/m      disp (0 or 2)     data (1 or

-----------------+----------------------+----------------------

MOV  mem,immed  | mov   [bx],15        | 88/86 10+EA (W88=14+E

| mov   color,7        | 286   3

|                      | 386   2

-----------------+----------------------+----------------------

Data Transfer Instructions

1011w reg      data (1 or 2)

----------------------+-------------------------+--------------

MOV  reg,immed       | mov   cx,256            | 88/86 4

| mov   dx,OFFSET string  | 286   2

|                         | 386   2

----------------------+-------------------------+--------------

101000dw       disp (0 or 2)

------------------+---------------------+----------------------

MOV  mem,accum   | mov   total,ax      | 88/86 10 (W88=14)

| mov   [di],al       | 286   3

|                     | 386   2

------------------+---------------------+----------------------

MOV  accum,mem   | mov   al,string[bx] | 88/86 10 (W88=14)

| mov   ax,fsize      | 286   5

|                     | 386   4

------------------+---------------------+----------------------

100011d0       mod,sreg, r/m     disp (0 or 2)

------------------+---------------------+----------------------

MOV  segreg,reg16| mov   ds,ax         | 88/86 2

|                     | 286   2

|                     | 386   2

------------------+---------------------+----------------------

MOV  segreg,mem16| mov   es,psp        | 88/86 8+EA (88=12+EA)

|                     | 286   5

|                     | 386   5

------------------+---------------------+----------------------

MOV  reg16,segreg| mov   ax,ds         | 88/86 2

|                     | 286   2

|                     | 386   2

------------------+---------------------+----------------------

MOV  mem16,segreg| mov   stack_save,ss | 88/86 9+EA (88=13+EA)

|                     | 286   3

|                     | 386   2

------------------+---------------------+----------------------

1.6.2. MOVS/MOVSB/MOVSW

Move String Data                     Flags:  O D I T S Z A P C

-----------------

Syntax:  MOVS dest,src

MOVSB

MOVSW

Moves a  string  from  one  area  of  memory  to

another. The source string must be  pointed  to  by

DS:SI and the destination address must  be  pointed

to by ES:DI (even if operands are given). For  each

element moved, DI and SI are adjusted according  to

the size of the operands  and  the  status  of  the

direction flag. They are increased if the direction

flag has been cleared with CLD, or decreased if the

direction flag has been set with STD.

If the MOVS form of  the  instruction  is  used,

operands must be provided to indicate the  size  of

the  data  elements  to  be  processed.  A  segment

override can be given for the source  operand  (but

not for the destination). If MOVSB (bytes) or MOVSW

(words) is used,  the  instruction  determines  the

Data Transfer Instructions

size of the data elements to be processed. Operands

are not allowed in MOVSB or MOVSW.

MOVS and its variations are  usually  used  with

the REP  prefix.  Before  a  move  using  a  repeat

prefix, CX should contain the number of elements to

move.

1010010w

-----------------------------+----------------------+----------

MOVS [ES:]dest,[segreg:]src | rep   movsb          | 88/86 18

|                      |  (W88=26)

MOVSB                       | movs  dest,es:source | 286   5

MOVSW                       |                      | 386   7

-----------------------------+----------------------+----------

1.6.3. WinOpen

;* WinOpen - Saves portion of screen to allocated memory,

;* then opens a window with specified fill attribute.

;* See also the WinClose procedure.

;*

;* Shows:   DOS Function - 48h (Allocate Memory Block)

;*          BIOS Interrupt - 10h,

;*           Function 6 (Initialize or Scroll Up Window)

;*          Instructions - movsw     stosw     rep

;*

;* Uses:    vconfig - Video configuration structure, declared

;*          in the DEMO.INC include file.

;*          The structure must first be initialized

;*          by calling the GetVidConfig procedure.

;*

;* Params:  row1 - Row at top of window

;*          col1 - Column at left edge of window

;*          row2 - Row at bottom of window

;*          col2 - Column at right edge of window

;*          attr - Fill attribute for window

;*

;* Return:  Short integer with segment address of allocated

;*          buffer, or 0 if unable to allocate memory

WinOpen PROC \

USES ds di si, \

row1:WORD, col1:WORD, row2:WORD, col2:WORD, attr:WORD

GetVidOffset row1, col1 ; Get offset in video segment

mov     si, ax          ; SI = video offset for window

mov     bx, row2

sub     bx, row1

inc     bx              ; BX = number of window rows

mov     cx, col2

sub     cx, col1

inc     cx              ; CX = number of columns

mov     ax, cx          ; Compute number of video

mul     bl              ;   cells in window

add     ax, 3           ; Plus 3 additional entries

shr     ax, 1           ; Shift right 3 times to

shr     ax, 1           ;   multiply by 2 bytes/cell,

shr     ax, 1           ;   divide by 16 bytes/para

Data Transfer Instructions

inc     ax              ; Add a paragraph

push    bx              ; Save number of rows

mov     bx, ax          ; BX = number of paragraphs

mov     ah, 48h         ; Request DOS Function 48h

int     21h             ; Allocate Memory Block

pop     bx

jnc     @F              ; If successful, continue

sub     ax, ax          ; Else return null pointer

jmp     SHORT exit

@@:     mov     es, ax          ; Point ES:DI to allocated

sub     di, di          ;   buffer

mov     ax, si

stosw                   ; Copy video offset to buffer

mov     ax, bx

stosw                   ; Number of rows to buffer

mov     ax, cx

stosw                       ; Number of cols to buffer

mov     ax, 160             ; Number of video cells/row

mov     ds, vconfig.sgmnt   ; DS = video segment

loop1:  push    si                  ; Save ptr to start of line

push    cx                  ;   and number of columns

cmp     vconfig.adapter, CGA    ; CGA adapter?

jne     @F                  ; No?  Skip video disable

For CGA adapters, WinOpen avoids  screen  "snow"

by disabling the video prior  block  memory  moves,

then reenabling it.  Although  this  technique  can

result in brief  flickering,  it  demonstrates  the

fastest way to access a  block  in  the  CGA  video

buffer without causing display snow. See  also  the

StrWrite procedure  for  another  solution  to  the

problem of CGA snow.

call    DisableCGA      ; Yes?  Disable video

@@:     rep     movsw           ; Copy one row to buffer

cmp     vconfig.adapter, CGA

jne     @F

call    EnableCGA       ; Reenable CGA video

@@:     pop     cx              ; Recover number of columns

pop     si              ;   and start of line

add     si, ax          ; Point to start of next line

dec     bx              ; Decrement row counter

jnz     loop1           ; Loop while rows remain

; Screen contents (including display attributes)

; are now copied to buffer.

; Next open window, overwriting the screen portion just saved.

mov     ax, 0600h           ; Scroll service

mov     bh, BYTE PTR attr   ; Fill attribute

mov     cx, col1        ; CX = row/col for upper left

mov     ch, BYTE PTR row1

mov     dx, col2        ; DX = row/col for lower right

mov     dh, BYTE PTR row2

int     10h             ; Blank window area on screen

mov     ax, es          ; Return address of allocated

exit:   ret                     ;   segment

WinOpen ENDP

Data Transfer Instructions

1.6.4. XCHG

Exchange                             Flags:  O D I T S Z A P C

-----------------

Syntax:  XCHG dest,src

Exchanges  the  values   of   the   source   and

destination operands.

1000011w    mod,reg,r/m      disp (0 or 2)

-----------------+---------------------+-----------------------

XCHG  reg,reg   | xchg   cx,dx        | 88/86 4

| xchg   l,dh         | 286   3

| xchg   al,ah        | 386   3

-----------------+---------------------+-----------------------

XCHG  reg,mem   | xchg   [bx],ax      | 88/86 17+EA (W88=25+EA

XCHG  mem,reg   | xchg   bx,pointer   | 286   5

|                     | 386   5

-----------------+---------------------+-----------------------

10010 reg

---------------------+------------------+----------------------

XCHG  accum,reg16   | xchg  ax,cx      |  88/86 3

XCHG  reg16,accum   | xchg  cx,ax      |  286   3

|                  |  386   3

---------------------+------------------+----------------------

1.6.5. IntToAsc

;* IntToAsc - Converts integer to ASCII string.

;* This procedure is useful only for assembly language,

;* and is not intended to be C-callable.

;*

;* Shows:   Instructions - cwd     aam     xchg

;*

;* Entry:   AX = integer (9999 max)

;*

;* Return:  DX:AX = 4-digit ASCII number

IntToAsc PROC

cwd                     ; Zero DX register

mov     cx, 100         ; Divide AX by 100, yields

div     cx              ;   AX=quotient, DX=remainder

aam                     ; Make digits unpacked BCD

or      ax, '00'        ; Convert to ASCII

xchg    ax, dx          ; Do same thing for DX

aam

or      ax, '00'

ret                     ; Return DX:AX = ASCII number

IntToAsc ENDP

Data Transfer Instructions

1.6.6. LODS/LODSB/LODSW

Load String Operand                  Flags:  O D I T S Z A P C

-----------------

Syntax: LODS src

LODSB

LODSW

Loads a string from memory into the  accumulator

register. The string to be loaded is the source and

must be pointed to by DS:SI (even if an operand  is

given). For  each  source  element  loaded,  SI  is

adjusted according to the size of the operands  and

the status of the direction flag. SI  is  increased

if the direction flag has been cleared with CLD  or

decreased if the direction flag has been  set  with

STD.

If the LODS form of the instruction is used,  an

operand must be provided to indicate  the  size  of

the  data  elements  to  be  processed.  A  segment

override can be given. If LODSB  (bytes)  or  LODSW

(words) is used,  the  instruction  determines  the

size of the  data  elements  to  be  processed  and

whether the element will be loaded  to  AL  or  AX.

Operands are not allowed with LODSB or LODSW.

LODS and its variations are  not  normally  used

with repeat prefixes, since there is no  reason  to

repeatedly load memory values to a register.

1010110w

----------------------+-------------------+--------------------

LODS  [segreg:]src   | lods  es:source   | 88/86 12 (W88=16)

LODSB                | lodsw             | 286   5

LODSW                |                   | 386   5

----------------------+-------------------+--------------------

1.6.7. WinClose

;* WinClose - "Closes" a window previously opened by the WinOpe

;* procedure. See also the WinOpen procedure.

;*

;* Shows:   DOS Function - 49h (Release Memory Block)

;*          Instructions - lodsw

;*          Operators - : (segment override)     SEG

;*

;* Uses:    vconfig - Video configuration structure, declared i

;*          the DEMO.INC include file. The structure must first

;*          be initialized by calling the GetVidConfig procedur

;*

;* Params:  addr - Segment address of buffer that holds screen

;*                 contents saved in WinOpen procedure

;*

;* Return:  None

WinClose PROC \

USES ds di si, \

addr:WORD

Data Transfer Instructions

mov     ds, addr            ; DS:SI points to buffer

sub     si, si

lodsw

mov     di, ax              ; DI = video offset of wind

lodsw

mov     bx, ax              ; BX = number of window row

lodsw

mov     cx, ax              ; CX = number of columns

mov     ax, SEG vconfig.sgmnt

mov     es, ax              ; Point ES to data segment

push    es:vconfig.sgmnt

pop     es                  ; ES = video segment

mov     ax, 160             ; Number of video cells/row

loop1:  push    di                  ; Save ptr to start of line

push    cx                  ;   and number of columns

cmp     vconfig.adapter, CGA    ; CGA adapter?

jne     @F                  ; No?  Skip video disable

; Disable CGA video prior to memory move to avoid screen snow.

; (See the WinOpen and StrWrite procedures for further

; discussions on CGA snow.)

call    DisableCGA          ; Yes?  Disable video

@@:     rep     movsw               ; Copy one row to buffer

cmp     vconfig.adapter, CGA

jne     @F

call    EnableCGA           ; Reenable CGA video

@@:     pop     cx                  ; Recover number of columns

pop     di                  ;   and start of line

add     di, ax              ; Point to start of next li

dec     bx                  ; Decrement row counter

jnz     loop1               ; Loop while rows remain

mov     ah, 49h             ; Request DOS Function 49h

mov     es, addr

int     21h                 ; Release Memory Block

ret

WinClose ENDP

1.6.8. STOS/STOSB/STOSW

Store String Data                    Flags:  O D I T S Z A P C

-----------------

Syntax: STOS dest

STOSB

STOSW

Stores the value in the accumulator to a string.

The string to be filled is the destination and must

be pointed to by  ES:DI  (even  if  an  operand  is

given). For  each  source  element  loaded,  DI  is

adjusted according to the size of the operands  and

the status of the direction flag. DI  is  increased

if the direction flag has been cleared with CLD  or

decreased if the direction flag has been  set  with

STD.

Data Transfer Instructions

If the STOS form of the instruction is used,  an

operand must be provided to indicate  the  size  of

the data  elements  to  be  processed.  No  segment

override is allowed.  If  STOSB  (bytes)  or  STOSW

(words) is used,  the  instruction  determines  the

size of the  data  elements  to  be  processed  and

whether the element will  be  from  AL  or  AX.  No

operand is allowed with STOSB or STOSW.

STOS and its variations are often used with  the

REP prefix.  Before  the  repeated  instruction  is

executed, CX should contain the number of  elements

to store.

1010101w

------------------+------------------------+-------------------

STOS  [ES:]dest  | stos  es:dstring       | 88/86 11 (W88=15)

STOSB            | rep    stosw           | 286   3

STOSW            | rep    stosb           | 386   4

------------------+------------------------+-------------------

1.6.9. LEA

Load Effective Address               Flags:  O D I T S Z A P C

-----------------

Syntax:  LEA reg,mem

Calculates the effective address (offset) of the

source memory operand and stores  the  result  into

the destination register.

10001101    mod, reg, r/m      disp (2)

------------------+-------------------------+------------------

LEA  reg,mem     | lea   bx,npointer       | 88/86 2+EA

|                         | 286   3

|                         | 386   2

------------------+-------------------------+------------------

1.6.10. VeriAnsi

;* VeriAnsi - Checks for ANSI driver by writing ANSI sequence

;* to report cursor position. If report compares with position

;* returned from GetCurPos procedure,

;* then ANSI driver is operating.

;*

;* Shows:   DOS Functions - 06h (Direct Console I/O)

;*                          0Ch (Flush Input Buffer then Input)

;*

;* Params:  None

;*

;* Return:  Short integer, 1 for yes or 0 for no

.DATA

PUBLIC report

report  DB      ESCAPE, '[6n$'  ; ANSI Report Cursor sequence

.CODE

Data Transfer Instructions

VeriAnsi PROC

mov     al, ch          ; Get original row # in AL

cbw                     ; AX = row # from GetCurPos

inc     ax              ; Add 1 to it

call    IntToAsc        ; Make ASCII digits

cmp     ax, bx          ; ANSI and BIOS reports match?

jne     e_exit          ; No?  Then ANSI not loaded

mov     ax, 0C06h       ; Flush remaining ANSI keys

mov     dl, 0FFh        ;   from buffer

int     21h

mov     ax, 1           ; Set 1 for true

jmp     SHORT exit      ;   and exit

e_exit: sub     ax, ax          ; Set 0 return code if no

exit:   ret                     ;   ANSI driver installed

VeriAnsi ENDP

1.6.11. LDS/LES

Load Far Pointer                     Flags:  O D I T S Z A P C

Syntax:  LDS reg,mem                         -----------------

Reads and stores the far  pointer  specified  by

the source memory operand.  The  pointer's  segment

value is stored in  the  segment  register  segment

specified by the instruction name. The offset value

is  stored  in  the  register  specified   by   the

destination operand. The LDS and  LES  instructions

are available on all processors.

11000101     mod, reg, r/m      disp (2)

--------------+---------------------+--------------------------

LDS  reg,mem | lds   si,fpointer   | 88/86 16+EA (88=24+EA)

|                     | 286   7

|                     | 386   7

--------------+---------------------+--------------------------

11000100     mod, reg, r/m      disp (2)

--------------+---------------------+--------------------------

LES  reg,mem | les   di,fpointer   | 88/86 16+EA (88=24+EA)

|                     | 286   7

|                     | 386   7

--------------+---------------------+--------------------------

1.6.12. LoadPtr

;* LoadPtr - Macro to load far address into segment:register

;* pair, or near address into register.

;*

;* Params:  sgmnt - Segment to be loaded with segment address

;*          reg - Register to be loaded with offset address

Data Transfer Instructions

;*          ptr - Pointer to address

;*

;* Shows:   Instructions - lds   les

;*          Directives - MACRO   IF      IFIDNI   ELSE    ELSEI

;*                       ENDIF   .ERR    %OUT     EXITM   ENDM

;*          Operators - < >       ;;

LoadPtr MACRO sgmnt, reg, ptr   ;; Macro definition

IF @DataSize                ;; If far pointer, and

IFIDNI <sgmnt>, <ds>    ;;   if 1st argument is DS,

lds reg, ptr        ;;   load DS:reg with far addre

EXITM

ENDIF

IFIDNI <sgmnt>, <es>    ;;   or if 1st argument is ES,

les reg, ptr        ;;   load ES:reg with far addre

EXITM

ENDIF

.ERR                    ;; Generate error if not DS or

%OUT 1st macro argument must be DS or ES

ELSE                        ;; If near pointer,

IFIDNI <sgmnt>, <es>    ;;   and if segment is ES,

push ds             ;;   ensure ES points to

pop  es             ;;   same segment as DS

ENDIF

mov reg, ptr        ;; Then load reg with near address

ENDIF

ENDM

1.6.13. XLAT/XLATB

Translate                            Flags:  O D I T S Z A P C

-----------------

Syntax:  XLAT [table]

Translates a value from  one  coding  system  to

another by looking up the value to be translated in

a table stored in memory. Before the instruction is

executed, BX should point to a table in memory  and

AL should contain  the  unsigned  position  of  the

value to be translated from the  table.  After  the

instruction, AL will contain the table  value  with

the specified position. No operand is required, but

one can be given in  order  to  specify  a  segment

override. DS is assumed unless a  segment  override

is given. XLATB is  recognized  as  a  synonym  for

XLAT.

11010111

-------------------------+----------------------+--------------

XLAT  [[segreg]:mem]    | xlat                 | 88/86 11

XLATB  [[segreg]:mem]   | xlatb  es:table      | 286   5

|                      | 386   5

-------------------------+----------------------+--------------

Data Transfer Instructions

1.6.14. BinToHex

;* BinToHex - Converts binary word to 6-byte hexadecimal

;*  number in ASCIIZ string.

;* String is right-justified and includes "h" radix.

;*

;* Shows:   Instruction - xlat

;*

;* Params:  num - Number to convert to hex string

;*          str - Pointer to 6-byte string

;*

;* Return:  None

.DATA

hex     DB      '0123456789ABCDEF'      ; String of hex numbers

.CODE

BinToHex PROC \

USES di, \

num:WORD, str:PTR BYTE

LoadPtr es, di, str     ; Point ES:DI to 6-byte string

mov     bx, OFFSET hex  ; Point DS:BX to hex numbers

mov     ax, num         ; Number in AX

mov     cx, 2           ; Loop twice for two bytes

loop1:  xchg    ah, al          ; Switch bytes

push    ax              ; Save number

shr     al, 1           ; Shift high nibble to low

shr     al, 1

shr     al, 1

shr     al, 1

xlat                ; Get equivalent ASCII number in AL

stosb               ; Copy to 6-byte string, increment

pop     ax          ; Recover number

push    ax          ; Save it again

and     al, 00001111b   ; Mask out high nibble

xlat                ; Get equivalent ASCII number in AL

stosb               ; Copy to 6-byte string, increment

pop     ax          ; Recover number

loop    loop1       ; Do next byte

mov     ax, 'h'     ; Put null, 'h' radix in AX

stosw               ; Copy to last two bytes in string

ret

BinToHex ENDP

1.7. Flag-Setting Instructions

Mnemonic

Name

CLC

Clear Carry Flag

CLD

Clear Direction Flag

CLI

Clear Interrupt Flag (disable maskable  interrupts)

Flag-Setting Instructions

CMC

Complement Carry Flag

STC

Set Carry Flag

STD

Set Direction Flag

STI

Set Interrupt Flag (recognize interrupts again)

POPF

Pop Flags (pop value at top  of  stack  into  flags

register)

PUSHF

Push Flags (push value  of  entire  flags  register

onto stack)

LAHF

Load Flags into AH Register

SAHF

Store AH into Flags (does not include all flags)

1.7.1. CLC

Clear Carry Flag                     Flags:  O D I T S Z A P C

-----------------

Syntax:  CLC                                                 0

Clears the carry flag.

11111000

-----------------+-------------------------+-------------------

CLC             | clc                     | 88/86 2

|                         | 286   2

|                         | 386   2

-----------------+-------------------------+-------------------

1.7.2. CopyFile

;* CopyFile - Copies a file from a specified directory to

;* another. Allows two different copy methods. See the OpenFile

;* CloseFile, ReadFile, and WriteFile procedures for specific

;* examples on opening, closing, reading from,

;* and writing to files.

;*

;* Shows:   DOS Functions - 3Ch (Create File)

;*                          5Bh (Create New File)

;*          Instruction - clc

;*

;* Params:  imode  - 0 = Create new target file or

;*                        overwrite existing file

;*                   1 = Abort and return error code if target

;*                        file already exists (only for DOS

;*                        versions 3.0 and higher)

;*          fspec1 - Pointer to ASCIIZ source file specificatio

;*          fspec2 - Pointer to ASCIIZ target file specificatio

;*

;* Return:  Short integer with error code

;*          0 if successful

Flag-Setting Instructions

;*          1 if error

.DATA

buffer  DB      BUFFER_SIZE DUP(?)  ; Buffer for diskette read

.CODE

EXTRN GetVer:PROC

CopyFile PROC \

USES ds si di, \

imode:WORD, fspec1:PTR BYTE, fspec2:PTR BYTE

LOCAL eof_flag:BYTE

; Open source file for read only

LoadPtr ds, dx, fspec1  ; Point DS:DX to source file

mov     ax, 3D00h   ; AH = function #, AL = access code

int     21h         ; Open File (for read only)

jc      e_exit

mov     si, ax          ; SI = file handle for source

; Open target file according to copy mode

LoadPtr ds, dx, fspec2  ; Point DS:DX to target file

cmp     imode, 1        ; Determine DOS function

je      check           ; Imode = 1?

mov     ah, 3Ch         ; No?  Request Create File

jmp     SHORT set       ;   (destroy existing)

check:  call    GetVer          ; Yes?  First check DOS version

cmp     ax, 300         ; 3.0 or higher?

jb      close           ; No?  Abort with error code

mov     ah, 5Bh         ; Request Create New File

set:    sub     cx, cx          ; Normal attribute for target

int     21h             ; DOS function for target file

jc      close           ; If open error, abort

mov     di, ax          ; DI = file handle for target

; Both files successfully opened.

; Now read from source and copy to target.

mov     ax, @data

mov     ds, ax              ; DS:DX = buffer. Read/writ

mov     dx, OFFSET buffer   ;   to and from here.

mov     eof_flag, 0         ; Initialize end-of-file fl

loop1:  mov     bx, si              ; Handle for source file

mov     cx, BUFFER_SIZE ; CX = number of bytes to read

mov     ah, 3Fh         ; Request DOS read

int     21h             ; Read from File

jc      close           ; If error, exit

cmp     ax, cx          ; All bytes read successfully?

je      @F              ; Yes?  Continue

inc     eof_flag        ; No?  Raise flag

@@:     mov     bx, di          ; Handle for target file

mov     cx, ax          ; Write number of bytes read

mov     ah, 40h         ; Request DOS write

int     21h         ; Write from buffer to target file

jc      close           ; If error, exit

cmp     eof_flag, 0     ; Finished?

je      loop1           ; No?  Loop to read next block

clc                     ; Yes?  Clear CY to indicate

Flag-Setting Instructions

;   success

close:  pushf                   ; Preserve flags while closing

mov     bx, di          ; Handle for target file

mov     ah, 3Eh         ; Request DOS Function 3Eh

int     21h             ; Close File

sub     ax, ax          ; Clear error code

popf                    ; Recover flags

jnc     exit            ; If successful, exit

e_exit: mov     ax, 1           ; Else set error code

exit:   ret

CopyFile ENDP

1.7.3. CLD

Clear Direction Flag                 Flags:  O D I T S Z A P C

-----------------

Syntax:  CLD                                   0

Clears the direction flag. All subsequent string

instructions will process up (from low addresses to

high  addresses),  by  increasing  the  appropriate

index registers.

11111100

-----------------+--------------------------+------------------

CLD             | cld                      | 88/86 2

|                          | 286   2

|                          | 386   2

-----------------+--------------------------+------------------

1.7.4. StrFindChar

;* StrFindChar - Finds first occurence of character in given

;* ASCIIZ string, searching either from beginning or end of

;* string. See StrWrite, WinOpen, WinClose, and StrCompare

;* procedures for other examples of string instructions.

;*

;* Shows:   Instructions - repne     scasb    cld     std

;*

;* Params:  ichar - Character to search for

;*          str - Pointer to ASCIIZ string in which to search

;*          direct - Direction flag:

;*                   0 = search from start to end

;*                   1 = search from end to start

;*

;* Return:  Null pointer if character not found, else pointer

;*           to string where character first encountered

StrFindChar PROC \

USES ds di si, \

ichar:BYTE, str:PTR BYTE, direct:WORD

Flag-Setting Instructions

LoadPtr es, di, str     ; ES:DI points to string

LoadPtr ds, si, str     ;   as does DS:SI

mov     cx, -1          ; Set scan counter to maximum

mov     bx, cx          ; BX = max string tail

cld                     ; Assume head-to-tail search

cmp     direct, 0       ; Assumption correct?

je      loop1           ; Yes?  Continue

mov     bx, di          ; No?  Set BX to byte before

dec     bx              ;   string head and scan

sub     al, al          ;   string for null terminator

push    cx              ;   to find string tail

repne   scasb

pop     cx              ; Recover scan counter

dec     di              ; Backup pointer to last

dec     di              ;   character in string and

mov     si, di          ;   begin search from there

std                     ; Set direction flag

loop1:  lodsb                   ; Get first char from string

cmp     si, bx          ; At head limit?

je      xmatch          ; Yes?  Then no match

or      al, al          ; At tail limit?

je      xmatch          ; Yes?  Then no match

cmp     al, ichar       ; Character match?

je      match           ; Yes?  Then exit

loop    loop1           ; No?  Resume search

xmatch: sub     ax, ax          ; Set null pointer if no match

sub     dx, dx

jmp     SHORT exit

match:  mov     ax, si          ; If match, point to first

dec     ax              ;   occurence

cmp     direct, 0       ; Head-to-tail search?

je      exit            ; Yes?  Then exit

inc     ax              ; No?  Then adjust pointer

inc     ax              ;   forward

mov     dx, ds          ; Pointer segment

exit:   ret

StrFindChar ENDP

1.7.5. CLI

Clear Interrupt Flag                 Flags:  O D I T S Z A P C

Syntax:  CLI                                 -----------------

0

Clears the interrupt flag.  When  the  interrupt

flag  is  cleared,  maskable  interrupts  are   not

recognized until the flag is set again with the STI

instruction.

11111010

-----------------+-------------------------+-------------------

CLI             | cli                     | 88/86 2

|                         | 286   3

|                         | 386   3

-----------------+-------------------------+-------------------

Flag-Setting Instructions

1.7.6. DisableCGA

;* DisableCGA - Disables CGA video by reprogramming

;* the control register.

;*

;* Shows:   Instructions - cli     sti     loopz     loopnz

;*

;* Uses:    vconfig - Video configuration structure, declared

;*          in the DEMO.INC include file. The structure must

;*          first be initialized by calling the GetVidConfig

;*          procedure.

;*

;* Params:  None

;*

;* Return:  None

DisableCGA PROC \

USES ax cx dx                   ; Preserve registers

mov     cx, -1          ; Set maximum loop count

mov     dx, 03DAh       ; Address of status register

wait1:  in      al, dx          ; Get video status

test    al, 8           ; Vertical retrace active?

loopnz  wait1           ; Yes?  Wait for end/timeout

cli                     ; Disallow interruptions

mov     cx, -1          ; Reset loop count

wait2:  in      al, dx          ; Get video status

test    al, 8           ; Start of vertical retrace?

loopz   wait2           ; No?  Wait for start/timeout

sub     dx, 2           ; DX = address of control reg

mov     al, 1           ; Value to disable CGA video

out     dx, al          ; Disable video

sti                     ; Reenable interrupts

ret

DisableCGA ENDP

1.7.7. CMC

Complement Carry Flag                Flags:  O D I T S Z A P C

Syntax:  CMC                                 -----------------

*

Complements (toggles) the carry flag.

11110101

----------------+--------------------------+-------------------

CMC            | cmc                      | 88/86 2

|                          | 286   2

|                          | 386   2

----------------+--------------------------+-------------------

Flag-Setting Instructions

1.7.8. STC

Set Carry Flag

Flags:  O D I T S Z A P C

Syntax:  STC                                 -----------------

1

Sets the carry flag.

11111001

-----------------+-------------------------+-------------------

STC             | stc                     | 88/86 2

|                         | 286   2

|                         | 386   2

-----------------+-------------------------+-------------------

1.7.9. STD

Set Direction Flag                   Flags:  O D I T S Z A P C

-----------------

Syntax:  STD                                   1

Sets the direction flag. All  subsequent  string

instructions will process down (from high addresses

to low addresses).

11111101

-----------------+-------------------------+-------------------

STD             | std                     | 88/86 2

|                         | 286   2

|                         | 386   2

-----------------+-------------------------+-------------------

1.7.10. STI

Set Interrupt Flag                   Flags:  O D I T S Z A P C

-----------------

Syntax:  STI                                     1

Sets the interrupt flag. When the interrupt flag

is set,  maskable  interrupts  are  recognized.  If

interrupts  were  disabled  by   a   previous   CLI

instruction,  pending  interrupts   will   not   be

executed immediately; they will be  executed  after

the instruction following STI.

11111011

-----------------+-------------------------+-------------------

STI             | sti                     | 88/86 2

|                         | 286   2

|                         | 386   3

-----------------+-------------------------+-------------------

Flag-Setting Instructions

1.7.11. POPF

Pop Flags                            Flags:  O D I T S Z A P C

-----------------

* * * * * * * * *

Syntax:  POPF

Pops the value on the top of the stack into  the

flags register. POPF always pops  into  the  16-bit

flags register.

10011101

------------------+-------------------------+------------------

POPF             | popf                    | 88/86 8 (88=12)

|                         | 286   5

|                         | 386   5

------------------+-------------------------+------------------

1.7.12. PUSHF

Push Flags                           Flags:  O D I T S Z A P C

-----------------

Syntax:  PUSHF

Pushes the flags register onto the stack.  PUSHF

always pushes the 16-bit flags register.

10011100

------------------+-------------------------+------------------

PUSHF            | pushf                   | 88/86 10 (88=14)

|                         | 286   3

|                         | 386   4

------------------+-------------------------+------------------

1.7.13. FindFirst

;* FindFirst - Finds first entry in given directory

;* matching specification.

;*

;* Shows:   DOS Function - 4Eh (Find First File)

;*          Keywords - USES

;*          Instructions - ret     pushf    popf

;*

;* Params:  attr - Attribute code

;*           (see header comments for CreateFile)

;*          fspec - Pointer to ASCIIZ file specification

;*          finfo - Pointer to 43-byte buffer to receive

;*                  data from matched entry

;*

;* Return:  Short integer with error code

;*          0 if successful

;*          1 if no match found

.DATA

Flag-Setting Instructions

old_dta DD      WORD PTR ?     ; Storage for old DTA address

.CODE

FindFirst PROC \

USES ds, \

attr:WORD, fspec:PTR BYTE, finfo:PTR BYTE

push    ds                      ; Pass far pointer

mov     ax, OFFSET @data:old_dta;   to old_dta

push    ax

call    GetDTA          ; Get current DTA address

add     sp, 4           ; Adjust stack

mov     cx, attr        ; Load CX with file attribute

LoadPtr ds, dx, finfo   ; DS:DX points to 43-byte buffe

push    ds              ; Make this new DTA

push    dx

call    SetDTA          ; Set 43-byte buffer as DTA

add     sp, 4           ; Adjust stack

LoadPtr ds, dx, fspec   ; Point DS:DX to file spec

mov     ah, 4Eh         ; AH = function number

int     21h             ; Find First File

pushf                           ; Preserve flags

push    WORD PTR @data:old_dta[2] ; Pass far pointer to

push    WORD PTR @data:old_dta[0] ;   SetDTA procedure

call    SetDTA          ; Restore DTA address to orig

sub     ax, ax          ; Set error code

add     sp, 4           ; Adjust stack

popf                    ; Recover flags

jnc     exit            ; Exit if successful match

inc     ax              ; Else set error code to 1

exit:   ret

FindFirst ENDP

1.7.14. LAHF

Load Flags into AH Register          Flags:  O D I T S Z A P C

Syntax:  LAHF                                -----------------

Transfers bits 0 to 7 of the flags  register  to

AH. This  includes  the  carry,  parity,  auxiliary

carry, zero, and sign  flags,  but  not  the  trap,

interrupt, direction, or overflow flags.

10011111

------------------+-------------------------+------------------

LAHF             | lahf                    | 88/86 4

|                         | 286   2

|                         | 386   2

------------------+-------------------------+------------------

Flag-Setting Instructions

1.7.15. SAHF

Store AH into Flags                  Flags:  O D I T S Z A P C

-----------------

Syntax:  SAHF                                        * * * * *

Transfers AH into bits  0  to  7  of  the  flags

register.  This   includes   the   carry,   parity,

auxiliary carry, zero, and sign flags, but not  the

trap, interrupt, direction, or overflow flags.

10011110

-----------------+-------------------------+-------------------

SAHF            | sahf                    | 88/86 4

|                         | 286   2

|                         | 386   3

-----------------+-------------------------+-------------------

1.7.16. Quadratic

;* Quadratic - Solves for the roots of

;*  a quadratic equation of form

;*                        A*x*x + B*x + C = 0

;* using floating-point instructions. This procedure requires

;* either a math coprocessor or emulation code. If executing

;* within the QuickAssembler environment, emulation is

;* automatically provided if a coprocessor is not installed.

;* If executing from the QCL command line, the /FPi switch must

;* be specified if a coprocessor is not installed. For example,

;* to create the MATHDEMO.EXE example program with floating-poi

;* emulation, enter the following line:

;*                 QCL /Cx mathdemo.c /FPi math.asm common.asm

;*

;* Shows: Instructions - sahf    fld1    fld    fadd    fmul

;*                       fxch    fsubr   fchs   fsubp   fstp

;*                       fst     fdivr   fwait  ftst

;*

;* Params:  a - Constant for 2nd-order term

;*          b - Constant for 1st-order term

;*          c - Equation constant

;*          r1 - Pointer to 1st root

;*          r2 - Pointer to 2nd root

;*

;* Return:  Short integer with return code

;*          0 if both roots found

;*          1 if single root (placed in r1)

;*          2 if indeterminate

Quadratic PROC \

USES ds di si, \

a:DWORD, b:DWORD, c:DWORD, r1:PTR DWORD, r2:PTR DWORD

LOCAL status:WORD       ; Intermediate status

LoadPtr es, di, r1      ; ES:DI points to 1st root

LoadPtr ds, si, r2      ; DS:SI points to 2nd root

sub     bx, bx          ; Clear error code

Flag-Setting Instructions

fld1                    ; Load top of stack with 1

fadd    st, st          ; Double it to make 2

fld     st              ; Copy to next register

fmul    a               ; ST register = 2a

ftst                    ; Test current ST value

fstsw   status          ; Copy status to local word

fwait                   ; Ensure coprocessor is done

mov     ax, status      ; Copy status into AX

sahf                    ; Load flag register

jnz     @F          ; If C3 set, a = 0, in which case

;   solution is x = -c / b

fld     b               ; Load b parameter

ftst                    ; Test current ST value

fstsw   status          ; Copy status to local word

fwait                   ; Ensure coprocessor is done

mov     ax, status      ; Copy status into AX

sahf                    ; Load flag register

jz      exit2       ; If C3 set, b = 0, so don't divide

fld     st              ; Copy b to next register

fld     c               ; Load C parameter

fchs                    ; Reverse sign

fxch                    ; Exchange ST and ST(1)

fdiv                        ; Divide c by b

fst     DWORD PTR es:[di]   ; Copy result

jmp     SHORT exit1         ; Return with code = 1

@@:     fmul    st(1), st       ; ST(1) register = 4a

fxch                    ; Exchange ST and ST(1)

fmul    c               ; ST register = 4ac

ftst                    ; Test current ST value

fstsw   status          ; Copy status to local word

fwait                   ; Ensure coprocessor is done

mov     ax, status      ; Copy status into AX

sahf                    ; Load flag register

jp      exit2           ; If C2 set, 4*a*c is infinite

fld     b               ; Else load b parameter

fmul    st, st          ; Square it; ST register = b*b

fsubr                   ; ST register = b*b - 4*a*c

ftst                    ; Test current ST value

fstsw   status          ; Copy status to local word

fwait                   ; Ensure coprocessor is done

mov     ax, status      ; Copy status into AX

sahf                    ; Load flag register

jc      exit2           ; If C0 set, b*b < 4ac

jnz     @F              ; If C3 set, b*b = 4ac, in whic

inc     bx              ;   case only 1 root so set fla

@@:     fsqrt                   ; Get square root

fld     b               ; Load b parameter

fchs                    ; Reverse sign

fxch                    ; Exchange ST and ST1

fld     st              ; Copy square root to next reg

fadd    st, st(2)       ; ST = -b + sqrt(b*b - 4*a*c)

fxch                    ; Exchange ST and ST1

fsubp   st(2), st       ; ST = -b - sqrt(b*b - 4*a*c)

fdiv    st, st(2)           ; Divide 1st dividend by 2*

fstp    WORD PTR es:[di]    ; Copy result, pop stack

fdivr                       ; Divide 2nd dividend by 2*

fstp    WORD PTR ds:[si]    ; Copy result, pop stack

jmp     SHORT exit          ; Return with code

Flag-Setting Instructions

exit2:  inc     bx          ; Error code = 2 for indeterminancy

exit1:  inc     bx          ; Error code = 1 for single root

exit:   mov     ax, bx

ret

Quadratic ENDP

1.8. Logic Instructions

The logic operations all perform bitwise operations  between

each bit in the source operand and the corresponding bit in the

destination operand.

Mnemonic

Name

AND

And

OR

Inclusive OR

XOR

Exclusive OR

NOT

One's Complement Negation

1.8.1. AND

Logical AND                          Flags:  O D I T S Z A P C

-----------------

Syntax:  AND dest,src                        0       * * ? * 0

Performs a bitwise logical AND on the source and destination

operands and stores the result in the destination operand.  For

each bit position in the operands, if both bits are  set,  then

the corresponding bit of the  result  is  set.  Otherwise,  the

corresponding bit of the result is cleared.

001000dw     mod,reg,r/m       disp (0 or 2)

---------------+------------------------+----------------------

AND  reg,reg  | and   dx,bx            | 88/86 3

|                        | 286   2

|                        | 386   2

---------------+------------------------+----------------------

AND  mem,reg  | and   bitmask,bx       | 88/86 16+EA (W88=24+E

| and   [bp+2],dx        | 286   7

|                        | 386   7

---------------+------------------------+----------------------

AND  reg,mem  | and   bx,masker        | 88/86 9+EA (W88=13+EA

| and   dx,marray[bx+di] | 286   7

|                        | 386   6

---------------+------------------------+----------------------

Logic Instructions

100000sw     mod,100,r/m       disp (0 or 2)     data (1 or 2)

---------------+---------------------+-------------------------

AND  reg,immed| and   dx,0F7h       | 88/86 4

|                     | 286   3

|                     | 386   2

---------------+---------------------+-------------------------

AND  mem,immed| and   masker,1001b  | 88/86 17+EA (W88=23+EA)

|                     | 286   7

|                     | 386   7

---------------+---------------------+-------------------------

0010010w     data (1 or 2)

-------------------+-------------------------+-----------------

AND  accum,immed  | and   ax,0B6h           | 88/86     4

|                         | 286       3

|                         | 386       2

-------------------+-------------------------+-----------------

1.8.2. Colors

;* Colors - Alters screen colors within a

;* specified area by using bit or move operations

;* on display attribute bytes in video memory.

;*

;* Shows:   Instructions - not   rol   ror   and   xor   or

;*

;* Params:  logic - Code number, 0 = NOT 2 = ROR 4 = XOR 6 = MO

;*                               1 = ROL 3 = AND 5 = OR

;*          attr - Attribute mask

;*          row1 - Row at top of window

;*          col1 - Column at left edge of window

;*          row2 - Row at bottom of window

;*          col2 - Column at right edge of window

;*

;* Return:  None

Colors  PROC \

USES ds si, \

logic:WORD, attr:WORD, row1:WORD, col1:WORD, row2:WORD, col2:WO

GetVidOffset row1, col1     ; Get offset in video segme

inc     ax

mov     si, ax              ; SI = offset for 1st attr

mov     bx, row2

sub     bx, row1

inc     bx                  ; BX = number of window row

mov     cx, col2

sub     cx, col1

inc     cx                  ; CX = number of columns

mov     ds, vconfig.sgmnt   ; DS = video segment

mov     ax, attr            ; AL = mask for and, xor, a

loop1:  push    si                  ; Save ptr to start of line

push    cx                  ;   and number of columns

cmp     vconfig.adapter, CGA    ; CGA adapter?

jne     @F                  ; No?  Skip video disable

; Disable CGA video prior to memory access to avoid screen

; snow. (See the WinOpen and StrWrite procedures for further

; discussions on CGA snow.)

Logic Instructions

call    DisableCGA          ; Yes?  Disable video

@@:     cmp     logic, 1            ; Rotate left?

jl      c_not               ; If less, do NOT

je      c_rol               ; If equal, do ROL

cmp     logic, 3            ; And?

jl      c_ror               ; If less, do ROR

je      c_and               ; If equal, do AND

cmp     logic, 5            ; Or?

jl      c_xor               ; If less, do XOR

je      c_or                ; If equal, do OR

; Otherwise, do MOV

c_mov:  mov     BYTE PTR [si], al   ; MOV attr parameter

add     si, 2               ;   into attribute byte

loop    c_mov

jmp     SHORT c_done

c_or:   or      BYTE PTR [si], al   ; OR with attr parameter

add     si, 2

loop    c_or

jmp     SHORT c_done

c_xor:  xor     BYTE PTR [si], al   ; XOR with attr parameter

add     si, 2

loop    c_xor

jmp     SHORT c_done

c_and:  and     BYTE PTR [si], al   ; AND with attr parameter

add     si, 2

loop    c_and

jmp     SHORT c_done

c_ror:  ror     BYTE PTR [si], 1    ; Rotate right 1 bit

add     si, 2

loop    c_ror

jmp     SHORT c_done

c_rol:  rol     BYTE PTR [si], 1    ; Rotate left 1 bit

add     si, 2

loop    c_rol

jmp     SHORT c_done

c_not:  not     BYTE PTR [si]       ; Flip bits

add     si, 2

loop    c_not

c_done: cmp     vconfig.adapter, CGA

jne     @F

call    EnableCGA   ; Reenable CGA video

@@:     pop     cx          ; Recover number of columns

pop     si          ; Recover offset for start of line

add     si, 160     ; Point to start of next line

dec     bx          ; Decrement row counter

jnz     loop1       ; Loop while rows remain

ret                 ; Exit when all lines complete

Colors  ENDP

1.8.3. OR

Inclusive OR                         Flags:  O D I T S Z A P C

-----------------

0       * * ? * 0

Syntax:  OR dest,src

Logic Instructions

Performs a bitwise logical OR on the source and  destination

operands and stores the result to the destination operand.  For

each bit position in the operands, if either or both  bits  are

set, the corresponding bit of the result it set. Otherwise, the

corresponding bit of the result is cleared.

000010dw    mod, reg, r/m      disp (0 or 2)

--------------+---------------------+--------------------------

OR  reg,reg  | or    ax,dx         | 88/86 3

|                     | 286   2

|                     | 386   2

--------------+---------------------+--------------------------

OR  mem,reg  | or    [bp+6],cx     | 88/86 16+EA (W88=24+EA)

| or    bits,dx       | 286   7

|                     | 386   7

--------------+---------------------+--------------------------

OR  reg,mem  | or    bx,masker     | 88/86 9+EA (W88=13+EA)

| or    dx,color[di]  | 286   7

|                     | 386   6

--------------+---------------------+--------------------------

100000sw    mod,001, r/m       disp (0 or 2)     data (1 or 2)

--------------+---------------------+--------------------------

OR  reg,immed| or    dx,110110b    | 88/86 4

|                     | 286   3

|                     | 386   2

--------------+---------------------+--------------------------

OR  mem,immed| or    flag_rec,8    | 88/86 (b=17,w=25)+EA

|                     | 286   7

|                     | 386   7

--------------+---------------------+--------------------------

0000110w    data (1 or 2)

------------------+-------------------------+------------------

OR  accum,immed  | or    ax,40h            | 88/86 4

|                         | 286   3

|                         | 386   2

------------------+-------------------------+------------------

1.8.4. XOR

Exclusive OR                         Flags:  O D I T S Z A P C

-----------------

Syntax:  XOR dest,src                        0       * * ? * 0

Performs  a  bitwise  exclusive  OR  on   the   source   and

destination operands and stores the result to the  destination.

For each bit position in the operands, if both bits are set  or

if both bits are cleared, the corresponding bit of  the  result

is cleared. Otherwise, the corresponding bit of the  result  is

set.

Logic Instructions

001100dw    mod,reg,r/m      disp (0 or 2)

-------------+---------------------+---------------------------

XOR  reg,reg| xor    cx,bx        | 88/86 3

| xor    ah,al        | 286   2

|                     | 386   2

-------------+---------------------+---------------------------

XOR  mem,reg| xor    [bp+10],cx   | 88/86 16+EA (W88=24+EA)

| xor    masked,bx    | 286   7

|                     | 386   6

-------------+---------------------+---------------------------

XOR  reg,mem| xor    cx,flags     | 88/86 9+EA (W88=13+EA)

| xor  bl,bitarray[di]| 286   7

|                     | 386   7

-------------+---------------------+---------------------------

100000sw    mod,110,r/m      disp (0 or 2)     data (1 or 2)

-----------------+-------------------------+-------------------

XOR  reg,immed  | xor    bx,10h           | 88/86 4

| xor    bl,1             | 286   3

|                         | 386   2

-----------------+-------------------------+-------------------

XOR  mem,immed  | xor    Boolean,1        | 88/86 17+EA

|                         | (W88=25+EA)

| xor    switches[bx],101b| 286   7

|                         | 386   7

-----------------+-------------------------+-------------------

0011010w    data (1 or 2)

---------------------+---------------------+-------------------

XOR  accum,immed    | xor    ax,01010101b | 88/86 4

|                     | 286   3

|                     | 386   2

---------------------+---------------------+-------------------

1.8.5. NOT

One's Complement Negation            Flags:  O D I T S Z A P C

-----------------

Syntax:  NOT dest

Toggles each bit of the operand by  clearing  set  bits  and

setting cleared bits.

1111011w    mod, 010,r/m      disp (0 or 2)

--------------+---------------------+--------------------------

NOT  reg     | not   ax            | 88/86 3

|                     | 286   2

|                     | 386   2

--------------+---------------------+--------------------------

NOT  mem     | not   masker        | 88/86 16+EA (W88=24+EA)

|                     | 286   7

|                     | 386   6

--------------+---------------------+--------------------------

Port I/O Instructions

1.9. Port I/O Instructions

Mnemonic

Name

IN

Input from Port

INS

Input from Port to String (80186/286/386 Only)

OUT

Output to Port

OUTS

Output String to Port (80186/286/386 Only)

1.9.1. IN

Input from Port                      Flags:  O D I T S Z A P C

Syntax:  IN accumulator,port                 -----------------

Transfers a byte or word from  a  port  to  the  accumulator

register. The port address is specified by the source  operand,

which can be DX or an 8-bit constant.  Constants  can  only  be

used for port numbers less than 255; use  DX  for  higher  port

numbers.

1110010w       data (1)

------------------+-------------------------+------------------

IN  accum,immed  | in    ax,60h            | 88/86 10 (W88=14)

|                         | 286   5

|                         | 386   12

------------------+-------------------------+------------------

1110110w

------------------+-------------------------+------------------

IN  accum,DX     | in    ax,dx             | 88/86 8 (W88=12)

| in    al,dx             | 286   5

|                         | 386   13

------------------+-------------------------+------------------

1.9.2. Sound

Port I/O Instructions

;* Sound - Sounds speaker with specified frequency and duration

;*

;* Shows:   Instructions - in     out

;*

;* Params:  freq - Desired frequency of sound in Hertz

;*          duration - Desired duration in clocks, where

;*                     18 clocks = approx 1 second

;*

;* Return:  None

Sound   PROC \

freq:WORD, duration:WORD

mov     al, 0B6h            ; Initialize channel 2 of

out     43h, al             ;   timer chip

mov     dx, 12h             ; Divide 1,193,182 Hertz

mov     ax, 34DEh           ;   (clock frequency) by

div     freq                ;   desired frequency

; Result is timer clock cou

out     42h, al             ; Low byte of count to time

mov     al, ah

out     42h, al             ; High byte of count to tim

in      al, 61h             ; Read value from port 61h

or      al, 3               ; Set first two bits

out     61h, al             ; Turn speaker on

push    duration

call    Pause               ; Pause for specified time

add     sp, 2               ; Clean stack on return

in      al, 61h             ; Get port value

xor     al, 3               ; Kill bits 0-1 to turn

out     61h, al             ;   speaker off

ret

Sound   ENDP

1.9.3. INS/INSB/INSW

Input from Port to String            Flags:  O D I T S Z A P C

80186/286/386 Only                           -----------------

Syntax:  INS dest,src

INSB

INSW

Receives a string from a port. The string is considered  the

destination and must be pointed to by ES:DI (even if an operand

is given). The input port is specified in DX. For each  element

received, DI is adjusted according to the size of  the  operand

and the status of the direction flag. DI is  increased  if  the

direction flag has been cleared with CLD or  decreased  if  the

direction flag has been set with STD.

If the INS form of the instruction is  used,  a  destination

operand must be provided to  indicate  the  size  of  the  data

elements to be processed and DX must be specified as the source

operand containing the port number. A segment override  is  not

allowed.  If  INSB  (bytes)  or  INSW  (words)  is  used,   the

instruction determines the size of  the  data  elements  to  be

received. No operands are allowed with INSB or INSW.

INS and its variations are usually used with the REP prefix.

Port I/O Instructions

Before the repeated instruction is executed, CX should  contain

the number of elements to be received.

0110110w

--------------------+-------------------------+----------------

INS  [ES:]dest,DX  | rep   insb              | 88/86 -

INSB               | ins   es:instr,dx       | 286   5

INSW               | rep   insw              | 386   15

--------------------+-------------------------+----------------

1.9.4. OUT

Output to Port                       Flags:  O D I T S Z A P C

-----------------

Syntax:  OUT port,accumulator

Transfers a byte or word to  a  port  from  the  accumulator

register. The port address  is  specified  by  the  destination

operand, which can be DX or an 8-bit constant.

1110011w       data (1)

------------------+--------------------------+-----------------

OUT immed8,accum | out   60h,al             | 88/86 10 (88=14)

|                          | 286   3

|                          | 386   10

------------------+--------------------------+-----------------

1110111w

------------------+--------------------------+-----------------

OUT  DX,accum    | out   dx,ax              | 88/86 8 (88=12)

| out   dx,al              | 286   3

|                          | 386   11

------------------+--------------------------+-----------------

1.9.5. OUTS/OUTSB/OUTSW

Output String to Port                Flags:  O D I T S Z A P C

-----------------

80186/286/386 Only

Syntax:  OUTS DX,src

OUTSB

OUTSW

Sends a string to a  port.  The  string  is  considered  the

source and must be pointed to by DS:SI (even if an  operand  is

given). The output port is specified in DX.  For  each  element

sent, SI is adjusted according to the size of the  operand  and

the status of the  direction  flag.  SI  is  increased  if  the

direction flag has been cleared with CLD or  decreased  if  the

direction flag has been set with STD.

If the OUTS form of the instruction is used, an operand must

be provided to indicate the size of data elements to be sent. A

segment override can  be  given.  If  OUTSB  (bytes)  or  OUTSW

(words) is used, the instruction determines  the  size  of  the

data elements to be sent. No operand is allowed with  OUTSB  or

Port I/O Instructions

OUTSW.

OUTS and its  variations  are  usually  used  with  the  REP

prefix. Before the instruction is executed, CX  should  contain

the number of elements to send.

0110111w

--------------------------+---------------------+--------------

OUTS  DX, [segreg:]src   | rep  outs  dx,buffer| 88/86 -

OUTSB                    | outsb               | 286   5

OUTSW                    | rep  outw           | 386   14

--------------------------+---------------------+--------------

1.10. Processor-Control Instructions

This group of instructions (except for NOP) generally  deals

with interactions in multiprocessor situations.

Mnemonic

Name

NOP

No Operation

ESC

Escape (sends an instruction to the coprocessor)

WAIT

Wait (stops and waits for coprocessor signal)

LOCK

Lock the Bus (locks out other processors)

HLT

Halt (stops execution until interrupt received)

1.10.1. NOP

No Operation                         Flags:  O D I T S Z A P C

-----------------

Syntax:  NOP

Performs no operation. NOP can be used for timing delays  or

alignment.

10010000

------------------+-------------------------+------------------

NOP              | nop                     | 88/86 3

|                         | 286   3

|                         | 386   3

------------------+-------------------------+------------------

1.10.2. ESC

Escape                               Flags:  O D I T S Z A P C

Syntax:  ESC                                 -----------------

Provides an instruction, and optionally a memory or register

Processor-Control Instructions

operand, for use by a coprocessor (such as the 8087, 80287,  or

80387). The  first  operand  must  be  a  6-bit  constant  that

specifies the bits of the coprocessor instruction.  The  second

operand can be either a register or memory operand to  be  used

by the coprocessor instruction.  The  CPU  puts  the  specified

information on the data bus where it can  be  accessed  by  the

coprocessor.   The   assembler   automatically   inserts    ESC

instructions in coprocessor instructions.

11011TTT      mod,LLL,r/m

TTT specifies the top three bits of the coprocessor opcode

and LLL specifies the lower three bits.

----------------+---------------------------+------------------

ESC  immed,reg | esc   5,al                | 88/86     2

|                           | 286       9-20

|                           | 386

----------------+---------------------------+------------------

ESC  immed,mem | esc   29,[bx]             | 88/86     8+EA

|                           | (W88=12+EA)

|                           | 286       9-20

|                           | 386

----------------+---------------------------+------------------

1.10.3. WAIT

Wait                                 Flags:  O D I T S Z A P C

-----------------

Syntax:  WAIT

Suspends CPU execution until a signal  is  received  that  a

coprocessor has finished a simultaneous operation. It should be

used to prevent a  coprocessor  instruction  from  modifying  a

memory location that is being modified at the same  time  by  a

processor instruction. WAIT is  the  same  as  the  coprocessor

FWAIT instruction.

10011011

-----------------+-------------------------+-------------------

WAIT            | wait                    | 88/86 4

|                         | 286   3

|                         | 386   6

-----------------+-------------------------+-------------------

1.10.4. LOCK

Lock the Bus                         Flags:  O D I T S Z A P C

-----------------

Syntax:  LOCK instruction

Locks out other processors  during  execution  of  the  next

instruction. This instruction is a prefix. It usually  precedes

an instruction that modifies a  memory  location  that  another

processor might attempt to modify at the same time.  See  Intel

documentation for details on multiprocessor environments.

Processor-Control Instructions

11110000

---------------------+-------------------------+---------------

LOCK  instruction   | lock  xchg ax,sem       | 88/86 2

|                         | 286   0

|                         | 386   0

---------------------+-------------------------+---------------

1.10.5. HLT

Halt                                 Flags:  O D I T S Z A P C

Syntax:  HLT                                 -----------------

Stops CPU execution until an interrupt restarts execution at

the instruction following HLT.

11110100

-----------------+--------------------------+------------------

HLT             | hlt                      | 88/86 2

|                          | 286   2

|                          | 386   5

-----------------+--------------------------+------------------

1.11. Stack-Oriented Instructions

Mnemonic

Name

PUSH

Push

PUSHF

Push Flags

PUSHA

Push All (80186/286/386 Only)

POP

Pop

POPF

Pop Flags

POPA

Pop All (80186/286/386 Only)

1.11.1. PUSH

Push                                 Flags:  O D I T S Z A P C

-----------------

Syntax:  PUSH src

Pushes the source operand onto the stack. This means that SP

is decreased by 2 and the source value is copied to SS:SP.  The

operand can be a memory  location,  a  general  purpose  16-bit

register, or a segment register. On the 80186-80386 processors,

the operand can also be a constant. On the 8088 and 8086,  PUSH

SP copies the value of SP after the push.  On  the  80186-80386

processors, PUSH SP copies the value of SP before the push.

Stack-Oriented Instructions

01010,reg

----------------+-------------------------+--------------------

PUSH reg16     | push  dx                | 88/86 11 (88=15)

|                         | 286   3

|                         | 386   2

----------------+-------------------------+--------------------

11111111    mod,110,r/m      disp (2)

---------------+-------------------------+---------------------

PUSH mem16    | push  [di]              | 88/86 16+EA (88=24+E

| push  fcount            | 286   5

|                         | 386   5

---------------+-------------------------+---------------------

00,sreg,110

------------------+-------------------------+------------------

PUSH  segreg     | push  es                | 88/86 10 (88=14)

| push  ss                | 286   3

| push  cs                | 386   2

------------------+-------------------------+------------------

00001111    10,sreg,000

------------------+-------------------------+------------------

PUSH  segreg     | push  fs                | 88/86 -

| push  gs                | 286   -

|                         | 386   2

------------------+-------------------------+------------------

011010s0    data (1 or 2)

------------------+-------------------------+------------------

PUSH  immed      | push  'a'               | 88/86 -

| push  15000             | 286   3

|                         | 386   2

------------------+-------------------------+------------------

1.11.2. GetMem

;* GetMem - Gets total size of memory and determines the larges

;* amount of unallocated memory available. GetMem invokes DOS

;* Function 48h (Allocate Memory) to request an impossibly larg

;* memory block. DOS denies the request, but returns instead th

;* size of the largest block available. This is the amount that

;* GetMem returns to the calling program. See the WinOpen

;* procedure for an example of calling Function 48h to allocate

;* unused memory.

;*

;* Shows:   BIOS Interrupt - 12h (Get Conventional Memory Size)

;*

;* Params:  None

;*

;* Return:  Long integer,

;*          high word = total memory in kilobytes (KB)

;*          low word  = largest block of available memory (KB)

GetMem  PROC

int     12h         ; Get total memory in K

push    ax          ; Save size of memory

mov     ah, 48h     ; Request memory allocation

mov     bx, 0FFFFh  ; Ensure request is denied for

;   impossibly large block

int     21h         ; Get largest available block in BX

Stack-Oriented Instructions

mov     ax, bx      ; Copy to AX

mov     cl, 6       ; Convert paragraphs to kilobytes b

shr     ax, cl      ;   dividing by 64

pop     dx          ; Recover total in DX

ret                 ; Return long integer DX:AX

GetMem  ENDP

1.11.3. PUSHA

Push All                             Flags:  O D I T S Z A P C

80186/286/386 Only                           -----------------

Syntax: PUSHA

Pushes the general-purpose registers  onto  the  stack.  The

registers are pushed in the following order: AX,  CX,  DX,  BX,

SP, BP, SI, DI. The value pushed for SP is the value before the

instruction. PUSHA always pushes 16-bit registers.

1.11.4. POP

Pop                                  Flags:  O D I T S Z A P C

-----------------

Syntax:  POP dest

Pops the top of the stack into the destination operand. This

means that the value at SS:SP  is  copied  to  the  destination

operand and SP is increased by 2. The destination  operand  can

be a memory location, a general purpose 16-bit register, or any

segment register except CS. Use RET to pop CS.

01011 reg

----------------+-------------------------+--------------------

POP  reg16     | pop   cx                | 88/86 8 (88=12)

|                         | 286   5

|                         | 386   4

----------------+-------------------------+--------------------

10001111     mod, 000,r/m      disp (2)

----------------+------------------------+---------------------

POP  mem16     | pop   param            | 88/86 17+EA (88=25+E

|                        | 286   5

|                        | 386   5

----------------+------------------------+---------------------

000,sreg,111

------------------+-------------------------+------------------

POP  segreg      | pop   es                | 88/86 8 (88=12)

| pop   ds                | 286   5

| pop   ss                | 386   7

------------------+-------------------------+------------------

Stack-Oriented Instructions

1.11.5. POPA

Pop All                              Flags:  O D I T S Z A P C

80186/286/386 Only                           -----------------

Syntax:  POPA

Pops  the  top  16  bytes  on  the  stack  into  the   eight

general-purpose registers. The  registers  are  popped  in  the

following order: DI, SI, BP, SP, BX, DX, CX, AX. The value  for

the SP register is actually discarded rather than copied to SP.

POPA always pops into 16-bit registers.

01100001

------------------+-------------------------+------------------

POPA             | popa                    | 88/86 -

|                         | 286   19

|                         | 386   24

------------------+-------------------------+------------------

1.11.6. PopAll

;* PopAll - Macro to restore registers pushed by the PushAll ma

;* the PushAll macro, PopAll uses the most efficient method ava

;* assembly time (not at run time).

;*

;* Shows:   Instruction - popa

;*

;* Params:  None

PopAll MACRO

IF @Cpu AND 2                       ;; If assembling on 801

popa                            ;;    use the efficient

ELSE                                ;;    instruction

pop ax                          ;; Otherwise pop the re

pop cx                          ;;    individually

pop dx

pop bx

pop sp

pop bp

pop si

pop di

ENDIF

ENDM

1.12. String Operations

In all the string instructions, DS:SI points to  the  source

operand and ES:DI points to the destination operand.

Mnemonic

Name

MOVS

Move String Data

Stack-Oriented Instructions

LODS

Load String Operand

STOS

Store String Data

SCAS

Scan String (compares to accumulator)

CMPS

Compare Strings

INS

Input to String from Port (80186/286/386 only)

OUTS

Input String to Port (80186/286/386 only)

REP

Repeat String (instruction prefix)

REPE/REPZ

Repeat While Equal

REPNE/REPNZ

Repeat While Not Equal

1.12.1. MOVS/MOVSB/MOVSW

Move String Data                     Flags:  O D I T S Z A P C

-----------------

Syntax:  MOVS dest,src

MOVSB

MOVSW

Moves a string from one  area  of  memory  to  another.  The

source string must be pointed to by DS:SI and  the  destination

address must be pointed to  by  ES:DI  (even  if  operands  are

given).  For  each  element  moved,  DI  and  SI  are  adjusted

according to the size of the operands and  the  status  of  the

direction flag. They are increased if the  direction  flag  has

been cleared with CLD, or decreased if the direction  flag  has

been set with STD.

If the MOVS form of the instruction is used,  operands  must

be provided to indicate the size of the  data  elements  to  be

processed. A segment override  can  be  given  for  the  source

operand (but not for the  destination).  If  MOVSB  (bytes)  or

MOVSW (words) is used, the instruction determines the  size  of

the data elements to be processed. Operands are not allowed  in

MOVSB or MOVSW.

MOVS and its  variations  are  usually  used  with  the  REP

prefix. Before a move using a repeat prefix, CX should  contain

the number of elements to move.

1010010w

-------------------------------+----------------------+--------

MOVS [ES:]dest,[segreg:]src   | rep   movsb          | 88/86 1

|                      | (W88=26

MOVSB                         | movs  dest,es:source | 286   5

MOVSW                         |                      | 386   7

-------------------------------+----------------------+--------

Stack-Oriented Instructions

1.12.2. LODS/LODSB/LODSW

Load String Operand                  Flags:  O D I T S Z A P C

-----------------

Syntax: LODS src

LODSB

LODSW

Loads a string from memory into  the  accumulator  register.

The string to be loaded is the source and must be pointed to by

DS:SI (even if an operand is given). For  each  source  element

loaded, SI is adjusted according to the size  of  the  operands

and the status of the direction flag. SI is  increased  if  the

direction flag has been cleared with CLD or  decreased  if  the

direction flag has been set with STD.

If the LODS form of the instruction is used, an operand must

be provided to indicate the size of the  data  elements  to  be

processed. A segment override can be given. If LODSB (bytes) or

LODSW (words) is used, the instruction determines the  size  of

the data elements to be processed and whether the element  will

be loaded to AL or AX. Operands are not allowed with  LODSB  or

LODSW.

LODS and its variations are not normally  used  with  repeat

prefixes, since there is no reason to  repeatedly  load  memory

values to a register.

1010110w

--------------------------+----------------------+-------------

LODS  [segreg:]src       | lods  es:source      | 88/86 12

|                      | (W88=26)

LODSB                    | lodsw                | 286   5

LODSW                    |                      | 386   5

--------------------------+----------------------+-------------

1.12.3. STOS/STOSB/STOSW

Store String Data                    Flags:  O D I T S Z A P C

-----------------

Syntax: STOS dest

STOSB

STOSW

Stores the value in the accumulator to a string. The  string

to be filled is the destination and must be pointed to by ES:DI

(even if an operand is given). For each source element  loaded,

DI is adjusted according to the size of the  operands  and  the

status of the direction flag. DI is increased if the  direction

flag has been cleared with CLD or decreased  if  the  direction

flag has been set with STD.

If the STOS form of the instruction is used, an operand must

be provided to indicate the size of the  data  elements  to  be

processed. No segment override is allowed. If STOSB (bytes)  or

STOSW (words) is used, the instruction determines the  size  of

the data elements to be processed and whether the element  will

be from AL or AX. No operand is allowed with STOSB or STOSW.

STOS and its variations are often used with the REP  prefix.

Before the repeated instruction is executed, CX should  contain

Stack-Oriented Instructions

the number of elements to store.

1010101w

------------------+------------------------+-------------------

STOS  [ES:]dest  | stos  es:dstring       | 88/86 11 (W88=15)

STOSB            | rep    stosw           | 286   3

STOSW            | rep    stosb           | 386   4

------------------+------------------------+-------------------

1.12.4. SCAS/SCASB/SCASW

Scan String Flags                    Flags:  O D I T S Z A P C

-----------------

*       * * * * *

Syntax:  SCAS dest

SCASB

SCASW

Scans a string to find a value specified in the  accumulator

register.  The  string  to  be  scanned   is   considered   the

destination and must be pointed to by ES:DI (even if an operand

is specified). For each element,  the  destination  element  is

subtracted from the accumulator value and the flags are updated

to reflect the result (although the result is not  stored).  DI

is adjusted according to the  size  of  the  operands  and  the

status of the direction flag. DI is increased if the  direction

flag has been cleared with CLD or decreased  if  the  direction

flag has been set with STD.

If the SCAS form of the instruction is used, an operand must

be provided to indicate the size of the  data  elements  to  be

processed. No segment override is allowed. If SCASB (bytes)  or

SCASW (words) is used, the instruction determines the  size  of

the data elements to  be  processed  and  whether  the  element

scanned for is in AL, AX. No operand is allowed with  SCASB  or

SCASW.

SCAS  and  its  variations  are  usually  used  with  repeat

prefixes. REPNE (or REPNZ) is used to find the first  match  of

the accumulator value. REPE (or REPZ) is used to find the first

nonmatch. Before the comparison, CX should contain the  maximum

number of elements to compare. After the comparison, CX will be

0 if no match or nonmatch was found. Otherwise SI and  DI  will

point to the element after the first match or nonmatch.

1010111w

-------------------+-------------------------+-----------------

SCAS  [ES:]dest   | repne  scasw            | 88/86 15 (W88=19

SCASB             | repe   scasb            | 286   7

SCASW             | scas   es:destin        | 386   7

-------------------+-------------------------+-----------------

1.12.5. CMPS/CMPSB/CMPSW

Stack-Oriented Instructions

Compare String                       Flags:  O D I T S Z A P C

-----------------

*       * * * * *

Syntax:  CMPS dest,src

CMPSB

CMPSW

Compares two strings. DS:SI must point to the source  string

and ES:DI  must  point  to  the  destination  string  (even  if

operands are  given).  For  each  comparison,  the  destination

element is subtracted from the source element and the flags are

updated to reflect the  result  (although  the  result  is  not

stored). DI and SI are adjusted according to the  size  of  the

operands and  the  status  of  the  direction  flag.  They  are

increased if the direction flag has been cleared  with  CLD  or

decreased if the direction flag has been set with STD.

If the CMPS form of the instruction is used,  operands  must

be provided to indicate the size of the  data  elements  to  be

processed. A segment override can be given for the source  (but

not for the destination). If CMPSB (bytes) or CMPSW (words)  is

used, the instruction determines the size of the data  elements

to be processed. Operands are not allowed with CMPSB or  CMPSW.

CMPS  and  its  variations  are  usually  used  with  repeat

prefixes. REPNE (or REPNZ) is used  to  find  the  first  match

between two strings. REPE (or REPZ) is used to find  the  first

nonmatch. Before the comparison, CX should contain the  maximum

number of elements to compare. After a  REPNE  CMPS,  the  zero

flag will be cleared if no match was found. After a REPE  CMPS,

the zero flag will be set if no nonmatch was  found.  Otherwise

SI and DI will point to the element after the  first  match  or

nonmatch.

1010011w

------------------------------+--------------------+-----------

CMPS  [segreg:]src,[ES:]dest |cmps  source,es:dest| 88/86 22

|                    |(W88=30)

CMPSB                        |repne cmpsw         | 286   8

CMPSW                        |repe  cmpsb         | 386   10

------------------------------+--------------------+-----------

1.12.6. REP

Repeat String                        Flags:  O D I T S Z A P C

-----------------

Syntax:  REP instruction

Repeats the string instruction the number of times indicated

by CX. For each  string  element,  the  string  instruction  is

performed and CX is decremented. When CX reaches  0,  execution

continues with the next instruction. REP is normally used  with

MOVS and STOS. (REP LODS is legal, but has the same  effect  as

LODS.) REP is additionally  used  with  INS  and  OUTS  on  the

80186-80386 processors. On all  processors  except  the  80386,

combining a repeat prefix with a  segment  override  may  cause

errors if an interrupt occurs during a string operation.

Stack-Oriented Instructions

11110010    1010010w

--------------------+-------------------------+----------------

REP  MOVS dest,src | rep   movs source,destin| 88/86 9+17n

|                         |  (W88=9+25n)

REP  MOVSB         | rep   movsw             | 286   5+4n

REP  MOVSW         |                         | 386   7+4n

--------------------+-------------------------+----------------

11110010    1010101w

--------------------+-------------------------+----------------

REP  STOS dest     | rep   stosb             | 88/86 9+10n

|                         |(W88=9+14n)

REP  STOSB         | rep   stos destin       | 286   4+3n

REP  STOSW         |                         | 386   5+5n

--------------------+-------------------------+----------------

11110010    0110110w

--------------------+-------------------------+----------------

REP  INS dest,DX   | rep   insb              | 88/86 -

REP  INSB          | rep   ins destin,dx     | 286   5+4n

REP  INSW          |                         | 386   13+6n

--------------------+-------------------------+----------------

11110010    0110111w

---------------------+-------------------------+---------------

REP  OUTS DX,src    | rep   outs dx,source    | 88/86 -

REP  OUTSB          | rep   outsw             | 286   5+4n

REP  OUTSW          |                         | 386   12+5n

---------------------+-------------------------+---------------

1.12.7. REPcondition

Repeat String Conditionally          Flags:  O D I T S Z A P C

-----------------

Syntax:  REPE instruction                              *

REPNE instruction

Repeats a string instruction as long as  condition  is  true

and the maximum count has not been reached. REPE and REPZ  (the

names are synonyms) repeat while the zero flag  is  set.  REPNE

and REPNZ (the names are synonyms) repeat while the  zero  flag

is cleared. The conditional repeat prefixes should only be used

with  SCAS  and  CMPS,  since  these  are   the   only   string

instructions that modify the zero flag.  Before  executing  the

instruction, CX should be set to the maximum  allowable  number

of repetitions. For each string element, the string instruction

is performed, CX is decremented, and the zero flag  is  tested.

On all processors except the 80386, combining a  repeat  prefix

with a segment override may cause errors if an interrupt occurs

during a string operation.

11110011     1010011w

-----------------------+-------------------------+-------------

REPE  CMPS dest,src   | repz   cmpsb            | 88/86 9+22n

|                         | (W88=9+30n)

REPE  CMPSB           | repe   cmps destin,src  | 286   5+9n

REPE  CMPSW           |                         | 386   5+9n

-----------------------+-------------------------+-------------

Stack-Oriented Instructions

11110011     1010111w

-----------------------+-------------------------+-------------

REPE  SCAS dest       | repe   scas destin      | 88/86 9+15n

|                         | (W88=9+19n)

REPE  SCASB           | repz   scasw            | 286   5+8n

REPE  SCASW           |                         | 386   5+8n

-----------------------+-------------------------+-------------

11110010     1010011w

-----------------------+-------------------------+-------------

REPNE  CMPS dest,src  | repne   cmpsw           | 88/86 9+22n

|                         | (W88=9+30n)

REPNE  CMPSB          | repnz   cmps destin,src | 286   5+9n

REPNE  CMPSW          |                         | 386   5+9n

-----------------------+-------------------------+-------------

11110010     1010111w

-----------------------+-------------------------+-------------

REPNE  SCAS dest      | repne    scas destin    | 88/86 9+15n

|                         | (W88=9+19n)

REPNE  SCASB          | repnz    scasb          | 286   5+8n

REPNE  SCASW          |                         | 386   5+8n

-----------------------+-------------------------+-------------

1.13. Type-Conversion Instructions

These instructions offer a fast way to convert from one data

type to another, while  preserving  the  sign.  Use  only  with

signed integer values. These instructions always use a value in

the accumulator (AL or AX).

Mnemonic

Name

CBW

Convert Byte to Word

CWD

Convert Word to Double

1.13.1. CBW

Convert Byte to Word                 Flags:  O D I T S Z A P C

-----------------

Syntax:  CBW

Converts a signed byte in AL to  a  signed  word  in  AX  by

extending the sign bit of AL into all bits of AH.

10011000

-----------------+-------------------------+------------------

CBW             | cbw                     | 88/86 2

|                         | 286   2

|                         | 386   3

-----------------+-------------------------+------------------

Type-Conversion Instructions

1.13.2. GetCurDisk

;* GetCurDisk - Gets designation of current disk.

;*

;* Shows:   DOS Function - 19h (Get Current Disk)

;*          Instruction - cbw

;*

;* Params:  None

;*

;* Return:  Short integer with drive designation

;*          0 = A, 1 = B, 2 = C, etc.

GetCurDisk PROC

mov     ah, 19h                 ; DOS Function 19h

int     21h                     ; Get Current Disk

cbw                             ; AX = drive designatio

ret

GetCurDisk ENDP

1.13.3. CWD

Convert Word to Double               Flags:  O D I T S Z A P C

-----------------

Syntax:  CWD

Converts the signed word in AX to a signed word in the DX:AX

register pair by extending the sign bit of AX into all bits  of

DX.

10011001

-----------------+-------------------------+------------------

CWD             | cwd                     | 88/86 5

|                         | 286   2

|                         | 386   2

-----------------+-------------------------+------------------

Directive Summary

2. Directive Summary

Category

Description

Conditional Assembly

Assemble blocks of code on certain conditions

Conditional Error

Generate assemble-time error on certain  conditions

Code Labels

Define code addresses and alignment

Data Allocation

Define global data items of various sizes

Listing Control

Control what is included in listing file

Macro

Define macros, equates, and repeat blocks

Miscellaneous

Comment, output, radix, and include directives

Multimodule

Declare items public for other modules

Processor

Enable instruction set of another processor

Segment

Give full segment definitions

Simplified Segment

Define segments using Microsoft conventions

Structure and Record

Define structure or record data types

2.1. Conditional-Assembly Directives

Mnemonic

Description

IF

Assembles block if expression is nonzero

IF1

Assembles block on pass 1

IF2

Assembles block on pass 2

IFE

Assembles block if expression is false

IFB

Assembles block if expression is blank

IFNB

Assembles block if expression is not blank

IFDEF

Assembles block if expression is defined

IFNDEF

Assembles block if expression is not defined

IFDIF[I]

Assembles block if arguments are different

IFIDN[I]

Assembles block if arguments are identical

ELSE

Marks beginning of alternate conditional block

ELSEIF

Marks alternate block with a new condition

ENDIF

Directive Summary

Terminates conditional block

2.1.1. IF block syntax

Syntax:  IFcondition

ifstatements

[ELSEIFcondition

elseifstatements]

.

.

.

[ELSE

elsestatements]

ENDIF

IFcondition may be any directive beginning with the  letters

"IF." There is a corresponding ELSEIF  directive  for  each  IF

directive.

The ifstatements, elseifstatements, and  elsestatements  are

each a series of one  or  more  assembly-code  statements.  The

respective conditions determine which block  is  assembled,  if

any. The ELSE and ELSEIF blocks are optional.

See Also:

IF, IFE, IF1, IFB, IFDEF, IFDIF

2.1.2. LoadPtr

Directive Summary

;* LoadPtr - Macro to load far address into segment:register

;*  pair, or near address into register.

;*

;* Params:  sgmnt - Segment to be loaded with segment address

;*          reg - Register to be loaded with offset address

;*          ptr - Pointer to address

;*

;* Shows:   Instructions - lds     les

;*          Directives - MACRO  IF     IFIDNI  ELSE   ELSEIF

;*                       ENDIF  .ERR   %OUT    EXITM  ENDM

;*          Operators - < >       ;;

LoadPtr MACRO sgmnt, reg, ptr  ;; Macro definition

IF @DataSize               ;; If far pointer, and

IFIDNI <sgmnt>, <ds>   ;;   if 1st argument is DS,

lds reg, ptr       ;;   load DS:reg with far addres

EXITM

ENDIF

.ERR                   ;; Generate error if not DS or E

%OUT 1st macro argument must be DS or ES

ELSE                                ;; If near pointer,

IFIDNI <sgmnt>, <es>            ;;   and if segment is

push ds                     ;;   ensure ES points t

pop  es                     ;;   same segment as DS

ENDIF

mov reg, ptr           ;; Then load reg with near addre

ENDIF

ENDM

2.1.3. IF

Syntax:

IF expression

IFE expression

IF causes assembly of a block of statements  (see  IF  Block

Syntax) if expression is true (nonzero). IFE causes assembly of

a block of statements if expression is false (zero).

ELSEIF and ELSEIFE correspond to IF and IFE, respectively.

See Also:

IF1, IFB, IFDEF, IFDIF

2.1.4. IF1, IF2

Syntax:

IF1

IF2

IF1 causes assembly of a block of statements (see  IF  Block

Syntax) during pass 1.  IF2  causes  assembly  of  a  block  of

statements during pass 2. These directives  conflict  with  the

One Pass Assembly option.

ELSEIF1 and ELSEIF2 correspond to IF1 and IF2, respectively.

Directive Summary

See Also:

IF, IFE, IFB, IFDEF, IFDIF

2.1.5. IFB

Syntax:

IFB <argument>

IFNB <argument>

IFB causes assembly of a block of statements (see  IF  Block

Syntax) if argument is blank. IFNB causes assembly of  a  block

of statements if argument is not blank.

Within a macro, a parameter name is  replaced  by  an  empty

string if you call the macro  without  giving  a  corresponding

argument.  IFB  and  IFNB  therefore  let  you  define  default

behavior when an argument is missing.

The  ELSEIFB  and  ELSEIFNB  correspond  to  IFB  and  IFNB,

respectively.

argument

Any name, number,  or  expression;  typically,  the

name of a parameter of a macro.

See Also:

IF, IFE, IF1, IFDEF, IFDIF

2.1.6. IFDEF

Syntax:

IFDEF name

IFNDEF name

IFDEF causes assembly of a block of statements (see IF Block

Syntax) if name is a previously defined symbol.  IFNDEF  causes

assembly of a block of statements if name has not been defined.

You can define symbols on the command line (or in  Assembler

Flags dialog box). Use of  this  feature  with  the  IFDEF  and

IFNDEF  directives  lets  you  control  which  statements   are

assembled from the command line.

ELSEIFDEF and ELSEIFNDEF correspond to IFDEF and IFNDEF.

name

Any valid name. If it is a forward reference, it is

undefined on Pass 1, but defined on  Pass  2.  This

may cause conflicts with one-pass assembly.

See Also:

IF, IFE, IF1, IFB, IFDEF, IFDIF

Directive Summary

2.1.7. IFDIF

Syntax:

IFDIF[I] <argument1>, <argument2>

IFIDN[I] <argument1>, <argument2>

Causes assembly of a  block  of  statements  (see  IF  Block

Syntax), depending on whether two arguments  are  different  or

identical. IFDIF assembles the block if argument1 and argument2

are different. IFIDN assembles the block if they are the  same.

IFDIFI   and   IFIDNI   do   the   same   actions,   but    are

case-insensitive.

ELSEIFDIF[I] and ELSEIFIDN[I]  correspond  to  IFDIF[I]  and

IFIDN[I].

argument

Any valid name,  number,  or  expression.  Must  be

enclosed in angle brackets.

See Also:

IF, IFE, IF1, IFB, IFDEF

2.2. Conditional-Error Directives

These directives generate error messages at run time and are

typically used for error checking within macro  calls  and  for

debugging.

Mnemonic

Generates error:

.ERR

If expression evaluates to nonzero

.ERR1

On pass 1

.ERR2

On pass 2

.ERRE

If expression is false

.ERRB

If expression is blank

.ERRNB

If expression is not blank

.ERRDEF

If expression is defined

.ERRNDEF

If expression is not defined

.ERRDIF[I]

If arguments are different

.ERRIDN[I]

If arguments are identical

Directive Summary

2.2.1. .ERR

Syntax:

.ERR

Generates a severe error message when assembled.  Generally,

this directive occurs in a conditional-assembly  block  and  is

used to indicate some error condition  discovered  at  assembly

time. Though this directive does not  add  functionality  to  a

program, it can  be  useful  for  verifying  that  the  program

assembled correctly.

See Also:

.ERR1, .ERRB, .ERRDEF, .ERRDIF, .ERRE, .ERRNZ

2.2.2. .ERR1/.ERR2

Syntax:

.ERR1

.ERR2

.ERR1 generates a severe error message when assembled during

pass 1. .ERR2 generates a severe error message  when  assembled

during  pass  2.  Generally,  these  directives  occur   in   a

conditional-assembly block and are used to indicate some  error

condition discovered at assembly time.

Unlike the .ERR directive, .ERR1 and .ERR2 conflict with the

One-Pass Assembly option.

See Also:

.ERR, .ERRB, .ERRDEF, .ERRDIF, .ERRE, .ERRNZ

2.2.3. .ERRE/.ERRNZ

Syntax:

.ERRE expression

.ERRNZ expression

Tests value of  expression  and  generates  a  severe  error

depending on the resulting value. .ERRE generates an  error  if

expression evaluates to false (zero). .ERRNZ generates an error

if expression evaluates to true (nonzero).

expression

An expression that evaluates to true or  false,  or

as  a  numeric  constant.  The  expression   cannot

contain forward references.

See Also:

.ERR, .ERR1, .ERRB, .ERRDEF, .ERRDIF

Directive Summary

2.2.4. .ERRB/.ERRNB

Syntax:

.ERRB <argument>

.ERRNB <argument>

.ERRB generates a severe error if argument is blank.  .ERRNB

generates an error if argument is not  blank.  Normally,  these

directives are used within macro definitions; a macro parameter

is replaced by an empty string if you call the macro and do not

give a corresponding argument. These directives let  you  write

macros that generate an error if the wrong number of  arguments

are used in the macro call.

argument

A name, number, or expression, though normally  the

name of a macro parameter.

See Also:

.ERR, .ERR1, .ERRDEF, .ERRDIF, .ERRE, .ERRNZ

2.2.5. .ERRDEF/.ERRNDEF

Syntax:

.ERRDEF name

.ERRNDEF name

.ERRDEF generates a  severe  error  message  if  name  is  a

previously defined symbol. .ERRNDEF generates  a  severe  error

message if name has not been defined.

One of the most common uses of the .ERRNDEF directive is  to

ensure that a symbol has been defined before you attempt to use

it. This situation may arise if the source code depends on  the

command line or Assembler Flags dialog box to define a  symbol.

See Also:

.ERR, .ERR1, .ERRB, .ERRDIF, .ERRE, .ERRNZ

2.2.6. .ERRDIF/.ERRIDN

Syntax:

.ERRDIF[I] <argument1>, <argument2>

.ERRIDN[I] <argument1>, <argument2>

.ERRDIF generates a severe error if argument1 and  argument2

are  different.  .ERRIDN  generates  an  error  if   they   are

identical. These directives are commonly used inside macros, to

test for bad values passed as parameters. .ERRDIFI and .ERRIDNI

do the same actions, but are case-insensitive.

argument1 and argument2 can each be a symbolic name, number,

or expression.

See Also:

.ERR, .ERR1, .ERRB, .ERRDEF, .ERRE, .ERRNZ

Directive Summary

2.3. Code-Label Directives

Mnemonic

Description

PROC

Declares start of procedure

ENDP

Declares end of procedure

LABEL

Declares a  label  of  specified  type  at  current

location

ALIGN

Starts location counter at next multiple of  number

given

EVEN

Starts location counter at next even address

ORG

Sets location counter to a specific offset  address

2.3.1. PROC

Syntax:  label PROC [NEAR|FAR] [langtype] [USES reglist], [arg

statements

RET [constant]

label ENDP

Defines a  procedure  with  name  label.  To  use  langtype,

reglist, or arguments, you must first use .MODEL and include  a

langtype.

langtype

C, BASIC,  PASCAL,  or  FORTRAN.  Overrides  .MODEL

langtype.

reglist

An optional list of registers  that  the  assembler

preserves   with   PUSH   instructions   (and   POP

instructions   upon   exit).   Separate    multiple

registers with spaces.

arguments

List  of  arguments.   The   assembler   translates

references to arguments into the  actual  reference

to  the  stack  location.  Each  argument  can   be

optionally  followed  by  a  colon  (:)  and  type.

Separate multiple arguments with commas.

RET

Return instruction. Size is NEAR or FAR,  depending

on the distance attribute of the procedure.

See Also:

LABEL, PUBLIC, EXTRN, ENDP, RET

Directive Summary

2.3.2. PUTSTR.ASM

;* PUTSTR.ASM illustrates multiple-modules and memory model

;* independent techniques. To create the program, paste this

;* module and HELLOM.ASM (get help on HELLOM.ASM to open it)

;* into separate program files. In the environment, you must

;* also create a program list for the program and define a mode

;* in the Defines field of the Assembler Flags dialog box.

;* For example, use "model=large". Alternately, you could assem

;* outside the environment with the following command line:

;*      qcl /Dmodel=large hellom.asm putstr.asm

;*

;* Shows:   Directives - PUBLIC    PROC     ENDP   PTR

;*          DOS function - 02h (Character Output)

; Model is symbol passed from command line or environment.

; Note that the % operator

; is required to receive external constant symbols.

%       .MODEL  model, c

; With simplified segment directives, procedures such as PutStr

; are automatically PUBLIC. Data is not automatically public

; and must be declared with PUBLIC in order to be accessed

; from other modules. With full segment directives,

; the following line would be required to make

; PutStr known to other modules.

;PUBLIC PutStr

.CODE                 ; Address of string (near or far

PutStr  PROC    str:PTR BYTE  ;   depending on model) is passed

;   the stack

mov     ah, 02h       ; Character output function

IF      @DataSize     ; If data is far (compact, large,

les     di, str       ;   or huge), address is ES:DI

next:   mov     dl, es:[di] ; Load each character through ES:[D

ELSE                  ; If data is near (tiny, small,

mov     di, str       ;   or medium), address is DI onl

next:   mov     dl, [di]      ; Load each character through [DI

ENDIF

or      dl, dl        ; Check to see if character is ze

jz      exit          ; If zero, done

int     21h           ; DOS displays character

inc     di            ; Point to next character

jmp     SHORT next    ; Loop back to load character

exit:   ret

PutStr  ENDP

END

2.3.3. ENDP

Syntax:

label ENDP

Labels the ending address of a  procedure  previously  begun

Directive Summary

with PROC.

See Also:

PROC, RET

2.3.4. LABEL

Syntax:

name LABEL type

Creates a new variable or label of a given size (type) at  a

specified location by assigning  the  current  location-counter

value and the given type to name.  Can  be  used  to  define  a

second entry point into a procedure.

name

Symbol assigned to the label.

type

BYTE, WORD, DWORD, QWORD, TBYTE, NEAR,  FAR,  PROC,

or a previously defined structure.

See Also:

PROC, THIS

2.3.5. ALIGN

Syntax:

ALIGN number

Aligns next variable or instruction  on  an  offset  address

that is a multiple of number. The assembler pads  each  skipped

byte with a NOP (in code segment) or 0  (in  data  segment)  as

needed.

number

Must be a power of 2 (i.e., 2,4,8...) and should be

equal to or greater than the size of the align type

of the current segment.

See Also:

EVEN

2.3.6. EVEN

Syntax:

EVEN

Aligns next  variable  or  instruction  on  an  even  offset

address, padding a byte with a NOP (in code segment) or  0  (in

Directive Summary

data segment) if necessary. The current  segment  must  not  be

BYTE-aligned.

See Also:

ALIGN

2.3.7. ORG

Syntax:

ORG expression

Begins assembly at a specific offset  address;  particularly

useful for COM-format programs, which must  begin  assembly  at

100H. The assembler implements this directive by adjusting  the

location counter.

expression

An expression that evaluates to a constant  number.

See Also:

AT, ALIGN, EVEN

2.4. Data-Allocation Directives

Mnemonic

Defines objects of type:

DB

Byte

DW

Word (2 bytes)

DD

Doubleword (4 bytes)

DQ

Quadword (8 bytes)

DT

Ten-Byte

2.4.1. DB

Syntax:

[name] DB initializer [,initializer]...

Allocates and optionally initializes one or  more  bytes  of

global data.

name

The symbol name assigned to  the  variable.  If  no

name is assigned, the data is  allocated,  but  the

starting address of the variable  has  no  symbolic

name.

initializer

A constant expression or question mark  (indicating

uninitialized data). Separate  multiple  values  by

Directive Summary

commas, or use the DUP operator.

See Also:

DW, DD, DQ, DT, DUP

2.4.2. data.asm

;* DATA.ASM illustrates how to declare variables with

;* data-definition directives.

;*

;* Shows:   Directives - DB    DW    DD    DQ    DT   LABEL

;*          Operator - DUP

.MODEL small

.STACK

.DATA

; Integers

bvar    DB      16             ; Byte initialized as 16

wvar    DW      4 * 3          ; Word initialized as 12

dvar    DD      4294967295     ; Double word as 4,294,967,295

qvar    DQ      ?              ; Quad word (8-byte) uninitializ

DB      1, 2, 3, 4, 5  ; Five initialized bytes

ten     DT      2345           ; 10-byte integer as 2,345

fp      DD      6.3153         ; 8-byte real number

; Strings and characters

abc     DB      'a', 'b', 'c'

; Three characters (same as 'abc')

hello   DB      'Hello', 13, 10, '$'

; String for DOS Function 9h

msg     DB      13, 10, "Another hello"

; String for DOS Function 40h

len     EQU     $ - msg         ; Length in bytes of msg

dir     DB      "\C\CPROGS", 0  ; String data for C

npdir   DW      BYTE PTR dir    ; Near pointer to char (C strin

fpdir   DD      BYTE PTR dir    ; Far pointer to char (C string

; Arrays and buffers

dbuf    DW      25 DUP (?)      ; 25 uninitialized doublewords

bbuf    DB      12 DUP ('month'); 60-byte array, each 5-byte

;   group initialized to 'month

fpdbuf  DD      WORD PTR dbuf   ; C array is pointer to array d

two_d   DW      5 DUP (5 DUP (0))

; Allocate 25 words initialized to 0

warr    LABEL   WORD

; Access next array as 50 words

darr    LABEL   DWORD

; Access same array as 25 doublewords

barr    DB      100 DUP (0, 16, 33, 48)

; Access array as 100 bytes, each

;   group initialize to bit mask

.CODE

.STARTUP

Directive Summary

2.4.3. DW

Syntax:

[name] DW initializer [,initializer]...

Allocates and optionally initializes one or  more  words  of

global data.

name

The symbol name assigned to  the  variable.  If  no

name is assigned, the data is  allocated,  but  the

starting address of the variable  has  no  symbolic

name.

initializer

A constant expression or question mark  (indicating

uninitialized data). Separate  multiple  values  by

commas, or use the DUP operator.

See Also:

DB, DD, DQ, DT, DUP

2.4.4. DD

Syntax:

[name] DD initializer [,initializer]...

Allocates and optionally initializes one or more doublewords

(4 bytes) of global data.

name

The symbol name assigned to  the  variable.  If  no

name is assigned, the data is  allocated,  but  the

starting address of the variable  has  no  symbolic

name.

initializer

A constant expression or question mark  (indicating

uninitialized data). Separate  multiple  values  by

commas, or use the DUP operator.

See Also:

DB, DW, DQ, DT, DUP

2.4.5. DQ

Syntax:

[name] DQ initializer [,initializer]...

Allocates and optionally initializes one or more quad  words

(8 bytes) of global data.

name

The symbol name assigned to  the  variable.  If  no

name is assigned, the data is  allocated,  but  the

starting address of the variable  has  no  symbolic

name.

initializer

A constant expression or question mark  (indicating

uninitialized data). Separate  multiple  values  by

commas, or use the DUP operator.

Directive Summary

See Also:

DB, DW, DD, DT, DUP

2.4.6. DT

Syntax:

[name] DT initializer [,initializer]...

Allocates and optionally initializes 10 bytes of storage for

each initializer.

name

The symbol name assigned to  the  variable.  If  no

name is assigned, the data is  allocated,  but  the

starting address of the variable  has  no  symbolic

name.

initializer

A constant expression or question mark  (indicating

uninitialized data). Separate  multiple  values  by

commas, or use the DUP operator.

See Also:

DB, DW, DD, DQ, DUP

2.5. Listing-File Directives

Mnemonic

Description

.LIST

Turns on listing of statements

.XLIST

Turns on macro expansion statements in listings

.LFCOND

Turns on listing of statements in false conditional

blocks

.SFCOND

Suppresses listing of false conditional blocks

.TFCOND

Toggles listing of false conditional blocks

.LALL

Turns on listing of all statements in macros

.SALL

Suppresses listing of all macro expansions

.XALL

Lists macro statements that generate code or data

TITLE

Defines program listing title

SUBTTL

Defines program listing subtitle

PAGE

Increments page number (printer-oriented LST  files

only)

Directive Summary

2.5.1. .LIST

Syntax:

.LIST

Turns on listing of most  source  statements.  This  is  the

default. Listing file must be enabled with an Assembler Flag or

command-line option.

See Also:

.XLIST, .LFCOND, .SFCOND, .TFCOND

2.5.2. .XLIST

Syntax:

.XLIST

Suppresses program listing. Used in conjunction  with  .LIST

to prevent a particular section of a  source  file  from  being

listed.

See Also:

.LIST, .LFCOND, .SFCOND, .TFCOND

2.5.3. .LFCOND

Syntax:

.LFCOND

Enables listing of  statements  in  conditional  blocks  for

which the condition evaluates to false (0).

See Also:

.SFCOND, .TFCOND, .LIST, .XLIST

2.5.4. .SFCOND

Syntax:

.SFCOND

Suppresses listing of statements in conditional blocks whose

condition  evaluates  to  false  (0)  and  were  therefore  not

assembled. This is the default behavior.

See Also:

.LFCOND, .TFCOND, .LIST, .XLIST

Directive Summary

2.5.5. .TFCOND

Syntax:

.TFCOND

Toggles the state  of  the  assembler  between  .LFCOND  and

.SFCOND; these two directives determine whether  or  not  false

conditional blocks are included in assembler listings.

See Also:

.LFCOND, .SFCOND, .LIST, .XLIST

2.5.6. .LALL

Syntax:

.LALL

Enables listing of all source  statements  in  a

macro expansion,  including  normal  comments,  but

excluding macro comments.

See Also:

.SALL, .XALL, .LIST, .XLIST

2.5.7. .SALL

Syntax:

.SALL

Suppresses listing of macro expansions in  a  listing  file.

Macro calls are listed, but not the source  code  generated  by

the call.

See Also:

.LALL, .XALL

2.5.8. .XALL

Syntax:

.XALL

Enables an assembly listing file  including  macro-expansion

statements that generate code or data. Comments,  equates,  and

segment definitions are ignored. This is the default.

See Also:

.LALL, .SALL

Directive Summary

2.5.9. TITLE

Syntax:

TITLE text

Specifies  title  to  be  used  on  each  page  of  assembly

listings. Specified text is printed flush left  on  the  second

line of each page of the listing. This directive is only useful

with printer-oriented listings.

Only one TITLE per  module  is  allowed.  Default  title  is

blank.

text

Any combination of up to 60 characters.

See Also:

PAGE, PAGE +, SUBTTL

2.5.10. SUBTTL

Syntax:

SUBTTL text

Specifies  the  subtitle  used  on  each  page  of  assembly

listings. Specified text is printed flush  left  on  the  third

line of the listing pages. This directive is useful  only  with

printer-oriented listing files.

Any number of subtitles can be given in a program. Each  new

SUBTTL replaces  the  current  subtitle.  Default  subtitle  is

blank.

text

Any combination of up to 60 characters.

See Also:

PAGE, PAGE +, TITLE

2.5.11. PAGE

Syntax:

PAGE [[length],width]

For listing files, sets maximum number  of  lines  per  page

(length) and maximum number of  characters  per  line  (width).

Generates  a  page  break  if  no  arguments  are  given.  This

directive is useful only with printer-oriented listings.

length

A number in the range 10 to 255 lines  (default  is

50).

width

A number in the range 60 to 132 characters (default

is 80).  To  specify  width  without  changing  the

default length, use a comma before width.

See Also:

PAGE +

Directive Summary

2.6. Macro

Directives

Description

MACRO

Marks  beginning  of  macro,  specifying  name  and

parameters

ENDM

Ends macro definition or repeat block

EXITM

Terminates expansion of the macro

LOCAL

Declares name to be local to macro

PURGE

Purges macro definitions from memory

REPT

Starts a block repeated n times

IRP

Starts a block  repeated  once  for  each  argument

given

IRPC

Starts a block repeated once for each character  in

a string

2.6.1. MACRO

Syntax:

name MACRO [parameter [,parameter]...]

statements

ENDM

Marks beginning of a  macro  block  called  name  and  takes

optional parameters. The assembler generates the statements  in

the macro block each time the macro is called in source code.

name

A unique symbol name. Can appear  later  in  source

code, to call the macro.

parameter

A valid symbol name. Each parameter can  appear  in

the statements and is replaced by the corresponding

item in the argument list  whenever  the  macro  is

called.

See Also:

ENDM, LOCAL, EXITM

2.6.2. ENDM

Syntax:

ENDM

Terminates macro and repeat blocks.

See Also:

MACRO, REPT, IRP, IRPC

Directive Summary

2.6.3. EXITM

Syntax:

EXITM

Terminates expansion of the current macro  or  repeat  block

and begins assembly of the next statement outside the block. If

encountered in a nested macro block, the assembler  returns  to

expanding the outer block.

See Also:

IRP, IRPC, REPT, MACRO, ENDM

2.6.4. LOCAL (in macros)

Syntax:

LOCAL localname [,localname]...

Declares that a  label  or  symbol  to  be  used

locally  within  a  macro  or  repeat  block.   The

assembler generates a different name for the symbol

each time it expands the macro, thus permitting you

to  call  a  macro  that  may  contain  local  jump

statements without generating the same label twice.

localname

The name of a  label  or  symbol  that  can  appear

elsewhere   in   the   macro.   Separate   multiple

localnames with commas.

See Also:

MACRO, ENDM, EXITM, REPT, IRP, IRPC

2.6.5. PURGE

Syntax:

PURGE macroname [,macroname]...

Deletes the specified macros or group of macros from memory.

A macro can purge itself if PURGE is on the last  line  of  the

macro. This directive should be used if your  system  does  not

have enough memory to keep all the macros needed for  a  source

file in memory at the same time.

Redefining  a  macro  automatically  purges   the   previous

definition.

macroname

A currently defined macro.

See Also:

MACRO, ENDM, EXITM

Directive Summary

2.6.6. REPT

Syntax:

REPT expression

statements

ENDM

Marks a block of statements to be assembled a  fixed  number

of times (determined by value of expression).

Repeat blocks must be terminated with ENDM.

expression

Must evaluate to a numeric constant.

statements

Any valid assembler statements.

See Also:

IRP, IRPC

2.6.7. IRP

Syntax:

IRP parameter,<argument [,argument]...>

statements

ENDM

Marks a block repeated once  for  each  argument  specified,

with  the  current  argument  replacing   parameter   on   each

repetition.

parameter

A valid symbol name. If the  parameter  appears  in

statements, then it  is  replaced  by  a  different

argument each time.

argument

Text, symbol, string, or numeric constant. Multiple

arguments must be separated by commas.

See Also:

ENDM, EXITM, IRPC, REPT

2.6.8. IRPC

Syntax:

IRPC parameter,string

statements

ENDM

Marks a block  repeated  once  for  each  character  in  the

string, with the character  replacing  the  parameter  on  each

repetition.

parameter

Directive Summary

A valid symbol name. If the  parameter  appears  in

statements, then it  is  replaced  by  a  different

character each time.

string

A literal string or a symbol defined with a  string

equate.

See Also:

ENDM, EXITM, IRP, REPT

2.7. Miscellaneous Directives

Mnemonic

Description

COMMENT

Marks beginning of a comment block

END

Marks end of source file and optionally sets  start

address

INCLUDE

Includes contents of another source file

INCLUDELIB

Declares default library to be searched by linker

%OUT

Displays a message during assembly

.RADIX

Sets default radix for constants

2.7.1. COMMENT

Syntax:

COMMENT delimiter [text]

text

[text] delimiter [text]

Treats  all  text  between  or  on  the  same  line  as  the

delimiters as a comment.

delimiter

First  nonblank  character  following  the  COMMENT

directive.

text

All text up to and including  the  line  containing

the next occurrence of the delimiter.

See Also:

semicolon (;), semicolon (;;)

Directive Summary

2.7.2. END

Syntax:

END [startaddress]

Marks the end of  a  module  and  optionally  indicates  the

address where execution will begin when the program is  loaded.

Only one module in a program can define a startaddress. Use  of

the .STARTUP directive removes the need to give a startaddress.

END also closes the last segment in the source file.

startaddress

Label or expression identifying the  address  where

you want program execution to begin.

See Also:

.STARTUP

2.7.3. INCLUDE

Syntax:

INCLUDE filespec

Inserts source code from the include file filespec into  the

current source file during assembly. Nested INCLUDE  directives

are allowed.

filespec

Must specify  an  existing  file  containing  valid

assembly-language statements. Can be a filename  or

a complete or relative file specification including

drive or directory name.

2.7.4. INCLUDELIB

Syntax:

INCLUDELIB libraryname

Informs the linker that the current module should be  linked

with a specified library file.  Can  be  used  to  specify  the

library file in the assembly source file instead of in the LINK

command line.

libraryname

Must be a valid filename or full path name.  If  no

extension is specified, the default extension  .LIB

is assumed.

2.7.5. %OUT Directive

Syntax:

%OUT text

Displays text to the standard  output  device.  Though  this

device is normally the screen, at the DOS command level you can

redirect it to a file or another  device.  This  directive  can

Directive Summary

help show which statements were assembled and can be useful for

debugging.

text

Any line of ASCII characters. For  multiple  lines,

use a separate %OUT directive for each.

See Also:

.ERR

2.7.6. .RADIX

Syntax:

.RADIX expression

Sets the default radix (base) for integer constants  to  the

value of expression. Expression defines whether the numbers are

binary, octal, decimal, hexadecimal, or numbers of  some  other

base.  Numbers  given  in  expression  are  always   considered

decimal.

Does not affect real numbers initialized as  variables  with

the DD, DQ, or DT directives; these always use decimal radix.

expression

Must evaluate to a number in the range 2 to 16.

See Also:

DD, DQ, DT

2.8. Multimodule Directives

To share variables between  modules,  declare  the  variable

PUBLIC in the module where it is defined  and  EXTRN  in  other

modules.

Mnemonic

Description

PUBLIC

Makes  each  variable  specified  public  to  other

modules

EXTRN

Defines one or more external variables

COMM

Declares one or more communal variables

INCLUDELIB

Declares default library to be searched by linker

Directive Summary

2.8.1. PUBLIC

Syntax:

PUBLIC [langtype] name [,[langtype] name]...

Makes one or more variables available to  other  modules  in

the program.

PUBLIC declarations can be placed  anywhere  in  the  source

module in which the symbol is  defined.  Text  macros  or  text

equates cannot be declared public.

langtype

C,  BASIC,  PASCAL,  or  FORTRAN.  The  C  langtype

appends an underscore to the name as output to  the

object file.

name

The name of a variable, label,  or  numeric  equate

defined within the current source file.

See Also:

EXTRN, COMM

2.8.2. EXTRN

Syntax:

EXTRN     [langtype]     name:type     [,[langtype]

name:type]...

Defines one or more external variables, labels, or  symbols,

each having a name and a specific  type.  The  symbol  must  be

defined and declared PUBLIC in another module in the program.

langtype

C,  BASIC,  PASCAL,  or  FORTRAN.  The  C  langtype

appends an underscore to the name as output to  the

object file.

name

A unique symbolic name

type

The same type given to the item in the module where

defined: i.e., NEAR, FAR, PROC, BYTE, WORD,  DWORD,

QWORD, TBYTE, or ABS

See Also:

PUBLIC, COMM

2.8.3. COMM

Syntax:

COMM definition [,definition]...

Creates one  or  more  communal  variables,  in  which  each

variable definition has the following form:

Directive Summary

[NEAR|FAR] [langtype]label:size[:count]

langtype

C,  BASIC,  PASCAL,  or  FORTRAN.  The  C  langtype

appends an underscore to the name as output to  the

object file.

label

The name of the variable.

size

A size specifier (BYTE, WORD,  etc.)  or  structure

name.

count

Specifies the number of  data  objects  (1  is  the

default).

See Also:

EXTRN, PUBLIC, PROC

2.8.4. INCLUDELIB

Syntax:

INCLUDELIB libraryname

Informs  the  linker  that  the  current  module

should be linked with a specified library file. Can

be used to specify the library file in the assembly

source file instead of in the LINK command line.

libraryname

Must be a valid filename or full path name.  If  no

extension is specified, the default extension  .LIB

is assumed.

2.9. Processor Directives

Mnemonic

Description

.186

Enables instruction set of 80186 processor

.286

Enables instruction set of 80286 processor

.287

Enables instruction set of 80287 math coprocessor

.8086

Disables processor instructions except for 8086 set

.8087

Disables coprocessor instructions except  for  8087

set

Directive Summary

2.9.1. .186

Syntax:

.186

Enables assembly of instructions for the 8086 processor, the

8087  coprocessor,  and  the  80186  processor,  and   disables

instructions specific to the 80286 and 80287 processors.

See Also:

.286, .287, .8086, .8087

2.9.2. .286

Syntax:

.286

Enables assembly of instructions for the 8086 processor plus

the additional real mode instructions for the 80286  processor,

and instructions for  the  80287  coprocessor.  This  directive

should be used only in programs executing on  .80286  or  80386

processors.

See Also:

.186, .287, .8086, .8087

2.9.3. .287

Syntax:

.287

Enables assembly of instructions for the 8087 floating-point

coprocessor and of the additional instructions for  the  80287.

Specifies IEEE format for encoding floating-point variables.

This directive takes advantage of the 80287 to optimize some

coprocessor instructions.  The  8087  does  not  support  these

optimized instructions.

See Also:

.186, .286, .8086, .8087

2.9.4. .8086

Syntax:

.8086

Enables assembly of  instructions  for  the  8086  and  8088

processors and  the  8087  coprocessor.  Disables  assembly  of

instructions unique to later processors. This directive  is  in

effect by default.

See Also:

Directive Summary

.186, .286, .287, .8087

2.9.5. .8087

Syntax:

.8087

Enables  assembly  of  instructions  for   the   8087   math

coprocessor, and disables assembly of the  instructions  unique

to later coprocessors. It specifies IEEE  format  for  encoding

floating-point  variables.  This  directive  is  in  effect  by

default.

See Also:

.186, .286, .287, .8086, .MSFLOAT

2.10. Segment Directives

Mnemonic

Description

SEGMENT

Marks beginning of a segment and defines attributes

ENDS

Marks end of a segment

GROUP

Groups one or more segments into a single  physical

segment

ASSUME

Informs assembler where  a  segment  register  will

point to

DOSSEG

Orders segments according to DOS segment convention

.ALPHA

Orders segments alphabetically by name

.SEQ

Orders segments according to source-code sequence

2.10.1. SEGMENT

Syntax:

name SEGMENT [align] [combine] ['class']

statements

name ENDS

Defines  a  program  segment  called  name  having  optional

segment attributes:  align,  combine,  use,  and  class.  These

attributes give the linker and assembler information on how  to

set up and combine segments.

name

Defines the name of the segment.

align

PARA (default), BYTE, WORD, DWORD, or PAGE

combine

PRIVATE (default), PUBLIC, STACK,  COMMON,  MEMORY,

Directive Summary

or AT address

class

Can contain a valid symbolic  name.  The  name  has

special meaning to the linker, indicating a segment

containing executable code.

See Also:

ENDS, ASSUME, GROUP

2.10.2. HELLOF.ASM

;* HELLOF.ASM - Illustrates full segment directives.

;*

;* Shows:   Directives - SEGMENT     ENDS       GROUP

;*          DOS Functions - 4Ch (Terminate Process

;*                               with Return Code)

;*

;* See HELLO.ASM for a simplified segment version or HELLOC.ASM

;* for a full-segment .COM file version.

DOSSEG                      ; Force DOS segment order

DGROUP  GROUP   _DATA, STACK        ; Stack and data in DGROUP

STACK   SEGMENT PARA STACK 'STACK'

DB      100h DUP (?)            ; 256-byte stack

STACK   ENDS

_DATA   SEGMENT WORD PUBLIC 'DATA'; Data segment with string da

msg     DB      "Hello, world.", 13, 10, "$"

_DATA   ENDS

_TEXT   SEGMENT WORD PUBLIC 'CODE'      ; Code segment

ASSUME  cs:_TEXT, ds:DGROUP, ss:DGROUP

start:  mov     ax, DGROUP   ; Initialize data segment

mov     ds, ax

cli                  ; Turn off interrupts

mov     ss, ax       ; Make SS and

mov     sp, OFFSET STACK  ; SP relative to DGROUP

sti                       ; SS now equals DS

mov     ah, 9h       ; Request DOS Function 9

mov     dx, OFFSET msg

; Load DX with offset of string (segment already in DS)

int     21h   ; Display String to Standard Output

mov     ax, 4C00h  ; Exit functions with 0 in AL

int     21h  ; Terminate Process with Return Code

_TEXT   ENDS

END     start ; End with reference to first

;   executable statement (CS:IP)

Directive Summary

2.10.3. ENDS

Syntax:

name ENDS

Marks the  end  of  a  segment  name  or  a  structure  name

previously begun with SEGMENT or STRUC.

See Also:

SEGMENT, STRUC

2.10.4. GROUP

Syntax:

name GROUP segment [,segment]...

Causes linker to recombine several segments into one  larger

segment, so that the items  in  all  segments  share  a  common

segment address. The total of all segments in the group must be

no larger than 64k.

name

Unique symbolic name, or previously defined  group.

If the same group name appears  in  more  than  one

GROUP statement, then the effect is cumulative; the

new segments are added to the group.

segment

Any previously defined segment or SEG expression.

See Also:

ASSUME, SEGMENT, SEG

2.10.5. ASSUME

Syntax:

ASSUME segregister:name [,segregister:name]...

Informs the assembler that at run time,  the  given  segment

register  will  point  to  the  named  segment  or  group.  The

assembler uses this information to correctly calculate offsets.

Each time you load a new value into a segment register,  you

should inform the assembler of this fact  by  using  an  ASSUME

statement.

segregister

CS, DS, ES, or SS

name

The name of a previously defined segment or  group,

or NOTHING,  will  tells  the  assembler  that  the

segment register currently holds no useful value.

See Also:

SEGMENT, GROUP

Directive Summary

2.10.6. HELLOC.ASM

;* illustratesfiles with full segment directives.

;* Assemble with the "Generate .COM File" linker flag or the

;* /TINY QCL option.

;*

;* Shows:    Directives - SEGMENT   ASSUME    ORG

;*

;* See HELLO.ASM for a simplified segment version or

;* HELLOF.ASM for a full-segment .EXE file version.

_TEXT   SEGMENT WORD PUBLIC 'CODE'

ASSUME  cs:_TEXT, ds:_TEXT

ORG     100h

first:  jmp     start                   ; Jump over data

msg     DB      "Hello, world.", 13, 10, "$"

start:  mov     ah, 9h     ; Request DOS Function 9

mov     dx, OFFSET msg ; Load DX with offset of string

;   (segment already in DS)

int     21h   ; Display String to Standard Output

mov     ax, 4C00h  ; Exit functions with 0 in AL

int     21h        ; Exit Program with Return Code

_TEXT   ENDS

END     first

2.10.7. DOSSEG

Syntax:

DOSSEG

Orders segments according to the  standard  DOS  convention;

overrides any other segment-order directives or  options.  This

convention places segments with class name 'CODE'  first,  then

other segments not part of DGROUP, and then DGROUP segments  in

this order: BEGDATA, BSS,  STACK.  Other  DGROUP  segments  are

placed between BEGDATA and BSS.

The DOSSEG directive also causes the linker to generate  two

special symbols, _end and _edata. Avoid using these  names  for

your own symbols.

See Also:

.SEQ, .ALPHA

2.10.8. HELLO.ASM

;* HELLO.ASM - Illustrates simplified segment directives.

;*

;* Shows:  Directives - TITLE     .MODEL    DOSSEG      .STACK

;*                      .DATA     .CODE     .STARTUP    OFFSET

;*                      .EXIT     END

Directive Summary

;*         DOS Functions - 09h (Display String)

;*

;* See HELLOF.ASM for a full-segment .EXE file version or

;* HELLOC.ASM for a full-segment .COM file version.

TITLE   HELLO

.MODEL  small, c  ; Small model (could be any model)

DOSSEG            ; Force DOS segment order

.STACK  100h      ; 256-byte stack

.DATA             ; Data segment

msg     DB      "Hello, world.", 13, 10, "$"

.CODE             ; Code segment

.STARTUP          ; Initialize data segment and

;   set SS = DS

mov     ah, 9h    ; Request DOS Function 9

mov     dx, OFFSET msg ; Load DX with offset of string

;   (segment already in DS)

int     21h       ; Display String

.EXIT   0         ; Exit  with return code 0

END

2.10.9. .ALPHA

Syntax:

.ALPHA

Outputs segments to the object file  by  alphabetical  order

(using the  names  of  the  segments).  The  default  order  is

sequential, which outputs segments in the order they appear  in

source code.

See Also:

.SEQ and DOSSEG

2.10.10. .SEQ

Syntax:

.SEQ

Outputs segments to the object file in the same  order  they

appeared in source code. This is the default order, but you can

use this directive to override the effect of the QCL /a option.

See Also:

DOSSEG, .ALPHA

Directive Summary

2.11. Simplified Segment Directives

Mnemonic

Description

.MODEL

Defines basic attributes of  the  module,  such  as

memory model

.STARTUP

Declares start address and initializes registers

.EXIT

Generates code to exit from program

.CODE

Starts a code segment

.STACK

Defines stack and optionally gives size of stack

.DATA

Starts a near-data segment

.DATA?

Starts a near-data segment for uninitialized data

.CONST

Starts a near-data segment for constant data

.FARDATA

Starts a far-data segment

.FARDATA?

Starts a far-data segment for uninitialized data

2.11.1. .MODEL

Syntax:

.MODEL memorymodel [,langtype] [,stacktype]

Initializes the program memory model. Should  be  placed  at

the beginning of the source file before  any  other  simplified

segment directive. .MODEL can only occur once.  Creates  ASSUME

and GROUP statements used by specified memory model.

If you use the language argument with .MODEL, all  procedure

names are public.

memorymodel

TINY, SMALL, COMPACT, MEDIUM, LARGE,  or  HUGE.  If

you  are  writing  an  assembler  routine   for   a

high-level  language,   this   should   match   the

memorymodel used by the compiler or interpreter.

langtype

C,  Pascal,  FORTRAN,  or  BASIC.   Instructs   the

assembler to follow the naming, calling, and return

conventions for the indicated language.

stacktype

Legal  values:  nearStack   (SS=DS)   or   farStack

(SS!=DS). nearStack is default.

See Also:

@DataSize, .CODE, .DATA, ASSUME, GROUP, EXTRN

Directive Summary

2.11.2. .STARTUP

Syntax:

.STARTUP

Generates startup code for the given model  and  stack  type

(default is nearStack) defined by .MODEL. Initializes  DS,  SS,

and SP as needed. Defines a start-address label,  so  that  you

don't need to give a start  address  with  END.  Use  only  for

stand-alone programs.

With TINY model, .STARTUP also generates ORG 100h.

If you use the .STARTUP directive, you should also  use  the

.EXIT directive at the  end  of  the  program.  Otherwise,  the

assembler generates an advisory warning.

See Also:

.MODEL, END, ORG, .EXIT

2.11.3. EXIT

Syntax:

.EXIT [exitcode]

Generates a call to interrupt 21H,  function  4CH,  to  exit

from the  program.  Use  of  the  optional  exitcode  parameter

returns an exit code to DOS. The  .MODEL  directive  must  have

previously been used. This directive  should  be  used  in  any

module  that  uses  the  .STARTUP  directive;   the   assembler

generates an advisory warning if you use .STARTUP without using

.EXIT.

exitcode

Optional return code. Can be a byte-sized register,

memory location or constant.  The  .EXIT  directive

moves this value into AL.

See Also:

.STARTUP

2.11.4. .CODE

Syntax:

.CODE [name]

Indicates the start of a  code  segment  and  ends  previous

segment, if any. The .MODEL directive must have previously been

used.  Each  .CODE  directive  generates  an  ASSUME  statement

associating CS with the current segment.

Directive Summary

name

Optional parameter  for  medium,  large,  and  huge

models: uses name_TEXT for the code  segment  name.

If name is not specified, the  assembler  generates

the  segment  name  _TEXT  (tiny,  small,   compact

models) or module_TEXT (other models).

See Also:

@code, .MODEL

2.11.5. .STACK

Syntax:

.STACK [size]

Defines the program stack segment (STACK) and ends  previous

segment, if any. The .MODEL directive must have previously been

used. You should generally not place any code or data  in  this

segment; the processor uses the stack  segment  to  store  data

during procedure and interrupt calls.

The .MODEL directive generates a GROUP statement that places

the STACK segment in DGROUP, unless the farStack  attribute  is

given.

size

Optional; specifies size of the stack in bytes.  If

none is given, the default is 1024.

See Also:

.DATA, .DATA?, .FARDATA, .MODEL, .CONST

2.11.6. .DATA

Syntax:

.DATA

Starts  the  initialized  data  segment  (_DATA)  and   ends

previous segment,  if  any.  The  .MODEL  directive  must  have

previously been used. This segment should  contain  all  global

data that have initial values.

The .MODEL directive generates a GROUP statement that places

_DATA in DGROUP.

See Also:

.FARDATA, .MODEL, .CONST, .STACK

Directive Summary

2.11.7. .DATA?

Syntax:

.DATA?

Starts  the  uninitialized  data  segment  (_BSS)  and  ends

previous segment,  if  any.  The  .MODEL  directive  must  have

previously been used. This segment should  contain  all  global

data that have no initial values.

Although uninitialized data  can  be  placed  in  the  .DATA

segment, use of .DATA? is recommended for maximum compatability

with high-level languages.

The .MODEL directive generates a GROUP statement that places

_DATA in DGROUP.

See Also:

.MODEL

2.11.8. .CONST

Syntax:

.CONST

Starts a constant data segment (with segment name CONST) and

ends previous segment, if any. The .MODEL directive  must  have

previously been used. This segment should contain strings, real

numbers, and other constant data that  may  be  accessed  by  a

high-level language.

The .MODEL directive generates a GROUP statement that places

CONST in DGROUP.

See Also:

.MODEL, .STACK, .DATA

2.11.9. .FARDATA

Syntax:

.FARDATA [name]

Starts a far-data segment for initialized data (segment name

FAR_DATA or name) ending previous segment, if any.  The  .MODEL

directive must have previously  been  used.  Data  in  segments

defined with .FARDATA is not placed in a group or combined with

.FARDATA segments for other source modules.

name

A unique  segment  name.  This  optional  parameter

allows you to  create  multiple  far-data  segments

within a source module.

Directive Summary

See Also:

.DATA, .MODEL

2.11.10. .FARDATA?

Syntax:

.FARDATA? [name]

Starts a far-data segment for  uninitialized  data  (segment

name FAR_BSS or name) ending  previous  segment,  if  any.  The

.MODEL directive  must  have  previously  been  used.  Data  in

segments defined with .FARDATA? is not placed  in  a  group  or

combined with .FARDATA? segments for other source modules.

name

A unique  segment  name.  This  optional  parameter

allows you to  create  multiple  far-data  segments

within a source module.

See Also:

.DATA, .MODEL

2.12. Record and Structure Directives

A record consists of bit fields. A structure is a collection

of any series of data types and can include other structures.

Mnemonic

Description

RECORD

Declares a  record  type  consisting  of  specified

fields

STRUC

Declares a structure type having specified fields

ENDS

Ends a structure definition

2.12.1. RECORD

Syntax:

recordname RECORD field [,field]...

Declares a record  type  consisting  of  the  specified  bit

fields. Multiple fields must be separated by a comma.  The  sum

of the widths for all fields must not exceed 16 bits.

recordname

Directive Summary

A unique symbolic name.

field

Each field has the following form, in  which  width

is a constant giving the  number  of  bits  in  the

field, and expression optionally sets an initial or

default value.

fieldname:width[= expression]

See Also:

STRUC, MASK, WIDTH

2.12.2. STRUC

Syntax:

name STRUC

fielddeclarations

name ENDS

Declares  a  structure  as  a  new  data  type.  Conditional

assembly blocks and comments are  legal,  but  not  nesting  of

other structures.

name

A unique symbolic name;  represents  the  structure

type. Structure variables can then be defined.

fielddeclarations

Any valid data definitions (using  DB,  DW,  etc.).

Initializers specify default values.

Once a structure type is declared, you can use it to  define

variables, in which each  field  not  initialized  assumes  the

default value:

[name] structuretype <[initializer [,initializer]...>

See Also:

RECORD, ENDS

2.12.3. ENDS

Syntax:

name ENDS

Marks the  end  of  a  segment  name  or  a  structure  name

previously begun with SEGMENT or STRUC.

See Also:

SEGMENT, STRUC

Operator Summary

3. Operator Summary

Category

Description

Arithmetic

Perform arithmetic calculations at assemble time

Macro

Alter evaluation within a macro definition

Logical and shift

Perform logic calculations at assemble time

Record

Generate bit mask or determine size of record

Type

Specify or analyze type of a variable

Address

Extract segment or offset address

Relational

Compare two values at assemble time

Miscellaneous

Comment and DUP (repeated initial values)

3.1. Arithmetic Operator Summary

+

Addition

-

Subtraction

*

Multiplication

/

Division

MOD

Modular division

.

Field operator

[]

Index operator

3.1.1. + (Plus, Binary)

Syntax:

expression1 + expression2

Plus   (binary).   Returns   value   of   expression1   plus

expression2.

expression

An integer constant or memory operand.  The  result

can be used as a memory operand.

See Also:

minus (-)

Operator Summary

3.1.2. + (Plus, Unary)

Syntax:

+expression

Plus (unary). Designates a positive number. Has higher level

of precedence than the binary (addition) plus.

expression An integer constant.

See Also:

minus (-)

3.1.3. - (Minus, Binary)

Syntax:

expression1 - expression2

Binary   minus.   Returns   value   of   expression1   minus

expression2. Has lower precedence  than  the  unary  (negative)

minus. Can be used to subtract one memory operand from another,

but only if the operands refer to  locations  within  the  same

segment. The result is a constant.

expression An integer constant or memory  operand.  The  result

can be used as a memory operand.

See Also:

plus (+)

3.1.4. - (Minus, Unary)

Syntax:

-expression

Unary minus. Reverses sign of expression. Has  higher  level

of precedence than the binary (subtraction) minus.

expression

An integer constant.

See Also:

plus (+)

Operator Summary

3.1.5. * (Multiplication)

Syntax:

expression1 * expression2

Multiplication operator. Returns product of two expressions.

expression1

Must be an integer constant.

expression2

Must be an integer constant.

See Also:

slash (/)

3.1.6. / (Division)

Syntax:

expression1 / expression2

Integer division. Returns quotient of expression1 divided by

expression2.

expression

An integer constant.

See Also:

asterisk (*)

3.1.7. MOD

Syntax:

expression1 MOD expression2

Returns the remainder (modulus) of dividing  expression1  by

expression2.

expression1

Must be an integer constant.

expression2

Must be an integer constant.

See Also:

slash (/)

Operator Summary

3.1.8. . (Structure-Field Reference)

Syntax:

variable.field

Returns the offset of field plus  the  offset  of  variable.

This operator is normally used to access a field of data within

a structure variable.

variable

A previously defined data item; usually a structure

variable.

field

A field within a previously defined structure type.

Each such field has an offset from the beginning of

the structure.

See Also:

STRUC

3.1.9. [] (Index Operator)

Syntax:

expression1[expression2]

[expression2]

In the syntax above, the brackets  are  intended  literally.

The expression outside the brackets is optional.

The brackets (also called the "index operator")  return  the

sum of expression1 and expression2. If expression  in  brackets

is a register, then the brackets indicate  an  indirect  memory

operand rather than a register-direct operand.

Two direct memory operands cannot be added.  Typically,  the

expression outside the brackets is an array  address,  and  the

expression inside is an offset to an array element  (always  in

terms of bytes).

See Also:

plus (+), period (.)

3.2. Macro Operator Summary

<>

Literal-text operator

!

Literal-character operator

;;

Macro comment (is not expanded in macro call)

%

Expression operator

Operator Summary

&

Substitute operator

3.2.1. <> (Literal Text Operator)

Syntax:

<text>

Treats text in a macro argument as a single literal  string.

Most often used in macro calls and with the  IRP  directive  to

ensure that values in a parameter list are treated as a  single

parameter. The assembler removes one set of angle brackets each

time it inserts an argument into a macro expansion.

text

Any  name,  number,  or  expression.  Can   contain

commas, spaces, tabs, or semicolons.

See Also:

EQU, IRP, exclamation (!)

3.2.2. ! (Literal Character Operator)

Syntax:

!character

Treats character in a macro argument as a literal  character

rather than  as  an  operator  or  symbol.  Used  with  special

characters such as the semicolon or ampersand  when  the  usual

assembly language meaning of the character must be  suppressed.

See Also:

anglebrackets (<>)

3.2.3. ;; (Macro Comment Delimeter)

Syntax:

;;text

Treats text to end of line as a macro comment.  The  comment

appears  in  the  macro  definition,  but  not  in   statements

generated by macro expansion.

See Also:

COMMENT, semicolon (;)

Operator Summary

3.2.4. % (Expression Operator)

Syntax:

%text

Treats  text  in  macro  argument  as  an  expression.   The

assembler computes the expression's  value  and  replaces  text

with the result. Used in macro calls when the programmer  needs

to pass the result of an  expression  rather  than  the  actual

expression to a macro.

When placed at beginnning of the line, % causes substitution

of all text macros on that line.

text

Can be  either  a  numeric  expression  or  a  text

equate.

See Also:

ampersand (&)

3.2.5. & (Substitute Operator)

Syntax:

parm1&parm2

&parm

parm&

Used within a  macro  or  repeat  block;  enables  parameter

substitution to take  place  for  both  parm1  and  parm2.  The

resulting strings are then joined together to form one  symbol.

You can join any number of strings with &.  This  operator  can

also be used to force parameter substitution  within  a  quoted

string.

See Also:

percent (%)

3.3. Logic/Shift Operator Summary

AND

Bitwise AND

OR

Bitwise inclusive OR

XOR

Bitwise exclusive OR

NOT

Logical negation of each bit

SHL

Shift bits left

SHR

Shift bits right

Operator Summary

3.3.1. AND

Logical AND                          Flags:  O D I T S Z A P C

-----------------

Syntax:  AND dest,src                        0       * * ? * 0

Performs a bitwise logical AND on the source and destination

operands and stores the result in the destination operand.  For

each bit position in the operands, if both bits are  set,  then

the corresponding bit of the  result  is  set.  Otherwise,  the

corresponding bit of the result is cleared.

001000dw     mod,reg,r/m       disp (0 or 2)

--------------+------------------------+-----------------------

AND  reg,reg | and   dx,bx            | 88/86 3

|                        | 286   2

|                        | 386   2

--------------+------------------------+-----------------------

AND  mem,reg | and   bitmask,bx       | 88/86 16+EA (W88=24+EA

| and   [bp+2],dx        | 286   7

|                        | 386   7

--------------+------------------------+-----------------------

AND  reg,mem | and   bx,masker        | 88/86 9+EA (W88=13+EA)

| and   dx,marray[bx+di] | 286   7

|                        | 386   6

--------------+------------------------+-----------------------

100000sw     mod,100,r/m       disp (0 or 2)     data (1 or 2)

---------------+---------------------+-------------------------

AND  reg,immed| and   dx,0F7h       | 88/86 4

|                     | 286   3

|                     | 386   2

---------------+---------------------+-------------------------

AND  mem,immed| and   masker,1001b  | 88/86 17+EA (W88=23+EA)

|                     | 286   7

|                     | 386   7

---------------+---------------------+-------------------------

0010010w     data (1 or 2)

-------------------+-------------------------+-----------------

AND  accum,immed  | and   ax,0B6h           | 88/86     4

|                         | 286       3

|                         | 386       2

-------------------+-------------------------+-----------------

3.3.2. OR

Inclusive OR                         Flags:  O D I T S Z A P C

-----------------

0       * * ? * 0

Syntax:  OR dest,src

Performs a bitwise logical OR on the source and  destination

operands and stores the result to the destination operand.  For

each bit position in the operands, if either or both  bits  are

set, the corresponding bit of the result it set. Otherwise, the

corresponding bit of the result is cleared.

Operator Summary

000010dw    mod, reg, r/m      disp (0 or 2)

--------------+---------------------+--------------------------

OR  reg,reg  | or    ax,dx         | 88/86 3

|                     | 286   2

|                     | 386   2

--------------+---------------------+--------------------------

OR  mem,reg  | or    [bp+6],cx     | 88/86 16+EA (W88=24+EA)

| or    bits,dx       | 286   7

|                     | 386   7

--------------+---------------------+--------------------------

OR  reg,mem  | or    bx,masker     | 88/86 9+EA (W88=13+EA)

| or    dx,color[di]  | 286   7

|                     | 386   6

--------------+---------------------+--------------------------

100000sw    mod,001, r/m       disp (0 or 2)     data (1 or 2)

--------------+---------------------+--------------------------

OR  reg,immed| or    dx,110110b    | 88/86 4

|                     | 286   3

|                     | 386   2

--------------+---------------------+--------------------------

OR  mem,immed| or    flag_rec,8    | 88/86 (b=17,w=25)+EA

|                     | 286   7

|                     | 386   7

--------------+---------------------+--------------------------

0000110w    data (1 or 2)

------------------+-------------------------+------------------

OR  accum,immed  | or    ax,40h            | 88/86 4

|                         | 286   3

|                         | 386   2

------------------+-------------------------+------------------

3.3.3. XOR

Exclusive OR                         Flags:  O D I T S Z A P C

-----------------

Syntax:  XOR dest,src                        0       * * ? * 0

Performs  a  bitwise  exclusive  OR  on   the   source   and

destination operands and stores the result to the  destination.

For each bit position in the operands, if both bits are set  or

if both bits are cleared, the corresponding bit of  the  result

is cleared. Otherwise, the corresponding bit of the  result  is

set.

001100dw    mod,reg,r/m      disp (0 or 2)

-------------+-------------------------+-----------------------

XOR  reg,reg| xor    cx,bx            | 88/86 3

| xor    ah,al            | 286   2

|                         | 386   2

-------------+-------------------------+-----------------------

XOR  mem,reg| xor    [bp+10],cx       | 88/86 16+EA (W88=24+EA

| xor    masked,bx        | 286   7

|                         | 386   6

-------------+-------------------------+-----------------------

XOR  reg,mem| xor    cx,flags         | 88/86 9+EA (W88=13+EA)

| xor    bl,bitarray[di]  | 286   7

|                         | 386   7

-------------+-------------------------+-----------------------

Operator Summary

100000sw    mod,110,r/m      disp (0 or 2)     data (1 or 2)

-----------------+----------------------+----------------------

XOR  reg,immed  | xor bx,10h           | 88/86 4

| xor bl,1             | 286   3

|                      | 386   2

-----------------+----------------------+----------------------

XOR  mem,immed  | xor Boolean,1        | 88/86 17+EA (W88=25+E

| xor switches[bx],101b| 286   7

|                      | 386   7

-----------------+----------------------+----------------------

0011010w    data (1 or 2)

-----------------+---------------------+-----------------------

XOR  accum,immed| xor    ax,01010101b | 88/86 4

|                     | 286   3

|                     | 386   2

-----------------+---------------------+-----------------------

3.3.4. NOT

One's Complement Negation            Flags:  O D I T S Z A P C

-----------------

Syntax:  NOT dest

Toggles each bit of the operand by  clearing  set  bits  and

setting cleared bits.

1111011w    mod, 010,r/m      disp (0 or 2)

--------------+---------------------+--------------------------

NOT  reg     | not   ax            | 88/86 3

|                     | 286   2

|                     | 386   2

--------------+---------------------+--------------------------

NOT  mem     | not   masker        | 88/86 16+EA (W88=24+EA)

|                     | 286   7

|                     | 386   6

--------------+---------------------+--------------------------

3.3.5. SAL/SAR/SHL/SHR

Shift                                Flags:  O D I T S Z A P C

-----------------

Syntax:  SHL dest,1                          *       * * ? * *

SHL dest,CL

SHL dest,number (80186/286/386 Only)

Shifts the bits in the destination  operand  the  number  of

times specified by the source operand. SAL and  SHL  shift  the

bits left; SAR and SHR shift right.

With SHL, SAL, and SHR, the bit shifted off the end  of  the

operand is copied into the  carry  flag  and  the  leftmost  or

rightmost bit opened by the shift is set to 0.  With  SAR,  the

bit shifted off the end of the operand is copied into the carry

flag and the leftmost bit  opened  by  the  shift  retains  its

previous value (thus preserving the sign of the  operand).  SAL

and SHL are synonyms; they have the same effect.

On the 8088 and 8086, the source operand can be either CL or

Operator Summary

1. On the 80186-80386 processors, the source operand can be  CL

or an 8-bit constant.  On  the  80186-80386  processors,  shift

counts larger than 31 are masked off, but on the 8088 and 8086,

larger shift counts  are  performed  despite  the  inefficiency

involved. The overflow flag  is  only  modified  by  single-bit

variations of the instruction; for multiple-bit  variations  it

is undefined.

1101000w     mod,TTT*,r/m      disp (0 or 2)

* TTT represents one of the following bit codes:

100 for SHL or SAL,

101 for SHR, or 111 for SAR.

-------------+--------------------------+----------------------

SAR  reg,1  | sar    di,1              | 88/86 2

| sar    cl,1              | 286   2

|                          | 386   3

-------------+--------------------------+----------------------

SAL  reg,1  | shr    dh,1              | 88/86 2

SHL  reg,1  | shl    si,1              | 286   2

SHR  reg,1  | sal    bx,1              | 386   3

-------------+--------------------------+----------------------

SAR  mem,1  | sar    count,1           | 88/86 15+EA (W88=23+E

|                          | 286   7

|                          | 386   7

-------------+--------------------------+----------------------

SAL  mem,1  | sal    WORD PTR m32[0],1 | 80/86 15+EA (W88=23+E

SHL  mem,1  | shl    index,1           | 286   7

SHR  mem,1  | shr    unsign[di],1      | 386   7

-------------+--------------------------+----------------------

1101001w     mod,TTT*,r/m      disp (0 or 2)

*TTT represents one of the following bit codes:

100 for SHL or SAL, 101

for SHR,  or 111 for SAR.

-------------+---------------------------+---------------------

SAR  reg,CL | sar    bx,cl              | 88/86 8+4n

| sar    dx,cl              | 286   5+n

|                           | 386   3

-------------+---------------------------+---------------------

SAL  reg,CL | shr    dx,cl              | 88/86 8+4n

SHL  reg,CL | shl    di,cl              | 286   5+n

SHR  reg,CL | sal    ah,cl              | 386   3

-------------+---------------------------+---------------------

SAR  mem,CL | sar    sign,cl            | 88/86 20+EA+4n

|                           | (W88=28+EA+4n)

| sar    WORD PTR [bp+8],cl | 286   8+n

|                           | 386   7

-------------+---------------------------+---------------------

SAL  mem,CL | shr    WORD PTR m32[2],cl | 88/86 20+EA+4n

|                           | (W88=28+EA+4n)

SHL  mem,CL | sal    BYTE PTR [di],cl   | 286   8+n

SHR  mem,CL | shl    index,cl           | 386   7

-------------+---------------------------+---------------------

Operator Summary

11000000w    mod,TTT*,r/m      disp (0 or 2)     data (1)

*TTT represents one of the following bit codes:

100 for SHL or SAL,

101 for SHR,  or 111 for SAR.

-----------------+-----------------------+---------------------

SAR  reg,immed8 | sar    bx,5           | 88/86 -

| sar    cl,5           | 286   5+n

|                       | 386   3

-----------------+-----------------------+---------------------

SAL  reg,immed8 | sal    cx,6           | 88/86 -

SHL  reg,immed8 | shl    di,2           | 286   5+n

SHR  reg,immed8 | shr    bx,8           | 386   3

-----------------+-----------------------+---------------------

SAR  mem,immed8 | sar    sign_count,3   | 88/86 -

| sar    WORD PTR [bx],5| 286   8+n

|                       | 386   7

-----------------+-----------------------+---------------------

SAL  mem,immed8 | shr    mem16,11       | 88/86 -

SHL  mem,immed8 | shl    unsign,4       | 286   8+n

SHR  mem,immed8 | sal    array[bx+di],14| 386   7

-----------------+-----------------------+---------------------

3.4. Record Operator Summary

MASK

Constructs bit mask

WIDTH

Returns width of record

3.4.1. MASK

Syntax:

MASK {recordfieldname|record}

Returns a bit  mask  for  the  bit  positions  in  a  record

occupied by the given record field. A bit in the mask  contains

a 1 if that bit corresponds to a  field  bit.  All  other  bits

contain 0.

recordfieldname

The name of  any  field  in  a  previously  defined

record.

record

The name of any previously defined record.

See Also:

WIDTH, NOT, RECORD

Operator Summary

3.4.2. WIDTH

Syntax:

WIDTH {recordfieldname|record}

Returns width (in bits) of a record  or  record  field.  The

width of a field is the number of bits assigned for that field;

the value of the field  is  the  starting  position  (from  the

right) of the field.

recordfieldname

The name of any field defined in any record.

record

Any defined record.

See Also:

MASK, RECORD

3.5. Type Operator Summary

HIGH

Returns high byte

LOW

Returns low byte

PTR

Temporarily alters type of data

SHORT

Specifies SHORT (* 128 bytes)

SIZE

Returns size of type

THIS

Creates symbol of specified type

TYPE

Returns number of bytes in variable

.TYPE

Returns a byte giving information on variable

3.5.1. HIGH

Syntax:

HIGH expression

Returns high-order (most significant)  byte  of  expression.

Cannot be used on  contents  of  a  memory  operand  or  on  an

address.

expression

Must evaluate to a constant value.

Operator Summary

See Also:

LOW

3.5.2. LOW

Syntax:

LOW expression

Returns low-order byte of  expression.  Cannot  be  used  on

contents of a memory operand or on an address.

expression

Must evaluate to a constant.

See Also:

HIGH

3.5.3. PTR

Syntax:

type PTR expression

Treats expression as having the specified  type.  Used  with

forward references to define explicitly what size or distance a

reference has. Also  used  to  enable  instructions  to  access

variables that would otherwise generate errors.

The PTR keyword can also appear in the argument  list  of  a

PROC statement, to indicate that the  parameter  is  a  pointer

variable.

type

For memory operands,  can  be  BYTE,  WORD,  DWORD,

QWORD, TBYTE. For labels,  can  be  NEAR,  FAR,  or

PROC.

expression

Any operand.

See Also:

TYPE, THIS

3.5.4. SHORT

Syntax:

SHORT label

Sets label type to SHORT. Short labels can be  used  in  JMP

Operator Summary

instructions when the distance is less than 128 bytes from  the

start of the next instruction. Instructions using short  labels

are a  byte  smaller  than  identical  instructions  using  the

default near labels.

See Also:

JMP

3.5.5. SIZE

Syntax:

SIZE variable

Returns the total number of bytes allocated for variable  if

variable was defined with the DUP operator. The returned  value

is equal to the value of LENGTH variable  times  the  value  of

TYPE variable.

If the variable was not declared with the DUP operator,  the

value returned is the same as TYPE variable;  it  simply  gives

the number of bytes in the data type.

variable

Array or other variable.

See Also:

DUP, LENGTH, TYPE

3.5.6. THIS

Syntax:

THIS type

Creates an operand of specified type with offset and segment

values equal to the current location-counter value.  Used  with

EQU or = to create labels and variables.

type

For memory operands,  can  be  BYTE,  WORD,  DWORD,

QWORD, or TBYTE. For labels, can be NEAR,  FAR,  or

PROC.

See Also:

EQU, equalsign (=), LABEL

Operator Summary

3.5.7. TYPE

Syntax:

TYPE expression

Returns a different result depending on type of  expression:

Expression

TYPE Operator returns

variable

Number of bytes in data object.

structure

Total size in bytes of the structure.

label

0FFFFh=NEAR label, 0FFFEh=FAR label.

constant

0.

The returned value can be used to specify the type for  PTR.

See Also:

PTR

3.5.8. .TYPE

Syntax:

.TYPE expression

Returns a byte defining the mode and scope of expression. If

expression is not valid, a 0  is  returned.  If  expression  is

valid, a byte is  returned  in  which  the  contents  mean  the

following:

Bit position   Bit Value = 0         Bit Value = 1

0         Not program related   Program related

1         Not data related      Data related

2         Not a consant         Constant value

3         Addressing mode is    Addressing mode is direc

not direct

4         Not a register        Expression is a register

5         Not defined           Defined

7         Local/public scope    External scope

See Also:

MACRO

Operator Summary

3.6. Address Operator Summary

SEG

Returns segment of object in memory

OFFSET

Returns offset of object in memory

3.6.1. SEG

Syntax:

SEG expression

Returns the segment  address  of  expression.  The  returned

value can be used as a memory operand.

expression

Any label, variable, segment name, group  name,  or

other memory operand. Cannot be a constant.

See Also:

OFFSET

3.6.2. OFFSET

Syntax:

OFFSET expression

Returns offset address of expression (usually  a  variable).

The value returned by  the  OFFSET  operator  is  an  immediate

operand.

If you use simplified segment directives, then the offset of

an item in one of the near data segments (DATA,  STACK,  CONST)

is calculated relative to the beginning of  DGROUP,  the  group

containing  all  these  segments.  If  you  give  full  segment

definitions, the returned value is a memory  operand  equal  to

the number of bytes between the item and the beginning  of  the

segment in which it is defined.

expression

Any  label,  variable,  or  other   direct   memory

operand.

See Also:

: (segment-override)

Operator Summary

3.7. Relational Operator Summary

These   operators    are    typically    used    with    the

conditional-assembly IF statement and return Boolean results.

EQ

Equal

NE

Not Equal

GT

Greater Than

GE

Greater Than or Equal to

LT

Less Than

LE

Less Than or Equal to

3.7.1. EQ

Syntax:

expression1 EQ expression2

Verifies equality. Returns true (-1) if  expression1  equals

expression2, or false (0) if it does not. Typically  used  with

conditional directives.

expression1

Must evaluate to a constant value.

expression2

Must evaluate to a constant value.

See Also:

equalsign (=), NE, LT, LE, GT, GE

3.7.2. NE

Syntax:

expression1 NE expression2

Returns true (-1) if expression1 does not equal expression2,

or  returns  false  (0)  if  it  does.  Typically   used   with

conditional directives.

expression1

Must evaluate to a constant value.

expression2

Must evaluate to a constant value.

See Also:

Operator Summary

EQ, LT, LE, GT, GE

3.7.3. GT

Syntax:

expression1 GT expression2

Returns  true  (-1)   if   expression1   is   greater   than

expression2, or returns false (0) if it is not. Typically  used

with conditional directives.

expression1

Must evaluate to a constant value.

expression2

Must evaluate to a constant value.

See Also:

EQ, NE, LT, LE, GE

3.7.4. GE

Syntax:

expression1 GE expression2

Returns true (-1) if expression1 is greater than or equal to

expression2, or returns false (0) if it is not. Typically  used

with conditional directives.

expression1

Must evaluate to a constant value.

expression2

Must evaluate to a constant value.

See Also:

EQ, NE, LT, LE, GT

3.7.5. LT

Syntax:

expression1 LT expression2

Returns true (-1) if expression1 is less  than  expression2,

or returns  false  (0)  if  it  is  not.  Typically  used  with

conditional directives.

expression1

Must evaluate to a constant value.

Operator Summary

expression2

Must evaluate to a constant value.

See Also:

EQ, NE, LE, GT, GE

3.7.6. LE

Syntax:

expression1 LE expression2

Returns true (-1) if expression1 is less than  or  equal  to

expression2, or false (0) if it is  not.  Typically  used  with

conditional directives.

expression1

Must evaluate to a constant value.

expression2

Must evaluate to a constant value.

See Also:

EQ, NE, LT, GT, GE

3.8. Miscellaneous Operator Summary

;

Comment (not assembled)

DUP

Repeat initialization values

?

Uninitialized value

\

Line continuation

$

Current location indicator

3.8.1. ; (Comment Delimeter)

Syntax:

;text

Treats text to end of line as a comment.  The  comment  must

follow all other fields in the statement.

See Also:

COMMENT, semicolons (;;)

Operator Summary

3.8.2. DUP

Syntax:

count DUP (initialvalue [,initialvalue]...)

Defines  data  structures.  Specifies  (count)   number   of

declarations of initialvalue. Arrays, buffers, and  other  data

structures consisting of multiple data objects of the same size

can be defined with the DUP operator. Creates  an  operand  for

use with data-definition directives.

DUP operators can be nested up to 17 levels.

count

Number of declarations of initialvalue.

initialvalue

The undefined symbol (?), another DUP operator,  or

expression that evaluates to an  integer  value  or

character constant. Multiple initial values must be

separated by commas.

See Also:

DB, DW, DD, DT, DQ

3.8.3. ? (Uninitialized Value)

Syntax:

?

Used as an initializer in  data  declarations;  indicates  a

value that the assembler allocates but does not initialize. Can

be used alone or with DUP, as in the following examples:

foo      DW      ?           ; Allocate one uninitialized wo

foo2     DB      ?,?,?       ; Allocate three uninitialized

foo3     DB      20 DUP (?)  ; Allocate 20 uninitialized byt

3.8.4. \ (Line Continutation)

Syntax:

text \

text

Enables an assmebly-language statement to continue  to  next

line, so that the line-break is ignored. Any  number  of  lines

can be joined with this method. A backslash is not considered a

line-continuation character if it occurs inside a comment.

Predefined Equates

4. Predefined Equates

Each of the following predefined equates has  its  own  help

topic:

@code              @CodeSize            @Cpu

@CurSeg            @data                @DataSize

@FileName          @Model               @Version

@WordSize

In addition to these equates, each of the  other  simplified

segment  directives  (.CONST,  .STACK,  .DATA?,  .FARDATA,  and

.FARDATA?) has a corresponding equate. For example, @fardata is

an equate giving the segment address generated by the  .FARDATA

directive.

4.1. @code

Syntax:

@code

Predefined equate representing the segment name  defined  by

.CODE; Can be used in ASSUME statements or any place a  segment

name would be valid.

See Also:

@CodeSize, @data

4.2. Vector

Predefined Equates

;* Vector - Macro to read current interrupt vector,

;* store it, and replace it.

;*

;* Shows:   Equates - @CodeSize     @code

;*

;* Params:  num - Vector number

;*          old - Pointer to doubleword for storing old vector

;*          new - Pointer to new handler

Vector MACRO num, old, new       ;; Macro definition

push ds                      ;; Save DS and ES registers

push es

mov ah, 35h                  ;; AH = DOS function number

mov al, num                  ;; AL = interrupt number

int 21h                      ;; Get Interrupt Vector

mov WORD PTR old[0], bx      ;; Store it

mov WORD PTR old[2], es

IF @CodeSize                 ;; If medium or large model,

lds dx, new              ;;    load DS from parameter

ELSE

mov bx, @code            ;; Else ensure DS points to

mov ds, bx               ;;    to code segment

mov dx, new              ;; DS:DX equals new vector

ENDIF

mov ah, 25h                  ;; AH = DOS function number

int 21h                      ;; Set Interrupt Vector

pop es                       ;; Restore ES and DS

pop ds

ENDM

4.3. @CurSeg

Syntax:

@CurSeg

Predefined equate returning the name of the current segment.

This equate may be useful for  ASSUME  statements  and  segment

overrides (especially inside of macros), as well as for  ending

the current segment:

@CurSeg ENDS

4.4. @FileName

Syntax:

@FileName

Predefined equate returning the base  name  of  the  current

source file. This directive is  useful  for  any  situation  in

which you want a name to change whenever the name of  the  file

changes.

Predefined Equates

4.5. @WordSize

Syntax:

@WordSize

Predefined text macro returning the size  of  a  word.  With

Quick-Assembler, this number is always 2. When the  program  is

assembled with the Microsoft Macro Assembler, then a value of 4

is possible. A value of 4 would indicate that  the  program  is

running in the 32-bit mode supported on 80386 processors.

4.6. @CodeSize

Syntax:

@CodeSize

Predefined equate giving a value of 0 for tiny,  small,  and

compact models, or 1 for medium, large, and  huge  models.  The

.MODEL directive must have previously been given.

See Also:

@DataSize, .MODEL

4.7. @data

Syntax:

@data

Predefined equate representing the group (DGROUP) containing

all near data segments: .DATA, .DATA?, .CONST., and .STACK.  To

access data in any of these segments, DS must contain the value

@data.  (The  .STARTUP  directive   automatically   does   this

initialization.) The .MODEL directive must have previously been

given.

See Also:

.CONST, .STACK, .STARTUP

4.8. @Model

Syntax:

@Model

A predefined  equate  that  returns  one  of  the  following

values, depending on memory model previously defined  with  the

.MODEL directive:

1) TINY

2) SMALL

3) COMPACT

4) MEDIUM

5) LARGE

6) HUGE

Predefined Equates

See Also:

@CodeSize, @DataSize

4.9. @Cpu

Syntax:

@Cpu

Predefined equate returning a different number, depending on

processor, as selected by previous processor directive (8086 is

default).

1

8086 processor enabled

3

80186 processor enabled

7

80286 processor enabled

4.10. @DataSize

Syntax:

@DataSize

Predefined equate giving a value of 0 for tiny,  small,  and

medium models, or 1 for compact, large, and  huge  models.  The

.MODEL directive must have previously been given.

See Also:

@CodeSize, .MODEL

4.11. @Version

Syntax:

@Version

Prefined macro returning  a  three-digit  string

giving the name  of  the  current  version  of  the

assembler.   For   QuickAssembler   Version   2.01,

@Version returns the string "520". For Version  5.1

of the Microsoft Macro Assembler, @Version  returns

the string "510".

Specifying Radixes

5. Specifying Radixes

Name           Base   Specifier       Digits

Binary           2      B             0 1

Octal            8      Q or O        0 1 2 3 4 5 6 7

Decimal         10      D             0 1 2 3 4 5 6 7 8 9

Hexadecimal     16      H             0 1 2 3 4 5 6 7 8 9

A B C D E F

To indicate the radix of a constant, place the specifier  at

the end of the number. Specifiers and hexadecimal digits can be

uppercase or lowercase. Hexadecimal numbers must  always  start

with a decimal digit (0 to 9). If the first digit is one of the

letters A  to  F,  prefix  the  number  with  a  leading  0  to

distinguish it from a symbolic name.

Useful Tables

6. Useful Tables

- Regular Expressions

- Assembly Operator Precedence

- Data Types and Specifiers

- Register and Flag Summary

- Key Scan Chart

- Program Segment Prefix

- Color Display Attributes

6.1. Regular Expressions

Special Characters in Regular Expressions

Period       .   Matches any single character.

Caret        ^   Matches text at the beginning of a line.

Dollar sign  $   Matches text at the end of a line. The dollar

sign must appear at the end of the text.

Asterisk     *   Matches zero or more repetitions of the

character preceding the asterisk.

Brackets    [ ]  Matches sets of the characters specified

within the brackets.

Caret ^  Matches any character EXCEPT those

specified within the brackets.

It must be the first character within

the brackets.

Dash  -  Matches characters in the ASCII order

between the characters on either side

inclusive.

Backslash   \    Removes special meaning from next character.

Assembler Operator Precedence

1) LENGTH, SIZE, WIDTH, MASK, (), [], <>

2) . (structure-field operator)

3) :

4) PTR, OFFSET, SEG, TYPE, THIS

5) HIGH, LOW

6) +, - (unary)

7) *, /, MOD, SHL, SHR

8) +, - (binary)

9) EQ, NE, LT, LE, GT, GE

10) NOT

11) AND

12) OR, XOR

13) SHORT, .TYPE

Useful Tables

6.2. Data Types

The LABEL, PROC, EXTRN, LOCAL, and COMM directives  can  all

use the type specifiers listed in the first column. The  second

and third columns show corresponding data  directive  (if  any)

and number of bytes per element.

BYTE       DB       1

WORD       DW       2

DWORD      DD       4

QWORD      DQ       8

TBYTE      DT      10

FAR                 4

NEAR                2

PROC               code size of memory model (2 or 4)

6.3. Register Summary

Accumulator

AX (AH/AL) Multiply, divide, I/O, fast arithmetic

Base

BX (BH/BL) Pointer to base address (data segment)

Count

CX (CH/CL) Count for loops, repeats, and shifts

Data

DX (DH/DL) Multiply, divide, and I/O

Source Index

SI Source string and index pointer

Destination Index

DI Destination string and index pointer

Base Pointer

BP Pointer to base address (stack segment)

Stack Pointer

SP Pointer to top of stack

Code Segment

CS Flags Flags

Data Segment

DS Instruction Pointer IP

Stack Segment

SS

Extra Segment

ES

6.4. Flags Summary

15               7             0

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| | | | |O|D|I|T|S|Z| |A| |P| |C|  Layout of flags register

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Useful Tables

Flag If

set:

O Overflow

Indicates result too large or small for destination

D Direction

String operations process down rather than up

I Interrupt

Enables interrupts (disables if cleared)

T Trap

Switches to single-step mode

S Sign

Indicates result has negative sign

Z Zero

Indicates result equal to 0

A Auxiliary Carry

Indicates carry or borrow to low four-order bits

P Parity

Indicates even number of bits in result

C Carry

Indicates carry or borrow to destination operand

6.5. Register Setup

When DOS loads an EXE program, it sets registers as follows:

ES

Points to segment of Program Segment Prefix (PSP)

DS

Points to segment of PSP

CS:IP

Points to CS:IP address in EXE header

SS:SP

Points to SS:SP address in EXE header

When DOS  loads  a  COM  program,  it  sets  all

segment registers to the beginning of the  program,

and IP to 100H.

6.6. Keyboard Scan Codes

Useful Tables

ESC       01    U          16   | or \      2B   F6          40

! or 1    02    I          17   Z           2C   F7          41

@ or 2    03    O          18   X           2D   F8          42

# or 3    04    P          19   C           2E   F9          43

$ or 4    05    { or [     1A   V           2F   F10         44

% or 5    06    } or ]     1B   B           30   NUMLOCK     45

^ or 6    07    ENTER      1C   N           31   SCROLL LOCK 46

& or 7    08    CTRL       1D   M           32   HOME or 7   47

* or 8    09    A          1E   < or ,      33   UP or 8     48

( or 9    0A    S          1F   > or .      34   PGUP or 9   49

) or 0    0B    D          20   ? or /      35   -           4A

_ or -    0C    F          21   RIGHT SHIFT 36   LEFT or 4   4B

+ or =    0D    G          22   PRTSC or *  37   5           4C

LEFT      0E    H          23   ALT         38   RIGHT or 6  4D

TAB       0F    J          24   SPACEBAR    39   +           4E

Q         10    K          25   CAPSLOCK    3A   END or 1    4F

W         11    L          26   F1          3B   DOWN or 2   50

E         12    : or ;     27   F2          3C   PGDN or 3   51

R         13    " or '     28   F3          3D   INS or 0    52

T         14      or `     29   F4          3E   DEL or .    53

Y         15    LEFT SHIFT 2A   F5          3F

6.7. Program Segment Prefix

Range       Size of Area    Description

00H - 01H   02H   (2)       INT 20H

02H - 03H   02H   (2)       Segment of first address after

end of block

04H - 04H   01H   (1)       Reserved

05H - 09H   05H   (5)       Opcode for far call to

DOS function dispatcher

0AH - 0DH   04H   (4)       Saved contents of terminate

routine vector

0EH - 11H   04H   (4)       Saved contents of CTRL+BREAK vector

12H - 15H   04H   (4)       Saved contents of error

routine vector

16H - 2BH   14H  (22)       Reserved

2CH - 2DH   02H   (2)       Segment address of environment

2EH - 5BH   28H  (40)       Reserved

5CH - 6BH   10H  (16)       First command-line argument

6CH - 7FH   10H  (16)       Second command-line argument

80H - FFH   80H (128)       Command line and/or Disk

Transfer Area (DTA)

6.8. Color Display Attributes

Useful Tables

Key to Bits: I=Intensity G=Green F=Flashing R=Red B=Blue

Background                              Foreground

Bits      Num  Color              Bits      Num  Color

F R G B                           I R G B

0 0 0 0   0    Black              0 0 0 0   0    Black

0 0 0 1   1    Blue               0 0 0 1   1    Blue

0 0 1 0   2    Green              0 0 1 0   2    Green

0 0 1 1   3    Cyan               0 0 1 1   3    Cyan

0 1 0 0   4    Red                0 1 0 0   4    Red

0 1 0 1   5    Magenta            0 1 0 1   5    Magenta

0 1 1 0   6    Brown              0 1 1 0   6    Brown

0 1 1 1   7    White              0 1 1 1   7    White

1 0 0 0   8    Black blink        1 0 0 0   8    Dark grey

1 0 0 1   9    Blue blink         1 0 0 1   9    Light blue

1 0 1 0   A    Green blink        1 0 1 0   A    Light green

1 0 1 1   B    Cyan blink         1 0 1 1   B    Light cyan

1 1 0 0   C    Red blink          1 1 0 0   C    Light red

1 1 0 1   D    Magenta blink      1 1 0 1   D    Light magenta

1 1 1 0   E    Brown blink        1 1 1 0   E    Yellow

1 1 1 1   F    White blink        1 1 1 1   F    Bright white

Interrupt 21H Summary (DOS)

7. Interrupt 21H Summary (DOS)

Category

Description

Character I/O

Old-style (DOS1) read and write to devices

Directory Operations

Get, set, create, and delete directory

Disk Management

Select drive and get drive status

File Operations

Open, close, and manage files

File Operations (FCB)

Functions use File Control Block--not recommended

Memory Management

Allocate or resize memory block

Miscellaneous

Miscellaneous system functions

Process Management

Execute or terminate process

Record Operations

Read and write to file

Record Operations (FCB)

Functions use File Control Block--not recommended

Time and Date

Get or set date and time

7.1.    Interrupt    21H,    Character-Oriented

Functions

Hex #  Dec #     Name

01H     1        Character Input with Echo

02H     2        Character Output

03H     3        Auxiliary Input

04H     4        Auxiliary Output

05H     5        Printer Output

06H     6        Direct Console I/O

07H     7        Unfiltered Character Input without Echo

08H     8        Character Input without Echo

09H     9        Display String

0AH    10        Buffered Keyboard Input

0BH    11        Check Input Status

0CH    12        Flush Input Buffer, Then Input

7.1.1. Int 21H Function 01H

Description:

Character Input with Echo

Reads a character from the standard input device and  echoes

it to the standard output device. If  no  character  is  ready,

waits until one is available. Input can be redirected (if input

Interrupt 21H, Character-Oriented Functions

has been redirected, there is no way to detect EOF).

Input

Output

AH=01H

AL=8-bit input data

- If the standard input is not redirected and the  character

read is a CTRL+C, an Int 23H is executed. If the  standard

input is redirected, a CTRL+C is detected at the  console,

and BREAK is ON, an Int 23H is executed.

- To read extended ASCII codes (such as the special function

keys F1 to F10) on the IBM PC and  compatibles,  you  must

call this function twice. The first call returns the value

00H to signal the presence of an extended code.

- See also Int  21H  Functions  06H,  07H,  and  08H,  which

provide character input with various combinations of  echo

and/or CTRL+C sensing.

- You can also read the keyboard by issuing a read (Int  21H

Function 3FH) using the predefined handle for the standard

input (0000H), if input has not been redirected, or  using

a handle obtained by opening the logical device CON.

7.1.2. Int 21H Function 02H

Description:

Character Output

Outputs a character to the standard  output  device.  Output

can be redirected (if output is redirected, there is no way  to

detect disk full).

Input                               Output

------------------------            -------

AH=02H                              Nothing

DL=8-bit data for output

- If  a  CTRL+C  is  detected  at  the  keyboard  after  the

requested character is output, an Int 23H is executed.

- If  the  standard  output  has  not  been  redirected,   a

backspace code (08H) causes the cursor to  move  left  one

position. If output has  been  redirected,  the  backspace

code does not receive any special treatment.

- You can also send strings to the display by  performing  a

write (Int 21H Function 40H) using the  predefined  handle

for the standard output (0001H), if output  has  not  been

redirected, or using a  handle  obtained  by  opening  the

logical device CON.

Interrupt 21H, Character-Oriented Functions

7.1.3. PUTSTR.ASM

;* PUTSTR.ASM illustrates multiple-modules and memory model

;* independent techniques. To create the program, paste this

;* module and HELLOM.ASM (get help on HELLOM.ASM to open it)

;* into separate program files. In the environment, you must

;* also create a program list for the program and define a mode

;* in the Defines field of the Assembler Flags dialog box.

;* For example, use "model=large". Alternately, you could assem

;* outside the environment with the following command line:

;*      qcl /Dmodel=large hellom.asm putstr.asm

;*

;* Shows:   Directives - PUBLIC    PROC     ENDP   PTR

;*          DOS function - 02h (Character Output)

; Model is symbol passed from command line or environment.

; Note that the % operator is required to receive external

; constant symbols.

%       .MODEL  model, c

; With simplified segment directives, procedures such

; as PutStr are automatically PUBLIC.

; Data is not automatically public and must be declared with

; PUBLIC in order to be accessed from other modules. With

; full segment directives, the following line would

; be required to make PutStr known to other modules.

;PUBLIC PutStr

.CODE                ; Address of string (near or far

PutStr  PROC    str:PTR BYTE ;   depending on model) is passed

;   the stack

mov     ah, 02h      ; Character output function

IF      @DataSize    ; If data is far (compact, large,

les     di, str      ;   or huge), address is ES:DI

next:   mov     dl, es:[di]  ; Load each character through ES:[

ELSE                 ; If data is near (tiny, small,

mov     di, str      ;   or medium), address is DI only

next:   mov     dl, [di]     ; Load each character through [DI]

ENDIF

or      dl, dl       ; Check to see if character is zer

jz      exit         ; If zero, done

int     21h          ; DOS displays character

inc     di           ; Point to next character

jmp     SHORT next   ; Loop back to load character

exit:   ret

PutStr  ENDP

END

Interrupt 21H, Character-Oriented Functions

7.1.4. Int 21H Function 03H

Description:

Auxiliary Input

Reads a character from the standard  auxiliary  device.  The

default is the first serial port (COM1).

Input             Output

------            -------------------

AH=03H            AL=8-bit input data

- In most MS-DOS systems, the serial  device  is  unbuffered

and is not interrupt driven. If the auxiliary device sends

data faster than your program can process  it,  characters

may be lost.

- At startup on the IBM PC,  PC-DOS  initializes  the  first

serial port to 2400 baud, no parity, 1  stop  bit,  and  8

data bits. Other implementations of MS-DOS may  initialize

the serial device differently.

- There is no way for a user program to read the  status  of

the auxiliary device or to detect I/O errors (such as lost

characters) through this function call.  On  the  IBM  PC,

more precise control may be obtained by calling  ROM  BIOS

Int  14H  or  by  driving  the  communications  controller

directly.

- If a CTRL+C is detected at the keyboard,  an  Int  23H  is

executed.

- You can also input from the auxiliary device by requesting

a read (Int 21H Function 3FH) using the predefined  handle

for the standard  auxiliary  device  (0003H)  or  using  a

handle obtained by opening the logical device AUX.

7.1.5. Int 21H Function 04H

Description:

Auxiliary Output

Outputs a character to the standard  auxiliary  device.  The

default is the first serial port (COM1).:

Input                               Output

------------------------            -------

AH=04H                              Nothing

DL=8-bit data for output

- If the output device is busy, this  function  waits  until

the device is ready to accept a character.

- There is no way to poll the status of the auxiliary device

using this function. On the IBM PC, more  precise  control

can be obtained by calling ROM BIOS Int 14H or by  driving

the communications controller directly.

- If a CTRL+C is detected at the keyboard,  an  Int  23H  is

Interrupt 21H, Character-Oriented Functions

executed.

- You can also send  strings  to  the  auxiliary  device  by

performing a  write  (Int  21H  Function  40H)  using  the

predefined  handle  for  the  standard  auxiliary   device

(0003H) or using a handle obtained by opening the  logical

device AUX.

7.1.6. Int 21H Function 05H

Description:

Printer Output

Sends a character to the standard list device.  The  default

is the printer  on  the  first  parallel  port  (LPT1),  unless

explicitly redirected by the user with the MODE command.

Input                                Output

-------------------------            -------

AH=05H                               Nothing

DL=8-bit data for output

- If the printer is busy,  this  function  waits  until  the

printer is ready to accept the character.

- There is no standardized way to poll  the  status  of  the

printer under MS-DOS.

- If a CTRL+C is detected at the keyboard,  an  Int  23H  is

executed.

- You can also send strings to the printer by  performing  a

write (Int 21H Function 40H) using the  predefined  handle

for the standard printer device (0004H) or using a  handle

obtained by opening the logical device PRN or LPT1.

7.1.7. Int 21H Function 06H

Description:

Direct Console I/O

Used by programs that need to read and  write  all  possible

characters and control codes without any interference from  the

operating system. Reads a character  from  the  standard  input

device or writes a character to the standard output device. I/O

may be redirected (if I/O has been redirected, there is no  way

to detect EOF or disk full).

Interrupt 21H, Character-Oriented Functions

Input                             Output

-----------------------------     -----------------------------

AH=06H                            If called with DL=00H-0FEH

DL=function requested             Nothing

=00H-FEH  if output request

=0FFH     if input request      If called with DL=FFH and a

character is ready

Zero flag=clear

AL=8-bit input data

If called with DL=FFH and a

character is ready

Zero flag=set

- No special action is taken upon entry  of  a  CTRL+C  when

this service is used.

- To read extended ASCII codes (such as the special function

keys F1 to F10) on the IBM PC and  compatibles,  you  must

call this function twice. The first call returns the value

00H to signal the presence of an extended code.

- See also Int  21H  Functions  01H,  07H,  and  08H,  which

provide character input with various combinations of  echo

and/or CTRL+C sensing, and Functions 02H  and  09H,  which

may be used to write characters to the standard output.

- You can also read the keyboard by issuing a read (Int  21H

Function 3FH) using the predefined handle for the standard

input (0000H), if input has  not  been  redirected,  or  a

handle obtained by opening the logical device CON.

- You can also send characters to the display by  issuing  a

write (Int 21H Function 40H) using the  predefined  handle

for the standard output (0001H), if output  has  not  been

redirected, or using a  handle  obtained  by  opening  the

logical device CON.

7.1.8. Int 21H Function 07H

Description:

Unfiltered Character Input Without Echo

Reads a character from the  standard  input  device  without

echoing it to the standard output device. If  no  character  is

ready, waits until one is available. Input  may  be  redirected

(if input has been redirected, there is no way to detect  EOF).

Input             Output

------            -------------------

AH=07H            AL=8-bit input data

- No special action is taken upon entry  of  a  CTRL+C  when

this function is used. If CTRL+C checking is required, use

Int 21H Function 08H instead.

- To read extended ASCII codes (such as the special function

keys F1 to F10) on the IBM PC and  compatibles,  you  must

call this function twice. The first call returns the value

00H to signal the presence of an extended code.

Interrupt 21H, Character-Oriented Functions

- See also Int  21H  Functions  01H,  06H,  and  08H,  which

provide character input with various combinations of  echo

and/or CTRL+C sensing.

- You can also read the keyboard by issuing a read (Int  21H

Function 3FH) using the predefined handle for the standard

input (0000H), if input has not been redirected, or  using

a handle obtained by opening the logical device CON.

7.1.9. Int 21H Function 08H

Description:

Character Input Without Echo

Reads a character from the  standard  input  device  without

echoing it to the standard output device. If  no  character  is

ready, waits until one is available. Input  may  be  redirected

(if input has been redirected, there is no way to detect  EOF).

Input             Output

------            -------------------

AH=08H            AL=8-bit input data

- If the standard input is not redirected, and the character

read is a CTRL+C, an Int 23H is executed. If the  standard

input is redirected, a CTRL+C is detected at the  console,

and BREAK is ON, an Int 23H is executed. To avoid possible

interruption  by  a  CTRL+C,  use  Int  21H  Function  07H

instead.

- To read extended ASCII codes (such as the special function

keys F1 to F10) on the IBM PC and  compatibles,  you  must

call this function twice. The first call returns the value

00H to signal the presence of an extended code.

- See also Int  21H  Functions  01H,  06H,  and  07H,  which

provide character input with various combinations of  echo

and/or CTRL+C sensing.

- You can also read the keyboard by issuing a read (Int  21H

Function 3FH) using the predefined handle for the standard

input (0000H), if input has not been redirected, or  using

a handle obtained by opening the logical device CON.

7.1.10. Int 21H Function 09H

Description:

Display String

Sends a string of characters to the standard output  device.

Output may be redirected (if output has been redirected,  there

is no way to detect disk full).

Interrupt 21H, Character-Oriented Functions

Input                                     Output

------------------------------            -------

AH=09H                                    Nothing

DS:DX=segment:offset of string

- The string must be terminated with the character $  (24H),

which is not transmitted. Any other ASCII codes, including

control codes, can be embedded in the string.

- See Int 21H Functions 02H  and  06H  for  single-character

output to the video display or standard output device.

- If a CTRL+C is detected at the keyboard,  an  Int  23H  is

executed.

- You can also send strings to the display by  performing  a

write (Int 21H Function 40H) using the  predefined  handle

for the standard  output  (0001H),  if  it  has  not  been

redirected, or using a  handle  obtained  by  opening  the

logical device CON.

7.1.11. Int 21H Function 0AH

Description:

Buffered Keyboard Input

Reads a string of bytes from the standard input  device,  up

to and including an ASCII carriage  return  (0DH),  and  places

them in a user-designated buffer. The characters are echoed  to

the standard output device. Input may be redirected  (if  input

has been redirected, there is no way to detect EOF).

Input                                     Output

------------------------------            ------

AH=0AH                                    Nothing

DS:DX=segment:offset of buffer

- The buffer used by this function has the following format:

Byte

Contents

0

maximum number of characters to read,  set  by

program

1

number of characters actually read  (excluding

carriage return), set by MS-DOS

2+

string read from keyboard or  standard  input,

terminated by a carriage return (0DH)

- If the buffer fills to one fewer than the  maximum  number

of characters it can hold, subsequent input is ignored and

the bell is sounded until a carriage return is detected.

- This  input   function   is   buffered   with   type-ahead

capability, and  all  of  the  standard  keyboard  editing

commands are active.

Interrupt 21H, Character-Oriented Functions

- If the standard input is not redirected, and a  CTRL+C  is

detected at the console, an Int 23H is  executed.  If  the

standard input is redirected, a CTRL+C is detected at  the

console, and BREAK is ON, an Int 23H is executed.

- See  Int  21H  Functions  01H,  06H,  07H,  and  08H   for

single-character input from the keyboard or standard input

device.

- You can also read strings from the keyboard by  performing

a read (Int 21H Function 3FH) using the predefined  handle

for the  standard  input  (0000H),  if  it  has  not  been

redirected, or using a  handle  obtained  by  opening  the

logical device CON.

7.1.12. Int 21H Function 0BH

Description:

Check Input Status

Checks whether a character is available  from  the  standard

input device. Input can be redirected.

Input          Output

------         ---------------------------------------------

AH=0BH         AL=00H if no character is available

=FFH if at least one character is available

- If a CTRL+C is detected, an Int 23H is executed.

- If the standard input is not redirected, and a  CTRL+C  is

detected at the console, an Int 23H is  executed.  If  the

standard input is redirected, a CTRL+C is detected at  the

console, and BREAK is ON, an Int 23H is executed.

- If a character is waiting, this function will continue  to

return a true flag until the character is consumed with  a

call to Int 21H Function 01H, 06H, 07H, 08H, 0AH, or  3FH.

- This function is equivalent to IOCTL Int 21H Function  44H

Subfunction 06H.

7.1.13. Int 21H Function 0CH

Description:

Flush Input Buffer and Then Input

Clears the standard input buffer and then invokes one of the

character input functions. Input can be redirected.

Interrupt 21H, Character-Oriented Functions

Input                                     Output

------------------------------            ---------------------

AH=0CH                                    If called with AL=01H

AL=number of input function to             07H, or 08H

be invoked after resetting             AL=8-bit input data

buffer (must be 01H, 06H,

07H, 08H, or 0AH)                      If called with AL=0AH

If AL=0AH                                 Nothing

DS:DX=segment:offset of input

buffer

- The function exists to allow a program to defeat  MS-DOS's

type-ahead feature. It discards any  characters  that  are

waiting in MS-DOS's internal  type-ahead  buffer,  forcing

the specified input  function  to  wait  for  a  character

(usually a keyboard entry) that is truly entered after the

program's request.

- The  presence  or  absence  of  CTRL+C   checking   during

execution of this function depends on the function  number

in register AL.

- A function number in AL other than 01H, 06H, 07H, 08H,  or

0AH simply flushes the input buffer and returns control to

the calling program.

7.2. Interrupt 21H, Directory-Control Functions

Hex #  Dec #     Name

39H     57       Create Directory

3AH     58       Delete Directory

3BH     59       Set Current Directory

47H     71       Get Current Directory

7.2.1. Int 21H Function 39H

Description:

Create Directory

Creates a directory using the specified drive and path.

Input                                 Output

------------------------------        ------------------------

AH=39H                                If function successful

DS:DX=segment:offset of ASCIIZ        Cary flag=clear

pathname                        If function unsuccessful

Carry flag=set

AX=error code

- The function fails if:

- any element of the pathname does not exist.

Interrupt 21H, Directory-Control Functions

- a directory with the same name at the end of the same

path already exists.

- the parent directory for the  new  directory  is  the

root directory and is full.

- [3.0+] the program is running on a  network  and  the

user running the program has insufficient access rights.

7.2.2. Int 21H Function 3AH

Description:

Delete Directory

Removes a directory using the specified drive and path.

Input                                 Output

------------------------------        ------------------------

AH=3AH                                If function successful

DS:DX=segment:offset of ASCIIZ        Carry flag=clear

pathname                        If function unsuccessful

Carry flag= set

AX=error code

- The function fails if:

- any element of the pathname does not exist.

-  the  specified  directory  is   also   the   current

directory.

- the specified directory contains any files.

- [3.0+] the program is running on a  network  and  the

user running the program has insufficient access rights.

7.2.3. Int 21H Function 3BH

Description:

Set Current Directory

Sets the current, or default, directory using the  specified

drive and path.

Input                                 Output

------------------------------        ------------------------

AH=3BH                                If function successful

DS:DX=segment:offset of ASCIIZ        Carry flag=clear

pathname                        If function unsuccessful

Carry flag= set

AX=error code

- The function fails if any element of the pathname does not

exist.

- Int 21H Function 47H can be used to obtain the name of the

Interrupt 21H, Directory-Control Functions

current directory before using Int  21H  Function  3BH  to

select another so  that  the  original  directory  can  be

restored later.

7.2.4. Int 21H Function 47H

Description:

Get Current Directory

Obtains an ASCIIZ string that describes the  path  from  the

root to the current directory, and the name of that  directory.

Input                             Output

-----------------------------     ----------------------------

AH=47H                            If function successful

DL=drive code (0=default,1=A,     Carry flag=clear

etc.)                         and buffer is filled in with

DS:SI=segment:offset of 64-        full pathname from root of

byte buffer                 current directory

If function unsuccessful

Carry flag=set

AX=error code

- The  returned  path  name  does  not  include  the   drive

identifier or a leading backslash (\).  It  is  terminated

with a null  (00H)  byte.  Consequently,  if  the  current

directory is the root directory, the  first  byte  in  the

buffer will contain 00H.

- The function fails if the drive code is invalid.

- The current directory may be set  with  Int  21H  Function

3BH.

7.3. Interrupt 21H, Disk-Management Functions

Hex #  Dec #     Name

0DH     13       Disk Reset

0EH     14       Select Disk

19H     25       Get Current Disk

1BH     27       Get Default Drive Data

1CH     28       Get Drive Data

2EH     46       Set Verify Flag

36H     54       Get Drive Allocation Information

54H     84       Get Verify Flag

Interrupt 21H, Disk-Management Functions

7.3.1. Int 21H Function 0DH

Description:

Disk Reset

Flushes all file buffers. All data that has  been  logically

written by user programs  but  has  been  temporarily  buffered

within MS-DOS is physically written to the disk.

Input             Output

------            -------

AH=0DH            Nothing

- This function does not update the disk directory  for  any

files that are  still  open.  If  your  program  fails  to

properly close all files before the disk is  removed,  and

files have changed size, the data forced out to  the  disk

by this function may still  be  inaccessible  because  the

directory entries will not be correct.

- [3.3+] Int 21H Function 68H (Commit File) should  be  used

in preference to this function because it also updates the

disk directory and file allocation table.

7.3.2. Int 21H Function 0EH

Description:

Select Disk

Selects the specified drive to be the current,  or  default,

disk drive and returns the total number of  logical  drives  in

the system.

Input                             Output

------------------------------    -----------------------------

AH=0EH                            AL=number of logical drives i

DL=drive code (0=A, 1=B, etc.)    system

- 63 drive designators (0 through 3FH) are available.

- [3.0+] 26 drive designators (0 through 19H) are available.

- To preserve upward compatibility, new applications  should

limit themselves to  the  drive  letters  A-Z  (0=A,  1=B,

etc.).

- Logical drives means the total number  of  block  devices:

floppy  disks,  simulated  disk  drives  (RAMdisks),   and

hard-disk drives. A single  physical  hard-disk  drive  is

frequently partitioned into two or more logical drives.

- In single-drive IBM PC-compatible systems, the value 2  is

returned in AL, because PC-DOS supports two logical drives

(A: and B:) on the single physical floppy-disk drive.  The

actual number of physical drives  in  the  system  can  be

determined with ROM BIOS Int 11H.

- [3.0+] The value returned in AL is either 5 or  the  drive

code corresponding to the  LASTDRIVE  entry  (if  any)  in

Interrupt 21H, Disk-Management Functions

CONFIG.SYS, whichever is greater.

7.3.3. Int 21H Function 19H

Description:

Get Current Disk

Returns the drive code of  the  current,  or  default,  disk

drive.

Input             Output

------            ------------------------------

AH=19H            AL=drive code (0=A, 1=B, etc.)

- To set the default drive, use Int 21H Function 0EH.

- Some other Int 21H functions use drive codes beginning  at

1 (that is, 1=A, 2=B, etc.) and reserve  drive  code  zero

for the default drive.

7.3.4. Int 21H Function 1BH

Description:

Get Default Drive Data

Obtains selected information about the  default  disk  drive

and a pointer to the media identification byte  from  its  file

allocation table.

Input        Output

------       -------------------------------------------------

AH=1BH       If function successful

AL=sectors per cluster

DS:BX=segment:offset of media ID byte

CX=size of physical sector (bytes)

DX=number of clusters for default drive

If function unsuccessful

(invalid drive or critical error)

AL=FFH

- The media ID byte has the following meanings:

0F0H

3.5-inch double-sided, 18 sectors or "other"

0F8H

fixed disk

0F9H

5.25-inch double-sided, 15 sectors or 3.5-inch

double-sided, 9 sectors

0FCH

5.25-inch single-sided, 9 sectors

0FDH

Interrupt 21H, Disk-Management Functions

5.25-inch double-sided, 9 sectors

0FEH

5.25-inch single-sided, 8 sectors

0FFH

5.25-inch double-sided, 8 sectors

- To obtain information about disks other than  the  one  in

the default drive, use Int 21H Function 1CH or 36H.

- The address returned in DS:BX points only to a copy of the

media ID byte from the disk's FAT; the memory  above  that

address cannot be assumed to contain the FAT or any  other

useful  information.  If  direct  access  to  the  FAT  is

required, use Int 25H to read it into memory.

7.3.5. Int 21H Function 1CH

Description:

Get Drive Data

Obtains allocation  information  about  the  specified  disk

drive and a pointer to the media identification byte  from  its

file allocation table.

Input              Output

--------------     --------------------------------------------

AH=1CH             If function successful

DL=drive code      AL=sectors per cluster

(0=default,     DS:BX=segment:offset of media ID byte

1=A, etc.)      CX=size of physical sector (bytes)

DX=number of clusters for default or

specified drive

If function unsuccessful (invalid drive or

critical error)

AL=FFH

- The media ID byte has the following meanings:

0F0H

3.5-inch double-sided, 18 sectors or "other"

0F8H

fixed disk

0F9H

5.25-inch double-sided, 15 sectors or 3.5-inch

double-sided, 9 sectors

0FCH

5.25-inch single-sided, 9 sectors

0FDH

5.25-inch double-sided, 9 sectors

0FEH

5.25-inch single-sided, 8 sectors

0FFH

5.25-inch double-sided, 8 sectors

- In general, this call is identical  to  Int  21H  Function

1BH, except for the ability to designate a  specific  disk

drive. See  also  Int  21H  Function  36H,  which  returns

similar information.

- The address returned in DS:BX points only to a copy of the

media ID byte from the disk's FAT; the memory  above  that

Interrupt 21H, Disk-Management Functions

address cannot be assumed to contain the FAT or any  other

useful  information.  If  direct  access  to  the  FAT  is

required, use Int 25H to read it into memory.

7.3.6. Int 21H Function 2EH

Description:

Set Verify Flag

Turns  off  or  turns  on  the  operating-system  flag   for

automatic read-after-write verification of data.

Input                                         Output

----------------------------------            -------

AH=2EH                                        Nothing

AL=00H  if turning off verify flag

01H  if turning on verify flag

DL=00H

- This  function  provides  increased  data   integrity   by

allowing the user to force a  read-after-write  verify  of

all data written  to  the  disk,  if  that  capability  is

supported by the manufacturer's disk driver.

- Because   read-after-write   verification    slows    disk

operations, the default setting of the verify flag is OFF.

- The current state of the verify  flag  can  be  determined

using Int 21H Function 54H.

- The state of the verify flag is  also  controlled  by  the

MS-DOS commands VERIFY OFF and VERIFY ON.

7.3.7. Int 21H Function 36H

Description:

Get Drive Allocation Information

Obtains selected information about a disk drive, from  which

the  drive's  capacity  and  remaining  free   space   can   be

calculated.

Input                          Output

-------------------------      -------------------------------

AH=36H                         If function successful

DL=drive code (0=default,      AX=sectors per cluster

1=A, etc.)                 BX=number of available clusters

CX=bytes per sector

DX=clusters per drive

If function unsuccessful

(drive invalid)

AX=FFFFH

Interrupt 21H, Disk-Management Functions

- This function regards "lost" clusters as being in use  and

does not report them as part of the  number  of  available

clusters, even though they are not assigned to a file.

- Similar information is returned by Int 21H  Functions  1BH

and 1CH.

7.3.8. Int 21H Function 54H

Description:

Get Verify Flag

Obtains   the   current   value   of   the   system   verify

(read-after-write) flag.

Input             Output

------            ----------------------------

AH=54H            AL=current verify flag value

00H  if verify off

01H  if verify on

- Because   read-after-write   verification    slows    disk

operations, the default state of the system verify flag is

OFF.

- The state of the system verify flag can be changed through

a call to Int 21H Function 2EH or by the  MS-DOS  commands

VERIFY ON and VERIFY OFF.

7.4. Interrupt 21H, File Operations

Hex #  Dec #     Name

3CH     60       Create File

3DH     61       Open File

3EH     62       Close File

41H     65       Delete File

43H     67       Get or Set File Attributes

45H     69       Duplicate Handle

46H     70       Redirect Handle

4EH     78       Find First File

4FH     79       Find Next File

56H     86       Rename File

57H     87       Get or Set File Date and Time

5AH     90       Create Temporary File

5BH     91       Create New File

67H    103       Set Handle Count (sets maximum number of files

Interrupt 21H, File Operations

7.4.1. Int 21H Function 3CH

Description:

Create File

Given  an  ASCIIZ  pathname,  creates  a  new  file  in  the

designated or default directory on the  designated  or  default

disk drive.  If  the  specified  file  already  exists,  it  is

truncated to zero length. In either case, the  file  is  opened

and a handle is returned that can be used by  the  program  for

subsequent access to the file.

Input                                   Output

----------------------------------      ----------------------

AH=3CH                                  If function successful

CX=file attribute                       Carry flag=clear

(bits can be combined)               AX=handle

Bit(s)  Significance (if set)

0       read-only

1       hidden                        If function failed

2       system                        Carry flag=set

3       volume label                  AX=error code

4       reserved (0)

5       archive

6-15    reserved (0)

DS:DX=segment:offset of ASCIIZ pathname

7.4.2. CreateFile

;* CreateFile - Creates file with specified attribute.

;*

;* Shows:   DOS Function - 3Ch (Create File)

;*

;* Params:

;* attr - Attribute code:  0 = normal        8 = volume label

;*                         1 = read only    16 = subdirectory

;*                         2 = hidden       32 = archive

;*                         4 = system

;* fspec - Pointer to ASCIIZ file specification

;*

;* Return:  Short integer with file handle or -1 for error

CreateFile PROC \

USES ds, \

attr:WORD, fspec:PTR BYTE

LoadPtr ds, dx, fspec  ; Point DS:DX to file spec

mov     cx, attr       ; CX = attribute

mov     ah, 3Ch        ; AH = function number

int     21h            ; Create File

jnc     exit           ; If ok, return AX = handle

mov     ax, -1         ; Else set error code

exit:   ret

CreateFile ENDP

Interrupt 21H, File Operations

7.4.3. Int 21H Function 3DH

Description:

Open File

Given an ASCIIZ pathname, opens the specified  file  in  the

designated or default directory on the  designated  or  default

disk drive. A handle is  returned  that  can  be  used  by  the

program for subsequent access to the file.

Input                                Output

------------------------------       --------------------------

AH=3DH                               If function successful

AL=access mode                         Carry flag=clear

Bit(s)   Significance                AX=handle

0-2      access mode               If function unsuccessful

000=read access             Carry flag=set

001=write access            AX=error code

010=read/write access

3        reserved (0)

4-6      sharing mode

(MS-DOS versions

3.0 and later)

000=compatibility mode

001=deny all

010=deny write

011=deny read

100=deny none

7        inheritance flag

(MS-DOS versions

3.0 and later)

0=child process inherits handle

1=child does not inherit handle

DS:DX=segment:offset of ASCIIZ pathname

7.4.4. Int 21H Function 3EH

Description:

Close File

Given a handle that was obtained by  a  previous  successful

open  or  create  operation,  flushes  all   internal   buffers

associated with the file to disk, closes the file, and releases

the handle for reuse. If the file was modified,  the  time  and

date stamp and file size are updated in  the  file's  directory

entry.

Interrupt 21H, File Operations

Input                Output

---------            ------------------------

AH=3EH               If function successful

BX=handle              Carry Flag=clear

If function unsuccessful

Carry flag= set

AX=error code

- If you accidentally call this function with a zero handle,

the standard input device  is  closed,  and  the  keyboard

appears to go dead. Be sure  you  always  call  the  close

function with a valid, non-zero handle.

7.4.5. Int 21H Function 41H

Description:

Delete File

Deletes a file  from  the  specified  or  default  disk  and

directory.

Input                             Output

------------------------------    ------------------------

AH=41H                            If function successful

DS:DX=segment:offset of ASCIIZ       Carry flag=clear

pathname                    If function unsuccessful

Carry flag= set

AX=error code

1) This function deletes a  file  by  replacing  the  first

character of its filename  in  the  directory  with  the

character e (E5H) and marking  the  file's  clusters  as

"free" in the disk's file allocation table.  The  actual

data stored in those clusters is not overwritten.

2) Only one file  at  a  time  may  be  deleted  with  this

function. Unlike the FCB-related  Delete  File  function

(Int 21H Function 13H), the * and ? wildcard  characters

are not allowed in the file specification.

3) The function fails if:

- any element of the pathname does not exist.

- the designated file exists but  has  the  read-only

attribute (Int 21H Function 43H can be used  to  examine

and modify  a  file's  attribute  before  attempting  to

delete it).

- [3.0+] the program is running on a network, and the

user running the program has insufficient access rights.

Interrupt 21H, File Operations

7.4.6. Int 21H Function 43H

Description:

Get or Set File Attributes

Obtains or alters  the  attributes  of  a  file  (read-only,

hidden, system, or archive).

Input                                Output

----------------------------------   --------------------------

AH=43H                               If function successful

AL=00H if getting file attribute        Carry flag=clear

01H if setting file attribute        CX=file attribute (see

CX=new file attribute, if AL = 01H         above)

(bits can be combined)            If function unsuccessful

bit 5=archive                      Carry flag=set

bit 2=system                       AX=error code

bit 1=hidden

bit 0=read-only

DS:DX=segment:offset of ASCIIZ

pathname

- Bits 3 and 4 of register CX must always be clear (0)  when

this function is called; in other words, you cannot change

an  existing  file  into  a  directory  or  volume  label.

However, you can  assign  the  "hidden"  attribute  to  an

existing directory with this function.

- [3.0+] If the program is running on a  network,  the  user

must have Create access rights to the directory containing

the file whose attribute is to be modified.

7.4.7. Int 21H Function 45H

Description:

Duplicate Handle

Given a handle for a currently open device or file,  returns

a new handle that refers to the same device or file at the same

position.

Input                           Output

--------------------------      ------------------------

AH=45H                          If function successful

BX=handle to be duplicated         Carry flag=clear

AX=new handle

If function unsuccessful

Carry flag=set

AX=error code

- A seek, read, or  write  operation  that  moves  the  file

pointer for one of the two handles  also  moves  the  file

pointer associated with the other.

- This function  can  be  used  to  efficiently  update  the

Interrupt 21H, File Operations

directory for a file that has changed in  length,  without

incurring the overhead of closing and then  reopening  the

file. The handle for the file is  simply  duplicated  with

this function and the duplicate  is  closed,  leaving  the

original handle open for further read/write operations.

- [3.3+] See also Int 21H Function 68H (Commit File).

7.4.8. Int 21H Function 45H

Description:

Duplicate Handle

Given a handle for a currently open device or file,  returns

a new handle that refers to the same device or file at the same

position.

Input                                 Output

--------------------------            ------------------------

AH=45H                                If function successful

BX=handle to be duplicated              Carry flag=clear

AX=new handle

If function unsuccessful

Carry flag=set

AX=error code

- A seek, read, or  write  operation  that  moves  the  file

pointer for one of the two handles  also  moves  the  file

pointer associated with the other.

- This function  can  be  used  to  efficiently  update  the

directory for a file that has changed in  length,  without

incurring the overhead of closing and then  reopening  the

file. The handle for the file is  simply  duplicated  with

this function and the duplicate  is  closed,  leaving  the

original handle open for further read/write operations.

- [3.3+] See also Int 21H Function 68H (Commit File).

7.4.9. Int 21H Function 46H

Description:

Redirect Handle

Given two handles, makes the second handle refer to the same

device or file at the same location as the  first  handle.  The

second handle is then said to be redirected.

Interrupt 21H, File Operations

Input                            Output

----------------------------     -----------------------------

AH=46H                           If function successful

BX=handle for file or device        Carry flag=clear

CX=handle to be redirected       If function unsuccessful

Carry flag=set

AX=error code

- If the handle passed in CX already refers to an open file,

that file is closed first.

- A seek, read, or  write  operation  that  moves  the  file

pointer for one of the two handles  also  moves  the  file

pointer associated with the other.

- This function is commonly used to  redirect  the  standard

input and output handles to another file or device  before

a child process is executed with Int 21H Function 4BH.

7.4.10. Int 21H Function 4EH

Description:

Find First File

Given a file specification in the form of an ASCIIZ  string,

searches the default or specified directory on the  default  or

specified drive for the first matching file.

Interrupt 21H, File Operations

Input                                     Output

------------------------------            ---------------------

AH=4EH                                    If function successfu

CX=attribute to use in search              ing file found)

(bits can be combined)                   Carry flag=clear

Bit(s)   Significance (if set)            and current disk tran

0        read-only                         filled in as follo

1        hidden                           Byte(s)   Descripti

2        system                           00H-14H   reserved

3        volume label                     15H       attribute

4        directory                                   file or

5        archive                          16H-17H   file time

6-15     reserved (0)                               bits 00H-

DS:DX=segment:offset of ASCIIZ                       ond incr

pathname                                            (0-29)

bits 05H-

(0-59)

bits 0BH-

(0-23)

18H-19H   file date

bits 00H-

(1-31)

bits 05H-

(1-12)

bits 09H-

(relativ

1AH-1DH   file size

1EH-2AH   ASCIIZ fi

extensio

If function unsucce

matching files)

Carry flag=set

AX=error code

AX=error code

7.4.11. Int 21H Function 4FH

Description:

Find Next File

Assuming a previous successful call to Int 21H Function 4EH,

finds the next file in the default or  specified  directory  on

the default or specified drive that matches the  original  file

specification.

Interrupt 21H, File Operations

Input                           Output

--------------------            -------------------------------

AH=4FH                          If function successful (matchin

Assumes DTA points               found)

to working buffer               Carry flag=clear

used by previous                and current disk transfer area

successful Int 21H               as follows:

Function 4EH or 4FH             Byte(s)   Description

00H-14H   reserved (0)

15H       attribute of matche

directory

16H-17H   file time

bits 00H-04H=2-seco

ments (0-29)

bits 05H-0AH=minute

bits 0BH-0FH=hours

18H-19H   file date

bits 00H-04H=day (1

bits 05H-08H=month

bits 09H-0FH=year (

to 1980)

1AH-1DH   file size

1EH-2AH   ASCIIZ filename and

If function unsuccessful (no

matching files)

Carry flag= set

AX=error code

- Use  of  this  call  assumes  that   the   original   file

specification passed to Int 21H Function 4EH contained one

or more * or ? wildcard characters.

- When this function is called, the  current  disk  transfer

area  (DTA)  must  contain  information  from  a  previous

successful call to Int 21H Function 4EH or 4FH.

7.4.12. FindNext

;* FindNext - Finds next entry in given directory

;* matching specification.

;* (Should be called only after successfully

;* calling the FindFirst procedure.)

;*

;* Shows:   DOS Function - 4Fh (Find Next File)

;*          Operator - OFFSET

;*

;* Params:  finfo - Pointer to 43-byte buffer.

;*                This must be the same buffer (or a duplicate)

;*                returned from the FindFirst procedure.

;*

;* Return:  Short integer with error code

;*          0 if successful

;*          1 if no more matches found

FindNext PROC \

USES ds, \

finfo:PTR BYTE

Interrupt 21H, File Operations

push    ds                      ; Pass far pointer

mov     ax, OFFSET @data:old_dta;   to old_dta

push    ax

call    GetDTA     ; Get current DTA address

add     sp, 4      ; Adjust stack

LoadPtr ds, dx, finfo ; DS:DX points to 43-byte buffer

push    ds            ; Make this new DTA

push    dx

call    SetDTA        ; Set 43-byte buffer as DTA

add     sp, 4         ; Adjust stack

mov     ah, 4Fh       ; AH = function number

int     21h           ; Find Next File

pushf                 ; Preserve flags

push    WORD PTR @data:old_dta[2] ; Pass far pointer to

push    WORD PTR @data:old_dta[0] ;   SetDTA procedure

call    SetDTA        ; Restore DTA address to orig

sub     ax, ax        ; Set error code

add     sp, 4         ; Adjust stack

popf                  ; Recover flags

jnc     exit          ; Exit if successful match

inc     ax            ; Else set error code to 1

exit:   ret

FindNext ENDP

7.4.13. Int 21H Function 56H

Description:

Rename File

Renames a  file  and/or  moves  its  directory  entry  to  a

different directory on the same disk. In  MS-DOS  versions  3.0

and  later,  this  function  can  also  be  used  to  rename  a

directory.

Input                               Output

-------------------------------     --------------------------

AH=56H                              If function successful

DS:DX=segment:offset of current       Carry flag=clear

ASCIIZ pathname                    If function unsuccessful

ES:DI=segment:offset of new           Carry flag=set

ASCIIZ pathname                      AX=error code

1) The function fails if:

- any element of the pathname does not exist.

- a file with the new pathname already exists.

-  the  current  pathname  specification  contains  a

different disk drive than does the new pathname.

- the file is being moved to the root  directory  and

the root directory is full.

- [3.0+] the program is running on a network and  the

user  has  insufficient  access  rights  to  either  the

existing file or the new directory.

2) The * and ?  wildcard  characters  are  not  allowed  in

Interrupt 21H, File Operations

either the current or new pathname specifications.

7.4.14. Int 21H Function 57H

Description:

Get or Set File Date and Time

Obtains or modifies the date and  time  stamp  in  a  file's

directory entry.

Input                                  Output

---------------------------------      ----------------------

If getting date and time               If function successful

AH=57H                                   Carry flag=clear

AL=00H                                   and, if called with

BX=handle                                AL=00H

If setting date and time                 CX=time

AH=57H                                   DX=date

AL=01H                                 If function unsuccessful

BX=handle                                Carry flag=set

CX=time                                  AX=error code

bits 0BH-0FH=hours (0-23)

bits 05H-0AH=minutes (0-59)

bits 00H-04H=2-second increments (0-29)

DX=date

bits 09H-0FH=year (relative to 1980)

bits 05H-08H=month (1-12)

bits 00H-04H=day (1-31)

- The file must have been previously opened or created via a

successful call to Int 21H Function 3CH, 3DH, 5AH, 5BH, or

6CH.

- If the 16-bit date for a file is set to zero, that  file's

date and time are not displayed on directory listings.

- A date and time set with this function will prevail,  even

if the file is modified afterwards before  the  handle  is

closed.

7.4.15. Int 21H Function 5AH

Description:

Create Temporary File DOS 3 Only

Creates a file  with  a  unique  name,  in  the  current  or

specified directory on the default or specified disk drive, and

returns a handle that can be used by the program for subsequent

access to the file. The name generated for  the  file  is  also

returned in a buffer specified by the program.

Interrupt 21H, File Operations

Input                                 Output

-----------------------------------   -------------------------

AH=5AH                                If function successful

CX=attribute (bits can be combined)     Carry flag=clear

Bit(s)  Significance (if set)          AX=handle

0       read-only                     DS:DX=segment:offset of

1       hidden                        complete ASCIIZ pathnam

2       system                      If function unsuccessful

3-4     reserved (0)                  Carry flag=set

5       archive                       AX=error code

6-15    reserved (0)

DS:DX=segment:offset of ASCIIZ path

1) The ASCIIZ path supplied  to  this  function  should  be

followed by at  least  13  additional  bytes  of  buffer

space. MS-DOS adds a backslash (\) to the supplied path,

if necessary, then appends  a  null-terminated  filename

that is a function of the current time.

2) Files created with this function are  not  automatically

deleted when the calling program terminates.

3) The function fails if

- any element of the pathname does not exist.

- the file is being created in the root directory and

the root directory is full.

4) [3.0+] If the program is running on a network, the  file

is  created  and  opened  for   read/write   access   in

compatibility sharing mode.

5) See also Int 21H Functions  3CH,  5BH,  and  6CH,  which

provide additional facilities for creating files.

7.4.16. Int 21H Function 5BH

Description:

Create New File DOS 3 Only

Given an ASCIIZ pathname, creates a file in  the  designated

or default directory on the designated  or  default  drive  and

returns a handle that can be used by the program for subsequent

access to the file. If  a  file  with  the  same  name  already

exists, the function fails.

Input                                Output

-----------------------------------  -------------------------

AH=5BH                               AH=5BH

CX=attribute (bits can be combined)  If function sucessful

Bit(s)   Significance (if set)        Carry flag=clear

0        read-only                  AX=handle

1        hidden                  If function unsucessful

2        system                     Carry flag=set

3        volume label               AX=error code

4        reserved (0)

5        archive

6-15     reserved (0)

DS:DX=segment:offset of ASCIIZ pathname

Interrupt 21H, File Operations

1) The function fails if:

- any element of the specified path does not exist.

- a file with the identical pathname (i.e., the  same

filename and extension  in  the  same  location  in  the

directory structure) already exists.

- the file is being created in the root directory and

the root directory is full.

- [3.0+] the program is running on a network and  the

user has insufficient access  rights  to  the  directory

that will contain the file.

2) The file is usually given the normal attribute (0)  when

it is created, and is opened for  both  read  and  write

operations. The attribute can subsequently  be  modified

with Int 21H Function 43H.

3) See also Int 21H Functions  3CH,  5AH,  and  6CH,  which

provide alternative ways of creating files.

4) This function may be used to implement semaphores  in  a

network or multitasking  environment.  If  the  function

succeeds, the program has  acquired  the  semaphore.  To

release the semaphore, the program  simply  deletes  the

file.

7.4.17. CreateNewFile

;* CreateNewFile - Creates a new file with specified attribute.

;* Differs from the CreateFile procedure in that it returns an

;* error if file already exists. For DOS versions 3.0 and highe

;*

;* Shows:   DOS Function - 5Bh (Create New File)

;*

;* Params:  attr - Attribute code

;*          (see header comments for CreateFile)

;*          fspec - Pointer to ASCIIZ file specification

;*

;* Return:  Short integer with file handle or -1 for error

CreateNewFile PROC \

USES ds, \

attr:WORD, fspec:PTR BYTE

LoadPtr ds, dx, fspec       ; Point DS:DX to file spec

mov     cx, attr            ; CX = attribute

mov     ah, 5Bh             ; AH = function number

int     21h                 ; Create New File

jnc     exit                ; Return AX = handle if ok

mov     ax, -1              ; Else set error code

exit:   ret

CreateNewFile ENDP

Interrupt 21H, File Operations

7.4.18. Int 21H Function 67H

Description:

Set Handle Count DOS 3.3 Only

Sets the maximum number of files and  devices  that  may  be

opened simultaneously using handles by the current process.

Input                                 Output

------------------------------        ------------------------

AH=67H                                If function successful

BX=number of desired handles          Carry flag=clear

If function unsuccessful

Carry flag=set

AX=error code

- This function call controls the size  of  the  table  that

relates handle numbers for the current process to MS-DOS's

internal, global table for  all  of  the  open  files  and

devices in the system. The default table is located in the

reserved area of the process's PSP and is large enough for

20 handles.

- The function fails if the requested number of  handles  is

greater than 20 and there is not sufficient free memory in

the system to allocate a new block to  hold  the  enlarged

table.

- If the number of handles  requested  is  larger  than  the

available entries in the system's global  table  for  file

and device handles  (controlled  by  the  FILES  entry  in

CONFIG.SYS), no error is returned. However,  a  subsequent

attempt to open a file or device, or create  a  new  file,

will fail if all of the entries  in  the  system's  global

file table are in use, even if the requesting process  has

not used up all of its own handles.

7.5. Interrupt 21H, File Operations, FCB

You can perform all the functions  below  more  easily  with

DOS2 functions. The services below are documented  so  you  can

analyze code written for DOS1.

Hex #  Dec #     Name

0FH     15       Open File

10H     16       Close File

11H     17       Find First File

12H     18       Find Next File

13H     19       Delete File

16H     22       Create File

17H     23       Rename File

23H     35       Get File Size

29H     41       Parse Filename

Interrupt 21H, File Operations, FCB

7.5.1. Int 21H Function 0FH

Description:

Open File

Opens  a  file  and  makes  it  available   for   subsequent

read/write operations.

Input                  Output

---------------------  ---------------------------------------

AH=0FH                 If function successful (file found)

DS:DX=segment:offset    AL=00

of file control   and FCB filled in by MS-DOS as follows:

block             drive field (offset 00H0=1 for drive A,

2 for drive B, etc.

current block field (offset 0CH)=00H

record size field (offset 0EH)=0080H

size field (offset 10H)=file size from

directory

date field (offset 14H)=date stamp from

directory

time field (offset 16H)=time stamp from

directory

If function unsuccessful (file not found

AL=0FFH

- If your program is going to use a record size  other  than

128 bytes, it should set the  record  size  field  at  FCB

offset 0EH after  the  file  is  successfully  opened  and

before any other disk operation.

- If random access is to be performed, the  calling  program

must also set the FCB relative-record field  (offset  21H)

after successfully opening the file.

- For format  of  directory  time  and  date,  see  Int  21H

Function 57H.

- Int 21H Function 3DH, which  allows  full  access  to  the

hierarchical  directory  structure,  should  be  used   in

preference to this function.

- [3.0+] If the program is running on a network, the file is

opened for  read/write  access  in  compatibility  sharing

mode.

7.5.2. Int 21H Function 10H

Description:

Close File

Closes a file, flushes  all  MS-DOS  internal  disk  buffers

associated  with  the  file  to  disk,  and  updates  the  disk

directory if the file has been modified or extended.

Interrupt 21H, File Operations, FCB

Input                          Output

----------------------------   --------------------------------

AH=10H                         If function successful (director

DS:DX=segment:offset of file      update successful)

control block              AL=00H

If function unsuccessful (file

not found in directory)

AL=FFH

- MS-DOS versions 1.x and 2.x do not reliably detect a  disk

change, and an error can occur if the user  changes  disks

while a file is still open on that  drive.  In  the  worst

case, the directory and file allocation table of the newly

inserted disk may be damaged or destroyed.

- Int 21H Function 3EH should be used in preference to  this

function.

7.5.3. Int 21H Function 11H

Description:

Find First File

Searches the current directory on the designated drive for a

matching filename.

Input                      Output

------------------------   --------------------------------

AH=11H                     If function successful (matching

DS:DX=segment:offset of       filename found)

file control block     AL=00H

and buffer at current disk trans-

fer area (DTA) address filled

in as an unopened normal FCB

or extended FCB, depending on

which type of FCB was input to

function

If function unsuccessful (no

matching filename found)

AL=FFH

- Use Int 21H Function 1AH to set the  DTA  to  point  to  a

buffer of adequate size before using this function call.

- The wildcard character ? is allowed in the filename in all

versions  of  MS-DOS.  In  versions  3.0  and  later,  the

wildcard character * may also be used in a filename. If  ?

or * is used, this function  returns  the  first  matching

filename.

- An extended FCB must be used to search for files that have

the system, hidden, read-only, directory, or  volume-label

attributes.

- If an extended FCB is used, its attribute byte  determines

the  type  of  search  that  will  be  performed.  If  the

attribute byte (byte 0) contains 00H, only ordinary  files

are found. If the volume-label attribute bit is set,  only

Interrupt 21H, File Operations, FCB

volume labels will be returned (if any  are  present).  If

any other attribute or combination of  attributes  is  set

(such as hidden, system, or read-only),  those  files  and

all ordinary files will be matched.

- Int 21H Function 4EH, which  allows  full  access  to  the

hierarchical  directory  structure,  should  be  used   in

preference to this function.

7.5.4. Int 21H Function 12H

Description:

Find Next File

Given that a previous call to Int 21H Function 11H has  been

successful, returns the next matching filename (if any).

Input                       Output

------------------------    --------------------------------

AH=12H                      If function successful (matching

DS:DX=segment:offset of        filename found)

file control block       AL=00H

and buffer at current disk trans-

fer area (DTA) address set up

as an unopened normal FCB or

extended FCB, depending on

which type of FCB was originally

input to Int 21H Function 11H

If function unsuccessful (no more

matching filenames found)

AL=FFH

- This function assumes that the FCB used as input has  been

properly  initialized  by  a  previous  call  to  Int  21H

Function 11H (and possible subsequent  calls  to  Int  21H

Function 12H) and that the  filename  or  extension  being

searched for contained at least one wildcard character.

- As with Int 21H Function 11H, it is important to  use  Int

21H Function 1AH to set the DTA to a  buffer  of  adequate

size before using this function.

- Int 21H Functions 4EH and 4FH, which allow full access  to

the hierarchical directory structure, should  be  used  in

preference to this function.

7.5.5. Int 21H Function 13H

Description:

Delete File

Deletes all matching files from the current directory on the

default or specified disk drive.

Interrupt 21H, File Operations, FCB

Input                        Output

--------------------------   --------------------------------

AH=13H                       If function successful (file or

DS:DX=segment:offset of                        files deleted)

file control block      AL=00H

If function unsuccessful (no

matching files were found, or

at least one matching file was

read-only)

AL=FFH

- The wildcard character ? is allowed in the filename; if  ?

is present and there is more than one  matching  filename,

all matching files will be deleted.

- Int 21H Function 41H, which  allows  full  access  to  the

hierarchical  directory  structure,  should  be  used   in

preference to this function.

- [3.0+] If the program is running on a  network,  the  user

must have Create rights to the  directory  containing  the

file to be deleted.

7.5.6. Int 21H Function 16H

Description:

Create File

Creates a new directory entry in the  current  directory  or

truncates any existing file with the same name to zero  length.

Opens the file for subsequent read/write operations.

Input                 Output

--------------------  ----------------------------------------

AH=16H                If function successful (file was created

DS:DX=segment:offset     or truncated)

of unopened file    AL=00H

control block       and FCB filled in by MS-DOS as follows:

drive field (offset 00H)=1 for drive A,

2 for drive B, etc.

current block field (offset 0CH)=00H

record size field (offset 0EH)=0080H

size field (offset 10H)=file size

from directory

date field (offset 14H)=date stamp

from directory

time field (offset 16H)=time stamp

from directory

If function unsuccessful (directory full)

AL=FFH

Interrupt 21H, File Operations, FCB

7.5.7. Int 21H Function 17H

Description:

Rename File

Alters the  name  of  all  matching  files  in  the  current

directory on the disk in the specified drive.

Input                   Output

---------------------   -------------------------------------

AH=17H                  If function successful (one or more

DS:DX=segment:offset       files renamed)

of "special" file     AL=00H

control block       If function unsuccessful (no matching

files, or new filename matched an

existing file)

AL=FFH

- The special file control block has a drive code, filename,

and extension in the usual position (bytes 0 through  0BH)

and a second filename starting 6  bytes  after  the  first

(offset 11H).

- The  ?  wildcard  character  can  be  used  in  the  first

filename. Every file matching the first file specification

will be renamed to match the second file specification.

- If the second file specification contains any  ?  wildcard

characters,  the  corresponding  letters  in   the   first

filename are left unchanged.

- The function terminates if the new name to be assigned  to

a file matches that of an existing file.

- An extended FCB can be used with this function to rename a

directory.

- Int 21H Function 56H, which  allows  full  access  to  the

hierarchical  directory  structure,  should  be  used   in

preference to this function.

7.5.8. Int 21H Function 23H

Description:

Get File Size

Searches for a matching file in the  current  directory;  if

one is found, updates the FCB with the file's size in terms  of

number of records.

Interrupt 21H, File Operations, FCB

Input                    Output

----------------------   -------------------------------------

AH=23H                   If function successful (matching file

DS:DX=segment:offset                            found)

of unopened file       AL=00

control block        and FCB relative-record field (offse

21H) set to the number of records in

the file, rounded up if necessary to

the next complete record

If function unsuccessful (no matching

file found)

AL=FFH

- An appropriate value must be placed in the FCB record size

field (offset 0EH) before calling this function. There  is

no default record size for this function. Compare with the

FCB-related open and create functions (Int  21H  Functions

0FH and 16H), which  initialize  the  FCB  for  a  default

record size of 128 bytes.

- The record size field can be set to 1 to find the size  of

the file in bytes.

- Because record numbers are zero based, this  function  can

be used to position the FCB's file pointer to the  end  of

file.

7.5.9. Int 21H Function 29H

Description:

Parse Filename

Parses a text string into  the  various  fields  of  a  file

control block (FCB).

Interrupt 21H, File Operations, FCB

Input                                Output

-----------------------------------  --------------------------

AH=29H                               AL=00H  if no wildcard cha

AL=flags to control parsing                  acters encountered

Bit 3=1  if the extension field in

FCB will be modified only      01H  if parsed string

if an extension is speci-           contained wild-

fied in the string being            card characters

parsed.

=0  if the extension field in

FCB will be modified re-       FFH  if drive specifier

gardless; if no extension           invalid

is present in the parsed

string, FCB extension is    DS:SI=segment:offset of

set to ASCII blanks.              first character afte

Bit 2=1  if the filename field in          parsed filename

FCB will be modified only

if a filename is specified  ES:DI=segment:offset of

in the string being parsed.       formatted unopened

=0  if the filename field in          file control block

FCB will be modified regard-

less; if no filename is pre-

sent in the parsed string,

FCB filename is set to ASCII

blanks.

Bit 1=1  if the drive ID byte in FCB

will be modified only if a

drive was specified in the

string being parsed.

=0  if the drive ID byte in FCB

will be modified regardless;

if no drive specifier is pre-

sent in the parsed

string, FCB drive-code field

is set to 0 (default).

Bit 0=1  if the leading separators will

be scanned off (ignored).

=0  if the leading separators will

not be scanned off.

DS:SI=segment:offset of text string

ES:DI=segment:offset of file controlblock

7.6. Interrupt 21H, Memory-Allocation Functions

Hex #  Dec #     Name

48H     72       Allocate Memory Block

49H     73       Release Memory Block

4AH     74       Resize Memory Block

58H     88       Get or Set Allocation Strategy

Interrupt 21H, Memory-Allocation Functions

7.6.1. Int 21H Function 48H

Description:

Allocate Memory Block

Allocates a block of memory and returns  a  pointer  to  the

beginning of the allocated area.

Input                     Output

-----------------------   ----------------------------------

AH=48H                    If function successful

BX=number of paragraphs     Carry flag=clear

of memory needed         AX=initial segment of allocated

block

If function unsuccessful

Carry flag=set

AX=error code

BX=size of largest available block

(paragraphs)

- If the function succeeds, the base address  of  the  newly

allocated block is AX:0000.

- The default allocation strategy used by MS-DOS  is  "first

fit"; that is, the memory block at the lowest address that

is large enough to satisfy the request is  allocated.  The

allocation strategy can be altered with Int  21H  Function

58H.

- When a .COM  program  is  loaded,  it  ordinarily  already

"owns" all of the memory in the  transient  program  area,

leaving none for dynamic allocation. The amount of  memory

initially allocated to a .EXE program at load time depends

on the MINALLOC and  MAXALLOC  fields  in  the  .EXE  file

header. See Int 21H Function 4AH.

7.6.2. Int 21H Function 49H

Description:

Release Memory Block

Releases a memory block and makes it available  for  use  by

other programs.

Input                         Output

-------------------------     ------------------------

AH=49H                        If function successful

ES=segment of block to be       Carry flag=clear

released                   If function unsuccessful

Carry flag=set

AX=error code

1) This  function  assumes  that  the  memory  block  being

released was previously obtained by a successful call to

Int 21H Function 48H.

Interrupt 21H, Memory-Allocation Functions

2) The function will fail or can cause unpredictable system

errors if:

- the program releases a memory block that  does  not

belong to it.

- the segment address passed in register ES is not  a

valid base address for an existing memory block.

7.6.3. Int 21H Function 4AH

Description:

Resize Memory Block

Dynamically shrinks or extends a memory block, according  to

the needs of an application program.

Input                          Output

----------------------------   --------------------------------

AH=4AH                         If function successful

BX=desired new block size in     Carry flag=clear

paragraphs                  If function unsuccessful

ES=segment of block to be        Carry flag=set

modified                      AX=error code

BX=maximum block size availabl

(paragraphs)

- This function modifies the size of a memory block that was

previously allocated with a call to Int 21H Function  48H.

- If the program is requesting an increase in the size of an

allocated block, and  this  function  fails,  the  maximum

possible size for  the  specified  block  is  returned  in

register BX. The program can use this value  to  determine

whether it should terminate, or  continue  in  a  degraded

fashion with less memory.

- A program that uses EXEC (Int 21H Function  4BH)  to  load

and execute a child program must call this function first,

passing the address of its own PSP in register ES and  the

amount of memory needed for its own code, data, and stacks

in register BX.

7.6.4. Int 21H Function 58H

Description:

Get or Set Allocation Strategy DOS 3 Only

Obtains or changes the code indicating  the  current  MS-DOS

strategy for allocating memory blocks.

Interrupt 21H, Memory-Allocation Functions

Input                               Output

------------------------            ---------------------------

If getting strategy code            If function successful

AH=58H                              Carry flag=clear

AL=00H                              and, if called with AL=00H

If setting strategy code            AX=error code

AH=58H                              If function unsuccessful

AL=01H                              Carry flag=set

BX=desired strategy code            AX=error code

00H=first fit

01H=best fit

02H=last fit

- The memory allocation strategies are:

First Fit: MS-DOS searches the available memory  blocks

from low addresses to high addresses, assigning the  first

one large enough to satisfy the block allocation  request.

Best Fit: MS-DOS searches all available  memory  blocks

and assigns the smallest available block that will satisfy

the request, regardless of its position.

Last Fit: MS-DOS searches the available  memory  blocks

from  high  addresses  to  low  addresses,  assigning  the

highest one large enough to satisfy the  block  allocation

request.

- The default MS-DOS memory allocation strategy is First Fit

(code 0).

7.7.  Interrupt   21H,   Miscellaneous   System

Functions

Hex #  Dec #     Name

25H     37       Set Interrupt Vector

30H     48       Get MS-DOS Version Number

33H     51       Get or Set Break Flag

35H     53       Get Interrupt Vector

38H     56       Get or Set Country Information

44H     68       IOCTL (I/O Control)

59H     89       Get Extended Error Information

5EH     94       Get Machine Name, Get or Set Printer Setup

5FH     95       Device Reallocation

63H     99       Get Lead Byte Table

65H    101       Get Extended Country Information

66H    102       Get or Set Code Page

7.7.1. Int 21H Function 25H

Description:

Set Interrupt Vector

Initializes a CPU interrupt vector to point to an  interrupt

Interrupt 21H, Miscellaneous System Functions

handling routine.

Input                                        Output

---------------------------------            -------

AH=25H                                       Nothing

AL=interrupt number

DS:DX=segment:offset of interrupt

handling routine

- This function should  be  used  in  preference  to  direct

editing of the  interrupt  vector  table  by  well-behaved

applications.

- Before an interrupt vector is modified, its original value

should be obtained with Int 21H Function 35H and saved, so

that it can be restored using this function before program

termination.

7.7.2. Exec

;* Exec - Executes a child process.

;* Exec handles the usual chores associated

;* with spawning a process:

;* (1) parsing the command line tail and loading the

;* FCBs with the first two arguments;

;* (2) setting and restoring the vectors

;* for Interrupts 1Bh, 23h, and 24h; and

;* (3) querying DOS for the child's return code.

;*

;* Shows:   DOS Functions - 29h (Parse Filename)

;*                          25h (Set Interrupt Vector)

;*                          35h (Get Interrupt Vector)

;*                          4Bh (Execute Program)

;*                          4Dh (Get Return Code)

;*

;* Params:  spec - Pointer to ASCIIZ specification

;*                 for program file

;*                 (must include .COM or .EXE extension)

;*          block - Pointer to parameter block structure

;*          break - Pointer to new Ctrl-Break

;*                  (Interrupt 1Bh) handler

;*          ctrlc - Pointer to new Ctrl-C

;*                  (Interrupt 23h) handler

;*          criterr - Pointer to new Critical Error

;*                  (Interrupt 24h) handler

;*

;* Return:  Short integer with child return code,

;*                or -1 for EXEC error

Exec    PROC \

USES ds si di, \

spec:PTR BYTE, block:PTR parmblk, break:PTR BYTE, \

ctrlc:PTR BYTE, criterr:PTR BYTE

jmp     SHORT @F      ; Jump over data area

old_1Bh DD      WORD PTR ?    ; Keep vectors for Interrupts

Interrupt 21H, Miscellaneous System Functions

old_23h DD      WORD PTR ?    ;   1Bh, 23h, and 24h in

old_24h DD      WORD PTR ?    ;   code segment

old_stk DD      WORD PTR ?    ; Keep stack pointer

@@:     Vector 1Bh, cs:old_1Bh, break

; Save, replace Int 1Bh vector

Vector 23h, cs:old_23h, ctrlc

; Save, replace Int 23h vector

Vector 24h, cs:old_24h, criterr

; Save, replace Int 24h vector

LoadPtr ds, bx, block ; Point DS:BX to parameter block

push    ds            ; Save segment address

les     di, [bx].fcb1 ; Point ES:DI to first FCB

lds     si, [bx].taddr; Point DS:SI to command line tai

inc     si            ; Skip over count byte

mov     ax, 2901h     ; Set AH to request Function 29h

; AL = flag to skip leading blank

int     21h        ; Parse command-line into first FCB

pop     es         ; Recover seg addr of parameter bloc

les     di, es:[bx].fcb2 ; Point ES:DI to second FCB

mov     ax, 2901h  ; Request DOS Function #29h again

int     21h        ; Parse command-line into second FCB

push    bp         ; Save only important register

mov     WORD PTR cs:old_stk[0], sp

mov     WORD PTR cs:old_stk[2], ss

LoadPtr es, bx, block  ; ES:BX points to param block

LoadPtr ds, dx, spec   ; DS:DX points to path spec

mov     ax, 4B00h      ; AH = DOS Function 4Bh

; AL = 0 for load and execute

int     21h            ; Execute Program

; Reset stack pointers

mov     sp, WORD PTR cs:old_stk[0]

mov     ss, WORD PTR cs:old_stk[2]

pop     bp             ; Recover saved register

; Restore vectors for Interrupts 1Bh, 23h, and 24h.

mov     ax, 251Bh      ; AH = DOS Function 25h

; AL = interrupt number

lds     dx, cs:old_1Bh ; DS:DX = original vector

int     21h            ; Set Interrupt 1Bh Vector

mov     al, 23h        ; AL = interrupt number

lds     dx, cs:old_23h ; DS:DX = original vector

int     21h            ; Set Interrupt 23h Vector

mov     al, 24h        ; AL = interrupt number

lds     dx, cs:old_24h ; DS:DX = original vector

int     21h            ; Set Interrupt 24h Vector

mov     ax, -1         ; Set error code

jc      exit           ; If EXEC error, exit

mov     ah, 4Dh        ; Else request child's code

int     21h            ; Get Return Code

sub     ah, ah         ; Make short integer

exit:   ret

Exec    ENDP

Interrupt 21H, Miscellaneous System Functions

7.7.3. Int 21H Function 30H

Description:

Get MS-DOS Version Number

Returns the version number  of  the  host  MS-DOS  operating

system. This  function  is  used  by  application  programs  to

determine the capabilities of their environment.

Input    Output

------   ---------------------------------------------------

AH=30H   If running under MS-DOS version 1

AL=00H   If running under MS-DOS version 2.0 and later

AL=major version number (MS-DOS 3.1 = 3, etc.)

AH=minor version number (MS-DOS 3.1 = 0AH, etc.)

BH=Original Equipment Manufacturer's (OEM's) serial

number (OEM dependent--usually 00H for IBM's

PC-DOS, 0FFH or other values for MS-DOS)

BL:CX=24-bit user serial number (optional, OEM

dependent)

- Because this function was not defined under MS-DOS 1.x, it

should always be called with AL = 00.  In  an  MS-DOS  1.x

environment, AL will be returned unchanged.

- Care must be taken not to exit in an unacceptable  fashion

if an MS-DOS 1.x environment is detected. For example, Int

21H Function 4CH (Terminate Process with Return Code), Int

21H Function 40H  (Write  to  File  or  Device),  and  the

standard error handle are not available in MS-DOS 1.x.  In

such cases a program should display an error message using

Int 21H Function 09H and then terminate with  Int  20H  or

Int 21H Function 00H.

7.7.4. Int 21H Function 33H

Description:

Get or Set Break Flag

Obtains or changes the  status  of  the  operating  system's

Break flag, which influences CTRL+C  checking  during  function

calls. Also returns the system boot drive in version 4.0.

Input                               Output

--------------------------------    --------------------------

If getting Break flag               DL=00H  Break flag is OFF

AH=33H                                 01H  Break flag is ON

AL=00H

If setting Break flag

AH=33H

AL=01H

DL=00H  if turning Break flag OFF

01H  if turning Break flag ON

Interrupt 21H, Miscellaneous System Functions

- When the system Break flag is on, the keyboard is examined

for a CTRL+C entry whenever any operating system input  or

output is  requested;  if  one  is  detected,  control  is

transferred to the CTRL+C  handler  (Int  23H).  When  the

Break flag is off, MS-DOS checks for a CTRL+C  entry  only

when executing the  traditional  character  I/O  functions

(Int 21H Functions 01H through 0CH).

- The Break flag is not part of the local environment of the

currently executing program; it affects all  programs.  An

application that alters the flag  should  first  save  the

flag's original status and then restore  the  flag  before

terminating.

7.7.5. Int 21H Function 35H

Description:

Get Interrupt Vector

Obtains the address of the current interrupt handler routine

for the specified machine interrupt.

Input                 Output

-------------------   -----------------------------------------

AH=35H                ES:BX=segment:offset of interrupt handler

AL=interrupt number

- Together with Int 21H Function 25H (Set Interrupt Vector),

this function is used by well-behaved application programs

to modify or inspect the machine interrupt vector table.

7.7.6. Int 21H Function 38H

Description:

Get or Set Country Information

Obtains internationalization  information  for  the  current

country. [3.0+] Obtains  internationalization  information  for

the current or specified country, or sets the  current  country

code.

Input                            Output

------------------------------   -----------------------------

If getting internationaliza-     If function successful

tion information                   Carry Flag=clear

AH=38H                             and, if getting internationa

AL=0  to get "current" coun-       ization information

try information             BX=country code

DS:DX=segment:offset of buffer    DS:DX=segment:offset of buffe

for returned information      holding internationalizatio

[3.0+]                              information filled in as

AH=38H                              follows:

Interrupt 21H, Miscellaneous System Functions

AL=0  to get "current" coun-        (for PC-DOS 2.0 and 2.1)

try information               Byte(s)   Contents

1-FEH  to get information          00H-01H   date format

for countries with            0=USA    m d y

code < 255                    1=Europe d m y

FFH    to get information            2=Japan  y m d

for countries with          02H-03H   ASCIIZ currency

code >= 255                           symbol

BX=country code, if AL = FFH        04H-05H   ASCIIZ thousands

DS:DX=segment:offset of buffer                separator

for returned information      06H-07H   ASCIIZ decimal

If setting current country code               separator

(MS-DOS versions 3.0 and later)     08H-1FH  reserved

AH=38H                              (for MS-DOS versions 2.0 an

AL=1-0FEH  country code for coun-    later, PC-DOS versions 3.0

tries with code < 255     and later)

0FFH  for countries with code    Byte(s)   Contents

>= 255                     00H-01H   date format

BX=country code, if AL = 0FFH         0=USA    m d y

DX=FFFFH                              1=Europe d m y

2=Japan  y m d

02H-06H   ASCIIZ currency

symbol string

07H-08H   ASCIIZ thousands

separator char-

acter

09H-0AH   ASCIIZ decimal

separator char-

acter

0BH-0CH   ASCIIZ date sep-

arator charactor

0DH-0EH   ASCIIZ time sep-

arator charactor

0FH       currency format

bit 0=0 if currency symbo

precedes value

bit 0=1 if currency symbo

follows value

bit 1=0 if no space betwe

value and currenc

symbol

bit 1=1 if one space betw

value and cur-

rency symbol

bit 2=0 if currency symbo

and decimal are

separate

bit 2=1 if currency symbo

replaces dec-

imal separator

10H      number of digits

after decimal in

currency

11H      time format

bit0=0 if 12-hour clock

bit0=1 if 24-hour clock

12H-15H  case-map call

address

16H-17H  ASCIIZ data-list

separator

18H-21H  reserved

If function unsuccessful

Interrupt 21H, Miscellaneous System Functions

Carry flag=set

AX=error code

7.7.7. Int 21H Function 44H

Description:

IOCTL (I/O Control)

Provides  a  direct  path  of   communication   between   an

application program and a device driver. Allows  a  program  to

obtain hardware-dependent information and to request operations

that are not supported by other MS-DOS function calls.

The IOCTL subfunctions and the MS-DOS versions in which they

first became available are:

Interrupt 21H, Miscellaneous System Functions

MS-DOS

Subfunction  Name                               version

-----------  ---------------------------------  -------

00H          Get Device Information             2.0

01H          Set Device Information             2.0

02H          Receive Control Data from          2.0

Character Device Driver

03H          Send Control Data to Character     2.0

Device Driver

04H          Receive Control Data from Block    2.0

Device Driver

05H          Send Control Data to Block         2.0

Device Driver

06H          Check Input Status                 2.0

07H          Check Output Status                2.0

08H          Check If Block Device Is           3.0

Removable

09H          Check If Block Device Is Remote    3.1

0AH          Check If Handle Is Remote          3.1

0BH          Change Sharing Retry Count         3.1

0CH          Generic I/O Control for Character

Devices

CL = 45H: Set Iteration Count      3.2

CL = 4AH: Select Code Page         3.3

CL = 4CH: Start Code Page          3.3

Preparation

CL = 4DH: End Code Page            3.3

Preparation

CL = 5FH: Set Display Information  4.0

CL = 65H: Get Iteration Count      3.2

CL = 6AH: Query Selected Code Page 3.3

CL = 6BH: Query Prepare List       3.3

CL = 7FH: Get Display Information  4.0

0DH          Generic I/O Control for Block Devices

CL = 40H: Set Device Parameters    3.2

CL = 41H: Write Track              3.2

CL = 42H: Format and Verify Track  3.2

CL = 47H: Set Access Flag          4.0

CL = 60H: Get Device Parameters    3.2

CL = 61H: Read Track               3.2

CL = 62H: Verify Track             3.2

CL = 67H: Get Access Flag          4.0

0EH          Get Logical Drive Map              3.2

0FH          Set Logical Drive Map              3.2

Only IOCTL Subfunctions 00H, 06H, and 07H may  be  used  for

handles associated with files.  Subfunctions  00H-08H  are  not

supported on network devices.

7.7.8. Int 21H Function 59H

Description:

Get Extended Error Information DOS 3 Only

Obtains  detailed  error  information   after   a   previous

unsuccessful Int 21H function call, including  the  recommended

remedial action.

Interrupt 21H, Miscellaneous System Functions

Input     Output

------    --------------------------------------------------

AH=59H    AX=extended error code

BX=00     BH=error class

01H      if out of resource (such as storage

or handles)

02H      if not error, but temporary situation

(such as locked region in file) that

can be expected to end

03H      if authorization problem

04H      if internal error in system software

05H      if hardware failure

06H      if system software failure not the

fault of the active process (such

as missing configuration files)

07H      if application program error

08H      if file or item not found

09H      if file or item of invalid type or format

0AH (10) if file or item interlocked

0BH (11) if wrong disk in drive, bad spot on

disk, or storage medium problem

0CH (12) if other error

BL=recommended action

01H      retry reasonable number of times,

then prompt user to select abort or ignor

02H      retry reasonable number of times

with delay between retries, then

prompt user to select abort or ignore

03H      get corrected information from user

(typically caused by incorrect

filename or drive specification)

04H      abort application with cleanup

(i.e., terminate the program in as

orderly a manner as possible,

releasing locks, closing files, etc.)

05H      perform immediate exit without cleanup

06H      ignore error

07H      retry after user intervention to

remove cause of error

CH=error locus

01H       unknown

02H       block device (disk or disk emulator)

03H       network

04H       serial device

05H       memory

and, for MS-DOS versions 3.0 and later

ES:DI=ASCIIZ volume label of disk to insert, if

AX=0022H (invalid disk change)

- This function may  be  called  after  any  other  Int  21H

function call that returned an error status, in  order  to

obtain more detailed information about the error type  and

the recommended action. If the previous Int  21H  function

call had no error, 0000H is returned in register AX.  This

function may also be called  during  the  execution  of  a

critical-error (Int 24H) handler.

- The contents of registers CL, DX, SI, DI, BP, DS,  and  ES

are destroyed by this function.

- Note that extended error codes  13H-1FH  (19-31)  and  22H

(34) correspond exactly to the error  codes  0-0CH  (0-12)

Interrupt 21H, Miscellaneous System Functions

and 0FH (15) returned by Int 24H.

- You should  not  code  your  programs  to  recognize  only

specific error  numbers  if  you  want  to  ensure  upward

compatibility, because new error codes are added  in  each

new version of MS-DOS.

7.7.9.    Interrupt    21H,    Function     5EH

(Machine/Printer)

Interrupt  Fnct  Subfnct    Name

21H        5EH   00H        Get Machine Name

21H        5EH   02H        Set Printer Setup String

21H        5EH   03H        Get Printer Setup String

7.7.9.1. Int 21H Function 5EH Subfunction 00H

Description:

Get Machine Name DOS 3.1 Only

Returns the address of an  ASCIIZ  (null-terminated)  string

identifying the local computer.  This  function  call  is  only

available when Microsoft Networks is running.

Input                    Output

---------------------    --------------------------------------

AH=5EH                   If function successful

AL=00H                     Carry flag=clear

DS:DX=segment:offset       CH=00H     if name not defined

of buffer to receive      <> 00H if name defined

string                    CL=netBIOS name number (if CH < > 0)

DX:DX=segment:offset of identifier

(if CH <> 0)

If function unsuccessful

Carry flag=set

AX=error code

- The computer identifier is a 15-byte string,  padded  with

spaces and terminated with a null (00H) byte.

- The  effect  of  this  call  is   unpredictable   if   the

file-sharing support module is not loaded.

7.7.9.2. Int 21H Function 5EH Subfunction 02H

Description:

Set Printer Setup String DOS 3.1 Only

Interrupt 21H, Miscellaneous System Functions

Specifies a string to be sent in front of all files directed

to a particular network printer, allowing  users  at  different

network nodes to specify individualized operating modes on  the

same  printer.  This  function  call  is  only  available  when

Microsoft Networks is running.

Input                       Output

-------------------------   ----------------------------------

AH=5EH                      If function successful

AL=02H                        Carry flag=clear

BX=redirection list index   If function unsuccessful

CX=length of setup string     Carry flag=set

DS:SI=segment:offset of       AX=error code

setup string

- The redirection  list  index  passed  in  register  BX  is

obtained  with   Function   5FH   Subfunction   02H   (Get

Redirection List Entry).

- See also Function 5EH Subfunction 03H, which may  be  used

to obtain the  existing  setup  string  for  a  particular

network printer.

7.7.9.3. Int 21H Function 5EH Subfunction 03H

Description:

Get Printer Setup String DOS 3.1 Only

Obtains the printer setup string for  a  particular  network

printer. This function call is only  available  when  Microsoft

Networks is running.

Input                            Output

------------------------------   ------------------------------

AH=5EH                           If function successful

AL=03H                             Carry flag=clear

BX=redirection list index          CX=length of printer setup

ES:DI=segment:offset of buffer        string

to receive setup string         If function unsuccessful

Carry flag=set

AX=error code

- The redirection  list  index  passed  in  register  BX  is

obtained  with   Function   5FH   Subfunction   02H   (Get

Redirection List Entry).

- See also Int 21H Function 5EH Subfunction  02H,  which  is

used to specify a setup string for a network printer.

Interrupt 21H, Miscellaneous System Functions

7.7.10. Interrupt  21H,  Function  5FH  (Device

Redirection)

Interrupt  Fnct  Subfnct    Name

21H        5FH   02H        Get Redirection List Entry

21H        5FH   03H        Redirect Device

21H        5FH   04H        Cancel Device Redirection

7.7.10.1. Int 21H Function 5FH Subfunction  02H

Description:

Get Redirection List Entry DOS 3.1 Only

Allows inspection of  the  system  redirection  list,  which

associates local logical names with network files, directories,

or  printers.  This  function  call  is  only  available   when

Microsoft  Networks  is  running  and  the  filesharing  module

(SHARE.EXE) has been loaded.

Input                         Output

---------------------------   ---------------------------------

AH=5FH                        If function successful

AL=02H                          Carry flag=clear

BX=redirection list index       BH=device status flag

DS:SI=segment:offset of            bit 0=0 if device valid

16-byte buffer to receive              =1 if not valid

local device name              BL=device type

ES:DI=segment:offset of            03H if printer

128-byte buffer to receive        04H if drive

network name                   CX=stored parameter value

DX=destroyed

BP=destroyed

DS:SI=segment:offset of ASCIIZ

local device name

ES:DI=segment:offset of ASCIIZ

network name

If function unsuccessful

Carry flag=set

AX=error code

- The  parameter  returned  in  CX  is  a  value  that   was

previously passed to MS-DOS in register CX  with  Int  21H

Function  5FH  Subfunction  03H  (Redirect   Device).   It

represents data that is private to the applications  which

store and retrieve it, and has no meaning to MS-DOS.

Interrupt 21H, Miscellaneous System Functions

7.7.10.2. Int 21H Function 5FH Subfunction  03H

Description:

Redirect Device DOS 3.1 Only

Establishes redirection across the network by associating  a

local device name with a network name. This  function  call  is

available only when  Microsoft  Networks  is  running  and  the

file-sharing module (SHARE.EXE) has been loaded.

Input                             Output

------------------------------    ----------------------------

AH=5FH                            If function successful

AL=03H                              Carry flag=clear

BL=device type                    If function unsuccessful

03H  if printer                  Carry flag=set

04H  if drive                    AX=error code

CX=parameter to save for caller

DS:SI=segment:offset of ASCIIZ

local device name

ES:DI=segment:offset of ASCIIZ

network name, followed by

ASCIIZ password

- The local  name  can  be  a  drive  designator  (a  letter

followed by a colon, such as "D:"), a printer name,  or  a

null string. Printer names must be one of  the  following:

PRN, LPT1, LPT2, or LPT3. If a null string followed  by  a

password is used, MS-DOS attempts to grant access  to  the

network directory with the specified password.

- The parameter passed in CX can be retrieved by later calls

to Int 21H Function 5FH  Subfunction  02H.  It  represents

data that is private to the applications which  store  and

retrieve it, and has no meaning to MS-DOS.

7.7.10.3. Int 21H Function 5FH Subfunction  04H

Description:

Cancel Device Redirection DOS 3.1 Only

Cancels a  previous  redirection  request  by  removing  the

association of a local device name with a  network  name.  This

function call is only  available  when  Microsoft  Networks  is

running  and  the  file-sharing  module  (SHARE.EXE)  has  been

loaded.

Input                         Output

-------------------------     ------------------------------

AH=5FH                        If function successful

AL=04H                          Carry flag=clear

DS:SI=segment:offset of       If function unsuccessful

ASCIIZ local device name       Carry flag=set

AX=error code

Interrupt 21H, Miscellaneous System Functions

- The supplied name can be  a  drive  designator  (a  letter

followed by a colon, such as "D:"), a printer name,  or  a

string starting with two backslashes  (\).  Printer  names

must be one of the following: PRN, LPT1, LPT2, or LPT3. If

the string with two backslashes is  used,  the  connection

between the local machine and  the  network  directory  is

terminated.

7.7.11. Int 21H Function 63H

Description:

Get Lead Byte Table

Obtains the address of the system table of legal  lead  byte

ranges for extended character sets,  or  sets  or  obtains  the

interim console flag. Int 21H Function 63H is available only in

MS-DOS version 2.25; it is not supported in MS-DOS versions 3.0

and later.

Input                             Output

-------------------------------   -----------------------------

AH=63H                            If function successful

AL=subfunction                      Carry flag=clear

00H  if getting address of        and, if called with AL=00H

system lead byte table       DS:SI=segment:offset of lea

01H  if setting or clearing             byte table

interim console flag         or, if called with AL=02H

02H  if obtaining value of        DL=value of interim console

interim console flag            flag

If AL=01H                         If function unsuccessful

DL=00H  if clearing interim         Carry flag=set

console flag                AX=error code

01H  if setting interim

console flag

- The lead byte table  consists  of  a  variable  number  of

2-byte entries, terminated by 2  null  (00H)  bytes.  Each

pair defines the beginning and ending value for a range of

lead bytes. The value of a legal lead byte  is  always  in

the range 80-0FFH.

- Entries in the lead byte table must be in ascending order.

If no legal lead bytes are defined in a given system,  the

table consists only of the two null bytes.

- If the interim console flag is set, Int 21H Functions  07H

(Unfiltered Character Input), 08H (Character Input without

Echo), and 0BH  (Keyboard  Status)  will  support  interim

characters.

- Unlike most other MS-DOS services, this function call does

not necessarily preserve any registers except SS:SP.

- [4.0] The address of the DBCS lead byte table can also  be

obtained with Int 21H Function 65H.

Interrupt 21H, Miscellaneous System Functions

7.7.12. Int 21H Function 65H

Description:

Get Extended Country Information DOS 3.3 Only

Obtains information about the specified country and/or  code

page.

Input                                      Output

----------------------------------------   --------------------

AH=65H                                     If function success-

AL=subfunction                                         ful

01H=Get General Internationalization       Carry flag=clear

Information                            and requested data

02H=Get Pointer to Uppercase Table         placed in calling

04H=Get Pointer to Filename Uppercase      program's buffer

Table                                If function unsuc-

06H=Get Pointer to Collating Table          cessful

07H=Get Pointer to DBCS Vector (MS-DOS     Carry flag=set

versions 4.0 and later)                AX=error code

BX=code page of interest (-1 = active CON device)

CX=length of buffer to receive information

DX=country ID (-1 = default)

ES:DI=address of buffer to receive information

- The information returned by this function is a superset of

the information returned by Int 21H Function 38H.

- This function may fail if either the country code  or  the

code page number is invalid or if the code page  does  not

match the country code.

- The function fails if the specified buffer length is  less

than five bytes. If the buffer to receive the  information

is at least five bytes long  but  is  too  short  for  the

requested information, the data is truncated and no  error

is returned.

- The format of the data returned by Subfunction 01H is:

Interrupt 21H, Miscellaneous System Functions

Byte(s)   Contents

-------   ----------------------------------------

00H       Information ID code (1)

01H-02H   Length of following buffer

03H-04H   Country ID

05H-06H   Code page number

07H-08H   Date format

0 = USA    m d y

1 = Europe d m y

2 = Japan  y m d

09H-0DH   ASCIIZ currency symbol

0EH-0FH   ASCIIZ thousands separator

10H-11H   ASCIIZ decimal separator

12H-13H   ASCIIZ date separator

14H-15H   ASCIIZ time separator

16H       Currency format flags

bit 0=0 if currency symbol precedes value

=1 if currency symbol follows value

bit 1=0 if no space between value and

currency 0 symbol

=1 if one space between value and

currency symbol

bit 2=0 if currency symbol and decimal

are separate

=1 if currency symbol replaces

decimal separator

17H       Number of digits after decimal in

currency

18H       Time format

bit 0=0 if 12-hour clock

=1 if 24-hour clock

19H-1CH   Case-map routine call address

1DH-1EH   ASCIIZ data list separator

1FH-28H   Reserved

- The format of the data returned by Subfunctions 02H,  04H,

06H, and 07H is:

Byte(s)   Contents

-------   -----------------------------------

00H       Information ID code (2, 4, 6, or 7)

01H-04H   Double-word pointer to table

- The uppercase and filename uppercase tables are a  maximum

of 130 bytes long. The first two bytes contain the size of

the table;  the  following  bytes  contain  the  uppercase

equivalents, if any, for character codes 80H-FFH. The main

use of these  tables  is  to  map  accented  or  otherwise

modified vowels to their  plain  vowel  equivalents.  Text

translated with the help of this  table  can  be  sent  to

devices that do not support  the  IBM  graphics  character

set, or used to create filenames that  do  not  require  a

special keyboard configuration for entry.

- The collating table is a maximum of 258  bytes  long.  The

first  two  bytes  contain  the  table  length,  and   the

subsequent bytes contain the values to  be  used  for  the

corresponding  character  codes  (0-FFH)  during  a   sort

operation. This table maps uppercase and  lowercase  ASCII

characters to the same collating codes so that sorts  will

be case-insensitive, and it maps accented vowels to  their

plain vowel equivalents.

Interrupt 21H, Miscellaneous System Functions

- [4.0+]  Subfunction   07H   returns   a   pointer   to   a

variable-length table that defines ranges for  double-byte

character set (DBCS) lead bytes. The table  is  terminated

by a pair of zero bytes, unless it must  be  truncated  to

fit in the buffer, and has the following format:

dw     length

db     start1,end1

db     start2,end2

(3-vertical ellipsis)

db     0,0

For example:

dw     4

db     81h,9fh

db     0e0h,0fch

db     0,0

- In  some  cases  a  truncated  translation  table  may  be

presented to the program by  MS-DOS.  Applications  should

always check the length at the beginning of the  table  to

ensure  that  it  contains  a  translation  code  for  the

particular character of interest.

7.7.13. Int 21H Function 66H

Description:

Get or Set Code Page DOS 3.3 Only

Obtains or selects the current code page.

Input                          Output

----------------------------   ------------------------------

AH=66H                         If function successful

AL=subfunction                   Carry flag=clear

01H=Get Code Page             and, if called with AL=01H

02H=Select Code Page          BX=active code page

BX=code page to select, if       DX=default code page

AL=02H                      If function unsuccessful

Carry flag=set

AX=error code

- When the Select Code Page subfunction is used, MS-DOS gets

the new code page from the COUNTRY.SYS  file.  The  device

must be previously prepared for code page  switching  with

the appropriate DEVICE= directive in the  CONFIG.SYS  file

and NLSFUNC and MODE CP PREPARE commands  (usually  placed

in the AUTOEXEC.BAT file).

Interrupt 21H, Process-Control Functions

7.8. Interrupt 21H, Process-Control Functions

Hex #  Dec #     Name

00H      0       Terminate Process

26H     38       Create New Program Segment Prefix (PSP)

31H     49       Terminate and Stay Resident

4BH     75       Execute Program (EXEC)

4CH     76       Terminate Process with Return Code

4DH     77       Get Return Code

62H     98       Get PSP Address

7.8.1. Int 21H Function 00H

Description:

Terminate Process

Terminates the current  process.  This  is  one  of  several

methods that a program can use to perform a final exit.

Input                                Output

-------------------------            -------

AH=00H                               Nothing

CS=segment address of

program segment prefix

1) MS-DOS then takes the following actions:

All memory belonging to the process is released.

File buffers are flushed, and any open handles  for

files or devices owned by the process are closed.

The  termination  handler  vector  (Int  22H)   is

restored from PSP:000AH.

The CTRL+C handler vector  (Int  23H)  is  restored

from PSP:000EH.

The critical-error  handler  vector  (Int  24H)  is

restored from PSP:0012H.

Control is transferred to the termination  handler.

If the program is returning to  COMMAND.COM,  control

transfers to the resident  portion,  and  the  transient

portion is reloaded if necessary. If a batch file is  in

progress, the next line  of  the  file  is  fetched  and

interpreted; otherwise, a prompt is issued for the  next

user command.

2) Any files that have been written to using FCBs should be

closed before performing this exit call; otherwise, data

may be lost.

3) Other methods of performing a final exit are:

- Int 20H

- Int 21H Function 31H

- Int 21H Function 4CH

- Int 27H

4) Int 21H Functions 31H and 4CH are the  preferred  method

for termination because they allow a return code  to  be

passed to the parent process.

5) [3.0+] If the program is running on a network, it should

Interrupt 21H, Process-Control Functions

remove all locks it has placed on  file  regions  before

terminating.

7.8.2. Int 21H Function 26H

Description:

Create New PSP

Copies the program segment prefix  (PSP)  of  the  currently

executing program to a specified segment address in free memory

and then updates the new PSP  to  make  it  usable  by  another

program.

Input                                               Output

----------------------------------------            -------

AH=26H                                              Nothing

DX=segment of new program segment prefix

- After the executing program's PSP is copied into  the  new

segment, the memory size information in  the  new  PSP  is

updated appropriately and  the  current  contents  of  the

termination (Int  22H),  CTRL+C  handler  (Int  23H),  and

critical-error  handler  (Int  24H)  vectors   are   saved

starting at offset 0AH.

- This function does not load another program or  in  itself

cause one to be executed.

- Int 21H Function 4BH (EXEC), which can be used to load and

execute programs  or  overlays  in  either  .COM  or  .EXE

format, should be used in preference to this function.

7.8.3. Int 21H Function 31H

Description:

Terminate and Stay Resident

Terminates execution of the currently  executing

program,  passing  a  return  code  to  the  parent

process, but reserves part or all of the  program's

memory so that it will not be overlaid by the  next

transient program to be loaded.

Input                                                     Outpu

----------------------------------------------            -----

AH=31H                                                    Nothi

AL=return code

DX=amount of memory to reserve (in paragraphs)

MS-DOS then takes the following actions:

1) Flushes the file buffers and closes any open

handles for files or devices  owned  by  the

Interrupt 21H, Process-Control Functions

process.

2) Restores the termination handler vector (Int

22H) from PSP:000AH.

3) Restores the CTRL+C handler vector (Int 23H)

from PSP:000EH.

4) Restores the critical-error  handler  vector

(Int 24H) from PSP:0012H.

5) Transfers   control   to   the   termination

handler.

6) If the program is returning to  COMMAND.COM,

control transfers to the  resident  portion,

and the transient  portion  is  reloaded  if

necessary. If a batch file is  in  progress,

the next line of the  file  is  fetched  and

interpreted; otherwise, a prompt  is  issued

for the next user command.

7) This function  call  is  typically  used  to

allow user-written  utilities,  drivers,  or

interrupt handlers to be loaded as  ordinary

.COM  or  .EXE  programs  and  then   remain

resident. Subsequent entrance to the code is

via a hardware or software interrupt.

8) This function attempts to  set  the  initial

memory allocation block  to  the  length  in

paragraphs  specified  in  register  DX.  If

other memory blocks have been  requested  by

the application using Int 21H Function  48H,

they will not be released by this  function.

9) Other methods of  performing  a  final  exit

are:

- Int 20H

- Int 21H Function 00H

- Int 21H Function 4CH

- Int 27H

10) The return code may be retrieved by a parent

process through Int 21H  Function  4DH  (Get

Return Code). It can also  be  tested  in  a

batch file with an IF ERRORLEVEL  statement.

By  convention,  a  return  code   of   zero

indicates  successful   execution,   and   a

non-zero return code indicates an error.

11) This function should not be called  by  .EXE

programs that are loaded at the high end  of

the transient  program  area  (i.e.,  linked

with the  /HIGH  switch)  because  doing  so

reserves the memory that is normally used by

the  transient  part  of   COMMAND.COM.   If

COMMAND.COM cannot be reloaded,  the  system

will fail.

12) This function should be used  in  preference

to Int 27H because it supports return codes,

allows  larger  amounts  of  memory  to   be

reserved, and does not require CS to contain

the segment of the program segment prefix.

13) [3.0+]  If  the  program  is  running  on  a

network, it should remove all locks  it  has

placed on file regions before terminating.

Interrupt 21H, Process-Control Functions

7.8.4. Int 21H Function 4BH

Description:

Execute Program (EXEC)

Allows  an  application  program  to  run  another  program,

regaining control when it is finished. Can also be used to load

overlays, although this use is uncommon.

Input                              Output

---------------------------------  ----------------------------

AH=4BH                             If function successful

AL=subfunction                       Carry flag=clear

00H=Load and Execute Program       all registers except for C

03H=Load Overlay                   IP may be destroyed

ES:BX=segment:offset of parameter    [3.0+] registers are pre-

block                              served in the usual fashio

DS:DX=segment:offset of program    If function unsuccessful

specification                      Carry flag=set

AX=error code

- The parameter block format for Subfunction 00H  (Load  and

Execute Program) is as follows:

Bytes    Contents

-------  ---------------------------------------------------

00H-01H  Segment pointer to environment block

02H-03H  Offset of command line tail

04H-05H  Segment of command line tail

06H-07H  Offset of first FCB to be copied into new PSP+5CH

08H-09H  Segment of first FCB

0AH-0BH  Offset of second FCB to be copied into new PSP+6CH

0CH-0DH  Segment of second FCB

- The parameter  block  format  for  Subfunction  03H  (Load

Overlay) is as follows:

Bytes    Contents

-------  ---------------------------------------------

00H-01H  Segment address where overlay is to be loaded

02H-03H  Relocation factor to apply to loaded image

- The  environment  block  must  be  paragraph  aligned.  It

consists of a sequence of ASCIIZ strings in the form:

db 'COMSPEC=A:COMMAND.COM',0

The entire set of strings is  terminated  by  an  extra

null (00H) byte.

- The command tail format consists of a count byte, followed

by an ASCII string, terminated by a carriage return (which

is not included in the count). The first character in  the

string should be an ASCII space  (20H)  for  compatibility

with the command tail passed to programs  by  COMMAND.COM.

For example:

db 6,' *.DAT',0dh

- Before a program uses Int 21H Function 4BH to run  another

Interrupt 21H, Process-Control Functions

program, it must release all memory  it  is  not  actually

using with a call to Int 21H  Function  4AH,  passing  the

segment  address  of  its  own  PSP  and  the  number   of

paragraphs to retain.

- Ordinarily, all active handles of the parent  program  are

inherited by the child program, although  the  parent  can

prevent this in  MS-DOS  3.0  and  later  by  setting  the

inheritance bit when the file or  device  is  opened.  Any

redirection of the standard input  and/or  output  in  the

parent process also affects the child process.

- The environment block can be used to pass  information  to

the child process. If the environment block pointer in the

parameter block is zero, the  child  program  inherits  an

exact copy of the parent's environment. In any  case,  the

segment address of the child's  environment  is  found  at

offset 002CH in the child's PSP.

- After return from the EXEC function call, the  termination

type and return code of the child program may be  obtained

with Int 21H Function 4DH.

7.8.5. Int 21H Function 4CH

Description:

Terminate Process with Return Code

Terminates the current process, passing a return code to the

parent process. This is one of several methods that  a  program

can use to perform a final exit.

If you use the .STARTUP directive, you should also  use  the

.EXIT directive, which automatically  generates  code  to  call

this function.

Input                      Output

---------------            -------

AH=4CH                     Nothing

AL=return code

- MS-DOS then takes the following actions:

- All memory belonging to the process is released.

- File buffers are flushed and any open handles for files or

devices owned by the process are closed.

- The termination handler vector (Int 22H) is restored  from

PSP:000AH.

- The CTRL+C handler  vector  (Int  23H)  is  restored  from

PSP:000EH.

- The critical-error handler vector (Int  24H)  is  restored

from PSP:0012H.

- Control is transferred to the termination handler.

1) If the program  is  returning  to  COMMAND.COM,  control

transfers to the  resident  portion  and  the  transient

portion is reloaded if necessary. If a batch file is  in

progress, the next line  of  the  file  is  fetched  and

interpreted; otherwise, a prompt is issued for the  next

user command.

Interrupt 21H, Process-Control Functions

2) This  is  the  preferred  method  of   termination   for

application programs because it allows a return code  to

be passed to the parent program and does not rely on the

contents of  any  segment  register.  Other  methods  of

performing a final exit are:

- Int 20H

- Int 21H Function 00H

- Int 21H Function 31H

- Int 27H

3) Any files that have been opened using FCBs and  modified

by  the  program  should  be   closed   before   program

termination; otherwise, data may be lost.

4) The return code can be retrieved by the  parent  process

with Int 21H Function 4DH (Get Return Code). It can also

be  tested  in  a  batch  file  with  an  IF  ERRORLEVEL

statement.  By  convention,  a  return  code   of   zero

indicates successful execution, and  a  non-zero  return

code indicates an error.

5) [3.0+] If the program is running on a network, it should

remove all locks it has placed on  file  regions  before

terminating.

7.8.6. Int 21H Function 4DH

Description:

Get Return Code

Used by a parent process, after the successful execution  of

an EXEC call (Int 21H Function 4BH), to obtain the return  code

and termination type of a child process.

Input         Output

------        -------------------------------------------------

AH=4DH        AH=exit type

00H if normal termination by Int 20H, Int 21H

00H Function 0, or Int 21H Function 4CH

01H if termination by user's entry of CTRL+C

02H if termination by critical-error handler

03H if termination by Int 21H Function 31H or

03H Int 27H

AL=return code passed by child process (0 if chi

terminated by Int 20H, Int 21H Function 0, or

Int 27H)

- This function will  yield  the  return  code  of  a  child

process  only  once.  A   subsequent   call   without   an

intervening  EXEC  (Int  21H  Function   4BH)   will   not

necessarily return any valid information.

- This function does not set the carry flag to  indicate  an

error. If no previous child process has been executed, the

values returned in AL and AH are undefined.

Interrupt 21H, Process-Control Functions

7.8.7. Int 21H Function 62H

Description:

Get PSP Address DOS 3 Only

Obtains the  segment  (paragraph)  address  of  the  program

segment prefix (PSP) for the currently executing program.

Input             Output

------            --------------------------------------------

AH=62H            BX=segment address of program segment prefix

1) Before a  program  receives  control  from  MS-DOS,  its

program segment prefix is  set  up  to  contain  certain

vital information, such as:

- the segment address of  the  program's  environment

block;

- the command line originally entered by the user;

- the original contents of the terminate, CTRL+C, and

critical-error handler vectors;

- and the top address of available RAM.

2) The segment address of the PSP is normally passed to the

program  in  registers  DS  and  ES  when  it  initially

receives control from MS-DOS.  This  function  allows  a

program to conveniently recover the PSP address  at  any

point during its execution, without having to save it at

program entry.

7.9. Interrupt 21H, Record Functions

Hex #  Dec #     Name

1AH      26      Set Data Transfer Area (DTA) Address

2FH      47      Get DTA Address

3FH      63      Read File or Device

40H      64      Write to File or Device

42H      66      Set File Pointer (current location in file)

5CH      92      Lock or Unlock Region of File

68H     104      Commit File (forces buffer to be written out)

7.9.1. Int 21H Function 1AH

Description:

Set DTA Address

Specifies the address of the disk transfer area (DTA) to  be

used for subsequent FCB-related function calls.

Interrupt 21H, Record Functions

Input                                   Output

----------------------------            -------

AH=1AH                                  Nothing

DS:DX=segment:offset of disk

transfer area

- If this function is never called by the program,  the  DTA

defaults to a 128-byte  buffer  at  offset  0080H  in  the

program segment prefix.

- In general,  it  is  the  programmer's  responsibility  to

ensure that the buffer area specified is large enough  for

any disk operation that will use it. The only exception to

this is that MS-DOS will detect and abort  disk  transfers

that would cause a segment wrap.

- Int 21H Function 2FH can be used to determine the  current

disk transfer address.

- The only handle-type operations that rely on  the  current

DTA address are the directory search  functions,  Int  21H

Functions 4EH and 4FH.

7.9.2. SetDTA

;* SetDTA - Sets address for new Disk Transfer Area.

;*

;* Shows:   DOS Function - 1Ah (Set DTA Address)

;*

;* Params:  dta - Far pointer to new transfer address

;*

;* Return:  None

SetDTA  PROC \

USES ds, \

dta:FAR PTR BYTE

lds     dx, [dta]               ; Point DS:DX to DTA

mov     ah, 1Ah                 ; DOS Function 1Ah

int     21h                     ; Set DTA Address

ret

SetDTA  ENDP

7.9.3. Int 21H Function 2FH

Description:

Get DTA Address

Obtains the current address of the disk transfer area  (DTA)

for FCB read/write operations.

Interrupt 21H, Record Functions

Input             Output

------            ------------------------------------------

AH=2FH            ES:BX=segment:offset of disk transfer area

The disk transfer area address is set  with  Int  21H  Function

1AH. The default DTA is a 128-byte buffer at offset 80H in  the

program segment prefix.

7.9.4. Int 21H Function 3FH

Description:

Read File or Device

Given a valid file handle from a  previous  open  or  create

operation, a buffer address, and a length in  bytes,  transfers

data at the current file pointer position from  the  file  into

the buffer and then updates the file pointer position.

Input                                  Output

------------------------------         -----------------------

AH=3FH                                 If function successful

BX=handle                              Carry flag=clear

CX=number of bytes to read             AX=bytes transferred

DS:DX=segment:offset of buffer         If function unsuccessful

Carry flag= set

AX=error code

- If reading from a character device (such as  the  standard

input) in cooked mode, at most one line of input  will  be

read (i.e., up to a carriage re-  turn  character  or  the

specified length, whichever comes first).

- If the carry flag is returned clear but AX = 0,  then  the

file pointer was already at end of file when  the  program

requested the read.

- If the carry flag is returned clear but AX <  CX,  then  a

partial record was read at end of  file  or  there  is  an

error.

- [3.0+] If the program is running on a  network,  the  user

must have Read access rights to the directory and file.

7.9.5. ReadFile

;* ReadFile - Read from open file to specified buffer.

;* See the CopyFile procedure for another example of using

;* DOS Function 3Fh to read files.

;*

;* Shows:   DOS Function - 3Fh (Read File or Device)

;*

;* Params:  handle - File handle

;*          len - Number of bytes to read

Interrupt 21H, Record Functions

;*          pbuff - Pointer to buffer

;*

;* Return:  Short integer with number of bytes read,

;*  or 0 if read error

ReadFile PROC \

USES ds di, \

handle:WORD, len:WORD, pbuff:PTR BYTE

LoadPtr ds, dx, pbuff  ; Point DS:DX to buffer

mov     di, dx         ; Keep string offset in DI

mov     bx, handle     ; BX = handle

mov     cx, len        ; CX = number of bytes to read

mov     ah, 3Fh        ; Request DOS read

int     21h            ; Read File or Device

jnc     exit           ; If ok, exit with bytes read

sub     ax, ax         ; Else set error code

exit:   ret

ReadFile ENDP

7.9.6. Int 21H Function 40H

Description:

Write File or Device

Given a valid file handle from a  previous  open  or  create

operation, a buffer address, and a length in  bytes,  transfers

data from the buffer into the file and then  updates  the  file

pointer position.

Input                                     Output

------------------------------            ---------------------

AH=40H                                    If function successfu

BX=handle                                 Carry Flag=clear

CX=number of bytes to write               AX=bytes transferred

DS:DX=segment:offset of buffer            Carry flag=set

AX=error code

- If the carry flag is returned clear but AX <  CX,  then  a

partial record was written or there is an error. This  can

be caused by a CTRL+Z (1AH) embedded in the  data  if  the

destination is a character device in cooked mode or  by  a

disk-full condition if the destination is a file.

- If the function is  called  with  CX  =  0,  the  file  is

truncated  or  extended  to  the  current   file   pointer

position.

- [3.0+] If the program is running on a  network,  the  user

must have Write access rights to the directory and file.

Interrupt 21H, Record Functions

7.9.7. Int 21H Function 42H

Description:

Set File Pointer

Sets the file location pointer  relative  to  the  start  of

file, end of file, or current file position.

Input                             Output

------------------------------    ----------------------------

AH=42H                            If function successful

AL=method code                     Carry flag=clear

00H absolute offset from          DX=most significant half of

start of file                    resulting file pointer

01H signed offset from            AX=least significant half of

current file pointer             resulting file pointer

02H signed offset from end       If function unsuccessful

of file                        Carry flag=set

BX=handle                          AX=error code

CX=most significant half of offset

DX=least significant half of offset

- This function uses a method code  and  a  double-precision

(32-bit) value to set the file pointer.  The  next  record

read or written in the file will begin  at  the  new  file

pointer location.

- Method 02H may be used to find the size  of  the  file  by

calling Int 21H Function 42H  with  an  offset  of  0  and

examining the pointer location that is returned.

- Using methods 01H or 02H, it is possible to set  the  file

pointer to a location that is before the start of file. If

this is done, no error is returned by this  function,  but

an error will be encountered upon a subsequent attempt  to

read or write the file.

- No matter  what  method  is  used  in  the  call  to  this

function, the file pointer returned in DX:AX is always the

resulting absolute byte offset from the start of file.

7.9.8. Int 21H Function 5CH

Description:

Lock or Unlock File Region DOS 3 Only

Locks or unlocks a specified  region  of  a  file  that  was

previously opened or created with Int 21H Functions  3CH,  3DH,

5AH,  or  5BH.  This  function  is  not  available  unless  the

file-sharing module (SHARE.EXE) is loaded.

Interrupt 21H, Record Functions

Input                            Output

------------------------------   -----------------------------

AH=5CH                           If function successful

AL=00H  if locking region          Carry flag=clear

01H  if unlocking region      If function unsuccessful

BX=handle                          Carry flag=set

CX=high part of region offset      AX=error code

DX=low part of region offset

SI=high part of region length

DI=low part of region length

- This   function   is   useful   for   file   and    record

synchronization  in  a  multi-  tasking   environment   or

network. Access to the file as a whole is control- led  by

the attribute and file-sharing parameters passed  in  open

or create calls, and by the file's attributes,  which  are

stored in its directory entry.

- The beginning  location  in  the  file  to  be  locked  or

unlocked  is  supplied  as  a  positive   double-precision

integer, which is a byte offset into the file. The  length

of the region to be locked or unlocked is  similarly  sup-

plied as a positive double-precision integer.

- For every call to lock a region of a file, there must be a

subsequent un- lock call with exactly the same file offset

and length.

- Locking beyond the current end of file is not an error.

- If a process terminates without releasing active locks  on

a file, the re- sult  is  undefined.  Therefore,  programs

using this function should install their own Int  23H  and

Int 24H handlers so the processes cannot be  termin-  ated

unexpectedly.

- Programs that are loaded  with  the  EXEC  call  (Int  21H

Function 4BH) inherit the handles of their parent but  not

any active locks.

- Duplicate handles created with Int 21H  Function  45H,  or

handles redirected to the file with Int 21H Function  46H,

are allowed access  to  locked  regions  within  the  same

process.

7.9.9. Int 21H Function 5CH

Description:

Lock or Unlock File Region DOS 3 Only

Locks or unlocks a specified  region  of  a  file  that  was

previously opened or created with Int 21H Functions  3CH,  3DH,

5AH,  or  5BH.  This  function  is  not  available  unless  the

file-sharing module (SHARE.EXE) is loaded.

Interrupt 21H, Record Functions

Input                            Output

------------------------------   -----------------------------

AH=5CH                           If function successful

AL=00H  if locking region          Carry flag=clear

01H  if unlocking region      If function unsuccessful

BX=handle                          Carry flag=set

CX=high part of region offset      AX=error code

DX=low part of region offset

SI=high part of region length

DI=low part of region length

- This   function   is   useful   for   file   and    record

synchronization  in  a  multi-  tasking   environment   or

network. Access to the file as a whole is control- led  by

the attribute and file-sharing parameters passed  in  open

or create calls, and by the file's attributes,  which  are

stored in its directory entry.

- The beginning  location  in  the  file  to  be  locked  or

unlocked  is  supplied  as  a  positive   double-precision

integer, which is a byte offset into the file. The  length

of the region to be locked or unlocked is  similarly  sup-

plied as a positive double-precision integer.

- For every call to lock a region of a file, there must be a

subsequent un- lock call with exactly the same file offset

and length.

- Locking beyond the current end of file is not an error.

- If a process terminates without releasing active locks  on

a file, the re- sult  is  undefined.  Therefore,  programs

using this function should install their own Int  23H  and

Int 24H handlers so the processes cannot be  termin-  ated

unexpectedly.

- Programs that are loaded  with  the  EXEC  call  (Int  21H

Function 4BH) inherit the handles of their parent but  not

any active locks.

- Duplicate handles created with Int 21H  Function  45H,  or

handles redirected to the file with Int 21H Function  46H,

are allowed access  to  locked  regions  within  the  same

process.

7.9.10. Int 21H Function 68H

Description:

Commit File DOS 3.3 Only

Forces all data in MS-DOS's internal buffers associated with

a specified handle to be physically written to the  device.  If

the handle refers to a file, and the file  has  been  modified,

the time and date stamp and file size in the  file's  directory

entry are updated.

Interrupt 21H, Record Functions

Input                Output

---------            ------------------------------

AH=68H               If function successful

BX=handle              Carry flag=clear

If function unsuccessful

Carry flag= set

AX=error code

- The effect of this function is equivalent to  closing  and

reopening a file, or to duplicating a handle for the  file

with Int 21H Function 45H and then closing the  duplicate.

However, this function has the advantage that it will  not

fail due to lack of handles, and the application does  not

risk losing control of the file in multitasking or network

environments.

- If this function is requested for a handle associated with

a character device, a success flag is returned  but  there

is no other effect.

7.10. Interrupt 21H, Record Functions, FCB

You can perform all the functions  below  more  easily  with

DOS2 functions. The services below are documented  so  you  can

analyze code written for DOS1.

Hex #  Dec #     Name

14H     20       Sequential Read

15H     21       Sequential Write

21H     33       Random Read

22H     34       Random Write

24H     36       Set Relative Record Number

27H     39       Random Block Read

28H     40       Random Block Write

7.10.1. Int 21H Function 14H

Description:

Sequential Read

Reads the next sequential block of data from a file and then

increments the file pointer appropriately.

Input                             Output

------------------------------    ----------------------------

AH=14H                            AL=00H  if read successful

DS:DX=segment:offset of prev-        01H  if end of file

iously opened file con-        02H  if segment wrap

trol block                     03H  if partial record at

end of file

Interrupt 21H, Record Functions, FCB

- The record  is  read  into  memory  at  the  current  disk

transfer area (DTA) address, specified by the most  recent

call to Int 21H Function 1AH. If the size  of  the  record

and the location of the buffer are  such  that  a  segment

overflow or wraparound would  occur,  the  function  fails

with a return code of 02H.

- The number of bytes of data to be read is specified by the

record size field (offset 0EH) of the file  control  block

(FCB).

- The file location  of  the  data  that  will  be  read  is

specified by the com- bination of the current block  field

(offset 0CH) and current record field (offset 20H) of  the

file  control  block  (FCB).   These   fields   are   also

automatically incremented by this function.

- If a partial record is read at the  end  of  file,  it  is

padded to the requested record length with zeros.

- [3.0+] If the program is running on a  network,  the  user

must have Read access rights to the  directory  containing

the file to be read.

7.10.2. Int 21H Function 15H

Description:

Sequential Write

Writes the next sequential block of data  into  a  file  and

then increments the file pointer appropriately.

Input                            Output

------------------------------   -----------------------------

AH=15H                           AL=00H  if write successful

DS:DX=segment:offset of prev-       01H  if disk is full

iously opened file con-       02H  if segment wrap

trol block

- The  record  is  written   (logically,   not   necessarily

physically) to the disk from memory at  the  current  disk

transfer area (DTA)  address,  spec-  ified  by  the  most

recent call to Int 21H Function 1AH. If the  size  of  the

record and the location of the  buffer  are  such  that  a

segment over- flow or wraparound would occur, the function

fails with a return code of 02H.

- The number of bytes of data to be written is specified  by

the record size field (offset 0EH)  of  the  file  control

block (FCB).

- The file location of the data  that  will  be  written  is

specified by the combination of the  current  block  field

(offset 0CH) and current record field (offset 20H) of  the

file  control  block  (FCB).   These   fields   are   also

automatically incremented by this function.

- [3.0+] If the program is running on a  network,  the  user

must have Write access rights to the directory  containing

the file to be written.

Interrupt 21H, Record Functions, FCB

7.10.3. Int 21H Function 21H

Description:

Random Read

Reads a selected record from a file into memory.

Input                   Output

--------------------    --------------------------------------

AH=21H                  AL=00H  if read successful

DS:DX=segment:offset       01H  if end of file

of previously        02H  if segment wrap, read canceled

opened file          03H  if partial record read at end

control block                      of file

- The record  is  read  into  memory  at  the  current  disk

transfer area ad- dress, specified by the most recent call

to  Int  21H  Function  1AH.  It   is   the   programmer's

responsibility to ensure that this area  is  large  enough

for any record that will be transferred. If the  size  and

loca- tion of the buffer are such that a segment  overflow

or wraparound would  occur,  the  function  fails  with  a

return code of 02H.

- The file location of the data to be read is determined  by

the combina- tion of  the  relative-record  field  (offset

21H) and the record size field (offset 0EH)  of  the  FCB.

The default record size is 128 bytes.

- The current block field (offset 0CH)  and  current  record

field  (offset  20H)  are  updated  to  agree   with   the

relative-record field as a side effect of the function.

- The relative-record field of the FCB is not incremented by

this  func-  tion;  it  is  the  responsibility   of   the

application to update the FCB appropriately if  it  wishes

to read successive records. Compare with Int 21H  Function

27H, which can read multiple  records  with  one  function

call  and  automatically  increments  the  relative-record

field.

- If a partial record is read at end of file, it  is  padded

to the requested record length with zeros.

- [3.0+] If the program is running on a  network,  the  user

must have Read access rights to the  directory  containing

the file to be read.

7.10.4. Int 21H Function 22H

Description:

Random Write

Writes data from memory into a selected record in a file.

Interrupt 21H, Record Functions, FCB

Input                   Output

--------------------    ---------------------------------------

AH=22H                  AL=00H  if write successful

DS:DX=segment:offset       01H  if disk full

of previously        02H  if segment wrap, write canceled

opened file

control block

- The  record  is  written   (logically,   not   necessarily

physically) to the file from memory at  the  current  disk

transfer address, specified by the most recent call to Int

21H Function 1AH. If the size and location of  the  buffer

are such that  a  segment  overflow  or  wraparound  would

occur, the function fails with a return code of 02H.

- The file location of the data to be written is  determined

by the combin- ation of the relative-record field  (offset

21H) and the record size field (offset 0EH)  of  the  FCB.

The default record size is 128 bytes.

- The current block field (offset 0CH)  and  current  record

field  (offset  20H)  are  updated  to  agree   with   the

relative-record field as a side effect of the function.

- The relative-record field of the FCB is not incremented by

this  func-  tion;  it  is  the  responsibility   of   the

application to update the FCB appropriately if  it  wishes

to write successive records. Compare with Int 21H Function

28H, which can write multiple records with one func-  tion

call  and  automatically  increments  the  relative-record

field.

- If a record is written beyond the current end of file, the

space between the old end of file and the  new  record  is

allocated but not initialized.

- [3.0+] If the program is running on a  network,  the  user

must have Write access rights to the directory  containing

the file to be written.

7.10.5. Int 21H Function 24H

Description:

Set Relative-Record Number

Sets the relative-record-number  field  of  a  file  control

block (FCB) to correspond  to  the  current  file  position  as

recorded in the opened FCB.

Input                       Output

------------------------    -----------------------------------

AH=24H                      AL is destroyed (other registers no

DS:DX=segment:offset of        affected)

previously opened     FCB relative-record field (offset

file control block       21H) updated

- This function is used when switching  from  sequential  to

random  I/O  within  a   file.   The   contents   of   the

relative-record field (offset 21H) are  derived  from  the

Interrupt 21H, Record Functions, FCB

record size (offset 0EH), current block (offset 0CH),  and

current record (offset 20H) fields  of  the  file  control

block.

- All four bytes of the FCB  relative-record  field  (offset

21H) should be initialized to  zero  before  calling  this

function.

7.10.6. Int 21H Function 24H

Description:

Set Relative-Record Number

Sets the relative-record-number  field  of  a  file  control

block (FCB) to correspond  to  the  current  file  position  as

recorded in the opened FCB.

Input                       Output

------------------------    -----------------------------------

AH=24H                      AL is destroyed (other registers no

DS:DX=segment:offset of        affected)

previously opened     FCB relative-record field (offset

file control block       21H) updated

- This function is used when switching  from  sequential  to

random  I/O  within  a   file.   The   contents   of   the

relative-record field (offset 21H) are  derived  from  the

record size (offset 0EH), current block (offset 0CH),  and

current record (offset 20H) fields  of  the  file  control

block.

- All four bytes of the FCB  relative-record  field  (offset

21H) should be initialized to  zero  before  calling  this

function.

7.10.7. Int 21H Function 27H

Description:

Random Block Read

Reads one or  more  sequential  records  from  a  file  into

memory, starting at a designated file location.

Input                           Output

-----------------------------   -------------------------------

AH=27H                          AL=00H  if all requested re-

CX=number of records to read                   cords read

DS:DX=segment:offset of prev-      01H  if end of file

iously opened file con-      02H  if segment wrap

trol block                   03H  if partial record read

at end of file

CX=actual number of records rea

Interrupt 21H, Record Functions, FCB

- The records are read  into  memory  at  the  current  disk

transfer area ad- dress, specified by the most recent call

to  Int  21H  Function  1AH.  It   is   the   programmer's

responsibility to ensure that this area  is  large  enough

for the group of records that will be transferred. If  the

size and location of the buffer are such  that  a  segment

overflow or wrapa- round would occur, the  function  fails

with a return code of 02H.

- The file location of the data to be read is determined  by

the combina- tion of  the  relative-record  field  (offset

21H) and the record size field (offset 0EH)  of  the  FCB.

The default record size is 128 bytes.

- After the disk transfer is performed,  the  current  block

(offset  0CH),   current   record   (offset   020H),   and

relative-record (offset 21H) fields of the FCB are updated

to point to the next record in the file.

- If a partial record is  read  at  the  end  of  file,  the

remainder of the record is padded with zeros.

- Compare with Int 21H Function 21H,  which  transfers  only

one record per function call and does not update  the  FCB

relative-record field.

- [3.0+] If the program is running on a  network,  the  user

must have Read access rights to the  directory  containing

the file to be read.

7.10.8. Int 21H Function 28H

Description:

Random Block Write

Writes one or more sequential records from memory to a file,

starting at a designated file location.

Input                            Output

------------------------------   -----------------------------

AH=28H                           AL=00H  if all requested

CX=number of records to write            records written

DS:DX=segment:offset of prev-       01H  if disk full

iously opened file con-       02H  if segment wrap

trol block                 CX=actual number of records

written

- The  records  are  written  (logically,  not   necessarily

physically) to  disk  from  memory  at  the  current  disk

transfer area address, specified by the most  recent  call

to Int 21H Function 1AH. If the size and location  of  the

buffer are such that  a  segment  overflow  or  wraparound

would occur, the function fails with a return code of 02H.

- The file location of the data to be written is  determined

by the combi- nation of the relative-record field  (offset

21H) and the record size field (offset 0EH)  of  the  FCB.

The default record size is 128 bytes.

- After the disk transfer is performed,  the  current  block

(offset  0CH),   current   record   (offset   020H),   and

relative-record (offset 21H) fields of the FCB are updated

to point to the next record in the file.

Interrupt 21H, Record Functions, FCB

- If this function is called with  CX  =  00H,  no  data  is

written to the disk but the file is extended or  truncated

to the length specified by combination of the record  size

(offset 0EH) and the relative-record (offset  21H)  fields

of the FCB.

- Compare with Int 21H Function 22H,  which  transfers  only

one record per function call and does not update  the  FCB

relative-record field.

- [3.0+] If the program is running on a  network,  the  user

must have Write access rights to the directory  containing

the file to be written.

7.11. Interrupt 21H, Time and Date Functions

Hex #  Dec #     Name

2AH     42       Get Date

2BH     43       Set Date

2CH     44       Get Time

2DH     45       Set Time

7.11.1. Int 21H Function 2AH

Description:

Get Date

Obtains the system day of the month, day of the week, month,

and year.

Input             Output

------            ---------------------------------------------

AH=2AH            CX=year (1980 through 2099)

DH=month (1 through 12)

DL=day (1 through 31)

Under MS-DOS versions 1.1 and later

AL=day of the week (0=Sunday, 1=Monday, etc.)

- This function's  register  format  is  the  same  as  that

required for Int 21H Function 2BH (Set Date).

- This function can be used together with Int  21H  Function

2BH to find the day of the week for an arbitrary date. The

current date is  first  obtained  with  Function  2AH  and

saved. The date of interest is then set with Function 2BH,

and the day of the week for that date is obtained  with  a

subsequent call to Function 2AH. Finally, the current date

is restored with an additional call to Function 2BH, using

the values ob- tained with the original Function 2AH call.

Interrupt 21H, Time and Date Functions

7.11.2. Int 21H Function 2BH

Description:

Set Date

Initializes the system clock driver to a specific date.  The

system time is not affected.

Input                         Output

---------------------------   ---------------------------------

AH=2BH                        AL=00H  if date set successfully

CX=year (1980 through 2099)      FFH  if date not valid (ignore

DH=month (1 through 12)

DL=day (1 through 31)

This function's register format is the same as that required

for Int 21H Function 2AH (Get Date).

7.11.3. Int 21H Function 2CH

Description:

Get Time

Obtains the time of day  from  the  system  real-time  clock

driver, con- verted to hours, minutes, seconds, and  hundredths

of seconds.

Input             Output

------            ---------------------------------------

AH=2CH            CH=hours (0 through 23)

CL=minutes (0 through 59)

DH=seconds (0 through 59)

DL=hundredths of seconds (0 through 99)

- This function's  register  format  is  the  same  as  that

required for Int 21H Function 2DH (Set Time).

- On most IBM PC-compatible  systems,  the  real-time  clock

does  not  have  a  resolution  of  single  hundredths  of

seconds. On such machines, the  values  returned  by  this

function in register DL are discontinuous.

7.11.4. Int 21H Function 2DH

Description:

Set Time

Initializes the system real-time clock to a specified  hour,

minute, second, and hundredth of second. The system date is not

affected.

Interrupt 21H, Time and Date Functions

Input                       Output

-------------------------   -----------------------------------

AH=2DH                      AL=00H  if time set successfully

CH=hours (0 through 23)        FFH  if time not valid (ignored)

CL=minutes (0 through 59)

DH=seconds (0 through 59)

DL=hundredths of seconds

(0 through 99)

This function's register format is the same as that required

for Int 21H Function 2CH (Get Time).

Miscellaneous DOS Interrupts

8. Miscellaneous DOS Interrupts

Hex #  Dec #     Name

20H     32       Terminate Program

25H     37       Read Disk Sector (ignore logical structure)

26H     38       Write Disk Sector (ignore logical structure)

27H     39       Terminate but Stay Resident

2FH     47       Print Spool Control (DOS3 only)

8.1. Interrupt 20H

Description:

Terminate Process

Terminates the current  process.  This  is  one  of  several

methods that a program can use to perform a final exit.

Input                                                   Output

------------------                                      -------

CS=segment address of program segment prefix            Nothing

If  the  program  is  returning  to   COMMAND.COM,   control

transfers to the resident portion, and the transient portion is

reloaded if necessary. If a batch file is in progress, the next

line of the file  is  fetched  and  interpreted;  otherwise,  a

prompt is issued for the next user command.

MS-DOS then takes the following actions:

1) All memory belonging to the process is released.

2) File buffers are flushed, and any open handles for files

or devices owned by the process are closed.

3) The termination handler vector  (Int  22H)  is  restored

from PSP:000AH.

4) The CTRL+C handler vector (Int  23H)  is  restored  from

PSP:000EH.

5) The critical-error handler vector (Int 24H) is  restored

from PSP:0012H (MS-DOS versions 2.0 and later).

6) Control is transferred to the termination handler.

7) Any files that have been written to using FCBs should be

closed before performing this exit call; otherwise, data

may be lost.

8) Other methods of performing a final exit are:

- Int 21H Function 00H

- Int 21H Function 31H

- Int 21H Function 4CH

- Int 27H

9) [3] If the program is running on a  network,  it  should

remove all locks it has placed on  file  regions  before

terminating.

10) Int 21H Functions 31H and 4CH are the  preferred  method

for termination because they allow a return code  to  be

passed to the parent process.

Miscellaneous DOS Interrupts

8.2. Interrupt 25H

Description:

Absolute Disk Read

Provides a direct linkage to the MS-DOS BIOS module to  read

data from a logical disk sector into memory.

Input                                  Output

------------------------------         ------------------------

For access to partitions               If function successful

<= 32 MB                              Carry flag=set

AL=drive number (0=A, 1=B,             If function unsuccessful

etc.)                                 Carry flag=set

CX=number of sectors to read           AX=error code (see Notes

DX=starting sector number

DS:BX=segment:offset of buffer

For access to partitions

> 32 MB (MS-DOS 4.0 and later)

AL=drive number (0=A, 1=B, etc.)

CX=-1

DS:BX=segment:offset of para-

meter block (see Notes)

- All  registers  except  the  segment  registers   may   be

destroyed.

- When this  function  returns,  the  CPU  flags  originally

pushed on the stack by the INT 25H instruction  are  still

on the stack. The stack must be cleared by a POPF  or  ADD

SP,2 to prevent uncontrolled  stack  growth  and  to  make

accessible any other values that were pushed on the  stack

before the call to INT 25H.

- Logical sector numbers are obtained by numbering each disk

sector sequentially from track 0, head 0,  sector  1,  and

continuing until the last sector on the disk  is  counted.

The head number is incremented before  the  track  number.

Logically adjacent sectors may not be physically adjacent,

due to interleaving that occurs at the device-driver level

for some disk types.

- The error code is interpreted as follows: The  lower  byte

(AL) is the same error code that is returned in the  lower

byte of DI when an Int 24H is issued. The upper byte  (AH)

contains:

80H

if attachment failed to respond

40H

if seek operation failed

20H

if controller failed

10H

data error (bad CRC)

08H

if direct memory access (DMA) failure

04H

if requested sector not found

02H

if bad address mark

01H

if bad command

Miscellaneous DOS Interrupts

- [4.0+] When accessing partitions larger than 32  MB  under

MS-DOS version 4, this function  uses  a  parameter  block

with the following format:

Bytes       Description

-------     -------------------------

00H-03H     32-bit sector number

04H-05H     number of sectors to read

06H-07H     offset of buffer

08H-09H     segment of buffer

8.3. Interrupt 26H

Description:

Absolute Disk Write

Provides a direct linkage to the MS-DOS BIOS module to write

data from memory to a logical disk sector.

Input                                 Output

------------------------------        -------------------------

For access to partitions              If function successful

<= 32 MB                               Carry flag=clear

AL=drive number (0=A, 1=B,            If function unsuccessful

etc.)                                  Carry flag=set

CX=number of sectors to write         AX=error code (see Notes)

DX=starting sector number

DS:BX=segment:offset of buffer

For access to partitions

> 32 MB (MS-DOS 4.0 and later)

AL=drive number (0=A, 1=B,

etc.)

CX=-1

DS:BX=segment:offset of para-

meter block (see Notes)

- All  registers  except  the  segment  registers   may   be

destroyed.

- When this  function  returns,  the  CPU  flags  originally

pushed onto the stack by the INT 26H instruction are still

on the stack. The stack must be cleared by a POPF  or  ADD

SP,2 to prevent uncontrolled  stack  growth  and  to  make

accessible any other values that were pushed on the  stack

before the call to INT 26H.

- Logical sector numbers are obtained by numbering each disk

sector sequentially from track 0, head 0,  sector  1,  and

continuing until the last sector on the disk  is  counted.

The head number is incremented before  the  track  number.

Logically adjacent sectors may not be physically adjacent,

due to interleaving that occurs at the device-driver level

for some disk types.

- The error code is interpreted as follows: The  lower  byte

(AL) is the same error code that is returned in the  lower

byte of DI when an Int 24H is issued. The upper byte  (AH)

Miscellaneous DOS Interrupts

contains:

80H

if attachment failed to respond

40H if seek operation failed

20H if controller failed

10H if data error (bad CRC)

08H if direct memory access (DMA) failure

04H if requested sector not found

03H if write-protect fault

02H if bad address mark

01H if bad command

- [4.0+] When accessing partitions larger than 32  MB  under

MS-DOS version 4, this function  uses  a  parameter  block

with the following format:

Bytes       Description

-------     -------------------------

00H-03H     32-bit sector number

04H-05H     number of sectors to read

06H-07H     offset of buffer

08H-09H     segment of buffer

8.4. Interrupt 27H

Description:

Terminate and Stay Resident

Terminates execution of the currently executing program, but

reserves part or all of its memory  so  that  it  will  not  be

overlaid by the next transient program to be loaded.

Input                                            Output

-------------------------------------            -------

DX=offset of the last byte plus one              Nothing

(relative to the program segment

prefix) of program to be protected

CS=segment of program segment prefix

1) MS-DOS then takes the following actions:

- File buffers are flushed and any open  handles  for

files or devices owned by the process are closed.

-  The  termination  handler  vector  (Int  22H)   is

restored from PSP:000AH.

- The CTRL+C handler vector  (Int  23H)  is  restored

from PSP:000EH.

- The critical-error  handler  vector  (Int  24H)  is

restored from PSP:0012H.

- Control is transferred to the termination  handler.

2) If the program  is  returning  to  COMMAND.COM,  control

transfers to the  resident  portion  and  the  transient

portion is reloaded if necessary. If a batch file is  in

progress, the next line  of  the  file  is  fetched  and

interpreted; otherwise a prompt is issued for  the  next

user command.

Miscellaneous DOS Interrupts

3) This  function  call  is   typically   used   to   allow

user-written utilities, drivers, or  interrupt  handlers

to be loaded as ordinary .COM or .EXE programs, and then

remain resident. Subsequent entrance to the code is  via

a hardware or software interrupt.

4) This  function  attempts  to  set  the  initial   memory

allocation block to the length  in  bytes  specified  in

register DX. If other memory blocks have been  requested

by the application via Int 21H Function 48H,  they  will

not be released by this function.

5) Other methods of performing a final exit are:

- Int 20H

- Int 21H Function 00H

- Int 21H Function 31H

- Int 21H Function 4CH

6) This function should not be called by .EXE programs that

are loaded at the high end of the transient program area

(i.e., linked with the /HIGH switch)  because  doing  so

reserves  the  memory  that  is  normally  used  by  the

transient part of COMMAND.COM. If COMMAND.COM cannot  be

reloaded, the system will fail.

7) This function does not work correctly when  DX  contains

values in the range 0FFF1H-0FFFFH. In this case,  MS-DOS

discards the high bit of the value in DX,  resulting  in

the reservation of 32 KB less memory than was  requested

by the program.

8) Int 21H Function 31H should be  used  in  preference  to

this function because it supports return  codes,  allows

larger amounts of memory to be reserved,  and  does  not

require CS to contain the segment of the program segment

prefix.

9) [3.0+] If the program is running on a network, it should

remove all locks it has placed on  file  regions  before

terminating.

8.5. Interrupt 2FH

Description:

Multiplex Interrupt

Provides a  general-purpose  avenue  of  communication  with

another process or with MS-DOS extensions, such  as  the  print

spooler, ASSIGN, SHARE, and APPEND.  The  multiplex  number  in

register AH specifies the process being communicated with.  The

range 00H-BFH is reserved for MS-DOS; applications may use  the

range C0H-FFH.

BIOS Interrupts

9. BIOS Interrupts

Interrupt    Function        Description

10H                          Video driver, all monitors

10H                          Video driver, graphics monitors

10H          10H             Video driver, colors

10H          11H             Video driver, fonts

10H          12H             Video configuration

13H                          Disk driver (input/output)

13H                          Disk driver (diagnostics)

14H                          Serial communications port driver

15H                          I/O subsystem

15H          C2H             I/O subsystem, mouse pointer (PS/2

16H                          Keyboard driver

17H                          Parallel port printer driver

1AH                          Clock

miscellaneous

9.1. Interrupt 10H (All Monitors)

Interrupt  Function   Name

10H        00H        Set Video Mode

10H        01H        Set Cursor Type

10H        02H        Set Cursor Position

10H        03H        Get Cursor Position

10H        04H        Get Light Pen Position

10H        05H        Set Display Page

10H        06H        Initialize or Scroll Up Window

10H        07H        Initialize or Scroll Down Window

10H        08H        Read Character and Attribute at Cursor

10H        09H        Write Character and Attribute at Cursor

10H        0AH        Write Character at Cursor

10H        0EH        Write Character in Teletype Mode

10H        0FH        Get Video Mode

10H        13H        Write String in Teletype Mode

9.1.1. Int 10H Function 00H

Description:

Set Video Mode  [MDA]  [CGA]  [PCjr]  [EGA]  [MCGA]

[VGA]

Selects the current video display  mode.  Also  selects  the

active video controller, if more than one is present.

Input                                Output

-------------------------            -------

AH=00H                               Nothing

AL=video mode (see Notes)

Interrupt 10H (All Monitors)

The video modes applicable to the various IBM machine models

and video adapters are as follows:

Mode    Resolution            Colors  Text/Graphics

----    -----------------     ------  -------------

00H     40-by-25

color burst off      16      text

01H     40-by-25              16      text

02H     80-by-25

color burst off      16      text

03H     80-by-25              16      text

04H     320-by-200             4      graphics

05H     320-by-200

color burst off       4      graphics

06H     640-by-200             2      graphics

07H     80-by-25               2*     text

08H     160-by-200            16      graphics

09H     320-by-200            16      graphics

0AH     640-by-200             4      graphics

0BH     reserved

0CH     reserved

0DH     320-by-200            16      graphics

0EH     640-by-200            16      graphics

0FH     640-by-350             2*     graphics

10H     640-by-350             4**    graphics

10H     640-by-350            16***   graphics

11H     640-by-480             2      graphics

12H     640-by-480            16      graphics

13H     320-by-200           256      graphics

*   Monochrome monitor only.

**  EGA with 64 KB of RAM.

*** EGA with 128 KB or more of RAM.

The presence or absence of color burst is  only  significant

when a composite monitor is being used. For RGB monitors, there

is no functional difference between modes 00H and 01H or  modes

02H and 03H. On the CGA, two palettes are available in mode 04H

and one in mode 05H.

On the PC/AT, PCjr, and PS/2, if bit 7 of  AL  is  set,  the

display buffer is not cleared when a new mode is  selected.  On

the PC or PC/XT, this capability is available only when an  EGA

(which has its own ROM BIOS) is installed.

9.1.2. GetVidConfig

;* GetVidConfig - Determines current video configuration and

;* initializes the vconfig structure.

;*

;* Shows: BIOS Interrupt - 10h, Function 0 (Set Video Mode)

;*                         10h, Function 0Fh (Get Video Mode)

;*                         10h, Function 1Ah (Get or Set Displa

;*                              Combination Code)

;*

;* Uses:    vconfig - Video configuration structure,

;*          declared in the DEMO.INC include file.

;*

;* Params:  None

Interrupt 10H (All Monitors)

;*

;* Return:  None

GetVidConfig PROC

mov     ax, 1A00h  ; Request video info for VGA

int     10h        ; Get Display Combination Code

chkVGA: cmp     al, 1Ah    ; Is VGA or MCGA present?

jne     chkEGA     ; No?  Then check for EGA

cmp     bl, 2      ; If VGA exists as secondary adapter

je      isCGA      ;   check for CGA and mono as primar

jb      isMONO

cmp     bl, 5      ; If EGA is primary, do normal

jbe     chkEGA     ;   EGA checking

chkMCGA:mov     vconfig.adapter, MCGA   ; Yes?  Assume MCGA

mov     vconfig.display, COLOR

cmp     bl, 8                ; Correct assumption?

ja      gmode                ; Yes?  Continue

isVGA:  mov     vconfig.adapter, VGA ; Assume it's VGA color

je      gmode                ; Yes?  Continue

mov     vconfig.display, MONO; No?  Must be VGA mono

jmp     SHORT gmode          ; Finished with VGA, so ju

chkEGA: mov     ah, 12h    ; Call EGA status function

mov     bl, 10h

sub     cx, cx     ; Clear status bits

int     10h        ; Get Configuration Information

jcxz    chkCGA     ; If CX is unchanged, not EGA

; Set structure fields for EGA

isEGA:  mov     vconfig.adapter, EGA

mov     vconfig.display, MONO   ; Assume EGA mono

or      bh, bh     ; Correct assumption?

jnz     gmode      ; Yes?  Continue

mov     vconfig.display, COLOR  ; No?  Must be EGA colo

jmp     SHORT gmode    ; Finished with EGA, so jump

chkCGA: int     11h            ; Get equipment list

and     al, 30h        ; If bits 4-5 set, monochrome

cmp     al, 30h        ; Monochrome text mode?

je      isMONO         ; Yes?  Continue

isCGA:  mov     vconfig.adapter, CGA    ; No?  Must be CGA

mov     vconfig.display, COLOR

mov     vconfig.CGAvalue, 29h ; Value for CGA 80x25 tex

jmp     SHORT gmode           ;   color, blink enable

isMONO: mov     vconfig.adapter, MDA    ; Set MONO

mov     vconfig.display, MONO

gmode:  mov     ah, 0Fh

int     10h               ; Get Video Mode

mov     vconfig.mode, al  ; Record mode

mov     vconfig.dpage, bh ;   and current page

cmp     al, 7             ; Monochrome text mode?

je      @F                ; Yes?  Continue

cmp     al, 3             ; Color text mode?

je      @F                ; Yes?  Continue

cmp     al, 2             ; Black/white 80-col mode?

je      @F                ; Yes?  Continue

Interrupt 10H (All Monitors)

mov     ax, 0003h         ; If not 80-col text mode,

mov     vconfig.mode, al  ;   request Function 0, mode

int     10h               ; Set Video Mode to 80-col

@@:     mov     al, vconfig.display ; Multiply display value

cbw                       ;   (which is either 0 or 1)

mov     bx, 800h          ;   by 800h, then add to B000

mul     bx                ;   for segment address of

add     ax, 0B000h        ;   video buffer

add     ah, vconfig.dpage ; Adding display page gives

mov     vconfig.sgmnt, ax ;   address of current page

mov     vconfig.rows, 24  ; Assume bottom row # = 24

cmp     vconfig.adapter, EGA    ; EGA or VGA?

jl      exit              ; No?  Exit

mov     ax, 1130h         ; Yes?  Request character inf

sub     bh, bh            ; Set BH to valid value

push    bp                ; BP will change, so save it

int     10h               ; Get number of rows/screen

mov     vconfig.rows, dl  ; Keep in structure

pop     bp                ; Restore BP

exit:   ret

GetVidConfig ENDP

9.1.3. Int 10H Function 01H

Description:

Set Cursor Type [MDA]  [CGA]  [PCjr]  [EGA]  [MCGA]

[VGA]

Selects the starting  and  ending  lines  for  the  blinking

hardware cursor in text display modes.

Input                                           Output

------------------------------------            -------

AH=01H                                          Nothing

CH bits 0-4=starting line for cursor

CL bits 0-4=ending line for cursor

In text display modes, the video hardware causes the  cursor

to blink, and the blink cannot be disabled. In graphics  modes,

the hardware cursor is not available.

The default values set by the ROM BIOS are:

Display                     Start         End

-------------------         -----         ---

monochrome mode 07H         11            12

text modes 00H-03H          6             7

On the EGA, MCGA, and VGA in text  modes  00H-03H,  the  ROM

BIOS  accepts  cursor  start  and  end  values  as  though  the

character  cell  were  8-by-8,  and  remaps   the   values   as

appropriate  for  the  true  character  cell  dimensions.  This

mapping is called cursor emulation.

You can turn off the cursor in several  ways.  On  the  MDA,

Interrupt 10H (All Monitors)

CGA, and VGA, setting register CH =20H  causes  the  cursor  to

disappear. Techniques that involve setting illegal starting and

ending  lines  for  the  current  display  mode  tend   to   be

unreliable. An alternative method is to position the cursor  to

a nondisplayable address, such as (x,y)=(0,25).

9.1.4. SetCurSize

;* SetCurSize - Sets cursor size.

;*

;* Shows:   BIOS Interrupt - 10h, Function 1 (Set Cursor Type)

;*

;* Params:  scan1 - Starting scan line

;*          scan2 - Ending scan line

;*

;* Return:  None

SetCurSize PROC \

scan1:WORD, scan2:WORD

mov     cx, scan2          ; CL = ending scan line

mov     ch, BYTE PTR scan1 ; CH = starting scan line

mov     ah, 1              ; Function 1

int     10h                ; Set Cursor Type

ret

SetCurSize ENDP

9.1.5. Int 10H Function 02H

Description: Set  Cursor  Position  [MDA]  [CGA]  [PCjr]  [EGA]

[MCGA] [VGA]

Positions the cursor on the display, using text coordinates.

Input                              Output

-----------------------            -------

AH=02H                             Nothing

BH=page

DH=row (y coordinate)

DL=column (x coordinate)

A separate cursor is maintained for each display  page,  and

each can be set independently with this function regardless  of

the currently active page.  The  number  of  available  display

pages depends on the video adapter and  current  display  mode.

See Int 10H Function 05H.

Text coordinates (x,y)=(0,0) are the  upper-left  corner  of

the screen.

The maximum value for each text coordinate  depends  on  the

video adapter and current display mode, as follows:

Interrupt 10H (All Monitors)

Mode    Maximum x    Maximum y

----    ---------    ---------

00H     39           24

01H     39           24

02H     79           24

03H     79           24

04H     39           24

05H     39           24

06H     79           24

07H     79           24

08H     19           24

09H     39           24

0AH     79           24

0BH     reserved

0CH     reserved

0DH     39           24

0EH     79           24

0FH     79           24

10H     79           24

11H     79           29

12H     79           29

13H     39           24

9.1.6. SetCurPos

;* SetCurPos - Sets cursor position.

;*

;* Shows:   BIOS Interrupt - 10h, Function 2

;*           (Set Cursor Position)

;*

;* Uses:    vconfig - Video configuration structure,

;*          declared in the DEMO.INC include file.

;*          The structure must first be initialized

;*          by calling the GetVidConfig procedure.

;*

;* Params:  row - Target row

;*          col - Target column

;*

;* Return:  None

SetCurPos PROC \

row:WORD, col:WORD

mov     dx, col           ; DL = column

mov     dh, BYTE PTR row  ; DH = row

mov     ah, 2             ; Function 2

mov     bh, vconfig.dpage ; Current page

int     10h               ; Set Cursor Position

ret

SetCurPos ENDP

Interrupt 10H (All Monitors)

9.1.7. Int 10H Function 03H

Description: Get  Cursor  Position  [MDA]  [CGA]  [PCjr]  [EGA]

[MCGA] [VGA]

Obtains the current position of the cursor on  the  display,

in text coordinates.

Input              Output

-------            ------------------------------

AH=03H             CH=starting line for cursor

BH=page            CL=ending line for cursor

DH=row ( y coordinate)

DL=column (x coordinate)

A separate cursor is maintained for each display  page,  and

each  can  be  inspected  independently  with   this   function

regardless  of  the  currently  active  page.  The  number   of

available display  pages  depends  on  the  video  adapter  and

current display mode. See Int 10H Function 05H.

9.1.8. Int 10H Function 04H

Description:

Get Light Pen Position [CGA] [PCjr] [EGA]

Obtains the current status and position of the light pen.

Input     Output

------    ---------------------------------------------------

AH=04H    AH=00H if light pen not down/not triggered

=01H if light pen down/triggered

BX=pixel column (graphics x coordinate)

CH=pixel row (graphics y coordinate, modes 04H-06H)

CX=pixel row (graphics y coordinate, modes 0DH-10H)

DH=character row (text y coordinate)

DL=character column (text x coordinate)

The range of text and graphics coordinates returned by  this

function depends on the current display mode.

On the  CGA,  the  graphics  coordinates  returned  by  this

function are not continuous.  The  y  coordinate  is  always  a

multiple of two; the x coordinate is either a multiple of  four

(for 320-by-200 graphics modes) or a  multiple  of  eight  (for

640-by-200 graphics modes).

Careful selection of background  and  foreground  colors  is

necessary to obtain maximum  sensitivity  from  the  light  pen

across the full screen width.

Interrupt 10H (All Monitors)

9.1.9. Int 10H Function 05H

Description: Set Display Page [CGA] [PCjr] [EGA] [MCGA] [VGA]

Selects the active display page for the video display.

Input                               Output

----------------------------------  --------------------

For CGA, EGA, MCGA, VGA             If CGA, EGA, MCGA, or

AH=05H                              VGA adapter: nothing

AL=page

0-7 for modes 00H and 01H (CGA,   If PCjr and if function

EGA, MCGA, VGA)               called with AL=80H-83H:

0-3 for modes 02H and 03H (CGA)

0-7 for modes 02H and 03H (EGA,   BH=CRT page register

MCGA, VGA)

0-7 for mode 07H (EGA, VGA)       BL=CPU page register

0-7 for mode 0DH (EGA, VGA)

0-3 for mode 0EH (EGA, VGA)

0-1 for mode 0FH (EGA, VGA)

0-1 or mode 10H (EGA, VGA)

For PCjr only

AH=05H

AL=subfunction

80H =read CRT/CPU page registers

81H =set CPU page register

82H =set CRT page register

83H =set both CPU and CRT page registers

BH=CRT page (Subfunctions 82H and 83H)

BL=CPU page (Subfunctions 81H and 83H)

9.1.10. Int 10H Function 06H

Description:

Initialize or Scroll Up Window [MDA]  [CGA]  [PCjr]

[EGA] [MCGA] [VGA]

Initializes a specified window of the display to ASCII blank

characters with a given attribute, or scrolls up  the  contents

of a window by a specified number of lines.

Input                                          Output

---------------------------------------------  -------

AH=06H                                         Nothing

AL=number of lines to scroll (if zero, entire

window is blanked)

BH=attribute to be used for blanked area

CH=y coordinate, upper-left corner of window

CL=x coordinate, upper-left corner of window

DH=y coordinate, lower-right corner of window

DL=x coordinate, lower-right corner of window

In video modes that support multiple  pages,  this  function

affects only the page currently being displayed.

If AL contains a value other than 00H, the area  within  the

Interrupt 10H (All Monitors)

specified window is scrolled up  by  the  requested  number  of

lines. Text that is scrolled beyond the top of  the  window  is

lost. The new lines that appear at the bottom of the window are

filled with ASCII blanks carrying the  attribute  specified  by

register BH.

To scroll down  the  contents  of  a  window,  see  Int  10H

Function 07H.

9.1.11. Int 10H Function 07H

Description: Initialize  or  Scroll  Down  Window  [MDA]  [CGA]

[PCjr] [EGA] [MCGA] [VGA]

Initializes a specified window of the display to ASCII blank

characters with a given attribute, or scrolls down the contents

of a window by a specified number of lines.

Input                                           Output

----------------------------------------------  -------

AH=07H                                          Nothing

AL=number of lines to scroll (if zero, entire

window is blanked)

BH=attribute to be used for blanked area

CH=y coordinate, upper-left corner of window

CL=x coordinate, upper-left corner of window

DH=y coordinate, lower-right corner of window

DL=x coordinate, lower-right corner of window

In video modes that support multiple  pages,  this  function

affects only the page currently being displayed.

If AL contains a value other than 00H, the area  within  the

specified window is scrolled down by the  requested  number  of

lines. Text that is scrolled beyond the bottom of the window is

lost. The new lines that appear at the top of  the  window  are

filled with ASCII blanks carrying the  attribute  specified  by

register BH.

To scroll up the contents of a window, see Int 10H  Function

06H.

9.1.12. Int 10H Function 08H

Description: Read Character and Attribute at Cursor [MDA] [CGA]

[PCjr] [EGA] [MCGA] [VGA]

Obtains the ASCII character and its attribute at the current

cursor position for the specified display page.

Input              Output

-------            ------------------------------

AH=08H             AH=attribute

BH=page            AL=character

In video modes that support multiple pages,  characters  and

their attributes can be read from any page, regardless  of  the

Interrupt 10H (All Monitors)

page currently being displayed.

9.1.13. ReadCharAttr

;* ReadCharAttr - Reads character and display attribute

;*                      at cursor location.

;*

;* Shows:   BIOS Interrupt - 10h, Function 8

;*          (Read Character and Attribute at Cursor)

;*

;* Uses:    vconfig - Video configuration structure,

;*          declared in the DEMO.INC include file.

;*          The structure must first be initialized

;*          by calling the GetVidConfig procedure.

;*

;* Params:  attr - Pointer to short integer for display attribu

;*

;* Return:  Short integer with ASCII value of character

ReadCharAttr PROC \

USES di, \

attr:PTR WORD

mov     ah, 8      ; Function 8

mov     bh, vconfig.dpage ; Current page

int     10h        ; Read Character and Attribute

sub     bh, bh

mov     bl, ah        ; BX = attribute

cbw                   ; AX = character

LoadPtr es, di, attr  ; ES:DI = pointer to int

mov     es:[di], bx   ; Copy attribute

ret

ReadCharAttr ENDP

9.1.14. Int 10H Function 09H

Description: Write Character  and  Attribute  at  Cursor  [MDA]

[CGA] [PCjr] [EGA] [MCGA] [VGA]

Writes an ASCII character and its attribute to  the  display

at the current cursor position.

Input                                                 Output

----------------------------------------------------  -------

AH=09H                                                Nothing

AL=character

BH=page

BL=attribute (text modes) or color (graphics modes)

CX=count of characters to write (replication factor)

In graphics modes, the replication factor in CX  produces  a

valid result only for the current row. If more  characters  are

written than there are remaining columns in  the  current  row,

the result is unpredictable.

Interrupt 10H (All Monitors)

All values of AL result in some  sort  of  display;  control

characters, including bell,  backspace,  carriage  return,  and

line feed, are not recognized as special characters and do  not

affect the cursor position.

After a character is  written,  the  cursor  must  be  moved

explicitly with Int 10H Function 02H to the next position.

To write a character without changing the attribute  at  the

current cursor position, use Int 10H Function 0AH.

If this function is used to  write  characters  in  graphics

mode and bit 7  of  BL  is  set  (1),  the  character  will  be

exclusive-OR'd (XOR) with the current  display  contents.  This

feature can be used to write characters and then "erase"  them.

For the CGA and PCjr in  graphics  modes  04H-06H,  the  bit

patterns for character codes 80H-FFH are obtained from a  table

whose address is stored in the vector for Int 1FH. On the PCjr,

the address of the table for character codes 00-7FH  is  stored

in the vector for Int 44H. Alternative character  sets  can  be

installed by loading them into memory and updating this vector.

For the EGA, MCGA, and VGA in graphics modes, the address of

the character definition table is stored in the vector for  Int

43H. See Int 10H Function 11H.

9.1.15. Int 10H Function 0AH

Description: Write Character at Cursor [MDA] [CGA] [PCjr] [EGA]

[MCGA] [VGA]

Writes an ASCII character to  the  display  at  the  current

cursor position. The character receives the  attribute  of  the

previous character displayed at the same position.

Input                                                 Output

----------------------------------------------------  -------

AH=0AH                                                Nothing

AL=character

BH=page

BL=color (graphics modes, PCjr only)

CX=count of characters to write (replication factor)

In graphics modes, the replication factor in CX  produces  a

valid result only for the current row. If more  characters  are

written than there are remaining columns in  the  current  row,

the result is unpredictable.

All values of AL result in some  sort  of  display;  control

characters, including bell,  backspace,  carriage  return,  and

line feed, are not recognized as special characters and do  not

affect the cursor position.

After a character is  written,  the  cursor  must  be  moved

explicitly with Int 10H Function 02H to the next position.

To write a character and attribute  at  the  current  cursor

position, use Int 10H Function 09H.

If this function is used to  write  characters  in  graphics

mode and bit 7  of  BL  is  set  (1),  the  character  will  be

exclusive-OR'd (XOR) with the current  display  contents.  This

feature can be used to write characters and then "erase"  them.

For the CGA and PCjr in  graphics  modes  04H-06H,  the  bit

patterns for character codes 80H-FFH are obtained from a  table

whose address is stored in the vector for Int 1FH. On the PCjr,

the address of the table for character codes 00-7FH  is  stored

in the vector for Int 44H. Alternative character  sets  can  be

Interrupt 10H (All Monitors)

installed by loading them into memory and updating this vector.

For the EGA, MCGA, and VGA in graphics modes, the address of

the character definition table is stored in the vector for  Int

43H. See Int 10H Function 11H.

9.1.16. Int 10H Function 0EH

Description:

Write Character in Teletype Mode [MDA] [CGA] [PCjr]

[EGA] [MCGA] [VGA]

Writes an ASCII character to  the  display  at  the  current

cursor position, using the  specified  color  (if  in  graphics

modes), and then increments the cursor position  appropriately.

Input                                           Output

------------------------------------            -------

AH=0EH                                          Nothing

AL=character

BH=page

BL=foreground color (graphics modes)

The special ASCII codes for  bell  (07H),  backspace  (08H),

carriage return (0DH), and line feed (0AH) are recognized,  and

the appropriate action  is  taken.  All  other  characters  are

written to the display (even if they are  control  characters),

and the cursor is moved to the next position.

In video modes that support multiple pages,  characters  can

be written to any page, regardless of the page currently  being

displayed.

Line wrapping and scrolling are provided. If the  cursor  is

at the end of a line, it is moved to the beginning of the  next

line. If the cursor reaches the end of the  last  line  on  the

screen, the screen is scrolled up by one line and the cursor is

placed at the beginning of a new blank line. The attribute  for

the entire new line is taken from the last character  that  was

written on the preceding line.

The default MS-DOS console driver (CON) uses  this  function

to write text to the screen. You cannot use  this  function  to

specify the attribute of a character. One method of  writing  a

character to the screen with a specific attribute is  to  first

write an ASCII blank (20H) with the desired  attribute  at  the

current cursor location using Int 10H Function 09H, then  write

the actual character with Int 10H Function 0EH. This technique,

although somewhat clumsy,  does  not  require  the  program  to

explicitly handle line wrapping and scrolling.

See also Int 10H Function 13H.

9.1.17. Int 10H Function 0FH

Description: Get Video Mode [MDA]  [CGA]  [PCjr]  [EGA]  [MCGA]

[VGA]

Obtains  the  current  display  mode  of  the  active  video

Interrupt 10H (All Monitors)

controller.

Input             Output

------            ------------------------------------------

AH=0FH            AH=number of character columns on screen

AL=display mode (see Int 10H Function 00H)

BH=active display page

This function can be  called  to  obtain  the  screen  width

before clearing the screen with Int 10H Functions 06H or 07H.

9.1.18. Int 10H Function 13H

Description: Write String in Teletype Mode [MDA]  [CGA]  [PCjr]

[EGA] [MCGA] [VGA]

Transfers a string to the video  buffer  for  the  currently

active display, starting at the specified position.

Input                                         Output

--------------------------------------------  -------

AH=13H                                        Nothing

AL=write mode

0  attribute in BL;

string contains character codes only;

and cursor position is not updated

after write

1  attribute in BL; string contains

character codes only; and cursor

position is updated after write

2  string contains alternating character

codes and attribute bytes; and cursor

position is not updated after write

3  string contains alternating character

codes and attribute bytes; and cursor

position is updated after write

BH=page

BL=attribute, if AL=00H or 01H

CX=length of character string

DH=y coordinate (row)

DL=x coordinate (column)

ES:BP=segment:offset of string

This function is not available on the  original  IBM  PC  or

PC/XT unless an EGA video adapter (which contains its  own  ROM

BIOS) is installed.

This function can be thought of as an extension to  Int  10H

Function 0EH. The  control  characters  bell  (07H),  backspace

(08H),  line  feed  (0AH),  and  carriage  return   (0DH)   are

recognized and handled appropriately.

Interrupt 10H (Graphics Monitors)

9.2. Interrupt 10H (Graphics Monitors)

Interrupt  Function         Name

10H        0BH              Set Palette, Background, or Border

10H        0CH              Write Graphics Pixel

10H        0DH              Read Graphics Pixel

10H        10H              Color (summary screen)

10H        11H              Fonts (summary screen)

10H        12H              Video Configuration (summary screen

10H        1AH              Get or Set Display Combination Code

10H        1BH              Get Functionality/State Information

10H        1CH              Save or Restore Video State

9.2.1. Int 10H Function 0BH

Description: Set Palette, Background, or  Border  [CGA]  [PCjr]

[EGA] [MCGA] [VGA]

Selects a palette, background, or border color.

Input                                               Output

--------------------------------------------------  -------

To set the background color and border color for    Nothing

graphics modes or the border color for text modes

AH=0BH

BH=00H

BL=color

To select the palette (320-by-200 4-color graphics modes)

AH=0BH

BH=01H

BL=palette (see Notes)

In text modes, this function selects only the border  color.

The background color of each individual character is controlled

by the upper 4 bits of that character's attribute byte.

On the CGA and EGA,  this  function  is  valid  for  palette

selection only in 320-by-200 4-color graphics modes.

In 320-by-200 4-color graphics modes, if register  BH  =01H,

the following palettes may be selected:

Palette  Pixel value  Color

-------  -----------  ------------------

0        0            same as background

1            green

2            red

3            brown or yellow

1        0            same as background

1            cyan

2            magenta

3            white

On  the  CGA  in  640-by-200  2-color  graphics  mode,   the

background color selected with this function actually  controls

the display color for non-zero pixels; zero pixels  are  always

displayed as black.

Interrupt 10H (Graphics Monitors)

On the PCjr in 640-by-200 2-color graphics mode, if BH  =00H

and bit 0 of register BL is cleared, pixel value 1 is displayed

as white; if bit 0 is set, pixel value 1 is displayed as black.

See also Int 10H Function 10H, which  is  used  for  palette

programming on the PCjr, EGA, MCGA, and VGA.

9.2.2. Int 10H Function 0CH

Description:

Write Graphics  Pixel  [CGA]  [PCjr]  [EGA]  [MCGA]

[VGA]

Draws a point on  the  display  at  the  specified  graphics

coordinates.

Input                                        Output

---------------------------------            -------

AH=0CH                                       Nothing

AL=pixel value

BH=page

CX=column (graphics x coordinate)

DX=row (graphics y coordinate)

The range  of  valid  pixel  values  and  (x,y)  coordinates

depends on the current video mode.

If bit 7  of  AL  is  set,  the  new  pixel  value  will  be

exclusive-OR'd (XOR) with the current contents of the pixel.

Register BH is ignored for display modes that  support  only

one page.

9.2.3. Int 10H Function 0DH

Description: Read Graphics  Pixel  [CGA]  [PCjr]  [EGA]  [MCGA]

[VGA]

Obtains the current value of the pixel on the display at the

specified graphics coordinates.

Input                                        Output

---------------------------------            --------------

AH=0DH                                       AL=pixel value

BH=page

CX=column (graphics x coordinate)

DX=row (graphics y coordinate)

The range of valid  (x,y)  coordinates  and  possible  pixel

values depends on the current video mode.

Register BH is ignored for display modes that  support  only

one page.

Interrupt 10H (Graphics Monitors)

9.2.4. Int 10H Function 1AH

Description:

Get or Set Display Combination Code [PS/2]

Returns a code describing the installed display  adapter(s),

or updates the ROM BIOS's  variable  describing  the  installed

adapter(s).

Input                         Output

-------------------------     --------------------------

AH=1AH                        If function supported

AL=subfunction                AL=1AH

00H=get display            and, if called with AL=00H

combination code          BH=inactive display code

01H=set display            BL=inactive display code

combination code

BH=inactive display

code (if AL =01H)

BL=active display

code (if AL =01H)

The display codes are interpreted as follows:

Code(s)     Video Subsystem Type

-------     -----------------------------------

00H         no display

01H         MDA with 5151 monitor

02H         CGA with 5153 or 5154 monitor

03H         reserved

04H         EGA with 5153 or 5154 monitor

05H         EGA with 5151 monitor

06H         PGA with 5175 monitor

07H         VGA with analog monochrome monitor

08H         VGA with analog color monitor

09H         reserved

0AH         MCGA with digital color monitor

0BH         MCGA with analog monochrome monitor

0CH         MCGA with analog color monitor

0DH-FEH     reserved

FFH         unknown

9.2.5. Int 10H Function 1BH

Description:

Get Functionality/State Information [PS/2]

Obtains information about the current display mode, as  well

as a pointer to a  table  describing  the  characteristics  and

capabilities of the video adapter and monitor.

Interrupt 10H (Graphics Monitors)

Input                               Output

----------------------------------- ---------------------------

AH=1BH                              If function supported

BX=implementation type (always 00H) AL=1BH

ES:DI=segment:offset of 64-byte     and information placed in

buffer                       caller's buffer (see Notes)

The caller's buffer  is  filled  in  with  information  that

depends on the current video display mode:

Byte(s)

Contents

00H-03H

pointer  to  functionality  information  (see  next

Note)

04H

current video mode

05H-06H

number of character columns

07H-08H

length of video refresh buffer (bytes)

09H-0AH

starting address in buffer of upper left corner  of

display

0BH-1AH

cursor position for video pages 0-7 as eight 2-byte

entries; first byte of each pair is  y  coordinate,

second byte is x coordinate

1BH

cursor starting line

1CH

cursor ending line

1DH

active display page

1EH-1FH

adapter base port address  (3BXH  monochrome,  3DXH

color)

20H

current setting of register 3B8H or 3D8H

21H

current setting of register 3B9H or 3D9H

22H

number of character rows

23H-24H

character height in scan lines

25H

active display code (see Int 10H Function 1AH)

26H

inactive display code (see Int 10H Function 1AH)

27H-28H

number of displayable colors (0 for monochrome)

29H

number of display pages

2AH

number of scan lines

00H=200 scan lines

01H=350 scan lines

02H=400 scan lines

03H=480 scan lines

04H-FFH=reserved

Interrupt 10H (Graphics Monitors)

2BH

primary character block (see Int 10H  Function  11H

Subfunction 03H)

2CH

secondary character block

2DH

miscellaneous state information

Bit(s)          Significance

------          ------------------------------------------

0               =1 if all modes on all displays active

(always 0 on MCGA)

1               =1 if gray-scale summing active

2               =1 if monochrome display attached

3               =1 if mode set default palette

loading disabled

4               =1 if cursor emulation active

(always 0 on=MCGA)

5               =state of I/B toggle (0=intensity, 1=blink

6-7             =reserved

2EH-30H

reserved

31H

video memory available

00H=64 KB

01H=128 KB

02H=192 KB

03H=256 KB

32H

save pointer state information

Bit(s)          Significance

------          ------------------------------------

0               =1 if 512-character set active

1               =1 if dynamic save area active

2               =1 if alpha font override active

3               =1 if graphics font override active

4               =1 if palette override active

5               =1 if display combination code (DCC)

extension active

6-7             =reserved

33H-3FH

reserved

Bytes 0-3 of the caller's buffer  contain  a  DWORD  pointer

(offset in lower word, segment in upper word) to the  following

information about the display adapter and monitor:

Byte(s)

Contents

00H

video modes supported

Interrupt 10H (Graphics Monitors)

Bit(s)          Significance

------          ------------------------

0               =1 if mode 00H supported

1               =1 if mode 01H supported

2               =1 if mode 02H supported

3               =1 if mode 03H supported

4               =1 if mode 04H supported

5               =1 if mode 05H supported

6               =1 if mode 06H supported

7               =1 if mode 07H supported

01H

video modes supported

Bit(s)          Significance

------          ------------------------

0               =1 if mode 08H supported

1               =1 if mode 09H supported

2               =1 if mode 0AH supported

3               =1 if mode 0BH supported

4               =1 if mode 0CH supported

5               =1 if mode 0DH supported

6               =1 if mode 0EH supported

7               =1 if mode 0FH supported

02H

video modes supported

Bit(s)          Significance

------          ------------------------

0               =1 if mode 10H supported

1               =1 if mode 11H supported

2               =1 if mode 12H supported

3               =1 if mode 13H supported

4-7             =reserved

03H-06H

reserved

07H

scan lines available in text modes

Bit(s)          Significance

------          --------------------

0               =1 if 200 scan lines

1               =1 if 350 scan lines

2               =1 if 400 scan lines

3-7             =reserved

08H

character blocks available in text modes  (see  Int

10H Function 11H)

09H

maximum number of active character blocks  in  text

modes

0AH

miscellaneous BIOS capabilities

Interrupt 10H (Graphics Monitors)

Bit(s)          Significance

------          --------------------------------------

0               =1 if all modes active on all displays

(always 0 for MCGA)

1               =1 if gray-scale summing available

2               =1 if character font loading available

3               =1 if mode set default palette

loading available

4               =1 if cursor emulation available

5               =1 if EGA (64-color) palette available

6               =1 if color register loading available

7               =1 if color register paging mode

select available

0BH

miscellaneous BIOS capabilities

Bit(s)          Significance

------          ----------------------------------------

0               =1 if light pen available

1               =1 if save/restore video state available

(always 0 on MCGA)

2               =1 if background intensity/blinking

control available

3               =1 if get/set display combination

code available

4-7             =reserved

0CH-0DH

reserved

0EH

save area capabilities

Bit(s)          Significance

------          --------------------------------------

0               =1 if supports 512-character sets

1               =1 if dynamic save area available

2               =1 if alpha font override available

3               =1 if graphics font override available

4               =1 if palette override available

5               =1 if display combination code

extension available

6-7             =reserved

0FH

reserved

9.2.6. Int 10H Function 1CH

Description:

Save or Restore Video State [PS/2]

Saves or  restores  the  digital-to-analog  converter  (DAC)

state and color registers, ROM BIOS video driver data area,  or

video hardware state.

Interrupt 10H (Graphics Monitors)

Input                              Output

-------------------------          --------------------------

AH=1CH                             If function supported

AL=subfunction                     AL=1CH

00H to get state buffer size    and, if called with AL=00H

01H to save state               BX=buffer block count

02H to restore state            (64 bytes per block)

CX=requested states                or, if called with AL=01H

Bit(s)     Significance         State information placed

------     ------------------   in caller's buffer

0          save/restore video   or, if called with AL=02H

hardware state      Requested state restored

1          save/restore video   according to contents of

BIOS data area      caller's buffer

2          save/restore video

DAC state and color

registers

3-15       reserved

ES:BX=segment:offset of buffer

Subfunction 00H is used to determine the size of buffer that

will be necessary to contain the specified  state  information.

The caller must supply the buffer.

The current video state  is  altered  during  a  save  state

operation (AL=01H). If the requesting program needs to continue

in the same video state, it can follow the save  state  request

with an immediate call to restore the video state.

This function is supported on the VGA only.

9.3. Interrupt 10H, Function 10H (Colors)

Interrupt  Fnct  Subfnct    Name

10H        10H   00H        Set Palette Register

10H        10H   01H        Set Border Color

10H        10H   02H        Set Palette and Border

10H        10H   03H        Toggle Blink/Intensity Bit

10H        10H   07H        Get Palette Register

10H        10H   08H        Get Border Color

10H        10H   09H        Get Palette and Border

10H        10H   10H        Set Color Register

10H        10H   12H        Set Block of Color Registers

10H        10H   13H        Set Color Page State

10H        10H   15H        Get Color Register

10H        10H   17H        Get Block of Color Registers

10H        10H   1AH        Get Color Page State

10H        10H   1BH        Set Gray-Scale Values

9.3.1. Int 10H Function 10H Subfunction 00H

Description:

Set Palette Register [PCjr] [EGA] [MCGA] [VGA]

Interrupt 10H, Function 10H (Colors)

Sets  the  correspondence  of  a  palette  register   to   a

displayable color.

Input                                     Output

------------------------------            -------

For PCjr, EGA, or VGA                     Nothing

AH=10H

AL=00H

BH=color value

BL=palette register (00-0FH)

For MCGA

AH=10H

AL=00H

BX=0712H

On the MCGA, this function can only be called with BX =0712H

and selects a color register set with eight consistent  colors.

9.3.2. Int 10H Function 10H Subfunction 01H

Description:

Set Border Color [PCjr] [EGA] [VGA]

Controls the color of the screen border (overscan).

Input                      Output

---------------            -------

AH=10H                     Nothing

AL=01H

BH=color value

9.3.3. Int 10H Function 10H Subfunction 02H

Description:

Set Palette and Border [PCjr] [EGA] [VGA]

Sets all palette registers and the border  color  (overscan)

in one operation.

Input                                         Output

----------------------------------            -------

AH=10H                                        Nothing

AL=02H

ES:DX=segment:offset of color list

The color list is 17 bytes long. The first 16 bytes are  the

color values to be loaded into palette registers 0-15, and  the

last byte is stored in the border color register.

In 16-color graphics modes, the following default palette is

set up:

Interrupt 10H, Function 10H (Colors)

Pixel value  Color

-----------  ------------

01H          blue

02H          green

03H          cyan

04H          red

05H          magenta

06H          brown

07H          white

08H          gray

09H          light blue

0AH          light green

0BH          light cyan

0CH          light red

0DH          light magenta

0EH          yellow

0FH          intense white

9.3.4. Int 10H Function 10H Subfunction 03H

Description:

Toggle  Blink/Intensity  Bit  [PCjr]  [EGA]  [MCGA]

[VGA]

Determines whether the most significant bit of  a  character

attribute will select blinking or intensified display.

Input                                Output

-------------------------            -------

AH=10H                               Nothing

AL=03H

BL=blink/intensity toggle

0 =enable intensity

1 =enable blinking

9.3.5. Int 10H Function 10H Subfunction 07H

Description:

Get Palette Register [VGA]

Returns the color  associated  with  the  specified  palette

register.

Input                           Output

--------------------            -------------------

AH=10H                          BH=color

AL=07H

BL=palette register

Interrupt 10H, Function 10H (Colors)

9.3.6. Int 10H Function 10H Subfunction 08H

Description:

Get Border Color [VGA]

Returns the current border color (overscan).

Input             Output

------            ------------------------------

AH=10H            BH=color

AL=08H

9.3.7. Int 10H Function 10H Subfunction 09H

Description:

Get Palette and Border [VGA]

Gets the contents of all palette registers  and  the  border

color (overscan) in one operation.

Input                         Output

-------------------------     ------------------------------

AH=10H                        ES:DX=segment:offset of buffer

AL=09H                           and buffer contains palette

ES:DX=segment:offset of          values in bytes 00H-0FH and

17-byte  buffer            border color in byte 10H

9.3.8. Int 10H Function 10H Subfunction 10H

Description:

Set Color Register [MCGA] [VGA]

Programs an individual color register with a  red-green-blue

(RGB) combination.

Input                        Output

-----------------            -------

AH=10H                       Nothing

AL=10H

BX=color register

CH=green value

CL=blue value

DH=red value

If gray-scale summing is enabled,  the  weighted  gray-scale

value is calculated as described under  Int  10H  Function  10H

Subfunction 1BH and is stored into all three components of  the

color register. See also Int 10H Function 12H Subfunction  33H.

Interrupt 10H, Function 10H (Colors)

9.3.9. Int 10H Function 10H Subfunction 10H

Description:

Set Color Register [MCGA] [VGA]

Programs an individual color register with a  red-green-blue

(RGB) combination.

Input                        Output

-----------------            -------

AH=10H                       Nothing

AL=10H

BX=color register

CH=green value

CL=blue value

DH=red value

If gray-scale summing is enabled,  the  weighted  gray-scale

value is calculated as described under  Int  10H  Function  10H

Subfunction 1BH and is stored into all three components of  the

color register. See also Int 10H Function 12H Subfunction  33H.

9.3.10. Int 10H Function 10H Subfunction 12H

Description:

Set Block of Color Registers [MCGA] [VGA]

Programs a group  of  consecutive  color  registers  in  one

operation.

Input                                          Output

-----------------------------------            -------

AH=10H                                         Nothing

AL=12H

BX=first color register

CX=number of color registers

ES:DX=segment:offset of color table

The table consists of a series of 3-byte entries, one  entry

per color register to be programmed. The bytes of an individual

entry specify the red, green, and blue values (in  that  order)

for the associated color register.

If gray-scale summing is enabled,  the  weighted  gray-scale

value for each register is calculated as  described  under  Int

10H Function 10H Subfunction 1BH and is stored into  all  three

components of the color register. See also Int 10H Function 12H

Subfunction 33H.

9.3.11. Int 10H Function 10H Subfunction 13H

Description:

Set Color Page State [VGA]

Interrupt 10H, Function 10H (Colors)

Selects the paging mode for the color registers, or  selects

an individual page of color registers.

Input                                           Output

------------------------------------            -------

To select the paging mode                       Nothing

AH=10H

AL=13H

BH=paging mode

00H for 4 pages of 64 registers

01H for 16 pages of 16 registers

BL=00H

To select a color register page

AH=10H

AL=13H

BH=page

BL=01H

This function is not valid in mode 13H (320-by-200 256-color

graphics).

9.3.12. Int 10H Function 10H Subfunction 15H

Description:

Get Color Register [MCGA] [VGA]

Returns the contents of a color register as its red,  green,

and blue components.

Input                        Output

-----------------            --------------

AH=10H                       CH=green value

AL=15H                       CL=blue value

BX=color register            DH=red value

9.3.13. Int 10H Function 10H Subfunction 17H

Description:

Get Block of Color Registers [MCGA] [VGA]

Allows the red, green, and blue components  associated  with

each of a set of color registers to be read in one operation.

Input                            Output

------------------------------   ------------------------------

AH=10H                           ES:DX=segment:offset of buffer

AL=17H                           and buffer contains color list

BX=first color register

CX=number of color registers

ES:DX=segment:offset of buffer

to receive color list

The color list returned in the caller's buffer consists of a

Interrupt 10H, Function 10H (Colors)

series of 3-byte entries corresponding to the color  registers.

Each 3-byte entry contains the register's red, green, and  blue

components in that order.

9.3.14. Int 10H Function 10H Subfunction 1AH

Description:

Get Color Page State [VGA]

Returns the color register paging  mode  and  current  color

page.

Input             Output

------            ----------------------------------

AH=10H            BH=color page

AL=1AH            BL=paging mode

00H if 4 pages of 64 registers

01H if 16 pages of 16 registers

See Int 10H  Function  10H  Subfunction  13H,  which  allows

selection of the paging mode or current color page.

9.3.15. Int 10H Function 10H Subfunction 1BH

Transforms the red, green, and blue values of one or more color

registers into the gray-scale equivalents.

Input                                   Output

----------------------------            -------

AH=10H                                  Nothing

AL=1BH

BX=first color register

CX=number of color registers

For each color register, the weighted sum of its red, green,

and blue values is calculated (30% red $PL 59%  green  $PL  11%

blue) and written back into all three components of  the  color

register. The original red, green, and blue values are lost.

9.4. Interrupt 10H, Function 11H (Fonts)

Interrupt 10H, Function 11H (Fonts)

Inter-

rupt  Fnct  Subfnct     Name

10H   11H   00H         Load User Font and

10H   11H   10H         Reprogram Controller

10H   11H   01H         Load ROM 8-by-14 Font and

10H   11H   11H         Reprogram Controller

10H   11H   02H         Load ROM 8-by-8 Font and

10H   11H   12H         Reprogram Controller

10H   11H   03H         Set Block Specifier

10H   11H   04H         Load ROM 8-by-16 Font and

10H   11H   14H         Reprogram Controller

10H   11H   20H         Set Int 1FH Font Pointer

10H   11H   21H         Set Int 43H for User's Font

10H   11H   22H         Set Int 43H for ROM 8-by-14 Font

10H   11H   23H         Set Int 43H for ROM 8-by-8 Font

10H   11H   24H         Set Int 43H for ROM 8-by-16 Font

10H   11H   30H         Get Font Information

9.4.1. Int 10H Function 11H Subfunction 00H/10H

Description:

Load  User  Font  and  Reprogram  Controller  [EGA]

[MCGA] [VGA]

Loads the user's font (character definition) table into  the

specified block of character generator RAM.

Input                                               Output

----------------------------------------            -------

AH=11H                                              Nothing

AL=00H or 10H (see Notes)

BH=points (bytes per character)

BL=block

CX=number of characters defined by table

DX=first character code in table

ES:BP=segment:offset of font table

This function provides font selection in text (alphanumeric)

display    modes.    For    font    selection    in    graphics

(all-points-addressable)  modes,  see  Int  10H  Function   11H

Subfunctions 20H-24H.

If AL=10H, page 0 must be  active.  The  points  (bytes  per

character),  rows,  and  length  of  the  refresh  buffer   are

recalculated. The controller is reprogrammed with  the  maximum

scan line (points -1), cursor start  (points  -2),  cursor  end

(points -1), vertical display end ((rows  *  points)  -1),  and

underline location (points -1, mode 7 only).

If  Subfunction  10H  is  called  at  any  time  other  than

immediately after a mode set, the results are unpredictable.

On the MCGA, a Subfunction 00H call should be followed by  a

Subfunction 03H call so that the ROM BIOS will  load  the  font

into the character generator's internal font pages.

Subfunction 10H is reserved on the MCGA. If  it  is  called,

Subfunction 00H is executed.

Interrupt 10H, Function 11H (Fonts)

9.4.2. SetLineMode

;* SetLineMode - Sets line mode for EGA or VGA.

;*

;* Shows:   BIOS Interrupt - 10h, Function 11h

;*                            (Character Generator Interface)

;*                           10h, Function 12h

;*                            (Video Subsystem Configuration)

;*          Instruction - cmp

;*

;* Uses:    vconfig - Video configuration structure,

;*          declared in the DEMO.INC include file.

;*          The structure must first be initialized

;*          by calling the GetVidConfig procedure.

;*

;* Params:  line - Requested line mode (25, 43, or 50)

;*

;* Return:  Short integer with error code

;*          0 if successful

;*          1 if error

SetLineMode PROC \

line:WORD

cmp     vconfig.adapter, EGA    ; EGA or VGA?

jge     @F                      ; Yes?  Continue

jmp     e_exit           ; No?  Exit with error

@@:     mov     ax, line         ; Check for valid parameter

cmp     al, 25

je      line25

cmp     al, 43

je      line43

cmp     al, 50

je      line50

jmp     SHORT e_exit ; If not 25, 43, or 50,

; exit w/ error

line25: mov     al, 11h      ; Set for EGA 25-line mode

cmp     vconfig.adapter, EGA    ; EGA?

je      lmode        ; Yes?  Continue

mov     ax, 1202h    ; No?  Function 12h for VGA

mov     bl, 30h      ; AL = 2 for 400 scan lines

int     10h          ; Reset to 400 scan lines

mov     ax, 0003     ; Reset mode (Function 0)

int     10h          ;   to mode 3 (80-col text)

mov     al, 14h      ; Request 8x16 char matrix

jmp     SHORT lmode

line43: mov     al, 12h      ; Set for EGA 43-line mode

cmp     vconfig.adapter, EGA    ; EGA?

je      lmode        ; Yes?  Continue

mov     ax, 1201h    ; No?  Function 12h for VGA

mov     bl, 30h      ; AL = 1 for 350 scan lines

int     10h          ; Reset to 350 scan lines

mov     ax, 0003     ; Reset mode (Function 0)

int     10h          ;   to mode 3 (80-col text)

mov     al, 12h      ; Request 8x8 character matrix

jmp     SHORT lmode

line50: cmp     vconfig.adapter, VGA    ; VGA?

jne     e_exit       ; No?  Exit with error

Interrupt 10H, Function 11H (Fonts)

mov     ax, 1202h    ; Yes?  Function 12h

mov     bl, 30h      ; AL = 2 for 400 scan lines

int     10h          ; Reset to 400 scan lines

mov     ax, 0003     ; Reset mode (Function 0)

int     10h          ;   to mode 3 (80-col text)

mov     al, 12h      ; Request 8x8 character matrix

lmode:  sub     bl, bl       ; Use table 0

mov     ah, 11h      ; Request Function 11h

int     10h          ; Set new line mode

mov     ah, 12h      ; Select alternate print

mov     bl, 20h      ;    screen for EGA and VGA

int     10h

cmp     vconfig.adapter, VGA    ; VGA?

je      exit         ; Yes?  Then exit

cmp     line, 12h    ; If EGA 43-line mode, set

je      port         ;   cursor through port to

;   avoid cursor emulation bug

mov     al, 7        ; Else use BIOS to set cursor

push    ax           ; Pass bottom scan line

mov     al, 6

push    ax           ; Pass top scan line

call    SetCurSize   ; Set normal cursor

add     sp, 4        ; Clean stack

jmp     SHORT exit   ; Exit

port:   mov     dx, 03D4h    ; Video controller address

mov     ax, 060Ah    ; Set AH = 06h (cursor start)

;     AL = 0Ah (register #)

out     dx, ax       ; Update port

mov     ax, 000Bh    ; Set AH = 00h (cursor end)

;     AL = 0Bh (register #)

out     dx, ax       ; Update port

jmp     SHORT exit   ; Normal exit

e_exit: mov     ax, 1        ; Set error code

jmp     SHORT @F

exit:   sub     ax, ax       ; Clear error code

@@:     ret

SetLineMode ENDP

9.4.3. Int 10H Function 11H Subfunction 01H/11H

Description: Load ROM 8-by-14  Font  and  Reprogram  Controller

[EGA] [VGA]

Loads the ROM BIOS  default  8-by-14  font  table  into  the

specified block of character generator RAM.

Interrupt 10H, Function 11H (Fonts)

Input                                Output

-------------------------            -------

AH=11H                               Nothing

AL=01H or 11H (see Notes)

BL=block

This function provides font selection in text (alphanumeric)

display    modes.    For    font    selection    in    graphics

(all-points-addressable)  modes,  see  Int  10H  Function   11H

Subfunctions 20H-24H.

If AL=11H, page 0 must be  active.  The  points  (bytes  per

character),  rows,  and  length  of  the  refresh  buffer   are

recalculated. The controller is reprogrammed with  the  maximum

scan line (points -1), cursor start  (points  -2),  cursor  end

(points -1), vertical display end ((rows  *  points)  -1),  and

underline location (points -1, mode 7 only).

If  Subfunction  11H  is  called  at  any  time  other  than

immediately after a mode set, the results are unpredictable.

Subfunctions 01H and 11H are reserved on the MCGA. If either

is called, Subfunction 04H is executed.

9.4.4. Int 10H Function 11H Subfunction 02H/12H

Description: Load ROM  8-by-8  Font  and  Reprogram  Controller

[EGA] [MCGA] [VGA]

Loads the ROM  BIOS  default  8-by-8  font  table  into  the

specified block of character generator RAM.

Input                                Output

-------------------------            -------

AH=11H                               Nothing

AL=02H or 12H (see Notes)

BL=block

This function provides font selection in text (alphanumeric)

display    modes.    For    font    selection    in    graphics

(all-points-addressable)  modes,  see  Int  10H  Function   11H

Subfunctions 20H-24H.

If AL =12H, page 0 must be active.  The  points  (bytes  per

character),  rows,  and  length  of  the  refresh  buffer   are

recalculated. The controller is reprogrammed with  the  maximum

scan line (points -1), cursor start  (points  -2),  cursor  end

(points -1), vertical display end ((rows  *  points)  -1),  and

underline location (points -1, mode 7 only).

If  Subfunction  12H  is  called  at  any  time  other  than

immediately after a mode set, the results are unpredictable.

On the MCGA, a Subfunction 02H call should be followed by  a

Subfunction 03H call, so that the ROM BIOS will load  the  font

into the character generator's internal font pages.

Subfunction 12H is reserved on the MCGA. If  it  is  called,

Subfunction 02H is executed.

Interrupt 10H, Function 11H (Fonts)

9.4.5. Int 10H Function 11H Subfunction 03H

Description:

Set Block Specifier [EGA] [MCGA] [VGA]

Determines  the  character  blocks  selected  by  bit  3  of

character attribute bytes in alphanumeric (text) display modes.

Input                                   Output

----------------------------            -------

AH=11H                                  Nothing

AL=03H

BL=character generator block

select code (see Notes)

On the EGA and MCGA, the bits of BL are used as follows:

Bits   Significance

----   --------------------------------------------------------

0-1    character block selected by attribute bytes with bit 3=0

2-3    character block selected by attribute bytes with bit 3=1

4-7    not used (should be 0)

On the VGA, the bits of BL are used as follows:

Bits   Significance

----   --------------------------------------------------------

0,1,4  character block selected by attribute bytes with bit 3=0

2,3,5  character block selected by attribute bytes with bit 3=1

6-7    not used (should be 0)

When using a 256-character set, both  fields  of  BL  should

select the same  character  block.  In  such  cases,  character

attribute bit 3 controls the foreground intensity.

When using 512-character sets, the fields  of  BL  designate

the blocks holding each half of the character set, and bit 3 of

the character attribute selects the upper or lower half of  the

character set.

When using a 512-character set, a call to Int  10H  Function

10H Subfunction 00H with BX =0712H is recommended  to  set  the

color planes to eight consistent colors.

9.4.6. Int 10H Function 11H Subfunction 04H

Description: Load ROM 8-by-16  Font  and  Reprogram  Controller

[MCGA] [VGA]

Loads the ROM BIOS  default  8-by-16  font  table  into  the

specified block of character generator RAM.

Input                                Output

-------------------------            -------

AH=11H                               Nothing

AL=04H or 14H (see Notes)

BL=block

Interrupt 10H, Function 11H (Fonts)

This function provides font selection in text (alphanumeric)

display    mode.    For    font    selection    in     graphics

(all-points-addressable)  modes,  see  Int  10H  Function   11H

Subfunctions 20H-24H.

If AL =14H, page 0 must be active.  The  points  (bytes  per

character),  rows,  and  length  of  the  refresh  buffer   are

recalculated. The controller is reprogrammed with  the  maximum

scan line (points -1), cursor start  (points  -2),  cursor  end

(points -1), vertical display end rows * points -1 for 350  and

400 line modes, or rows * points * 2 -1 for  200  line  modes),

and underline location (points -1, mode 7 only).

If  Subfunction  14H  is  called  at  any  time  other  than

immediately after a mode set, the results are unpredictable.

On the MCGA, a Subfunction 04H call should be followed by  a

Subfunction 03H call so that the ROM BIOS will  load  the  font

into the character generator's internal font pages.

Subfunction 14H is reserved on the MCGA. If  it  is  called,

Subfunction 04H is executed.

9.4.7. Int 10H Function 11H Subfunction 14H

Description: Load ROM 8-by-16  Font  and  Reprogram  Controller

[MCGA] [VGA]

Loads the ROM BIOS  default  8-by-16  font  table  into  the

specified block of character generator RAM.

Input                            Output

-------------------------        -------

AH=11H                           Nothing

AL=04H or 14H (see Notes)

BL=block

This function provides font selection in text (alphanumeric)

display    mode.    For    font    selection    in     graphics

(all-points-addressable)  modes,  see  Int  10H  Function   11H

Subfunctions 20H-24H.

If AL =14H, page 0 must be active.  The  points  (bytes  per

character),  rows,  and  length  of  the  refresh  buffer   are

recalculated. The controller is reprogrammed with  the  maximum

scan line (points -1), cursor start  (points  -2),  cursor  end

(points -1), vertical display end rows * points -1 for 350  and

400 line modes, or rows * points * 2 -1 for  200  line  modes),

and underline location (points -1, mode 7 only).

If  Subfunction  14H  is  called  at  any  time  other  than

immediately after a mode set, the results are unpredictable.

On the MCGA, a Subfunction 04H call should be followed by  a

Subfunction 03H call so that the ROM BIOS will  load  the  font

into the character generator's internal font pages.

Subfunction 14H is reserved on the MCGA. If  it  is  called,

Subfunction 04H is executed.

Interrupt 10H, Function 11H (Fonts)

9.4.8. Int 10H Function 11H Subfunction 20H

Description:

Set Int 1FH Font Pointer [EGA] [MCGA] [VGA]

Sets the Int 1FH pointer to  the  user's  font  table.  This

table is used for character codes  80H-FFH  in  graphics  modes

04H-06H.

Input                                         Output

----------------------------------            -------

AH=11H                                        Nothing

AL=20H

ES:BP=segment:offset of font table

This  function   provides   font   selection   in   graphics

(all-points-addressable) display modes. For font  selection  in

text  (alphanumeric)  modes,   see   Int   10H   Function   11H

Subfunctions 00H-14H.

If this  subfunction  is  called  at  any  time  other  than

immediately after a mode set, the results are unpredictable.

9.4.9. Int 10H Function 11H Subfunction 21H

Description:

Set Int 43H for User's Font [EGA] [MCGA] [VGA]

Sets the vector for Int 43H to  point  to  the  user's  font

table and updates the video  ROM  BIOS  data  area.  The  video

controller is not reprogrammed.

Input                                           Output

------------------------------------------      -------

AH=11H                                          Nothing

AL=21H

BL=character rows specifier

00H if user specified (see register DL)

01H =14 (0EH) rows

02H =25 (19H) rows

03H =43 (2BH) rows

CX=points (bytes per character)

DL=character rows per screen (if BL =00H)

ES:BP=segment:offset of user font table

This  function   provides   font   selection   in   graphics

(all-points-addressable) display modes. For font  selection  in

text  (alphanumeric)  modes,   see   Int   10H   Function   11H

Subfunctions 00H-14H.

If this  subfunction  is  called  at  any  time  other  than

immediately after a mode set, the results are unpredictable.

Interrupt 10H, Function 11H (Fonts)

9.4.10. Int 10H Function 11H Subfunction 22H

Description:

Set Int 43H for ROM 8-by-14 Font [EGA] [MCGA] [VGA]

Sets the vector for Int 43H to point to the ROM BIOS default

8-by-14 font and updates the video  ROM  BIOS  data  area.  The

video controller is not reprogrammed.

Input                                             Output

------------------------------------------        -------

AH=11H                                            Nothing

AL=22H

BL=character rows specifier

00H if user specified (see register DL)

01H=14 (0EH) rows

02H=25 (19H) rows

03H=43 (2BH) rows

DL=character rows per screen (if BL=00H)

This  function   provides   font   selection   in   graphics

(all-points-addressable) display modes. For font  selection  in

text  (alphanumeric)  modes,   see   Int   10H   Function   11H

Subfunctions 00H-14H.

If this  subfunction  is  called  at  any  time  other  than

immediately after a mode set, the results are unpredictable.

When this subfunction is called on the MCGA, Subfunction 24H

is substituted.

9.4.11. Int 10H Function 11H Subfunction 23H

Description:

Set Int 43H for ROM 8-by-8 Font [EGA] [MCGA]  [VGA]

Sets the vector for Int 43H to point to the ROM BIOS default

8-by-8 font and updates the video ROM BIOS data area. The video

controller is not reprogrammed.

Input                                             Output

------------------------------------------        -------

AH=11H                                            Nothing

AL=23H

BL=character rows specifier

00H if user specified (see register DL)

01H=14 (0EH) rows

02H=25 (19H) rows

03H=43 (2BH) rows

DL=character rows per screen (if BL =00H)

This  function   provides   font   selection   in   graphics

(all-points-addressable) display modes. For font  selection  in

text  (alphanumeric)  modes,   see   Int   10H   Function   11H

Subfunctions 00H-14H.

If this  subfunction  is  called  at  any  time  other  than

immediately after a mode set, the results are unpredictable.

Interrupt 10H, Function 11H (Fonts)

9.4.12. Int 10H Function 11H Subfunction 24H

Description:

Set Int 43H for ROM 8-by-16 Font [MCGA] [VGA]

Sets the vector for Int 43H to point to the ROM BIOS default

8-by-16 font and updates the video  ROM  BIOS  data  area.  The

video controller is not reprogrammed.

Input                                          Output

----------------------------------------       --------

AH=11H                                         Nothing

AL=24H

BL=row specifier

01H=14 (0EH) rows

02H=25 (19H) rows

03H=43 (2BH) rows

DL=character rows per screen (if BL=00H)

This  function   provides   font   selection   in   graphics

(all-points-addressable) display modes. For font  selection  in

text  (alphanumeric)  modes,   see   Int   10H   Function   11H

Subfunctions 00H-14H.

If this  subfunction  is  called  at  any  time  other  than

immediately after a mode set, the results are unpredictable.

9.4.13. Int 10H Function 11H Subfunction 30H

Description:

Get Font Information [EGA] [MCGA] [VGA]

Returns a pointer to the character definition  table  for  a

font, and the points (bytes per character) and  rows  for  that

font.

Input                        Output

---------------------------  ----------------------------------

AH=11H                       CX=points (bytes per character)

AL=30H                       DL=rows (character rows on screen-

BH=font code                 ES:BP=segment:offset of font table

00H=current Int 1FH contents

01H=current Int 43H contents

02H=ROM 8-by-14 font

(EGA, VGA only)

03H=ROM 8-by- 8 font

(characters 00H-7FH)

04H=ROM 8-by- 8 font

(characters 80H-FFH)

05H=ROM alternate 9-by-14 font

(EGA, VGA only)

06H=ROM 8-by-16 font

(MCGA, VGA only)

07H=ROM alternate 9-by-16 font

(VGA only)

Interrupt 10H, Function 12H (Video Config.)

9.5.  Interrupt  10H,   Function   12H   (Video

Config.)

Interrupt  Fnct  Subfnct  Name

10H        12H   10H      Get Configuration Information

10H        12H   20H      Select Alternate PrintScreen

10H        12H   30H      Set Scan Lines

10H        12H   31H      Enable/Disable Default Palette Loadin

10H        12H   32H      Enable/Disable Video

10H        12H   33H      Enable/Disable Gray-Scale Summing

10H        12H   34H      Enable/Disable Cursor Emulation

10H        12H   35H      Switch Active Display

10H        12H   36H      Enable/Disable Screen Refresh

9.5.1. Int 10H Function 12H Subfunction 10H

Description:

Get Configuration Information [EGA] [VGA]

Obtains  configuration  information  for  the  active  video

subsystem.

Input             Output

------            --------------------------------

AH=12H            BH=display type

BL=10H               0H if color display

1H if monochrome display

BL=memory installed on EGA board

00H if 64 KB

01H if 128 KB

03H if 256 KB

CH=feature bits (see Notes)

CL=switch setting (see Notes)

The feature bits are set from Input  Status  register  0  in

response to an output on the specified Feature Control register

bits:

Feature     Feature Control     Input Status

Bit(s)      Output Bit          Bit

-------     ---------------     ------------

0           0                   5

1           0                   6

2           1                   5

3           1                   6

4-7         not used

The bits in the switch settings byte indicate the  state  of

the EGA's configuration DIP switch (1 =off, 0 =on).

Interrupt 10H, Function 12H (Video Config.)

Bit     Significance

---     ----------------------

0       configuration switch 1

1       configuration switch 2

2       configuration switch 3

3       configuration switch 4

4-7     not used

9.5.2. Int 10H Function 12H Subfunction 20H

Description:

Select Alternate PrintScreen [EGA] [VGA]

Selects an alternate print-screen routine for  the  EGA  and

VGA that works properly if the screen length is not  25  lines.

The ROM BIOS default  print-screen  routine  always  prints  25

lines.

Input             Output

------            -------

AH=12H            Nothing

BL=20H

9.5.3. Int 10H Function 12H Subfunction 30H

Description:

Set Scan Lines [VGA]

Selects the number of scan lines for alphanumeric modes. The

selected value takes effect the next time Int 10H Function  00H

is called to select the display mode.

Input                            Output

---------------------            ------------------------

AH=12H                           If the VGA is active

AL=scan line code                AL=12H

00H=200 scan lines            If the VGA is not active

01H=350 scan lines            AL=00H

BL=30H

9.5.4. Int 10H Function 12H Subfunction 31H

Description:

Enable/Disable Default Palette Loading [MCGA] [VGA]

Enables or disables loading of  a  default  palette  when  a

Interrupt 10H, Function 12H (Video Config.)

video display mode is selected.

Input                                     Output

----------------------------------------  ---------------------

AH=12H                                    If function supported

AL=00H to enable default palette loading    AL=12H

01H to disable default palette loading

BL=31H

9.5.5. Int 10H Function 12H Subfunction 32H

Description:

Enable/Disable Video [MCGA] [VGA]

Enables or disables CPU access to the  video  adapter's  I/O

ports and video refresh buffer.

Input                               Output

------------------------            ---------------------

AH=12H                              If function supported

AL=00H to enable access             AL=12H

01H to disable access

BL=32H

9.5.6. Int 10H Function 12H Subfunction 33H

Description:

Enable/Disable Gray-Scale Summing [MCGA] [VGA]

Enables or disables gray-scale  summing  for  the  currently

active display.

Input                                 Output

------------------------------------  ---------------------

AH=12H                                If function supported

AL=00H to enable gray-scale summing     AL=12H

01H to disable gray-scale summing

BL=33H

When enabled, gray-scale summing occurs during display  mode

selection, palette programming, and color register loading.

9.5.7. Int 10H Function 12H Subfunction 34H

Description:

Enable/Disable Cursor Emulation [VGA]

Enables or  disables  cursor  emulation  for  the  currently

Interrupt 10H, Function 12H (Video Config.)

active display. When cursor emulation is enabled, the ROM  BIOS

automatically remaps Int 10H Function 01H cursor  starting  and

ending lines for the current character cell dimensions.

Input                                Output

----------------------------------   ---------------------

AH=12H                               If function supported

AL=00H to enable cursor emulation      AL=12H

01H to disable cursor emulation

BL=34H

9.5.8. Int 10H Function 12H Subfunction 35H

Description:

Switch Active Display [MCGA] [VGA]

Allows selection of one of two video adapters in the system,

when memory usage or port addresses conflict  between  the  two

adapters.

Input                         Output

----------------------------  -------------------------------

AH=12H                        If function supported

AL=switching function           AL=12H

00H to disable initial        and, if called with AL=00H

video adapter             or 02H

01H to enable system board    Video adapter state information

video adapter             saved in caller's buffer

02H to disable active         or, if called with AL =03H

video adapter             Video adapter state restored

03H to enable inactive        from information in caller's

video adapter             buffer.

BL=35H

ES:DX=segment:offset of 128-byte buffer

(if AL =00H, 02H, or 03H)

This subfunction cannot be used unless both  video  adapters

have a disable capability (Int  10H  Function  12H  Subfunction

32H).

If there is no conflict between the system board  video  and

the adapter board video in memory or  port  usage,  both  video

controllers can be active simultaneously and  this  subfunction

is not required.

9.5.9. Int 10H Function 12H Subfunction 36H

Description:

Enable/Disable Screen Refresh [VGA]

Enables or disables the  video  refresh  for  the  currently

active display.

Interrupt 10H, Function 12H (Video Config.)

Input                               Output

------------------------            ---------------------

AH=12H                              If function supported

AL=00H to enable refresh            AL=12H

01H to disable refresh

BL=36H

9.6. Function 13H (Disk I/O)

Disk I/O

Interrupt  Function         Name

13H        00H              Reset Disk System

13H        01H              Get Disk System Status

13H        02H              Read Sector

13H        03H              Write Sector

13H        04H              Verify Sector

13H        05H              Format Track

13H        06H              Format Bad Track

13H        07H              Format Drive

13H        08H              Get Drive Parameters

13H        09H              Initialize Fixed Disk Characteristi

13H        0AH              Read Sector Long

13H        0BH              Write Sector Long

13H        0CH              Seek

13H        0DH              Reset Fixed Disk System

13H        0EH              Read Sector Buffer

13H        0FH              Write Sector Buffer

Disk Diagnostics

Interrupt  Function         Name

13H        10H              Get Drive Status

13H        11H              Recalibrate Drive

13H        12H              Controller RAM Diagnostic

13H        13H              Controller Drive Diagnostic

13H        14H              Controller Internal Diagnostic

13H        15H              Get Disk Type

13H        16H              Get Disk Change Status

13H        17H              Set Disk Type

13H        18H              Set Media Type for Format

13H        19H              Park Heads

13H        1AH              Format ESDI Drive

Function 13H (Disk I/O)

9.6.1. Int 13H Function 00H

Description:

Reset Disk System [PC] [AT] [PS/2]

Resets the disk controller, recalibrates its attached drives

(the read/ write arm is moved to cylinder 0), and prepares  for

disk I/O.

Input                   Output

---------------------   ------------------------------------

AH=00H                  If function successful

DL=drive                  Carry flag=clear

00H-7FH floppy disk     AH=00H

80H-FFH fixed disk    If function unsuccessful

Carry flag=set

AH=status (see Int 13H Function 01H)

This function should be called after a  failed  floppy  disk

Read, Write, Verify, or  Format  request  before  retrying  the

operation.

If called with DL >NM =80H (i.e.,  selecting  a  fixed  disk

drive), the floppy disk controller  and  then  the  fixed  disk

controller are reset. See also  Int  13H  Function  0DH,  which

allows the fixed disk controller to be reset without  affecting

the floppy disk controller.

9.6.2. Int 13H Function 01H

Description:

Get Disk System Status [PC] [AT] [PS/2]

Returns the status of the most recent disk operation.

Input                      Output

-----------------------    ------------------------------------

AH=01H                     AH=00H

DL=drive                   AL=status of previous disk operation

00H-7FH  floppy disk       00H no error

80H-FFH  fixed disk        01H invalid command

02H address mark not found

03H disk write-protected (F)

04H sector not found

05H reset failed (H)

06H floppy disk removed (F)

07H bad parameter table (H)

08H DMA overrun (F)

09H DMA crossed 64 KB boundary

0AH bad sector flag (H)

0BH bad track flag (H)

0CH media type not found (F)

0DH invalid number of sectors on

format   (H)

0EH control data address mark

detected   (H)

0FH DMA arbitration level out of

Function 13H (Disk I/O)

range   (H)

10H uncorrectable CRC* or ECC**

data error

11H ECC corrected data error (H)

20H controller failed

40H seek failed

80H disk timedout

(failed to respond)

AAH drive not ready (H)

BBH undefined error (H)

CCH write fault (H)

E0H status register error (H)

FFH sense operation failed (H)

H=fixed disk only

F=floppy disk only

*  Cyclic Redundancy Check code

** Error Checking and Correcting code

On fixed disks, error code 11H (ECC  data  error)  indicates

that a recoverable error was detected during a  preceding  Read

Sector (Int 13H Function 02H) function.

9.6.3. Int 13H Function 02H

Description:

Read Sector [PC] [AT] [PS/2]

Reads one or more sectors from disk into memory.

Input                              Output

--------------------------------   ---------------------------

AH=02H                             If function successful

AL=number of sectors transferred     Carry flag=clear

AL=number of sectors                 AH=00H

CH=cylinder                          AL=number of sectors

CL=sector                               transferred

DH=head                            If function unsuccessful

DL=drive                             Carry flag   =set

00H-7FH floppy disk                AH=status

80H-FFH fixed disk                 (see Int 13H Function 01H)

ES:BX=segment:offset of buffer

On fixed disks, the upper 2  bits  of  the  10-bit  cylinder

number are placed in the upper 2 bits of register CL.

On fixed disks, error code 11H indicates that a  read  error

occurred that was corrected  by  the  ECC  algorithm;  in  this

event, register AL contains the burst length. The data returned

is probably good, although there is a  small  chance  that  the

data was not corrected properly. If a multisector transfer  was

requested,  the  operation  was  terminated  after  the  sector

containing the read error.

On floppy disk drives, an error may result  from  the  drive

motor being off at the time of the request. The ROM  BIOS  does

not automatically wait for the drive to come up to speed before

attempting the read operation. The  requesting  program  should

reset the floppy disk system (Int 13H Function 00H)  and  retry

the operation  three  times  before  assuming  that  the  error

results from some other cause.

Function 13H (Disk I/O)

9.6.4. Int 13H Function 03H

Description:

Write Sector [PC] [AT] [PS/2]

Writes one or more sectors from memory to disk.

Input                            Output

------------------------------   -----------------------------

AH=03H                           If function successful

AL=number of sectors               Carry flag=clear

CH=cylinder                        AH=00H

CL=sector                          AL=number of sectors

DH=head                               transferred

DL=drive

00H-7FH floppy disk            If function unsuccessful

80H-FFH fixed disk               Carry flag=set

ES:BX=segment:offset of buffer     AH=status

(see Int 13H Function 01H)

On fixed disks, the upper 2  bits  of  the  10-bit  cylinder

number are placed in the upper 2 bits of register CL.

On floppy disk drives, an error may result  from  the  drive

motor being off at the time of the request. The ROM  BIOS  does

not automatically wait for the drive to come up to speed before

attempting the write operation. The requesting  program  should

reset the floppy disk system (Int 13H Function 00H)  and  retry

the operation  three  times  before  assuming  that  the  error

results from some other cause.

9.6.5. Int 13H Function 04H

Description:

Verify Sector [PC] [AT] [PS/2]

Verifies the address fields of one or more sectors. No  data

is transferred to or from memory by this operation.

Input                            Output

------------------------------   -----------------------------

AH=04H                           If function successful

AL=number of sectors               Carry flag=clear

CH=cylinder                        AH=00H

CL=sector                          AL=number of sectors verifie

DH=head                          If function unsuccessful

DL=drive                           Carry flag=set

00H-7FH floppy disk              AH=status

80H-FFH fixed disk               (see Int 13H Function 01H)

ES:BX=segment:offset of buffer

(see Notes)

On PCs, PC/XTs, and PC/ATs with ROM BIOS dated earlier  than

11/15/85, ES:BX should point to a valid buffer.

On fixed disks, the upper 2  bits  of  the  10-bit  cylinder

number are placed in the upper 2 bits of register CL.

This function can be used to test whether a  readable  media

Function 13H (Disk I/O)

is in a floppy disk drive. An error may result from  the  drive

motor being off at the time of the  request,  because  the  ROM

BIOS does not automatically wait for the drive to  come  up  to

speed before attempting the verify  operation.  The  requesting

program should reset the floppy disk system (Int  13H  Function

00H) and retry the operation three times before assuming that a

readable floppy disk is not present.

9.6.6. Int 13H Function 05H

Description:

Format Track [P] [AT] [PS/2]

Initializes disk sector and  track  address  fields  on  the

specified track.

Input                             Output

-------------------------------   ----------------------------

AH=05H                            If function successful

AL=interleave                       Carry flag=clear

(PC/XT fixed disks)               AH=00H

CH=cylinder

DH=head                           If function unsuccessful

DL=drive                            Carry flag=set

00H-7FH floppy disk              AH=status

80H-FFH fixed disk               (see Int 13H Function 01H)

ES:BX=segment:offset of address

field list (except PC/XT)

fixed disk, see Notes)

On floppy disks, the address field list consists of a series

of 4-byte entries, one  entry  per  sector,  in  the  following

format:

Byte     Contents

----     --------------------------------------

0        cylinder

1        head

2        sector

3        sector-size code

00H if 128 bytes per sector

01H if 256 bytes per sector

02H if 512 bytes per sector (standard)

03H if 1024 bytes per sector

On floppy disks, the number of sectors per  track  is  taken

from the BIOS floppy disk  parameter  table  whose  address  is

stored in the vector for Int 1EH.

When this function is used for floppy disks on the PC/AT  or

PS/2, it should be preceded by a call to Int 13H  Function  17H

to select the type of medium to be formatted.

On fixed disks, the upper 2  bits  of  the  10-bit  cylinder

number are placed in the upper 2 bits of register CL.

On PC/XT-286, PC/AT, and PS/2 fixed disks, ES:BX points to a

512-byte buffer containing byte pairs for  each  physical  disk

sector as follows:

Function 13H (Disk I/O)

Byte        Contents

---         -------------------

0           00H for good sector

80H for bad sector

1           sector number

For example, to format  a  track  with  17  sectors  and  an

interleave of two, ES:BX would point to the  following  34-byte

array at the beginning of a 512-byte buffer:

db   00h,01h,00h,0ah,00h,02h,00h,0bh,00h,03h,00h,0ch

db   00h,04h,00h,0dh,00h,05h,00h,0eh,00h,06h,00h,0fh

db   00h,07h,00h,10h,00h,08h,00h,11h,00h,09h

9.6.7. Int 13H Function 06H

Description:

Format Bad Track [PC]

Initializes a track, writing disk address  fields  and  data

sectors and setting bad sector flags.

Input                    Output

----------------------   ------------------------------------

AH=06H                   If function successful

AL=interleave              Carry flag=clear

CH=cylinder                AH=00H

DH=head                  If function unsuccessful

DL=drive                   Carry flag=set

80H-FFH  fixed disk      AH=status (see Int 13H Function 01H)

This function is defined for PC/XT fixed disk drives only.

For additional information, see Notes for Int  13H  Function

05H.

9.6.8. Int 13H Function 07H

Description:

Format Drive [PC]

Formats the entire drive, writing disk  address  fields  and

data sectors, starting at the specified cylinder.

Input                             Output

----------------------            --------------------------

AH=07H                            If function successful

AL=interleave                     Carry flag=clear

CH=cylinder                       AH=00H

DL=drive                          If function unsuccessful

80H-FFH  fixed disk            Carry flag=set

AH=status (see Int 13H

Function 01H)

This function is defined for PC/XT fixed disk drives only.

Function 13H (Disk I/O)

For additional information, see Notes for Int  13H  Function

05H.

9.6.9. Int 13H Function 08H

Description:

Get Drive Parameters [PC] [AT] [PS/2]

Returns various parameters for the specified drive.

Input                    Output

----------------------   -------------------------------------

AH=08H                   If function successful

DL=drive                   Carry flag=clear

00H-7FH floppy disk      BL=drive type

80H-FFH fixed disk         (PC/AT and PS/2 floppy disks)

01H if 360 KB, 40 track, 5.25"

02H if 1.2 MB, 80 track, 5.25"

03H if 720 KB, 80 track, 3.5"

04H if 1.44 MB, 80 track, 3.5"

CH=low 8 bits of maximum

cylinder number

CL=bits 6-7 high-order 2 bits of

maximum cylinder number

bits 0-5 maximum sector number

DH=maximum head number

DL=number of drives

ES:DI=segment:offset of disk drive

parameter table

If function unsuccessful

Carry flag=set

AH=status (see Int 13H Function 01H)

On the PC and PC/XT, this function  is  supported  on  fixed

disks only.

The value returned in register DL reflects the  true  number

of physical drives attached to the adapter  for  the  requested

drive.

9.6.10. Int 13H Function 09H

Description:

Initialize Fixed  Disk  Characteristics  [PC]  [AT]

[PS/2]

Initializes the fixed disk  controller  for  subsequent  I/O

operations, using  the  values  found  in  the  ROM  BIOS  disk

parameter block(s).

Function 13H (Disk I/O)

Input                                 Output

------------------------------------  -------------------------

AH=09H                                If function successful

DL=drive                                Carry flag=clear

80H-FFH fixed disk                   AH=00H

and, on the PC/XT                     If function unsuccessful

Vector for Int 41H must point           Carry flag=set

to disk parameter block                AH=status

or, on the PC/AT and PS/2            (see Int 13H Function 01H

Vector for Int 41H must point

to disk parameter block for drive 0

Vector for Int 46H must point

to disk parameter block for drive 1

This function is supported on fixed disks only.

For PC and PC/XT fixed disks, the parameter block format  is

as follows:

Byte(s)      Contents

-------      ---------------------------------------

00H-01H      maximum number of cylinders

02H          maximum number of heads

03H-04H      starting reduced write current cylinder

05H-06H      starting write precompensation cylinder

07H          maximum ECC burst length

08H          drive options

bit 7=1 if disable disk-access retries

bit 6=1 if disable ECC retries

bits 3-5=0

bits 0-2=drive option

09H          standard time-out value

0AH          time-out value for format drive

0BH          time-out value for check drive

0CH-0FH      reserved

For PC/AT and PS/2 fixed disks, the parameter  block  format

is as follows:

Byte(s)      Contents

-------      ---------------------------------------------

00H-01H      maximum number of cylinders

02H          maximum number of heads

03H-04H      reserved

05H-06H      starting write precompensation cylinder

07H          maximum ECC burst length

08H          drive options

bits 6-7=nonzero (10, 01, or 11) if retries di

bit 5=1 if manufacturer's defect map present

at maximum cylinder + 1

bit 4=not used

bit 3=1 if more than 8 heads

bits 0-2=not used

09H-0BH      reserved

0CH-0DH      landing zone cylinder

0EH          sectors per track

0FH          reserved

Function 13H (Disk I/O)

9.6.11. Int 13H Function 0AH

Description:

Read Sector Long [PC] [AT] [PS/2]

Reads a sector or sectors from disk into memory, along  with

a 4-byte ECC code for each sector.

Input                         Output

---------------------------   -----------------------------

AH=0AH                        If function successful

AL=number of sectors            Carry flag  =clear

CH=cylinder                     AH=00H

CL=sector (see Notes)           AL=number of sectors transferre

DH=head                       If function unsuccessful

DL=drive                        Carry flag=set

80H-FFH fixed disk            AH=status

ES:BX=segment:offset of buffer     (see Int 13H Function 01H)

This function is supported on fixed disks only.

The upper 2 bits of the 10-bit cylinder number are placed in

the upper 2 bits of register CL.

Unlike the normal Read Sector  function  (Int  13H  Function

02H), ECC errors are not automatically  corrected.  Multisector

transfers are terminated after any sector with a read error.

9.6.12. Int 13H Function 0BH

Description:

Write Sector Long [PC] [AT] [PS/2]

Writes a  sector  or  sectors  from  memory  to  disk.  Each

sector's worth of data must be followed by its 4-byte ECC code.

Input                         Output

----------------------------  ------------------------------

AH=0BH                        If function successful

AL=number of sectors            Carry flag=clear

CH=cylinder                     AH=00H

CL=sector (see Notes)           AL=number of sectors transferre

DH=head                       If function unsuccessful

DL=drive                        Carry flag=set

80H-FFH fixed disk            AH=status

ES:BX=segment:offset of buffer     (see Int 13H Function 01H)

This function is supported on fixed disks only.

The upper 2 bits of the 10-bit cylinder number are placed in

the upper 2 bits of register CL.

Function 13H (Disk I/O)

9.6.13. Int 13H Function 0CH

Description:

Seek [PC] [AT] [PS/2]

Positions  the  disk  read/write  heads  to  the   specified

cylinder, but does not transfer any data.

Input                         Output

---------------------------   --------------------------------

AH=0CH                        If function successful

CH=lower 8 bits of cylinder     Carry flag=clear

CL=upper 2 bits of cylinder     AH=00H

in bits 6-7                If function unsuccessful

DH=head                         Carry flag=set

DL=drive                        AH=status (see Int 13H

80H-FFH fixed disk                      Function 01H)

This function is supported on fixed disks only.

The upper 2 bits of the 10-bit cylinder number are placed in

the upper 2 bits of register CL.

The Read Sector, Read Sector Long, Write Sector,  and  Write

Sector Long functions include an  implied  seek  operation  and

need not be preceded by an explicit call to this function.

9.6.14. Int 13H Function 0DH

Description:

Reset Fixed Disk System [PC] [AT] [PS/2]

Resets the  fixed  disk  controller,  recalibrates  attached

drives (moves the read/write arm to cylinder 0),  and  prepares

for subsequent disk I/O.

Input                   Output

---------------------   ------------------------------------

AH=0DH                  If function successful

DL=drive                  Carry flag=clear

80H-FFH fixed disk      AH=00H

If function unsuccessful

Carry flag=set

AH=status (see Int 13H Function 01H)

This function is supported on fixed disks only.  It  differs

from Int 13H Function 00H in that the floppy disk controller is

not reset.

9.6.15. Int 13H Function 0EH

Description:

Read Sector Buffer [PC]

Function 13H (Disk I/O)

Transfers the contents of the fixed disk adapter's  internal

sector buffer to system  memory.  No  data  is  read  from  the

physical disk drive.

Input                           Output

------------------------------  --------------------------

AH=0EH                          If function successful

ES:BX=segment:offset of buffer    Carry flag=clear

If function unsuccessful

Carry flag=set

AH=status

(see Int 13H Function 01H)

This function is supported by the PC/XT's fixed disk adapter

only. It is not defined for fixed disk adapters on the PC/AT or

PS/2.

9.6.16. Int 13H Function 0FH

Description:

Write Sector Buffer [PC]

Transfers  data  from  system  memory  to  the  fixed   disk

adapter's internal sector buffer. No data  is  written  to  the

physical disk drive.

Input                            Output

------------------------------   --------------------------

AH=0FH                           If function successful

ES:BX=segment:offset of buffer     Carry flag=clear

If function unsuccessful

Carry flag=set

AH=status

(see Int 13H Function 01H)

This function is supported by the PC/XT's fixed disk adapter

only. It is not defined for fixed disk adapters on the PC/AT or

PS/2.

This function should be called to initialize the contents of

the sector buffer before formatting  the  drive  with  Int  13H

Function 05H.

9.6.17. Int 13H Function 10H

Description:

Get Drive Status [PC] [AT] [PS/2]

Tests whether the specified fixed disk drive is  operational

and returns the drive's status.

Function 13H (Disk I/O)

Input                   Output

---------------------   ------------------------------------

AH=10H                  If function successful

DL=drive                  Carry flag=clear

80H-FFH fixed disk      AH=00H

If function unsuccessful

Carry flag=set

AH=status (see Int 13H Function 01H)

This function is supported on fixed disks only.

9.6.18. Int 13H Function 11H

Description:

Recalibrate Drive [PC] [AT] [PS/2]

Causes the fixed disk adapter to recalibrate itself for  the

specified drive, positioning the read/arm to  cylinder  0,  and

returns the drive's status.

Input                  Output

---------------------  ------------------------------------

AH=11H                 If function successful

DL=drive                 Carry flag=clear

80H-FFH fixed disk     AH=00H

If function unsuccessful

Carry flag=set

AH=status (see Int 13H Function 01H)

This function is supported on fixed disks only.

9.6.19. Int 13H Function 12H

Description:

Controller RAM Diagnostic [PC]

Causes the fixed  disk  adapter  to  carry  out  a  built-in

diagnostic test  on  its  internal  sector  buffer,  indicating

whether the test was passed by the returned status.

Input             Output

------            ------------------------------------

AH=12H            If function successful

Carry flag=clear

If function unsuccessful

Carry flag=set

AH=status (see Int 13H Function 01H)

This function is supported on PC/XT fixed disks only.

Function 13H (Disk I/O)

9.6.20. Int 13H Function 13H

Description:

Controller Drive Diagnostic [PC]

Causes the fixed disk adapter  to  run  internal  diagnostic

tests of the attached drive, indicating whether  the  test  was

passed by the returned status.

Input             Output

------            ------------------------------------

AH=13H            If function successful

Carry flag=clear

If function unsuccessful

Carry flag=set

AH=status (see Int 13H Function 01H)

This function is supported on PC/XT fixed disks only.

9.6.21. Int 13H Function 14H

Description:

Controller Internal Diagnostic [PC] [AT] [PS/2]

Causes the fixed  disk  adapter  to  carry  out  a  built-in

diagnostic self-test, indicating whether the test was passed by

the returned status.

Input             Output

------            ------------------------------------

AH=14H            If function successful

Carry flag=clear

AH=00H

If function unsuccessful

Carry flag=set

AH=status (see Int 13H Function 01H)

This function is supported on fixed disks only.

9.6.22. Int 13H Function 15H

Description:

Get Disk Type [AT] [PS/2]

Returns a code indicating the type of floppy or  fixed  disk

referenced by the specified drive code.

Function 13H (Disk I/O)

Input                   Output

----------------------  ------------------------------------

AH=15H                  If function successful

DL=drive                  Carry flag=clear

00H-7FH floppy disk     AH=drive type code

80H-FFH fixed disk        00H if no drive present

01H if floppy disk drive

without change-line support

02H if floppy disk drive

with change-line support

03H if fixed disk

and, if fixed disk (AH=03H)

CX:DX=number of 512-byte sectors

If function unsuccessful

Carry flag=set

AH=status (see Int 13H Function 01H)

This function is not supported on the PC or PC/XT.

9.6.23. Int 13H Function 16H

Description:

Get Disk Change Status [AT] [PS/2]

Returns the status of the change  line,  indicating  whether

the disk in the drive may have been  replaced  since  the  last

disk access.

Input                    Output

----------------------   ------------------------------

AH=16H                   If change line inactive

DL=drive                    (disk has not been changed)

00H-7FH floppy disk      Carry flag=clear

AH=00H

If change line active

(disk may have been changed)

Carry flag=set

AH=06H

If this function returns with the carry flag set,  the  disk

has not necessarily  been  changed;  the  change  line  can  be

activated by simply unlocking and locking the disk  drive  door

without removing the floppy disk.

This function is not supported for floppy disks on the PC or

PC/XT.

9.6.24. Int 13H Function 17H

Description:

Set Disk Type [AT] [PS/2]

Selects a floppy disk type for the specified drive.

Function 13H (Disk I/O)

Input                           Output

-----------------------------   ----------------------------

AH=17H                          If function successful

AL=floppy disk type code          Carry flag=clear

00H not used                    AH=00H

01H 320/360 KB floppy disk    If function unsuccessful

in 360 KB drive             Carry flag=set

02H 320/360 KB floppy disk      AH=status

in 1.2 MB drive             (see Int 13H Function 01H)

03H 1.2 MB floppy disk

in 1.2 MB drive

04H 720 KB floppy disk

in 720 KB drive

DL=drive

00H-7FH floppy disk

This function is not supported for floppy disks on the PC or

PC/XT.

If the change line is active for the specified drive, it  is

reset. The ROM BIOS then sets the data rate for  the  specified

drive and media type.

9.6.25. Int 13H Function 18H

Description:

Set Media Type for Format [AT] [PS/2]

Selects media characteristics for the specified drive.

Input                   Output

----------------------  -------------------------------------

AH=18H                  If function successful

CH=number of cylinders    Carry flag=clear

CL=sectors per track      AH=00H

DL=drive                  ES:DI=segment:offset of disk

00H-7FH floppy disk           parameter table for media type

If function unsuccessful

Carry flag=set

AH=status (see Int 13H Function 01H)

A floppy disk must be present in the drive.

This function should be called prior to  formatting  a  disk

with Int 13H Function 05H so that the  ROM  BIOS  can  set  the

correct data rate for the media.

If the change line is active for the specified drive, it  is

reset.

9.6.26. Int 13H Function 19H

Description:

Park Heads [PS/2]

Moves the read/write arm to a track that  is  not  used  for

Function 13H (Disk I/O)

data storage, so that data will not be damaged when  the  drive

is turned off.

Input                            Output

---------------------            ---------------------------

AH=19H                           If function successful

DL=drive                         Carry flag =clear

80H-FFH fixed disk            If function unsuccessful

Carry flag=set

AH=status (see Int 13H

Function 01H)

This function is defined for PS/2 fixed disks only.

9.6.27. Int 13H Function 1AH

Description:

Format ESDI Drive [PS/2]

Initializes disk sector and track address fields on a  drive

attached to the ESDI Fixed Disk Drive Adapter/A.

Input                                  Output

------------------------------------   -----------------------

AH=1AH                                 If function successful

AL=relative block address                Carry flag=clear

(RBA) defect table count              AH=00H

0 if no RBA table                    If function unsuccessful

>0 if RBA table used                    Carry flag=set

CL=format modifier bits                  AH=status

Bit(s)     Significance (if set)    (see Int 13H

------     ----------------------        Function 01H)

0          ignore primary

defect map

1          ignore secondary

defect map

2          update secondary

defect map (see Notes)

3          perform extended

surface analysis

4          generate periodic

interrupt (see Notes)

5-7        reserved (must be 0)

DL=drive

80H-FFH fixed disk

ES:BX=segment:offset of RBA table

This operation is sometimes called a "low level format"  and

prepares the disk for physical  read/write  operations  at  the

sector level. The drive must be subsequently  partitioned  with

the FDISK command and then given a "high level format" with the

FORMAT command to install a file system.

If bit 4 of register CL is  set,  Int  15H  is  called  with

AH=0FH and AL=phase code after each cylinder  is  formatted  or

analyzed. The phase code is defined as:

Function 13H (Disk I/O)

0=reserved

1=surface analysis

2=formatting

See also Int 15H Function 0FH.

If bit 2 of register CL is set, the drive's secondary defect

map is updated to reflect errors found during surface analysis.

If both bit 2 and bit 1 are set, the secondary  defect  map  is

replaced.

For an extended surface analysis, the disk should  first  be

formatted by calling this function with bit 3 cleared and  then

analyzed by calling this function with bit 3 set.

9.7. Interrupt 14H (COM Port)

Interrupt  Function   Name

14H        00H        Initialize Communications Port

14H        01H        Write Character to Communications Port

14H        02H        Read Character from Communications Port

14H        03H        Get Communications Port Status

14H        04H        Extended Initialize Communications Port

14H        05H        Extended Communications Port Control

9.7.1. Int 14H Function 00H

Description:

Initialize Communications Port [PC] [AT] [PS/2]

Initializes a serial communications port to a  desired  baud

rate, parity, word length, and number of stop bits.

Interrupt 14H (COM Port)

Input                          Output

-----------------------------  -------------------------------

AH=00H                         AH=port status

AL=initialization parameter     Bit(s)  Significance (if set)

(see Notes)                  ------  ----------------------

DX=communications port number   7       timed-out

(0=COM1, 1=COM2,   etc.)      6       transmit shift

register empty

5       transmit holding

register empty

4       break detected

3       framing error detected

2       parity error detected

1       overrun error detected

0       receive data ready

AL=modem status

Bit(s)  Significance (if set)

------  ----------------------

7       receive line signal

detect

6       ring indicator

5       data-set-ready

4       clear-to-send

3       change in receive line

signal detect

2       trailing edge ring

indicator

1       change in

data-set-ready status

0       change in

clear-to-send status

The initialization parameter byte is defined as follows:

7 6 5         4 3          2              1 0

Baud Rate     Parity       Stop Bits      Word Length

---------     -------      ---------      -----------

000=110       X0=none      0=1 bit        10=7 bits

001=150       01=odd       1=2 bits       11=8 bits

010=300       11=even

011=600

100=1200

101=2400

110=4800

111=9600

To initialize the serial port for data  rates  greater  than

9600 baud on PS/2 machines, see Int 14H Functions 04H and  05H.

9.7.2. Int 14H Function 01H

Description:

Write Character to Communications  Port  [PC]  [AT]

[PS/2]

Writes a character to the  specified  serial  communications

port, returning the current status of the port.

Interrupt 14H (COM Port)

Input                          Output

-----------------------------  ----------------------------

AH=01H                         If function successful

AL=character                     AH bit 7=0

DX=communications port number    AH bits 0-6=port status

(0=COM1, 1=COM2,   etc.)        Bit(s)  Significance (if set)

------  ------------------

6       transmit shift

register empty

5       transmit holding

register empty

4       break detected

3       framing error

detected

2       parity error detec

1       overrun error

detected

0       receive data ready

AL=character (unchanged)

If function unsuccessful (timed-

AH bit 7=1

AL=character (unchanged)

9.7.3. Int 14H Function 02H

Description:

Read Character from Communications Port  [PC]  [AT]

[PS/2]

Reads a character from the specified  serial  communications

port, also returning the port's status.

Input                          Output

-----------------------------  ----------------------------

AH=02H                         If function successful

DX=communications port number    AH bit 7=0

(0=COM1, 1=COM2,   etc.)      AH bits 0-6=status

Bit(s)  Significance (if s

------  ------------------

4       break detected

3       framing error dete

2       parity error detec

1       overrun error dete

AL=character

If function unsuccessful (timed-

AH bit 7=1

9.7.4. Int 14H Function 03H

Description:

Get Communications Port Status [PC] [AT] [PS/2]

Interrupt 14H (COM Port)

Returns the status of the  specified  serial  communications

port.

Input               Output

-----------------   ------------------------------------------

AH=03H              AH=port status (see Int 14H Function 00H)

DX=communications   AL=modem status (see Int 14H Function 00H)

port number

(0=COM1,1=COM2,etc.)

9.7.5. Int 14H Function 04H

Description:

Extended Initialize Communications Port [PS/2]

Initializes a serial communications port to a  desired  baud

rate, parity, word length, and number of stop bits. Provides  a

superset  of  Int  14H  Function  00H  capabilities  for   PS/2

machines.

Input                          Output

---------------------------    -----------------------------

AH=04H                         AH=port status

AL=break flag                     (see Int 14H Function 00H)

00H no break                 AL=modem status

01H break                       (see Int 14H Function 00H)

BH=parity

00H none

01H odd

02H even

03H stick parity odd

04H stick parity even

BL=stop bits

00H 1 stop bit

01H 2 stop bits if

word length=6-8 bits

01H 1.5 stop bits if

word length=5 bits

CH=word length

00H 5 bits

01H 6 bits

02H 7 bits

03H 8 bits

CL=baud rate

00H 110 baud

01H 150 baud

02H 300 baud

03H 600 baud

04H 1200 baud

05H 2400 baud

06H 4800 baud

07H 9600 baud

08H 19,200 baud

DX=communications port number

(0=COM1, 1=COM2,   etc.)

Interrupt 14H (COM Port)

9.7.6. Int 14H Function 05H

Description:

Extended Communications Port Control [PS/2]

Reads or sets the  modem  control  register  (MCR)  for  the

specified serial communications port.

Input                              Output

---------------------------------  ---------------------------

AH=05H                             If called with AL=00H

AL=subfunction                       BL=modem control register

00H to read modem                  contents (see above)

control register          If called with AL=01H

01H to write modem              AH=port status

control register            (see Int 14H Function 00H)

BL=modem control register            AL=modem status

contents (if AL=01H)             (see Int 14H Function 00H)

Bit(s)     Significance

------     -------------------

0          data-terminal ready

1          request-to-send

2          Out1

3          Out2

4          loop (for testing)

5-7        reserved

DX=communications port number

(0=COM1, 1=COM2, etc.)

9.8. Interrupt 15H (I/O Subsystem)

Interrupt 15H (I/O Subsystem)

Interrupt  Fnct  Subfnct  Name

15H        00H            Turn On Cassette Motor

15H        01H            Turn Off Cassette Motor

15H        02H            Read Cassette

15H        03H            Write Cassette

15H        0FH            Format ESDI Drive Periodic Interrupt

15H        21H   00H      Read POST Error Log

15H        21H   01H      Write POST Error Log

15H        4FH            Keyboard Intercept

15H        80H            Device Open

15H        81H            Device Close

15H        82H            Process Termination

15H        83H            Event Wait

15H        84H            Read Joystick

15H        85H            SysReq Key

15H        86H            Delay

15H        87H            Move Extended Memory Block

15H        88H            Get Extended Memory Size

15H        89H            Enter Protected Mode

15H        90H            Device Wait

15H        91H            Device Post

15H        C0H            Get System Environment

15H        C1H            Get Address of Extended BIOS Data Are

15H        C2H            Mouse Pointer (summary)

15H        C3H            Set Watchdog Time-Out

15H        C4H            Programmable Option Select

9.8.1. Int 15H Function 00H

Description:

Turn On Cassette Motor [PC]

Turns on the motor of the cassette tape drive.

Input             Output

------            ------------------------------

AH=00H            If function successful

Carry flag=clear

If function unsuccessful

Carry flag=set

AH=status

86H if cassette not present

This function is available only on the PC and the  PCjr.  It

is not supported on the PC/XT and all subsequent models.

9.8.2. Int 15H Function 01H

Description:

Turn Off Cassette Motor [PC]

Turns off the motor of the cassette tape drive.

Interrupt 15H (I/O Subsystem)

Input             Output

------            ------------------------------

AH=01H            If function successful

Carry flag=clear

If function unsuccessful

Carry flag=set

AH=status

86H if cassette not present

This function is available only on the PC and the  PCjr.  It

is not supported on the PC/XT and all subsequent models.

9.8.3. Int 15H Function 02H

Description:

Read Cassette [PC]

Reads one or more 256-byte blocks of data from the  cassette

tape drive to memory.

Input                            Output

------------------------------   -----------------------------

AH=02H                           If function successful

CX=number of bytes to read         Carry flag=clear

ES:BX=segment:offset of buffer     DX=number of bytes

actually read

ES:BX=segment:offset + 1 of

last byte read

If function unsuccessful

Carry flag=set

AH=status

01H if CRC error

02H if bit signals scrambled

04H if no data found

80H if invalid command

86H if cassette not present

This function is available only on the PC and the  PCjr.  It

is not supported on the PC/XT and all subsequent models.

9.8.4. Int 15H Function 03H

Description:

Write Cassette [PC]

Writes one or more 256-byte blocks of data  from  memory  to

the cassette tape drive.

Interrupt 15H (I/O Subsystem)

Input                            Output

------------------------------   ------------------------------

AH=03H                           If function successful

CX=number of bytes to write        Carry flag=clear

ES:BX=segment:offset of buffer     CX=00H

ES:BX=segment:offset + 1 of

last byte written

If function unsuccessful

Carry flag=set

AH=status

80H if invalid command

86H if cassette not present

9.8.5. Int 15H Function 0FH

Description:

Format ESDI Drive Periodic Interrupt [PS/2]

Invoked by the  ROM  BIOS  on  the  ESDI  Fixed  Disk  Drive

Adapter/A during a format or surface analysis  operation  after

each cylinder is completed.

Input                   Output

---------------------   ---------------------------------------

AH=0FH                  If formatting or analysis should contin

AL=phase code             Carry flag=clear

0=reserved            If formatting or analysis should

1=surface analysis       be terminated

2=formatting            Carry flag=set

This function call can be captured by a program so  that  it

will be notified as each cylinder is formatted or analyzed. The

program can count interrupts for each phase  to  determine  the

current cylinder number.

The ROM BIOS default handler for this function returns  with

the carry flag set.

9.8.6. Int 15H Function 21H Subfunction 00H

Description:

Read POST Error Log [PS/2]

Returns error information that was  accumulated  during  the

most recent power-on self-test (POST).

Interrupt 15H (I/O Subsystem)

Input             Output

------            --------------------------------------

AH=21H            If function successful

AL=00H            Carry flag=clear

AH=00H

BX=number of POST error codes stored

ES:DI=segment:offset of POST error log

If function unsuccessful

Carry flag=set

AH=status

80H=invalid command

86H=function not supported

The error log consists of  single-word  entries.  The  first

byte of an entry is the device error code; the  second  is  the

device identifier.

This function is not available on the PS/2 Models 25 and 30.

9.8.7. Int 15H Function 21H Subfunction 01H

Description:

Write POST Error Log [PS/2]

Adds an entry to the power-on self-test (POST) error log.

Input                  Output

--------------------   -----------------------------

AH=21H                 If function successful

AL=01H                   Carry flag=clear

BH=device identifier     AH=00H

BL=device error code   If function unsuccessful

Carry flag=set

AH=status

01H=error list full

80H=invalid command

86H=function not supported

This function is not available on the PS/2 Models 25 and 30.

9.8.8. Int 15H Function 4FH

Description:

Keyboard Intercept [PS/2]

Invoked for  each  keystroke  by  the  ROM  BIOS's  Int  09H

keyboard interrupt handler.

Interrupt 15H (I/O Subsystem)

Input                   Output

------------            -----------------------------

AH=4FH                  If scan code consumed

AL=scan code            Carry flag=clear

If scan code not consumed

Carry flag=set

AL=unchanged or new scan code

An operating system or a resident utility can  capture  this

function to filter  the  raw  keyboard  data  stream.  The  new

handler can substitute a new scan code, return  the  same  scan

code, or return the carry flag clear causing the  keystroke  to

be discarded. The ROM BIOS default routine simply  returns  the

scan code unchanged.

A program can call Int 15H Function C0H to determine whether

the host machine's ROM BIOS supports this keyboard intercept.

9.8.9. Int 15H Function 80H

Description:

Device Open [AT] [PS/2]

Acquires ownership of a logical device for a process.

Input                    Output

-------------            ------------------------

AH=80H                   If function successful

BX=device ID             Carry flag=clear

CX=process ID            AH=00H

If function unsuccessful

Carry flag=set

AH=status

This function call, along with Int  15H  Functions  81H  and

82H, defines a simple protocol that can be  used  to  arbitrate

usage of devices by multiple processes. A multitasking  program

manager would be expected to capture Int 15H  and  provide  the

appropriate service. The default BIOS routine for this function

simply returns with the carry flag clear and AH=00H.

9.8.10. Int 15H Function 81H

Description:

Device Close [AT] [PS/2]

Releases ownership of a logical device for a process.

Interrupt 15H (I/O Subsystem)

Input                    Output

-------------            ------------------------

AH=81H                   If function successful

BX=device ID             Carry flag=clear

CX=process ID            AH=00H

If function unsuccessful

Carry flag=set

AH=status

A multitasking program manager would be expected to  capture

Int 15H and provide  the  appropriate  service.  The  ROM  BIOS

default routine for this function simply returns with the carry

flag clear and AH=00H. See also Int 15H Functions 80H and  82H.

9.8.11. Int 15H Function 82H

Description:

Process Termination [AT] [PS/2]

Releases ownership of all logical devices for a process that

is about to terminate.

Input                    Output

-------------            ------------------------

AH=82H                   If function successful

BX=process ID            Carry flag=clear

AH=00H

If function unsuccessful

Carry flag=set

AH=status

A multitasking program manager would be expected to  capture

Int 15H and provide  the  appropriate  service.  The  ROM  BIOS

default routine for this function simply returns with the carry

flag clear and AH=00H. See also Int 15H Functions 80H and  81H.

9.8.12. Int 15H Function 83H

Description:

Event Wait [AT] [PS/2]

Requests setting of a semaphore after a  specified  interval

or cancels a previous request.

Interrupt 15H (I/O Subsystem)

Input                       Output

--------------------------  -----------------------------------

If requesting event wait    If called with AL=00H,

AH=83H                         and function successful

AL=00H                        Carry flag=clear

CX:DX=microseconds          If called with AL=00H, and function

ES:BX=segment:offset of        unsuccessful (Event Wait already

semaphore byte           active)

If canceling event wait       Carry flag=set

AH=83H                      If called with AL=01H

AL=01H                        Nothing

The function call returns immediately. If  the  function  is

successful, bit 7  of  the  semaphore  byte  is  set  when  the

specified  interval  has  elapsed.  The  calling   program   is

responsible for clearing the semaphore before  requesting  this

function.

The actual duration of an event wait is always  an  integral

multiple  of  976  microseconds.  The  CMOS   date/clock   chip

interrupts are used to implement this function.

Use of this function allows programmed, hardware-independent

delays at a finer resolution than can be obtained  through  use

of the MS-DOS Get Time function (Int 21H  Function  2CH,  which

returns time in hundredths of a second).

See also Int 15H Function 86H, which  suspends  the  calling

program for the specified interval in milliseconds.

This function is not supported on the PS/2 Models 25 and 30.

9.8.13. Int 15H Function 84H

Description:

Read Joystick [AT] [PS/2]

Returns  the  joystick  switch  settings  and  potentiometer

values.

Input                            Output

-------------------------------  ------------------------------

AH=84H                           If function successful

DX=subfunction                     Carry flag=clear

00H to read switch settings     and, if called with DX=00H

01H to read resistive inputs    AL=switch settings (bits 4-7

or, if called with DX=01H

AX=A(x) value

BX=A(y) value

CX=B(x) value

DX=B( y) value

If function unsuccessful

Carry flag=set

An error condition is returned if  DX  does  not  contain  a

valid subfunction number.

If no game adapter is installed, AL is returned as  00H  for

Subfunction 00H (i.e., all switches open); AX, BX, CX,  and  DX

are returned containing 00H for Subfunction 01H.

Using a 250 KOhm joystick, the potentiometer values  usually

lie within the range 0-416 (0000-01A0H).

Interrupt 15H (I/O Subsystem)

9.8.14. Int 15H Function 85H

Description:

SysReq Key [AT] [PS/2]

Invoked by the ROM BIOS keyboard driver when the SysReq  key

is detected.

Input                              Output

-------------------------------    ----------------------------

AH=85H                             If function successful

AL=key status                        Carry flag=clear

00H if key make (depression)      AH=00H

01H if key break (release)      If function unsuccessful

Carry flag=set

AH=status

The ROM BIOS handler for  this  function  call  is  a  dummy

routine that always returns a success status unless called with

an invalid subfunction number in  AL.  A  multitasking  program

manager would be expected to capture Int 15H so that it can  be

notified when the user strikes the SysReq key.

9.8.15. Int 15H Function 86H

Description:

Delay [AT] [PS/2]

Suspends the calling program for  a  specified  interval  in

microseconds.

Input                         Output

--------------------------    --------------------------

AH=86H                        If function successful

CX:DX=microseconds to wait       (wait was performed)

Carry flag=clear

If function unsuccessful

(wait was not performed)

Carry flag=set

The actual duration  of  the  wait  is  always  an  integral

multiple of 976 microseconds.

Use of this function allows programmed, hardware-independent

delays at a finer resolution than can be obtained  through  use

of the MS-DOS Get Time function (Int 21H  Function  2CH,  which

returns time in hundredths of a second).

See also Int 15H Function 83H, which  triggers  a  semaphore

after a specified interval but does  not  suspend  the  calling

program.

Interrupt 15H (I/O Subsystem)

9.8.16. Int 15H Function 87H

Description:

Move Extended Memory Block [AT] [PS/2]

Transfers data  between  conventional  memory  and  extended

memory.

Input                             Output

------------------------------    -----------------------------

AH=87H                            If function successful

CX=number of words to move          Carry flag=clear

ES:SI=segment:offset of Global      AH=00H

Descriptor Table            If function unsuccessful

(see Notes)                   Carry flag=set

AH=status

01H if RAM parity error

02H if exception interrupt

error

03H if gate address line 2

failed

Conventional memory lies  at  addresses  below  the  640  KB

boundary, and is used for  the  execution  of  MS-DOS  and  its

application programs. Extended memory lies at addresses above 1

MB, and can only be accessed by an 80286 or 80386  CPU  running

in protected mode. As much as 15 MB of extended memory  can  be

installed in a PC/AT or compatible.

The Global Descriptor Table (GDT) used by this function must

be set up as follows:

Byte(s)     Contents

-------     -------------------------------------------------

00H-0FH     reserved (should be 0)

10H-11H     segment length in bytes (2 * CX - 1 or greater)

12H-14H     24-bit source address

15H         access rights byte (always 93H)

16H-17H     reserved (should be 0)

18H-19H     segment length in bytes (2 * CX - 1 or greater)

1AH-1CH     24-bit destination address

1DH         access rights byte (always 93H)

1EH-2FH     reserved (should be 0)

The table is composed of six 8-byte descriptors to  be  used

by the CPU in protected mode. The four descriptors  in  offsets

00H-0FH and 20H-2FH are filled in by the ROM  BIOS  before  the

CPU mode switch.

The addresses  used  in  the  descriptor  table  are  linear

(physical) 24-bit addresses in the range 000000H-FFFFFFH -- not

segments and offsets -- with the least significant byte at  the

lowest address and the most significant  byte  at  the  highest

address.

The block move is performed with interrupts disabled;  thus,

use of this  function  may  interfere  with  the  operation  of

communications programs, network  drivers,  or  other  software

that relies on prompt servicing of hardware interrupts.

Programs and drivers that access extended memory  with  this

function cannot be executed in the Compatibility Environment of

OS/2.

This function is not supported on the PS/2 Models 25 and 30.

Interrupt 15H (I/O Subsystem)

9.8.17. Int 15H Function 88H

Description:

Get Extended Memory Size [AT] [PS/2]

Returns the amount  of  extended  memory  installed  in  the

system.

Input             Output

------            ------------------------------------

AH=88H            AX=amount of extended memory (in KB)

Extended memory is memory at addresses above 1 MB, which can

only be accessed by an 80286 or 80386 CPU running in  protected

mode. Because MS-DOS is a real-mode operating system,  extended

memory can be used for storage of volatile data but  cannot  be

used for execution of programs.

Programs and  drivers  that  use  this  function  cannot  be

executed in the Compatibility Environment of OS/2.

This function is not supported on the PS/2 Models 25 and 30.

9.8.18. Int 15H Function 89H

Description:

Enter Protected Mode [AT] [PS/2]

Switches the CPU from real mode into protected mode.

Input                               Output

----------------------------------  ---------------------------

AH=89H                              If function successful

BH=interrupt number for IRQ0,          (CPU is in protected mod

written to ICW2 of 8259 PIC #1     Carry flag=clear

(must be evenly divisible by 8,    AH=00H

determines IRQ0-IRQ7)              CS=user-defined selector

BL=interrupt number for IRQ8,         DS=user-defined selector

written to ICW2 of 8259 PIC #2     ES=user-defined selector

(must be evenly divisible by 8,    SS=user-defined selector

determines IRQ8-IRQ15)           If function unsuccessful

ES:SI=segment:offset of Global         (CPU is in real mode)

Descriptor Table (GDT)          Carry flag=set

AH=FFH

The Global Descriptor Table must contain  eight  descriptors

set up as follows:

Interrupt 15H (I/O Subsystem)

Offset            Descriptor usage

------            -----------------------------------

00H               dummy descriptor (initialized to 0)

08H               Global Descriptor Table (GDT)

10H               Interrupt Descriptor Table (IDT)

18H               user's data segment (DS)

20H               user's extra segment (ES)

28H               user's stack segment (SS)

30H               user's code segment (CS)

38H               BIOS code segment

The user must initialize the first  seven  descriptors;  the

eighth is filled in by the ROM BIOS to  provide  addressability

for its own execution. The calling program may modify  and  use

the eighth descriptor for any purpose after  return  from  this

function call.

This function is not supported on the PS/2 Models 25 and 30.

9.8.19. Int 15H Function 90H

Description:

Device Wait [AT] [PS/2]

Invoked by the ROM BIOS fixed disk,  floppy  disk,  printer,

network, and keyboard drivers prior to performing a  programmed

wait for I/O completion.

Input                                      Output

-------------------------------  ------------------------------

AH=90H                           If no wait (driver must perfor

AL=device type                               its own time-out)

00H-7FH serially reusable        Carry flag=clear

devices                  AH=00H

80H-BFH reentrant devices      If wait was performed

C0H-FFH wait-only calls,         Carry flag=set

no corresponding

Post function

ES:BX=segment:offset of request

block for device

types   80H-FFH

Predefined device types are:

00H disk (may time-out)

01H floppy disk (may time-out)

02H keyboard (no time-out)

03H pointing device (PS/2, may time-out)

80H network (no time-out)

FCH fixed disk reset (PS/2, may time-out)

FDH floppy disk drive motor start (may time-out)

FEH printer (may time-out)

For network adapters, ES:BX  points  to  a  network  control

block (NCB).

A multitasking program manager would be expected to  capture

Int 15H Function 90H so that it can dispatch other tasks  while

I/O is in progress. The default BIOS routine for this  function

simply returns with the carry flag clear and AH=00H.

Interrupt 15H (I/O Subsystem)

9.8.20. Int 15H Function 91H

Description:

Device Post [AT] [PS/2]

Invoked by the ROM BIOS fixed disk,  floppy  disk,  network,

and keyboard drivers to signal that I/O is complete and/or  the

device is ready.

Input                                             Output

--------------------------------------            ------

AH=91H                                            AH=00H

AL=device type

00H-7FH serially reusable devices

80H-BFH reentrant devices

ES:BX=segment:offset of request block

for device types 80H-BFH

Predefined device types that may use Device Post are:

00H  disk (may time-out)

01H  floppy disk (may time-out)

02H  keyboard (no time-out)

03H  pointing device (PS/2, may time-out)

80H  network (no time-out)

The ROM BIOS printer routine does not invoke  this  function

because printer output is not interrupt driven.

A multitasking program manager would be expected to  capture

Int 15H Function 91H so that it can be  notified  when  I/O  is

completed and awaken the  requesting  task.  The  default  BIOS

routine for this function simply returns with  the  carry  flag

clear and AH=00H.

9.8.21. Int 15H Function C0H

Description:

Get System Environment [AT] [PS/2]

Returns a  pointer  to  a  table  containing  a  variety  of

information about the system configuration.

Input    Output

------   ------------------------------------------------------

AH=C0H   ES:BX=segment:offset of configuration table (see Notes

The format of the system configuration table is as  follows:

Interrupt 15H (I/O Subsystem)

Byte(s)            Contents

-------            --------------------------------------------

00H-01H            length of table in bytes

02H                system model (see following Note)

03H                system submodel (see following Note)

04H                BIOS revision level

05H                configuration flags

Bit(s)  Significance (if set)

------  --------------------------------------

7       DMA channel 3 used

6       slave 8259 present (cascaded IRQ2)

5       real-time clock available

4       keyboard intercept

(Int 15H Function 4FH) available

3       Wait for External Event is available

2       extended BIOS data area allocated

1       Micro Channel implemented

0       reserved

06H-09H            reserved

The system model and type bytes are assigned as follows:

Machine                   Model Byte            Submodel Byte

-------                   ----------            -------------

PC                        FFH

PC/XT                     FEH

PC/XT                     FBH                   00H or 01H

PCjr                      FDH

PC/AT                     FCH                   00H or 01H

PC/XT-286                 FCH                   02H

PC Convertible            F9H

PS/2 Model 30             FAH                   00H

PS/2 Model 50             FCH                   04H

PS/2 Model 60             FCH                   05H

PS/2 Model 80             F8H                   00H or 01H

9.8.22. Int 15H Function C1H

Description:

Get Address of Extended BIOS Data Area [PS/2]

Returns the segment address of the base of the extended BIOS

data area.

Input             Output

------            -------------------------------------

AH=C1H            If function successful

Carry flag=clear

ES=segment of extended BIOS data area

If function unsuccessful

Carry flag=set

The extended BIOS data area is allocated at the high end  of

conventional  memory  during  the   POST   (Power-On-Self-Test)

sequence. The word at 0040:0013H (memory size)  is  updated  to

reflect the reduced amount of memory available for  MS-DOS  and

Interrupt 15H (I/O Subsystem)

application programs. The first byte in the extended BIOS  data

area is initialized to its length in KB.

A program can determine whether the extended BIOS data  area

exists with Int 15H Function C0H.

9.8.23. Int 15H Function C3H

Description:

Set Watchdog Time-Out [PS/2]

Enables or disables a watchdog timer.

Input                                  Output

-------------------------------------  ------------------------

AH=C3H                                 If function successful

AL=subfunction                           Carry flag=clear

00H to disable watchdog time-out     If function unsuccessful

01H to enable watchdog time-out        Carry flag=set

BX=watchdog timer counter (if AL=01H)

The watchdog timer generates an NMI interrupt.

This function is not available on the PS/2 Models 25 and 30.

9.8.24. Int 15H Function C4H

Description:

Programmable Option Select [PS/2]

Returns  the  base  Programmable  Option   Select   register

address, enables a slot for setup, or enables an adapter.

Input                               Output

--------------------------------    -------------------------

AH=C4H                              If function successful

AL=subfunction                        Carry flag=clear

00H to return base POS adapter      and, if called with AL=00

register address                DX=base POS adapter

01H to enable slot                  register address

02H to enable adapter             If function unsuccessful

BL=slot number (if AL=01H)            Carry flag=set

This function is available only on machines using the  Micro

Channel Architecture (MCA) bus.

After a slot  is  enabled  with  Subfunction  01H,  specific

information can be obtained for the adapter  in  that  slot  by

performing port input operations:

Interrupt 15H (I/O Subsystem)

Port            Function

----            ----------------------------------

100H            MCA ID (low byte)

101H            MCA ID (high byte)

102H            Option Select Byte 1

bit 0=1 if enabled,=0 if disabled

103H            Option Select Byte 2

104H            Option Select Byte 3

105H            Option Select Byte 4

bits 6-7=channel check indicators

106H            Subaddress Extension (low byte)

107H            Subaddress Extension (high byte)

9.8.25. Interrupt 15H (Mouse Pointer)

Interrupt  Fnct  Subfnct    Name

15H        C2H   00H        Enable/Disable Pointing Device

15H        C2H   01H        Reset Pointing Device

15H        C2H   02H        Set Sample Rate

15H        C2H   03H        Set Resolution

15H        C2H   04H        Get Pointing Device Type

15H        C2H   05H        Initialize Pointing Device Interfac

15H        C2H   06H        Set Scaling or Get Status

15H        C2H   07H        Set Pointing Device Handler Address

9.8.25.1. Int 15H Function C2H Subfunction  00H

Description:

Enable/Disable Pointing Device [PS/2]

Enables or disables the system's  mouse  or  other  pointing

device.

Input                    Output

----------------------   -------------------------------

AH=C2H                   If function successful

AL=00H                     Carry flag=clear

BH=enable/disable flag     AH=00H

00H=disable            If function unsuccessful

01H=enable               Carry flag=set

AH=status

01H if invalid function call

02H if invalid input

03H if interface error

04H if resend

05H if no far call installed

Interrupt 15H (I/O Subsystem)

9.8.25.2. Int 15H Function C2H Subfunction  01H

Description:

Reset Pointing Device [PS/2]

Resets the system's mouse or other pointing device,  setting

the sample rate, resolution, and other characteristics to their

default values.

Input     Output

------    ----------------------------------------------------

AH=C2H    If function successful

AL=01H      Carry flag=clear

AH=00H

BH=device ID

If function unsuccessful

Carry flag=set

AH=status (see Int 15H Function C2H Subfunction 00H

After a reset operation, the state of the pointing device is

as follows:

- disabled;

- sample rate at 100 reports per second;

- resolution at 4 counts per millimeter;

- and scaling at 1 to 1.

The data package size is unchanged by this function.

The application can use  the  other  Int  15H  Function  C2H

subfunctions to initialize the pointing device to other  sample

rates, resolution, and scaling, and then enable the device with

Int 15H Function C2H Subfunction 00H.

See  also  Int  15H  Function  C2H  Subfunction  05H,  which

incidentally resets the pointing device in a similar manner.

9.8.25.3. Int 15H Function C2H Subfunction  02H

Description:

Set Sample Rate [PS/2]

Sets the sampling  rate  of  the  system's  mouse  or  other

pointing device.

Input                           Output

-----------------------------   ---------------------------

AH=C2H                          If function successful

AL=02H                            Carry flag=clear

BH=sample rate value              AH=00H

00H=10 reports per second     If function unsuccessful

01H=20 reports per second       Carry flag=set

02H=40 reports per second       AH=status (see Int 15H

03H=60 reports per second       Function C2H

04H=80 reports per second       Subfunction 00H)

05H=100 reports per second

06H=200 reports per second

Interrupt 15H (I/O Subsystem)

The default sample rate is 100 reports per  second  after  a

reset operation (Int 15H Function C2H Subfunction 01H).

9.8.25.4. Int 15H Function C2H Subfunction  03H

Description:

Set Resolution [PS/2]

Sets the resolution of the system's mouse or other  pointing

device.

Input                            Output

------------------------------   ---------------------------

AH=C2H                           If function successful

AL=03H                             Carry flag=clear

BH=resolution value                AH=00H

00H=1 count per millimeter     If function unsuccessful

01H=2 counts per millimeter      Carry flag=set

02H=4 counts per millimeter      AH=status (see Int 15H

03H=8 counts per millimeter         Function C2H

Subfunction 00H)

The default resolution is 4 counts per  millimeter  after  a

reset operation (Int 15H Function C2H Subfunction 01H).

9.8.25.5. Int 15H Function C2H Subfunction  04H

Description:

Get Pointing Device Type [PS/2]

Returns the identification code for the  system's  mouse  or

other pointing device.

Input             Output

------            ------------------------------------------

AH=C2H            If function successful

AL=04H            Carry flag=clear

AH=00H

BH=device ID

If function unsuccessful

Carry flag=set

AH=status (see Int 15H Function

C2H Subfunction 00H)

9.8.25.6. Int 15H Function C2H Subfunction  05H

Description:

Initialize Pointing Device Interface [PS/2]

Interrupt 15H (I/O Subsystem)

Sets the data package size for the system's mouse  or  other

pointing device, and initializes the resolution, sampling rate,

and scaling to their default values.

Input                  Output

--------------------   -----------------------------------

AH=C2H                 If function successful

AL=05H                   Carry flag=clear

BH=data package size     AH=00H

in bytes (1-8)      If function unsuccessful

Carry flag=set

AH=status (see Int 15H Function C2H

Subfunction 00H)

After this operation, the state of the pointing device is as

follows:

- disabled;

- sample rate at 100 reports per second;

- resolution at 4 counts per millimeter;

- and scaling at 1 to 1.

9.8.25.7. Int 15H Function C2H Subfunction  06H

Description:

Set Scaling or Get Status [PS/2]

Returns the current status of the system's  mouse  or  other

pointing device or sets the device's scaling factor.

Input                          Output

---------------------------    ------------------------------

AH=C2H                         If function successful

AL=06H                           Carry flag=clear

BH=extended command              AH=00H

00H=return device status       and, if called with BH=00H

01H=set scaling at 1:1         BL=status byte

02H=set scaling at 2:1           Bit(s)  Significance

------  ------------

0       =1 if right button

pressed

1       =reserved

2       =1 if left button

pressed

3       =reserved

4       =0 if 1:1 scaling

1 if 2:1 scaling

5       =0 if device disable

1 if device enabled

6       =0 if stream mode

1 if remote mode

7       =reserved

CL=resolution

00H=1 count per millimeter

01H=2 counts per millimeter

02H=4 counts per millimeter

03H=8 counts per millimeter

Interrupt 15H (I/O Subsystem)

DL=sample rate

0AH=10 reports per second

14H=20 reports per second

28H=40 reports per second

3CH=60 reports per second

50H=80 reports per second

64H=100 reports per second

C8H=200 reports per second

If function unsuccessful

Carry flag=set

AH=status (see Int 15H

Function C2H Subfunction 00

9.8.25.8. Int 15H Function C2H Subfunction  07H

Description:

Set Pointing Device Handler Address [PS/2]

Notifies the ROM BIOS pointing device driver of the  address

for a routine to be called each time pointing  device  data  is

available.

Input                        Output

----------------------       ---------------------------

AH=C2H                       If function successful

AL=07H                         Carry flag=clear

ES:BX=segment:offset         If function unsuccessful

of user routine          Carry flag=set

AH=status (see Int 15H

Function C2H

Subfunction 00H)

The user's handler for pointing device data is entered via a

far call with four parameters on the stack:

SS:SP+0AH status

SS:SP+08H x coordinate

SS:SP+06H y coordinate

SS:SP+04H z coordinate (always 0)

The handler must exit via a far  return  without  removing  the

parameters from the stack.

The  status  parameter  passed  to  the  user's  handler  is

interpreted as follows:

Bit(s)     Significance (if set)

------     --------------------------

0          left button pressed

1          right button pressed

2-3        reserved

4          sign of x data is negative

5          sign of y data is negative

6          x data has overflowed

7          y data has overflowed

8-15       reserved

Interrupt 16H (Keyboard)

9.9. Interrupt 16H (Keyboard)

Interrupt  Function         Name

16H        00H              Read Character from Keyboard

16H        01H              Get Keyboard Status

16H        02H              Get Keyboard Flags

16H        03H              Set Repeat Rate

16H        04H              Set Keyclick

16H        05H              Push Character and Scan Code

16H        10H              Read Character from Enhanced Keyboa

16H        11H              Get Enhanced Keyboard Status

16H        12H              Get Enhanced Keyboard Flags

9.9.1. Int 16H Function 00H

Description:

Read Character from Keyboard [PC] [AT] [PS/2]

Reads a character from  the  keyboard,  also  returning  the

keyboard scan code.

Input             Output

------            ---------------------

AH=00H            AH=keyboard scan code

AL=ASCII character

9.9.2. GetKeyClock

;* GetKeyClock - Waits for keypress while updating time at

;* specified location on screen.

;*

;* Shows:   BIOS Interrupt - 16h, Functions 0 and 10h

;*                           (Read Character)

;*                           16h, Functions 1 and 11h

;*                           (Get Keyboard Status)

;*          DOS Functions - 2Ah (Get Date)

;*                          2Ch (Get Time)

;*

;* Uses:    vconfig - Video configuration structure,

;*          declared in the DEMO.INC include file.

;*          The structure must first be

;*          initialized by calling the GetVidConfig procedure.

;*

;* Params:  row - Screen row for clock display

;*          col - Screen column for clock display

;*

;* Return:  Short integer with key scan code in high byte

;*          and ASCII character code in low byte.

;*          Low byte is 0 for special keys (such as the "F"

;*          keys) which don't generate characters.

Interrupt 16H (Keyboard)

.DATA

PUBLIC datestr

datestr DB      '  -  -     :  :  ', 0  ; Date/time string

.CODE

EXTRN StrWrite:PROC

GetKeyClock PROC \

row:WORD, col:WORD

LOCAL   service:BYTE

call    GetShift     ; Check for extended keyboard

mov     service, 11h ; Request for Function 11h

cmp     dx, 1        ; Extended keyboard available?

je      key1         ; Yes?  Set AH appropriately

mov     service, 1   ; No? Set AH for Function 1

key1:   mov     ah, service

int     16h          ; Check if Key is Ready

jnz     exit         ; Ready?  Exit procedure

; Not ready?  Check text mode

cmp     vconfig.mode, 7  ; Monochrome text mode?

je      @F               ; Yes?  Continue

cmp     vconfig.mode, 3  ; Color text mode?

je      @F               ; Yes?  Continue

cmp     vconfig.mode, 2  ; Black/white?

jne     key1          ; No?  Skip clock update and

;   poll keyboard again

; If 80-column text, get date and time from

; DOS before again polling keyboard,

; and display at upper right corner of screen.

@@:     mov     ah, 2Ch                 ; Request time

int     21h                     ; Get Time

mov     dl, dh

push    dx                      ; Save seconds,

push    cx                      ;   minutes,

mov     cl, ch                  ;   and

push    cx                      ;   hours

mov     ah, 2Ah                 ; Request date

int     21h                     ; Get Date

sub     cx, 1900  ; Subtract century, CL = year

push    cx                      ; Save year,

push    dx                      ;   day,

mov     dl, dh                  ;   and

push    dx                      ;   month

mov     cx, 6

sub     bx, bx

loop1:  pop     ax        ; Recover all 6 numbers in AL

aam               ; Convert to unpacked BCD

xchg    al, ah    ; Switch bytes for word move

or      ax, '00'  ; Make ASCII numerals

mov     WORD PTR datestr[bx], ax; Copy to string

add     bx, 3                   ;   at every third byte

loop    loop1

DispText row, col, <OFFSET datestr>

jmp     key1        ; Loop again for keypress

Interrupt 16H (Keyboard)

exit:   mov     ah, service ; 1 or 11h, depending on keybd

dec     ah          ; Set AH to 0 or 10h

int     16h         ; Get key to remove it from

ret                 ;   keyboard buffer

GetKeyClock ENDP

9.9.3. Int 16H Function 01H

Description:

Get Keyboard Status [PC] [AT] [PS/2]

Determines whether a character is ready for input, returning

a flag and also the character itself, if one is waiting.

Input             Output

------            --------------------------

AH=01H            If key waiting to be input

Zero flag=clear

AH=keyboard scan code

AL=character

If no key waiting

Zero flag=set

The character returned by this function when the  zero  flag

is clear is not removed from the type-ahead  buffer.  The  same

character and scan code will be returned by the  next  call  to

Int 16H Function 00H.

9.9.4. Int 16H Function 02H

Description:

Get Keyboard Flags [PC] [AT] [PS/2]

Returns the ROM BIOS flags byte that describes the state  of

the various keyboard toggles and shift keys.

Input             Output

------            ----------------------------------

AH=02H            AL=flags

Bit(s)  Significance (if set)

------  -----------------------

7       Insert on

6       Caps Lock on

5       Num Lock on

4       Scroll Lock on

3       Alt key is down

2       Ctrl key is down

1       left Shift key is down

0       right Shift key is down

The keyboard flags byte is stored in the ROM BIOS data  area

at 0000:0417H.

Interrupt 16H (Keyboard)

9.9.5. Int 16H Function 03H

Description:

Set Repeat Rate [PC] [AT] [PS/2]

Sets the ROM BIOS key repeat ("typematic") rate and delay.

Input                                                 Output

------------------------------------------            -------

On the PC/AT and PS/2                                 Nothing

AH=03H

AL=05H

BH=repeat delay (see Notes)

BL=repeat rate (see Notes)

On the PCjr

AH=03H

AL=subfunction

00H to restore default rate and delay

01H to increase initial delay

02H to decrease repeat rate by one-half

03H to increase delay and decrease

repeat rate by one-half

04H to turn off keyboard repeat

Subfunctions 00H-04H are available on the PCjr but  are  not

supported by the PC or  PC/XT  ROM  BIOS.  Subfunction  05H  is

available on PC/ATs with ROM BIOS's dated 11/15/85  and  later,

and on the PS/2.

On the PC/AT and PS/2, the value in BH controls  the  amount

of delay before the first repeat key is  generated,  and  is  a

multiple of 250 milliseconds:

Value            Delay (msec.)

-----            -------------

00H              250

01H              500

02H              750

03H              1000

On the PC/AT and PS/2, the value  for  the  repeat  rate  in

characters per second can be chosen from the following table:

Value            Repeat rate (characters per second)

-----            -----------------------------------

00H              30.0

01H              26.7

02H              24.0

03H              21.8

04H              20.0

05H              18.5

06H              17.1

07H              16.0

08H              15.0

09H              13.3

0AH              12.0

0BH              10.9

0CH              10.0

0DH              9.2

0EH              8.6

0FH              8.0

Interrupt 16H (Keyboard)

10H              7.5

11H              6.7

12H              6.0

13H              5.5

14H              5.0

15H              4.6

16H              4.3

17H              4.0

18H              3.7

19H              3.3

1AH              3.0

1BH              2.7

1CH              2.5

1DH              2.3

1EH              2.1

1FH              2.0

9.9.6. Int 16H Function 04H

Description:

Set Keyclick [PC]

Turns the keyboard click on or off.

Input                                        Output

---------------------------------            -------

AH=04H                                       Nothing

AL=subfunction

00H to turn off keyboard click

01H to turn on keyboard click

This function is supported by the PCjr BIOS only.

9.9.7. Int 16H Function 05H

Description:

Push Character and Scan Code [AT] [PS/2]

Places a character and scan code in the keyboard  type-ahead

buffer.

Input                   Output

------------            -----------------------------

AH=05H                  If function successful

CH=scan code            Carry flag=clear

CL=character            AL=00H

If function unsuccessful

(type-ahead buffer is full)

Carry flag=set

AL=01H

This function can be used by keyboard  enhancers  and  other

utilities to interpolate keys into  the  data  stream  seen  by

Interrupt 16H (Keyboard)

application programs.

9.9.8. Int 16H Function 10H

Description:

Read Character from Enhanced Keyboard [AT] [PS/2]

Reads a character and scan code from the keyboard type-ahead

buffer.

Input             Output

------            ---------------------

AH=10H            AH=keyboard scan code

AL=ASCII character

Use this function for the enhanced keyboard instead  of  Int

16H Function 00H. It allows applications  to  obtain  the  scan

codes for the additional F11, F12, and cursor control keys.

9.9.9. Int 16H Function 11H

Description:

Get Enhanced Keyboard Status [AT] [PS/2]

Determines whether a character is ready for input, returning

a flag and also the character itself, if one is waiting.

Input             Output

------            --------------------------

AH=11H            If key waiting to be input

Zero flag=clear

AH=keyboard scan code

AL=character

If no key waiting

Zero flag=set

Use this function for the enhanced keyboard instead  of  Int

16H Function 00H.  It  allows  applications  to  test  for  the

additional F11, F12, and cursor control keys.

The character returned by this function when the  zero  flag

is clear is not removed from the type-ahead  buffer.  The  same

character and scan code will be returned by the  next  call  to

Int 16H Function 10H.

9.9.10. Int 16H Function 12H

Description:

Get Enhanced Keyboard Flags [AT] [PS/2]

Obtains the status of various enhanced keyboard special keys

and keyboard driver states.

Interrupt 16H (Keyboard)

Input             Output

------            -----------------------------------

AH=12H            AX=flags

Bit(s)  Significance (if set)

------  -----------------------

00H     right Shift key is down

01H     left Shift key is down

02H     either Ctrl key is down

03H     either Alt key is down

04H     Scroll Lock toggle is on

05H     Num Lock toggle is on

06H     Caps Lock toggle is on

07H     Insert toggle is on

08H     left Ctrl key is down

09H     left Alt key is down

0AH     right Ctrl key is down

0BH     right Alt key is down

0CH     Scroll key is down

0DH     Num Lock key is down

0EH     Caps Lock key is down

0FH     SysReq key is down

Use this function for the enhanced keyboard instead  of  Int

16H Function 02H.

9.10. Interrupt 17H (Parallel Printer Port)

Interrupt  Function         Name

17H        00H              Write Character to Printer

17H        01H              Initialize Printer Port

17H        02H              Get Printer Status

9.10.1. Int 17H Function 00H

Description:

Write Character to Printer [PC] [AT]

Sends  a  character  to  the  specified   parallel   printer

interface port and returns the current status of the port.

Interrupt 17H (Parallel Printer Port)

Input                         Output

---------------------------   -------------------------------

AH=00H                        AH=status

AL=character                  Bit(s)  Significance (if set)

DX=printer number             ------  ---------------------

(0=LPT1, 1=LPT2, 2=LPT3)    7       printer not busy

6       printer acknowledge

5       out of paper

4       printer selected

3       I/O error

2       unused

1       unused

0       printer timed-out

9.10.2. Int 17H Function 01H

Description:

Initialize Printer Port [PC] [AT] [PS/2]

Initializes the specified parallel  printer  interface  port

and returns its status.

Input                          Output

---------------------------    -----------------------------

AH=01H                         AH=status

DX=printer number                 (see Int 17H Function 00H)

(0=LPT1, 1=LPT2, 2=LPT3)

9.10.3. Int 17H Function 02H

Description:

Get Printer Status [PC] [AT] [PS/2]

Returns the current status of the specified parallel printer

interface port.

Input                           Output

---------------------------     -----------------------------

AH=02H                          AH=status

DX=printer number                  (see Int 17H Function 00H)

(0=LPT1, 1=LPT2, 2=LPT3)

9.10.4. VeriPrint

Interrupt 17H (Parallel Printer Port)

;* VeriPrint - Checks if LPT1 (PRN) is available.

;*

;* Shows:   BIOS Interrupt - 17h (Parallel Port Printer Driver)

;*

;* Params:  None

;*

;* Return:  Short integer, 1 for yes or 0 for no

VeriPrint PROC

mov     ah, 2         ; Check printer status for

sub     dx, dx        ;   parallel printer (port 0)

int     17h

xchg    dx, ax        ; Put 0 (for error) in AX

test    dh, 00101001b ; Are any error bits on?

jne     exit          ; Yes?  Leave 0

test    dh, 10010000b ; Are both operation bits on?

jz      exit          ; No?  Leave 0

inc     ax            ; Yes?  Return 1

exit:   ret

VeriPrint ENDP

9.11. Interrupt 1AH (Clock)

Interrupt  Function         Name

1AH        00H              Get Tick Count

1AH        01H              Set Tick Count

1AH        02H              Get Time

1AH        03H              Set Time

1AH        04H              Get Date

1AH        05H              Set Date

1AH        06H              Set Alarm

1AH        07H              Reset Alarm

1AH        0AH              Get Day Count

1AH        0BH              Set Day Count

1AH        80H              Set Sound Source

9.11.1. Int 1AH Function 00H

Description:

Get Tick Count [AT] [PS/2]

Returns the contents of the clock tick counter.

Interrupt 1AH (Clock)

Input             Output

------            -------

AH=00H            AL=rolled-over flag

00H if midnight not passed since last read

<>00H if midnight was passed since last rea

CX:DX=tick count (high 16 bits in CX)

This function is supported by the PC/XT and PCjr  ROM  BIOS,

but is not present in the ROM BIOS for the original PC.

The returned value is the cumulative number of  clock  ticks

since midnight. There are 18.2 clock ticks per second. When the

counter reaches 1,573,040, it  is  cleared  to  zero,  and  the

rolled-over flag is set.

The rolled-over flag is cleared by this  function  call,  so

the flag will only be returned nonzero once per day.

Int 1AH Function 01H can be  used  to  set  the  clock  tick

counter to an arbitrary 32-bit value.

9.11.2. Pause

;* Pause - Waits for specified number of clocks

;* to elapse, then returns.

;*

;* Shows:   BIOS Interrupt - 1Ah, Function 0

;*          (Real-Time Clock Driver)

;*          Operators - LOCAL     []

;*

;* Params:  duration - Desired duration in clocks, where

;*                     18 clocks = approx 1 second

;*

;* Return:  None

Pause   PROC \

duration:WORD

LOCAL time:DWORD

sub     ah, ah

int     1Ah           ; Get Clock Count in CX:DX

add     dx, duration  ; Add pause time to it

adc     cx, 0

mov     WORD PTR time[0], dx ; Result is target time;

mov     WORD PTR time[2], cx ;   keep in local variable

loop1:  int     1AH                  ; Now repeatedly poll cloc

cmp     dx, WORD PTR time[0] ;   count until the target

jb      loop1                ;   time is reached

cmp     cx, WORD PTR time[2]

jb      loop1

ret

Pause   ENDP

Interrupt 1AH (Clock)

9.11.3. Int 1AH Function 01H

Description:

Set Tick Count [AT] [PS/2]

Stores a 32-bit value in the clock tick counter.

Input                                            Output

-------------------------------------            -------

AH=01H                                           Nothing

CX:DX=tick count (high 16-bits in CX)

This function is supported by the PC/XT and PCjr  ROM  BIOS,

but is not present in the ROM BIOS for the original PC.

Int 1AH Function 00H is used to read the value of the  clock

tick counter.

The rolled-over flag is cleared by this function call.

9.11.4. Int 1AH Function 02H

Description:

Get Time [AT] [PS/2]

Reads the current time from the CMOS time/date chip.

Input             Output

------            --------------------------------------

AH=02H            CH=hours in binary coded decimal (BCD)

CL=minutes in BCD

DH=seconds in BCD

DL=daylight-saving-time code

00H if standard time

01H if daylight saving time

and, if clock running

Carry flag=clear

or, if clock stopped

Carry flag=set

9.11.5. Int 1AH Function 03H

Description:

Set Time [AT] [PS/2]

Sets the time in the CMOS time/date chip.

Interrupt 1AH (Clock)

Input                                             Output

--------------------------------------            -------

AHM=03H                                           Nothing

CH=hours in binary coded decimal (BCD)

CL=minutes in BCD

DH=seconds in BCD

DL=daylight-saving-time code

00H if standard time

01H if daylight saving time

9.11.6. Int 1AH Function 04H

Description:

Get Date [AT] [PS/2]

Reads the current date from the CMOS time/date chip.

Input     Output

------    ---------------------------------------------------

AH=04H    CH=century (19 or 20) in binary coded decimal (BCD)

CL=year in BCD

DH=month in BCD

DL=day in BCD

and, if clock running

Carry flag=clear

or, if clock stopped

Carry flag=set

9.11.7. Int 1AH Function 05H

Description:

Set Date [AT] [PS/2]

Sets the date in the CMOS time/date chip.

Input                                                Output

---------------------------------------------------  -------

AH=05H                                               Nothing

CH=century (19 or 20) in binary coded decimal (BCD)

CL=year in BCD

DH=month in BCD

DL=day in BCD

Interrupt 1AH (Clock)

9.11.8. Int 1AH Function 06H

Description:

Set Alarm [AT] [PS/2]

Sets an alarm in the CMOS date/time chip.

Input                                   Output

--------------------------------------  -----------------------

AH=06H                                  If function successful

CH=hours in binary coded decimal (BCD)    Carry flag=clear

CL=minutes in BCD                       If function unsuccessfu

DH=seconds in BCD                         (alarm already set,

or clock stopped)

Carry flag=set

A side effect of this function  is  that  the  clock  chip's

interrupt level (IRQ8) is enabled.

Only one alarm may be active at any given  time.  The  alarm

occurs every 24 hours at the specified time until it  is  reset

with Int 1AH Function 07H.

The program using this function must place  the  address  of

its interrupt handler for the alarm in the vector for Int  4AH.

9.11.9. Int 1AH Function 07H

Description:

Reset Alarm [AT] [PS/2]

Cancels any pending alarm  request  on  the  CMOS  date/time

chip.

Input             Output

------            -------

AH=07H            Nothing

This function does not disable the  clock  chip's  interrupt

level (IRQ8).

9.11.10. Int 1AH Function 0AH

Description:

Get Day Count [PS/2]

Returns the contents of the system's day counter.

Interrupt 1AH (Clock)

Input             Output

------            ------------------------------

AH=0AH            If function successful

Carry flag=clear

CX=count of days since 1-1-1980

If function unsuccessful

Carry flag=set

9.11.11. Int 1AH Function 0BH

Description:

Set Day Count [PS/2]

Stores an arbitrary value in the system's day counter.

Input                             Output

----------------------            --------------------------

AH=0BH                            If function successful

CX=count of days since            Carry flag=clear

January 1, 1980                If function unsuccessful

Carry flag=set

9.11.12. Int 1AH Function 80H

Description:

Set Sound Source [PC]

Sets up the source for tones that will appear on the  PCjr's

"Audio Out" or RF modulator.

Input                                          Output

--------------------------------------------   -------

AH=80H                                         Nothing

AL=sound source

00H if 8253 programmable timer, channel 2

01H if cassette input

02H if "Audio In" line on I/O channel

03H if sound generator chip

This function is supported on the PCjr only.

9.12. Int 11H

Description:

Get Equipment Configuration [PC] [AT] [PS/2]

Obtains the equipment list code word from the ROM BIOS.

Miscellaneous BIOS

Input     Output

-------   -----------------------------------------------------

Nothing   AX=equipment list code word

Bit(s)          Significance

------          -------------------------------------

14-15           number of printers installed

13              =1 if internal modem installed

(PC and XT only)

=1 if serial printer attached (PCjr)

12              =1 if game adapter installed

9-11            number of RS-232 ports installed

8               reserved

6-7             number of floppy disk drives

(if bit 0 =1)

00=1

01=2

10=3

11=4

4-5             initial video mode

00  reserved

01  40-by-25 color text

10  80-by-25 color text

11  80-by-25 monochrome

2-3             system board RAM size (PC, see Note)

00=16 KB

01=32 KB

10=48 KB

11=64 KB

2               =1 if pointing device installed (PS/2

1               =1 if math coprocessor installed

0               =1 if floppy disk drive(s) installed

Bits 2-3 of the returned value are used only in the ROM BIOS

for the original IBM PC with the 64 KB system board and on  the

PCjr.

9.13. VeriCop

;* VeriCop - Checks for coprocessor.

;*

;* Shows:   BIOS Interrupt - 11h (Get Equipment Information)

;*

;* Params:  None

;*

;* Return:  Short integer, 1 for yes or 0 for no

VeriCop PROC

int     11h        ; Check peripherals

test    al, 10b    ; Coprocessor?

mov     ax, 0      ; Assume no, don't alter flags

jz      exit       ; No?  Done

inc     ax         ; Yes?  Set to 1

exit:   ret

VeriCop ENDP

Miscellaneous BIOS

9.14. Int 12H

Description:

Get Conventional Memory Size [PC] [AT] [PS/2]

Returns the amount of conventional memory available for  use

by MS-DOS and application programs.

Input              Output

-------            ----------------------

Nothing            AX=memory size (in KB)

On some early PC models, the amount of  memory  returned  by

this function is controlled by the settings of the dip switches

on the system board and may not reflect all the memory that  is

physically present.

On the PC/AT, the value returned is the amount of functional

memory found during the power-on self-test, regardless  of  the

memory size configuration information stored in CMOS RAM.

The value returned does  not  reflect  any  extended  memory

(above the 1 MB boundary) that may be  installed  on  80286  or

80386 machines such as the PC/AT or PS/2 (Models 50 and above).

9.15. GetMem

;* GetMem - Gets total size of memory and determines the

;* largest amount of unallocated memory available.

;* GetMem invokes DOS Function 48h (Allocate Memory)

;* to request an impossibly large memory block. DOS denies the

;* request, but returns instead the size of the largest block

;* available. This is the amount that GetMem returns to the

;* calling program. See the WinOpen procedure for an example

;* of calling Function 48h to allocate unused memory.

;*

;* Shows:   BIOS Interrupt - 12h (Get Conventional Memory Size)

;*

;* Params:  None

;*

;* Return:  Long integer, high word = total memory

;*                         in kilobytes (KB)

;*                        low word  = largest block

;*                         of available memory (KB)

GetMem  PROC

int     12h        ; Get total memory in K

push    ax         ; Save size of memory

mov     ah, 48h    ; Request memory allocation

mov     bx, 0FFFFh ; Ensure request is denied for

;   impossibly large block

int     21h        ; Get largest available block in BX

mov     ax, bx     ; Copy to AX

mov     cl, 6      ; Convert paragraphs to kilobytes by

shr     ax, cl     ;   dividing by 64

pop     dx         ; Recover total in DX

ret                ; Return long integer DX:AX

Miscellaneous BIOS

GetMem  ENDP

9.16. Int 18H

Description:

ROM BASIC [PC] [AT] [PS/2]

Transfers control to ROM BASIC.

Input              Output

-------            -------

Nothing            Nothing

This function is invoked when the system  is  turned  on  or

restarted if attempts to read a boot sector from the fixed disk

or floppy disk drives are unsuccessful.

9.17. Int 19H

Description:

Reboot System [PC] [AT] [PS/2]

Reboots the operating system from the floppy disk  or  fixed

disk drive.

Input              Output

-------            -------

Nothing            Nothing

The bootstrap routine reads Sector 1, Track 0 into memory at

location 0000:7C00H and transfers control to the same  address.

If attempts to read a boot sector from the floppy disk or fixed

disk are unsuccessful, control is transferred to ROM  BASIC  by

execution of an Int 18H.

If location 0000:0472H does not contain the value  1234H,  a

memory test will be performed before reading the boot sector.

MASM 5.1

I n d e x

@code  . . . . . . . . . . . . . . . . . . 143

@CodeSize  . . . . . . . . . . . . . . . . 145

@Cpu   . . . . . . . . . . . . . . . . . . 146

@CurSeg  . . . . . . . . . . . . . . . . . 144

@data  . . . . . . . . . . . . . . . . . . 145

@DataSize  . . . . . . . . . . . . . . . . 146

@FileName  . . . . . . . . . . . . . . . . 144

@Model   . . . . . . . . . . . . . . . . . 145

@Version   . . . . . . . . . . . . . . . . 146

@WordSize  . . . . . . . . . . . . . . . . 145

% (Expression Operator)  . . . . . . . . . 128

%OUT Directive   . . . . . . . . . . . . . 107

! (Literal Character Operator)   . . . . . 127

& (Substitute Operator)  . . . . . . . . . 128

* (Multiplication)   . . . . . . . . . . . 125

+ (Plus, Binary)   . . . . . . . . . . . . 123

+ (Plus, Unary)  . . . . . . . . . . . . . 124

- (Minus, Binary)  . . . . . . . . . . . . 124

- (Minus, Unary)   . . . . . . . . . . . . 124

. (Structure-Field Reference)  . . . . . . 126

.186   . . . . . . . . . . . . . . . . . . 111

.286   . . . . . . . . . . . . . . . . . . 111

.287   . . . . . . . . . . . . . . . . . . 111

.8086  . . . . . . . . . . . . . . . . . . 111

.8087  . . . . . . . . . . . . . . . . . . 112

.ALPHA   . . . . . . . . . . . . . . . . . 116

.CODE  . . . . . . . . . . . . . . . . . . 118

.CONST   . . . . . . . . . . . . . . . . . 120

.DATA  . . . . . . . . . . . . . . . . . . 119

.DATA?   . . . . . . . . . . . . . . . . . 120

.ERR   . . . . . . . . . . . . . . . . . . 91

.ERR1/.ERR2  . . . . . . . . . . . . . . . 91

.ERRB/.ERRNB   . . . . . . . . . . . . . . 92

.ERRDEF/.ERRNDEF   . . . . . . . . . . . . 92

.ERRDIF/.ERRIDN  . . . . . . . . . . . . . 92

.ERRE/.ERRNZ   . . . . . . . . . . . . . . 91

.FARDATA   . . . . . . . . . . . . . . . . 120

.FARDATA?  . . . . . . . . . . . . . . . . 121

.LALL  . . . . . . . . . . . . . . . . . . 101

.LFCOND  . . . . . . . . . . . . . . . . . 100

.LIST  . . . . . . . . . . . . . . . . . . 100

.MODEL   . . . . . . . . . . . . . . . . . 117

.RADIX   . . . . . . . . . . . . . . . . . 108

.SALL  . . . . . . . . . . . . . . . . . . 101

.SEQ   . . . . . . . . . . . . . . . . . . 116

.SFCOND  . . . . . . . . . . . . . . . . . 100

.STACK   . . . . . . . . . . . . . . . . . 119

.STARTUP   . . . . . . . . . . . . . . . . 118

.TFCOND  . . . . . . . . . . . . . . . . . 101

.TYPE  . . . . . . . . . . . . . . . . . . 137

.XALL  . . . . . . . . . . . . . . . . . . 101

MASM 5.1

.XLIST   . . . . . . . . . . . . . . . . . 100

/ (Division)   . . . . . . . . . . . . . . 125

; (Comment Delimeter)  . . . . . . . . . . 141

;; (Macro Comment Delimeter)   . . . . . . 127

<> (Literal Text Operator)   . . . . . . . 127

? (Uninitialized Value)  . . . . . . . . . 142

AAA  . . . . . . . . . . . . . . . . . . . 20

AAD  . . . . . . . . . . . . . . . . . . . 22

AAM  . . . . . . . . . . . . . . . . . . . 21

AAS  . . . . . . . . . . . . . . . . . . . 21

AddLong  . . . . . . . . . . . . . . . . . 12

ADC  . . . . . . . . . . . . . . . . . . . 13

ADD  . . . . . . . . . . . . . . . . . . . 12

ALIGN  . . . . . . . . . . . . . . . . . . 95

AND  . . . . . . . . . . . . . . . . . . . 65, 129

ASSUME   . . . . . . . . . . . . . . . . . 114

CALL   . . . . . . . . . . . . . . . . . . 38

CBW  . . . . . . . . . . . . . . . . . . . 84

CLC  . . . . . . . . . . . . . . . . . . . 55

CLD  . . . . . . . . . . . . . . . . . . . 57

CLI  . . . . . . . . . . . . . . . . . . . 58

CMC  . . . . . . . . . . . . . . . . . . . 59

CMP  . . . . . . . . . . . . . . . . . . . 26

CMPS   . . . . . . . . . . . . . . . . . . 29, 81

CMPSB  . . . . . . . . . . . . . . . . . . 29, 81

CMPSW  . . . . . . . . . . . . . . . . . . 29, 81

COMM   . . . . . . . . . . . . . . . . . . 109

COMMENT  . . . . . . . . . . . . . . . . . 106

CWD  . . . . . . . . . . . . . . . . . . . 85

DAA  . . . . . . . . . . . . . . . . . . . 22

DAS  . . . . . . . . . . . . . . . . . . . 23

DB   . . . . . . . . . . . . . . . . . . . 96

DD   . . . . . . . . . . . . . . . . . . . 98

DEC  . . . . . . . . . . . . . . . . . . . 16

DIV  . . . . . . . . . . . . . . . . . . . 19

DisableCGA   . . . . . . . . . . . . . . . 59

DOSSEG   . . . . . . . . . . . . . . . . . 115

DQ   . . . . . . . . . . . . . . . . . . . 98

DT   . . . . . . . . . . . . . . . . . . . 99

DUP  . . . . . . . . . . . . . . . . . . . 142

DW   . . . . . . . . . . . . . . . . . . . 98

END  . . . . . . . . . . . . . . . . . . . 107

ENDM   . . . . . . . . . . . . . . . . . . 103

ENDP   . . . . . . . . . . . . . . . . . . 94

ENDS   . . . . . . . . . . . . . . . . . . 114, 122

ENTER  . . . . . . . . . . . . . . . . . . 43

EQ   . . . . . . . . . . . . . . . . . . . 139

ESC  . . . . . . . . . . . . . . . . . . . 73

EVEN   . . . . . . . . . . . . . . . . . . 95

EXIT   . . . . . . . . . . . . . . . . . . 118

EXITM  . . . . . . . . . . . . . . . . . . 104

EXTRN  . . . . . . . . . . . . . . . . . . 109

FindNext   . . . . . . . . . . . . . . . . 177

MASM 5.1

Flags  . . . . . . . . . . . . . . . . . . 149

GetMem   . . . . . . . . . . . . . . . . . 331

GE   . . . . . . . . . . . . . . . . . . . 140

GetVidConfig   . . . . . . . . . . . . . . 237

GROUP  . . . . . . . . . . . . . . . . . . 114

GT   . . . . . . . . . . . . . . . . . . . 140

HELLOM.ASM   . . . . . . . . . . . . . . . 39

HIGH   . . . . . . . . . . . . . . . . . . 134

HLT  . . . . . . . . . . . . . . . . . . . 75

IDIV   . . . . . . . . . . . . . . . . . . 19

IF   . . . . . . . . . . . . . . . . . . . 88

IF1  . . . . . . . . . . . . . . . . . . . 88

IFB  . . . . . . . . . . . . . . . . . . . 89

IFDEF  . . . . . . . . . . . . . . . . . . 89

IFDIF  . . . . . . . . . . . . . . . . . . 90

IMUL   . . . . . . . . . . . . . . . . . . 17

Int 10H Function 00H   . . . . . . . . . . 236

Int 10H Function 01H   . . . . . . . . . . 239

Int 10H Function 02H   . . . . . . . . . . 240

Int 10H Function 03H   . . . . . . . . . . 242

Int 10H Function 04H   . . . . . . . . . . 242

Int 10H Function 05H   . . . . . . . . . . 243

Int 10H Function 06H   . . . . . . . . . . 243

Int 10H Function 07H   . . . . . . . . . . 244

Int 10H Function 08H   . . . . . . . . . . 244

Int 10H Function 09H   . . . . . . . . . . 245

Int 10H Function 0AH   . . . . . . . . . . 246

Int 10H Function 0BH   . . . . . . . . . . 249

Int 10H Function 0CH   . . . . . . . . . . 250

Int 10H Function 0DH   . . . . . . . . . . 250

Int 10H Function 0EH   . . . . . . . . . . 247

Int 10H Function 0FH   . . . . . . . . . . 247

Int 10H Function 10H Subfunction 00H   . . 256

Int 10H Function 10H Subfunction 01H   . . 257

Int 10H Function 10H Subfunction 02H   . . 257

Int 10H Function 10H Subfunction 03H   . . 258

Int 10H Function 10H Subfunction 07H   . . 258

Int 10H Function 10H Subfunction 08H   . . 259

Int 10H Function 10H Subfunction 09H   . . 259

Int 10H Function 10H Subfunction 10H   . . 259, 260

Int 10H Function 10H Subfunction 12H   . . 260

Int 10H Function 10H Subfunction 13H   . . 260

Int 10H Function 10H Subfunction 15H   . . 261

Int 10H Function 10H Subfunction 17H   . . 261

Int 10H Function 10H Subfunction 1AH   . . 262

Int 10H Function 10H Subfunction 1BH   . . 262

Int 10H Function 11H Subfunction 00H/10H   263

Int 10H Function 11H Subfunction 01H/11H   265

Int 10H Function 11H Subfunction 02H/12H   266

Int 10H Function 11H Subfunction 03H   . . 267

Int 10H Function 11H Subfunction 04H   . . 267

Int 10H Function 11H Subfunction 14H   . . 268

Int 10H Function 11H Subfunction 20H   . . 269

Int 10H Function 11H Subfunction 21H   . . 269

Int 10H Function 11H Subfunction 22H   . . 270

Int 10H Function 11H Subfunction 23H   . . 270

Int 10H Function 11H Subfunction 24H   . . 271

Int 10H Function 11H Subfunction 30H   . . 271

Int 10H Function 12H Subfunction 10H   . . 272

MASM 5.1

Int 10H Function 12H Subfunction 20H   . . 273

Int 10H Function 12H Subfunction 30H   . . 273

Int 10H Function 12H Subfunction 31H   . . 273

Int 10H Function 12H Subfunction 32H   . . 274

Int 10H Function 12H Subfunction 33H   . . 274

Int 10H Function 12H Subfunction 34H   . . 274

Int 10H Function 12H Subfunction 35H   . . 275

Int 10H Function 12H Subfunction 36H   . . 275

Int 10H Function 13H   . . . . . . . . . . 248

Int 10H Function 1AH   . . . . . . . . . . 251

Int 10H Function 1BH   . . . . . . . . . . 251

Int 10H Function 1CH   . . . . . . . . . . 255

Int 11H  . . . . . . . . . . . . . . . . . 329

Int 12H  . . . . . . . . . . . . . . . . . 331

Int 13H Function 00H   . . . . . . . . . . 277

Int 13H Function 01H   . . . . . . . . . . 277

Int 13H Function 02H   . . . . . . . . . . 278

Int 13H Function 03H   . . . . . . . . . . 279

Int 13H Function 04H   . . . . . . . . . . 279

Int 13H Function 05H   . . . . . . . . . . 280

Int 13H Function 06H   . . . . . . . . . . 281

Int 13H Function 07H   . . . . . . . . . . 281

Int 13H Function 08H   . . . . . . . . . . 282

Int 13H Function 09H   . . . . . . . . . . 282

Int 13H Function 0AH   . . . . . . . . . . 284

Int 13H Function 0BH   . . . . . . . . . . 284

Int 13H Function 0CH   . . . . . . . . . . 285

Int 13H Function 0DH   . . . . . . . . . . 285

Int 13H Function 0EH   . . . . . . . . . . 285

Int 13H Function 0FH   . . . . . . . . . . 286

Int 13H Function 10H   . . . . . . . . . . 286

Int 13H Function 11H   . . . . . . . . . . 287

Int 13H Function 12H   . . . . . . . . . . 287

Int 13H Function 13H   . . . . . . . . . . 288

Int 13H Function 14H   . . . . . . . . . . 288

Int 13H Function 15H   . . . . . . . . . . 288

Int 13H Function 16H   . . . . . . . . . . 289

Int 13H Function 17H   . . . . . . . . . . 289

Int 13H Function 18H   . . . . . . . . . . 290

Int 13H Function 19H   . . . . . . . . . . 290

Int 13H Function 1AH   . . . . . . . . . . 291

Int 14H Function 00H   . . . . . . . . . . 292

Int 14H Function 01H   . . . . . . . . . . 293

Int 14H Function 02H   . . . . . . . . . . 294

Int 14H Function 03H   . . . . . . . . . . 294

Int 14H Function 04H   . . . . . . . . . . 295

Int 14H Function 05H   . . . . . . . . . . 296

Int 15H Function 00H   . . . . . . . . . . 297

Int 15H Function 01H   . . . . . . . . . . 297

Int 15H Function 02H   . . . . . . . . . . 298

Int 15H Function 03H   . . . . . . . . . . 298

Int 15H Function 0FH   . . . . . . . . . . 299

Int 15H Function 21H Subfunction 00H   . . 299

Int 15H Function 21H Subfunction 01H   . . 300

Int 15H Function 4FH   . . . . . . . . . . 300

Int 15H Function 80H   . . . . . . . . . . 301

Int 15H Function 81H   . . . . . . . . . . 301

Int 15H Function 82H   . . . . . . . . . . 302

Int 15H Function 83H   . . . . . . . . . . 302

Int 15H Function 84H   . . . . . . . . . . 303

Int 15H Function 85H   . . . . . . . . . . 304

Int 15H Function 86H   . . . . . . . . . . 304

MASM 5.1

Int 15H Function 87H   . . . . . . . . . . 305

Int 15H Function 88H   . . . . . . . . . . 306

Int 15H Function 89H   . . . . . . . . . . 306

Int 15H Function 90H   . . . . . . . . . . 307

Int 15H Function 91H   . . . . . . . . . . 308

Int 15H Function C0H   . . . . . . . . . . 308

Int 15H Function C1H   . . . . . . . . . . 309

Int 15H Function C2H Subfunction 00H   . . 311

Int 15H Function C2H Subfunction 01H   . . 312

Int 15H Function C2H Subfunction 02H   . . 312

Int 15H Function C2H Subfunction 03H   . . 313

Int 15H Function C2H Subfunction 04H   . . 313

Int 15H Function C2H Subfunction 05H   . . 313

Int 15H Function C2H Subfunction 06H   . . 314

Int 15H Function C2H Subfunction 07H   . . 315

Int 15H Function C3H   . . . . . . . . . . 310

Int 15H Function C4H   . . . . . . . . . . 310

Int 16H Function 00H   . . . . . . . . . . 316

Int 16H Function 01H   . . . . . . . . . . 318

Int 16H Function 02H   . . . . . . . . . . 318

Int 16H Function 03H   . . . . . . . . . . 319

Int 16H Function 04H   . . . . . . . . . . 320

Int 16H Function 05H   . . . . . . . . . . 320

Int 16H Function 10H   . . . . . . . . . . 321

Int 16H Function 11H   . . . . . . . . . . 321

IN   . . . . . . . . . . . . . . . . . . . 70

Int 16H Function 12H   . . . . . . . . . . 321

Int 17H Function 00H   . . . . . . . . . . 322

Int 17H Function 01H   . . . . . . . . . . 323

Int 17H Function 02H   . . . . . . . . . . 323

INC  . . . . . . . . . . . . . . . . . . . 14

INCLUDE  . . . . . . . . . . . . . . . . . 107

INCLUDELIB   . . . . . . . . . . . . . . . 107, 110

INS  . . . . . . . . . . . . . . . . . . . 71

Int 18H  . . . . . . . . . . . . . . . . . 332

Int 19H  . . . . . . . . . . . . . . . . . 332

Int 1AH Function 00H   . . . . . . . . . . 324

Int 1AH Function 01H   . . . . . . . . . . 326

Int 1AH Function 02H   . . . . . . . . . . 326

Int 1AH Function 03H   . . . . . . . . . . 326

Int 1AH Function 04H   . . . . . . . . . . 327

Int 1AH Function 05H   . . . . . . . . . . 327

Int 1AH Function 06H   . . . . . . . . . . 328

Int 1AH Function 07H   . . . . . . . . . . 328

Int 1AH Function 0AH   . . . . . . . . . . 328

Int 1AH Function 0BH   . . . . . . . . . . 329

Int 1AH Function 80H   . . . . . . . . . . 329

Int 21H Function 00H   . . . . . . . . . . 209

Int 21H Function 01H   . . . . . . . . . . 153

Int 21H Function 02H   . . . . . . . . . . 154

Int 21H Function 03H   . . . . . . . . . . 156

Int 21H Function 04H   . . . . . . . . . . 156

Int 21H Function 05H   . . . . . . . . . . 157

Int 21H Function 06H   . . . . . . . . . . 157

Int 21H Function 07H   . . . . . . . . . . 158

Int 21H Function 08H   . . . . . . . . . . 159

Int 21H Function 09H   . . . . . . . . . . 159

Int 21H Function 0AH   . . . . . . . . . . 160

Int 21H Function 0BH   . . . . . . . . . . 161

Int 21H Function 0CH   . . . . . . . . . . 161

Int 21H Function 0DH   . . . . . . . . . . 165

Int 21H Function 0EH   . . . . . . . . . . 165

MASM 5.1

Int 21H Function 0FH   . . . . . . . . . . 183

Int 21H Function 10H   . . . . . . . . . . 183

Int 21H Function 11H   . . . . . . . . . . 184

Int 21H Function 12H   . . . . . . . . . . 185

Int 21H Function 13H   . . . . . . . . . . 185

Int 21H Function 14H   . . . . . . . . . . 222

Int 21H Function 15H   . . . . . . . . . . 223

Int 21H Function 16H   . . . . . . . . . . 186

Int 21H Function 17H   . . . . . . . . . . 187

Int 21H Function 19H   . . . . . . . . . . 166

Int 21H Function 1AH   . . . . . . . . . . 215

Int 21H Function 1BH   . . . . . . . . . . 166

Int 21H Function 1CH   . . . . . . . . . . 167

Int 21H Function 21H   . . . . . . . . . . 224

Int 21H Function 22H   . . . . . . . . . . 224

Int 21H Function 23H   . . . . . . . . . . 187

Int 21H Function 24H   . . . . . . . . . . 225, 226

Int 21H Function 25H   . . . . . . . . . . 192

Int 21H Function 26H   . . . . . . . . . . 210

Int 21H Function 27H   . . . . . . . . . . 226

Int 21H Function 28H   . . . . . . . . . . 227

Int 21H Function 29H   . . . . . . . . . . 188

Int 21H Function 2AH   . . . . . . . . . . 228

Int 21H Function 2BH   . . . . . . . . . . 229

Int 21H Function 2CH   . . . . . . . . . . 229

Int 21H Function 2DH   . . . . . . . . . . 229

Int 21H Function 2EH   . . . . . . . . . . 168

Int 21H Function 2FH   . . . . . . . . . . 216

Int 21H Function 30H   . . . . . . . . . . 195

Int 21H Function 31H   . . . . . . . . . . 210

Int 21H Function 33H   . . . . . . . . . . 195

Int 21H Function 35H   . . . . . . . . . . 196

Int 21H Function 36H   . . . . . . . . . . 168

Int 21H Function 38H   . . . . . . . . . . 196

Int 21H Function 39H   . . . . . . . . . . 162

Int 21H Function 3AH   . . . . . . . . . . 163

Int 21H Function 3BH   . . . . . . . . . . 163

Int 21H Function 3CH   . . . . . . . . . . 170

Int 21H Function 3DH   . . . . . . . . . . 171

Int 21H Function 3EH   . . . . . . . . . . 171

Int 21H Function 3FH   . . . . . . . . . . 217

Int 21H Function 40H   . . . . . . . . . . 218

Int 21H Function 41H   . . . . . . . . . . 172

Int 21H Function 42H   . . . . . . . . . . 219

Int 21H Function 43H   . . . . . . . . . . 173

Int 21H Function 44H   . . . . . . . . . . 198

Int 21H Function 45H   . . . . . . . . . . 173, 174

Int 21H Function 46H   . . . . . . . . . . 174

Int 21H Function 47H   . . . . . . . . . . 164

Int 21H Function 48H   . . . . . . . . . . 190

Int 21H Function 49H   . . . . . . . . . . 190

Int 21H Function 4AH   . . . . . . . . . . 191

Int 21H Function 4BH   . . . . . . . . . . 212

Int 21H Function 4CH   . . . . . . . . . . 213

Int 21H Function 4DH   . . . . . . . . . . 214

Int 21H Function 4EH   . . . . . . . . . . 175

Int 21H Function 4FH   . . . . . . . . . . 176

Int 21H Function 54H   . . . . . . . . . . 169

Int 21H Function 56H   . . . . . . . . . . 178

Int 21H Function 57H   . . . . . . . . . . 179

Int 21H Function 58H   . . . . . . . . . . 191

Int 21H Function 59H   . . . . . . . . . . 199

MASM 5.1

Int 21H Function 5AH   . . . . . . . . . . 179

Int 21H Function 5BH   . . . . . . . . . . 180

Int 21H Function 5CH   . . . . . . . . . . 219, 220

Int 21H Function 5EH Subfunction 00H   . . 201

Int 21H Function 5EH Subfunction 02H   . . 201

Int 21H Function 5EH Subfunction 03H   . . 202

Int 21H Function 5FH Subfunction 02H   . . 203

Int 21H Function 5FH Subfunction 03H   . . 204

Int 21H Function 5FH Subfunction 04H   . . 204

Int 21H Function 62H   . . . . . . . . . . 215

Int 21H Function 63H   . . . . . . . . . . 205

INSB   . . . . . . . . . . . . . . . . . . 71

Int 21H Function 65H   . . . . . . . . . . 206

INSW   . . . . . . . . . . . . . . . . . . 71

Int 21H Function 66H   . . . . . . . . . . 208

Int 21H Function 67H   . . . . . . . . . . 182

Int 21H Function 68H   . . . . . . . . . . 221

Interrupt 15H (Mouse Pointer)  . . . . . . 311

Interrupt 20H  . . . . . . . . . . . . . . 231

Interrupt 21H, Function 5EH

(Machine/Printer)  . . . . . . . . . . . . 201

INT  . . . . . . . . . . . . . . . . . . . 41

Interrupt 21H, Function 5FH (Device

Redirection)   . . . . . . . . . . . . . . 203

Interrupt 25H  . . . . . . . . . . . . . . 232

Interrupt 26H  . . . . . . . . . . . . . . 233

Interrupt 27H  . . . . . . . . . . . . . . 234

Interrupt 2FH  . . . . . . . . . . . . . . 235

IntToAsc   . . . . . . . . . . . . . . . . 21

IRET   . . . . . . . . . . . . . . . . . . 42

IRP  . . . . . . . . . . . . . . . . . . . 105

IRPC   . . . . . . . . . . . . . . . . . . 105

JA   . . . . . . . . . . . . . . . . . . . 34

JAE  . . . . . . . . . . . . . . . . . . . 34

JB   . . . . . . . . . . . . . . . . . . . 34

JBE  . . . . . . . . . . . . . . . . . . . 34

JC   . . . . . . . . . . . . . . . . . . . 34

JCXZ/JECXZ   . . . . . . . . . . . . . . . 34

JE   . . . . . . . . . . . . . . . . . . . 34

JG   . . . . . . . . . . . . . . . . . . . 34

JGE  . . . . . . . . . . . . . . . . . . . 34

JL   . . . . . . . . . . . . . . . . . . . 34

JLE  . . . . . . . . . . . . . . . . . . . 34

JMP  . . . . . . . . . . . . . . . . . . . 32

JNA  . . . . . . . . . . . . . . . . . . . 34

JNAE   . . . . . . . . . . . . . . . . . . 34

JNB  . . . . . . . . . . . . . . . . . . . 34

JNBE   . . . . . . . . . . . . . . . . . . 34

JNC  . . . . . . . . . . . . . . . . . . . 34

JNE  . . . . . . . . . . . . . . . . . . . 34

JNG  . . . . . . . . . . . . . . . . . . . 34

JNGE   . . . . . . . . . . . . . . . . . . 34

JNL  . . . . . . . . . . . . . . . . . . . 34

JNLE   . . . . . . . . . . . . . . . . . . 34

JNO  . . . . . . . . . . . . . . . . . . . 34

JNP  . . . . . . . . . . . . . . . . . . . 34

JNS  . . . . . . . . . . . . . . . . . . . 34

JNZ  . . . . . . . . . . . . . . . . . . . 34

JO   . . . . . . . . . . . . . . . . . . . 34

JP   . . . . . . . . . . . . . . . . . . . 34

JPE  . . . . . . . . . . . . . . . . . . . 34

MASM 5.1

JPO  . . . . . . . . . . . . . . . . . . . 34

JS   . . . . . . . . . . . . . . . . . . . 34

JZ   . . . . . . . . . . . . . . . . . . . 34

Keyboard Scan Codes  . . . . . . . . . . . 150

LABEL  . . . . . . . . . . . . . . . . . . 95

LAHF   . . . . . . . . . . . . . . . . . . 62

LDS  . . . . . . . . . . . . . . . . . . . 52

LE   . . . . . . . . . . . . . . . . . . . 141

LEA  . . . . . . . . . . . . . . . . . . . 51

LEAVE  . . . . . . . . . . . . . . . . . . 43

LES  . . . . . . . . . . . . . . . . . . . 52

LoadPtr  . . . . . . . . . . . . . . . . . 52

LOCAL (in macros)  . . . . . . . . . . . . 104

LOCK   . . . . . . . . . . . . . . . . . . 74

LODS   . . . . . . . . . . . . . . . . . . 49, 80

LODSB  . . . . . . . . . . . . . . . . . . 49, 80

LODSW  . . . . . . . . . . . . . . . . . . 49, 80

LOOP   . . . . . . . . . . . . . . . . . . 37

LOOPE  . . . . . . . . . . . . . . . . . . 38

LOOPNE   . . . . . . . . . . . . . . . . . 38

LOW  . . . . . . . . . . . . . . . . . . . 135

LT   . . . . . . . . . . . . . . . . . . . 140

MACRO  . . . . . . . . . . . . . . . . . . 103

MASK   . . . . . . . . . . . . . . . . . . 133

MOD  . . . . . . . . . . . . . . . . . . . 125

MOV  . . . . . . . . . . . . . . . . . . . 44

MOVS   . . . . . . . . . . . . . . . . . . 45, 79

MOVSB  . . . . . . . . . . . . . . . . . . 45, 79

MOVSW  . . . . . . . . . . . . . . . . . . 45, 79

MUL  . . . . . . . . . . . . . . . . . . . 18

NE   . . . . . . . . . . . . . . . . . . . 139

NEG  . . . . . . . . . . . . . . . . . . . 17

NOP  . . . . . . . . . . . . . . . . . . . 73

NOT  . . . . . . . . . . . . . . . . . . . 69, 131

OFFSET   . . . . . . . . . . . . . . . . . 138

OR   . . . . . . . . . . . . . . . . . . . 67, 129

ORG  . . . . . . . . . . . . . . . . . . . 96

OUT  . . . . . . . . . . . . . . . . . . . 72

OUTS   . . . . . . . . . . . . . . . . . . 72

OUTSB  . . . . . . . . . . . . . . . . . . 72

OUTSW  . . . . . . . . . . . . . . . . . . 72

PAGE   . . . . . . . . . . . . . . . . . . 102

PopAll   . . . . . . . . . . . . . . . . . 78

POP  . . . . . . . . . . . . . . . . . . . 77

POPA   . . . . . . . . . . . . . . . . . . 78

POPF   . . . . . . . . . . . . . . . . . . 61

PROC   . . . . . . . . . . . . . . . . . . 93

PSP  . . . . . . . . . . . . . . . . . . . 151

PTR  . . . . . . . . . . . . . . . . . . . 135

PUBLIC   . . . . . . . . . . . . . . . . . 109

PURGE  . . . . . . . . . . . . . . . . . . 104

PUSH   . . . . . . . . . . . . . . . . . . 75

PUSHA  . . . . . . . . . . . . . . . . . . 77

PUSHF  . . . . . . . . . . . . . . . . . . 61

RCL  . . . . . . . . . . . . . . . . . . . 24

MASM 5.1

RCR  . . . . . . . . . . . . . . . . . . . 24

RECORD   . . . . . . . . . . . . . . . . . 121

REP  . . . . . . . . . . . . . . . . . . . 82

REPE   . . . . . . . . . . . . . . . . . . 83

REPNE  . . . . . . . . . . . . . . . . . . 83

REPT   . . . . . . . . . . . . . . . . . . 105

RET  . . . . . . . . . . . . . . . . . . . 40

ReadCharAttr   . . . . . . . . . . . . . . 245

Register   . . . . . . . . . . . . . . . . 149

RETF   . . . . . . . . . . . . . . . . . . 40

RETN   . . . . . . . . . . . . . . . . . . 40

Register Setup   . . . . . . . . . . . . . 150

ROL  . . . . . . . . . . . . . . . . . . . 24

ROR  . . . . . . . . . . . . . . . . . . . 24

SAHF   . . . . . . . . . . . . . . . . . . 63

SAL  . . . . . . . . . . . . . . . . . . . 25

SAL/SAR/SHL/SHR  . . . . . . . . . . . . . 131

SAR  . . . . . . . . . . . . . . . . . . . 25

SBB  . . . . . . . . . . . . . . . . . . . 15

SCAS   . . . . . . . . . . . . . . . . . . 81

SCASB  . . . . . . . . . . . . . . . . . . 81

SCASW  . . . . . . . . . . . . . . . . . . 81

SEG  . . . . . . . . . . . . . . . . . . . 138

SetCurPos  . . . . . . . . . . . . . . . . 241

SetCurSize   . . . . . . . . . . . . . . . 240

SEGMENT  . . . . . . . . . . . . . . . . . 112

SHL  . . . . . . . . . . . . . . . . . . . 25

SHORT  . . . . . . . . . . . . . . . . . . 135

SHR  . . . . . . . . . . . . . . . . . . . 25

SIZE   . . . . . . . . . . . . . . . . . . 136

Sound  . . . . . . . . . . . . . . . . . . 70

String Operations  . . . . . . . . . . . . 78

STC  . . . . . . . . . . . . . . . . . . . 60

STD  . . . . . . . . . . . . . . . . . . . 60

STI  . . . . . . . . . . . . . . . . . . . 60

STOS   . . . . . . . . . . . . . . . . . . 50, 80

STOSB  . . . . . . . . . . . . . . . . . . 50, 80

STOSW  . . . . . . . . . . . . . . . . . . 50, 80

STRUC  . . . . . . . . . . . . . . . . . . 122

SUB  . . . . . . . . . . . . . . . . . . . 14

SUBTTL   . . . . . . . . . . . . . . . . . 102

SubLong  . . . . . . . . . . . . . . . . . 16

TEST   . . . . . . . . . . . . . . . . . . 30

THIS   . . . . . . . . . . . . . . . . . . 136

TITLE  . . . . . . . . . . . . . . . . . . 102

Type Operator Summary  . . . . . . . . . . 134

TYPE   . . . . . . . . . . . . . . . . . . 137

VeriAnsi   . . . . . . . . . . . . . . . . 51

VeriCop  . . . . . . . . . . . . . . . . . 330

WAIT   . . . . . . . . . . . . . . . . . . 74

WinOpen  . . . . . . . . . . . . . . . . . 46

WIDTH  . . . . . . . . . . . . . . . . . . 134

XCHG   . . . . . . . . . . . . . . . . . . 48

XLAT   . . . . . . . . . . . . . . . . . . 53

XLATB  . . . . . . . . . . . . . . . . . . 53

XOR  . . . . . . . . . . . . . . . . . . . 68, 130

MASM 5.1

[] (Index Operator)  . . . . . . . . . . . 126

\ (Line Continutation)   . . . . . . . . . 142