How to use 80-Track Drives on a PCjr

by P. Eskildsen

Many of us dyed-in-the-wool PCjr users want to try everything possible

with our machines to get the last ounce of performance from a machine

that, although much maligned by many (mostly ill-informed, so-called

PC-journalists), is still quite versatile and still

remains the most economical, genuine IBM.

I have tried to use 80-track (quad density) drives of 720k to 1.2mb on my

Jr with mixed success and have searched for a good, reliable solution. At

one time I thought that the Weltec 1.2mb floppy (which sort of emulates

an AT drive but works from a PC-type FDC) was the answer. BUT the drive

is *VERY SLOW* and not reliable on a PCjr. PLUS you must purchase special

disks which are expensive. AND the company is not responsive to consumer

problems (I'm still trying after three months to get my defective unit

repaired, replaced or returned).

Well, here it is!

DOS 3.2 was created for the new IBM Convertible (laptop) and includes

support for every disk format IBM has ever used.

The 80-track 720k format used on the convertible's 3 1/2" floppies sounded

similar to the most common 9-sector format used by many 80-track quad

density drives on other machines.

YOU NEED

IBM PCDOS 3.2 on 5 1/4" bootable disk

IBM PCjr 128k minimun

2d drive PCjr modification (mine was the Creative Firmware

board that plugs into the modem slot - but others may

work as well)

80-Track drive (5 1/4" OR 3 1/2")

External power supply, cabinet and cable for the drive

WHAT TO DO

Make up a bootable DOS 3.2 DSDD system disk

At the DOS prompt enter:

copy con config.sys [press enter]

device=driver.sys /d:1 [press enter]

device=driver.sys /d:1 [press enter]

^Z [Fn6 followed by enter - writes the file to disk]

Set up the 80-track drive

for Mitsubishi M4853 jumper DS0, & HM only on the

block of 8 jumpers adjacent to the 34 'finger'

edge card connector.

for CDC 1503 (full-height) jumper leave positions 'M'

and '1' jumpered on the 14 pin socket close to J1.

(You can break the straps on the jumper block or

just remove the plug-in block and make jumpers out

of a couple of 5/8" lengths of solid hook-up wire).

Setup for Home Computer Magazine modified FDC

for CDC 1503 positions 'M' and '2' are jumpered

WHAT YOU GET

You now have a disk that will boot with DOS 3.2 and gives:

Drive A: & B: are 360k DSDD

Drive C: & D: are 720k DSQD

You can format standard SSDD (cheap) disks as DSQD in drive C:

or D: as QD.

format c:/v [enter]

You can do diskcopy from C: to D:

You can do diskcomp from C: to D:

The directory of your QD disk can be read by the AT 1.2mb drive

You can read directories of 40-track DSDD disks in the QD drive

You get cheap pretty high density storage at a cheap price.

Your disk access time on the QD drive is a little faster than

a standard DSDD drive (the step rate and distance is shorter)

I have *NEVER* had a SSDD disk give any bad sectors when

formatted as QD so media cost/kb of storage is very low.

This procedure may make a reasonable backup alternative for

PCjr owners with hard disks. (I have tried Irwin and other

tape drives on my 25mb HD PCjr without success)

WHAT YOU DON'T GET

Your QD drive will not read or write files to a 40-track disk

Your QD disk files can not be read or written to by the AT 1.2mb

FD drive even though that is also an 80-track drive.

Your QD disks if formatted with '/s' can not boot an AT (or PC or

XT without patching)

p.s.  There is no reason why the above principles won't work on an AT, PC

or XT with appropriate modifications, but I haven't tried it myself.

Have at it!

pse 8-14-86 Lafayette, CA

# N81N         #RBBS#1 (kman)           #  402-391-6739#