SB-021

Service notes on TS-520                             10-21-77

TRANSMITTER

1.   No switching of final relay:  Bad Q8 on X54-0001-00 2SA562Y.

2.   No drive:  C3 100pf in final box is cold soldered.

3.   Low output:  Check the plate load cap for touching plates.

4.   3.5 MHz low output:  Replace burned 47pf 3KV at load wafer.

5.   No TX:  Final choke burned out in final VOX L8.

6.   No RX or TX:  Bad common amp. Q1 3SK35GR in IF unit X48-1060-00.

7.   No ACC:  No plate current Q2 3SK41 If unit X48-1060-00.

8.   No TX relay:  Q8 2SA562Y on VOX unit.  Also R18 1K resistor relay

control.

9.   No TX or RX:  Check 9 volt terminal on FIX AVR (X43-1100-00) is loose.

10.  No CW TX: Bad 9 volt regulator.  Check D3 WZ061 on X43-1100-00.

11.  No TX:  Solder touch between TIF and key terminal on X44-1080-00.

12.  No drive to TX:  Check RL1 on GEN unit, small reed relay (X52-0010-

01).

13.  No ALC meter action on 10 meters:  This is normal for TS-520.

14.  CW chirp:  Q4 2SA606L on Fix  channel AVR X43-1100-00.

15.  ALC meter to maximum when TX:  RFI terminal at X44-1080-00 is not

soldered.

16.  No TX output:  82pf 3KV in final is burned.

17.  No TX output:  Q3 on RF unit 3SK41 X44-1080-00.

18.  No bias:  10 ohm cathode resistors burned in final VOX.

19.  No drive or TX output:  R70-71 RF unit, R5 Rectifier unit.

20.  No VOX:  Q8 in VOX unit X54-0001-00.

21.  No TX:  12BY7 is bad.  Shorts and burns out.  R71 and 70.

22.  No carrier output:  Q1 on Carrier X50-0009-01.

23.  No drive - no screen voltage:  R5 470 ohm open on rectifier X43-1090-

00.

24.  No drive - no ALC:  Q2 3SK35Y IF unit.  Q3 3SK41L RF unit.

25.  No RF output:  C13 0.04uf at pin 1 of S2001A is bad.

26.  Low mic gain:  Q1 2SC1000 on Gen unit.

27.  No TX:  Misconnection of brown wire after installation of CW filter.

RECEIVER

1.   Low Sensitivity:  Touching RLT and 9V at function switch.

2.   Receiver sensitivity:  No -6v at AVR:  Q8 2SC735 on X43-1100-00 is

bad.

3.   Frequency Shift:  Bad carreir Xtal 3395 on carrier unit.

4.   VFO drift:  C6 22pf, VFO unit.

5.   No receive:  Bad Q4, 3rd IF amp 3SK35GR on X48-1060-00.

6.   Receiver sensitivity and bias current jump, -6v unstable:  T1

oscillator transformer for -6v is bad on X43-1100-00.

7.   Erratic AGC action:  C11 100uf 16Von X431100-00.  Fix channel AVR is

bad.

8.   Frequency shift when warmed up:  Q1 in VFO 3SK22GR.

9.   No RX or TX:  If unit C59 touch to T12 IF transformer.

10.  Receiver noise:  Q5 3SK35Y X44-1080-00.

11.  Receiver sensitivity down:  NBI terminal is pulled loose.  Receiver at

IF board.

12.  No S meter reading and RF gain CCW.  No meter deflection:  Q3 on IF

unit.  Q5 on RF unit.

13.  VFO frequency jumped:  There is loose tension inside VFO.

14.  VFO frequency shift:  Cold solder of VFO output lead to VFO output

socket.

15.  Blows fuse when first on:  Check Q1 and Q2 DC-DC converter transistor.

One is possibly shorted.

16.  Noise blanker inoperative:  R6 4.7K resistor is broken at Q2 on X54-

1080-10.

17.  Loud audio squeal:  Q4 3SB473 on X49-0008-01 shorted.

18.  Low receiver sensitivity:  Diode D4 1S1587 on RF unit.

19.  VFO intermittant:  Wire to L1 in VFO is not soldered.

20.  No receive:  Q4 3SK35Y, Q5 3SK35GR on RF unit.

21.  No receive:  Q9 3SK35GR on IF unit.

22.  Hash noise in receiver:  Q3 on RF unit 3SK41.

23.  Blows fuse:  C31 power supply is shorted, 100uf 500v.  Also check

associated diode D3, D4.

24.  No RX on all bands:  WWV switch is broken or a wire is loose or not

soldered.

25.  Receiver sensitivity:  Q5 on RF amp 3SK35Y.

26.  Receiver sensitivity:  NBO terminal on X48-1060-00 is loose.

27.  Low audio:  Check for purple wire touch to ground at 12 pin connector

in rear panel.

(c) 102177TKC

Downloaded From P-80 International Information Systems 304-744-2253