SB-803

TS-180S CW Delay During VOX Memory Operation                 2-7-80

Some operators may report missing the first dit during CW VOX operation on

the memories.  This will occur only in early units, and may be improved by

changing one capacitor in the DFC.

On the DFC VCO unit X50-1550-00, check C3 at the base of Q3.  If C3 is a

22uF electrolytic, change to a .01uf disc.  No adjustments are necessary.

JEB/yn

(c) 2780TKC

Downloaded From P-80 International Information Systems 304-744-2253