SB-822

TR-2400 Battery Charger Failure                        4-22-80

The stock charger (W09-0307-05) supplied with the TR-2400 may be rapaired

and/or tested as necessary.

Open circuit, the charger should deliver 18-20V.  Current when charging

sould be 45mA.

Contained in the charger, in series with the output is a 120 ohm 1/2 watt

current limiting resistor.  Under normal use (including momentary short

circuit test) this part should not fail.

If you receive a charger with no output, the unit may be opened by

fracturing the glued seam by a sharp moderate hammer blow.  Place the

charger on its side ofon a hard, flat surface.  A layer of cloth at top and

bottom will protect the case from marring.  Strike the seam first on one

side, and then the other.  The case should open in two pieces.

Replace the 120 ohm 1/2 W resistor with a 1 W resistor.  After replacing

and before closing the unit, perform voltage and current checks.   A spot

of glue or cement at each corner will reseal the charger.

JEB/yn

(c) 42280TKC

Downloaded From P-80 International Information Systems 304-744-2253