SB-850

TS-180S Low Pass Filter Optional Changes                       8-24-81

Supercedes Bulletin dated 9/14/79

1.   Transmitter ALC levels may be increased by these optional component

changes.  Check for correct component values when installing the CK-18 kit.

On the LPF Unit X51-1180-00 change:

7 MHz

C14 from 470pf to 390pf  (CM93D2H391J)

C17 from 470pf to 390pf  (CM93D2H391J)

18 and 21 MHz

C25 from 27pf to 39pf    (CC45CH2H390J)

C26 from 82pf to 68pf    (CC45CH2H680J)

C28 from 82pf to 120pf   (CC45CH2H121J)

C29 from 27pf to 39pf    (CC45CH2H390J)

24.5 and 28 MHz

C30 from 18pf to 39pf    (CC45CH2H390J)

C34 from 18pf to 39pf    (CC45CH2H390J)

All capacitors should be 500V silver mica or disc ceramic, 5%.

ALC Adjustment.  After these component changes, perform ALC alignments 4-4,

4-5, and 4-6 as listed in the Service Manual.  Be sure to adjust to spec.;

incorrect meter zero (4-4), output exceeding 95W (4-5), or failure to peak

T1 (RF unit) and L15 (IF unit) (4-6) will result in low or no ALC and

possible future damage to the power amplifier.

2.   Fast protection with a linear amplifier.  While working in the LPF

unit, check the value of C73 (base of Q1).  If it is 1uf, change to a 3.3

uf @ 25v dc tantalum (CS15E1E3R3M).  This will compensate fro slow linear

amplifier relay closure.

Note:  These changes are at the owners option and may not be performed in

warranty.

JEB/sh

(c) 82481TKC

Downloaded From P-80 International Information Systems 304-744-2253