SB-906

TM-2570 Low TX Power Output                 3-18-86

Some owners of the TM-2570 have reportecd low transmitter power when the

radio is used with an antenna having reasonably low VSWR (1.5:1)

Procedure:

1.   Remove the bottom cover on the heat sink.

2.   Ensure that C72 is installed correctly.  (Refer to Service Manual)

3.   Squeeze the coils of L16 closer together.

4.   Make sure that C72 and L14 are as far away from each other as

possible, move the capacitor, not the inductor.

5.   Readjust VR1-VR4, according to the instructions provided in the

Service Manual.  The protection circuit, VR4, should be adjusted for 8A at

146.000 Mhz rather than the values indicated in the Service manual.  Change

your service manual to reflect this value.

6.   Replace the bottom cover to complete the procedure.

Time required for this modification is 1/2 hour or less.

(c) 121589TKC

Downloaded From P-80 International Information Systems 304-744-2253