MININEC Antenna Analysis

MN is an attempt to make the MININEC antenna analysis program easier to

use, especially by amateur radio operators.  No changes have been made to the

modeling algorithm contained in MININEC version 10, as released by the Naval

Ocean Systems Center (except to fix a bug involving S-parameter loads), but

substantial changes have been made in the way the program interacts with the

user.  This note is just intended to help you with some of the new features.

The easiest way to learn to use the program in the first place is by trying it

out on the sample antenna files provided.

New Features

The most important new feature is a provision for antenna files.  This

allows the antenna characteristics to be captured from a text file instead of

from the keyboard.  This saves you the considerable trouble of reentering a

complicated antenna geometry by hand every time you start up the program.  It

allows you to interrupt your work, save it away, and come back to it later when

it is convenient, without having to start from scratch. It also allows you to

build up your own library of designs, from which you can easily retrieve and

modify antennas for new applications.  You can use any text editor to create

the antenna file, as long as it comes out in ordinary ASCII characters.

Commands are available for listing, loading, viewing, and editing antenna

files.

If you have a Hercules Graphics Card, or compatible monochrome graphics

card, you can plot the azimuth and elevation directive patterns on your screen

in polar form using the optional Plotting Program.  The standard ARRL log dB

plotting scale is used, so that your patterns can be directly related to those

in amateur antenna publications.  The plots are saved automatically and can be

viewed at a later time without redoing the analysis.  A comparison mode allows

you to switch screens instantly between the plots of two antennas, permitting

very revealing comparisons.

A new command has been added to compute gain, front-to-back ratio,

maximum sidelobe level, and beamwidth.  You no longer have to extract these

antenna parameters manually from the directive pattern data.

You may specify the antenna dimensions in feet, inches, meters,

centimeters, or millimeters.  Elevation angle (the angle with respect to the

horizon) is used instead of zenith angle (the angle with respect to overhead).

A file containing a record of the data generated during the antenna analysis is

automatically created and given the same name as the antenna file, with the

extension .RUN.

The all-capital-letters printing style has been done away with, and

upper or lower case is accepted from the keyboard.  Defaults are provided for

most keyboard inputs and are indicated by [brackets].  Entering just a carriage

return selects the default.

MN will run either with or without an 8087 math coprocessor chip.  If

it takes more than one minute to fill and factor the mutual impedance matrix,

the program will beep when it is done to alert you that results are ready.

Antenna Files

An antenna file must have the extension .ANT.  The antenna file can be

specified as a command line parameter when starting the program; otherwise you

will be prompted for it.  In either case you need not enter the .ANT extension,

it will be supplied by the program.  The format is illustrated below by a

sample file for a 3 element beam.  No comments are actually permitted within a

file, but they may be added freely at the end.

3 element Yagi		{1 line title for the antenna}

free space	{Can be any other character string for antennas over ground}

24.94 MHz		{ "Hz", "KHz", or "GHz" may also be used}

3 wires, inches  	{ "feet", "meters", "centimeters", or "millimeters"}

10  0,-111.189,0  0,111.189,0  1	{For each wire: # segments, XYZ coords

10  -69,-117.666,0  -69,117.666,0  1	 of each end, radius}

10  73,-108.144,0  73,108.144,0  1

1 source		{Number of sources (feedpoints)}

5,100,0			{Segment number, voltage, phase for each source}

0 loads			{Number of loads}

The numbers can be separated by any combination of spaces and commas,

but not by tabs.  The words "wires", "source", and "loads" actually may consist

of any character strings, but must be present.  Everything is case insensitive.

X and Y are in the horizontal plane and +Z is up.  0 deg azimuth angle

is in the +X direction, and 90 deg azimuth is in the +Y direction.  The horizon

is at 0 deg elevation angle and +90 deg is overhead.  Unidirectional antennas

are assumed to be aimed in the +X direction by the subroutine that computes

gain, F/B, max sidelobe level, and beamwidth.  The Z coordinate can be set to 0

if only free space modeling is performed.

Segments and Wires

MN allows you to specify how many segments each wire is divided into

for analysis purposes.  The more segments used the higher the accuracy, but the

longer the analysis takes.  The number required depends on the complexity of

the antenna and the accuracy of the results required.  Generally 6-10 segments

per half wave are adequate for most purposes.  However, for each antenna you

should verify that the number of segments chosen is adequate by trying a larger

number to see if the results change significantly.  The number of wires is

limited to 50, and the total number of segments to 100 (twice the number of

segments as the original MININEC).

Segments include internally generated logical jumpers between the ends

of individual wires, as well as wire sections having finite length and carrying

current.  Segments are also called pulses.  Two wires having their ends at the

same XYZ coordinates will be considered to be connected by MN, and current will

be allowed to flow between the two wires as if they were soldered together.

For example, each loop of a cubical Quad antenna consists of 4 wires whose

endpoints share some common coordinates.

Sources

Feedpoints are called sources and are permitted at any pulse.  You must

have the feed symmetry in mind when you divide the driven wires into segments.

In the Yagi example above, the driven element uses an even number of segments

(10).  One is consumed internally by MN, leaving an odd number for antenna

currents (9), and thus a single central pulse at the feed point (pulse #5).

The first time an antenna file is run the .RUN output file generated by MN

should be examined to verify that the pulses have been distributed and numbered

the way you intended.  In fact, the easiest way to figure out which pulse

number to specify as the feed point is by making an initial guess, loading the

antenna file, quitting, and then examining the antenna geometry section of the

.RUN file to see how the segments really were allocated.

Loads

You may specify up to 50 loads for one antenna.  This number is large

to allow modeling of CCD antennas having many capacitors.  Here is an example

of an antenna using one resistive load:

ZL1ACW's big rhombic

free space

24.94 MHz

4 wires, feet

25   -182.7 0 35      0 81.35 35   .00337	{.00337 is the radius of

25   0 81.35 35       182.7 0 35   .00337	 #12 wire in feet}

25   182.7 0 35       0 -81.35 35  .00337

25   0 -81.35 35      -182.7 0 35  .00337

1 source

100,100,0

1 load

resistor	{This may be anything other than "s-parameter" for RL/RC loads}

50,740,0	{Pulse #, load resistance, load reactance}

S-parameter loads may be specified also.  This allows complex frequency

dependent loads to be modeled, unlike RL/RC loads whose reactance values do not

vary with frequency.  Here is an example of one:

W3DZZ trap dipole for 80 through 10 meters

free space

14.150 MHz

4 wires, feet

10  0 -54 0   0 -32 0  .00337

10  0 -32 0   0   0 0  .00337

10  0   0 0   0  32 0  .00337

10  0  32 0   0  54 0  .00337

1 source

20,100,0

2 loads

s-parameter

10,2		{Pulse #, order of s-parameter function}

0 1		{Numerator, denominator coefficents of s^0}

8.2 0		{    "           "          "       "  s^1}

0 4.92E-4	{    "           "          "       "  s^2}

30,2		{Second load ... }

0 1

8.2 0

0 4.92E-4	{Scientific notation allowed}

The traps for this antenna are 8.2uH in parallel with 60 pF.  The

S-parameter representation for a parallel LC circuit is:

Ls/(1+LCs^2)

L should be in uH and C in uF, since the program works in MHz

internally.

Real Ground

For antennas over ground the program will ask a series of questions to

establish the ground characteristics.  You can specify up to seven different

ground "media", each having its own dielectric constant and conductivity,

distance from the antenna, and height.  In addition, MN can accomodate ground

screens consisting of radials.  To model radials, enter 2 for number of media,

circular for boundary type, and the length of the radial wires for the X

coordinate of the next media interface.  Enter earth dielectric constant and

conductivity for both media.  The effective impedance of the ground screen is

then added in parallel to that of the first ground media.

Usually the most suitable reference for an antenna modeled over real

ground is not a dipole in free space, but a dipole or monopole having the same

polarization over the same kind of earth, at the same height, and operating at

the same frequency.  Earth reflection coefficients vary with polarization and

frequency, as well as with dielectric constant and conductivity.  Expect to see

negative gains in dBd terms at very low elevation angles, using realistic earth

characteristics, for antennas that exhibit gain in free space.  (Remember that

dBd refers to a dipole in free space, not to a dipole substituted for the test

antenna.)  In particular, keep in mind that vertical antennas have very little

response at elevation angles near 0 deg over real ground (even over salt

water).  The same is true for horizontal antennas over all types of ground

unless they are very high.  The file EARTH.DOC is included as a guide for

choosing the conductivity and dielectric constant of your particular earth when

modeling antennas over real ground.

Gain & F/B

The routine which finds beamwidth and maximum sidelobe level, and which

produces azimuth data for the plot, only searches the radiation pattern from 0

to 180 degrees.  To save time, it does not search the 0 to -180 degree half of

the pattern, which is assumed to be symmetric.  For antennas which do not

possess mirror symmetry, you can make a second run with the Y coordinates of

all wires negated to analyze the antenna in the missing half plane.  The

pattern search is done in 2 degree steps, and for an elevation angle of 0

degrees for free space models.  For antennas over ground, it prompts you for

the elevation angle to use while searching.  If the maximum lobe is found at 0

degrees azimuth angle (the normal case for unidirectional antennas aimed in the

+X direction) then the 3 dB beamwidth is displayed.  If a sidelobe is found

which is larger than the rear lobe then its level and angle are displayed.  If

the main lobe is found at an angle other than 0 its level and angle are shown.

The radiation pattern used in all cases is the total pattern, which is

the RMS sum of horizontal and vertical components.  This is a realistic model

for evaluating HF antenna performance on randomly polarized incoming skywave

signals.  It also facilitates the analysis of imperfect antennas.  For example,

quads show only about 25-30 dB front-to-side ratio due to incidental vertically

polarized radiation from the out-of-phase sides of the quad loops.  In

addition, the performance of Beverage antennas can be accurately modeled, since

this antenna consists of a long horizontal wire which responds principally to

vertically polarized signals, where the pickup of horizontal fields may reduce

directivity.  Note that for linearly polarized antennas the magnitude computed

for orthogonal polarization terms is typically down more than 130 dB in MN

analysis, so that the use of the total horizontal and vertical field will not

lead to inaccurate results where only the response at one polarization is

desired.

Aborting Calculations

Because some of the calculations can be quite time consuming, a

provision has been made for aborting from various program loops by pressing the

<Esc> key.  Aborting a calculation will not disturb any calculations already

completed, and the order of calculation has been arranged so that the most

interesting results are available first.  This allows you to use the abort

feature to get faster results, as well as to escape from command mistakes.  For

example, if you are only interested in obtaining input impedance, gain, or F/B,

you may hit <Esc> after they are displayed to terminate the azimuth pattern

search.  This search is done to find the beamwidth and maximum sidelobe level,

and also generates data for the azimuth plot.

Editing Antenna Files

Your favorite text editor or word processor may be invoked from within

MN to edit the current antenna file.  The name of your editor should be placed

in a file called MN.CFG.  MN will append the current antenna filename to the

editor name and execute this string as a DOS command whenever the E command is

entered.  After you exit the editor MN will reread the antenna file.  All

antenna parameters will be reset to those in the newly edited file, so any

temporary changes made using the Change commands will be lost.  The .RUN file

is also overwritten each time the antenna file is read back in.

Brian Beezley, K6STI

507 Taylor St.

Vista, CA 92084