Explanation of various PTP files

We have included several files as examples for how to set up your PK-232 for

several modes. The .SET files are there to place the PK-232 in the correct mode

and do not require explanation. If you have questions on these files, consult

your PK-232 manual.

Baudot Operation

When I want to go to baudot mode with my PK-232 I press F1, and then type

12,baudot.* . This single action causes my MOD file to change to BAUDOT.MOD, my

DEF file to change to BAUDOT.DEF and the commands in BAUDOT.SET to be sent to

the PK-232. I am then in complete (PTP and PK-232) baudot mode.

In the BAUDOT.MOD file that I use for baudot operation, I use my F7 key for a

transmit key. The key is set up as follows:

!^CX^M^Y

The exclamation point causes this F key to become an immediate key and precede

the contents of the type-ahead buffer which would contain any information you

keyed while you were in receive. The ^CX^M causes the PK-232 to switch to

transmit mode. The ^Y then causes the contents of the type-ahead buffer to be

sent to the PK-232. You are now in transmit mode and have just transmitted the

contents of the PTP type-ahead buffer (if their were any) and can begin typing

your information.

To go to receive mode, I define F9 as a "K" key as follows:

DE N4PY N4PY KKK^M^D^A

The action here goes like this: you are in transmit mode and when you press F9

"DE N4PY N4PY KKK" followed by carriage return is sent. Next the ^D (control D)

is sent to the PK-232 to tell the PK-232 to change to receive mode when the

PK-232 transmit buffer is empty. The ^A is then intercepted by PTP and tells PTP

to turn on the PTP type-ahead buffer. This key value is not sent to the PK-232.

You are now in receive mode with the PTP type-ahead buffer turned on.

I have the F8 key defined to be a receive key as follows:

^D^A

The ^D causes the PK-232 to go to receive mode when the PK-232 transmit buffer

is empty and the ^A tells PTP to turn on its type-ahead buffer.

I have the F10 key defined as a "73" key as follows:

73 DE N4PY N4PY N4PY^M^D

This causes " 73 DE N4PY N4PY N4PY" followed by carriage return to be sent to

the PK-232. The ^D causes the PK-232 to go to receive mode when the PK-232

transmit buffer is empty. I am then finished with the QSO and I leave the PTP

type-ahead buffer off at this point so I can type commands to the PK-232. If I

wanted to begin another QSO and use type-ahead prior to going to transmit mode,

I would hit control A on the keyboard to turn the PTP type-ahead buffer on.

In the BAUDOT.DEF file I define ALT-F1 as my brag key as follows:

^OB:BRAG

This causes a file called BRAG on the B drive to be sent. I have placed

pertinent station desciption information in this file.

I have defined ALT-F2 as I my CQ key. It is defined as follows:

CQ CQ CQ DE N4PY N4PY N4PY ^MCQ CQ CQ DE N4PY N4PY N4PY ^MCQ CQ CQ N4PY N4PY

N4PY KKK^M^M^D

This causes the CQ message to be sent and then the ^D causes the PK-232 to

return to receive mode when the PK-232 transmit buffer is empty.

I define my control F1 key to turn transmit reverse and receive reverse off in

the PK-232. I operate my TENTEC OMNI in reverse sideband and when I am on bands

20, 15, 12, and 10 meters this puts me in lower sideband so I do not want

reverse tones in the PK-232. The key is defined as follows:

RXREV OFF^M^W^WTXREV OFF^M

This causes RXREV OFF command to be sent followed by a .4 second wait followed

by TXREV OFF command.

I define my control F2 key to turn transmit reverse and receive reverse on in

the PK-232.  I operate my TENTEC OMNI in reverse sideband and when I am on bands

160, 80, 40, and 30 meters this puts me in upper sideband so I want reverse

tones in the PK-232.  The key is defined as follows:

RXREV ON^M^W^WTXREV ON^M

This causes RXREV ON command to be sent followed by a .4 second wait followed

by TXREV ON command.

Other pertinent lines in the DEF file for this mode are:

HOLDBUF

400

This causes the type-ahead buffer to be 400 characters long.

INITTA

ON

This sets the initial state for the type-ahead buffer to be on so that I can

immediately begin placing information in the PTP type-ahead buffer in response

to a CQ message.

PACKET

NO

This causes the PTP type-ahead buffer to not dump on carriage return and

prevents other packet oriented functions from not taking place.

PROMPT

Baudot

This causes the prompt on the center stripe to say "Baudot" to remind me what

mode I am in just in case I forgot.

Amtor Operation

When I want to go to amtor mode with my PK-232 I press F1, and then type

12,amtor.* . This single action causes my MOD file to change to AMTOR.MOD, my

DEF file to change to AMTOR.DEF and the commands in AMTOR.SET to be sent to

the PK-232. I am then in complete (PTP and PK-232) amtor mode.

In the AMTOR.MOD file that I use for amtor operation, I use my F7 key for a

link disconnect key. The key is set up as follows:

^M^M^MLINK DISCONNECTED^M73 DE N4PY^D

The message LINK DISCONNECTED is sent to the receiving station along with DE

N4PY. The ^D causes the PK-232 to actually disconnect the link.

To go to receive mode, I define F9 as a "K" key as follows:

^MDE N4PY^M^M^M+?

This causes a carriage return to be sent followed by "DE N4PY" followed by 3

carriage returns followed by the amtor changeover command of +?.

I have the F8 key defined to be a receive key as follows:

^M^M+?

This causes 2 carriage returns to be sent to the PK-232 followed by the amtor

changeover command.

I have the F10 key defined as a "73" key as follows:

^M^M73 DE N4PY^M

This causes 2 carriage returns to be sent followed by "73 DE N4PY" followed by

carriage return. I then either press F7 to disconnect the link or press F8 to

switch to receive.

In the AMTOR.DEF file I define ALT-F1 as my brag key as follows:

^OB:BRAG

This causes a file called BRAG on the B drive to be sent. I have placed

pertinent station desciption information in this file.

I have defined ALT-F2 as I my CQ key. I use this key in amtor FEC mode only. It

is defined as follows:

CQ CQ CQ DE N4PY N4PY N4PY (NNPY)^MCQ CQ CQ DE N4PY N4PY N4PY (NNPY)^MCQ CQ CQ

N4PY N4PY N4PY (NNPY)^MSELCALL IS NNPY NNPY^M^D

This causes the CQ message to be sent and then the ^D causes the PK-232 to

return to receive mode when the PK-232 transmit buffer is empty.

I define ALT-F7 to be my FEC transmit key. I define the key as follows:

!FE^M^W^W^W^W^W^W^W^W^W^W^W^W^W^W^W^W^M^M^Y

The exclamation point causes this key to go ahead of the contents of the PTP

type-ahead buffer (if their were any). FE^M causes the command FE followed by a

carriage return to be sent to the PK-232 placing the PK-232 in FEC transmit

mode. Next there are 17 delay characters to cuase a 3.4 second delay before

sending any information to allow a potential receiving station to sync to my

signal. The ^Y then sends the contents of the PTP type-ahead buffer to the

PK-232 if their were any. I would then normally press the ALT-F2 key to send my

CQ message.

I define my ALT-F8 key as my receive key when using FEC mode. It is defined as

follows:

^MDE N4PY DE N4PY DE N4PY ^M^M^D

This causes a carriage return to be sent to the PK-232 followed by the message

"DE N4PY DE N4PY DE N4PY" followed by 2 carriage returns. The ^D causes the

PK-232 to return to receive mode.

I define my control F1 key to turn transmit reverse and receive reverse off in

the PK-232. I operate my TENTEC OMNI in reverse sideband and when I am on bands

20, 15, 12, and 10 meters this puts me in lower sideband so I do not want

reverse tones in the PK-232. The key is defined as follows:

RXREV OFF^M^W^WTXREV OFF^M

This causes RXREV OFF command to be sent followed by a .4 second wait followed

by TXREV OFF command.

I define my control F2 key to turn transmit reverse and receive reverse on in

the PK-232.  I operate my TENTEC OMNI in reverse sideband and when I am on bands

160, 80, 40, and 30 meters this puts me in upper sideband so I want reverse

tones in the PK-232.  The key is defined as follows:

RXREV ON^M^W^WTXREV ON^M

This causes RXREV ON command to be sent followed by a .4 second wait followed

by TXREV ON command.

Other pertinent lines in the DEF file for this mode are:

DCD

YES

This causes PTP to look at the DCD line to detect connections in amtor mode

CONNECT BELL

YES

This causes PTP to sound a bell when an amtor connection is made.

HOLDBUF

400

This causes the type-ahead buffer to be 400 characters long.

INITTA

OFF

This sets the initial state for the type-ahead buffer to be off as I do not use

the PTP type-ahead buffer in amtor mode. Because the PK-232 is always in

converse mode when in an amtor QSO, the PK-232 type-ahead feature works fine in

this mode.

PACKET

NO

This causes the PTP type-ahead buffer to not dump on carriage return and

prevents other packet oriented functions from not taking place.

PROMPT

Amtor

This causes the prompt on the center stripe to say "Amtor" to remind me what

mode I am in just in case I forgot.

Kantronics KAM

We do not have access to a Kantronics KAM unit and so are not familiar with its

operation. However, with the versatility of function keys in PTP, we feel the

KAM can easily be used with PTP. We have included a sample .MOD file for both

baudot and amtor for use with the KAM. We do not know if the key setup is

correct for the KAM but feel it is close and also serves as an example of how to

set up files for other units.

The KAMBAUDO.MOD file is set up for KAM baudot operation as follows:

The F7 key is defined as a transmit key as follows:

!^CT^M^Y

The exclamation point causes this key to go ahead of the contents of the PTP

type-ahead buffer. The ^CT causes the KAM to go to transmit mode. The ^M then

sends a carriage return and finally the ^Y sends the contents ( if their are any

) of the PTP type-ahead buffer to KAM.

The F8 key is defined as a receive key as follows:

^CE^A

The ^CE causes the KAM to go to receive mode when the KAM transmit buffer is

empty and the ^A turns on the PTP type-ahead buffer to allow PTP type-ahead

while in receive mode.

The F9 key is defined as a "K" key as follows:

DE N4PY N4PY KKK^M^CE^A

This causes " DE N4PY N4PY KKK" followed ny a carriage return to be sent to KAM

followed by ^CE which tells KAM to return to receive mode after the contents of

the KAM transmit buffer have been transmitted. The ^A tell PTP to turn on the

PTP type-ahead buffer.

The F10 key is defined as a "73" key as follows:

73 DE N4PY N4PY N4PY^M^CE

This causes the text " 73 DE N4PY N4PY N4PY" followed by carriage return to be

sent to KAM. The ^CE places the KAM in receive mode after the KAM transmit

buffer is empty.

The KAMAMTOR.MOD file is set up for KAM amtor operation as follows:

The F7 key is defined as a link disconnect key as follows:

^M^M^MLINK DISCONNECTED^M73 DE N4PY^CX

This causes 3 carriage returns to be sent to KAM followed by "LINK DISCONNECTED"

followed by carriage return. Then the text "73 DE N4PY" is sent. The ^CX then

actually disconnects the link.

The F8 key is defined as a receive key as follows:

^M^M+?

Two carriage returns are sent followed by the amtor changeover command +?.

The F9 key is defined as a "K" key as follows:

^MDE N4PY^M^M^M+?

This causes a carriage return followed by "DE N4PY" followed by 3 carriage

returns to be sent to the KAM followed by the amtor changeover command.

The F10 key is defined as a "73" key as follows:

^M^M73 DE N4PY^M

This causes two carriage returns followed by the text "DE N4PY" followed by

carriage return to be sent to KAM. I then type either F8 to changeover to

receive or type F7 to disconnect the link.