Notes about BuildCD

===================

(c) Copyright 1993 Commodore-Amiga, Inc.  All Rights Reserved

Confidential.

The BuildCD software allows you to cut "gold disc" CD-ROMs using an

Amiga and a Phillips CDD-521 write-once CD-ROM cutter from data files

which have been prepared for CD-ROM such as an ISO-9660 image file,

MPEG tracks, and CD Audio tracks.

BuildCD features:

o Easy-to-use gadtools interface

o Cuts 0 or 1 Data tracks first, and any number of Audio or MPEG tracks

in any order

o Displays CD space and time used for all tracks

o Loads and saves track sheets for frequestly cut titles

Required Hardware

=================

You will need an Amiga with very fast SCSI, fast SCSI harddisks

with lots of storage space, a Philips CDD 521 Compact Disk Recorder

(which can cost up to $5000), plus blank gold disks.

The following Amiga configurations haved been tested:

1. 25MHz A3000

2. A4000 [25MHz MC68040, 8Mb FAST RAM] with the A4091 or FastLane

SCSI-2 host adapters.

We have not tested with an A2000/A2091.

The Philips drive requires a steady uninterrupted 300K/second sustained

transfer rate.  We do not know if this is possible with the any hardware

other than what we have tested.

Hard Disk Drives

================

For harddisk space, you will need approximately 3x the amount of space

your source data takes up, with 1/3 of that on a partition which can be

reformatted often and which can hold all of your final data for your CD.

We have successfully cut disks with samples of the following drives:

o Seagate/Imprimis/CDC Wren IV,VI,VII

o Seagate ST11200

o Maxtor "Magic" 540, 1240

Reselect

========

Important!!!  The Philips drive can not deal with other drives that have

Reselect On.  If a drive with Reselect On is on the bus when cutting

disks, you will get errors and will cut a bad disk.  You must be sure

that all SCSI devices on the bus have Reselect OFF.  For harddisks, this

can generally be accomplished with the ReselectOff script and icon

(ignore the error output for unit numbers you don't have).  For some

drives and other SCSI devices, you may have to consult the documentation

which came with the device to see if there is a jumper for reselection.

The Fish Disk tool "RDBInfo" (Disk 779) may be used to check the reslection

state of individual SCSI units.

Other Required Software/Hardware

================================

You need to create the data for your disk and get it into a standard

CD-ROM format before you can use BuildCD to place the data on the disk.

Suggested tools for getting your data in CD-ROM formats:

- ISO Image (data track)

For the Amiga, use the Amiga ISOCD program which AmigaCD licensees

receive without charge.  Only the Amiga ISOCD program can create

a bootable disk for CDTV or CD32.  Only licensees may have and use

this program.  Commodore subsidiaries and other licensees may not

build title disks for a developer unless the developer or publisher

has signed an AmigaCD license and amendment for that title.

For other systems (Mac and PC), there are commercial software

packages available for building ISO image files.  These packages

can not build a bootable Amiga CD32 disk.

- Audio tracks

Digitize with the Studio-16 sampler, and then use the tool called

"Conversion" by Jim Hawkins to place the sound sample bytes

in the correct order for a CD track.

- MPEG tracks

We have these made by providing our video to a company that converts

our video to MPEG.  See the list of some MPEG services at the end of

this document.  Before cutting the MPEG binary you receive from the

MPEG service, you may need to align and pad it properly by making

a copy of the stream with the tool called "Padstream".

CCOPY

=====

CCopy must be used to copy your CD-ready data (ISO image, audio tracks,

mpeg tracks) to a clean FastFileSystem partition (we'll call this the

"build" partition).  You then use BuildCD to cut the CD from the data

on your build partition.

CCopy is a special version of COPY which attempts to place files

in a manner optimal for sustained reads when transferring to the Philips

drive.  For CCopy to work properly, it must be used to copy data to an

Amiga FastFileSystem partition which is either freshly formatted,

OR has never had files deleted from it.  (i.e. you can CCopy multiple

ISO images, audio tracks, MPEG tracks, etc. to a fresh FFS partition

as long as you don't delete any files from the partition.  Deleting files

would leave a fragmented disk structure and subsequent CCopy'ed

files would not be laid down contiguously, possibly making it

impossible to sustain the transfer rate needed to cut a CD).

If possible, use the V40 Amiga filesystem on this partition and use

V40 HDToolbox to set the block size for the partition to 2048 instead

of 512.  Do this before formatting the partition.  Note - to make sure

the partition is actually using the 2048 blocksize, use the CLI "Info"

command after formatting and multiply the number of free blocks shown

by 2048 and make sure the result is close to the size of the partition

in bytes.  Use of 2048-size blocks will allow BuildCD to read the data

more quickly from your drive when cutting the CD.

MPEG

====

To include a properly encoded and interleaved SIF format MPEG track on a

CD32 disc...

1 - Be sure that the MPEG file is properly encoded and interleaved to comply

with the VideoCD standard.  Things to check for...

a - Encoded at the proper VideoCD bit-rates for audio and video.

b - Audio and Video properly interleaved for VideoCD

c - Ready to be written as Mode 2 Form 2 data (properly padded for

Mode 2 Form 2 sector size.)

d - Two seconds of blank sectors at head of data.  (only strictly

necessary for files that will be played by the ROM player

interface.)

If either "c" or "d" above are either not true or unsure, run the

"padstream" program on the MPEG data file.

Command format:

PadStream Source/A,Dest/A,VideoCD/S

Where source is the name of the file in question, Dest is the

file that will be written, and VideoCD is a command line switch

that should be used if the file will be part of a VideoCD

compliant disc that is intended to be played by the ROM VideoCD

player program.

2 - Use ISOCD to build your data file as normal.

3 - Use BuildCD 1.31 or greater to cut track 1 as data (with your ISOCD

generated ISO image as the data file), and tracks 2-99 as either

audio and/or MPEG tracks.  Be sure to specify MPEG for MPEG tracks

and Audio for Audio tracks.

MPEG ENCODING

=============

o What is the cost for MPEG encoding ?

Costs vary from between $60/min (Laser Pacific, if you tell them you're

a CD-I developer) and $170/min (Philips if you tell them you're a CD32

developer.)

o Are their any Micro based MPEG encoding solutions?

None yet that have the quality of a professional encoding job.  There

is a software based solution from a company called Helfrich in Germany.

It works using their Piccollo graphics card.  The software does a pretty

good job, but you obviously have to be willing to let it sit a long time

with each frame if you want decent quality.

We've heard that there are some PC solutions in the works, but haven't

actually seen anything available yet.

o Some MPEG Service Bureaus

Ms. Jane Sinclair

President

Advanced Multimedia

26600 Telegraph Road

Suite 181

Southfield, MI 48075 USA

P: 1 313 3502130

F: 1 313 3503618

Mr. Heinz Kroger

AVM AG

CH 4106 Therwil

Basel, Switzerland

P: 41 61 7218621

F: 41 61 7218070

Hiromi Aoyagi

Denshi Media Services

Taito 1 Chome

Tokyo, Taito-KU Japan

P: 81 3 32768071

F: 81 3 32750951

Mr. Tia Chou

Digital Video Systems, Inc.

536 Weddell Drive

Suite 9

Sunnyvale, CA 94089 USA

P: 1 408 7344238

F: 1 408 7348660

Mr. James E. Brathwaite

Epic Interactive Media Co., Ltd.

VPS House

52 Old Steine

Brighton

E. Sussex BN2 1NH

United Kingdom

P: 44 273 728600

Dennis Garlington

GTE Interactive Media

Suite 200A

Carlsbad, CA 92009 USA

P: 1 619 4318801

F: 1 619 4318755

Jeanette Depatie

Heuris Logic

1758 Muegge

St. Charles, MO 63303 USA

P: 1 314 9490415

Mr. Bruno Rahir

International Creative Digital Image SA

Boulevard Tirou 130/4th Floor

Carleroi, 6000

Belgium

P: 32 071 300910

F: 32 071 331229

Mr. Leon Silverman

Vice President of Marketing

Laser-Pacific Media Corp.

809 N. Cahuenga Blvd.

Los Angeles, CA 90038 USA

P: (213) 462-6266

Adrian Farmer - Deputy Chairman

Crack Downes  - Technical

Nimbus Information Systems

Raglan, Llantarnam Park, Cwmbran

Gwent NP44 3AB

P: (0633) 867777

F: (0633) 867799

Mr. Stephan Maris

N.V. Philips Industrial Activities

Kampische Steenweg 293

Hasselt, 3500

Belgium

P: 32 011 242167

F: 32 011 242168

Mr. Bentley Nelson

Pacific Video Resources

2339 Third Street M-4

San Francisco, CA 94107 USA

P: 1 415 8645679

F: 1 415 8642059

Paddy Roache

Philips Interactive Media Systems

City House

420-430 London Road

Croydon CR8 3QR

United Kingdom

P: 44 81 6894444

F: 44 81 6893395