TABLE OF CONTENTS

lowlevel.library/AddKBInt

lowlevel.library/AddTimerInt

lowlevel.library/AddVBlankInt

lowlevel.library/ElapsedTime

lowlevel.library/GetKey

lowlevel.library/GetLanguageSelection

lowlevel.library/QueryKeys

lowlevel.library/ReadJoyPort

lowlevel.library/RemKBInt

lowlevel.library/RemTimerInt

lowlevel.library/RemVBlankInt

lowlevel.library/SetJoyPortAttrsA

lowlevel.library/StartTimerInt

lowlevel.library/StopTimerInt

lowlevel.library/SystemControlA

lowlevel.library/AddKBInt                           lowlevel.library/AddKBInt

NAME

AddKBInt -- adds a routine to the keyboard interrupt. (V40)

SYNOPSIS

intHandle = AddKBInt(intRoutine, intData);

D0                   A0          A1

APTR AddKBInt(APTR, APTR);

FUNCTION

This routine extends the functionality of the keyboard interrupt to

include intRoutine. Since this is an extention of the normal

keyboard interrupt all of the keyboard handshaking is handled. The

keyboard error codes are filtered out and not passed to intRoutine.

The routine is called whenever the user enters a key on the

keyboard.

The routine is called from within an interrupt, so normal

restrictions apply. The routine must preserve the following

registers: A2, A3, A4, A7, D2-D7. Other registers are

scratch, except for D0, which MUST BE SET TO 0 upon

exit. On entry to the routine, A1 holds 'intData' and A5

holds 'intRoutine', and D0 contains the rawkey code read

from the keyboard.

The routine is not called when a reset is received from the

keyboard.

This is a low level function that does not fit the normal Amiga

multitasking model. The interrupt installed will have no knowledge

of which window/screen currently has input focus.

If your program is to exit without reboot, you MUST call RemKBInt()

before exiting.

Only one interrupt routine may be added to the system.  ALWAYS

check the return value in case some other task has previously

used this function.

INPUTS

intRoutine - the routine to invoke every vblank. This routine should

be as short as possible to minimize its effect on overall

system performance.

intData - data passed to the routine in register A1. If more than one

long word of data is required this should be a pointer to

a structure that contains the required data.

RESULT

intHandle - a handle used to manipulate the interrupt, or NULL

if it was not possible to attach the routine.

SEE ALSO

RemKBInt()

lowlevel.library/AddTimerInt                     lowlevel.library/AddTimerInt

NAME

AddTimerInt -- adds an interrupt that is executed at regular

intervals. (V40)

SYNOPSIS

intHandle = AddTimerInt(intRoutine, intData);

D0                      A0          A1

APTR AddTimerInt(APTR, APTR);

FUNCTION

Calling this routine causes the system to allocate a CIA timer

and set up 'intRoutine' to service any interrupts caused by the timer.

Although the timer is allocated it is neither running, nor enabled.

StartIntTimer() must be called to establish the time interval and

start the timer.

The routine is called from within an interrupt, so normal

restrictions apply. The routine must preserve the following

registers: A2, A3, A4, A7, D2-D7. Other registers are

scratch, except for D0, which MUST BE SET TO 0 upon

exit. On entry to the routine, A1 holds 'intData' and A5

holds 'intRoutine'.

Only a single CIA timer will be allocated by this routine. So this

routine may only be called once without an intervening call to

RemTimerInt().

The CIA timer used by this routine is not guaranteed to always be

the same. This routine utilizes the CIA resource and uses an

unallocated CIA timer.

If your program is to exit without reboot, you MUST match all

calls to this function with calls to RemTimerInt() before exiting.

Even if you only use the function once in your program; checking

the return value will make your program more tolerant for

mulititasking on the Amiga computer platforms.

INPUTS

intRoutine - the routine to invoke upon timer interrupts. This routine

should be as short as possible to minimize its effect on

overall system performance.

intData - data passed to the routine in register A1. If more than one

long word of data is required this should be a pointer to

a structure that contains the required data.

RESULT

intHandle - a handle used to manipulate the interrupt, or NULL

if it was not possible to attach the routine.

SEE ALSO

RemTimerInt(), StopTimerInt(), StartTimerInt()

lowlevel.library/AddVBlankInt                   lowlevel.library/AddVBlankInt

NAME

AddVBlankInt -- adds a routine executed every vertical blank. (V40)

SYNOPSIS

intHandle = AddVBlankInt(intRoutine, intData);

D0		         a0	  a1

APTR AddVBlankInt(APTR, APTR);

FUNCTION

Lets you attach a routine to the system which will get called

everytime a vertical blanking interrupt occurs.

The routine is called from within an interrupt, so normal

restrictions apply. The routine must preserve the following

registers: A2, A3, A4, A7, D2-D7. Other registers are

scratch, except for D0, which MUST BE SET TO 0 upon

exit. On entry to the routine, A1 holds 'intData' and A5

holds 'intRoutine'.

If your program is to exit without reboot, you MUST call

RemVBlankInt() before exiting.

Only one interrupt routine may be added to the system.  ALWAYS check

the return value in case some other task has previously used this

function.

INPUTS

intRoutine - the routine to invoke every vblank. This routine should

be as short as possible to minimize its effect on overall

system performance.

intData - data passed to the routine in register A1. If more than one

long word of data is required this should be a pointer to

a structure that contains the required data.

RESULT

intHandle - a handle used to manipulate the interrupt, or NULL

if it was not possible to attach the routine.

SEE ALSO

RemVBlankInt()

lowlevel.library/ElapsedTime                     lowlevel.library/ElapsedTime

NAME

ElapsedTime -- returns the time elapsed since it was last called. (V40)

SYNOPSIS

fractionalSeconds = ElapsedTime(context);

D0                              A0

ULONG ElapsedTime(struct EClockVal *);

FUNCTION

This function utilizes the timer.device/ReadEClock() function to get

an accurate elapsed time value. Since the context needs to be

established the first call to this routine will return a nonsense

value.

The return value for this function only allows for sixteen bits

worth for the integer number of seconds and sixteen bits for the

factional number of seconds.

With sixteen bits worth of integer seconds this function can be

used to timer an interval up to about 16 hours. If the actual time

interval is larger this function will return this maximum value.

The sixteen bits for fractional seconds gives a resolution of

approximately 20 microseconds. However, it is not recomended

to expect this function to be accurate for a time interval of

less than 200 microseconds.

INPUTS

context - pointer to an EClockVal structure. The first time you

call this function, you should initialize the structure

to 0s. You should then reuse the same structure for

subsequent calls to this function, as this is how the

elapsed time is calculated.

RESULT

fractionalSeconds - The elapsed time as a fixed point 32-bit

number with the point fixed in the middle.

That is, the upper order sixteen bits represent

the number of seconds elapsed. The low order

sixteen bit represent the fractional number of

seconds elapsed. This value is limited to about

sixteen hours. Although this value is precise

to nearly 20 microseconds it is only accurate to

within 200 microseconds.

WARNING

The first call to this function will return a non-sense value. Only

rely on its result starting with the second call.

SEE ALSO

timer.device/ReadEClock()

lowlevel.library/GetKey                               lowlevel.library/GetKey

NAME

GetKey -- returns the currently pressed rawkey code and qualifiers.

(V40)

SYNOPSIS

key = GetKey();

D0

ULONG GetKey(VOID);

FUNCTION

This function returns the currently pressed non-qualifier key and

all pressed qualifiers.

This function is safe within an interrupt.

This is a low level function that does not fit the normal Amiga

multitasking model. The values returned by this function are

not modified by which window/screen currently has input focus.

RESULT

key - key code for the last non-qualifier key pressed in the low

order word. If no key is pressed this word will be FF. The

upper order word contains the qualifiers which can be found

within the long word as follows:

Qualifier               Key

LLKB_LSHIFT             Left Shift

LLKB_RSHIFT             Rigt Shift

LLKB_CAPSLOCK           Caps Lock

LLKB_CONTROL            Control

LLKB_LALT               Left Alt

LLKB_RALT               Right Alt

LLKB_LAMIGA             Left Amiga

LLKB_RAMIGA             Right Amiga

SEE ALSO

<libraries/lowlevel.h>

lowlevel.library/GetLanguageSelection   lowlevel.library/GetLanguageSelection

NAME

GetLanguageSelection -- returns the current language selection. (V40)

SYNOPSIS

language = GetLanguageSelection();

D0

ULONG GetLanguageSelection (VOID);

FUNCTION

Determine what the user has specified as a language.

RESULT

language - user specified language, or zero if none has yet been

specified. See <libraries/lowlevel.h> for a definition

of the currently supported language.

SEE ALSO

<libraries/lowlevel.h>, locale.doc

lowlevel.library/QueryKeys                         lowlevel.library/QueryKeys

NAME

QueryKeys -- return the states for a set of keys. (V40)

SYNOPSIS

QueryKeys(queryArray, arraySize);

A0          D1

VOID QueryKeys(struct KeyQuery *, UBYTE);

FUNCTION

Scans the keyboard to determine which of the rawkey codes

listed in the QueryArray are currently pressed. The state for each

key is returned in the array.

This function may be invoked from within an interrupt, but the size

of QueryArray should be kept as small as possible.

This is a low level function that does not fit the normal Amiga

multitasking model. The values returned have no knowledge

of which window/screen currently has input focus.

INPUTS

queryArray - an array of KeyQuery structures. The kq_KeyCode fields

of these structures should be filled with the rawkey

codes you wish to query about. Upon return from this

function, the kq_Pressed field of these structures

will be set to TRUE if the associated key is down,

and FALSE if not.

arraySize - number of key code entries in queryArray

SEE ALSO

<libraries/lowlevel.h>

lowlevel.library/ReadJoyPort                     lowlevel.library/ReadJoyPort

NAME

ReadJoyPort -- return the state of the selected joy/mouse port. (V40)

SYNOPSIS

portState = ReadJoyPort(portNumber);

D0                      D0

ULONG ReadJoyPort(ULONG);

FUNCTION

This function is used to determine what device is attached to the

joy port and the current position/button state. The user may attach

a mouse, game controller, or joystick to the port and this function

will dynamically detect which device is attached and return the

appropriatly formatted portState.

To determine the type of controller that is attached, this function

clocks the game controller and/or interprets changes in the joy

port data. Valid clocked data from the game controller is immediately

detected. However, to accuratly determine if a mouse or joystick is

attached, several calls to this function are required along with some

movement at the joy port by the user.

This function always executes immediatly.

This is a low level single threaded function that does not fit the

normal Amiga multitasking model. Only one task can be executing

this routine at any time. All others will return immediately with

JP_TYPE_NOTAVAIL.

The nature of this routine is not meant to encourage non-multitasking

friendly programming practices like polling loops. If your task

is waiting for a transition to be returned use a WaitTOF() between

calls to minimize the total system impact.

When called the first time, for each port, this function attempts to

acquire certain system resources. In order to aquire these resources

this function MUST be called from a task, or a DOS process. If this

function fails to aquire the necessary resources, it will

return with JP_TYPE_NOTAVAIL. Once the resources are aquired (return

value other than JP_TYPE_NOTAVAIL) this function may be used in

interrupts.

INPUTS

portNumber - port to read, in the range 0 to 3.

RESULT

portState - bit map that identifies the device and the current

state of that device. The format of the bit map is

dependant on the type of device attached.

The following constants from <libraries/lowlevel.h>

are used to determine which device is attached and

the state of that device.

The type of device can be determined by applying

the mask JP_TYPE_MASK to the return value and comparing

the resultant value with the following:

JP_TYPE_NOTAVAIL        port data unavailable

JP_TYPE_GAMECTLR        game controller

JP_TYPE_MOUSE           mouse

JP_TYPE_JOYSTK          joystick

JP_TYPE_UNKNOWN         unknown device

If type = JP_TYPE_GAMECTLR the bit map of portState is:

JPF_BUTTON_BLUE         Blue - Stop

JPF_BUTTON_RED          Red - Select

JPF_BUTTON_YELLOW       Yellow - Repeat

JPF_BUTTON_GREEN        Green - Shuffle

JPF_BUTTON_FORWARD      Charcoal - Forward

JPF_BUTTON_REVERSE      Charcoal - Reverse

JPF_BUTTON_PLAY         Grey - Play/Pause

JPF_JOY_UP              Up

JPF_JOY_DOWN            Down

JPF_JOY_LEFT            Left

JPF_JOY_RIGHT           Right

If type = JP_TYPE_JOYSTK the bit map of portState is:

JPF_BUTTON_BLUE         Right

JPF_BUTTON_RED          Fire

JPF_JOY_UP              Up

JPF_JOY_DOWN            Down

JPF_JOY_LEFT            Left

JPF_JOY_RIGHT           Right

If type = JP_TYPE_MOUSE the bit map of portState is:

JPF_BUTTON_BLUE         Right mouse

JPF_BUTTON_RED          Left mouse

JPF_BUTTON_PLAY         Middle mouse

JP_MVERT_MASK           Mask for vertical counter

JP_MHORZ_MASK           Mask for horizontal counter

SEE ALSO

SetJoyPortAttrs()

lowlevel.library/RemKBInt                           lowlevel.library/RemKBInt

NAME

RemKBInt -- remove a previously installed keyboard interrupt. (V40)

SYNOPSIS

RemKBInt(intHandle);

A1

VOID RemKBInt(APTR);

FUNCTION

Remove a keyboard interrupt routine previously added with AddKBInt().

INPUTS

intHandle - handle obtained from AddKBInt(). This may be NULL,

in which case this function does nothing.

SEE ALSO

AddKBInt()

lowlevel.library/RemTimerInt                     lowlevel.library/RemTimerInt

NAME

RemTimerInt -- remove a previously installed timer interrupt. (V40)

SYNOPSIS

RemTimerInt(intHandle);

A1

VOID RemTimerInt(APTR);

FUNCTION

Removes a timer interrupt routine previously installed with

AddTimerInt.

INPUTS

intHandle - handle obtained from AddTimerInt(). This may be NULL,

in which case this function does nothing.

SEE ALSO

AddTimerInt(), StopTimerInt(), StartTimerInt()

lowlevel.library/RemVBlankInt                   lowlevel.library/RemVBlankInt

NAME

RemVBlankInt -- remove a previously installed vertical blank routine.

(V40)

SYNOPSIS

RemVBlankInt(intHandle);

A1

VOID RemVBlankInt(APTR);

FUNCTION

Removes a vertical blank interrupt routine previously added with

AddVBlankInt().

INPUTS

intHandle - handle obtained from AddVBlankInt(). This may be NULL,

in which case this function does nothing.

SEE ALSO

AddVBlankInt()

lowlevel.library/SetJoyPortAttrsA           lowlevel.library/SetJoyPortAttrsA

NAME

SetJoyPortAttrsA -- change the attributes of a port.  (V40.27)

SetJoyPortAttrs -- varargs stub for SetJoyPortAttrsA().  (V40.27)

SYNOPSIS

success = SetJoyPortAttrsA(portNumber, tagList);

D0                         D0          A1

BOOL SetJoyPortAttrsA(ULONG, struct TagItem *);

Success = SetJoyPortAttrs(portNumber, firstTag, ...);

BOOL SetJoyPortAttrs(Tag, ...);

FUNCTION

This function allows modification of several attributes held by

ReadJoyPort() about both it's operation and the type of controller

currently plugged into the port.

ReadJoyPort()'s default behavior is to attempt to automatically

sense the type of controller plugged into any given port, when

asked to read that port. This behavior is beneficial, to allow

simple detection of the type of controller plugged into the port.

Unfortunately, rare cases are possible where extremely fine

mouse movements appear to be real joystick movements. Also, this

ability to auto-sense the controller type causes most reads to

take longer than if there were no auto-sensing.

SetJoyPortAttrs() is intended to provide for both of these cases.

It allows the programmer to notify ReadJoyPort() to stop spending

time attempting to sense which type of controller is in use -- and,

optionally, to force ReadJoyPort() into utilizing a certain

controller type.

INPUTS

portNumber - the joyport in question (0-3).

tagList - a pointer to an array of tags providing parameters

to SetJoyPortAttrs(); if NULL, the function

will return TRUE, but do nothing.

TAGS

SJA_Type (ULONG) - Sets the current controller type to the mouse,

joystick, or game controller. Supply one of

SJA_TYPE_GAMECTLR, SJA_TYPE_MOUSE, SJA_TYPE_JOYSTK,

or SJA_TYPE_AUTOSENSE. If SJA_TYPE_AUTOSENSE is used,

ReadJoyPort() will attempt to determine the type of

controller plugged into the given port automatically.

If one of the other types is used, ReadJoyPort() will

make no attempt to read the given controller as

anything other than the type specified. The default

type is SJA_AUTOSENSE.

Note -- if you set the type to anything other than

auto-sense, it's your responsibility to return it

to auto-sense mode before exiting.

SJA_Reinitialize (VOID) - Return a given port to it's initial state,

forcing a port to deallocate any allocated resources;

return the implied type to SJA_TYPE_AUTOSENSE.

RESULT

success - TRUE if everything went according to plan, or FALSE upon

failure

SEE ALSO

ReadJoyPort(), <libraries/lowlevel.h>

lowlevel.library/StartTimerInt                 lowlevel.library/StartTimerInt

NAME

StartTimerInt -- start the timer associated with the timer interrupt.

(V40)

SYNOPSIS

StartTimerInt(intHandle, timeInterval, continuous);

A1         D0            D1

VOID StartTimerInt(APTR, ULONG, BOOL);

FUNCTION

This routine starts a stopped timer that is assocatied with a

timer interrupt created by AddTimerInt().

INPUTS

intHandle - handle obtained from AddTimerInt().

timeInterval - number of micoseconds between interrupts. The

maximum value allowed is 90,000. If higher values

are passed there will be unexpected results.

continuous - FALSE for a one shot interrupt. TRUE for multiple

interrupts.

SEE ALSO

AddTimerInt(), RemTimerInt(), StopTimerInt()

lowlevel.library/StopTimerInt                   lowlevel.library/StopTimerInt

NAME

StopTimerInt -- stop the timer associated with the timer interrupt.

(V40)

SYNOPSIS

StopTimerInt(intHandle);

A1

VOID StopTimerInt(APTR);

FUNCTION

Stops the timer associated with the timer interrupt handle passed.

This is used to stop a continuous timer started by

StartTimerInt().

INPUTS

intHandle - handle obtained from AddTimerInt().

SEE ALSO

AddTimerInt(), RemTimerInt(), StartTimerInt()

lowlevel.library/SystemControlA               lowlevel.library/SystemControlA

NAME

SystemControlA - Method for selectively disabling OS features. (V40)

SystemControl - varargs stub for SystemControlA().

SYNOPSIS

failTag = SystemControlA(tagList);

D0                       A1

ULONG SystemControlA(struct TagItem *);

failTag = SystemControl(firstTag, ...);

ULONG SystemControl(Tag, ...);

FUNCTION

This function is used to alter the operation of the system. Some of

the alterations involve controlling what are normally regarded as

system resources. In order to minimize confusion only one task is

allowed to control any part of the system resources. This prevents

the possiblity of two tasks fighting, each controlling a part of the

system. If a tag is identified as task exclusive, it means that

only one task can hold (set to TRUE) that tag. If

another task attempts to set the same tag to TRUE, the call to

SystemControl() will fail.

It is important to remember that SystemControl() can fail.

This is a low level function and certain tags do not fit the normal

Amiga multitasking model.

INPUTS

tagList - pointer to an array of tags listing the features of the

system to be enabled/disabled.

TAGS

SCON_TakeOverSys (BOOL)

TRUE -	Takes over the CPU to ensure that a program gets every

ounce of CPU time (with the exception of crucial

interrupts). When in this mode, the CPU will belong

completely to the program. Task switching will be

disabled and the program will get all CPU cycles. This

means any calls to the OS that involve multitasking in

some way will not execute correctly. Other tasks will

not run until this tag is used with FALSE. However,

during a Wait() on a signal, multitasking will

automatically be turned back on until the signal is

received. Once received, multitasking will again be

disabled and the CPU will be exclusive to the owning

program.

FALSE - Relinquishes the CPU and reenables multitasking.

This tag is task exculsive. This tag nests. A task may

take over the CPU several times before relinquishing

it.

SCON_KillReq (BOOL)

TRUE - 	Disables system requesters.  These are the reasons for

NOT disabling system requesters:

1- No calls in the program will cause a system

requester.

2- The only thing that could cause a requester

to appear is the lack of a CD in the drive and

SCON_CDReboot is set to CDReboot_On, therefore a

requester can't appear.

3- The only disk I/O is via a CD with SCON_CDReboot

set to CDReboot_On and/or nonvolatile.library.

When requesters should not be disabled.

GAME PROGRAMS:

No DOS calls are used after loading; or SCON_CDReboot

is CDReboot_On; and nonvolatile.library is used for

loading and saving user data.

This fits the above case since; After loading either

DOS calls are not used fitting reason 1, or the game

is accessing the CD and has SCON_CDReboot set to

CDReboot_On fitting reason 2. The game accesses high

scores, game position, etc through nonvolatile.library,

fitting reason 3.

FALSE -	Enables requesters for the program.

This tag nests. Tasks may disable requesters several times before

enabling them.  However, there must be a matching number of calls.

SCON_CDReboot (ULONG)

CDReboot_On - Ejecting the CD will cause a reboot of the

system. Use this only if the program cannot deal with

error conditions.

CDReboot_Off - Ejecting the CD will not cause a reboot of the

system. Use this if the program needs to insert CDs

while running.

CDReboot_Default - Restore the default reboot behavior for this

system. This should be used upon exit, if this tag had

been used to change the reboot behaviour. For the CD32

this value is synonomous with CDReboot_On. For Amiga

computers this value is synonomous with CDReboot_Off.

Note that the default reboot behavior differs depending on the

platform. If a program requires a specific behavior it must

use this function to set the behavior. For example, a CD audio

mixer would use this tag with the data CDReboot_Off. This will

allow the changing of audio CDs on the game machine as well as

Amiga computers.

If, however, there is no error detection code at all this tag

should be used with the data CDReboot_On.

It is hoped that no program will require CDReboot_On. If all

programs check for error condition and recover gracefully such a

call should never be necessary. With the default behavior the

CD32 will always reset on disk ejects, and programs

run from Amiga computers will not reset. Thus, leaving the

default will increase the market for a program to include both

types of platforms.

This tag does not nest.

SCON_StopInput (BOOL) - When TRUE, stops input.device from using any

CPU cycles. Also prevents input.device from passing along any

events from either the keyboard and/or port 0.

This tag is task exclusive. This tag is NOT reversible.

Attempting to reverse will result in confused/garbled input

events.

SCON_AddCreateKeys (ULONG) - Starts creating rawkey codes for the

joystick/game controller on the given unit. The unit value is

checked for validity and must be either 0 or 1. Each different unit

used results in some code added to the VBlank interrupt chain.

This tag nests. The tag SCON_RemCreateKeys is used to undo this

tag. Tasks may create rawkey codes several times before stopping

them.

Note that when operating in an Intuition window, the controller's

blue button is the equivilent of the mouse menu button. Therefore,

Intuition will be capturing most blue button events. If

notificiation of these events is important, review the

documentation for WFLG_RMBTRAP in the

intuition.library/OpenWindow() autodoc.

SCON_RemCreateKeys (ULONG) - stops rawkey codes for the joystick/game

controller on the given unit. The unit value is checked for

validity and must be either 0 or 1.

RESULT

failTag - zero if all tags succeeded. A non-zero return indicates a

tag that has failed. It is possible that other tags may

fail as well.

If any tag fails there will be no change in the system due

to other tags.

SEE ALSO

<libraries/lowlevel.h>