For your convenience, the autodoc for the new V40 WriteChunkyPixels

function is attached below.  All CD32 developers should have the full

V40 OS for their development machine, and V40 autodocs, includes,

libs, and fd's.  Archives of the V40 Workbench disks, V40 kickfiles,

and V40 development files may be downloaded from our closed

developer conferences on BIX (US), ADSP (Europe), and CIX (UK).

graphics.library/WriteChunkyPixels         graphics.library/WriteChunkyPixels

NAME

WriteChunkyPixels -- write the pen number value of a rectangular array

of pixels starting at a specified x,y location and continuing

through to another x,y location within a certain RastPort. (V40)

SYNOPSIS

WriteChunkyPixels(rp,xstart,ystart,xstop,ystop,array,bytesperrow)

A0 D0     D1     D2    D3    A2     D4

VOID WriteChunkyPixels(struct  RastPort *, LONG, LONG,

LONG, LONG, UBYTE *, LONG);

FUNCTION

For each pixel in a rectangular region, decode the pen number selector

from a linear array of pen numbers into the bit-planes used to describe

a particular rastport.

INPUTS

rp     -  pointer to a RastPort structure

(xstart,ystart) -  starting point in the RastPort

(xstop,ystop)   -  stopping point in the RastPort

array  - pointer to an array of UBYTEs from which to fetch the

pixel data.

bytesperrow - The number of bytes per row in the source array.

This should be at least as large as the number of pixels

being written per line.

RESULT

NOTE

xstop must be >= xstart

ystop must be >= ystart

The source array can be in fast RAM.

===chunky-to-planar conversion HW:

GfxBase->ChunkyToPlanarPtr is either NULL, or a pointer to a HW

register used to aid in the process of converting 8-bit chunky

pixel data into the bit-plane format used by the Amiga custom

display chips. If NULL, then such hardware is not present.

If an expansion device provides hardware which operates compatibly,

than it can install the HW address into this pointer at boot time,

and the system will use it.

This pointer may be used for direct access to the chunky-to-planar

conversion HW, if more is desired than the straight chunky-pixel

copy that is performed by WriteChunkyPixels().

If using the hardware directly, it should only be accessed when

the task using it has control of the blitter (via OwnBlitter()),

since this is the locking used to arbitrate usage of this device.

The hardware may be viewed as a device which accepts 32 8-bit

chunky pixels and outputs 8 longwords of bitplane data.

For proper operation, exactly 8 longwords (containing 32 pixels)

of chunky data should be written to *(GfxBase->ChunkyToPlanarPtr).

After the data is written, bitplane data (starting with plane 0)

can be read back a longword at a time. There is no need to read

back all 8 longwords if the high-order bitplanes are not needed.

Since WriteChunkyPixels is not (currently) particularly fast on

systems without the chunky-to-planar hardware, time critical

applications (games, etc) may want to use their own custom conversion

routine if GfxBase->ChunkyToPlanarPtr is NULL, and call

WriteChunkyPixels() otherwise.

This pointer is only present in GfxBase in versions of graphics.library

>= 40, so this should be checked before the pointer is read.

BUGS

Not very fast on systems without chunky-to-planar conversion

hardware.

SEE ALSO

WritePixel()  graphics/rastport.h