Loadtiga needs some parameters to run properly. They can either be passed as tool types

or given on the command line when 'loadtiga' is executed from the shell.

From Workbench:

BOARDx_MODE=<mode description>

BOARDx_OSC1=<oscillator description>

BOARDx_OSC2=<oscillator description>

For x, give the A2410 board number (0 .. 3)

MODE description:

================

Valid MODEs are:

MODE_1024x768_NI

MODE_800x600_NI

MODE_640x400_I

MODE_1024x768_I

MODE_1024x1024_NI

Oscillator description:

======================

Valid OSC1 and OSC2 descriptions are:

14.318MHZ

36.000MHZ

44.98MHZ

66.667MHZ

80.00MHZ

OSC1 and OSC2 are the two oscillators in the left/back corner of the TIGA board.

There is a '1' and '2' written on the board, so that it should be easy to identify

the different oscillators. On my board Oscillator 2 is the one nearest the backplane.

Each oscillator corresponds to a certain resolution.

14.318MHZ		MODE_640x400_I

36.000MHZ		MODE_800x600_NI

44.98MHZ		MODE_1024x768_I

66.667MHZ		MODE_1024x768_NI

80.00MHZ		MODE_1024x1024_NI

During your setup you should make sure that the MODE you select can be produced by

one of your oscillators.

Example:

=======

Corresponding to the shipment configuration of the A2410 board your tooltypes

would, for A2410 board number 0 (zero), look like :

BOARD0_MODE=MODE_800x600_NI

BOARD0_OSC1=36.000MHz

BOARD0_OSC2=66.667MHz