|=================================================|

|                                                 |

|      Drawing Interchange and File Formats       |

|                  Release 12                     |

|   Copyright (c) 1982-1990, 1992 Autodesk, Inc.  |

|              All Rights Reserved                |

|                                                 |

|=================================================|

AutoCAD can be used by itself as a complete drawing editor. In some

applications, however, other programs must examine drawings created

by AutoCAD or generate drawings to be viewed, modified, or plotted

with AutoCAD.

For example, if you've made an architectural drawing with AutoCAD,

using inserted parts to represent windows, doors, and so on, you can

process the drawing file and produce a bill of materials of all

items used in the drawing, or even make energy-use calculations

based on the area and the number and type of windows used. Another

possible application is to use AutoCAD to describe structures and

then send the descriptions to a more powerful computer for finite-

element structural analysis. You can compute stresses and

displacements and send back information to display the deformed

structure as an AutoCAD drawing.

Since the AutoCAD drawing database (.dwg file) is written in a

compact format that changes significantly as new features are added

to AutoCAD, we do not document its format and do not recommend that

you attempt to write programs to read it directly. To assist in

interchanging drawings between AutoCAD and other programs, a Drawing

Interchange file format (DXF) has been defined. All implementations

of AutoCAD accept this format and are able to convert it to and from

their internal drawing file representation.

AutoCAD also supports the Initial Graphics Exchange Specification

(IGES) file format. The information comprising an AutoCAD drawing

can be written out in IGES format, and IGES files can be read and

converted to the AutoCAD internal format.

ASCII Drawing Interchange (DXF) Files

*************************************

This section describes the AutoCAD DXF (drawing interchange file)

format and the commands provided to read and write these files. DXF

files are standard ASCII text files. They can easily be translated

to the formats of other CAD systems or submitted to other programs

for specialized analysis. AutoCAD can also produce or read a binary

form of the full DXF file. This feature is described in detail later

in this chapter.

DXFOUT Command - Writing a DXF File

===================================

You can generate a drawing interchange file from an existing drawing

by means of the DXFOUT command:

Command: dxfout

When AutoCAD prompts you, respond with a filename or press 5 to

accept the default.

The default name for the output file is the same as that of the

current drawing, but with a file type of .dxf. If you specify an

explicit filename, you do not need to include a file type; .dxf is

assumed. If a file with the same name already exists, the existing

file is deleted. If you specify the file using a file dialogue box,

and a file with the same name already exists, AutoCAD tells you;

allowing you to OK or cancel the deletion. Next, DXFOUT asks what

precision you want for floating-point numbers and permits output of

a partial DXF file containing only selected objects.

Enter decimal places of accuracy (0 to 16)/Entities/Binary <6>:

The Binary option is described later in this chapter.

If you respond with "entities" (or just "e"), DXFOUT asks you to

select the objects you want written to the DXF file. Only the

objects you select are included in the output file - symbol tables

(including Block Definitions) will not be included. Once you've

selected the desired objects, AutoCAD again prompts you for the

numeric precision:

Enter decimal places of accuracy (0 to 16)/Binary <6>:

DXFIN Command - Loading a DXF File

==================================

A drawing interchange file can be converted into an AutoCAD drawing

by means of the DXFIN command:

Command: dxfin

When AutoCAD prompts you, respond with the name of the drawing

interchange file to be loaded.

Full DXFIN

----------

To load a complete DXF file, you must use DXFIN in an empty drawing,

before any entities have been drawn and before any additional Block

definitions, layers, linetypes, text styles, dimension styles, named

views, named coordinate systems, or named viewport configurations

have been created.

NOTE: If the drawing you are using as a prototype is not empty, you

might find it helpful to open a new drawing using the No

Prototype... button of the Create New Drawing dialogue box, as

described in chapter 4 of the "AutoCAD Reference Manual." You should

also be aware that some third-party applications include an acad.lsp

or .mnl file that modifies your drawing upon startup.

If any errors are detected during the input, the new drawing is

discarded. Otherwise, an automatic ZOOM All is performed to set the

drawing extents.

Partial DXFIN

-------------

If the current drawing is not empty, DXFIN loads only the ENTITIES

section of the DXF file, adding the entities found there to the

current drawing. In this case, DXFIN displays the message:

Not a new drawing -- only ENTITIES section will be input.

If errors are detected during such partial DXF input, the drawing is

returned to the state it was in before the DXFIN command. Otherwise,

the newly added entities are drawn.

Auditing DXF Files

------------------

To ensure that corrupt data is not imported into your drawing, you

can instruct AutoCAD to perform an audit after importing DXF files

into your drawing with DXFIN. When you use DXFIN, the default action

is to perform no automatic auditing. To activate automatic auditing,

use the CONFIG command:

Command: config

Your current AutoCAD configuration appears. Press 5 to continue.

>From the Configuration menu select this option:

7. Configure operating parameters

>From the Operating parameter menu select this option:

9. Automatic Audit after IGESIN, DXFIN, or DXBIN

Answer Y to this question:

Do you want an automatic audit after IGESIN, DXFIN, or DXBIN?

<N>: y

Return to the graphics screen by pressing 5 three times.

NOTE: This kind of audit only displays the errors AutoCAD finds; it

does not correct them. To correct problems, use the AUDIT command on

the drawing while you are in AutoCAD, or manually edit the DXF file.

DXF File Format

===============

This section describes the format of a DXF file in detail. It

contains technical information that you need only if you write your

own programs to process DXF files or work with entity information

obtained by certain AutoLISP and ADS functions.

It would probably be helpful to produce a DXF file from a small

drawing, print it out, and refer to it occasionally while reading

the information presented next.

General File Structure

----------------------

A Drawing Interchange File is simply an ASCII text file with a file

type of .dxf and specially formatted text. The overall organization

of a DXF file is as follows:

1. HEADER section - General information about the drawing is found

in this section of the DXF file. Each parameter has a variable

name and an associated value (see table 11-3 for a list of the

header variables).

2. TABLES section - This section contains definitions of named

items.

o Linetype table (LTYPE)

o Layer table (LAYER)

o Text Style table (STYLE)

o View table (VIEW)

o User Coordinate System table (UCS)

o Viewport configuration table (VPORT)

o Dimension Style table (DIMSTYLE)

o Application Identification table (APPID)

3. BLOCKS section - This section contains Block Definition entities

describing the entities that make up each Block in the drawing.

4. ENTITIES section - This section contains the drawing entities,

including any Block References.

5. END OF FILE

If you use DXFOUT's Entities option, the resulting DXF file contains

only the ENTITIES section and the END OF FILE marker, and the

ENTITIES section reflects only the objects you select for output.

NOTE: If you select an INSERT entity, the corresponding Block

definition is not included in the output file.

A DXF file is composed of many groups, each of which occupies two

lines in the DXF file. The first line of a group is a group code,

which is a positive nonzero integer output in FORTRAN I3 - that is,

right-justified and blank filled in a three-character field (the

exception to this is the four-digit extended entity data group

codes, which are output in FORTRAN I4). The second line of the group

is the group value, in a format that depends on the type of group

specified by the group code. Although DXFOUT output has a fixed

format, the DXFIN format is free.

The specific assignment of group codes depends on the item being

described in the file. However, the type of the value this group

supplies is derived from the group code in the following way:

Table 11-1. Group code ranges

+================================+

| Group code  | Following value  |

| range       |                  |

|-------------|------------------|

| 0 - 9       | String           |

|-------------|------------------|

| 10 - 59     | Floating-point   |

|-------------|------------------|

| 60 - 79     | Integer          |

|-------------|------------------|

| 140 - 147   | Floating-point   |

|-------------|------------------|

| 170 - 175   | Integer          |

|-------------|------------------|

| 210 - 239   | Floating-point   |

|-------------|------------------|

| 999         | Comment (string) |

|-------------|------------------|

| 1010 - 1059 | Floating-point   |

|-------------|------------------|

| 1060 - 1079 | Integer          |

|-------------|------------------|

| 1000 - 1009 | String           |

+--------------------------------+

Thus a program can easily read the value following a group code

without knowing the particular use of this group in an item in the

file. The appearance of values in the DXF file is not affected by

the setting of the UNITS command: coordinates are always represented

as decimal (or possibly scientific notation if very large) numbers,

and angles are always represented in decimal degrees with zero

degrees to the east of origin.

Variables, table entries, and entities are described by a group that

introduces the item, giving its type and/or name, followed by

multiple groups that supply the values associated with the item. In

addition, special groups are used for file separators such as

markers for the beginning and end of sections, tables, and the file

itself.

Entities, table entries, and file separators are always introduced

with a 0 group code that is followed by a name describing the item.

NOTE: The maximum DXF file string length is 256 characters. If your

AutoCAD drawing contains strings that exceed this number, those

strings are truncated during DXFOUT. If your DXF file contains

strings that exceed this number, DXFIN will fail.

Group Codes

-----------

Group codes are used both to indicate the type of the value of the

group, as explained earlier, and to indicate the general use of the

group. The specific function of the group code depends on the actual

variable, table item, or entity description. This section indicates

the general use of groups, noting as "(fixed)" any that always have

the same function.

Table 11-2. AutoCAD entity group codes (by number)

+===============================================================+

| Group code | Value type                                       |

|------------|--------------------------------------------------|

| 0          | Identifies the start of an entity, table entry,  |

|            | or file separator. The type of entity is given   |

|            | by the text value that follows this group        |

|------------|--------------------------------------------------|

| 1          | The primary text value for an entity             |

|------------|--------------------------------------------------|

| 2          | A name: Attribute tag, Block name, and so on.    |

|            | Also used to identify a DXF section or table name|

|------------|--------------------------------------------------|

| 3-4        | Other textual or name values                     |

|------------|--------------------------------------------------|

| 5          | Entity handle expressed as a hexadecimal string  |

|            | (fixed)                                          |

|------------|--------------------------------------------------|

| 6          | Line type name (fixed)                           |

|------------|--------------------------------------------------|

| 7          | Text style name (fixed)                          |

|------------|--------------------------------------------------|

| 8          | Layer name (fixed)                               |

|------------|--------------------------------------------------|

| 9          | Variable name identifier (used only in HEADER    |

|            | section of the DXF file)                         |

|------------|--------------------------------------------------|

| 10         | Primary X coordinate (start point of a Line or   |

|            | Text entity, center of a Circle, etc.)           |

|------------|--------------------------------------------------|

| 11-18      | Other X coordinates                              |

|------------|--------------------------------------------------|

| 20         | Primary Y coordinate. 2n values always           |

|            | correspond to 1n values and immediately follow   |

|            | them in the file                                 |

|------------|--------------------------------------------------|

| 21-28      | Other Y coordinates                              |

|------------|--------------------------------------------------|

| 30         | Primary Z coordinate. 3n values always           |

|            | correspond to 1n and 2n values and immediately   |

|            | follow them in the file                          |

|------------|--------------------------------------------------|

| 31-37      | Other Z coordinates                              |

|------------|--------------------------------------------------|

| 38         | This entity's elevation if nonzero (fixed).      |

|            | Exists only in output from versions prior to R11 |

|------------|--------------------------------------------------|

| 39         | This entity's thickness if nonzero (fixed)       |

|------------|--------------------------------------------------|

| 40-48      | Floating-point values (text height, scale        |

|            | factors, etc.)                                   |

|------------|--------------------------------------------------|

| 49         | Repeated value - multiple 49 groups may appear   |

|            | in one entity for variable length tables (such   |

|            | as the dash lengths in the LTYPE table). A 7x    |

|            | group always appears before the first 49 group   |

|            | to specify the table length                      |

|------------|--------------------------------------------------|

| 50-58      | Angles                                           |

|------------|--------------------------------------------------|

| 62         | Color number (fixed)                             |

|------------|--------------------------------------------------|

| 66         | "Entities follow" flag (fixed)                   |

|------------|--------------------------------------------------|

| 67         | Identifies whether entity is in model space or   |

|            | paper space                                      |

|------------|--------------------------------------------------|

| 68         | Identifies whether viewport is on but fully off  |

|            | screen, is not active, or is off                 |

|------------|--------------------------------------------------|

| 69         | Viewport identification number                   |

|------------|--------------------------------------------------|

| 70-78      | Integer values such as repeat counts, flag       |

|            | bits, or modes                                   |

|------------|--------------------------------------------------|

| 210,       | X, Y, and Z components of extrusion direction    |

| 220,       | (fixed)                                          |

| 230        |                                                  |

|------------|--------------------------------------------------|

| 999        | Comments                                         |

|------------|--------------------------------------------------|

| 1000       | An ASCII string (up to 255 bytes long) in        |

|            | extended entity data                             |

|------------|--------------------------------------------------|

| 1001       | Registered application name (ASCII string up to  |

|            | 31 bytes long) for XDATA (fixed)                 |

|------------|--------------------------------------------------|

| 1002       | Extended entity data control string ("{" or "}") |

|            | (fixed)                                          |

|------------|--------------------------------------------------|

| 1003       | Extended entity data Layer name                  |

|------------|--------------------------------------------------|

| 1004       | Chunk of bytes (up to 127 bytes long) in         |

|            | extended entity data                             |

|------------|--------------------------------------------------|

| 1005       | Extended entity data database handle             |

|------------|--------------------------------------------------|

| 1010,      | Extended entity data X, Y, and Z coordinates     |

| 1020,      |                                                  |

| 1030       |                                                  |

|------------|--------------------------------------------------|

| 1011,      | Extended entity data X, Y, and Z coordinates of  |

| 1021,      | 3D world space position                          |

| 1031       |                                                  |

|------------|--------------------------------------------------|

| 1012,      | Extended entity data X, Y, and Z components of   |

| 1022,      | 3D world space displacement                      |

| 1032       |                                                  |

|------------|--------------------------------------------------|

| 1013,      | Extended entity data X, Y, and Z components of   |

| 1023,      | 3D world space direction                         |

| 1033       |                                                  |

|------------|--------------------------------------------------|

| 1040       | Extended entity data Floating-point value        |

|------------|--------------------------------------------------|

| 1041       | Extended entity data distance value              |

|------------|--------------------------------------------------|

| 1042       | Extended entity data scale factor                |

|------------|--------------------------------------------------|

| 1070       | Extended entity data 16-bit signed integer       |

|------------|--------------------------------------------------|

| 1071       | Extended entity data 32-bit signed long          |

+---------------------------------------------------------------+

Comments

========

The 999 group code indicates that the following line is a comment

string. DXFOUT does not currently include such groups in a DXF

output file, but DXFIN honors them and ignores the comments. Thus,

you can use the 999 group to include comments in a DXF file you've

edited. For example:

999

This is a comment.

999

This is another comment.

File Sections

=============

The DXF file is subdivided into four editable sections, plus the END

OF FILE marker. File separator groups are used to delimit these file

sections. The following is an example of a void DXF file with only

the section markers and table headers present:

0            (Begin HEADER section)

SECTION

2

HEADER

<<<<Header variable items go here>>>>

0

ENDSEC       (End HEADER section)

0           (Begin TABLES section)

SECTION

2

TABLES

0

TABLE

2

VPORT

70

(viewport table maximum item count)

<<<<viewport table items go here>>>>

0

ENDTAB

0

TABLE

2

APPID, DIMSTYLE, LTYPE, LAYER, STYLE, UCS, VIEW, or VPORT

70

(Table maximum item count)

<<<<Table items go here>>>>

0

ENDTAB

0

ENDSEC       (End TABLES section)

0            (Begin BLOCKS section)

SECTION

2

BLOCKS

<<<<Block definition entities go here>>>>

0

ENDSEC       (End BLOCKS section)

0            (Begin ENTITIES section)

SECTION

2

ENTITIES

<<<<Drawing entities go here>>>>

0

ENDSEC       (End ENTITIES section)

0

EOF          (End of file)

HEADER Section

--------------

The HEADER section of the DXF file contains settings of variables

associated with the drawing. These variables are set with various

commands and are the type of information displayed by the STATUS

command. Each variable is specified in the header section by a 9

group giving the variable's name, followed by groups that supply the

variable's value. The following list shows the header variables and

their meanings.

Although this list is very similar to the list of system variables

in Appendix A of this manual, the two lists are not identical. Be

sure you're referring to the proper list.

NOTE: $AXISMODE and $AXISUNIT are no longer functional in Release

12.

Table 11-3. DXF system variables

+===============================================================+

| Variable       | Type   | Description                         |

|----------------|--------|-------------------------------------|

| $ACADVER       | 1      | The AutoCAD drawing database        |

|                |        | version number; AC1006 = R10,       |

|                |        | AC1009 = R11 and R12                |

|----------------|--------|-------------------------------------|

| $ANGBASE       | 50     | Angle 0 direction                   |

|----------------|--------|-------------------------------------|

| $ANGDIR        | 70     | 1 = clockwise angles, 0 =           |

|                |        | counterclockwise                    |

|----------------|--------|-------------------------------------|

| $ATTDIA        | 70     | Attribute entry dialogs, 1 = on,    |

|                |        | 0 = off                             |

|----------------|--------|-------------------------------------|

| $ATTMODE       | 70     | Attribute visibility: 0 = none,     |

|                |        | 1 = normal, 2 = all                 |

|----------------|--------|-------------------------------------|

| $ATTREQ        | 70     | Attribute prompting during INSERT,  |

|                |        | 1 = on, 0 = off                     |

|----------------|--------|-------------------------------------|

| $AUNITS        | 70     | Units format for angles             |

|----------------|--------|-------------------------------------|

| $AUPREC        | 70     | Units precision for angles          |

|----------------|--------|-------------------------------------|

| $AXISMODE      | 70     | Axis on if nonzero (not functional  |

|                |        | in Release 12)                      |

|----------------|--------|-------------------------------------|

| $AXISUNIT      | 10, 20 | Axis X and Y tick spacing           |

|                |        | (not functional in Release 12)      |

|----------------|--------|-------------------------------------|

| $BLIPMODE      | 70     | Blip mode on if nonzero             |

|----------------|--------|-------------------------------------|

| $CECOLOR       | 62     | Entity color number; 0 = BYBLOCK,   |

|                |        | 256 = BYLAYER                       |

|----------------|--------|-------------------------------------|

| $CELTYPE       | 6      | Entity linetype name, or BYBLOCK    |

|                |        | or BYLAYER                          |

|----------------|--------|-------------------------------------|

| $CHAMFERA      | 40     | First chamfer distance              |

|----------------|--------|-------------------------------------|

| $CHAMFERB      | 40     | Second chamfer distance             |

|----------------|--------|-------------------------------------|

| $CLAYER        | 8      | Current layer name                  |

|----------------|--------|-------------------------------------|

| $COORDS        | 70     | 0 = static coordinate display,      |

|                |        | 1 = continuous update, 2 = "d<a"    |

|                |        | format                              |

|----------------|--------|-------------------------------------|

| $DIMALT        | 70     | Alternate unit dimensioning         |

|                |        | performed if nonzero                |

|----------------|--------|-------------------------------------|

| $DIMALTD       | 70     | Alternate unit decimal places       |

|----------------|--------|-------------------------------------|

| $DIMALTF       | 40     | Alternate unit scale factor         |

|----------------|--------|-------------------------------------|

| $DIMAPOST      | 1      | Alternate dimensioning suffix       |

|----------------|--------|-------------------------------------|

| $DIMASO        | 70     | 1 = create associative dimensioning,|

|                |        | 0 = draw individual entities        |

|----------------|--------|-------------------------------------|

| $DIMASZ        | 40     | Dimensioning arrow size             |

|----------------|--------|-------------------------------------|

| $DIMBLK        | 2      | Arrow block name                    |

|----------------|--------|-------------------------------------|

| $DIMBLK1       | 1      | First arrow block name              |

|----------------|--------|-------------------------------------|

| $DIMBLK2       | 1      | Second arrow block name             |

|----------------|--------|-------------------------------------|

| $DIMCEN        | 40     | Size of center mark/lines           |

|----------------|--------|-------------------------------------|

| $DIMCLRD       | 70     | Dimension line color, range is      |

|                |        | 0 = BYBLOCK, 256 = BYLAYER          |

|----------------|--------|-------------------------------------|

| $DIMCLRE       | 70     | Dimension extension line color,     |

|                |        | range is 0 = BYBLOCK, 256 = BYLAYER |

|----------------|--------|-------------------------------------|

| $DIMCLRT       | 70     | Dimension text color, range is      |

|                |        | 0 = BYBLOCK, 256 = BYLAYER          |

|----------------|--------|-------------------------------------|

| $DIMDLE        | 40     | Dimension line extension            |

|----------------|--------|-------------------------------------|

| $DIMDLI        | 40     | Dimension line increment            |

|----------------|--------|-------------------------------------|

| $DIMEXE        | 40     | Extension line extension            |

|----------------|--------|-------------------------------------|

| $DIMEXO        | 40     | Extension line offset               |

|----------------|--------|-------------------------------------|

| $DIMGAP        | 40     | Dimension line gap                  |

|----------------|--------|-------------------------------------|

| $DIMLFAC       | 40     | Linear measurements scale factor    |

|----------------|--------|-------------------------------------|

| $DIMLIM        | 70     | Dimension limits generated if       |

|                |        | nonzero                             |

|----------------|--------|-------------------------------------|

| $DIMPOST       | 1      | General dimensioning suffix         |

|----------------|--------|-------------------------------------|

| $DIMRND        | 40     | Rounding value for dimension        |

|                |        | distances                           |

|----------------|--------|-------------------------------------|

| $DIMSAH        | 70     | Use separate arrow blocks if nonzero|

|----------------|--------|-------------------------------------|

| $DIMSCALE      | 40     | Overall dimensioning scale factor   |

|----------------|--------|-------------------------------------|

| $DIMSE1        | 70     | First extension line suppressed     |

|                |        | if nonzero                          |

|----------------|--------|-------------------------------------|

| $DIMSE2        | 70     | Second extension line suppressed    |

|                |        | if nonzero                          |

|----------------|--------|-------------------------------------|

| $DIMSHO        | 70     | 1 = Recompute dimensions while      |

|                |        | dragging, 0 = drag original image   |

|----------------|--------|-------------------------------------|

| $DIMSOXD       | 70     | Suppress outside-extensions         |

|                |        | dimension lines if nonzero          |

|----------------|--------|-------------------------------------|

| $DIMSTYLE      | 2      | Dimension style name                |

|----------------|--------|-------------------------------------|

| $DIMTAD        | 70     | Text above dimension line if nonzero|

|----------------|--------|-------------------------------------|

| $DIMTFAC       | 40     | Dimension tolerance display scale   |

|                |        | factor                              |

|----------------|--------|-------------------------------------|

| $DIMTIH        | 70     | Text inside horizontal if nonzero   |

|----------------|--------|-------------------------------------|

| $DIMTIX        | 70     | Force text inside extensions if     |

|                |        | nonzero                             |

|----------------|--------|-------------------------------------|

| $DIMTM         | 40     | Minus tolerance                     |

|----------------|--------|-------------------------------------|

| $DIMTOFL       | 70     | If text outside extensions, force   |

|                |        | line extensions between extensions  |

|                |        | if nonzero                          |

|----------------|--------|-------------------------------------|

| $DIMTOH        | 70     | Text outside horizontal if nonzero  |

|----------------|--------|-------------------------------------|

| $DIMTOL        | 70     | Dimension tolerances generated if   |

|                |        | nonzero                             |

|----------------|--------|-------------------------------------|

| $DIMTP         | 40     | Plus tolerance                      |

|----------------|--------|-------------------------------------|

| $DIMTSZ        | 40     | Dimensioning tick size: 0 = no ticks|

|----------------|--------|-------------------------------------|

| $DIMTVP        | 40     | Text vertical position              |

|----------------|--------|-------------------------------------|

| $DIMTXT        | 40     | Dimensioning text height            |

|----------------|--------|-------------------------------------|

| $DIMZIN        | 70     | Zero suppression for "feet & inch"  |

|                |        | dimensions                          |

|----------------|--------|-------------------------------------|

| $DWGCODEPAGE   | 70     | Drawing code page. Set to the       |

|                |        | system code page when a new drawing |

|                |        | is created, but not otherwise       |

|                |        | maintained by AutoCAD               |

|----------------|--------|-------------------------------------|

| $DRAGMODE      | 70     | 0 = off, 1 = on, 2 = auto           |

|----------------|--------|-------------------------------------|

| $ELEVATION     | 40     | Current elevation set by ELEV       |

|                |        | command                             |

|----------------|--------|-------------------------------------|

| $EXTMAX        | 10, 20,| X, Y, and Z drawing extents         |

|                | 30     | upper-right corner (in WCS)         |

|----------------|--------|-------------------------------------|

| $EXTMIN        | 10, 20,| X, Y, and Z drawing extents         |

|                | 30     | lower-left corner (in WCS)          |

|----------------|--------|-------------------------------------|

| $FILLETRAD     | 40     | Fillet radius                       |

|----------------|--------|-------------------------------------|

| $FILLMODE      | 70     | Fill mode on if nonzero             |

|----------------|--------|-------------------------------------|

| $HANDLING      | 70     | Handles enabled if nonzero          |

|----------------|--------|-------------------------------------|

| $HANDSEED      | 5      | Next available handle               |

|----------------|--------|-------------------------------------|

| $INSBASE       | 10, 20,| Insertion base set by BASE command  |

|                | 30     | (in WCS)                            |

|----------------|--------|-------------------------------------|

| $LIMCHECK      | 70     | Nonzero if limits checking is on    |

|----------------|--------|-------------------------------------|

| $LIMMAX        | 10, 20 | XY drawing limits upper-right       |

|                |        | corner (in WCS)                     |

|----------------|--------|-------------------------------------|

| $LIMMIN        | 10, 20 | XY drawing limits lower-left        |

|                |        | corner (in WCS)                     |

|----------------|--------|-------------------------------------|

| $LTSCALE       | 40     | Global linetype scale               |

|----------------|--------|-------------------------------------|

| $LUNITS        | 70     | Units format for coordinates and    |

|                |        | distances                           |

|----------------|--------|-------------------------------------|

| $LUPREC        | 70     | Units precision for coordinates     |

|                |        | and distances                       |

|----------------|--------|-------------------------------------|

| $MAXACTVP      | 70     | Sets maximum number of viewports to |

|                |        | be regenerated                      |

|----------------|--------|-------------------------------------|

| $MENU          | 1      | Name of menu file                   |

|----------------|--------|-------------------------------------|

| $MIRRTEXT      | 70     | Mirror text if nonzero              |

|----------------|--------|-------------------------------------|

| $ORTHOMODE     | 70     | Ortho mode on if nonzero            |

|----------------|--------|-------------------------------------|

| $OSMODE        | 70     | Running object snap modes           |

|----------------|--------|-------------------------------------|

| $PDMODE        | 70     | Point display mode                  |

|----------------|--------|-------------------------------------|

| $PDSIZE        | 40     | Point display size                  |

|----------------|--------|-------------------------------------|

| $PELEVATION    | 40     | Current paper space elevation       |

|----------------|--------|-------------------------------------|

| $PEXTMAX       | 10, 20,| Maximum X, Y, and Z extents for     |

|                | 30     | paper space                         |

|----------------|--------|-------------------------------------|

| $PEXTMIN       | 10, 20,| Minimum X, Y, and Z extents for     |

|                | 30     | paper space                         |

|----------------|--------|-------------------------------------|

| $PLIMCHECK     | 70     | Limits checking in paper space      |

|                |        | when nonzero                        |

|----------------|--------|-------------------------------------|

| $PLIMMAX       | 10, 20 | Maximum X and Y limits in paper     |

|                |        | space                               |

|----------------|--------|-------------------------------------|

| $PLIMMIN       | 10, 20 | Minimum X and Y limits in paper     |

|                |        | space                               |

|----------------|--------|-------------------------------------|

| $PLINEGEN      | 70     | Governs the generation of linetype  |

|                |        | patterns around the vertices        |

|                |        | of a 2D Polyline                    |

|                |        | 1 = linetype is generated in        |

|                |        | a continuous pattern around         |

|                |        | vertices of the Polyline            |

|                |        | 0 = each segment of the Polyline    |

|                |        | starts and ends with a dash         |

|----------------|--------|-------------------------------------|

| $PLINEWID      | 40     | Default Polyline width              |

|----------------|--------|-------------------------------------|

| $PSLTSCALE     | 70     | Controls paper space linetype       |

|                |        | scaling                             |

|                |        | 1 = no special linetype scaling     |

|                |        | 0 = viewport scaling governs        |

|                |        | linetype scaling                    |

|----------------|--------|-------------------------------------|

| $PUCSNAME      | 2      | Current paper space UCS name        |

|----------------|--------|-------------------------------------|

| $PUCSORG       | 10, 20,| Current paper space UCS origin      |

|                | 30     |                                     |

|----------------|--------|-------------------------------------|

| $PUCSXDIR      | 10, 20,| Current paper space UCS X axis      |

|                | 30     |                                     |

|----------------|--------|-------------------------------------|

| $PUCSYDIR      | 10, 20,| Current paper space UCS Y axis      |

|                | 30     |                                     |

|----------------|--------|-------------------------------------|

| $QTEXTMODE     | 70     | Quick text mode on if nonzero       |

|----------------|--------|-------------------------------------|

| $REGENMODE     | 70     | REGENAUTO mode on if nonzero        |

|----------------|--------|-------------------------------------|

| $SHADEDGE      | 70     | 0 = faces shaded, edges not         |

|                |        | highlighted                         |

|                |        | 1 = faces shaded, edges highlighted |

|                |        | in black                            |

|                |        | 2 = faces not filled, edges in      |

|                |        | entity color                        |

|                |        | 3 = faces in entity color,          |

|                |        | edges in black                      |

|----------------|--------|-------------------------------------|

| $SHADEDIF      | 70     | Percent ambient/diffuse light,      |

|                |        | range 1-100, default 70             |

|----------------|--------|-------------------------------------|

| $SKETCHINC     | 40     | Sketch record increment             |

| $SKPOLY        | 70     | 0 = sketch lines, 1 = sketch        |

|                |        | polylines                           |

|----------------|--------|-------------------------------------|

| $SPLFRAME      | 70     | Spline control polygon display,     |

|                |        | 1 = on, 0 = off                     |

|----------------|--------|-------------------------------------|

| $SPLINESEGS    | 70     | Number of line segments per spline  |

|                |        | patch                               |

|----------------|--------|-------------------------------------|

| $SPLINETYPE    | 70     | Spline curve type for PEDIT Spline  |

|                |        | (See your AutoCAD Reference Manual) |

|----------------|--------|-------------------------------------|

| $SURFTAB1      | 70     | Number of mesh tabulations in first |

|                |        | direction                           |

|----------------|--------|-------------------------------------|

| $SURFTAB2      | 70     | Number of mesh tabulations in       |

|                |        | second direction                    |

|----------------|--------|-------------------------------------|

| $SURFTYPE      | 70     | Surface type for PEDIT Smooth       |

|                |        | (See your AutoCAD Reference Manual) |

|----------------|--------|-------------------------------------|

| $SURFU         | 70     | Surface density (for PEDIT Smooth)  |

|                |        | in M direction                      |

|----------------|--------|-------------------------------------|

| $SURFV         | 70     | Surface density (for PEDIT Smooth)  |

|                |        | in N direction                      |

|----------------|--------|-------------------------------------|

| $TDCREATE      | 40     | Date/time of drawing creation       |

|----------------|--------|-------------------------------------|

| $TDINDWG       | 40     | Cumulative editing time for         |

|                |        | this drawing                        |

|----------------|--------|-------------------------------------|

| $TDUPDATE      | 40     | Date/time of last drawing update    |

|----------------|--------|-------------------------------------|

| $TDUSRTIMER    | 40     | User elapsed timer                  |

|----------------|--------|-------------------------------------|

| $TEXTSIZE      | 40     | Default text height                 |

|----------------|--------|-------------------------------------|

| $TEXTSTYLE     | 7      | Current text style name             |

|----------------|--------|-------------------------------------|

| $THICKNESS     | 40     | Current thickness set by ELEV       |

|                |        | command                             |

|----------------|--------|-------------------------------------|

| $TILEMODE      | 70     | 1 for previous release              |

|                |        | compatibility mode, 0 otherwise     |

|----------------|--------|-------------------------------------|

| $TRACEWID      | 40     | Default Trace width                 |

|----------------|--------|-------------------------------------|

| $UCSNAME       | 2      | Name of current UCS                 |

|----------------|--------|-------------------------------------|

| $UCSORG        | 10, 20,| Origin of current UCS (in WCS)      |

|                | 30     |                                     |

|----------------|--------|-------------------------------------|

| $UCSXDIR       | 10, 20,| Direction of current UCS's X axis   |

|                | 30     | (in World coordinates)              |

|----------------|--------|-------------------------------------|

| $UCSYDIR       | 10, 20,| Direction of current UCS's Y axis   |

|                | 30     | (in World coordinates)              |

|----------------|--------|-------------------------------------|

| $UNITMODE      | 70     | Low bit set = display fractions,    |

|                |        | feet-and-inches, and surveyor's     |

|                |        | angles in input format              |

|----------------|--------|-------------------------------------|

| $USERI1 - 5    | 70     | Five integer variables intended     |

|                |        | for use by third-party developers   |

|----------------|--------|-------------------------------------|

| $USERR1 - 5    | 40     | Five real variables intended        |

|                |        | for use by third-party developers   |

|----------------|--------|-------------------------------------|

| $USRTIMER      | 70     | 0 = timer off, 1 = timer on         |

|----------------|--------|-------------------------------------|

| $VISRETAIN     | 70     | 0 = don't retain Xref-dependent     |

|                |        | visibility settings, 1 = retain     |

|                |        | Xref-dependent visibility settings  |

|----------------|--------|-------------------------------------|

| $WORLDVIEW     | 70     | 1 = set UCS to WCS during           |

|                |        | DVIEW/VPOINT, 0 = don't change UCS  |

+---------------------------------------------------------------+

The following header variables existed prior to AutoCAD Release 11

but now have independent settings for each active viewport. DXFIN

honors these variables when read from DXF files, but if a VPORT

symbol table with *ACTIVE entries is present (as is true for any DXF

file produced by Release 11 or higher), the values in the VPORT

table entries override the values of these header variables.

Table 11-4. Revised VPORT header variables

+===============================================================+

| Variable       | Type   | Description                         |

|----------------|--------|-------------------------------------|

| $FASTZOOM      | 70     | Fast zoom enabled if nonzero        |

|----------------|--------|-------------------------------------|

| $GRIDMODE      | 70     | Grid mode on if nonzero             |

|----------------|--------|-------------------------------------|

| $GRIDUNIT      | 10, 20 | Grid X and Y spacing                |

|----------------|--------|-------------------------------------|

| $SNAPANG       | 50     | Snap grid rotation angle            |

|----------------|--------|-------------------------------------|

| $SNAPBASE      | 10, 20 | Snap/grid base point (in UCS)       |

|----------------|--------|-------------------------------------|

| $SNAPISOPAIR   | 70     | Isometric plane: 0 = left, 1 = top, |

|                |        | 2 = right                           |

|----------------|--------|-------------------------------------|

| $SNAPMODE      | 70     | Snap mode on if nonzero             |

|----------------|--------|-------------------------------------|

| $SNAPSTYLE     | 70     | Snap style: 0 = standard,           |

|                |        | 1 = isometric                       |

|----------------|--------|-------------------------------------|

| $SNAPUNIT      | 10, 20 |Snap grid X and Y spacing            |

|----------------|--------|-------------------------------------|

| $VIEWCTR       | 10, 20 |XY center of current view on screen  |

|----------------|--------|-------------------------------------|

| $VIEWDIR       | 10, 20,| Viewing direction (direction from   |

|                | 30     | target, in WCS)                     |

|----------------|--------|-------------------------------------|

| $VIEWSIZE      | 40     | Height of view                      |

+----------------|--------|-------------------------------------+

The date/time variables ($TDCREATE and $TDUPDATE) are output as real

numbers in the following format:

<Julian date>.<Fraction>

The elapsed time variables ($TDINDWG and $TDUSRTIMER) have a similar

format:

<Number of days>.<Fraction>

The date and time variables are described on page 299.

TABLES Section

--------------

The TABLES section contains several tables, each of which contains a

variable number of table entries.

The order of the tables may change, but the LTYPE table will always

precede the LAYER table. Each table is introduced with a 0 group

with the label TABLE. This is followed by a 2 group identifying the

particular table (VPORT, LTYPE, LAYER, STYLE, VIEW, DIMSTYLE, UCS or

APPID) and a 70 group that specifies the maximum number of table

entries that may follow. Table names are always output in uppercase

characters.

The tables in a drawing can contain deleted items, but these are not

written to the DXF file. Thus, fewer table entries may follow the

table header than are indicated by the 70 group, so don't use the

count in the 70 group as an index to read in the table. This group

is provided so that a program which reads DXF files can allocate an

array large enough to hold all the table entries that follow.

Following this header for each table are the table entries. Each

table item consists of a 0 group identifying the item type (same as

table name, e.g., LTYPE or LAYER), a 2 group giving the name of the

table entry, a 70 group specifying flags relevant to the table entry

(defined for each following table), and additional groups that give

the value of the table entry. The end of each table is indicated by

a 0 group with the value ENDTAB.

The 70 group flag bit values that apply to all table entries are

described in the following chart. Additional 70 group values that

apply to LAYER, STYLE, and VIEW table entries are described in the

appropriate sections below.

Table 11-5. Group 70 bit codes that apply to all table entries

+===============================================================+

| Flag bit | Meaning                                            |

| value    |                                                    |

|----------|----------------------------------------------------|

| 16       | If set, table entry is externally dependent on     |

|          | an Xref                                            |

|----------|----------------------------------------------------|

| 32       | If this bit and bit 16 are both set, the           |

|          | externally dependent Xref has been successfully    |

|          | resolved                                           |

|----------|----------------------------------------------------|

| 64       | If set, the table entry was referenced by at least |

|          | one entity in the drawing the last time the drawing|

|          | was edited. (This flag is for the benefit of       |

|          | AutoCAD commands; it can be ignored by most        |

|          | programs that read DXF files, and need not be set  |

|          | by programs that write DXF files)                  |

+---------------------------------------------------------------+

The following are the groups used for each type of table item. All

groups are present for each table item.

APPID     2 (user-supplied application name), 70 (standard flag

values).

These table entries maintain a set of names for all

applications registered with a drawing.

DIMSTYLE  2 (dimension style name), 70 (standard flag values), and

the following, described by dimension variable name:

3 (dimpost), 4 (dimapost), 5 (dimblk), 6 (dimblk1),

7 (dimblk2), 40 (dimscale), 41 (dimasz), 42 (dimexo),

43 (dimdli), 44 (dimexe), 45 (dimrnd), 46 (dimdle),

47 (dimtp), 48 (dimtm), 140 (dimtxt), 141 (dimcen),

142 (dimtsz), 143 (dimaltf), 144 (dimlfac), 145 (dimtvp),

146 (dimtfac), 147 (dimgap), 71 (dimtol), 72 (dimlim),

73 (dimtih), 74 (dimtoh), 75 (dimse1), 76 (dimse2),

77 (dimtad), 78 (dimzin), 170 (dimalt), 171 (dimaltd),

172 (dimtofl), 173 (dimsah), 174 (dimtix), 175 (dimsoxd),

176 (dimclrd), 177 (dimclre), 178 (dimclrt).

LTYPE     2 (linetype name), 70 (standard flag values), 3

(descriptive text for linetype), 72 (alignment code; value

is always 65, the ASCII code for `A'), 73 (number of dash

length items), 40 (total pattern length), and optionally:

49 (dash length 1), 49 (dash length 2), and so on.

LAYER     2 (layer name), 70 (standard flag values), 62 (color

number, negative if layer is off), 6 (linetype name).

In addition to the standard flags, the 70 group flag is

bit coded as follows:

Table 11-6. Group 70 bit codes for LAYER table

+=====================================================+

| Flag bit | Meaning                                  |

| value    |                                          |

|----------|------------------------------------------|

| 1        | If set, layer is frozen                  |

|----------|------------------------------------------|

| 2        | If set, layer is frozen by default in    |

|          | new Viewports                            |

|----------|------------------------------------------|

| 4        | If set, layer is locked                  |

+-----------------------------------------------------+

If no value (0) is set, the layer is on and thawed. The

fourth bit (8) and the eighth bit (128) are not used.

Xref-dependent layers are output during DXFOUT. For these

layers, the associated linetype name in the DXF file is

always CONTINUOUS.

STYLE     2 (style name), 70 (standard flag values), 40 (fixed text

height; 0 if not fixed), 41 (width factor), 50 (oblique

angle), 71 (text generation flags), 42 (last height used),

3 (primary font filename), 4 (big-font file name; blank

if none).

If the third bit (4) is set in the 70 group flags, this

is a vertically oriented text style.

A STYLE table item is used to record shape file LOAD

requests also. In this case the first bit (1) is set in

the 70 group flags and only the 3 group (shape filename)

is meaningful (all the other groups are output, however).

The text generation flags are a bit-coded field with the

following bit meanings:

Table 11-7. Group 71 bit codes for STYLE table

+=====================================================+

| Flag bit | Meaning                                  |

| value    |                                          |

|----------|------------------------------------------|

| 2        | Text is backward (mirrored in X)         |

|----------|------------------------------------------|

| 4        | Text is upside down (mirrored in Y)      |

+-----------------------------------------------------+

UCS       2 (UCS name), 70 (standard flag values), 10, 20, 30

(origin), 11, 21, 31 (X axis direction), 12, 22, 32

(Y axis direction). All in World coordinates.

VIEW      2 (name of view), 70 (standard flag values), 40 and

41 (view height and width, in DCS), 10 and 20 (view

center point, in DCS), 11, 21, 31 (view direction from

target, in WCS), 12, 22, 32 (target point, in WCS),

42 (lens length), 43 and 44 (front and back clipping

planes - offsets from target point), 50 (twist angle),

71 view mode (see VIEWMODE system variable in appendix A).

If the first bit (1) is set in the 70 group flags, this

is a paper space view.

(See chapter 2 of the "AutoLISP Programmer's Reference"

for information on DCS, the Display Coordinate System.)

VPORT     2 (viewport name), 70 (standard flag values), 10 and 20

(lower-left corner of viewport; 0.0 to 1.0), 11 and

21 (upper-right corner), 12 and 22 (view center point,

in WCS), 13 and 23 (snap base point), 14 and 24 (snap

spacing, X and Y), 15 and 25 (grid spacing, X and Y), 16,

26, 36 (view direction from target point), 17, 27, 37

(view target point), 40 (view height), 41 (viewport aspect

ratio), 42 (lens length), 43 and 44 (front and back

clipping planes; offsets from target point),50 (snap

rotation angle), 51 (view twist angle), 68 (status field),

69 (ID), 71 (view mode; see VIEWMODE system variable in

appendix A), 72 (circle zoom percent), 73 (fast zoom

setting), 74 (UCSICON setting), 75 (snap on/off), 76 (grid

on/off), 77 (snap style), 78 (snap isopair).

The VPORT table is unique in that it may contain several

entries with the same name (indicating a multiple-viewport

configuration). The entries corresponding to the active

viewport configuration all have the name *ACTIVE. The

first such entry describes the current viewport.

BLOCKS Section

--------------

The Blocks section of the DXF file contains all the Block

Definitions. This section contains the entities that make up the

Blocks used in the drawing, including anonymous Blocks generated by

the HATCH command and by associative dimensioning. The format of the

entities in this section is identical to those in the Entities

section described later, so see that section for details. All

entities in the Blocks section appear between Block and Endblk

entities. Block and Endblk entities appear only in the Blocks

section. Block definitions are never nested (that is, no Block or

Endblk entity ever appears within another Block-Endblk pair),

although a Block definition can contain an INSERT entity.

External References are written in the DXF file as any Block

Definition, except they also include a text string (group code 1) of

the path and filename of the External Reference. This is the text

string format:

Xref filename

ENTITIES Section

----------------

Entity items appear in both the BLOCK and ENTITIES sections of the

DXF file. The appearance of entities in the two sections is

identical.

The following gives the format of each entity as it appears in the

file. Some groups that define an entity always appear, and some are

optional and appear only if they differ from their default values.

In the following discussion, groups that always occur are given by

their group number and function, while optional groups are indicated

by -optional N following the group description. N is the default

value if the group is omitted.

Programs that read DXF files should not assume that the groups

describing an entity occur in the order given here. The end of the

groups that make up an entity is indicated by the next 0 group,

beginning the next entity or indicating the end of the section.

Remember that a DXF file is a complete representation of the drawing

database, and that as AutoCAD is further enhanced, new groups will

be added to entities to accommodate additional features.

Accommodating DXF files from future releases of AutoCAD will be

easier if you write your DXF processing program in a table-driven

way, ignoring any groups not presently defined, and making no

assumptions about the order of groups in an entity.

Each entity begins with a 0 group identifying the entity type. The

names used for the entities are given on the following pages. Every

entity contains an 8 group that gives the name of the layer on which

the entity resides. Each entity may have elevation, thickness,

linetype, or color information associated with it.

If handles are enabled, every entity has a 5 group containing its

handle (as a string representing a hexadecimal number).

The following groups are included only if the entity has nondefault

values for these properties. When a group is omitted, its default

value upon input (when using DXFIN) is indicated in the third

column. If the value of a group is equal to the default, it is

omitted upon output (when using DXFOUT).

Table 11-8. Group codes common to all entities

+==================================================================+

| Group  | Meaning                                 | If omitted,   |

| code   |                                         | defaults to...|

|--------|-----------------------------------------|---------------|

| 6      | Linetype name (if not BYLAYER). The     | BYLAYER       |

|        | special name BYBLOCK indicates a        |               |

|        | floating linetype                       |               |

|--------|-----------------------------------------|---------------|

| 38     | Elevation (if nonzero). Exists only in  | 0             |

|        | output from versions prior to R11.      |               |

|        | Otherwise, Z coordinates are supplied   |               |

|        | as 3x-groups as part of each of the     |               |

|        | entity's defining points                |               |

|--------|-----------------------------------------|---------------|

| 39     | Thickness (if nonzero)                  | 0             |

|--------|-----------------------------------------|---------------|

| 62     | Color number (if not BYLAYER). Zero     | BYLAYER       |

|        | indicates the BYBLOCK (floating) color. |               |

|        | 256 indicates the BYLAYER color         |               |

|--------|-----------------------------------------|---------------|

| 67     | Absent or zero indicates entity is in   | 0             |

|        | model space. One indicates entity is    |               |

|        | in paper space, other values are        |               |

|        | reserved                                |               |

|--------|-----------------------------------------|---------------|

| 210,   | These groups are included for each Line,| 0,0,1         |

| 220,   | Point, Circle, Shape, Text, Arc, Trace, |               |

| 230    | Solid, Block Reference, Polyline,       |               |

|        | Dimension, Attribute, and Attribute     |               |

|        | Definition entity if its extrusion      |               |

|        | direction is not parallel to the World  |               |

|        | Z axis. They indicate the X, Y, and Z   |               |

|        | components of the entity's extrusion    |               |

|        | direction                               |               |

+------------------------------------------------------------------+

The rest of the groups that make up an entity item are described

next. Many of the entities include "flag" groups. These are integer

codes (6x or 7x groups) that encode various pieces of information

regarding the entity, and are specific to the particular entity

type. In the following descriptions, the term bit-coded means that

the flag contains various true/false values coded as the sum of the

bit values given. Any bits not defined in the following section

should be ignored in these fields and set to zero when constructing

a DXF file.

LINE      10, 20, 30 (start point), 11, 21, 31 (endpoint).

POINT     10, 20, 30 (point).

Point entities have an optional 50 group that determines

the orientation of PDMODE images. The group value is the

negative of the Entity Coordinate Systems (ECS) angle of

the UCS X axis in effect when the point was drawn. The X

axis of the UCS in effect when the point was drawn is

always parallel to the XY plane for the point's ECS, and

the angle between the UCS X axis and the ECS X axis is a

single 2D angle. The value in group 50 is the angle from

horizontal (the effective X axis) to the ECS X axis.

Entity Coordinate Systems (ECS) are described later in

this section.

CIRCLE    10, 20, 30 (center), 40 (radius).

ARC       10, 20, 30 (center), 40 (radius), 50 (start angle), 51

(end angle).

TRACE     Four points defining the corners of the trace: (10, 20,

30), (11, 21, 31), (12, 22, 32), and (13, 23, 33).

SOLID     Four points defining the corners of the solid: (10, 20,

30), (11, 21, 31), (12, 22, 32), and (13, 23, 33). If only

three points were entered (forming a triangular solid),

the third and fourth points will be the same.

TEXT      10, 20, 30 (insertion point), 40 (height), 1 (text value),

50 (rotation angle -optional 0), 41 (relative X-scale

factor -optional 1), 51 (oblique angle -optional 0), 7

(text style name -optional STANDARD), 71 (text generation

flags -optional 0), 72 (horizontal justification type -

optional 0), 73 (vertical justification type -optional 0)

11, 21, 31 (alignment point -optional, appears only if 72

or 73 group is present and nonzero).

The "text generation flags" are a bit-coded field with

meanings as follows:

Table 11-9. Group 71 bit codes for Text entity

+=====================================================+

| Flag bit | Meaning                                  |

| value    |                                          |

|----------|------------------------------------------|

| 2        | Text is backward (mirrored in X)         |

|----------|------------------------------------------|

| 4        | Text is upside down (mirrored in Y)      |

+-----------------------------------------------------+

The justification-type value (group codes 72 and 73, not

bit-coded) indicates the text-justification style used on

the text, as shown in the following table:

Table 11-10.  Group 72 & 73 bit codes for Text entity

+======================================================+

| Group 73   | Group 72 (horizontal alignment)         |

|(vertical   |-----------------------------------------|

| alignment) |0    |1      |2     |3      |4     |5    |

|------------|-----|-------|------|-------|------|-----|

|3 (Top)     |TLeft|TCenter|TRight|       |      |     |

|------------|-----|-------|------|-------|------|-----|

|2 (Middle)  |MLeft|MCenter|MRight|       |      |     |

|------------|-----|-------|------|-------|------|-----|

|1 (Bottom)  |BLeft|BCenter|BRight|       |      |     |

|------------|-----|-------|------|-------|------|-----|

|0 (Baseline)|Left |Center |Right |Aligned|Middle| Fit |

+------------------------------------------------------+

If the justification is anything other than baseline/left

(groups 72 and 73 both 0), group codes 11, 21, and 31

specify the alignment point (or the second alignment point

for Align or Fit).

DXFOUT handles ASCII control characters in text strings by

expanding the character into a ^ (caret) followed by the

appropriate letter. For example, an ASCII Control-G (BEL,

decimal code 7) is output as ^G. If the text itself

contains a caret character, it is expanded to ^ (caret,

space). DXFIN performs the complementary conversion.

SHAPE     10, 20, 30 (insertion point), 40 (size), 2 (shape name),

50 (rotation angle -optional 0), 41 (relative X-scale

factor -optional 1), 51 (oblique angle -optional 0).

BLOCK     2 (Block name), 3 (this is also the Block name), 70 (Block

type flag), 10, 20, 30 (Block base point), and if the

Block is an Xref Block it will also contain group code 1

(Xref pathname). Block entities appear only in the BLOCKS

section, not in the ENTITIES section. The "Block type

flag" (group 70) is bit-coded, with the following bit

meanings:

Table 11-11. Group 70 bit codes for Block table

+=======================================================+

| Flag bit | Meaning                                    |

| value    |                                            |

|----------|--------------------------------------------|

| 1        | This is an anonymous Block generated by    |

|          | hatching, associative dimensioning, other  |

|          | internal operations, or an application     |

|----------|--------------------------------------------|

| 2        | This Block has Attributes                  |

|----------|--------------------------------------------|

| 4        | This Block is an external reference (Xref) |

|----------|--------------------------------------------|

| 8        | not used                                   |

|----------|--------------------------------------------|

| 16       | This Block is externally dependent         |

|----------|--------------------------------------------|

| 32       | This is a resolved external reference, or  |

|          | dependent of an external reference         |

|----------|--------------------------------------------|

| 64       | This definition is referenced              |

+-------------------------------------------------------+

ENDBLK    No groups. Appears only in BLOCKS section.

INSERT    66 (Attributes follow flag -optional 0), 2 (Block name),

10, 20, 30 (insertion point), 41 (X- scale factor

-optional 1), 42 (Y scale factor -optional 1), 43 (Z-

scale factor -optional 1), 50 (rotation angle -optional

0), 70 and 71 (column and row counts -optional 1), 44 and

45 (column and row spacing -optional 0).

If the value of the "Attributes follow" flag is 1, a

series of Attribute (Attrib) entities is expected to

follow the Insert, terminated by a sequence end (Seqend)

entity.

ATTDEF    10, 20, 30 (text start), 40 (text height), 1 (default

value, see "Text" on page 260 for handling of ASCII

control characters), 3 (prompt string), 2 (tag string), 70

(Attribute flags), 73 (field length -optional 0), 50 (text

rotation - optional 0), 41 (relative X scale factor

-optional 1), 51 (oblique angle -optional 0), 7 (text

style name -optional STANDARD), 71 (text generation flags

-optional 0, see "Text" on page 260), 72  (horizontal text

justification type - optional 0, see "Text" on page 260),

74 (vertical text justification type -optional 0 see group

73 in "Text" on page 260), 11, 21, 31 (alignment point

-optional, appears only if 72 or 74 group is present and

nonzero).

The "Attribute flags" (group code 70) are a bit-coded

field in which the bits have the following meanings:

Table 11-12. Group 70 bit codes for Attdef entity

+=======================================================+

| Flag bit | Meaning                                    |

| value    |                                            |

|----------|--------------------------------------------|

| 1        | Attribute is invisible (does not display)  |

|----------|--------------------------------------------|

| 2        | This is a constant Attribute               |

|----------|--------------------------------------------|

| 4        | Verification is required on input of this  |

|          | Attribute                                  |

|----------|--------------------------------------------|

| 8        | Attribute is preset (no prompt during      |

|          | insertion)                                 |

+-------------------------------------------------------+

ATTRIB    10, 20, 30 (text start), 40 (text height), 1 (value, see

"Text" on page 260 for handling ASCII control characters),

2 (Attribute tag), 70 (Attribute flags; see Attdef), 73

(field length -optional 0), 50 (text rotation -optional

0), 41 (relative X scale factor -optional 1), 51 (oblique

angle -optional 0), 7 (text style name -optional

STANDARD), 71 (text generation flags -optional 0, see

"Text" on page 260), 72 (horizontal text justification

type -optional 0, see "Text" on page 260), 74 (vertical

text justification type -optional 0, see group 73 in

"Text" on page 260), 11, 21, 31 (alignment point

-optional, appears only if 72 or 74 group is present

and nonzero).

POLYLINE  66 (vertices-follow flag), 10, 20, 30 (polyline elevation

- 30 supplies elevation, 10 and 20 are always set to

zero), 70 (Polyline flag -optional 0), 40 (default

starting width - optional 0), 41 (default ending width

-optional 0), 71 and 72 (polygon mesh M and N vertex counts

-optional 0), 73 and 74 (smooth surface M and N densities

-optional 0), 75 (curves and smooth surface type -optional

0). The default widths apply to any vertex that doesn't

supply widths (see later).

The "vertices follow" flag is always 1, indicating that a

series of Vertex entities is expected to follow the

Polyline, terminated by a sequence end (Seqend) entity.

The polyline flag (group code 70) is a bit-coded field

with bits defined as follows:

Table 11-13. Group 70 bit codes for Polyline entity

+======================================================+

| Flag bit | Meaning                                   |

| value    |                                           |

|----------|-------------------------------------------|

| 1        | This is a closed Polyline (or a polygon   |

|          | mesh closed in the M direction)           |

|----------|-------------------------------------------|

| 2        | Curve-fit vertices have been added        |

|----------|-------------------------------------------|

| 4        | Spline-fit vertices have been added       |

|----------|-------------------------------------------|

| 8        | This is a 3D Polyline                     |

|----------|-------------------------------------------|

| 16       | This is a 3D polygon mesh.                |

|          | Group 75 indicates the smooth surface type|

|          | as follows:                               |

|          | 0 = no smooth surface fitted              |

|          | 5 = quadratic B-spline surface            |

|          | 6 = cubic B-spline surface                |

|          | 8 = Bezier surface                        |

|----------|-------------------------------------------|

| 32       | The polygon mesh is closed in the N       |

|          | direction                                 |

|----------|-------------------------------------------|

| 64       | This Polyline is a polyface mesh          |

|----------|-------------------------------------------|

| 128      | The linetype pattern is generated         |

|          | continuously around the vertices of       |

|          | this Polyline                             |

+------------------------------------------------------+

A polyface mesh is represented in DXF as a variant of a

Polyline entity. The Polyline header is identified as

introducing a polyface mesh by the presence of the 64 bit

in the Polyline flags (70) group. The 71 group specifies

the number of vertices in the mesh, and the 72 group, the

number of faces. While these counts are correct for all

meshes created with the PFACE command, applications are

not required to place correct values in these fields, and

AutoCAD actually never relies upon their accuracy.

Following the Polyline header is a sequence of Vertex

entities that specify the vertex coordinates and faces

that compose the mesh. Vertices such as these are

described in the following subsection on Vertex.

Applications might want to represent polygons with an

arbitrarily large number of sides in polyface meshes.

However, the AutoCAD entity structure imposes a limit on

the number of vertices that a given face entity can

specify. You can represent more complex polygons by

decomposing them into triangular wedges. Their edges

should be made invisible to prevent visible artifacts of

this subdivision from being drawn. The PFACE command

performs this subdivision automatically, but when

applications generate polyface meshes directly, the

applications must do this themselves.

The number of vertices per face is the key parameter in

this subdivision process. The PFACEVMAX system variable

provides an application with the number of vertices per

face entity. This value is read-only, and is set to 4.

Polyface meshes created with the PFACE command are always

generated with all the vertex coordinate entities first,

followed by the face definition entities. The code within

AutoCAD that processes polyface meshes does not, at

present, require this ordering; it works even with

interleaved vertex coordinates and face definitions as

long as no face specifies a vertex with an index that

appears after it in the database. Programs that read

polyface meshes from DXF would be wise to be as tolerant

of odd vertex and face ordering as AutoCAD is.

VERTEX    10, 20, 30 (location), 40 (starting width -optional, see

earlier), 41 (ending width -optional, see above), 42

(bulge -optional 0), 70 (vertex flags -optional 0), 50

(curve fit tangent direction -optional). The bulge is the

tangent of 1/4 the included angle for an arc segment, made

negative if the arc goes clockwise from the start point to

the endpoint; a bulge of 0 indicates a straight segment,

and a bulge of 1 is a semicircle. The meaning of the bit-

coded Vertex flag (group code 70) is shown in the

following table

Table 11-14. Group 70 bit codes for Vertex entity

+========================================================+

| Flag bit | Meaning                                     |

| value    |                                             |

|----------|---------------------------------------------|

| 1        | Extra vertex created by curve-fitting       |

|----------|---------------------------------------------|

| 2        | Curve-fit tangent defined for this vertex.  |

|          | A curve-fit tangent direction of 0 may be   |

|          | omitted from the DXF output, but is         |

|          | significant if this bit is set              |

|----------|---------------------------------------------|

| 4        | Unused (never set in DXF files)             |

|----------|---------------------------------------------|

| 8        | Spline vertex created by spline-fitting     |

|----------|---------------------------------------------|

| 16       | Spline frame control point                  |

|----------|---------------------------------------------|

| 32       | 3D Polyline vertex                          |

|----------|---------------------------------------------|

| 64       | 3D polygon mesh vertex                      |

|----------|---------------------------------------------|

| 128      | Polyface mesh vertex                        |

+--------------------------------------------------------+

Every Vertex that is part of a polyface mesh has the 128

bit set in its Vertex flags (70) group. If the entity

specifies the coordinates of a vertex of the mesh, the 64

bit is set as well and the 10, 20, and 30 groups give the

vertex coordinates. The vertex indexes are determined by

the order in which the Vertex entities appear within the

Polyline, with the first numbered 1.

If the Vertex defines a face of the mesh, its Vertex flags

(70) group has the 128 bit set but not the 64 bit. The 10,

20, and 30 (location) groups of the face entity are

irrelevant and are always written as zero in a DXF file.

The vertex indexes that define the mesh are given by 71,

72, 73, and 74 groups, the values of which are integers

specifying one of the previously defined vertices by

index. If the index is negative, the edge that begins with

that vertex is invisible. The first zero vertex marks the

end of the vertices of the face. Since the 71 through 74

groups are optional fields with default values of zero,

they are present in DXF only if nonzero.

SEQEND    No fields. This entity marks the end of vertices (Vertex

type name) for a Polyline, or the end of Attribute

entities (Attrib type name) for an Insert entity that has

Attributes (indicated by 66 group present and nonzero in

Insert entity).

3DFACE    Four points defining the corners of the face: (10, 20,

30), (11, 21, 31), (12, 22, 32), and (13, 23, 33). 70

(invisible           edge flags -optional 0). If only

three points are entered (forming a triangular face), the

third and fourth points will be the same. The meanings of

the bit-coded "invisible edge flags" are shown in the

following table:

Table 11-15. Group 70 bit codes for 3D Face entity

+===========================================+

| Flag bit | Meaning                        |

| value    |                                |

|----------|--------------------------------|

| 1        | First edge is invisible        |

|----------|--------------------------------|

| 2        | Second edge is invisible       |

|----------|--------------------------------|

| 4        | Third edge is invisible        |

|----------|--------------------------------|

| 8        | Fourth edge is invisible       |

+-------------------------------------------+

VIEWPORT  10,20,30 (center point of entity in paper space

coordinates), 40 (width in paper space units), 41 (height

in paper space units), 68 (viewport status field), 69

(viewport ID, permanent during editing sessions, but

mutable between sessions; the paper space viewport entity

always has an ID of 1).

The value of the viewport status field (68) is interpreted

as follows:

-1                On, but is fully off-screen or is one of

the viewports not active because the

$MAXACTVP count is currently being

exceeded.

0                 Off.

<positive value>  On, active and the value indicates the

order of "stacking" for the viewports,

with 1 applying to the active viewport,

which is also the highest, 2 applying

to the next viewport in the stack,

and so on.

In addition, the extended entity data groups in the

following table apply to viewports.

NOTE: In contrast to normal entity data, the same extended

entity group code can appear multiple times, and order is

important.

Table 11-16. Extended entity group codes for Viewports

+========================================================+

| Group   | Description                                  |

|---------|----------------------------------------------|

| 1001    | Application name. This field will always be  |

|         | the string "ACAD"                            |

|---------|----------------------------------------------|

| 1000    | Begin viewport data. This field will always  |

|         | be the string "MVIEW". Other data groups may |

|         | appear in the future                         |

|---------|----------------------------------------------|

| 1002    | Begin window descriptor data. This field will|

|         | always be the string "{"                     |

|---------|----------------------------------------------|

| 1070    | Extended entity data version number. For     |

|         | Releases 11 and 12, this field will always be|

|         | the integer 16                               |

|---------|----------------------------------------------|

| 1010    | View target point X value                    |

|---------|----------------------------------------------|

| 1020    | View target point Y value                    |

|---------|----------------------------------------------|

| 1030    | View target point Z value                    |

|---------|----------------------------------------------|

| 1010    | View direction vector X value                |

|---------|----------------------------------------------|

| 1020    | View direction vector Y value                |

|---------|----------------------------------------------|

| 1030    | View direction vector Z value                |

|---------|----------------------------------------------|

| 1040    | View twist angle                             |

|---------|----------------------------------------------|

| 1040    | View height                                  |

|---------|----------------------------------------------|

| 1040    | View center point X value                    |

|---------|----------------------------------------------|

| 1040    | View center point Y value                    |

|---------|----------------------------------------------|

| 1040    | Perspective lens length                      |

|---------|----------------------------------------------|

| 1040    | Front clip plane Z value                     |

|---------|----------------------------------------------|

| 1040    | Back clip plane Z value                      |

|---------|----------------------------------------------|

| 1070    | View mode                                    |

|---------|----------------------------------------------|

| 1070    | Circle zoom                                  |

|---------|----------------------------------------------|

| 1070    | Fast zoom setting                            |

|---------|----------------------------------------------|

| 1070    | UCSICON setting                              |

|---------|----------------------------------------------|

| 1070    | Snap ON/OFF                                  |

|---------|----------------------------------------------|

| 1070    | Grid ON/OFF                                  |

|---------|----------------------------------------------|

| 1070    | Snap style                                   |

|---------|----------------------------------------------|

| 1070    | Snap ISOPAIR                                 |

|---------|----------------------------------------------|

| 1040    | Snap angle                                   |

|---------|----------------------------------------------|

| 1040    | Snap base point UCS X coordinate             |

|---------|----------------------------------------------|

| 1040    | Snap base point UCS Y coordinate             |

|---------|----------------------------------------------|

| 1040    | Snap X spacing                               |

|---------|----------------------------------------------|

| 1040    | Snap Y spacing                               |

|---------|----------------------------------------------|

| 1040    | Grid X spacing                               |

|---------|----------------------------------------------|

| 1040    | Grid Y spacing                               |

|---------|----------------------------------------------|

| 1070    | Hidden in plot flag                          |

|---------|----------------------------------------------|

| 1002    | Begin frozen layer list (possibly empty).    |

|         | This field will always be the string "{"     |

|---------|----------------------------------------------|

| 1003... | The names of layers frozen in this viewport. |

|         | This list may include Xref-dependent layers. |

|         | Any number of 1003 groups may appear here    |

|---------|----------------------------------------------|

| 1002    | End frozen layer list. This field will always|

|         | be the string "}"                            |

|---------|----------------------------------------------|

| 1002    | End viewport data. This field will always be |

|         | the string "}"                               |

+--------------------------------------------------------+

DIMENSION 2 (name of pseudo-Block containing the current dimension

entity geometry), 3 (dimension style name), 10, 20, 30

(definition point for all dimension types), 11, 21, 31

(middle point of dimension text), 12, 22, 32 (dimension

block translation vector), 70 (Dimension type), 1

(dimension text explicitly entered by the user. If null or

"<>", the dimension measurement is drawn as the text, if

" " [one blank space], the text is suppressed. Anything

else is drawn as the text). 13, 23, 33 (definition point

for linear and angular dimensions), 14, 24, 34 (definition

point for linear and angular dimensions), 15, 25, 35

(definition point for diameter, radius, and angular

dimensions), 16, 26, 36 (point defining dimension arc for

angular dimensions), 40 (leader length for radius and

diameter dimensions), 50 (angle of rotated, horizontal, or

vertical linear dimensions).

The dimension type (group code 70) is an integer-coded

field with the following values:

Table 11-17. Group 70 integer codes for Dimension entity

+========================================================+

| Group   | Description                                  |

|---------|----------------------------------------------|

| 0       | Rotated, horizontal, or vertical             |

|---------|----------------------------------------------|

| 1       | Aligned                                      |

|---------|----------------------------------------------|

| 2       | Angular                                      |

|---------|----------------------------------------------|

| 3       | Diameter                                     |

|---------|----------------------------------------------|

| 4       | Radius                                       |

|---------|----------------------------------------------|

| 5       | Angular 3-point                              |

|---------|----------------------------------------------|

| 6       | Ordinate                                     |

|---------|----------------------------------------------|

| 64      | Ordinate type. This is a bit value (bit 7)   |

|         | used only with integer value 6. If set,      |

|         | ordinate is X-type; if not set, ordinate is  |

|         | Y-type                                       |

|---------|----------------------------------------------|

| 128     | This is a bit value (bit 8) added to the     |

|         | other group 70 values if the dimension text  |

|         | has been positioned at a user-defined        |

|         | location rather than at the default location |

+--------------------------------------------------------+

In addition, all dimension types have an optional group

(code 51) that indicates the horizontal direction for the

Dimension entity. This determines the orientation of

dimension text and dimension lines for horizontal,

vertical, and rotated linear dimensions. The group value

is the negative of the Entity Coordinate Systems (ECS)

angle of the UCS X axis in effect when the Dimension was

drawn. The X axis of the UCS in effect when the Dimension

was drawn is always parallel to the XY plane for the

Dimension's ECS, and the angle between the UCS X axis and

the ECS X axis is a single 2D angle. The value in group 51

is the angle from horizontal (the effective X axis) to the

ECS X axis. Entity Coordinate Systems (ECS) are described

later in this section.

Linear dimension types with an oblique angle have an

optional group (code 52). When added to the rotation angle

of the linear dimension (group code 50) this gives the

angle of the extension lines. The optional group code 53

is the rotation angle of the dimension text away from its

default orientation (the direction of the dimension line).

For all dimension types, the following groups represent 3D

WCS points:

10, 20, 30

13, 23, 33

14, 24, 34

15, 25, 35

For all dimension types, the following groups represent 3D

ECS points:

11, 21, 31

12, 22, 32

16, 26, 36

Linear    (13,23,33)  The point used to specify the first extension

line.

(14,24,34)  The point used to specify the second extension

line.

(10,20,30)  The point used to specify the dimension line.

Refer to figure 11-1 on page 267 of the "AutoCAD

Customization Manual"

Angular   (13,23,33) and (14,24,34)  The endpoints of the first

extension line.

(10,20,30) and (15,25,35)  The endpoints of the second

extension line.

(16,26,36)                 The point used to specify the

dimension line arc.

Refer to figure 11-2 on page 267 of the "AutoCAD

Customization Manual"

Angular   (15,25,35)  The vertex of the angle.

(3-point) (13,23,33)  The endpoints of the first extension line.

(13,23,33)  The endpoints of the first extension line.

(14,24,34)  The endpoints of the second extension line.

(10,20,30)  The point used to specify the dimension line

arc.

Refer to figure 11-3 on page 267 of the "AutoCAD

Customization Manual"

Diameter  (15,25,35)  The point used to pick the circle/arc to

dimension.

(10,20,30)  The point on that circle directly across from

the pick point.

Refer to figure 11-4 on page 268 of the "AutoCAD

Customization Manual"

Radius    (15,25,35)  The point used to pick the circle/arc to

dimension.

(10,20,30)  The center of that circle.

Refer to figure 11-5 on page 268 of the "AutoCAD

Customization Manual"

Ordinate  (13,23,33)  The point used to select the feature.

(14,24,34)  The point used to locate the leader end point.

Refer to figure 11-6 on page 268 of the "AutoCAD

Customization Manual"

Entity Coordinate Systems (ECS)

-------------------------------

To save space in the drawing database (and in the DXF file), the

points associated with each entity are expressed in terms of the

entity's own Entity Coordinate System (ECS). The Entity Coordinate

System allows AutoCAD to use a much more compact means of

representation for entities. With ECS, the only additional

information needed to describe the entity's position in 3D space is

the 3D vector describing the Z axis of the ECS, and the elevation

value.

For a given Z axis (or extrusion) direction, there are an infinite

number of coordinate systems, defined by translating the origin in

3D space and by rotating the X and Y axes around the Z axis.

However, for the same Z axis direction, there is only one Entity

Coordinate System. It has the following properties:

o  Its origin coincides with the WCS origin.

o  The orientation of the X and Y axes within the XY plane are

calculated in an arbitrary, but consistent manner. AutoCAD

performs this calculation using the arbitrary axis algorithm

(described later).

For some entities, the ECS is equivalent to the World Coordinate

System and all points (DXF groups 10 - 37) are expressed in World

coordinates. See the following table.

Table 11-18. Coordinate systems associated with an entity type

+==================================================================+

| Entities                   | Notes                               |

|----------------------------|-------------------------------------|

| Line, Point, 3DFace,       | These entities do not lie in a      |

| 3D Polyline, 3D Vertex,    | particular plane. All points are    |

| 3D Mesh, 3D Mesh vertex    | expressed in World coordinates. Of  |

|                            | these entities, only Lines and      |

|                            | Points can be extruded; their       |

|                            | extrusion direction can differ from |

|                            | the World Z axis                    |

|----------------------------|-------------------------------------|

| Circle, Arc, Solid, Trace, | These entities are planar in        |

| Text, Attrib, Attdef,      | nature. All points are expressed    |

| Shape, Insert, 2D Polyline,| in Entity coordinates. All of these |

| 2D Vertex                  | entities can be extruded; their     |

|                            | extrusion direction can differ from |

|                            | the World Z axis                    |

|----------------------------|-------------------------------------|

| Dimension                  | Some of a Dimension's points are    |

|                            | expressed in WCS, and some in ECS   |

|----------------------------|-------------------------------------|

| Viewport                   | Expressed in World coordinates      |

|----------------------------|-------------------------------------|

| Others                     | The remaining entities have no      |

|                            | point data and their coordinate     |

|                            | systems are therefore irrelevant    |

+------------------------------------------------------------------+

Once AutoCAD has established the ECS for a given entity, here's how

it works:

o The elevation value stored with an entity indicates how far along

the Z axis to shift the XY plane from the WCS origin to make it

coincide with the plane that the entity is in. How much of this is

the user-defined elevation is unimportant.

o Any 2D points describing the entity that were entered through the

UCS are transformed into the corresponding 2D points in the ECS,

which (more often than not) is shifted and rotated with respect to

the UCS.

These are a few ramifications of this process:

o You cannot reliably find out what UCS was in effect when an entity

was acquired.

o When you enter the XY coordinates of an entity in a given UCS and

then do a DXFOUT, you probably won't recognize those XY

coordinates in the DXF file. You'll have to know the method by

which AutoCAD calculates the X and Y axes in order to work with

these values.

o The elevation value stored with an entity and output in DXF files

will be a sum of the Z-coordinate difference between the UCS XY

plane and the ECS XY plane, and the elevation value that the user

specified   at the time the entity was drawn.

Arbitrary Axis Algorithm

------------------------

The arbitrary axis algorithm is used by AutoCAD internally to

implement the arbitrary but consistent generation of Entity

Coordinate Systems for all entities except Lines, Points, 3D Faces,

and 3D Polylines, which contain points in World coordinates.

Given a unit-length vector to be used as the Z axis of a coordinate

system, the arbitrary axis algorithm generates a corresponding X

axis for the coordinate system. The Y axis follows by application of

the right-hand rule.

The method is to examine the given Z axis (also called the normal

vector) and see if it is close to the positive or negative World Z

axis. If it is, cross the World Y axis with the given Z axis to

arrive at the arbitrary X axis. If not, cross the World Z axis with

the given Z axis to arrive at the arbitrary X axis. The boundary at

which the decision is made was chosen to be both inexpensive to

calculate and completely portable across machines. This is achieved

by having a sort of "square" polar cap, the bounds of which is 1/64,

which is precisely specifiable in 6 decimal fraction digits and in 6

binary fraction bits.

In mathematical terms, the algorithm does the following (all vectors

are assumed to be in 3D space, specified in the World Coordinate

System):

Let the given normal vector be called N.

Let the World Y axis be called Wy, which is always (0,1,0).

Let the World Z axis be called Wz, which is always (0,0,1).

We are looking for the arbitrary X and Y axes to go with the normal

N. They'll be called Ax and Ay. N could also be called Az (the

arbitrary Z axis):

If (abs (Nx) < 1/64) and (abs (Ny) < 1/64) then

Ax = Wy  N (where "" is the cross-product operator).

Otherwise,

Ax = Wz  N.

Scale Ax to unit length.

The method of getting the Ay vector would be:

Ay = N  Ax. Scale Ay to unit length.

Extended Entity Data

====================

Extended entity data is created by applications such as the Advanced

Modeling Extension (AME), or by routines written with AutoLISP or

ADS. Extended entity data is also produced by creating PostScript

output with PSOUT. If an entity contains extended data, it follows

the entity's normal definition data.

The group codes 1000 through 1071 describe extended entity data. The

following is an example of an entity containing extended entity data

in DXF format.

___

0                  |

INSERT             |

8                 |

0                  |

5                 |

5                  |

F11                |

15                 |-- Normal entity definition data.

2                 |

BLOCK_A            |

10                |

0.0                |

20                |

0.0                |

30                |

0.0             ___|

1001               |

AME_SOL            |

1002               |

{                  |

1070               |

0                 |

1071               |

1.95059E+06       |

1070               |

519               |

1010               |

2.54717            |

1020               |

2.122642           |-- Extended entity data.

1030               |

2.049201           |

1005               |

ECD                |

1005               |

EE9                |

1005               |

0                  |

1040               |

0.0                |

1040               |

1.0                |

1000               |

MILD_STEEL      ___|

Figure 11-7.  Example of extended entity data

Organization of Extended Entity Data

====================================

As you can see in the above example, group code 1001 indicates the

beginning of extended entity data. This is followed by one or more

1000 group codes. Application names are string values (in the

example, the application name is AME_SOL). In contrast to normal

entity data, the same group code can appear multiple times, and

order is important.

Extended entity data are grouped by registered application name, and

each registered application's group begins with a 1001 group code

with the registered application name as the string value. Registered

application names correspond to APPID symbol table entries, which

are essentially placeholders for registered application names.

An application can use as many APPID names as needed, although one

will often suffice. APPID names are permanent, although they can be

purged if they aren't currently used in the drawing.

Each APPID name can have no more than one data group attached to

each entity. Within an application's group, the sequence of extended

entity data groups and their meaning is defined by the application.

NOTE: PostScript images and PostScript fill requests for Polylines

are stored in the AutoCAD database as extended entity data belonging

to the AUTOCAD_POSTSCRIPT_FIGURE application.

As the example in the previous figure shows, the group codes for

extended entity data begin at 1000 and currently extend to 1071. The

following list of extended entity data group codes are supported by

AutoCAD, which maintains and manipulates their values as described:

Table 11-19. extended entity data group codes and descriptions

+==================================================================+

| Entity Name    | Group code | Description                        |

|----------------|------------|------------------------------------|

| String         | 1000       | Strings in extended entity data    |

|                |            | can be up to 255 bytes long (with  |

|                |            | the 256th byte reserved for the    |

|                |            | null character)                    |

|----------------|------------|------------------------------------|

| Application    | 1001       | Application names can be up to 31  |

| name           | also a     | bytes long (the 32d byte is        |

|                | string     | reserved for the null character).  |

|                | value      | Use of application names is        |

|                |            | described in more detail later in  |

|                |            | this section                       |

|                |            | CAUTION: Do not add a 1001 group   |

|                |            | into your extended entity data, as |

|                |            | AutoCAD will assume it is the      |

|                |            | beginning of a new application     |

|                |            | extended entity data group         |

|----------------|------------|------------------------------------|

| Control string | 1002       | An extended data control string can|

|                |            | be either "{"or "}": these braces  |

|                |            | enable applications to organize    |

|                |            | their data by subdividing the data |

|                |            | into lists. The left brace begins a|

|                |            | list, and a right brace terminates |

|                |            | the most recent list; lists can be |

|                |            | nested                             |

|                |            | When AutoCAD reads the extended    |

|                |            | entity data for a particular       |

|                |            | application, it checks to ensure   |

|                |            | that braces are balanced correctly |

|----------------|------------|------------------------------------|

| Layer name     | 1003       | Name of the layer associated with  |

|                |            | the extended entity data           |

|----------------|------------|------------------------------------|

| Binary data    | 1004       | Binary data is organized into      |

|                |            | variable-length chunks.The maximum |

|                |            | length of each chunk is 127 bytes. |

|                |            | Binary data is represented as a    |

|                |            | string of hexadecimal digits, two  |

|                |            | per binary byte, in ASCII DXF files|

|----------------|------------|------------------------------------|

| Database       | 1005       | Handles of entities in the         |

| handle         |            | drawing database                   |

|                |            | NOTE: When a drawing with handles  |

|                |            | and extended entity data handles   |

|                |            | is imported into another drawing   |

|                |            | using INSERT, INSERT *, XREF Bind, |

|                |            | XBIND, or partial DXFIN, the       |

|                |            | extended entity data handles are   |

|                |            | translated in the same manner as   |

|                |            | their corresponding entity handles,|

|                |            | thus maintaining their binding.    |

|                |            | This is also done in the EXPLODE   |

|                |            | Block operation, or for any other  |

|                |            | AutoCAD operation. When AUDIT      |

|                |            | detects an extended entity data    |

|                |            | handle that doesn't match the      |

|                |            | handle of an entity in the drawing |

|                |            | file, it is considered an error.   |

|                |            | If AUDIT is fixing entities, it    |

|                |            | sets the handle to 0.              |

|----------------|------------|------------------------------------|

| 3 reals        | 1010,      | Three real values, in the order    |

|                | 1020,      | X, Y, Z. They can be used as       |

|                | 1030       | a point or vector record. AutoCAD  |

|                |            | never alters their value           |

|----------------|------------|------------------------------------|

| World space    | 1011,      | Unlike a simple 3D point,the World |

| position       | 1021,      | space coordinates are moved,       |

|                | 1031       | scaled, rotated, and mirrored      |

|                |            | along with the parent entity to    |

|                |            | which the extended data belongs.   |

|                |            | The world space position is also   |

|                |            | stretched when the STRETCH command |

|                |            | is applied to the parent entity    |

|                |            | and this point lies within the     |

|                |            | select window                      |

|----------------|------------|------------------------------------|

| World space    | 1012,      | Also a 3D point that is scaled,    |

| displacement   | 1022,      | rotated, and mirrored along with   |

|                | 1032       | the parent (but not moved or       |

|                |            | stretched)                         |

|----------------|------------|------------------------------------|

| World          | 1013,      | Also a 3D point that is rotated    |

| direction      | 1023,      | and mirrored along with the parent |

|                | 1033       | (but not moved, scaled, or         |

|                |            | stretched).                        |

|----------------|------------|------------------------------------|

| Real           | 1040       | A real value                       |

|----------------|------------|------------------------------------|

| Distance       | 1041       | A real value that is scaled along  |

|                |            | with the parent entity             |

|----------------|------------|------------------------------------|

| Scale factor   | 1042       | Also a real value that is scaled   |

|                |            | along with the parent. The         |

|                |            | difference between a distance and  |

|                |            | a scale factor is application-     |

|                |            | defined                            |

|----------------|------------|------------------------------------|

| Integer        | 1070       | A 16-bit integer (signed or        |

|                |            | unsigned)                          |

|----------------|------------|------------------------------------|

| Long           | 1071       | A 32-bit signed (long) integer     |

+------------------------------------------------------------------+

For more information on extended entity data and the APPID table,

refer to the "AutoCAD Development System Programmer's Reference" and

the "AutoLISP Programmer's Reference."

Writing DXF Interface Programs

==============================

Writing a program that communicates with AutoCAD via the DXF

mechanism often appears far more difficult than it really is. The

DXF file contains a seemingly overwhelming amount of information,

and examining a DXF file manually may lead to the conclusion that

the task is hopeless.

However, the DXF file has been designed to be easy to process by

program, not manually. The format was intentionally constructed to

make it easy to ignore information you don't need while easily

reading the information you do need. Just remember to handle the

groups in any order and ignore any group you don't care about.

As an example, the following is a Microsoft BASIC program that reads

a DXF file and extracts all the Line entities from the drawing

(ignoring lines that appear inside Blocks). It prints the endpoints

of these lines on the screen. As an exercise you might try entering

this program into your computer, running it on a DXF file from one

of your drawings, then enhancing it to print the center point and

radius of any circles it encounters. This program is not put forward

as an example of clean programming technique nor the way a general

DXF processor should be written; it is presented as an example of

just how simple a DXF-reading program can be.

1000   REM

1010   REM Extract lines from DXF file

1020   REM

1030   G1% = 0

1040   LINE INPUT "DXF file name: "; A$

1050   OPEN "i", 1, A$ + ".dxf"

1060   REM

1070   REM Ignore until section start encountered

1080   REM

1090   GOSUB 2000

1100   IF G% <> 0 THEN 1090

1110   IF S$ <> "SECTION" THEN 1090

1120   GOSUB 2000

1130   REM

1140   REM Skip unless ENTITIES section

1150   REM

1160   IF S$ <> "ENTITIES" THEN 1090

1170   REM

1180   REM Scan until end of section, processing LINEs

1190   REM

1200   GOSUB 2000

1210   IF G% = 0 AND S$ = "ENDSEC" THEN 2200

1220   IF G% = 0 AND S$ = "LINE" THEN GOSUB 1400 : GOTO 1210

1230   GOTO 1200

1400   REM

1410   REM Accumulate LINE entity groups

1420   REM

1430   GOSUB 2000

1440   IF G% = 10 THEN X1 = X : Y1 = Y : Z1 = Z

1450   IF G% = 11 THEN X2 = X : Y2 = Y : Z2 = Z

1460   IF G% = 0 THEN PRINT "Line from (";X1;",";Y1;",";Z1;") to

(";X2;",";Y2;",";Z2;")":RETURN

1470   GOTO 1430

2000   REM

2010   REM Read group code and following value

2020   REM For X coordinates, read Y and possibly Z also

2030   REM

2040   IF G1% < 0 THEN G% = -G1% : G1% = 0 ELSE INPUT #1, G%

2050   IF G% < 10 OR G% = 999 THEN LINE INPUT #1, S$ : RETURN

2060   IF G% >= 38 AND G% <= 49 THEN INPUT #1, V : RETURN

2080   IF G% >= 50 AND G% <= 59 THEN INPUT #1, A : RETURN

2090   IF G% >= 60 AND G% <= 69 THEN INPUT #1, P% : RETURN

2100   IF G% >= 70 AND G% <= 79 THEN INPUT #1, F% : RETURN

2110   IF G% >= 210 AND G% <= 219 THEN 2130

2115   IF G% >= 1000 THEN LINE INPUT #1, T$ : RETURN

2120   IF G% >= 20 THEN PRINT "Invalid group code";G% : STOP

2130   INPUT #1, X

2140   INPUT #1, G1%

2150   IF G1% <> (G%+10) THEN PRINT "Invalid Y coord code"; G1% :

STOP

2160   INPUT #1, Y

2170   INPUT #1, G1%

2180   IF G1% <> (G%+20) THEN G1% = -G1% ELSE INPUT #1, Z

2190   RETURN

2200   CLOSE 1

Writing a program that constructs a DXF file is more difficult,

because you must maintain consistency within the drawing in order

for AutoCAD to find the file acceptable. AutoCAD lets you omit many

items in a DXF file and still obtain a usable drawing. The entire

HEADER section can be omitted if you don't need to set any header

variables. Any of the tables in the TABLES section can be omitted if

you don't need to make any entries, and the entire TABLES section

can be dropped if nothing in it is required. If you define any

linetypes in the LTYPE table, this table must appear before the

LAYER table. If no Block Definitions are used in the drawing, the

BLOCKS section can be omitted. If present, however, the BLOCKS

section must appear before the ENTITIES section. Within the ENTITIES

section, you can reference layer names even though you haven't

defined them in the LAYER table. Such layers are automatically

created with color 7 and the CONTINUOUS linetype. The EOF item must

be present at the end-of-file.

The following Microsoft BASIC program constructs a DXF file

representing a polygon with a specified number of sides, leftmost

origin point, and side length. This program supplies only the

ENTITIES section of the DXF file, and places all entities generated

on the default layer 0. This may be taken as an example of a minimum

DXF generation program. Since this program doesn't create the

drawing header, the drawing limits, extents, and current view will

be invalid after performing a DXFIN on the drawing generated by this

program. You can do a ZOOM E to fill the screen with the drawing

generated. Then adjust the limits manually.

1000   REM

1010   REM Polygon generator

1020   REM

1030   LINE INPUT "Drawing (DXF) file name: "; A$

1040   OPEN "o", 1, A$ + ".dxf"

1050   PRINT #1, 0

1060   PRINT #1, "SECTION"

1070   PRINT #1, 2

1080   PRINT #1, "ENTITIES"

1090   PI = ATN(1) * 4

1100   INPUT "Number of sides for polygon: "; S%

1110   INPUT "Starting point (X,Y): "; X, Y

1120   INPUT "Polygon side: "; D

1130   A1 = (2 * PI) / S%

1140   A = PI / 2

1150   FOR I% = 1 TO S%

1160   PRINT #1, 0

1170   PRINT #1, "LINE"

1180   PRINT #1, 8

1190   PRINT #1, "0"

1200   PRINT #1, 10

1210   PRINT #1, X

1220   PRINT #1, 20

1230   PRINT #1, Y

1240   PRINT #1, 30

1250   PRINT #1, 0.0

1260   NX = D * COS(A) + X

1270   NY = D * SIN(A) + Y

1280   PRINT #1, 11

1290   PRINT #1, NX

1300   PRINT #1, 21

1310   PRINT #1, NY

1320   PRINT #1, 31

1330   PRINT #1, 0.0

1340   X = NX

1350   Y = NY

1360   A = A + A1

1370   NEXT I%

1380   PRINT #1, 0

1390   PRINT #1, "ENDSEC"

1400   PRINT #1, 0

1410   PRINT #1, "EOF"

1420   CLOSE 1

The DXFIN command is relatively forgiving with respect to the format

of data items. As long as a properly formatted item appears on the

line on which the data is expected, DXFIN will accept it (of course,

string items should not have leading spaces unless these are

intended to be part of the string). This program takes advantage of

this flexibility in input format, and does not try to generate a

file appearing exactly like one generated by AutoCAD.

In the case of error loading a DXF file using DXFIN, AutoCAD reports

the error with a message indicating the nature of the error and the

last line processed in the DXF file before the error was detected.

This may not be the line on which the error occurred, especially in

the case of errors such as omission of required groups.

Binary Drawing Interchange Files

********************************

The ASCII DXF file format described in the preceding sections of

this chapter is a complete representation of an AutoCAD drawing in

an ASCII text form easily processed by other programs. In addition,

AutoCAD can produce or read a binary form of the full DXF file, and

accepts limited input in another binary file format. These binary

files are described in the following sections.

Binary DXF Files

================

The DXFOUT command provides a Binary option that writes binary DXF

files. Such a file contains all of the information present in an

ASCII DXF file, but in a more compact form that takes, typically,

25% less file space and can be read and written more quickly

(typically 5 times faster) by AutoCAD. Unlike ASCII DXF files, which

entail a trade-off between size and floating-point accuracy, binary

DXF files preserve all of the accuracy in the drawing database.

AutoCAD Release 10 was the first version to support this form of DXF

file; it cannot be read by older versions.

A binary DXF file begins with a 22-byte sentinel consisting of:

AutoCAD Binary DXF<CR><LF><SUB><NUL>

Following the sentinel are (group, value) pairs as in an ASCII DXF

file, but represented in binary form. The group code is a single-

byte binary value, and the value that follows is one of the

following:

o  A two-byte integer with the least-significant byte first and the

most-significant byte last.

o  An eight-byte IEEE double precision floating-point number stored

with the least-significant byte first and the most-significant

byte last.

o  An ASCII string terminated by a zero (NUL) byte.

The type of the datum following a group is determined from the group

code according to the same rules used in decoding ASCII DXF files.

Translation of angles to degrees, and dates to fractional Julian

date representation, is performed for binary files as well as for

ASCII DXF files. The comment group, 999, is not used in binary DXF

files.

Extended entity data group codes are represented in Binary DXF as a

single byte with the value 255, followed by a 2-byte integer value

containing the actual group code, followed by the actual value.

Extended entity data long (group code 1071) values occupy 4 bytes of

data. Extended entity data binary chunks (group code 1004) are

represented as a single-byte, unsigned integer length, followed by

the specified number of bytes of chunk data. For example, to

transfer an extended entity data long group, the following values

would appear, occupying 1, 2, and 4 bytes respectively:

255       Escape group code.

1071      True group code.

999999    Value for the 1071 group code.

DXFOUT writes binary DXF files with the same file type (.dxf) as for

ASCII DXF files. The DXFIN command automatically recognizes a binary

file (by means of its sentinel string) and loads the file. There is

no need for you to identify it as a binary file.

If DXFIN encounters an error in a binary DXF file, it reports the

byte address within the file where the error was detected.

Binary Drawing Interchange (DXB) Files

**************************************

The DXF file formats described earlier in this chapter are complete

representations of an AutoCAD drawing that can be written and read

by AutoCAD and other programs. However, AutoShade and programs

executed via the external commands facility (chapter 3) often  need

to supply simple geometric input to AutoCAD. For these purposes,

another file format even more compact than the binary DXF format is

supported. This format, called DXB (for drawing interchange binary)

is limited in the entities it can represent.

DXBIN Command

=============

To load a DXB file produced by a program such as AutoShade, enter

the DXBIN command:

Command: dxbin

When AutoCAD prompts you, respond with the name of the file you want

to load. You don't need to include a file type; .dxb is assumed.

DXB File Format

===============

IMPORTANT: This information is for experienced programmers and is

subject to change without notice.

The format of a DXB file is as follows:

Header: "AutoCAD DXB 1.0" CR LF ^Z NUL   (19 bytes)

Data: Zero or more data records

Terminator: NUL                          (1 byte)

Each data record begins with a single byte identifying the record

type, followed by data items. The data items have various forms of

representation and encoding. In the descriptions following, each

data item is prefixed with a letter and a hyphen. The meaning of the

letter codes is as follows:

w-   16-bit integer, byte reversed in the standard 80x86 style

(least- significant byte first, most-significant byte second).

f-   IEEE 64-bit floating-point value stored with lsb first, msb

last (as stored by an 80x87).

l-   32-bit integer with the bytes reversed 80x86 style.

n-   Number which may be either a 16-bit integer or a floating-point

number depending on the most recent setting of the number mode

data item. The number mode defaults to 0, signifying integers.

If set to 1, all n- items will be read as floating-point.

u-   Item which is either a 32-bit integer or a floating-point

number depending on the most recent number mode setting. If a

32-bit integer, the value is scaled by multiplying it by 65536

(2^16). If a floating-point value, no scaling is applied.

a-   Item representing an angle. If number mode is integer, this is

a 32-bit integer representing an angle in units of millionths

of a degree (range 0 to 360,000,000). If a floating-point

number, represents degrees.

In the following table, the lengths include the item-type byte and

assume the number mode is set to zero (integer mode). If number mode

is floating-point, add 6 bytes to the length for each n- item

present and 4 bytes for each a-, or u- item present.

Table 11-20. Byte length for item types

+==================================================================+

| Item type     | Code     | Data items                 | Length   |

|               |(decimal) |                            |(bytes)   |

|---------------|----------|----------------------------|----------|

| Line          | 1        | n-fromx n-fromy            | 13       |

|               |          | n-tox n-toy                |          |

|               |          | n-fromx n-fromy n-fromz    |          |

|               |          | n-tox n-toy n-toz          |          |

|---------------|----------|----------------------------|----------|

| Point         | 2        | n-x n-y                    | 5        |

|---------------|----------|----------------------------|----------|

| Circle        | 3        | n-ctrx n-ctry n-rad        | 7        |

|---------------|----------|----------------------------|----------|

| Arc           | 8        | n-ctrx n-ctry n-rad        | 19       |

|               |          | a-starta a-enda            |          |

|---------------|----------|----------------------------|----------|

| Trace         | 9        | n-x1 n-y1 n-x2 n-y2        | 17       |

|               |          | n-x3 n-y3 n-x4 n-y4        |          |

|---------------|----------|----------------------------|----------|

| Solid         | 11       | n-x1 n-y1 n-x2 n-y2        | 17       |

|               |          | n-x3 n-y3 n-x4 n-y4        |          |

|---------------|----------|----------------------------|----------|

| Seqend        | 17       | (none)                     | 1        |

|---------------|----------|----------------------------|----------|

| Polyline      | 19       | w-closureflag              | 3        |

|---------------|----------|----------------------------|----------|

| Vertex        | 20       | n-x n-y                    | 5        |

|---------------|----------|----------------------------|----------|

| 3Dface        | 22       | n-x1 n-y1 n-z1             | 25       |

|               |          | n-x2 n-y2 n-z2             |          |

|               |          | n-x3 n-y3 n-z3             |          |

|               |          | n-x4 n-y4 n-z4             |          |

|---------------|----------|----------------------------|----------|

| Scale Factor  | 128      | f-scalefac                 | 9        |

|---------------|----------|----------------------------|----------|

| New Layer     | 129      | "layername" NUL            |layername |

|               |          |                            |length + 2|

|---------------|----------|----------------------------|----------|

| Line          | 130      | n-tox n-toy                | 5        |

| Extension     |          |                            |          |

|---------------|----------|----------------------------|----------|

| Trace         | 131      | n-x3 n-y3 n-x4 n-y4        | 9        |

| Extension     |          |                            |          |

|---------------|----------|----------------------------|----------|

| Block Base    | 132      | n-bx n-by                  | 5        |

|---------------|----------|----------------------------|----------|

| Bulge         | 133      | u-2h/d                     | 5        |

|---------------|----------|----------------------------|----------|

| Width         | 134      | n-startw n-endw            | 5        |

|---------------|----------|----------------------------|----------|

| Number Mode   | 135      | w-mode                     | 3        |

|---------------|----------|----------------------------|----------|

| New Color     | 136      | w-colornum                 | 3        |

|---------------|----------|----------------------------|----------|

| 3Dline        | 137      | n-tox n-toy n-toz          | 7        |

| Extension     |          |                            |          |

+------------------------------------------------------------------+

The Line Extension item extends the last line or line extension from

its To point to a new To point:. The Trace Extension item similarly

extends the last trace solid, or Trace Extension from its x3,y3-

x4,y4 ending line to a new x3,y3--x4,y4 line.

The Scale Factor is a floating-point value by which all integer

coordinates are multiplied to obtain the floating-point coordinates

used by the actual entities. The initial scale factor when a file is

read is 1.0. The New Layer item creates a layer if none exists,

giving the new layer the same defaults as the LAYER New command, and

sets that layer as the current layer for subsequent entities. At the

end of the DXB file load, the layer in effect before the command is

restored.

The Block Base item specifies the base (origin) point of a created

Block. The Block base must be defined before the first entity record

is encountered. If DXB is not defining a Block, this specification

will be ignored.

A Polyline consists of straight segments of fixed width connecting

the vertices, except as overridden by the Bulge and Width items

described below. The closure flag should be 0 or 1; if it is 1, then

there is an implicit segment from the last vertex (immediately

before the Seqend) to the first vertex.

A Bulge item, encountered between two Vertex items (or after the

last Vertex of a closed Polyline), indicates that the two vertices

are connected by an arc rather than a straight segment. If the line

segment connecting the vertices would have length d, and the

perpendicular distance from the midpoint of that segment to the arc

is h, then the magnitude of the Bulge is (2 * h / d). The sign is

negative if the arc from the first vertex to the second is

clockwise. A semicircle thus has a bulge of 1 (or -1). If the number

mode is 0 (integer), Bulge items are scaled by 2 16. If the number

mode has been set to floating-point, then the floating-point value

supplied is just 2*h/d (not scaled).

The Width item indicates the starting and ending widths of the

segment (straight or curved) connecting two vertices. This width

stays in effect until the next width item or the Seqend. If there is

a Width item between the Polyline item and the first Vertex, it is

stored as a default width for the Polyline; this saves considerable

database space if the Polyline has several segments of this width.

The Number Mode item controls the mode of items with types given in

the table above as n-, a-, or u-. If the value supplied is zero,

these values will be integers, otherwise floating-point. The storage

and implicit scaling conventions for these values in both modes are

described earlier.

Lines share the same cells to remember the last to-point, so you

shouldn't mix extension groups for the two entities without an

initial group before the extension. There is no extension group for

3Dfaces, as there's no obvious edge to extend from.

The New Color group specifies the color for subsequent entities in

the DXB file. The w-colornum word argument is in the range from 0 to

256. 0 means color by block, 1-255 are the standard AutoCAD colors,

and 256 means color by layer. A color outside the range from 0 to

256 sets the color back to the current entity color (you can do this

deliberately, and it can be quite handy). The initial entity color

of material added by DXBIN is the current entity color.

All points specified in the DXB file are interpreted in terms of the

current UCS at the time the DXBIN command is executed.

Writing DXB Files

=================

There is no direct AutoCAD command to write a DXB file, but the

special ADI plotter driver can write such a file. If you want to

create a DXB file from an AutoCAD drawing, configure the ADI plotter

and select its DXB file output option.

Initial Graphics Exchange Specification (IGES) Files

****************************************************

Using the commands described in this section, you can instruct

AutoCAD to read and write IGES-format interchange files.

NOTE: The format of IGES files and the mapping performed to

translate between AutoCAD drawing information and IGES are described

in the separate AutoCAD/IGES Interface Specifications document.

IGESOUT Command

===============

You can generate an Initial Graphics Exchange Specification (IGES)

interchange file from an existing AutoCAD drawing by means of the

IGESOUT command:

Command: igesout

When AutoCAD prompts you, respond with a filename or press 5 to

accept the default.

The default name for the output file is the same as that of the

current drawing, but with a file type of .igs. If you specify an

explicit filename without including a file type, .igs is assumed. If

a file with the same name already exists, it is deleted. If FILEDIA

is on, and a file with the same name already exists, AutoCAD tells

you; allowing you to OK or cancel the deletion.

IGESIN Command

==============

An IGES interchange file can be converted into an AutoCAD drawing by

means of the IGESIN command:

Command: igesin

When AutoCAD prompts you, respond with the name of the IGES file to

be loaded.

To load a complete IGES file, you must use IGESIN in an empty

drawing, before any entities have been drawn and before any

additional Block definitions, layers, linetypes, text styles, named

views, named coordinate systems, or named viewport configurations

have been created.

NOTE: If the drawing you are using as a prototype is not empty, you

might find it helpful to open a new drawing using the No

Prototype... button of the Create New Drawing dialogue box, as

described in chapter 4 of the AutoCAD Reference Manual. You should

also be aware that some third-party applications include an acad.lsp

or .mnl file that modifies your drawing upon startup.

If a serious error is encountered, the input process stops and an

error message is displayed reporting where the error was found. The

partial drawing is not discarded.